RECOMMENDATION  ITU-R  BR.602-3

EXCHANGE  OF  TELEVISION  RECORDINGS  FOR  PROGRAMME  EVALUATION

(Questions ITU-R 18/11 and ITU-R 105/11)

(1982-1990-1992-1994)

Rec. ITU-R BR.602-3

The ITU Radiocommunication Assembly,

considering

a)	that a significant number of television recordings are exchanged by broadcasting organizations for purposes of programme evaluation;

b)	that the VHS format (specified in the International Electrotechnical Commission (IEC) Publication 774) provides good interchangeability between video cassettes recorded on machines supplied by several manufacturers as well as advantages in price, cassette size and playing time;

c)	that this format provides programme quality consistent in time and adequate for viewing purposes and programme evaluation;

d)	that this format was originally designed for the consumer market; consequently:

–	the recorders and the cassettes are comparatively inexpensive;

–	the recorders can be easily operated;

–	they are comparatively reliable and robust;

–	they are quite widespread and easily available;

e)	that the use of tape cassettes offers advantages in handling and shipping,

recommends

1.	that the use of video cassettes conforming with the VHS format should be used for the international exchange of recorded programmes for programme evaluation, in both the 625-lines, 50 field/s and the 525-lines, 60 field/s standards;

Note 1 – Several formats may be used for the same application subject to prior agreement. These are S-VHS format (IEC/SC 60B (Secretariat) 199), Video 8 format (IEC Publication 843), and Hi8 format (IEC/SC 60B (Secretariat) 198).

2.	that:

–	only the “normal speed” recording mode (standard play) should be used;

–	the sound must be recorded on the two longitudinal tracks;

–	for stereo sound, the left-channel signal must be recorded on track 1 (inner track) and the right-channel signal on track 2 (edge track).