Recommendation ITU-R BS.2094-2

(02/2025)

BS Series: Broadcasting service (sound)

Common definitions for the audio definition model

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from https://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2025

ITU 2025

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  BS.2094-2

Common definitions for the audio definition model

(2016-2017-2025)

Scope

This Recommendation contains a set of common definitions for multichannel audio configurations that use the audio definition model (Recommendation ITU-R BS.2076) to describe them.

Keywords

ADM, audio, multichannel, channel-based, tracks, metadata, bw64, exchange, audio programme, BWF, immersive, HOA, Higher Order Ambisonics

The ITU Radiocommunication Assembly,

considering

a)	that storage media based on information technology, including data disks and tapes, have penetrated all areas of audio production for radio broadcasting, namely non-linear editing, on-air play-out and archives;

b)	that the adoption of a single file format for signal interchange would greatly simplify the interoperation of individual pieces of equipment, and remote studios, and it would facilitate the desirable integration of editing, on-air play-out, and archiving;

c)	that compatibility with currently-available commercial file formats would minimize the industry efforts required to implement a new format in the equipment;

d)	that future audio systems will require metadata associated with the audio to be carried in the file;

e)	that future audio systems will use a variety of multichannel configurations including channel-, object-, and scene-based audio such as specified in Recommendation ITU-R BS.2051;

f)	that future audio systems will use the audio definition model (Recommendation ITU-R BS.2076) to describe the technical format of the audio being delivered and exchanged;

g)	that the majority of audio in existence and produced in the short-term future is and will be channel-based using a commonly used set of configurations;

h)	that using different metadata descriptions for identical audio configurations will cause compatibility problems and unnecessary overheads,

recommends

that, for exchanging any audio that uses the audio definition model (Recommendation ITU-R BS.2076) as the metadata model, a set of common definitions be used to describe any audio configurations as defined in Annex 1.

TABLE OF CONTENTS

Page

Policy on Intellectual Property Right (IPR)		ii

Annex 1 – (normative)  Common definitions for the audio definition model		3

1	Introduction		3

2	Contents of the common definitions		3

3	Common definitions usage		3

4	Set of common definitions		4

4.1	Common definitions for the ‘DirectSpeakers’ audio type		5

4.1.1	Common audioChannelFormats for ‘DirectSpeakers’		5

4.1.2	Common audioStreamFormats and audioTrackFormats for ‘DirectSpeakers’		8

4.1.3	Common audioPackFormats for ‘DirectSpeakers’		8

4.2	Common definitions for the ‘Matrix’ audio type		30

4.3	Common definitions for the ‘Objects’ audio type		30

4.4	Common definitions for the ‘HOA’ audio type		31

4.4.1	Common audioChannelFormats for ‘HOA’		31

4.4.2	Common audioPackFormats for ‘HOA’		32

4.5	Common definitions for the ‘Binaural’ audio type		36

4.5.1	Common audioChannelFormats for ‘Binaural’		37

4.5.2	Common audioStreamFormats and audioTrackFormats for ‘Binaural’		37

4.5.3	Common audioPackFormats for ‘Binaural’		37

4.6	Using URIs		38

5	Attachments		38

Annex 2 (informative) – Overview of changes in this edition (Recommendation ITU-R BS.2094-2)		39

Annex 1 
(normative)

Common definitions for the audio definition model

# 1	Introduction

Recommendation ITU-R BS.2076 ‒ Audio Definition Model, is a metadata model used to describe the technical content and format of audio. It can be used to describe any type of audio signal to allow it to be rendered correctly, whether it be object-, scene-, or channel-based. While the audio definition model (ADM) is extremely flexible and allows any type of audio format to be defined, the vast majority of audio in existence is based on a few commonly used channel-based configurations. Therefore, it is not efficient for these commonly used formats to need to be explicitly defined every time they are used. It would also be problematic for identical formats to be defined in different ways by different organisations or applications.

To provide consistency and efficiency in the use of the ADM, a set of common definitions has been drawn up. These are based on what is commonly used in the audio industry, including those specified in Recommendations ITU-R BS.2051 and ITU-R BS.775 to produce definitions that should be used when these configurations are used in files and streams. This set of common definitions is intended to be a living resource, with new definitions added in the future should they be required.

# 2	Contents of the common definitions

As the vast majority of existing audio (and in the short-term future) is channel-based, most of the common definitions will be for channel-based audio. The initial set will concentrate on commonly used channel-based configurations. The other assumption used is that the track formats are pulse code modulation (PCM), so it does not include coded audio.

The ADM consists of several different elements used for defining audio. They either describe the content or the format. The set of common definitions is only concerned about the format elements as these can be defined without knowledge of the content of the audio. These elements are:

–	audioTrackFormat

–	audioStreamFormat

–	audioChannelFormat

–	audioBlockFormat

–	audioPackFormat

Apart from audioPackFormat the elements are very closely tied together with one of each used for each channel definition. However, it does not mean that all four of these elements are required to be used at all times. It is perfectly acceptable to use a common audioChannelFormat/ audioBlockFormat definition with custom-made audioTrackFormat and audioStreamFormat definitions.

# 3	Common definitions usage

The common definitions exist as an extensible markup language (XML) file which can be either stored locally with the application that is handling the audio files, or referred to remotely. It is not intended to be carried in the audio file itself, as one of the intentions of it is to reduce the amount of metadata required to be carried in audio files.

Any code that reads audio files that contain ADM-defined audio (so BWF, BW64, RF64 typically) should always aim to read the common definitions first before reading the ADM metadata stored within the audio file itself. The relationship between the audio file and the common definitions file is shown in Fig. 1.

Figure 1

Relationship between audio file and common definitions file

The <chna> chunk contains an ID reference to both an audioTrackFormat and an audioPackFormat definition for each track in the file. These references should be looked up in the common definitions file first to see whether that contains the IDs, and if not then the audio file’s <axml> chunk should be referred to. When inspecting the content-related ADM metadata in the <axml> chunk, in particular the audioObject elements, it may have references to IDs that exist in the common definitions file, most probably audioPackFormat IDs. This ordering of the chunks is not strict, and it may be preferable to place the <axml> chunk after the <data> chunk.

# 4	Set of common definitions

The set of common definitions consists of commonly used channel-based configurations, some of which are recognised standards and some are common proprietary configurations. The definitions consist of a set of audioChannelFormat definitions for channels associated with many different speaker locations, including all those in Recommendation ITU-R BS.2051. Each of these channel definitions has an associated audioStreamFormat and audioTrackFormat definition for PCM audio signals to cover the most typical use case. The set of audioPackFormat definitions for the various speaker combinations use only channels from this common set.

The audioChannelFormat and the audioPackFormat IDs follow this format:

–	audioChannelFormatID: AC_yyyy0xxx

–	audioPackFormatID: AP_yyyy0xxx

where the first four hexadecimal digits (yyyy) indicate what the type of audio is. If this value is set to 0001 then the type is ‘DirectSpeakers’, if it is 0002 then it is ‘Matrix’, 0003 for ‘Objects’, 0004 for ‘HOA’ and 0005 for ‘Binaural’. The final four digits (xxxx) have values below 1000, so belong to the set of common definitions as listed here.

## 4.1	Common definitions for the ‘DirectSpeakers’ audio type

### 4.1.1	Common audioChannelFormats for ‘DirectSpeakers’

TABLE 1A

audioChannelFormat definitions for ‘DirectSpeakers’ for polar coordinate system

| audioChannelFormatID | audioChannelFormatName | Azimuth | Elevation | speakerLabel |
| --- | --- | --- | --- | --- |
| AC_00010001 | FrontLeft | 30 | 0 | M+030 |
| AC_00010002 | FrontRight | −30 | 0 | M-030 |
| AC_00010003 | FrontCentre | 0 | 0 | M+000 |
| AC_00010004 (1) | LowFrequencyEffects | 0 | −30 | LFE |
| AC_00010005 | SurroundLeft | 110 | 0 | M+110 |
| AC_00010006 | SurroundRight | −110 | 0 | M-110 |
| AC_00010007 | FrontLeftOfCentre | 22.5 | 0 | M+022 |
| AC_00010008 | FrontRightOfCentre | −22.5 | 0 | M-022 |
| AC_00010009 | BackCentre | 180 | 0 | M+180 |
| AC_0001000a | SideLeft | 90 | 0 | M+090 |
| AC_0001000b | SideRight | −90 | 0 | M-090 |
| AC_0001000c | TopCentre | 0 | 90 | T+000 |
| AC_0001000d | TopFrontLeft | 30 | 30 | U+030 |
| AC_0001000e | TopFrontCentre | 0 | 30 | U+000 |
| AC_0001000f | TopFrontRight | −30 | 30 | U-030 |
| AC_00010010 | TopSurroundLeft | 110 | 30 | U+110 |
| AC_00010011 | TopBackCentre | 180 | 30 | U+180 |
| AC_00010012 | TopSurroundRight | −110 | 30 | U-110 |
| AC_00010013 | TopSideLeft | 90 | 30 | U+090 |
| AC_00010014 | TopSideRight | −90 | 30 | U-090 |
| AC_00010015 | BottomFrontCentre | 0 | −30 | B+000 |
| AC_00010016 | BottomFrontLeftMid | 45 | −30 | B+045 |
| AC_00010017 | BottomFrontRightMid | −45 | −30 | B-045 |
| AC_00010018 | FrontLeftWide | 60 | 0 | M+060 |
| AC_00010019 | FrontRightWide | −60 | 0 | M-060 |
| AC_0001001a | N/A (see Note) |  |  |  |
| AC_0001001b | N/A (see Note) |  |  |  |
| AC_0001001c | BackLeftMid | 135 | 0 | M+135 |
| AC_0001001d | BackRightMid | −135 | 0 | M-135 |
| AC_0001001e | TopBackLeftMid | 135 | 30 | U+135 |

TABLE 1A (end)

| audioChannelFormatID | audioChannelFormatName | Azimuth | Elevation | speakerLabel |
| --- | --- | --- | --- | --- |
| AC_0001001f | TopBackRightMid | −135 | 30 | U-135 |
| AC_00010020 (1) | LowFrequencyEffects1 | 45 | −30 | LFE1 |
| AC_00010021 (1) | LowFrequencyEffects2 | −45 | −30 | LFE2 |
| AC_00010022 | TopFrontLeftMid | 45 | 30 | U+045 |
| AC_00010023 | TopFrontRightMid | −45 | 30 | U-045 |
| AC_00010024 | FrontLeftScreen | Left screen edge (or 25 if unknown) | 0 | M+SC |
| AC_00010025 | FrontRightScreen | Right screen edge (or −25 if unknown) | 0 | M-SC |
| AC_00010026 | FrontLeftMid | 45 | 0 | M+045 |
| AC_00010027 | FrontRightMid | −45 | 0 | M-045 |
| AC_00010028 | UpperTopBackCentre | 180 | 45 | UH+180 |
| AC_00010029 | BackLeft | 150 | 0 | M+150 |
| AC_0001002a | BackRight | −150 | 0 | M-150 |
| AC_0001002b | BottomFrontLeft | 30 | −30 | B+030 |
| AC_0001002c | BottomFrontRight | −30 | −30 | B-030 |
| (1) These channels also feature a low pass frequency cut-off of 120 Hz. For further characteristics of LFE signal, refer to Annex 7 of Recommendation ITU-R BS.775. NOTE – Earlier versions (Recommendation ITU-R BS.2094-0 and ITU-R BS.2094-1) of this Recommendation contained the common definitions for these audioChannelFormatIDs and audioChannelFormatNames but have now been deprecated. These audioChannelFormatIDs shall not be used for other audioChannelFormats to avoid compatibility issues. |  |  |  |  |

TABLE 1B

audioChannelFormat definitions for ‘DirectSpeakers’ for Cartesian coordinate system

| audioChannelFormatID | audioChannelFormatName | X | Y | Z | speakerLabel (2) |
| --- | --- | --- | --- | --- | --- |
| AC_00010801 | FrontLeft_Cart | −1 | 1 | 0 | M_FL |
| AC_00010802 | FrontRight_Cart | 1 | 1 | 0 | M_FR |
| AC_00010803 | FrontCentre_Cart | 0 | 1 | 0 | M_FC |
| AC_00010804 (1) | LowFrequencyEffects_Cart | 0 | 1 | −1 | LFE |
| AC_00010805 | BackLeft_Cart | −1 | −1 | 0 | M_BL |
| AC_00010806 | BackRight_Cart | 1 | −1 | 0 | M_BR |
| AC_00010807 | FrontLeftCentreMid_Cart | −0.5 | 1 | 0 | M_FL50 |
| AC_00010808 | FrontRightCentreMid_Cart | 0.5 | 1 | 0 | M_FR50 |
| AC_00010809 | BackCentre_Cart | 0 | −1 | 0 | M_BC |
| AC_0001080a | SideLeft_Cart | −1 | 0 | 0 | M_SL |
| AC_0001080b | SideRight_Cart | 1 | 0 | 0 | M_SR |

TABLE 1B (end)

| audioChannelFormatID | audioChannelFormatName | X | Y | Z | speakerLabel (2) |
| --- | --- | --- | --- | --- | --- |
| AC_0001080c | TopCentre_Cart | 0 | 0 | 1 | U_TC |
| AC_0001080d | TopFrontLeft_Cart | −1 | 1 | 1 | U_FL |
| AC_0001080e | TopFrontCentre_Cart | 0 | 1 | 1 | U_FC |
| AC_0001080f | TopFrontRight_Cart | 1 | 1 | 1 | U_FR |
| AC_00010810 | TopBackLeft_Cart | −1 | −1 | 1 | U_BL |
| AC_00010811 | TopBackCentre_Cart | 0 | −1 | 1 | U_BC |
| AC_00010812 | TopBackRight_Cart | 1 | −1 | 1 | U_BR |
| AC_00010813 | TopSideLeft_Cart | −1 | 0 | 1 | U_SL |
| AC_00010814 | TopSideRight_Cart | 1 | 0 | 1 | U_SR |
| AC_00010815 | BottomFrontCentre_Cart | 0 | 1 | −1 | B_FC |
| AC_00010816 | BottomFrontLeft_Cart | −1 | 1 | −1 | B_FL |
| AC_00010817 | BottomFrontRight_Cart | 1 | 1 | −1 | B_FR |
| AC_00010818 | FrontSideLeftFar_Cart | −1 | 0.667 | 0 | M_FSL67 |
| AC_00010819 | FrontSideRightFar_Cart | 1 | 0.667 | 0 | M_FSR67 |
| AC_00010820 (1) | LowFrequencyEffects1_Cart | −1 | 1 | −1 | LFE1 |
| AC_00010821 (1) | LowFrequencyEffects2_Cart | 1 | 1 | −1 | LFE2 |
| AC_00010824 | FrontLeftScreen_Cart | Left screen edge (or −0.5 if unknown) | 1 | 0 | M_FLSc |
| AC_00010825 | FrontRightScreen_Cart | Right screen edge (or 0.5 if unknown) | 1 | 0 | M_FRSc |
| AC_00010828 | TopBackCentreFar_Cart | 0 | −0.667 | 1 | U_BC67 |
| AC_00010829 | BackLeftCentreMid_Cart | −0.5 | −1 | 0 | M_BL50 |
| AC_0001082a | BackRightCentreMid_Cart | 0.5 | −1 | 0 | M_BR50 |
| (1) These channels also feature a low pass frequency cut-off of 120 Hz. For further characteristics of LFE signal, refer to Annex 7 of Recommendation ITU-R BS.775. (2) The speakerLabel of an audioChannelFormat in the Cartesian coordinate system uniquely identifies a channel in the “virtual cube”. The loudspeaker, as defined in Recommendation ITU-R BS.2051, that this channel will be played back on is undetermined until the target system layout is known. Table 2B in this document provides speakerLabel mappings for the common definition audioPackFormats when the target system layout matches the audioPackFormat. |  |  |  |  |  |

Each channel definition is listed in Table 1A, where the audioChannelFormatID, audioChannelFormatName and speakerLabel elements are listed. The azimuth and elevation columns represent the position that would be specified within the position sub-element (the distance is 1.0 for all these channel definitions).

The XML below shows how the first entry in Table 1A is represented in XML.

<audioChannelFormat audioChannelFormatID="AC_00010001" audioChannelFormatName="FrontLeft" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AC_00010001_00000001"> <speakerLabel>urn:itu:bs:2051:0:speaker:M+030</speakerLabel> <position coordinate="azimuth">30.0</position> <position coordinate="elevation">0.0</position> <position coordinate="distance">1.0</position> </audioBlockFormat> </audioChannelFormat>

### 4.1.2	Common audioStreamFormats and audioTrackFormats for ‘DirectSpeakers’

The common format type for audioStreamFormat and audioTrackFormat is ‘PCM’.

As previously mentioned, the audioStreamFormat and audioTrackFormat, which relate to each audioChannelFormat definition use the same ID prefixes and the same names with ‘PCM_’ appended, such as ‘PCM_FrontLeft’. The XML code below shows both the audioStreamFormat and audioTrackFormat definition for the first entry in the Table.

<audioStreamFormat audioStreamFormatID="AS_00010001" audioStreamFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00010001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioTrackFormat audioTrackFormatID="AT_00010001_01" audioTrackFormatName="PCM_FrontLeft" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00010001</audioStreamFormatIDRef> </audioTrackFormat>

### 4.1.3	Common audioPackFormats for ‘DirectSpeakers’

The audioPackFormat definitions cover a range of speaker configurations. Tables 2A and 2B show the set of configurations that have been defined. The ones that are in Recommendation ITU-R BS.2051 are indicated. The last column gives the references to each channel the pack consists of. Instead of giving the whole ID (e.g. AP_00010001), only the last two digits are listed for clarity; so they would be AP_000100xx where xx is the digits listed to give the full audioPackFormatIDRef.

The audioPackFormat names have been given both a traditional name and the name formatted in the style of Recommendation ITU-R BS.2051 (U+M+L) separated by an underscore. Spaces are not used in the names as they can cause problems.

To simplify mapping, each audioPackFormat definition in Table 2A has a corresponding entry in Table 2B.

The audioPackFormat elements defined in Tables 2A and 2B for each of the coordinate systems have been designed so that the last 2-digits of the audioPackFormatIDs from one coordinate system corresponds to the other coordinate system. The 3 last digits in the range of 0x001 to 0x7ff indicates a polar based pack, 0x800 to 0xfff indicates a Cartesian based pack.

TABLE 2A

audioPackFormat definitions for ‘DirectSpeakers’ for polar coordinate system

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| AP_00010001 mono_(0+1+0) (BS.775) | AC_00010003 | FrontCentre | M+000 | Mono |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| AP_00010002 stereo_(0+2+0) (BS.775, BS.2051 (A)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
| AP_0001000a 3.0_(0+3+0) (BS.775) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
| AP_0001000b 4.0_(0+4+0) (BS.775) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010009 | BackCentre | M+180 | Mono Surround |
| AP_0001000c 5.0_(0+5+0) (BS.775, BS.2051 (B) without LFE) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
| AP_00010003 5.1_(0+5+0) (BS.775, BS.2051 (B)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
| AP_0001000d 6.1_(0+6+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010009 | BackCentre | M+180 | Centre Surround |
| AP_0001000e 7.1front_(0+7+0) (ISO/IEC 23091-3:2018) | AC_00010001 | FrontLeft | M+030 | Left Front Centre |
|  | AC_00010002 | FrontRight | M-030 | Right Front Centre |
|  | AC_00010003 | FrontCentre | M+000 | Centre Front |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010026 | FrontLeftMid | M+045 | Left Front |
|  | AC_00010027 | FrontRightMid | M-045 | Right Front |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| AP_0001001b 7.0back_(0+7+0) (BS.2051 (I) without LFE) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
| AP_0001000f 7.1back_(0+7+0) (BS.2051 (I)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
| AP_0001001c 7.0top_(2+5+0) (BS.2051 (C) without LFE) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Left Top Front |
|  | AC_0001000f | TopFrontRight | U-030 | Right Top Front |
| AP_00010004 7.1top_(2+5+0) (BS.2051 (C)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Left Top Front |
|  | AC_0001000f | TopFrontRight | U-030 | Right Top Front |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| AP_00010012 7.1screen_5.1+sc_(0+7+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010024 | FrontLeftScreen | M+SC | Front Left Screen |
|  | AC_00010025 | FrontRightScreen | M-SC | Front Right Screen |
| AP_0001001d 7.0topside_5.0.2_(2+5+0) (N/A without LFE) | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
| AP_00010013 7.1topside_5.1.2_(2+5+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
| AP_00010020 Auro-3D_8.0_(4+4+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Height Left |
|  | AC_0001000f | TopFrontRight | U-030 | Height Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Height Left Surround |
|  | AC_00010012 | TopSurroundRight | U-110 | Height Right Surround |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| AP_00010014 9.1screen_5.1.2+sc_(2+7+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
|  | AC_00010024 | FrontLeftScreen | M+SC | Front Left Screen |
|  | AC_00010025 | FrontRightScreen | M-SC | Front Right Screen |
| AP_00010016 9.1topside_7.1.2_(2+7+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Back Left |
|  | AC_0001001d | BackRightMid | M-135 | Back Right |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
| AP_0001001e 9.0_5.0.4_(4+5+0) (BS.2051 (D) without LFE) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Left Top Front |
|  | AC_0001000f | TopFrontRight | U-030 | Right Top Front |
|  | AC_00010010 | TopSurroundLeft | U+110 | Left Top Rear |
|  | AC_00010012 | TopSurroundRight | U-110 | Right Top Rear |
| AP_00010005 9.1_5.1.4_(4+5+0) (BS.2051 (D)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Left Top Front |
|  | AC_0001000f | TopFrontRight | U-030 | Right Top Front |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
|  | AC_00010010 | TopSurroundLeft | U+110 | Left Top Rear |
|  | AC_00010012 | TopSurroundRight | U-110 | Right Top Rear |
| AP_00010006 10.1_(4+5+1) (BS.2051 (E)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Left Top Front |
|  | AC_0001000f | TopFrontRight | U-030 | Right Top Front |
|  | AC_00010010 | TopSurroundLeft | U+110 | Left Top Rear |
|  | AC_00010012 | TopSurroundRight | U-110 | Right Top Rear |
|  | AC_00010015 | BottomFrontCentre | B+000 | Centre Bottom Front |
| AP_00010021 Auro-3D_10.1_ (5+5+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Height Left |
|  | AC_0001000f | TopFrontRight | U-030 | Height Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Height Left Surround |
|  | AC_00010012 | TopSurroundRight | U-110 | Height Right Surround |
|  | AC_0001000c | TopCentre | T+000 | Top |
| AP_00010007 10.2_(3+7+0) (BS.2051 (F)) | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Left Height |
|  | AC_00010023 | TopFrontRightMid | U-045 | Right Height |
|  | AC_0001000a | SideLeft | M+090 | Left Side |
|  | AC_0001000b | SideRight | M-090 | Right Side |
|  | AC_0001001c | BackLeftMid | M+135 | Left Back |
|  | AC_0001001d | BackRightMid | M-135 | Right Back |
|  | AC_00010028 | UpperTopBackCentre | UH+180 | Centre Height |
|  | AC_00010020 | LowFrequencyEffects1 | LFE1 | LowFrequencyEffects-1 |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
|  | AC_00010021 | LowFrequencyEffects2 | LFE2 | LowFrequencyEffects-2 |
| AP_00010015 11.1screen_5.1.4+sc_(4+7+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Top Front Left |
|  | AC_0001000f | TopFrontRight | U-030 | Top Front Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Top Surround Left |
|  | AC_00010012 | TopSurroundRight | U-110 | Top Surround Right |
|  | AC_00010024 | FrontLeftScreen | M+SC | Front Left Screen |
|  | AC_00010025 | FrontRightScreen | M-SC | Front Right Screen |
| AP_0001001f 11.0_7.0.4_(4+7+0) (BS.2051 (J) without LFE) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Left Top Front |
|  | AC_00010023 | TopFrontRightMid | U-045 | Right Top Front |
|  | AC_0001001e | TopBackLeftMid | U+135 | Left Top Back |
|  | AC_0001001f | TopBackRightMid | U-135 | Right Top Back |
| AP_00010017 11.1_7.1.4_(4+7+0) (BS.2051 (J)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Left Top Front |
|  | AC_00010023 | TopFrontRightMid | U-045 | Right Top Front |
|  | AC_0001001e | TopBackLeftMid | U+135 | Left Top Back |
|  | AC_0001001f | TopBackRightMid | U-135 | Right Top Back |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| AP_00010022 Auro-3D_11.1_(6+5+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_0001000d | TopFrontLeft | U+030 | Height Left |
|  | AC_0001000f | TopFrontRight | U-030 | Height Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Height Left Surround |
|  | AC_00010012 | TopSurroundRight | U-110 | Height Right Surround |
|  | AC_0001000c | TopCentre | T+000 | Top |
|  | AC_0001000e | TopFrontCentre | U+000 | Height Centre |
| AP_00010023 Auro-3D_11.1_(4+7+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010029 | BackLeft | M+150 | Left Back |
|  | AC_0001002a | BackRight | M-150 | Right Back |
|  | AC_0001000d | TopFrontLeft | U+030 | Height Left |
|  | AC_0001000f | TopFrontRight | U-030 | Height Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Height Left Surround |
|  | AC_00010012 | TopSurroundRight | U-110 | Height Right Surround |
| AP_00010025 360RA_13.0_(5+5+3) (N/A) | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010005 | SurroundLeft | M+110 | Left Rear |
|  | AC_00010006 | SurroundRight | M-110 | Right Rear |
|  | AC_0001000e | TopFrontCentre | U+000 | Upper Centre |
|  | AC_0001000d | TopFrontLeft | U+030 | Upper Left |
|  | AC_0001000f | TopFrontRight | U-030 | Upper Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Upper Left Rear |
|  | AC_00010012 | TopSurroundRight | U-110 | Upper Right Rear |
|  | AC_00010015 | BottomFrontCentre | B+000 | Lower Centre |
|  | AC_0001002b | BottomFrontLeft | B+030 | Lower Left |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
|  | AC_0001002c | BottomFrontRight | B-030 | Lower Right |
| AP_00010008 13.1screen_7.1.4+sc_(4+9+0) (BS.2051 (G)) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Left Top Front |
|  | AC_00010023 | TopFrontRightMid | U-045 | Right Top Front |
|  | AC_0001001e | TopBackLeftMid | U+135 | Left Top Back |
|  | AC_0001001f | TopBackRightMid | U-135 | Right Top Back |
|  | AC_00010024 | FrontLeftScreen | M+SC | Left Screen |
|  | AC_00010025 | FrontRightScreen | M-SC | Right Screen |
| AP_00010018 13.1_9.1.4_(4+9+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
|  | AC_00010018 | FrontLeftWide | M+060 | Left Wide |
|  | AC_00010019 | FrontRightWide | M-060 | Right Wide |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Left Top Front |
|  | AC_00010023 | TopFrontRightMid | U-045 | Right Top Front |
|  | AC_0001001e | TopBackLeftMid | U+135 | Left Top Back |
|  | AC_0001001f | TopBackRightMid | U-135 | Right Top Back |
| AP_00010024 Auro-3D_13.1_(6+7+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_00010005 | SurroundLeft | M+110 | Left Surround |
|  | AC_00010006 | SurroundRight | M-110 | Right Surround |
|  | AC_00010029 | BackLeft | M+150 | Left Back |
|  | AC_0001002a | BackRight | M-150 | Right Back |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
|  | AC_0001000d | TopFrontLeft | U+030 | Height Left |
|  | AC_0001000f | TopFrontRight | U-030 | Height Right |
|  | AC_00010010 | TopSurroundLeft | U+110 | Height Left Surround |
|  | AC_00010012 | TopSurroundRight | U-110 | Height Right Surround |
|  | AC_0001000c | TopCentre | T+000 | Top |
|  | AC_0001000e | TopFrontCentre | U+000 | Height Centre |
| AP_00010019 15.1_9.1.6_(6+9+0) (N/A) | AC_00010001 | FrontLeft | M+030 | Left |
|  | AC_00010002 | FrontRight | M-030 | Right |
|  | AC_00010003 | FrontCentre | M+000 | Centre |
|  | AC_00010004 | LowFrequencyEffects | LFE | Low Frequency Effects |
|  | AC_0001000a | SideLeft | M+090 | Left Side Surround |
|  | AC_0001000b | SideRight | M-090 | Right Side Surround |
|  | AC_0001001c | BackLeftMid | M+135 | Left Rear Surround |
|  | AC_0001001d | BackRightMid | M-135 | Right Rear Surround |
|  | AC_00010018 | FrontLeftWide | M+060 | Left Wide |
|  | AC_00010019 | FrontRightWide | M-060 | Right Wide |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Left Top Front |
|  | AC_00010023 | TopFrontRightMid | U-045 | Right Top Front |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
|  | AC_0001001e | TopBackLeftMid | U+135 | Left Top Back |
|  | AC_0001001f | TopBackRightMid | U-135 | Right Top Back |
| AP_00010010 22.0_(9+10+3) (BS.2051 (H) without LFE) | AC_00010018 | FrontLeftWide | M+060 | Front Left |
|  | AC_00010019 | FrontRightWide | M-060 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_0001001c | BackLeftMid | M+135 | Back Left |
|  | AC_0001001d | BackRightMid | M-135 | Back Right |
|  | AC_00010001 | FrontLeft | M+030 | Front Left Centre |
|  | AC_00010002 | FrontRight | M-030 | Front Right Centre |
|  | AC_00010009 | BackCentre | M+180 | Back Centre |
|  | AC_0001000a | SideLeft | M+090 | Side Left |
|  | AC_0001000b | SideRight | M-090 | Side Right |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Top Front Left |
|  | AC_00010023 | TopFrontRightMid | U-045 | Top Front Right |
|  | AC_0001000e | TopFrontCentre | U+000 | Top Front Centre |
|  | AC_0001000c | TopCentre | T+000 | Top Centre |
|  | AC_0001001e | TopBackLeftMid | U+135 | Top Back Left |

TABLE 2A (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
|  | AC_0001001f | TopBackRightMid | U-135 | Top Back Right |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
|  | AC_00010011 | TopBackCentre | U+180 | Top Back Centre |
|  | AC_00010015 | BottomFrontCentre | B+000 | Bottom Front Centre |
|  | AC_00010016 | BottomFrontLeftMid | B+045 | Bottom Front Left |
|  | AC_00010017 | BottomFrontRightMid | B-045 | Bottom Front Right |
| AP_00010009 22.2_(9+10+3) (BS.2051 (H)) This pack describes a different variant of the 22.2 system where there are 5 front main channels and loudspeakers, and corresponds with the Cartesian pack AP_00010809. This pack simplifies mapping to the Cartesian pack AP_00010809. | AC_00010018 | FrontLeftWide | M+060 | Front Left |
|  | AC_00010019 | FrontRightWide | M-060 | Front Right |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010020 | LowFrequencyEffects1 | LFE1 | LowFrequencyEffects-1 |
|  | AC_0001001c | BackLeftMid | M+135 | Back Left |
|  | AC_0001001d | BackRightMid | M-135 | Back Right |
|  | AC_00010001 | FrontLeft | M+030 | Front Left Centre |
|  | AC_00010002 | FrontRight | M-030 | Front Right Centre |
|  | AC_00010009 | BackCentre | M+180 | Back Centre |
|  | AC_00010021 | LowFrequencyEffects2 | LFE2 | LowFrequencyEffects-2 |
|  | AC_0001000a | SideLeft | M+090 | Side Left |
|  | AC_0001000b | SideRight | M-090 | Side Right |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Top Front Left |
|  | AC_00010023 | TopFrontRightMid | U-045 | Top Front Right |
|  | AC_0001000e | TopFrontCentre | U+000 | Top Front Centre |
|  | AC_0001000c | TopCentre | T+000 | Top Centre |
|  | AC_0001001e | TopBackLeftMid | U+135 | Top Back Left |
|  | AC_0001001f | TopBackRightMid | U-135 | Top Back Right |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
|  | AC_00010011 | TopBackCentre | U+180 | Top Back Centre |
|  | AC_00010015 | BottomFrontCentre | B+000 | Bottom Front Centre |
|  | AC_00010016 | BottomFrontLeftMid | B+045 | Bottom Front Left |
|  | AC_00010017 | BottomFrontRightMid | B-045 | Bottom Front Right |
| AP_0001001a 22.2-3front_(9+10+3) (N/A) This pack describes a different variant of the 22.2 system where there are 3 front main channels and loudspeakers, and is corresponds with the Cartesian pack AP_0001081a. This pack | AC_00010018 | FrontLeftWide | M+060 | Left Wide |
|  | AC_00010019 | FrontRightWide | M-060 | Right Wide |
|  | AC_00010003 | FrontCentre | M+000 | Front Centre |
|  | AC_00010020 | LowFrequencyEffects1 | LFE1 | LowFrequencyEffects-1 |
|  | AC_0001001c | BackLeftMid | M+135 | Back Left |
|  | AC_0001001d | BackRightMid | M-135 | Back Right |

TABLE 2A (end)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |
| --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column |
| simplifies mapping to the Cartesian pack AP_0001081a. | AC_00010001 | FrontLeft | M+030 | Front Left |
|  | AC_00010002 | FrontRight | M-030 | Front Right |
|  | AC_00010009 | BackCentre | M+180 | Back Centre |
|  | AC_00010021 | LowFrequencyEffects2 | LFE2 | LowFrequencyEffects-2 |
|  | AC_0001000a | SideLeft | M+090 | Side Left |
|  | AC_0001000b | SideRight | M-090 | Side Right |
|  | AC_00010022 | TopFrontLeftMid | U+045 | Top Front Left |
|  | AC_00010023 | TopFrontRightMid | U-045 | Top Front Right |
|  | AC_0001000e | TopFrontCentre | U+000 | Top Front Centre |
|  | AC_0001000c | TopCentre | T+000 | Top Centre |
|  | AC_0001001e | TopBackLeftMid | U+135 | Top Back Left |
|  | AC_0001001f | TopBackRightMid | U-135 | Top Back Right |
|  | AC_00010013 | TopSideLeft | U+090 | Top Side Left |
|  | AC_00010014 | TopSideRight | U-090 | Top Side Right |
|  | AC_00010011 | TopBackCentre | U+180 | Top Back Centre |
|  | AC_00010015 | BottomFrontCentre | B+000 | Bottom Front Centre |
|  | AC_00010016 | BottomFrontLeftMid | B+045 | Bottom Front Left |
|  | AC_00010017 | BottomFrontRightMid | B-045 | Bottom Front Right |
| AP_00010011 N/A (see NOTE) |  |  |  |  |
| NOTE – Earlier versions (Recommendation ITU-R BS.2094-0 and ITU-R BS.2094-1) of this Recommendation contained the common definition of this audioPackFormatID and audioPackFormatName but have now been deprecated. The audioPackFormatID and audioPackFornatName shall not be used for other audioPackFormats to avoid compatibility issues. |  |  |  |  |

TABLE 2B

audioPackFormat definitions for ‘DirectSpeakers’ for Cartesian coordinate system

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
| AP_00010801 mono_(0+1+0) (BS.775) | AC_00010803 | FrontCentre_Cart | M_FC | Mono | M+000 |
| AP_00010802 stereo_(0+2+0) (BS.775, BS.2051 (A)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
| AP_0001080a 3.0_(0+3+0) (BS.775) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
| AP_0001080b 4.0_(0+4+0) (BS.775) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010809 | BackCentre_Cart | M_BC | Mono Surround | M+180 |
| AP_0001080c 5.0_(0+5+0) (BS.775, BS.2051 (B) without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
| AP_00010803 5.1_(0+5+0) (BS.775, BS.2051 (B)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
| AP_0001080d 6.1_(0+6+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010809 | BackCentre_Cart | M_BC | Centre Surround | M+180 |
| AP_0001080e 7.1front_(0+7+0) (N/A) | AC_00010807 | FrontLeftCentreMid_Cart | M_FL50 | Left Front Centre | M+030 |
|  | AC_00010808 | FrontRightCentreMid_Cart | M_FR50 | Right Front Centre | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010801 | FrontLeft_Cart | M_F | Left Front | M+045 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right Front | M-045 |
| AP_0001081b 7.0back_(0+7+0) (BS.2051 (I) without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
| AP_0001080f 7.1back_(0+7+0) (BS.2051 (I)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
| AP_0001081c 7.0top_(2+5+0) (BS.2051 (C) without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-030 |
| AP_00010804 7.1top_(2+5+0) (BS.2051 (C)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-030 |
| AP_00010812 7.1screen_5.1+sc_(0+7+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010824 | FrontLeftScreen_Cart | M_FLSc | Front Left Screen | M+SC |
|  | AC_00010825 | FrontRightScreen_Cart | M_FRSc | Front Right Screen | M-SC |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
| AP_0001081d 7.0topside_5.0.2_(2+5+0) (N/A without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
| AP_00010813 7.1topside_5.1.2_(2+5+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
| AP_00010820 Auro-3D_8.0_(4+4+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Height Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Height Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Height Left Surround | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Height Right Surround | U-110 |
| AP_00010814 9.1screen_5.1.2+sc_(2+7+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
|  | AC_00010824 | FrontLeftScreen_Cart | M_FLSc | Front Left Screen | M+SC |
|  | AC_00010825 | FrontRightScreen_Cart | M_FRSc | Front Right Screen | M-SC |
| AP_00010816 9.1topside_7.1.2_(2+7+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Back Left | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Back Right | M-135 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
| AP_0001081e 9.0_5.0.4_(4+5+0) (BS.2051 (D) without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Rear | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Rear | U-110 |
| AP_00010805 9.1_5.1.4_(4+5+0) (BS.2051 (D)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Rear | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Rear | U-110 |
| AP_00010806 10.1_(4+5+1) (BS.2051 (E)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Rear | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Rear | U-110 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_00010815 | BottomFrontCentre_Cart | B_FC | Centre Bottom Front | B+000 |
| AP_00010821 Auro-3D_10.1_ (5+5+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Height Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Height Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Height Left Surround | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Height Right Surround | U-110 |
|  | AC_0001080c | TopCentre_Cart | U_TC | Top | T+000 |
| AP_00010807 10.2_(3+7+0) (BS.2051 (F)) | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Height | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Height | U-045 |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Back | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Back | M-135 |
|  | AC_00010828 | TopBackCentreFar_Cart | U_BC67 | Centre Height | UH+180 |
|  | AC_00010820 | LowFrequencyEffects1_Cart | LFE1 | LowFrequencyEffects-1 | LFE1 |
|  | AC_00010821 | LowFrequencyEffects2_Cart | LFE2 | LowFrequencyEffects-2 | LFE2 |
| AP_00010815 11.1screen_5.1.4+sc_(4+7+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Top Front Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Top Front Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Top Surround Left | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Top Surround Right | U-110 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_00010824 | FrontLeftScreen_Cart | M_FLSc | Front Left Screen | M+SC |
|  | AC_00010825 | FrontRightScreen_Cart | M_FRSc | Front Right Screen | M-SC |
| AP_0001081f 11.0_7.0.4_(4+7+0) (BS.2051 (J) without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-045 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Back | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Back | U-135 |
| AP_00010817 11.1_7.1.4_(4+7+0) (BS.2051 (J)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-045 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Back | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Back | U-135 |
| AP_00010822 Auro-3D_11.1_(6+5+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Height Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Height Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Height Left Surround | U+110 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Height Right Surround | U-110 |
|  | AC_0001080c | TopCentre_Cart | U_TC | Top | T+000 |
|  | AC_0001080e | TopFrontCentre_Cart | U_FC | Height Centre | U+000 |
| AP_00010823 Auro-3D_11.1_(4+7+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010829 | BackLeftCentreMid_Cart | M_BL50 | Left Back | M+150 |
|  | AC_0001082a | BackRightCentreMid_Cart | M_BR50 | Right Back | M-150 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Height Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Height Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Height Left Surround | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Height Right Surround | U-110 |
| AP_00010825 360RA_13.0_(5+5+3) (N/A) | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear | M-110 |
|  | AC_0001080e | TopFrontCentre_Cart | U_FC | Upper Centre | U+000 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Upper Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Upper Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Upper Left Rear | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Upper Right Rear | U-110 |
|  | AC_00010815 | BottomFrontCentre_Cart | B_FC | Lower Centre | B+000 |
|  | AC_00010816 | BottomFrontLeft_Cart | B_FL | Lower Left | B+030 |
|  | AC_00010817 | BottomFrontRight_Cart | B_FR | Lower Right | B-030 |
| AP_00010808 13.1screen_7.1.4+sc_(4+9+0) (BS.2051 (G)) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-045 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Back | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Back | U-135 |
|  | AC_00010824 | FrontLeftScreen_Cart | M_FLSc | Left Screen | M+SC |
|  | AC_00010825 | FrontRightScreen_Cart | M_FRSc | Right Screen | M-SC |
| AP_00010818 13.1_9.1.4_(4+9+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
|  | AC_00010818 | FrontSideLeftFar_Cart | M_FSL67 | Left Wide | M+060 |
|  | AC_00010819 | FrontSideRightFar_Cart | M_FSR67 | Right Wide | M-060 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-045 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Back | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Back | U-135 |
| AP_00010824 Auro-3D_13.1_(6+7+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Surround | M+110 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Surround | M-110 |
|  | AC_00010829 | BackLeftCentreMid_Cart | M_BL50 | Left Back | M+150 |
|  | AC_0001082a | BackRightCentreMid_Cart | M_BR50 | Right Back | M-150 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Height Left | U+030 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Height Right | U-030 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Height Left Surround | U+110 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Height Right Surround | U-110 |
|  | AC_0001080c | TopCentre_Cart | U_TC | Top | T+000 |
|  | AC_0001080e | TopFrontCentre_Cart | U_FC | Height Centre | U+000 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
| AP_00010819 15.1_9.1.6_(6+9+0) (N/A) | AC_00010801 | FrontLeft_Cart | M_FL | Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Right | M-030 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Centre | M+000 |
|  | AC_00010804 | LowFrequencyEffects_Cart | LFE | Low Frequency Effects | LFE |
|  | AC_0001080a | SideLeft_Cart | M_SL | Left Side Surround | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Right Side Surround | M-090 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Left Rear Surround | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Right Rear Surround | M-135 |
|  | AC_00010818 | FrontSideLeftFar_Cart | M_FSL67 | Left Wide | M+060 |
|  | AC_00010819 | FrontSideRightFar_Cart | M_FSR67 | Right Wide | M-060 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Left Top Front | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Right Top Front | U-045 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Left Top Back | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Right Top Back | U-135 |
| AP_00010810 22.0_(9+10+3) (BS.2051 (H) without LFE) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+060 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-060 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Back Left | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Back Right | M-135 |
|  | AC_00010807 | FrontLeftCentreMid_Cart | M_FL50 | Front Left Centre | M+030 |
|  | AC_00010808 | FrontRightCentreMid_Cart | M_FR50 | Front Right Centre | M-030 |
|  | AC_00010809 | BackCentre_Cart | M_BC | Back Centre | M+180 |
|  | AC_0001080a | SideLeft_Cart | M_SL | Side Left | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Side Right | M-090 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Top Front Left | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Top Front Right | U-045 |
|  | AC_0001080e | TopFrontCentre_Cart | U_FC | Top Front Centre | U+000 |
|  | AC_0001080c | TopCentre_Cart | U_TC | Top Centre | T+000 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Top Back Left | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Top Back Right | U-135 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
|  | AC_00010811 | TopBackCentre_Cart | U_BC | Top Back Centre | U+180 |
|  | AC_00010815 | BottomFrontCentre_Cart | B_FC | Bottom Front Centre | B+000 |
|  | AC_00010816 | BottomFrontLeft_Cart | B_FL | Bottom Front Left | B+045 |

TABLE 2B (continued)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_00010817 | BottomFrontRight_Cart | B_FR | Bottom Front Right | B-045 |
| AP_00010809 22.2_(9+10+3) (BS.2051 (H)) | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+060 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-060 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010820 | LowFrequencyEffects1_Cart | LFE1 | LowFrequencyEffects-1 | LFE1 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Back Left | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Back Right | M-135 |
|  | AC_00010807 | FrontLeftCentreMid_Cart | M_FL50 | Front Left Centre | M+030 |
|  | AC_00010808 | FrontRightCentreMid_Cart | M_FR50 | Front Right Centre | M-030 |
|  | AC_00010809 | BackCentre_Cart | M_BC | Back Centre | M+180 |
|  | AC_00010821 | LowFrequencyEffects2_Cart | LFE2 | LowFrequencyEffects-2 | LFE2 |
|  | AC_0001080a | SideLeft_Cart | M_SL | Side Left | M+090 |
|  | AC_0001080b | SideRight_Cart | M_SR | Side Right | M-090 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Top Front Left | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Top Front Right | U-045 |
|  | AC_0001080e | TopFrontCentre_Cart | U_FC | Top Front Centre | U+000 |
|  | AC_0001080c | TopCentre_Cart | U_TC | Top Centre | T+000 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Top Back Left | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Top Back Right | U-135 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
|  | AC_00010811 | TopBackCentre_Cart | U_BC | Top Back Centre | U+180 |
|  | AC_00010815 | BottomFrontCentre_Cart | B_FC | Bottom Front Centre | B+000 |
|  | AC_00010816 | BottomFrontLeft_Cart | B_FL | Bottom Front Left | B+045 |
|  | AC_00010817 | BottomFrontRight_Cart | B_FR | Bottom Front Right | B-045 |
| AP_0001081a 22.2-3front_(9+10+3) (N/A) | AC_00010818 | FrontSideLeftFar_Cart | M_FSL67 | Left Wide | M+060 |
|  | AC_00010819 | FrontSideRightFar_Cart | M_FSR67 | Right Wide | M-060 |
|  | AC_00010803 | FrontCentre_Cart | M_FC | Front Centre | M+000 |
|  | AC_00010820 | LowFrequencyEffects1_Cart | LFE1 | LowFrequencyEffects-1 | LFE1 |
|  | AC_00010805 | BackLeft_Cart | M_BL | Back Left | M+135 |
|  | AC_00010806 | BackRight_Cart | M_BR | Back Right | M-135 |
|  | AC_00010801 | FrontLeft_Cart | M_FL | Front Left | M+030 |
|  | AC_00010802 | FrontRight_Cart | M_FR | Front Right | M-030 |
|  | AC_00010809 | BackCentre_Cart | M_BC | Back Centre | M+180 |
|  | AC_00010821 | LowFrequencyEffects2_Cart | LFE2 | LowFrequencyEffects-2 | LFE2 |
|  | AC_0001080a | SideLeft_Cart | M_SL | Side Left | M+090 |

TABLE 2B (end)

| audioPackFormatID audioPackFormatName (Defined in Document) | audioChannelFormatIDRef | audioChannelFormatName | speakerLabel | Informal, not contained in BS.2094 XML |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel name in definition document, see first column | Playback system speaker (1) |
|  | AC_0001080b | SideRight_Cart | M_SR | Side Right | M-090 |
|  | AC_0001080d | TopFrontLeft_Cart | U_FL | Top Front Left | U+045 |
|  | AC_0001080f | TopFrontRight_Cart | U_FR | Top Front Right | U-045 |
|  | AC_0001080e | TopFrontCentre_Cart | U_FC | Top Front Centre | U+000 |
|  | AC_0001080c | TopCentre_Cart | U_TC | Top Centre | T+000 |
|  | AC_00010810 | TopBackLeft_Cart | U_BL | Top Back Left | U+135 |
|  | AC_00010812 | TopBackRight_Cart | U_BR | Top Back Right | U-135 |
|  | AC_00010813 | TopSideLeft_Cart | U_SL | Top Side Left | U+090 |
|  | AC_00010814 | TopSideRight_Cart | U_SR | Top Side Right | U-090 |
|  | AC_00010811 | TopBackCentre_Cart | U_BC | Top Back Centre | U+180 |
|  | AC_00010815 | BottomFrontCentre_Cart | B_FC | Bottom Front Centre | B+000 |
|  | AC_00010816 | BottomFrontLeft_Cart | B_FL | Bottom Front Left | B+045 |
|  | AC_00010817 | BottomFrontRight_Cart | B_FR | Bottom Front Right | B-045 |
| (1) The playback system speaker specified in this column is only relevant when the loudspeaker configuration of the playback system matches the audioPackFormat specified. |  |  |  |  |  |

To show how an audioPackDefinition is represented in XML the following code shows the stereo pack definition.

<audioPackFormat audioPackFormatID="AP_00010002" audioPackFormatName="urn:itu:bs:2051:0:pack:stereo_(0+2+0)" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00010001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00010002</audioChannelFormatIDRef> </audioPackFormat>

## 4.2	Common definitions for the ‘Matrix’ audio type

Currently, there are no common definitions for the ‘Matrix’ audio type. However, in future revisions relevant information may be added to this section for common matrix configurations used.

## 4.3	Common definitions for the ‘Objects’ audio type

Currently, there are no common definitions for the ‘Objects’ audio type. However, in future revisions relevant information may be added to this section.

## 4.4	Common definitions for the ‘HOA’ audio type

### 4.4.1	Common audioChannelFormats for ‘HOA’

TABLE 3

AudioChannelFormat definitions for 'HOA'

| audioChannelFormatID | audioChannelFormatName | Order | Degree | Normalization |
| --- | --- | --- | --- | --- |
| AC_00040001 | SN3D_ACN_0 | 0 | 0 | SN3D |
| AC_00040002 | SN3D_ACN_1 | 1 | –1 | SN3D |
| AC_00040003 | SN3D_ACN_2 | 1 | 0 | SN3D |
| AC_00040004 | SN3D_ACN_3 | 1 | 1 | SN3D |
| AC_00040005 | SN3D_ACN_4 | 2 | –2 | SN3D |
| AC_00040006 | SN3D_ACN_5 | 2 | –1 | SN3D |
| … | … | … | … | … |
| AC_00040079 | SN3D_ACN_120 | 10 | 10 | SN3D |
| AC_00040101 | N3D_ACN_0 | 0 | 0 | N3D |
| AC_00040102 | N3D_ACN_1 | 1 | –1 | N3D |
| AC_00040103 | N3D_ACN_2 | 1 | 0 | N3D |
| AC_00040104 | N3D_ACN_3 | 1 | 1 | N3D |
| AC_00040105 | N3D_ACN_4 | 2 | –2 | N3D |
| AC_00040106 | N3D_ACN_5 | 2 | –1 | N3D |
| … | … | … | … | … |
| AC_00040179 | N3D_ACN_120 | 10 | 10 | N3D |
| AC_00040201 | FuMa_W | 0 | 0 | FuMa |
| AC_00040202 | FuMa_X | 1 | 1 | FuMa |
| AC_00040203 | FuMa_Y | 1 | –1 | FuMa |
| AC_00040204 | FuMa_Z | 1 | 0 | FuMa |
| AC_00040205 | FuMa_R | 2 | 0 | FuMa |
| AC_00040206 | FuMa_S | 2 | 1 | FuMa |
| AC_00040207 | FuMa_T | 2 | –1 | FuMa |
| AC_00040208 | FuMa_U | 2 | 2 | FuMa |
| AC_00040209 | FuMa_V | 2 | –2 | FuMa |
| AC_0004020a | FuMa_K | 3 | 0 | FuMa |
| AC_0004020b | FuMa_L | 3 | 1 | FuMa |
| AC_0004020c | FuMa_M | 3 | –1 | FuMa |
| AC_0004020d | FuMa_N | 3 | 2 | FuMa |
| AC_0004020e | FuMa_O | 3 | –2 | FuMa |
| AC_0004020f | FuMa_P | 3 | 3 | FuMa |
| AC_00040210 | FuMa_Q | 3 | –3 | FuMa |

Each channel definition is listed in Table 3, where the audioChannelFormatID, and audioChannelFormatName elements are listed. The order, degree, and normalization columns represent the settings that would be specified within the audioBlockFormat sub-elements. The XML below shows how the first entry in this table is represented in XML.

<audioChannelFormat audioChannelFormatID="AC_00040001" audioChannelFormatName="SN3D_ACN_0" typeLabel="0004" typeDefinition="HOA"> <audioBlockFormat audioBlockFormatID="AB_00040001_00000001"> <normalization>SN3D</normalization> <order>0</order> <degree>0</degree> </audioBlockFormat> </audioChannelFormat>

### 4.4.2	Common audioPackFormats for ‘HOA’

The audioPackFormat definitions cover HOA configurations where all channels have the same normalization scheme and a complete series of channels up to a given order. Table 4 shows the set of configurations that have been defined.

As audioPackFormats can be nested, this allows the higher order packs to contain the lower order pack it contains plus the higher order channels. This reduces the size of each pack, and allows for flexibility in systems where different orders may require different processing.

The audioPackFormat names consist of the order, normalization and type. Spaces are not used in the names as they can cause problems.

audioPackFormatID scheme:

for type 0-1: 	AP_00040<type><normalization><order>

for type 2-3: 	AP_00040<type><normalization><order combination>

TABLE 4

HOA type values

| <type> | Value |
| --- | --- |
| 0 | 3D / periphonic |
| 1 | 2D / pantophonic / horizontal-only |
| 2 | pantophonic and periphonic superimposed (#H#P) |
| 3 | Complete mixed-order sets (#H#V) |

TABLE 5

HOA normalization values

| <normalization> | Value |
| --- | --- |
| 0 | SN3D |
| 1 | N3D |
| 2 | FuMa |

TABLE 6

HOA order combination values

| <type> | <order combination> | Value |
| --- | --- | --- |
| 2 | 0 | 2H1P |
| 2 | 1 | 3H1P |
| 3 | 0 | 2H1V |

TABLE 7

AudioPackFormat definitions

| audioPackFormatID audioPackFormatName | audioChannelIDRef | audioChannelName | Order | Degree | Norm. |
| --- | --- | --- | --- | --- | --- |
| AP_00040001 3D_order1_SN3D _ACN | AC_00040001 | SN3D_ACN_0 | 0 | 0 | SN3D |
|  | AC_00040002 | SN3D_ACN_1 | 1 | −1 | SN3D |
|  | AC_00040003 | SN3D_ACN_2 | 1 | 0 | SN3D |
|  | AC_00040004 | SN3D_ACN_3 | 1 | 1 | SN3D |
| AP_00040011 3D_order1_N3D _ACN | AC_00040101 | N3D_ACN_0 | 0 | 0 | N3D |
|  | AC_00040102 | N3D_ACN_1 | 1 | −1 | N3D |
|  | AC_00040103 | N3D_ACN_2 | 1 | 0 | N3D |
|  | AC_00040104 | N3D_ACN_3 | 1 | 1 | N3D |
| AP_00040021 3D_order1_FuMa | AC_00040201 | FuMa_W | 0 | 0 | FuMa |
|  | AC_00040202 | FuMa_X | 1 | 1 | FuMa |
|  | AC_00040203 | FuMa_Y | 1 | −1 | FuMa |
|  | AC_00040204 | FuMa_Z | 1 | 0 | FuMa |
| AP_00040002 3D_order2_SN3D _ACN | audioPackFormatIDRef: AP_00040001 |  |  |  |  |
|  | AC_00040005 | SN3D_ACN_4 | 2 | −2 | SN3D |
|  | AC_00040006 | SN3D_ACN_5 | 2 | −1 | SN3D |
|  | AC_00040007 | SN3D_ACN_6 | 2 | 0 | SN3D |
|  | AC_00040008 | SN3D_ACN_7 | 2 | 1 | SN3D |
|  | AC_00040009 | SN3D_ACN_8 | 2 | 2 | SN3D |
| AP_00040012 3D_order2_N3D _ACN | audioPackFormatIDRef: AP_00040011 |  |  |  |  |
|  | AC_00041005 | N3D_ACN_4 | 2 | −2 | N3D |
|  | AC_00041006 | N3D_ACN_5 | 2 | −1 | N3D |
|  | AC_00041007 | N3D_ACN_6 | 2 | 0 | N3D |
|  | AC_00041008 | N3D_ACN_7 | 2 | 1 | N3D |
|  | AC_00041009 | N3D_ACN_8 | 2 | 2 | N3D |
| AP_00040022 3D_order2_FuMa | audioPackFormatIDRef: AP_00040021 |  |  |  |  |
|  | AC_00040205 | FuMa_R | 2 | 0 | FuMa |
|  | AC_00040206 | FuMa_S | 2 | 1 | FuMa |
|  | AC_00040207 | FuMa_T | 2 | −1 | FuMa |
|  | AC_00040208 | FuMa_U | 2 | 2 | FuMa |
|  | AC_00040209 | FuMa_V | 2 | −2 | FuMa |
| AP_00040003 3D_order3_SN3D _ACN | audioPackFormatIDRef: AP_00040002 |  |  |  |  |
|  | AC_0004000a | SN3D_ACN_9 | 3 | −3 | SN3D |
|  | AC_0004000b | SN3D_ACN_10 | 3 | −2 | SN3D |

TABLE 7 (continued)

| audioPackFormatID audioPackFormatName | audioChannelIDRef | audioChannelName | Order | Degree | Norm. |
| --- | --- | --- | --- | --- | --- |
|  | AC_0004000c | SN3D_ACN_11 | 3 | −1 | SN3D |
|  | AC_0004000d | SN3D_ACN_12 | 3 | 0 | SN3D |
|  | AC_0004000e | SN3D_ACN_13 | 3 | 1 | SN3D |
|  | AC_0004000f | SN3D_ACN_14 | 3 | 2 | SN3D |
|  | AC_00040010 | SN3D_ACN_15 | 3 | 3 | SN3D |
| AP_00040013 3D_order3_N3D _ACN | audioPackFormatIDRef: AP_00040012 |  |  |  |  |
|  | AC_0004010a | N3D_ACN_9 | 3 | −3 | N3D |
|  | AC_0004010b | N3D_ACN_10 | 3 | −2 | N3D |
|  | AC_0004010c | N3D_ACN_11 | 3 | −1 | N3D |
|  | AC_0004010d | N3D_ACN_12 | 3 | 0 | N3D |
|  | AC_0004010e | N3D_ACN_13 | 3 | 1 | N3D |
|  | AC_0004010f | N3D_ACN_14 | 3 | 2 | N3D |
|  | AC_00040110 | N3D_ACN_15 | 3 | 3 | N3D |
| AP_00040023 3D_order3_FuMa | audioPackFormatIDRef: AP_00040022 |  |  |  |  |
|  | AC_0004020a | FuMa_K | 3 | 0 | FuMa |
|  | AC_0004020b | FuMa_L | 3 | 1 | FuMa |
|  | AC_0004020c | FuMa_M | 3 | −1 | FuMa |
|  | AC_0004020d | FuMa_N | 3 | 2 | FuMa |
|  | AC_0004020e | FuMa_O | 3 | −2 | FuMa |
|  | AC_0004020f | FuMa_P | 3 | 3 | FuMa |
|  | AC_00040210 | FuMa_Q | 3 | −3 | FuMa |
| AP_00040004 3D_order4_SN3D _ACN | audioPackFormatIDRef: AP_00040003 |  |  |  |  |
|  | AC_00040011 | SN3D_ACN_16 | 4 | −4 | SN3D |
|  | AC_00040012 | SN3D_ACN_17 | 4 | −3 | SN3D |
|  | AC_00040013 | SN3D_ACN_18 | 4 | −2 | SN3D |
|  | AC_00040014 | SN3D_ACN_19 | 4 | −1 | SN3D |
|  | AC_00040015 | SN3D_ACN_20 | 4 | 0 | SN3D |
|  | AC_00040016 | SN3D_ACN_21 | 4 | 1 | SN3D |
|  | AC_00040017 | SN3D_ACN_22 | 4 | 2 | SN3D |
|  | AC_00040018 | SN3D_ACN_23 | 4 | 3 | SN3D |
|  | AC_00040019 | SN3D_ACN_24 | 4 | 4 | SN3D |
| AP_00040014 3D_order4_N3D _ACN | audioPackFormatIDRef: AP_00040013 |  |  |  |  |
|  | AC_00040111 | N3D_ACN_16 | 4 | −4 | N3D |
|  | AC_00040112 | N3D_ACN_17 | 4 | −3 | N3D |
|  | AC_00040113 | N3D_ACN_18 | 4 | −2 | N3D |
|  | AC_00040114 | N3D_ACN_19 | 4 | −1 | N3D |
|  | AC_00040115 | N3D_ACN_20 | 4 | 0 | N3D |
|  | AC_00040116 | N3D_ACN_21 | 4 | 1 | N3D |
|  | AC_00040117 | N3D_ACN_22 | 4 | 2 | N3D |
|  | AC_00040118 | N3D_ACN_23 | 4 | 3 | N3D |
|  | AC_00040119 | N3D_ACN_24 | 4 | 4 | N3D |

TABLE 7 (continued)

| audioPackFormatID audioPackFormatName | audioChannelIDRef | audioChannelName | Order | Degree | Norm. |
| --- | --- | --- | --- | --- | --- |
| AP_00040005 3D_order5_SN3D _ACN | audioPackFormatIDRef: AP_00040004 |  |  |  |  |
|  | AC_0004001a | SN3D_ACN_25 | 5 | −5 | SN3D |
|  | AC_0004001b | SN3D_ACN_26 | 5 | −4 | SN3D |
|  | AC_0004001c | SN3D_ACN_27 | 5 | −3 | SN3D |
|  | AC_0004001d | SN3D_ACN_28 | 5 | −2 | SN3D |
|  | AC_0004001e | SN3D_ACN_29 | 5 | −1 | SN3D |
|  | AC_0004001f | SN3D_ACN_30 | 5 | 0 | SN3D |
|  | AC_00040020 | SN3D_ACN_31 | 5 | 1 | SN3D |
|  | AC_00040021 | SN3D_ACN_32 | 5 | 2 | SN3D |
|  | AC_00040022 | SN3D_ACN_33 | 5 | 3 | SN3D |
|  | AC_00040023 | SN3D_ACN_34 | 5 | 4 | SN3D |
|  | AC_00040024 | SN3D_ACN_35 | 5 | 5 | SN3D |
| AP_00040015 3D_order5_N3D _ACN | audioPackFormatIDRef: AP_00040014 |  |  |  |  |
|  | AC_0004011a | N3D_ACN_25 | 5 | −5 | N3D |
|  | AC_0004011b | N3D_ACN_26 | 5 | −4 | N3D |
|  | AC_0004011c | N3D_ACN_27 | 5 | −3 | N3D |
|  | AC_0004011d | N3D_ACN_28 | 5 | −2 | N3D |
|  | AC_0004011e | N3D_ACN_29 | 5 | −1 | N3D |
|  | AC_0004011f | N3D_ACN_30 | 5 | 0 | N3D |
|  | AC_00040120 | N3D_ACN_31 | 5 | 1 | N3D |
|  | AC_00040121 | N3D_ACN_32 | 5 | 2 | N3D |
|  | AC_00040122 | N3D_ACN_33 | 5 | 3 | N3D |
|  | AC_00040123 | N3D_ACN_34 | 5 | 4 | N3D |
|  | AC_00040124 | N3D_ACN_35 | 5 | 5 | N3D |
| AP_00040006 3D_order6_SN3D _ACN | audioPackFormatIDRef: AP_00040005 |  |  |  |  |
|  | AC_00040025 | SN3D_ACN_36 | 6 | −6 | SN3D |
|  | AC_00040026 | SN3D_ACN_37 | 6 | −5 | SN3D |
|  | AC_00040027 | SN3D_ACN_38 | 6 | −4 | SN3D |
|  | AC_00040028 | SN3D_ACN_39 | 6 | −3 | SN3D |
|  | AC_00040029 | SN3D_ACN_40 | 6 | −2 | SN3D |
|  | AC_0004003a | SN3D_ACN_41 | 6 | −1 | SN3D |
|  | AC_0004003b | SN3D_ACN_42 | 6 | 0 | SN3D |
|  | AC_0004002c | SN3D_ACN_43 | 6 | 1 | SN3D |
|  | AC_0004002d | SN3D_ACN_44 | 6 | 2 | SN3D |
|  | AC_0004002e | SN3D_ACN_45 | 6 | 3 | SN3D |
|  | AC_0004002f | SN3D_ACN_46 | 6 | 4 | SN3D |
|  | AC_00040030 | SN3D_ACN_47 | 6 | 5 | SN3D |
|  | AC_00040031 | SN3D_ACN_48 | 6 | 6 | SN3D |
| AP_00040016 3D_order6_N3D _ACN | audioPackFormatIDRef: AP_00040015 |  |  |  |  |
|  | AC_00040125 | N3D_ACN_36 | 6 | −6 | N3D |
|  | AC_00040126 | N3D_ACN_37 | 6 | −5 | N3D |
|  | AC_00040127 | N3D_ACN_38 | 6 | −4 | N3D |
|  | AC_00040128 | N3D_ACN_39 | 6 | −3 | N3D |
|  | AC_00040129 | N3D_ACN_40 | 6 | −2 | N3D |

TABLE 7 (end)

| audioPackFormatID audioPackFormatName | audioChannelIDRef | audioChannelName | Order | Degree | Norm. |
| --- | --- | --- | --- | --- | --- |
|  | AC_0004013a | N3D_ACN_41 | 6 | −1 | N3D |
|  | AC_0004013b | N3D_ACN_42 | 6 | 0 | N3D |
|  | AC_0004013c | N3D_ACN_43 | 6 | 1 | N3D |
|  | AC_0004013d | N3D_ACN_44 | 6 | 2 | N3D |
|  | AC_0004013e | N3D_ACN_45 | 6 | 3 | N3D |
|  | AC_0004013f | N3D_ACN_46 | 6 | 4 | N3D |
|  | AC_00040140 | N3D_ACN_47 | 6 | 5 | N3D |
|  | AC_00040141 | N3D_ACN_48 | 6 | 6 | N3D |
| AP_00040111 2D_Order1_N3D_ACN | AC_00040101 | N3D_ACN_0 | 0 | 0 | N3D |
|  | AC_00040102 | N3D_ACN_1 | 1 | −1 | N3D |
|  | AC_00040104 | N3D_ACN_3 | 1 | 1 | N3D |
| AP_00040112 2D_Order2_N3D_ACN | audioPackFormatIDRef: AP_00040111 |  |  |  |  |
|  | AC_00040105 | N3D_ACN_4 | 2 | −2 | N3D |
|  | AC_00040109 | N3D_ACN_8 | 2 | 2 | N3D |
| AP_00040210 2H1P_N3D_ACN | audioPackFormatIDRef: AP_00040011 |  |  |  |  |
|  | AC_00040105 | N3D_ACN_4 | 2 | −2 | N3D |
|  | AC_00040109 | N3D_ACN_8 | 2 | 2 | N3D |
| AP_00040211 3H1P_N3D_ACN | audioPackFormatIDRef: AP_00040210 |  |  |  |  |
|  | AC_0004010a | N3D_ACN_9 | 3 | −3 | N3D |
|  | AC_00040110 | N3D_ACN_15 | 3 | 3 | N3D |
| AP_00040310 2H1V_N3D_ACN | audioPackFormatIDRef: AP_00040011 |  |  |  |  |
|  | AC_00040105 | N3D_ACN_4 | 2 | −2 | N3D |
|  | AC_00040106 | N3D_ACN_5 | 2 | −1 | N3D |
|  | AC_00040108 | N3D_ACN_7 | 2 | 1 | N3D |
|  | AC_00040109 | N3D_ACN_8 | 2 | 2 | N3D |

To show how an audioPackDefinition is represented in XML the following code shows the first-order SN3D ACN (AmbiX) pack definition.

<audioPackFormat audioPackFormatID="AP_00040001" audioPackFormatName="3D_order1_SN3D_ACN" typeLabel="0004" typeDefinition="HOA"> <audioChannelFormatIDRef>AC_00040001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00040002</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00040003</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00040004</audioChannelFormatIDRef> </audioPackFormat>

## 4.5	Common definitions for the ‘Binaural’ audio type

Currently, there are no common definitions for the ‘Binaural’ audio type. However, in future revisions relevant information may be added to this section.

### 4.5.1	Common audioChannelFormats for ‘Binaural’

TABLE 8

AudioChannelFormat definitions for ‘Binaural’

| audioChannelFormatID | audioChannelFormatName |
| --- | --- |
| AC_00050001 | LeftEar |
| AC_00050002 | RightEar |

Each channel definition is listed in Table 8, where the audioChannelFormatID, and audioChannelFormatName elements are listed. The XML below shows how the first entry in this table is represented in XML.

<audioChannelFormat audioChannelFormatID="AC_00050001" audioChannelFormatName="LeftEar" typeLabel="0005" typeDefinition="DirectSpeakers"> <audioBlockFormat audioBlockFormatID="AC_00050001_00000001"> </audioBlockFormat> </audioChannelFormat>

### 4.5.2	Common audioStreamFormats and audioTrackFormats for ‘Binaural’

The common format type for audioStreamFormat and audioTrackFormat is ‘PCM’.

As previously mentioned, the audioStreamFormat and audioTrackFormat, which relate to each audioChannelFormat definition use the same ID prefixes and the same names with ‘PCM_’ appended, such as ‘PCM_LeftEar’. The XML code below shows both the audioStreamFormat and audioTrackFormat definition for the first entry in the table.

<audioStreamFormat audioStreamFormatID="AS_00050001" audioStreamFormatName="PCM_LeftEar" formatLabel="0001" formatDefinition="PCM"> <audioChannelFormatIDRef>AC_00050001</audioChannelFormatIDRef> <audioTrackFormatIDRef>AT_00050001_01</audioTrackFormatIDRef> </audioStreamFormat> <audioTrackFormat audioTrackFormatID="AT_00050001_01" audioTrackFormatName="PCM_LeftEar" formatLabel="0001" formatDefinition="PCM"> <audioStreamFormatIDRef>AS_00050001</audioStreamFormatIDRef> </audioTrackFormat>

### 4.5.3	Common audioPackFormats for ‘Binaural’

The audioPackFormat definition covers a single configuration. Table 9 shows the configuration that has been defined. The last column gives the references to each channel the pack consists of. Instead of giving the whole ID (e.g. AP_00010001), only the last two digits are listed for clarity; so they would be AP_000100xx where xx is the digits listed to give the full audioPackFormatIDRef.

TABLE 9

AudioPackFormat definitions

| audioPackFormatID audioPackFormatName | audioChannelIDRef | audioChannelName |
| --- | --- | --- |
| AP_00050001 Binaural | AC_00050001 AC_00050002 | LeftEar RightEar |

To show how an audioPackDefinition is represented in XML the following code shows the binaural pack definition.

<audioPackFormat audioPackFormatID="AP_00050001" audioPackFormatName="Binaural" typeLabel="0001" typeDefinition="DirectSpeakers"> <audioChannelFormatIDRef>AC_00050001</audioChannelFormatIDRef> <audioChannelFormatIDRef>AC_00050002</audioChannelFormatIDRef> </audioPackFormat>

## 4.6	Using URIs

As the set of common definitions may increase in the future, it helps to have a method of provenance for particular elements. In the channel definitions for polar coordinates the speakerLabel elements correspond to those used in Recommendation ITU-R BS.2051, which use the L+aaa style of naming.

The two places that use URIs are the speakerLabel element (within audioBlockFormat) and the audioPackFormatName attribute (part of audioPackFormat). For example, for Recommendation ITU-R BS.2051-0 the URI prefix is given as: urn:itu:bs:2051:0. It should be noted that this is not necessarily the most recent edition of Recommendation ITU-R BS.2051. An example of code using the URIs specifying M+030 and stereo is shown below.

<speakerLabel>urn:itu:bs:2051:0:speaker:M+030</speakerLabel>

<audioPackFormat audioPackFormatID="AP_00010002" audioPackFormatName="urn:itu:bs:2051:0:pack:stereo_(0+2+0)" typeLabel="0001" typeDefinition="DirectSpeakers">

After each urn prefix follows a string to classify what is being named (either speaker or pack in this case), and then this is followed by the actual name.

# 5	Attachments

This file is a Microsoft Excel spreadsheet containing both the channel definitions and pack definitions:

The following file contains the Common Definitions using the Recommendation ITU-R BS.2076 model in XML. It has been automatically generated from the spreadsheet. Due to the limitations of Word, an XML file cannot be embedded. So this is a Word document containing just the XML as text. To use, open the file, select all, then copy into a plain ASCII text file (using a text editor) and save it as “common_adm_def_v15.xml”.

If there is a difference between the text in this Recommendation and the attached files (XLSX, XML), the text of this Recommendation shall have priority.

Annex 2
(informative)

Overview of changes in this edition (Recommendation ITU-R BS.2094-2)

Modifications are as follows:

–	The audio channel labels of Low Frequency Effects -Left and -Right are changed to Low Frequency Effects −1 and −2 respectively in modified Tables 1 and 2.

–	New common definitions of audioChannelFormat for M+/-150 and B+/−030 with polar coordinate system are added in Table 1A.

–	New common definitions of audioPackFormat for the following sound systems with polar coordinate system are added in Table 2A.

•	Six sound systems without LFE used in production (7.0back, 7.0top, 7.0topside, 9.0, 11.0, 22.0.

•	Eight sound systems used in streaming / package media (13.1_9.1.4, 15.1_9.1.6, Auro-3D_8.0 (4+4+0), Auro-3D_10.1 (5+5+0), Auro-3D_11.1 (6+5+0), Auro-3D_11.1 (4+7+0), Auro-3D_13.1 (6+7+0), 360RealityAudio).

–	The common definitions of audioPackFormat for Auro-3D_(9+9+0) and audioChannelFormat for M+/−135_Difff with polar coordinate system are deprecated from modified Tables 2A and 1A.

–	New common definitions of audioChannelFormat and audioPackFormat for Cartesian coordinate system are added in Tables 1B and 2B.

–	Addition of text in § 5 regarding the priority of Recommendation text versus attached XML:

•	"If there is a difference between the text in this Recommendation and the attached files (XLSX, XML), the text of this Recommendation shall have priority."