RECOMMENDATION  ITU-R  BS.467*

Technical characteristics to be checked for
frequency-modulation stereophonic broadcasting

Pilot-tone system

(1970)

The ITU Radiocommunication Assembly,

recommends

1	that during programme transmission instruments should indicate the percentage of peak-modulation in the main carrier by the following:

1.1	the main channel, M;

1.2	the stereophonic sub-carrier, S;

1.3	the pilot signal;

1.4	all signals specified in Recommendation ITU-R BS.450, simultaneously;

2	that in addition, the following characteristics should be measured during periods of test and adjustment:

2.1	the frequency response of the M and individual A and B channels;

2.2	harmonic distortion in the individual A and B channels;

2.3	the signal-to-noise ratio in the individual A and B channels;

2.4	the crosstalk attenuation between the A and B channels;

2.5	the crosstalk from the main channel, M, into the stereophonic sub-channel, S, and from the stereophonic sub-channel, S into the main channel;

2.6	the frequency of the pilot signal;

2.7	the degree of suppression of the sub-carrier;

2.8	the phase of the sub-carrier relative to the pilot signal;

2.9	the total unintentional amplitude-modulation of the main carrier.

*	 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2002 in accordance with Resolution ITU-R 44.