RECOMMENDATION  560-3*

RADIO-FREQUENCY  PROTECTION  RATIOS  IN  LF,  MF  AND  HF  BROADCASTING

(Question 44/10, Study Programme 44A/10)

(1978-1982-1986-1990)

Rec. 560-3

The CCIR

UNANIMOUSLY  RECOMMENDS

that the radio-frequency protection ratios for sound broadcasting in bands 5 (LF), 6 (MF), and 7 (HF) as given in § 1 and 2 should be applied.

# 1.	Radio-frequency protection ratio in bands 5 (LF) and 6 (MF)

The radio-frequency protection ratio (as defined in Recommendation 638), for co-channel transmissions (± 50 Hz) should be 40 dB when both the wanted and the unwanted signals are stable (ground wave).

When the wanted signal is stable and the unwanted signal fluctuates (including short-term fluctuations), the radio-frequency protection ratio should be 40 dB at the reference time (see Annex I to Recommendation 435) for at least 50% of the nights of the year. This protection ratio corresponds to the ratio of the wanted field strength and the annual median value of the hourly medians of the interfering field strength at the reference time.

The protection so defined is provided:

–	for 50% of the nights at the reference time;

–	for more than 50% of the nights at times other than the reference time;

–	for 100% of the days during daylight hours.

The radio-frequency protection ratio values specified above will permit a service of excellent reception quality. For planning purposes, however, lower values may be required. In this respect, proposals have been made by some countries and organizations (see Annex III).

Note 1 – The minimum usable field strength to which this protection ratio of 40 dB applies varies in the different regions and with frequency. Within the European zone, this minimum is of the order of 1 mV/m.

Note 2 – A co-channel protection ratio of 26 dB was used by the Regional Administrative MF Broadcasting Conference (Region 2) (Rio de Janeiro, 1981) for both ground-wave and sky-wave services. Region 2 has two noise zones, 1 and 2, the former for most of the Region, the latter for a defined tropical area. In noise zone 1, the nominal usable field strength is 100 mV/m day-time and 500 mV/m night-time for Class A stations which have secondary service areas. It is 500 mV/m day-time for Classes B and C, and 2500 and 4000 mV/m respectively, night-time.

In noise zone 2, these values are generally 2.5 times more than the above figures.

Night-time protection, computed for two hours after sunset, is afforded for 50% of the nights of the year, except that the countries of North America agreed to protection from each other for 90% of the nights.

Note 3 – Co-channel protection ratios of 30 dB and 27 dB were used by the Regional Administrative LF/MF Broadcasting Conference for Regions 1 and 3, Geneva, 1975, for ground-wave and sky-wave services, respectively.

# 2.	Relative radio-frequency protection ratio curves in bands 5 (LF), 6 (MF) and 7 (HF)

The relative radio-frequency protection ratio is the difference, expressed in decibels, between the protection ratio when the carriers of the wanted and unwanted transmitters have a frequency difference of Df (Hz or kHz) and the protection ratio when the carriers of these transmitters have the same frequency.

Once a value for the co-channel radio-frequency protection ratio (which is equal to the audio-frequency protection ratio) has been determined, then the radio-frequency protection ratio, expressed as a function of the carrier-frequency spacing, is given by the curves of Fig. 1 (see also Annex I):

–	curve A, when a limited degree of modulation compression is applied at the transmitter input, such as in good quality transmissions, and when the bandwidth of the audio-frequency modulating signal is of the order of 10 kHz;

–	curve B, when a high degree of modulation compression (at least 10 dB greater than in the preceding case) is applied by means of an automatic device and when the bandwidth of the audio-frequency modulating signal is of the order of 10 kHz;

–	curve C, when a limited degree of modulation compression (as in the case of curve A) is applied and when the bandwidth of the audio-frequency modulating signal is of the order of 4.5 kHz;

–	curve D, when a high degree of modulation compression (as in the case of curve B) is applied by means of an automatic device and when the bandwidth of the audio-frequency modulating signal* is of the order of 4.5 kHz.

FIGURE  1       [D01]      =  10  cm

The curves A, B, C and D (see also Annex I) are valid only when the wanted and unwanted transmissions are compressed to the same extent. They have been obtained mainly from measurements and calculations with a reference receiver representative of good quality receivers used for reception in bands 5 (LF) and 6 (MF). The overall frequency response curve of the European Broadcasting Union (EBU) reference receiver used passes through –3 dB, –24 dB and –59 dB at 2 kHz, 5 kHz and 10 kHz, respectively [Petke, 1973].

REFERENCES

PETKE, G. [October, 1973] Determination by calculation of the RF protection ratio for AM transmission systems. EBU Rev. (Tech.), 141, 227-234.

ANNEX  I

The shape of the relative radio-frequency protection ratio curves depends on the receiver selectivity, on the bandwidth of the audio-frequency modulating signal, and also on the ratio of the energy of the carrier and of the sidebands. This latter phenomenon is most important between 250 Hz and 5 kHz approximately, where the disturbance is essentially due to the whistle produced by the carrier-frequency beat. The shape of the curves in Fig. 1 therefore depends on the average modulation depth and on the dynamic compression of the modulation signals.

Curve A represents average values derived from calculations and from tests made with various receivers mainly designed for reception in band 5 (LF) and band 6 (MF), with modulation compression typical of that currently applied in the studios, i.e. with compression permitting a maximum dynamic range of at least 30 dB.

Curve B applies to the use of compression, as applied by an automatic device, of at least 10 dB higher than in the preceding case.

Both curves A and B, as distinct from curves C and D, apply to a bandwidth of the audio-frequency modulating signal of the order of 10 kHz.

Curves C and D apply to the use of compression of the same order of magnitude as in the cases of curves A and B, respectively. The bandwidth of the audio-frequency modulating signal, is, however, restricted to about 4.5 kHz. This degree of bandwidth limitation reduces interference from adjacent channels without, in practice, leading to any significant degradation of the reception quality.

It should be noted that, in some circumstances, listeners are able to reduce the interfering effect of an unwanted transmission, spaced by more than approximately 3 kHz, by adjusting their receivers (slight detuning, selectivity control, tone control, etc.). Under these conditions, the curves of Fig. 1 are no longer applicable for spacings of more than about 3 kHz. However, the practice of detuning leads to distortion and cannot be used when two interfering emissions of approximately equal strength are present, on both sides of the wanted carrier frequency. Moreover, many receivers are not equipped with a selectivity control or tone control.

Note 1 – In addition to the relative radio-frequency protection ratios given in this Recommendation there are other factors of importance in determining optimum frequency spacings (see Question 44/10).

Note 2 – Caution should be exercised when relative values of radio-frequency protection ratio beyond –50 dB are obtained from the curves because, in practice, non-linear distortion originating in the transmitter may lead to poorer protection than indicated.

ANNEX  II

# Presentation of experimental results

Whenever possible, the results of measurements of the radio-frequency protection ratio between two broadcast signals should be presented in terms of the following characteristics and parameters:

–	type of modulation,

–	separation, between the carrier-frequencies (kHz) (this should lie between 0 and at least 10 kHz),

–	modulation depth of both signals,

–	occupied bandwidth,

–	modulation processing (compression and pre-emphasis),

–	type of the programmes of the wanted and unwanted signals,

–	characteristics of fading if present,

–	radio-frequency input voltage of the wanted signal (the radio-frequency input voltage should be chosen in such a way that the protection ratios are not significantly affected by non-linearities within the radio-frequency and intermediate frequency stages of the receiver),

–	passband of the receiver before demodulation,

–	overall response curve at audio-frequencies of the receiver, including the loudspeaker,

–	the grade of listener satisfaction aimed at and the statistical distribution of such grades,

–	the measuring method (subjective or objective).

ANNEX  III

PROTECTION  RATIOS  IN  LF,  MF  AND  HF  BROADCASTING

# 1.	Introduction

This Annex is a summary of the information available on the subject of protection ratios for amplitude-modulation sound-broadcasting services. It is confined, however, to results obtained since 1948.

Agreed values of protection ratios are essential for the solution of frequency assignment problems in amplitude-modulation sound broadcasting. Moreover, they may serve as basic reference data for the evaluation of the relative merits and the effectiveness to be expected with various amplitude-modulation transmission systems.

The protection ratios quoted refer, in all cases, to the ratios at the input to the receiver, no account having been taken of the effect of using directional receiving antennas.

Protection ratios depend on a multiplicity of parameters, among which transmission standards and receiver characteristics play an important role. Apart from technical factors there are others of a physiological and a psychological nature which have to be respected. It is, therefore, extraordinarily difficult to determine generally agreed values of protection ratios, even if both the transmission standards and the receiver characteristics are given (see Recommendation 559).

It is well known that the radio-frequency protection ratios for transmitters working in the same channel and transmitting the same programme can be improved considerably by synchronizing techniques, thereby increasing the coverage areas of these transmitters (see also Report 616). Actual values for these protection ratios depend on various factors, including the synchronization method (see § 10). A value of 8 dB was laid down at the Regional Administrative LF/MF Broadcasting Conference for Regions 1 and 3 [ITU, 1975].

2.	Audio-frequency protection ratio

The audio-frequency protection ratio is the agreed minimum value of the audio signal-to-interference ratio considered necessary to achieve a subjectively defined reception quality (see Recommendation 638).

This ratio may have different values according to the type of service desired. It depends greatly on the type of the wanted and the unwanted programme. It is essential, therefore, to carry out a considerable amount of subjective listening tests, before a minimum value of the audio frequency signal-to-interference ratio can be agreed upon.

It must clearly be stated that, due to physiological and psychological effects, it is completely impossible to fix reasonable values of the audio-frequency protection ratio by methods other than subjective tests.

3.	Radio-frequency protection ratio

The radio-frequency protection ratio is the value of the radio-frequency wanted-to-interfering signal ratio that enables, under specified conditions, the audio-frequency protection ratio to be obtained at the output of a receiver.

The radio-frequency protection ratio may, thus, be determined by means of subjective tests, as in the case of the audio-frequency protection ratio. When proceeding in this way the number of parameters to be taken into account and, hence, the amount of work to be done, will prove to be far greater than in the preceding case. Comparable results can only be obtained if the test conditions are rather similar.

However, the assessment of radio-frequency protection ratios can be considerably facilitated, once the audio-frequency protection ratio has been determined. Due to the fact that the majority of physiological and psychological effects only influence the audio-frequency protection ratio, it is possible to derive, under specified technical conditions and for a given value of the audio-frequency protection ratio, values of radio-frequency protection ratios, either by objective measuring methods or by graphical [Parreaux, 1972] or numerical [Petke, 1973; Gröschel, 1971] methods (see Recommendation 559).

It must be emphasised that the last mentioned three methods for the establishment of radio-frequency protection ratios, are based on the same basic ideas. They should lead, therefore, in principle, to the same results and, in fact, they do so, if the three methods are used with sufficiently high precision.

The lack of suitably reliable values for radio-frequency protection ratios in the past was mainly a consequence of the very complicated relationship between the radio-frequency protection ratio and the overall amplitude/frequency response of the receivers. The latter depends on the selectivity of the radio-frequency and the intermediate-frequency stages, the selectivity of the demodulator and the amplitude/frequency response of the audio-frequency stages. This difficulty has been partly overcome as a consequence of the establishment of the objective two-signal measuring method.

Numerical methods previously mentioned may however, be used to relate data on receiver selectivity characteristics, as provided by the receiver manufacturers, to values of radio-frequency protection ratio. Although the calculations are complicated and need electronic aids, they make possible, (in contrast to the objective measuring method), the determination of the overall frequency response of the receiver for a given radio-frequency protection ratio curve.

4.	General principle of non-subjective methods

All non-subjective methods assume the use of standardized conditions at both the transmitting and the receiving end of the transmission system, as described in Recommendation 559.

In all interference problems, there are two different types of annoyance:

–	the cross-talk from the interfering channel into the wanted channel, caused by modulation, and

–	the beat-note produced by both carriers.

The beat-note predominates in annoyance when the carrier-frequency separation is between about 0.25 kHz and 5 kHz, at least for the majority of receivers in use.

5.	Radio-frequency protection ratios for ground-wave services

5.1	Stable wanted and interfering signals (ground-wave signal interfered with by another ground-wave signal)

In § 1 of this Recommendation, a value of 40 dB is given for use in bands 5 (LF) and 6 (MF) for co-channel transmissions.

With this value of radio-frequency protection ratio a high quality of reception is possible. For planning purposes, however, it may be necessary to adopt lower values. This problem has been studied by the EBU [CCIR, 1970-74a] and in Japan [CCIR, 1970-74b]. The values that have been proposed are 30 dB and 26 dB, respectively, and in fact, a value of 30 dB was agreed by the Regional Administrative LF/MF Conference for Regions 1 and 3, whereas 26 dB was used by the Regional Administrative MF Broadcasting Conference (Region 2).

Relative values of radio-frequency protection ratios as a function of the separation between the carrier frequencies of the wanted and the interfering signal are given in the form of curves in § 2 of this Recommendation. These curves are based partly on measurements made in accordance with the objective two-signal method of measurement and partly on computations (see Recommendation 559).

The influence of dynamic compression and audio-frequency bandwidth limitation can also be seen from these curves. It should be noted, however, that the full improvement in protection resulting from bandwidth limitation can only be obtained when the non-linearity of the transmitter is small.

5.2	Stable wanted and fluctuating interfering signal [Belger et al., 1965]

5.2.1	Short-term fading

Short-term fading of the interfering signal modifies the character of the disturbance felt by the listener: if, for a given audio-frequency signal-to-interference ratio, the interfering signal is made to fluctuate, the disturbance is subjectively felt to be more severe. [CCIR, 1963-66a, b and c] indicate that, to obtain the same degree of satisfaction to the listener, the protection must be increased by about 5 dB in the latter case.

In § 1 of this Recommendation, the value for short-term fading has been incorporated in the radio-frequency protection ratio.

5.3	Long-term field-strength variations

Detailed information is contained in Report 266 and Recommendation 435.

6.	Radio-frequency protection ratios for sky-wave services

A characteristic of the sky-wave service, especially when reception is being made with envelope detectors, is that propagation effects usually bring about a degradation of the received signal quality, for example, distortion in the case of selective fading. Because of this fact, it is considered that lower values of protection ratios should be applied to a sky-wave service as compared with a ground-wave service, the precise values depending upon whether the service is a primary one, as for broadcasting in band 7 (HF), or a secondary one, as for broadcasting in bands 5 (LF) and 6 (MF), where the primary service is provided by the ground-wave.

No value is recommended for use when the service is provided by the sky wave.

6.1	Bands 5 (LF) and 6 (MF)

As a result of the studies carried out by the EBU [CCIR, 1970-74a], in bands 5 (LF) and 6 (MF), a co-channel RF protection-ratio value of 27 dB has been proposed and in fact adopted, by the Regional Administrative LF/MF Conference for Regions 1 and 3.

6.2	Band 7 (HF)

In band 7, following the studies carried out in India [CCIR, 1978-82a], the USA, and USSR and in the EBU, the radio-frequency protection ratio to be used for co-channel transmission (± 10 Hz, see Note) should be in the range of 27-40 dB for steady state conditions. According to the subjective assessments of reception quality carried out in Japan [CCIR, 1982-86a] and the People’s Republic of China [CCIR, 1982-86b], a co-channel protection ratio of 27 dB for steady-state conditions corresponds to grade 4 of the five-grade impairment scale (see Recommendation 562) with a carrier-frequency difference of 100 Hz or less.

Note – The permissible difference in carrier frequencies in band 7 (HF) can be as high as 600 Hz and is applicable to transmitters working at 20 MHz until January 1990 according to Appendix 7 of the Radio Regulations. After this date, this value applies only to transmitters of 10 kW power or less. For all other transmitters, the permissible frequency tolerance will be 10 Hz.

For planning purposes, a minimum value of 27 dB for stable conditions and a frequency difference of £ 100 Hz between the carriers is proposed.

For the determination of appropriate fading allowances, some information can be found in Reports 266 and 894. It should be noted, however, that other factors, such as the correlation between the fading of the wanted and the unwanted signal, need also to be taken into account.

We may conveniently distinguish between two types of within-the-hour fade: short-term fades due to interference between individual signal components with a correlation period of up to a few seconds, and long-term phenomena in which the signals averaged over a few minutes fade in periods of up to tens of minutes.

Interference-type within-the-hour wanted signal fades affect only the subjective signal quality, which is significantly improved by receiver AGC. A suitable duration for purposes of quality evaluation is 1 min.

For the evaluation of fading margins or channel reliability, we may conveniently apply the statistical data for within-the-hour fades, characterized by periods of a few minutes or longer. The standard deviation of this type of fade may vary within wide limits depending on the ionospheric conditions and the location of the path, the path length and its direction. This applies to the wanted and interfering signals alike.

7.	Data available on protection ratios

Annex I to Recommendation 639 deals with the effect of a limitation of the bandwidth of emission on radio-frequency protection ratios. Further information on protection ratios may be found in [Belger and von Rautenfeld, 1958; Liedtke, 1965]. Additional data for broadcasting in band 7 (HF) are contained in Recommendation 411. The curves reproduced in Report 302 represent data at present available on the subject of Study Programme 45E/10 and refer principally to the protection ratios required to provide an acceptable broadcasting service in the Tropical Zone in the shared bands.

8.	Measurement results

Measurement of protection ratios for stable signals were carried out in the USSR [CCIR, 1978-82b]. The following quality criteria were used as the basis of an experiment:

–	perceptibility of interference in the background of the wanted programme,

–	tolerance to interference while listening to the wanted speech programme.

Quantitative assessments using these criteria were made in the USSR by a number of experts subjectively assessing an agreement between a given fragment of the programme and a predetermined criterion.

The results of measurement are given in Figs. 2, 3 and 4.

Figure 2 shows the results of the co-channel protection ratio measurements. The value of K denotes the proportion of the experts who found the interference tolerable. The interfering programme was modern dance music. The shaded parts correspond to a change of passband from wide to narrow.

Figure 3 shows the values of protection ratio as a function of the wanted and interfering carrier-frequency spacing for K = 0.9. Shaded parts of the curves correspond to the scatter determined by the receiver passband.

Figure 4 gives the receiver selectivity curves, where the shaded parts indicate the dispersion of curves for receivers of different classes. Also shown in Fig. 4 is the selectivity curve of the EBU MBF receiver.

In addition, the effect of the modulating signal bandwidth on protection ratios was studied for the highest-class receiver. The modulating frequency bandwidths of both the wanted and interfering signals were limited using two identical switched filters with the cut-off frequencies f = 10 kHz, 6.8 kHz and 3.4 kHz and with frequency response slopes of 90 dB/decade. The results of measurements showed that a change of modulating signal bandwidths has no pronounced effect on protection ratios.

9.	Subjective assessment of the quality of reception

9.1	Investigations carried out in the USSR

Statistical and subjective tests were carried out in the USSR on the effects of distortion and interference in a broadcast channel.

The tests were performed using a statistically based subjective method, using special equipment which enabled a comparison to be made between an undistorted sound programme and a second programme, into which predetermined levels of distortion had been injected.

The object of these experiments was to determine the perceptibility of distortion and the following groups of listeners participated:

–	qualified experts (sound-broadcasting producers),

–	observers without special musical education and without training in the observation of distortion.

The results of these experiments were published in the form of graphs, showing the percentage of perceptibility as a function of the level of the distortion or interference injected.

All these tests were made on the basis of a large amount of statistical data. The correctness of the data obtained was checked by the methods of mathematical statistics. The results were given in terms of:

–	linear distortion of different types (at various levels and for different frequency ranges),

–	non-linear distortion (cubic, quadratic and “central cut-off” types),

–	background noise (sinusoidal),

–	white noise.

A comparison of the reception quality determined by the objective method or subjective tests shows differences in the necessary protection ratio. This difference may reach 10 dB for a frequency spacing of 9 kHz [CCIR, 1974-78a].

9.2	Investigations carried out in Japan

Results of a subjective assessment carried out in Japan [CCIR, 1982-86a] concerning the relationship between reception quality and radio-frequency wanted-to-interfering signal ratio is shown in Fig. 5(a) for co-channel interference and Fig. 5(b) for adjacent-channel interference. Listening tests were made using three receivers (A, F, H) by ten experts.

FIGURE  2       [D02]      =  15  cm  =  page pleine

FIGURE  3       [D03]      =  19  cm  =  page pleine

FIGURE  4       [D04]      =  15  cm  =  page pleine

FIGURE  5       [D05]      =  22  cm  =  page pleine

10.	Radio-frequency protection ratios for synchronized broadcasting transmitters

10.1	Investigations carried out in the USSR

These investigations were carried out to determine values of signal-to-interference ratio applicable to reception of transmissions from synchronized transmitter groups comprising two or three transmitters. Both phase and frequency methods of synchronization were considered.

10.1.1	Explanation of the term “radio-frequency protection ratio”

The term “protection ratio” in this context means the ratio of the field strength of the strongest signal from one of the transmitters in the synchronized group to the resultant field strength of the remaining transmitters in the same group.

10.1.2	Determination of the protection ratio

For the purpose of determining the protection ratio, use was made of a statistical method based on subjective impressions of reception quality from a transmitter in a synchronized group, compared with reception quality of a single non-synchronized transmitter station. Twenty-six experts were employed – all of whom were technical and scientific broadcasting staff.

Protection ratio values for non-fading signals were determined under laboratory conditions and later verified under operational conditions.

For fading signals only operational tests using a synchronized network were carried out.

For all these tests the depth of maximum modulation was 90%.

Figure 6 shows the variations in protection ratio as a function of the phase difference between the carriers of two stations during day-time in the absence of fading. The parameter used in these curves is the percentage of experts who rate the total signal as being at least satisfactory. It will be seen from this figure that, to satisfy 90% of the listeners, the protection ratio for a network consisting of two synchronized stations for reception without fading was 4 dB.

FIGURE  6       [D06]      =  7  cm

Figure 7 shows the variation in protection ratio as a function of the difference in frequency between two synchronized transmitters for the percentage of experts who found the reception quality to be satisfactory. This figure shows that, for non-fading signals with two synchronized transmitters and a protection ratio of 4 dB, it is necessary to have a synchronization accurate to 0.015 to 0.02 Hz, to satisfy 90% of listeners. With a frequency difference of 0.1 Hz the protection ratio has to be increased to 6 dB.

FIGURE  7       [D07]      =  7  cm

Figure 8 contains similar results for phase synchronization operation of three transmitters. To satisfy 90% of the listeners the protection ratio should not be less than 3.1 dB. (The standard value is 4 dB.)

It is concluded that when reception is affected by fading it will be necessary to increase the protection ratio to 7 or 8 dB in the case of two synchronized transmitters, and to 6 dB in the case of three transmitters.

FIGURE  8       [D08]      =  12  cm

10.2	Investigations carried out within the EBU

Synchronizing techniques as developed up to 1964 (and in most cases still in use) in several countries, notably, Austria, France, Federal Republic of Germany, Italy, Netherlands, Norway, Sweden, United Kingdom, Australia and the United States of America, are described in [EBU, 1974] which contains an extensive bibliography, as well as a survey of the theoretical basis of these techniques.

10.3	Investigations carried out within the OIRT

In [Augustin and Schulze, 1973], it was found that it is possible to simulate all the essential effects of synchronized or non-synchronized common channel systems (same programme) in practice, by a model. Such a model was developed by the Rundfunk- und Fernsehtechnisches Zentralamt of the Deutsche Post, Berlin. That model offers both economical and operational advantages, when carrying out studies on reception problems in the area, where the ground waves of synchronized transmitters interfere; i.e., without taking into account ionospheric fading effects.

With the use of the above-mentioned model system, it is possible to study synchronized or non-synchronized common channel systems in the laboratory. In particular it has been found that the following measures are advantageous [Augustin and Schulze, 1973]. If the carrier frequencies differ by only about 0.1 Hz and if the delay times of the sound signals between the studio and the transmitters of this system have been equalized, the following advantages will result:

–	decrease of the interference zone to almost zero, and

–	decrease of the selective fading effects, so that only amplitude fading effects still remain, which are entirely compensated for by the automatic gain control of the receiver, without unacceptable distortions.

In conclusion, it can be said that these effects permit a protection ratio of 0 dB for day-time reception.

These theoretical results have been confirmed in field tests by using two transmitters of 20 kW operating in band 6 (MF) with a spacing of about 80 km.

10.4	Investigations carried out in the United Kingdom

10.4.1	Equalization of modulation delay

Laboratory tests conducted by the BBC [Whythe, 1976] to study the effect of modulation delay in a common-channel, same-programme system gave the results shown in Table I applicable to two carriers, with the order of 0.1 Hz frequency difference, modulated by a music programme. The impairment was slightly less severe when using a speech programme.

Given the field-strength contours for a two-transmitter situation the results of Table I can be used to assess the improvement to daytime reception that modulation-delay equalization could provide over any affected region. In most cases, improvement is gained over the greatest area if the modulation delays are equated at the point where the equal-field-strength locus crosses the straight line joining the two transmitters. It may be preferable, however, to equate the modulation delays at some other point, for example, near a highly populated part of the affected region, in order to improve daytime reception for the greatest number of listeners.

TABLE  I

| Carrier-amplitude | Subjective grade of impairment exceeded for 10% of the time for modulation-delay inequality of: (1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ratio (dB) | 0 | 30 ms | 50 ms | 84 ms | 250 ms | 1 ms |
| 0 | 4 | 3 | 2 | 1 | 1 | 1 |
| 3 | 5 | 5 | 4.5 | 2.5 | 1 | 1 |
| 6 | 5 | 5 | 5 | 5 | 3.5 | 2 |
| 9 | 5 | 5 | 5 | 5 | 5 | 5 |

(1)	5-grade subjective impairment scale (Recommendation 562).

A nine-months field trial between the BBC medium wave transmitters (1214 kHz) at Brookmans Park (50 kW, directional antenna) and Droitwich (30 kW, omni-directional antenna), which are 145 km apart, largely confirmed the results of Table I and showed the arrangement to be practicable and stable and to impose very little demand operationally. The trial revealed the following practical aspects:

–	It was found practicable to achieve and maintain phase equalization to within ± 30° over the modulation band 50 Hz to 4 kHz at the chosen receiving point.

–	After inserting the phase equalization, the residual distortion was nevertheless still significant in practice, in regions where the two field strengths differed by less than 1.5 dB; in other words, a protection ratio of 1.5 dB is still required for good reception.

10.4.2	Carrier phase locking

A system of carrier phase locking has been introduced in the United Kingdom [Millard et al., 1979] for three common frequency transmitters on 200 kHz. The transmitters in question are situated in Droitwich in the centre of England, at Westerglen near Edinburgh and at Burghead in north-east Scotland. The phase locking is used in conjunction with audio delay equalization (see § 10.4.1) so that the signals received in the approximately equal field strength areas between adjacent stations (i.e. between Droitwich and Westerglen and between Westerglen and Burghead) are maintained with carriers in phase and with the timings of the modulation envelopes equalized. Furthermore, the three transmitters are driven with rubidium standard units of high frequency stability. In the critical areas, therefore, the standing wave patterns remain fixed and distortion is minimized and a stable, unvarying situation is provided for listeners. Receivers with ferrite-rod antennas may experience poor reception only on or near lines joining the transmitters. Elsewhere the magnetic fields act in different directions and do not cancel [Knight, 1980].

Experience to date indicates that the system is providing a good nationwide service.

Consideration is being given to the application of the technique to MF broadcasting.

10.5	Investigations carried out in India

Subjective tests were carried out in India with a view to determining the protection ratio applicable to synchronized transmitter groups in band 7 (HF). These tests indicated that a large majority of the listeners did not observe any degradation in the quality of programme even in a zone where the field strength of the synchronized transmitters differed only by 2-3 dB at a distance of approximately 2000 km from the transmitters. It has therefore been concluded that in the case of synchronized transmitters in band 7, the appropriate co-channel protection ratio would be as low as 3 dB if the transmitters are driven by a common oscillator and operate with antennas of similar vertical radiation characteristics [CCIR, 1978-82c; CCIR, 1982-86c].

10.6	Investigations carried out in Japan

A synchronized network composed of seven MF transmitters has been operated in Okayama, Japan, since January 1984, using an identical carrier frequency in the MF broadcasting band [CCIR, 1982-86d]. It is necessary that the following conditions are met, in order to improve the quality of signals received in the interference zones:

–	transmitters comprising the network have an identical carrier frequency and are phase-locked to each other, using a synchronizing signal transmitted via radio-relay links between the broadcasting transmitters. In addition, the phase of the two carrier signals is maintained in phase in the area where both carrier signals propagate in the same direction;

–	the field strength of each transmitter is set at not less than 1 mV/m in the interference zones where the number of listeners is large;

–	the phase difference of modulation signals is maintained within 5° over the frequency band from 80 Hz to 4 kHz in the interference zone.

Seven transmitters of an identical carrier frequency (1494 kHz) having a total power of 9.2 kW offer a coverage area of about 80 km by 90 km which corresponds approximately to the area (for the Okayama district) which a single high-power transmitter of 150 kW at the main station would provide. When the above three conditions are satisfied, a good enough quality of received signals is obtained in the interference zones, as long as the S/I ratio is not less than 1 dB (Fig. 9).

Due to its size, no interference due to short-range fading or multipath propagation was experienced in the coverage area. However, in large coverage areas this type of interference may lead to some difficulty in achieving the same results.

FIGURE  9       [D09]      =  15  cm

REFERENCES

AUGUSTIN, E. and SCHULZE, R. [1973] Probleme des Gleichwellenrundfunks: ein optimales Gleichwellensystem (Problems of synchronized broadcasting transmissions: An optimum system of synchronized transmitters). Techn. Mitt. RFZ, 4, 95-97.

BELGER, E. and VON RAUTENFELD, F. [1958] Der erforderliche Gleichkanal- und Nachbarkanal-Störabstand beim amplitudenmodulierten Tonrundfunk (The protection ratios required for common channels and adjacent channels in amplitude-modulated sound broadcasting). Rundfunktechn. Mitt., 2, 172-177.

BELGER, E., VON RAUTENFELD, F. and VOGT, K. [April, 1965] The wanted-to-interfering signal ratio required with an interfering signal fluctuating in time, in amplitude-modulation sound broadcasting. EBU Rev., Part A – Technical, 90, 69-72.

EBU [1974] Synchronization groups of transmitters in LF and MF broadcasting. Doc. Tech. No. 3210.

GRÖSCHEL, G. [1971] Out-of-band radiation and its influence on the adjacent channel with amplitude-modulated sound broadcasting. Nachrichtentechn. Fachber., 41, 41-49.

ITU [1975] Final Acts of the Regional Administrative LF/MF Broadcasting Conference for Regions 1 and 3, Geneva, Switzerland.

KNIGHT, P. [1980] Reception of phase-locked LF and MF transmissions by receivers having ferrite-rod antennas. Electron. Lett., Vol. 16, 1, 40.

LIEDTKE, G. R. [April, 1965] The effect of receiver characteristics and radiated bandwidth on the RF wanted-to-interfering signal ratio in amplitude-modulation sound broadcasting. EBU Rev. Tech., Part A, 90, 65-68.

MILLARD, G. H., THODAY, R. D. C. and PRICE, H. M. [1979] Synchronization of the Radio-4 (United Kingdom) transmitter chain on 200 kHz. BBC Research Report 1979/27.

PARREAUX, D. [August, 1972] A graphical method of determining protection ratios in the case of LF and MF amplitude-modulated transmissions. EBU Rev. Tech., 134, 163-167.

PETKE, G. [October, 1973] Determination by calculation of the RF protection ratio for AM transmission systems. EBU Rev. Tech., 141, 227-234.

WHYTHE, D. J. [1976] Reduction of mush-area distortion in common-frequency MF transmitter networks. BBC Research Report 1976/5.

CCIR Documents

[1963-66]: a. X/5 (EBU); b. X/31 (Germany (Federal Republic of)); c. X/36 (France).

[1970-74]: a. 10/266 (EBU); b. 10/286 (Japan).

[1974-78]: a. 10/324 (USSR).

[1978-82]: a. 10/245 (India); b. 10/209 (Rev. 1) (USSR); c. 10/82 (India).

[1982-86]: a. 10/6 (Japan); b. 10/112 (China (People’s Republic of)); c. 10/69 (India); d. 10/209 (Japan).

BIBLIOGRAPHY

BACHMUTOVA, N. I., PYLAEVA, Z. D., TISHIN, S. A. and YAMPOLSKAYA, N. G. [1975] Protection ratios in LF/MF European broadcasting networks. Elektrosviaz, 7, 22-24, Moscow, USSR.

KOKOREV, A. V. [1976] Choice of adjacent channel protection ratios for LF/MF broadcasting systems. NIIR Proc., 4.

KOKOREV, A. V. and SHLÜGER, I. S. [1977] Protection ratios in band 7 (HF). Elektrosviaz, 10, 33-38, Moscow, USSR.

SHAMSHIN, V. A. and SHLÜGER, I. S. [October, 1975] Adjacent channel protection ratios for LF/MF sound broadcasting (150-1605 kHz). Telecomm. J., Vol. 42, X, 606-607.

CCIR Documents

[1963-66]: X/4 + Add. 1 (Germany (Federal Republic of)).

[1986-90]: 10/59 (USSR).

ANNEX  IV

PLANNING  PARAMETERS  IN  BAND  7  (HF)
CONSIDERED  BY  THE  WARC  HFBC(2)

The WARC HFBC(2) considered that RF protection ratios, minimum usable field strengths and signal fading allowances are basic planning parameters which may be improved as a result of further studies and recommended that the IFRB should use the following parameters in its Technical Standards.

1.	Radio-frequency protection ratios

1.1	Protection ratio for unsynchronized transmissions

The HFBC Planning System shall endeavour to satisfy the requirements with a minimal co-channel RF protection ratio of 17 dB without taking account of the fading allowances and multiple interference entries. In cases of congestion this ratio may be lowered until the congestion is resolved.

1.2	Protection ratio for synchronized transmissions

The co-channel protection ratio between synchronized transmissions in the same network should be:

| Distance L between synchronized transmitters (km) | Protection ratio (dB) |
| --- | --- |
| L £ 700 700 < L £ 2500 2500 < L | 0 4 8 |

1.3	Relative radio-frequency protection ratios

The relative RF protection ratios (a) for carrier frequency separations* (Df), with reference to the co-channel protection ratio, should be:

| Df (kHz) | a (dB) |
| --- | --- |
| 0 ± 5 ± 10 ± 15 ± 20 | 0 –3 –35 –49 –54 |

# 2.	Minimum usable field strength

The minimum usable field strength should be determined by adding 34 dB to the greater of:

–	the field strength due to atmospheric radio noise as contained in Report 322,

–	3.5 dB(mV/m), which is the intrinsic receiver noise level.

# 3.	Signal fading allowance

3.1	Short-term (within the hour) fading

The upper-decile amplitude deviation from the median of a single signal is to be taken as 5 dB and the lower-decile is to be taken as –8 dB.

3.2	Long-term (day-to-day) fading

The magnitude of the long-term fading, as determined by the ratio of the operating frequency to the basic MUF, is given in Table III of CCIR Report 266.

For synchronized transmissions, the fading allowance associated with the predominant signal should be used. In cases where the contributing wanted field strengths are equal and Note 1 of Table III of CCIR Report 266 applies to at least one of the paths, the values for geomagnetic latitudes ³ 60° should be used.

3.3	Combined distribution of fading applicable to wanted and unwanted signals

The fading allowances for 10% and 90% of the time are each to be taken as 10 dB, except where the provisions of the following notes apply. In the latter case, 14 dB is to be used.

Note 1 – If any point on that part of the great circle which passes through the transmitter and the receiver, and which lies between control points located 1000 km from each end of the path reaches a corrected geomagnetic latitude of 60° or more, the values for ³ 60° must be used.

Note 2 – These values relate to the path of the wanted signal only.

Note 3 – For synchronized emissions, the fading allowance associated with the predominant wanted signal is to be used. For those conditions where the constituent wanted field strengths are equal and Note 1 above applies to at least one of the paths, the value of 14 dB is to be used for the decile values.

_________________

*	Annex III to this Recommendation replaces Report 794, which is hereby cancelled.

*	The WARC HFBC(2) decided, that the upper limit of the audio-frequency band (at –3 dB) of the transmitter shall not exceed 4.5 kHz and the lower limit shall be 150 Hz, with lower frequencies attenuated at a slope of 6 dB per octave.

If audio-frequency signal processing is used, the dynamic range of the modulating signal shall be not less than 20 dB.

*	Frequency separations Df < –20 kHz, as well as Df > +20 kHz, need not be considered.