RECOMMENDATION  ITU-R  BS.647-2*,**

A digital audio interface for broadcasting studios

(1986-1990-1992)

The ITU Radiocommunication Assembly,

considering

a)	that there will be a need to interconnect in the digital domain the various different items of digital audio equipment that will be found in broadcasting studios; these will include digital audio tape recorders, the audio system of digital video tape recorders, mixers, etc.;

b)	that there would be advantages if all equipment used the same interface connections;

c)	that the interface should make allowance for processing headroom in terms of resolution;

d)	that the interface should allow for auxiliary data of various kinds;

e)	that changes to this Recommendation should be made only after due consideration and study,

recommends

1	that the interface described in Annex 1 should be adopted;

2	that proposals for the amendment of this Recommendation awaiting consideration should first be attached to Annex 2 to this Recommendation prior to their possible adoption by the ITU-R and the consequent amendment of Annex 1.

Annex 1

# 1	Scope

This Annex specifies a recommended interface for the serial digital transmission of two channels of periodically sampled and linearly represented digital audio data in a broadcasting complex, up to a distance of a few hundred metres.

Although this transmission specification is independent of sampling frequency the interface is primarily intended to be used at 48 kHz as this is the recommended sampling frequency for use in broadcasting studios (Recommendation ITU-R BS.646).

The Annex does not cover connection to any common carrier equipment nor does it specifically address any of the questions relating to the synchronizing of large systems, although by its nature the format permits easy synchronization of receiving devices to the transmitting device.

NOTE 1 – In this interface specification for broadcasting studio use, mention is also made of an interface for consumer use. The two interfaces are not identical.

NOTE 2 – An engineering guideline document to accompany this interface specification is in the course of preparation.

# 2	Terminology

For the purpose of this specification the following definitions of terms apply.

## 2.1	Sampling frequency

The sampling frequency is the frequency of the samples representing an audio signal. When more than one audio signal is transmitted through the same interface, the sampling frequencies shall be identical.

## 2.2	Audio sample word

The audio sample word represents the amplitude of a digital audio sample. Representation is linear in 2 s complement binary form. Positive numbers correspond to positive analogue voltages at the input of the Analogue to Digital Convertor (ADC). The number of bits per word can be specified from 16 to 24 in two coding ranges (less than or equal to 20 bits and less than or equal to 24 bits).

## 2.3	Auxiliary sample bits

The 4 least significant bits (LSB) can be assigned as auxiliary sample bits and used for auxiliary information when the number of audio sample bits is less than or equal to 20.

## 2.4	Validity bit

This bit indicates whether the audio sample bits in the subframe (time slots 4-27 or 8-27 depending on the audio word length as described in § 3.1 are suitable for conversion to an analogue audio signal.

## 2.5	Channel status

The channel status carries, in a fixed format derived from the block (see § 2.11), information associated with each audio channel which is decodable by any interface user.

## 2.6	User data

The user data channel is provided to carry any other information.

## 2.7	Parity bit

The parity bit is provided to permit the detection of an odd number of errors resulting from malfunctions in the interface.

## 2.8	Preambles

Preambles are specific patterns used for synchronization. There are three different preambles (see § 3.4).

## 2.9	Subframe

The subframe is a fixed structure used to carry the information described in § 2.2 to 2.8 (see § 3.1 and 3.2).

## 2.10	Frame

The frame is a sequence of two successive and associated subframes.

## 2.11	Block

The block is a group of 192 consecutive frames. The start of a block is designated by a special subframe preamble (see § 3.4).

## 2.12	Channel coding

The channel coding describes the method by which the binary digits are represented for transmission through the interface.

# 3	Interface format

## 3.1	Subframe format

Each subframe is divided into 32 time slots, numbered from 0 to 31 (see Fig. 1).

Time slots 0 to 3 (preamble) carry one of the three permitted preambles (see Fig. 2 and § 3.2 and 3.4).

Time slots 4 to 27 (audio sample word) carry the audio sample word in linear 2 s complement representation. The most significant bit (MSB) is carried by time slot 27.

When a 24-bit coding range is used, the LSB is in time slot 4 (see Fig. 1a).

When a 20-bit coding range is sufficient, time slots 8-27 carry the audio sample word with the LSB in time slot 8. Time slots 4 to 7 may be used for other applications. Under these circumstances, the bits in the time slots 4 to 7 are designated auxiliary sample bits (see Fig. 1b).

If the source provides fewer bits than the interface allows (either 24 or 20) the unused LSB shall be set to logic “0”.

Time slot 28 (validity bit) carries the validity bit associated with the audio sample word such that the validity bit shall be logic “0” if the audio sample word is suitable for conversion to an analogue audio signal and it shall be logic “1” if it is not.

There is no default state for the validity bit.

Time slot 29 (user data bit) carries one bit of the user data channel associated with the audio channel transmitted in the same subframe (see § 3).

Time slot 30 (channel status bit) carries one bit of the channel status information associated with the audio channel transmitted in the same subframe (see § 4).

Time slot 31 (parity bit) carries a parity bit such that time slots 4 to 31 inclusive will carry an even number of logic “1” and an even number of logic “0” (even parity).

NOTE 1 – The preambles have even parity as an explicit property.

## 3.2	Frame format

A frame is uniquely composed of two subframes (see Fig. 2). The rate of transmission of frames corresponds exactly to the source sampling frequency.

The first subframe normally starts with preamble “X”, however the preamble changes to preamble “Z” once every 192 frames. This defines the block structure used to organize the channel status information. The second subframe always starts with preamble “Y”.

The modes of transmission are signalled by setting bits 0 to 3 of byte 1 of channel status.

Two-channel mode

In two-channel mode the samples from both channels are transmitted in consecutive subframes. Channel 1 is in subframe 1 and channel 2 is in subframe 2.

Stereophonic mode

In stereophonic mode the interface is used to transmit stereophonic audio in which the two channels are presumed to have been simultaneously sampled. The left or “A” channel is in subframe 1 and the right or “B” channel is in subframe 2.

Single channel mode (monophonic)

In monophonic mode the transmitted bit rate shall remain at the normal 2 channel rate and the audio sample word shall be placed in subframe 1. Time slots 4 to 31 of subframe 2 shall either carry the identical bits to subframe 1 or shall be set to logic “0”. A receiver shall normally default to channel 1 unless manual override is provided.

Primary/secondary mode

In some applications requiring two channels where one of the channels is the main or primary channel while the other is a secondary channel, the primary channel is in subframe 1 and the secondary channel is in subframe 2.

## 3.3	Channel coding

To minimize the direct current (d.c.) component on the transmission line, to facilitate clock recovery from the data stream and to make the interface insensitive to the polarity of connections, time slots 4 to 31 are encoded in biphase-mark.

Each bit to be transmitted is represented by a symbol comprising two consecutive binary states. The first state of a symbol is always different from the second state of the previous symbol. The second state of the symbol is identical to the first if the bit to be transmitted is logic “0”, however, it is different if the bit is logic “1” (see Fig. 3).

## 3.4	Preambles

Preambles are specific patterns providing synchronization and identification of the subframes and blocks.

To achieve synchronization within one sampling period and to make this process completely reliable, these patterns violate the biphase mark code rules, thereby avoiding the possibility of data imitating the preambles.

A set of three preambles is used. These preambles are transmitted in the time allocated to four time slots at the start of each subframe (time slots 0 to 3) and are represented by eight successive states. The first state of the preamble is always different from the second state of the previous symbol (representing the parity bit). Depending on this state the preambles are:

| Preceding state | 0 1 1 1 0 0 0 1 | 1 1 1 1 0 0 0 1 |  |
| --- | --- | --- | --- |
|  | Channel coding |  |  |
| “X” | 1 1 1 0 0 0 1 0 | 0 0 0 1 1 1 0 1 | Subframe 1 |
| “Y” | 1 1 1 0 0 1 0 0 | 0 0 0 1 1 0 1 1 | Subframe 2 |
| “Z” | 1 1 1 0 1 0 0 0 | 0 0 0 1 0 1 1 1 | Subframe 1  block start |

Like biphase code, these preambles are d.c. free and provide clock recovery. They differ in at least two states from any valid biphase sequence.

Figure 4 represents preamble “X”.

NOTE 1 – Owing to the even parity bit in time slot 31, all preambles will start with a transition in the same direction (see § 3.1). Thus only one of these sets of preambles will, in practice, be transmitted through the interface. However, it is necessary for either set to be decodable because a polarity reversal may occur in the connection.

## 3.5	User data format

User data bits may be used in any way desired by the user.

Possible formats for the user data channel are indicated by channel status byte 1 bits 4-7.

The default value of the user data bit shall be logic “0”.

## 3.6	Channel status format

The channel status for each audio signal carries information associated with that audio signal, and thus it is possible for different channel status data to be carried in the two subframes of the digital audio signal. Examples of information to be carried in the channel status are: length of audio sample words, number of audio channels, sampling frequency, time code, alphanumeric source and destination codes, and pre-emphasis.

Channel status information is organized in 192-bit blocks, sub-divided into 24 bytes (Fig. 5). The first bit of each block is carried in the frame with preamble “Z”.

The specific organization follows, wherein the suffix 0 designates the first byte or bit:

| NOTE 1 – The significance of byte 0, bit 0 is such that a transmission from an interface conforming to IEC 958 “consumer use” can be identified, and receiver conforming only to IEC 958 “consumer use” will correctly identify a transmission from a “professional use” interface as defined in this standard. Connection of a “professional use” transmitter with a “consumer use” receiver or vice versa might result in unpredictable operation. Byte 0 |  |  |  |
| --- | --- | --- | --- |
| Bit 0 | 0 3 4 1 3 4 | Consumer use of channel status block (see Note 1) Professional use of channel status block |  |
| Bit 1 | 0 3 4 1 3 4 | Normal audio mode Non-audio mode |  |
| Bits 2-4 | Encoded audio signal emphasis |  |  |
| Bit State | 2 3 4 0 0 0 1 0 0 1 1 0 1 1 1 | Emphasis not indicated. Receiver defaults to no emphasis with manual override enabled No emphasis. Receiver manual override disabled 50/15 s emphasis. Receiver manual override disabled ITU-T Recommendation J.17 emphasis (with 6.5 dB insertion loss at 800 Hz). Receiver manual override disabled |  |
|  | All other states of bits 2-4 are reserved and shall not be used until further defined |  |  |
| Bit 5 | 1 3 4 0 3 4 |  | Source sampling frequency unlocked Default and source sampling frequency locked |
| Bits 6-7 | Encoded sampling frequency |  |  |
| Bit State | 6 7 0 0 0 0 0 1 0 1 0 0 1 1 0 |  | Sampling frequency not indicated. Receiver default to 48 kHz and manual override or auto set enabled 48 kHz sampling frequency. Manual override or auto set disabled 44.1 kHz sampling frequency. Manual override or auto set disabled 32 kHz sampling frequency. Manual override or auto set disabled |

| (1) This system is specified in Supplement 1 to EBU Tech. 3250: format of the user data channel of the digital audio interface. Byte 1 |  |  |
| --- | --- | --- |
| Bits 0-3 | Encoded channel mode |  |
| Bit State | 0 1 2 3 0 0 0 0 | Mode not indicated. Receiver default to two-channel mode. Manual override enabled |
|  | 0 0 0 1 | Two channel mode. Manual override disabled |
|  | 0 0 1 0 | Single channel mode (monophonic). Manual override disabled |
|  | 0 0 1 1 | Primary/secondary mode (subframe 1 is primary). Manual override disabled |
|  | 0 1 0 0 | Stereophonic mode (channel 1 is left channel). Manual override disabled |
|  | 0 1 0 1 0 1 1 0 | Reserved for user defined applications Reserved for user defined applications |
|  | 1 1 1 1 | Vector to byte 3. Reserved for future applications |
|  | All other states of bits 0-3 are reserved and shall not be used until further defined |  |
| Bits 4-7 | Encoded user bits management |  |
| Bit State | 4 5 6 7 0 0 0 0 | Default, no user information indicated |
|  | 0 0 0 1 0 0 1 0 0 0 1 1 | 192 bit block structure. Preamble “Z” indicates the start of a block Packet system based on high level data link control (HDLC) protocol (1) User defined |
|  | All other states of bits 4-7 are reserved and shall not be used until further defined |  |

| NOTE 1 – The signal coding used for the coordination channel is described in Appendix 1. Byte 2 |  |  |  |
| --- | --- | --- | --- |
| Bits 0-2 | Encoded use of auxiliary sample bits |  |  |
| Bit State | 0 1 2 0 0 0 | Maximum audio sample word length is 20 bits (default). Use of auxiliary sample bits not defined |  |
|  | 0 0 1 | Maximum audio sample word length is 24 bits. Auxiliary sample bits used for main audio sample data |  |
|  | 0 1 0 | Maximum audio sample word length is 20 bits. Auxiliary sample bits in this channel used to carry a single coordination signal (see Note 1) |  |
|  | 0 1 1 | Reserved for user defined applications |  |
|  | All other states of bits 0-2 are reserved and shall not be used until further defined |  |  |
| Bits 3-5 | Encoded audio sample word length of transmitted signal (see Notes 2, 3 and 4) |  |  |
|  |  | Audio sample word length if maximum length is 24 bits as indicated by bits 0-2 above | Audio sample word length if maximum length is 20 bits as indicated by bits 0-2 above |
| Bit State | 3 4 5 0 0 0 0 0 1 0 1 0 0 1 1 1 0 0 1 0 1 | Word length not indicated (default) 23 bits 22 bits 21 bits 20 bits 24 bits | Word length not indicated (default) 19 bits 18 bits 17 bits 16 bits 20 bits |
|  | All other states of bits 3-5 are reserved and shall not be used until further defined |  |  |
| Bits 6-7 | Reserved and shall be set to logic “0” until further defined |  |  |

NOTE 2 – The default state of bits 3-5 indicates that the number of active bits within the 20 or 24 bits coding range is not specified by the transmitter. The receiver should default to the maximum number of bits specified by the coding range and enable manual override or auto set.

NOTE 3 – The non-default states of bits 3-5 indicate the number of active bits within the 20 or 24 bits coding range which might be active. This is also an indirect expression of the number of LSBs that are certain to be inactive which is equal to 20 or 24 minus the number corresponding to the bit state. The receiver should disable manual override and auto set for these bit states.

NOTE 4 – Irrespective of the audio sample word length as indicated by any of the states of bits 3-5, the MSB is in time slot 27 of the transmitted subframe as specified in § 3.1.

| Byte 3 |  |
| --- | --- |
| Bits 0-7 | Vectored target byte from byte 1 |
|  | Reserved for future use as multi-channel function description Shall be set to logic “0” at present |

| Byte 4 |  |  |
| --- | --- | --- |
| Bits 0-1 | Digital audio reference signal (per AES11 – “Synchronization of digital audio equipment in studio operations”) |  |
| Bit State | 0 1 0 0 0 1 1 0 1 1 | Not a reference signal (default) Grade 1 reference signal Grade 2 reference signal Reserved and shall not be used until further defined |
| Bits 2-7 | Reserved and shall be set to logic “0” until further defined |  |

| Byte 5 |  |
| --- | --- |
| Bits 0-7 | Reserved and shall be set to logic “0” until further defined |

| Bytes 6-9 |  |
| --- | --- |
| Alphanumeric channel origin data. First character in message is byte 6 |  |
| Bits 0-7 (each byte) | 7-bit ISO 646 (ASCII) data with no parity bit LSBs are transmitted first with logic “0” in bit 7 Non-printed control characters (codes 01h to 1Fh and 7Fh) are not permitted Default value shall be logic “0” (code 00h; ASCII null) |

| Bytes 10-13 |  |
| --- | --- |
| Alphanumeric channel destination data. First character in message is byte 10 |  |
| Bits 0-7 (each byte) | 7-bit ISO 646 (ASCII) data with no parity bit LSBs are transmitted first with logic “0” in bit 7 Non-printed control characters (codes 01h to 1Fh and 7Fh) are not permitted Default value shall be logic “0” (code 00h; ASCII null) |

| NOTE 1 – This has the same function as a recording index counter. Bytes 14-17 |  |
| --- | --- |
| Local sample address code (32-bit binary) Value is of first sample of current block |  |
| Bits 0-7 (each byte) | LSBs are transmitted first Default value shall be logic “0” |

| NOTE 1 – This is the time-of-day laid down during the source encoding of the signal and shall remain unchanged during subsequent operations. A value of all zeros for the binary sample address code shall, for the purposes of transcoding to real time, or to time codes in particular, be taken as midnight (i.e. 00 h, 00 min, 00 s, 00 frame). Transcoding of the binary number to any conventional time code requires accurate sampling frequency information to provide the sample accurate time. Bytes 18-21 |  |
| --- | --- |
| Time-of-day sample address code (32-bit binary) Value is of first sample of current block |  |
| Bits 0-7 (each byte) | LSBs are transmitted first Default value shall be logic “0” |

| Byte 22 |  |
| --- | --- |
| Flag used to identify whether the information carried by the channel status data is reliable. According to the following table, if reliable the appropriate bits shall be set to logic “0” (default), if unreliable the bits shall be set to logic “1” |  |
| Bits 0-3 | Reserved and shall be set to logic “0” until further defined |
| Bit 4 | Bytes 10 to 15 |
| Bit 5 | Bytes 16 to 13 |
| Bit 6 | Bytes 14 to 17 |
| Bit 7 | Bytes 18 to 21 |

| NOTE 1 – Appendix 2 includes a diagram of the shift register circuit used to generate the code and two examples of channel status data, and the corresponding CRC byte. Byte 23 |  |
| --- | --- |
| Channel status data cyclic redundancy check character (CRCC) |  |
| Bits 0-7 | Generating polynomial is: G(x) = x8  x4  x3  x2  1 The CRCC conveys information to test valid reception of the entire channel status data block (bytes 0 to 22 inclusive) For serial implementations the initial condition of all ones should be used in generating the check bits with the LSB transmitted first Default value shall be logic “0” for minimum implementation of channel status only (see § 4.2.1) |

# 4	Interface format implementation

## 4.1	General

To promote compatible operation between items of equipment built to this specification it is necessary to establish which information bits and operational bits need to be encoded and sent by a transmitter and decoded by an interface receiver.

Documentation shall be provided describing the channel status features supported by the interface transmitters and receivers.

## 4.2	Transmitter

Transmitters shall follow all the formatting and channel coding rules established in earlier sections of this Annex. Along with the audio sample word, all transmitters shall correctly encode and transmit the validity bit, user bit, parity bit, and the three preambles. The channel status shall be encoded to one of the implementations given below.

The following three implementations are defined: “Minimum”, “Standard”, and “Enhanced”. These terms are used to communicate in a simple manner the level of implementation of the interface transmission involving the many features of channel status. Irrespective of the level of implementation the reserved states of bits defined in § 3 shall remain unchanged.

### 4.2.1	Minimum implementation of channel status

The Minimum implementation represents the lowest level of implementation of the interface that meets the requirements of this Annex. In the Minimum implementation, transmitters shall encode and transmit channel status Byte 0, Bit 0 with a state of logic “1” signifying “professional use of channel status block”. All other channel status bits of Bytes 0 to Byte 23 inclusive shall be transmitted with the default state of logic “0”. In this circumstance, the receiver will adopt the default conditions specified in Bytes 0 to 2.

If additional bytes of channel status (which do not fully comply with the standard implementation (see § 4.2.2)) are implemented as required by an application the interface transmitter shall be classified as a Minimum implementation of channel status.

It should be noted that this implementation imposes severe operational restrictions on the receiving devices which may be connected to it. For example, receivers implementing Byte 23 will show a CRC error when the default value of logic “0” is received as the cyclic redundancy check character (CRCC). Also reception of the default value for Byte 0, Bits 6-7 might cause improper operation in receiving devices not supporting manual override or auto set capabilities.

### 4.2.2	Standard implementation of channel status

The Standard implementation provides a fundamental level of implementation which should prove sufficient for general applications in professional audio or broadcasting. In addition to conforming to the requirements described above for the Minimum implementation, a Standard implementation interface transmitter shall correctly encode and transmit all channel status bits in Byte 0, Byte 1, Byte 2 and Byte 23 (CRCC) in the manner specified in this Annex.

### 4.2.3	Enhanced implementation of channel status

In addition to conforming to the requirements described above for the standard implementation, the Enhanced implementation shall provide further capabilities.

## 4.3	Receivers

Implementation in receivers is highly dependent on the application. Proper documentation shall be provided on the level of implementation of the interface receiver for decoding the transmitted information (Validity, User, Channel Status, Parity), and on whatever subsequent action is taken by the equipment of which it is a part.

# 5	Electrical requirements

## 5.1	General characteristic

The electrical parameters of the interface are based on those defined in ITU-T Recommendation V.11 for balanced voltage digital circuits which allow signal transmission up to a few hundred metres in length.

In order to improve the balance of the transmitter or the receiver or both beyond that recommended by the ITU-T, a circuit conforming to the general configuration shown in Fig. 6 shall be used.

In this circuit the series capacitors C2 and C3 isolate the transformers and prevent damage from connection to a source containing a d.c. voltage. In addition to achieving higher rejection of common mode signals the transformers reduce grounding and electromagnetic interference (EMI) problems. Although equalization may be employed at the receiver, no equalization prior to transmission is permitted.

The interconnecting cable shall be balanced and screened (shielded) with nominal characteristic impedance of 110  at frequencies from 0.1 to 6.0 MHz.

## 5.2	Line driver characteristics

### 5.2.1	Output impedance

The line driver shall have a balanced output with an internal impedance of 110   20%, at frequencies from 0.1 to 6.0 MHz when measured at the output terminals.

### 5.2.2	Signal amplitude

The signal amplitude shall lie between 2 and 7 V peak-to-peak, when measured across a 110  resistor connected to the output terminals, without any interconnecting cable present.

### 5.2.3	Balance

Any common mode component at the output of the equipment shall be more than 30 dB below the signal at frequencies from d.c. to 6 MHz.

### 5.2.4	Rise and fall times

The rise and fall times, determined between the 10% and 90% amplitude points, shall be between 5 ns and 30 ns when measured across a 110  resistor connected to the output terminals, without any interconnecting cable present.

NOTE 1 – Operation towards the lower limit of 5 ns will improve the received eye pattern but will increase EMI at the transmitter. Equipment must meet local regulations regarding EMI.

### 5.2.5	Data jitter

Data transitions shall occur within  20 ns of an ideal jitter free clock measured at the half-voltage points.

NOTE 1 – This specification applies only to the signal after channel coding. Tighter specifications apply to the audio sample clock.

## 5.3	Line receiver characteristics

### 5.3.1	Terminating impedance

The receiver shall present a substantially resistive impedance of 110   20% to the interconnecting cable over the frequency band 0.1 to 6.0 MHz, when measured across the input terminals. The application of more than one receiver to any one line might create transmission errors due to the resulting impedance mismatch.

### 5.3.2	Maximum input signals

The receiver shall correctly interpret the data when connected directly to a line driver working between the extreme voltage limits specified by § 5.2.2.

NOTE 1 – In EBU Document Tech. 3250 (1985) the specification for line driver signal amplitude was 10 V peak-to-peak maximum.

### 5.3.3	Minimum input signals

The receiver shall correctly sense the data when a random input signal produces the eye diagram characterized by a Vmin of 200 mV and Tmin of 50% of Tnom (see Fig. 7).

### 5.3.4	Receiver equalization

Optional equalization can be applied in the receiver to enable interconnecting cable longer than 100 m to be used. A suggested frequency equalizing characteristic is shown in Fig. 8. The receiver shall meet the requirements specified in § 5.3.2 and 5.3.3.

### 5.3.5	Common mode rejection

There shall be no data errors introduced by the presence of a common mode signal of up to 7 V peak at frequencies from d.c. to 20 kHz.

## 5.4	Connectors

The standard connector for both outputs and inputs shall be the circular latching three-pin connector described in IEC 268-12 (this type of connector is normally called “XLR”).

An output connector fixed on an item of equipment shall use male pins with a female shell. The corresponding cable connector shall thus have female pins with a male shell.

An input connector fixed on an item of equipment shall use female pins with a male shell and the corresponding cable connector shall thus have male pins with a female shell. The pin usage shall be:

–	pin 1:  cable shield or signal earth;

–	pin 2:  signal;

–	pin 3:  signal.

(It should be noted that the relative polarity of pins 2 and 3 is not important in the digital case.)

Equipment manufacturers should clearly label digital audio inputs and outputs as such, including the terms “digital audio input” or “digital audio output” as appropriate.

In such cases where panel space is limited and the function of the connector might be confused with an analogue signal connector, the abbreviations “DI” and “DO” should be used to designate digital audio inputs and outputs, respectively.

Appendix 1

The provision of additional, voice-quality channels via
the digital audio interface

When a 20-bit coding range is sufficient for the audio signal, the 4 auxiliary sample bits can be used for a voice-quality coordination signal (talk back).

The voice-quality signal is sampled at exactly one-third of the sampling frequency for the main audio, coded uniformly with 12 bits/sample represented in two’s complement form. It is sent 4 bits at a time in the auxiliary sample bits of the interface subframes. One such signal may be sent in subframe 1 and another in subframe 2. The “Z” preamble at the start of each block is used as a frame alignment word for the voice-quality signals. The two subframes of frame 0 each contain the 4 LSBs of a sample of their respective voice-quality signal as shown in Fig. 9. Fig. 9 also shows two voice-quality signals, one in each subframe.

Appendix 2

Generation of cyclic redundancy check code (Byte 23) for channel status

The channel status block format of 192 bits includes a CRC (cyclic redundancy check) code occupying the last 8 bits of the block (Byte 23). The specification for the code is given by the generating polynomial:

G(x)    x8    x4    x3    x2    1

An example of a hardware realization in the serial form is given in Fig. 10. The initial condition of all the stages is logic “1”.

Examples of channel status data and the resultant CRCC

Example 1:

| Byte | Bits set to “1” |
| --- | --- |
| 0 | 0, 2, 3, 4, 5 |
| 1 | 1 |
| 4 | 1 |

All other bits in channel status Bytes 0 to 22 inclusive are set to “0”.

| CRCC | Byte 23 |  |  | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel status bit: 184 |  |  |  |  |  |  |  |  |  |  | 191 |

Example 2:13

| Byte | Bits set to “1” |
| --- | --- |
| 0 | 0 |

All other bits in channel status Bytes 0 to 22 inclusive are set to “0”.

| CRCC | Byte 23 |  |  | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel status bit: 184 |  |  |  |  |  |  |  |  |  |  | 191 |

No particular level of implementation should be taken as implied by the examples given above.

Annex 2

Evolution of the digital audio interface

# 1	Introduction

Changes may be proposed in order to improve the facilities offered by the interface, or to enable it to meet the requirements of particular applications. It is important that the advantages and disadvantages of making such changes be thoroughly examined, not only by those proposing the changes, but also by other users of the interface.

This Annex presents a number of amendments and ideas which have been proposed but not necessarily adopted. The publication of this information is intended to fulfil a number of purposes:

–	to draw the attention of the user to possible future problems;

–	to draw the attention of the user to possible new requirements and facilities;

–	to help the user make effective use of the interface in present and future applications;

–	to alert the makers of interface equipment to possible areas of change;

–	to stimulate further study.

# 2	Alternative time codes

The original specification of the digital audio interface stipulates the use of “sample address codes”; these are binary counts of samples (i.e. at the audio sampling frequency). This is an ideal form of time code for many purely audio applications in the digital environment, and disk-based audio editing equipment using sample-address time codes has been developed by a number of organizations.

However, in some applications (e.g. television and recording) there may be the need to interconnect two items of apparatus which both use a BCD (binary coded decimal) time code (sometimes termed “SMPTE/EBU” time-code). The items of apparatus could be compatible in respect of the time code, but unable to exchange time-code information via the digital audio interface without transcoding to and from sample-address form.

Possible solutions to this problem are described below.

## 2.1	Replacement of sample address by BCD time code

The sample address time codes may be replaced directly by BCD time codes, using the same number of bytes in the same positions in the channel status words of the interface (4 bytes per time code). To enable the interface to carry either type of time code, it is advisable to use one of the bits in channel status to “flag” the substitution of BCD time code for the sample address code.

## 2.2	Use of a time code incorporating elements of both BCD and sample address codes in channel status

It is possible to devise a time code which combines elements of both types of time code. For example, the most significant part of the time code could be a BCD representation of hours, minutes and seconds, and time within the seconds could be indicated by a binary count of samples. Another possibility would be for television fields and interface blocks to be indicated in the BCD count, with the binary count of samples operating within the interface block count.

Thus, it would be possible to provide a time code satisfying the requirements of apparatus using BCD time code, incorporating the resolution of the sample-address code. However, the disadvantages with this proposal are that more bytes would be needed in channel status for this type

of code, necessitating a re-organization of channel status, and the requirement for equipment using sample-address code to encode and decode the BCD part of the time code at the interfaces.

## 2.3	The provision of additional time codes in the user channel

The user data channel of the interface could be used to carry BCD time codes, in addition to the sample address codes in channel status. The user data channel is likely to be too valuable to dedicate solely to the transport of time codes, but an HDLC (high level data link control) system for carrying time codes and other data in the user channel has been developed and this may be suitable as a possible standard for carrying a wide range of information in the user data channel.

# 3	Copying

The International Electrotechnical Commission (IEC) has proposed a copyright protection scheme and the means of indicating whether a recording is the original or a copy, in the consumer interface. Consideration should be given to the possibility of incorporating such information in the channel status of the professional interface.

*	 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2003 in accordance with Resolution ITU-R 44.

*	*	This Recommendation should be brought to the attention of the IEC and the Audio Engineering Society (AES).