RECOMMENDATION  ITU-R  BS.706-2*

Data system in monophonic AM sound broadcasting (AMDS)

(l990-1992-1998)

The ITU Radiocommunication Assembly,

considering

a)	the growing interest for a data transmission system for AM broadcasting and its applications;

b)	that it is desirable to have one system applicable to all AM broadcasting bands and that the future introduction of SSB in HF and of synchronous detection should be taken into account;

c)	that existing systems cannot be implemented on single-sideband (SSB) transmission in band 7 (HF);

d)	that certain applications of an AM data transmission system could correspond to similar features in the FM radio data system (RDS), as defined in Recommendation ITU-R BS.643, taking account of the lower bit rate available;

e)	that the design of such a system should take into account the mass production of receivers;

f)	that data signals can be added to existing AM broadcast transmissions in such a way that they are inaudible, thus achieving good compatibility with reception of the normal monophonic sound programme signals,

recommends

1	that a system for data transmission in AM broadcasting (AMDS) should fulfil the requirements listed in Annex 1;

2	that since a system is not available for SSB in band 7 (HF) and for AM-Stereo in band 6 (MF) a system for data transmission in AM-DSB broadcasting bands 5 (LF), 6 (MF) and 7 (HF) shall fulfil the requirements listed in Annex 1 and shall comply with the minimum specification listed in Annex 2.

NOTE 1 – Information regarding operational or projected systems is given in Annex 3.

ANNEX  1

Requirements for an AM radio-data transmission system

# 1	Compatibility aspects

## 1.1	Compatibility with the main programme

The supplementary data system must be compatible with the main audio programme under all operational conditions including:

–	transmitters operated with energy-saving carrier-control techniques;

–	synchronized networks of transmitters;

–	SSB transmissions (if the introduction of an AM data system in HF broadcasting is feasible);

–	transmitters which are used as a high stability frequency reference;

–	mobile reception and, where necessary, reception with a stereophonic AM receiver in band 6 (MF).

## 1.2	Compatibility with other programmes in co- or adjacent channels

The protection ratios used in planning should not be affected, i.e. no additional interference should be caused to the audio programme signal by the data signals.

# 2	Reliability of data reception

The area in which the data signal can be reliably received, should be at least as large as that where the main programme service for ground- and sky-wave propagation conditions is provided.

# 3	Applications

Because of the low data-rate which is expected to be available in an AM radio-data system, it may not be feasible to support simultaneously more than a few of the applications listed below.

It is expected that a large part of the data-transmission capacity will usually be used for features related to automatic or assisted tuning functions. These features are therefore labelled “primary”. Other applications are labelled “secondary” and may be introduced to meet the needs of individual broadcasters. Note that although similar terms are used, these features may not correspond exactly with those used in RDS (see Recommendation ITU-R BS.643).

Primary

–	Programme Identification (PI) code including:

–	unique country code for each ITU country;

–	unique language code.

–	List of Alternative Frequencies (AFs).

–	Programme Service (PS) name: this comprises at least four alpha-numeric characters and is intended for display.

–	Scheduling information (SI).

–	Traffic Programme (TP) identification and Traffic Announcement (TA) identification.

Secondary (examples)

–	Differential GPS data (dGPS)

–	Clock-Time (CT) and date (UTC and MJD)

–	Programme Item Number (PIN)

–	Decoder Identification code (DI) (e.g. stereo)

–	Programme Type code (PTY)

–	Transparent Data Channel (TDC)

–	In-House (IH) applications

–	Traffic Message Channel (TMC)

–	Radio Paging (RP).

ANNEX  2

Specification of a data system for use in monophonic AM sound-broadcasting

Frequency bands:		LF, MF and HF

Method of modulation: 	phase modulation of the main carrier

Maximum phase deviation: 	depending on bit rate according to Fig. 1

Data format for bit rates below 100 bit/s: 	depending on application

Data format for bit rates  100 bit/s:	according to Annex 4

ANNEX  3

Information on operational and projected AM data systems (AMDS)

Table 1 gives information on AMDS systems as applied in different countries.

TABLE  1

Application of AMDS systems

| Characteristics | United Kingdom | France | Germany |
| --- | --- | --- | --- |
| – Operating frequency band – Possible frequency bands – Phase deviation – Bit rate – Data coding – Data structure – Achieved bit error ratio (normal conditions) – Application In operation since Reference | LF LF, MF  22.5º 25 bit/s Bi-phase Blocks of 50 bits with 32 useful Coded weather information Electricity supply switching 1985 | LF LF, MF, HF  28.5º 40 bit/s NRZ Blocks of 50 bits with 32 useful Time information 1986 French national standard NFC 90-002 | LF, MF, HF LF, MF, HF  15º 200 bit/s NRZ Blocks of 47 bits with 36 useful < 10–3 for LF and MF (50% time) Automatic or assisted tuning Traffic information Field tests since 1988 |

ANNEX  4

1	Baseband coding

1.1	Structure of baseband coding

1.2	Features of data transmission

1.3	Error protection

1.4	Block and Group synchronization

2	Data format

2.1	Definitions relating to the data transmission

2.2	Programme identification

2.2.1	PI code

2.2.2	BI code

3	Group types

3.1	Usage of Groups

3.2	Group sequences

4	Description of Groups

4.1	Group type 0/Basic tuning and switching information BTI

4.2	Group type 1/Radiotext RT

4.3	Group type 2/Alternative frequencies AF

4.4	Group type 3/Traffic message channel TMC

4.5	Group type 4/In-house applications IH

4.6	Group type 5/Transparent Data Channel TDC

4.7	Group type 6/Scheduling information SI

4.7.1	Coding of START and END

4.8	Group type 7/Scheduling information supplementary SIS

4.8.1	Information addressed by the usage code UCI

4.8.2	Coding of latitude and longitude

4.9	Group type 8/Additional tuning information ATI

4.9.1	Information addressed by the usage code UC2

4.10	Group type 9/differential GPS-data dGPS

4.11	Group type 10/TIME information UTC

5	Glossary of terms

6	Index of Tables

7	Index of Figures

8	Index of Formulas

# 1	Baseband coding

## 1.1	Structure of baseband coding

Figure 3 shows the structure of the baseband coding. The largest element within the structure is called a Group. One Group consists of 2 Blocks with 47 bits each. Each Block contains one information word (36 bits) and one check word (11 bits). To distinguish the Blocks of a Group and to improve the Block synchronization, each Block is superimposed by an offset word (11 bits, modulo-2 addition).

Group = 94 bits

| Block 1 | Block 2 |
| --- | --- |

Block = 47 bits

| Information word | Check word |
| --- | --- |

Information word = 36 bits

| m35 | m34 | … | m01 | m00 |
| --- | --- | --- | --- | --- |

Check word (+ offset word) = 11 bits

| FIGURE 3 c10 | c09 | … | c01 | c00 |
| --- | --- | --- | --- | --- |

Baseband coding structure

## 1.2	Features of data transmission

For all information words, check words, binary numbers or addresses, the most significant bit will be transmitted first (Fig. 4). The order assigned to the binary number or address bit transmitted last is 20.

The transmission of data is fully synchronized, i.e. there are no gaps between Groups and Blocks.

Group

| Block 1 will be transmitted before Block 2 | Block 2 |
| --- | --- |

| GT | PI or BIMSB code | … | Check word | GT | … | Check word |
| --- | --- | --- | --- | --- | --- | --- |

| Group type code 4 Bits | 16 Bits | Infor- mation 16 Bits | + Offset word A 11 Bits | Group type code 4 Bits | Information 32 Bits | + Offset word B 11 Bits |
| --- | --- | --- | --- | --- | --- | --- |

Group type code (GT)

| a3 | a2 | a1 | a0 |
| --- | --- | --- | --- |

| FIGURE 4 most significant bit will be transmitted first |  |  | least significant bit |
| --- | --- | --- | --- |

Data format and addressing

TABLE  2

Data elements

| Data element | Note | Chapter |
| --- | --- | --- |
| Check word | Error detection, error correction, Block and Group synchronization | 2.1 - 2.4 |
| PI code | Programme identification code | 3.2.1 |
| BI code | Broadcast identification code | 3.2.2 |
| GT | Group type | 4 |
| Information | is defined by the Group type code | 5.1 - 5.11 |
| X | unused capacity | 5.1 - 5.11 |

## 1.3	Error protection

In order to enable the receiver/decoder to detect and correct transmission errors, each Block is assigned a check word (11 bits). This check word

c(x) (c10, c09, ... , c00 shown in Fig. 3) is the sum total (modulo 2) of:

–	the remainder obtained after multiplication of the 36-bit information word m(x) by x11 and a following division (modulo 2) by the generator polynomial g(x), and

–	an 11-bit binary sequence d(x), called offset word such that:

c(x)=d(x)+m(x)⋅x11g(x)     |mod  g(x)	(1)

whereby the generator polynomial (degree 11) may be described by the following formula:

g(x)  x11  x8  x6  1	(2)

Different offset words A and B are used for each Block of a Group.

The 11-bit binary sequence for the offset words A and B are shown in the Table below:

TABLE  3

Offset words

| Offset | Offset word d(x) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | d10 | d9 | d8 | d7 | d6 | d5 | d4 | d3 | d2 | d1 | d0 |
| A | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| B | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |

The error protection code offers the following features:

–	detects all single and double errors in a Block;

–	detects any single burst spanning 10 bits or less;

–	detects about 99.90% of bursts spanning 11 bits; and

–	about 99.95% of all longer bursts.

The code is an optimal burst error correcting code and is capable of correcting any single burst of span 5 bits or less. Depending on the number of errors within a Block or within the content of a Block the receiver/decoder may utilize either the error detection mode or the error correction mode or both of them.

The probability of unrecognized errors depends on the number of errors which are corrected. Field trials have shown that no more than 2 errors in a Block should be corrected.

## 1.4	Block and Group synchronization

The beginning and end of a data Block as well as the beginning and end of a Group may be detected in the receiver/decoder by using the two offset words A and B. These offset words destroy the cyclic property of the basic code so that in the modified code, cyclic shifts of code words do not give rise to other code words. By means of this method the Block synchronization becomes reliable.

# 2	Data format

## 2.1	Definitions relating to the data transmission

There is no specific repetition cycle fixed for the various types of Groups (see § 4), i.e. a large degree of flexibility is provided. This allows the user to create combinations of different types of information, which comply with his requirements.

The selected data format comprises 5 Group types which are still unused and therefore provides a sufficient degree of adaptability for future applications.

In order to ensure efficient utilization of the AM data channel, the number of different types of information is minimized within a given Group. Thereby the user will not be forced to waste data capacity by the transmission of unused information.

The first 4 bits of each Block are assigned to the Group type code (Fig. 5) specifying the application of the Group. By this process each Block of a Group can be decoded within the receiver/decoder without reference to the other Block. Thus the access time for data which are transmitted in several Blocks is reduced during interference.

## 2.2	Programme identification

The programme identification is a code which enables the receiver to distinguish between country of origin, programme area, language and the identification of the programme itself.

The code is not intended for direct display. It is assigned to each individual broadcast programme to identify transmitters radiating the same sound programme. By this code the mobile receiver/decoder is enabled to search automatically for an alternative frequency in case of bad reception of the just received frequency. In cases of stationary reception and especially in the HF bands the listener may start search tuning for transmissions originated in a specific country with a desired language.

Except for Group type 5 (Transparent Data Channel) the programme identification is always included in the first Block of each Group (see Fig. 5). Depending on the service two different codes for the programme identification are available. These are the BI code and the PI code.

### 2.2.1	PI code

The usage of the PI code is equivalent to RDS. For reasons of compatibility with RDS, transmissions that are implemented on LF and/or MF and also on VHF/FM Band II, have to be identified with the PI code. This is necessary for automatic search tuning on all bands and moreover to support cross-border functions especially for the TMC service (Traffic Message Channel).

TABLE  4

PI structure

| Bits | Identification |
| --- | --- |
| 1 to 4 | Country identification |
| 5 to 8 | Programme type in terms of area coverage |
| 9 to 16 | Programme reference number |

Extended country codes (ECC) may optionally be transmitted to render the country identification in bits 1 to 4 of the PI code unique. The extended country code consists of eight bits and is coded according to RDS-Standards.

### 2.2.2	BI code

The BI code (Broadcast identification) should be used to facilitate maximum flexibility with respect to tuning and searching for a particular HF transmission. An eight-bit extension (bits 17 to 24) allows identification of up to 32 broadcasters (organization number has to be agreed nationally) and up to 8 different simultaneous programmes from a single broadcaster (programme marker).

TABLE  5

BI structure

| Bits | Identification |
| --- | --- |
|  | BIMSB |
| 1 to 8 | Country identification (255 countries can be coded)(1) |
| 9 to 16 | Language code (255 languages can be coded) |
|  | BILSB |
| 17 to 21 | Organization number (32 organizations within a country) |
| 22 to 24 | Programme marker (8 programmes) |
| (1) These country codes are given in Appendix A. |  |

NOTES: Use of PI vs. BI code:

a)	For reasons of compatibility with RDS, those programmes that are implemented on LF and/or MF and VHF/FM Band II, have to be identified with PI-code. The use of the ECC code will be optional.

b)	All other applications can use BI code instead of the PI code.

c)	For HF, the BI code should be used to facilitate maximum flexibility with respect to tuning and searching for a particular transmission.

d)	The usage of BI or PI code is signalled by the codeflag CF.

e)	A short designation of the respective country could be derived from the Country identification or ECC to be shown on the receiver display. 2-Letter ISO 3166 codes are recommended.

# 3	Group types

The application of a Group is determined by the Group type code, i.e. the first 4 bits in each Block (Fig. 5). The assigned Group types for the different applications are listed in Table 6.

TABLE  6

Group types

| Group type |  |  |  |  |  | Type of |
| --- | --- | --- | --- | --- | --- | --- |
| Dec | Binary Code |  |  |  | Application | information |
|  | A3 | A2 | A1 | A0 |  | (1) |
| 0 | 0 | 0 | 0 | 0 | Basic tuning and switching information BTI | interactive |
| 1 | 0 | 0 | 0 | 1 | Radiotext RT | interactive |
| 2 | 0 | 0 | 1 | 0 | Alternative frequencies AF | interactive |
| 3 | 0 | 0 | 1 | 1 | Traffic message channel TMC | interactive |
| 4 | 0 | 1 | 0 | 0 | In-House applications IH | various |
| 5 | 0 | 1 | 0 | 1 | Transparent data channel TDC | various |
| 6 | 0 | 1 | 1 | 0 | Scheduling information SI | off-line |
| 7 | 0 | 1 | 1 | 1 | Supplementary scheduling information SIS | off-line |
| 8 | 1 | 0 | 0 | 0 | Additional tuning information ATI | interactive |
| 9 | 1 | 0 | 0 | 1 | Differential Global Positioning System data dGPS | interactive |
| 10 | 1 | 0 | 1 | 0 | Time information UTC | interactive |
| 11 | 1 | 0 | 1 | 1 | Undefined |  |
| 12 | 1 | 1 | 0 | 0 | Undefined |  |
| 13 | 1 | 1 | 0 | 1 | Undefined |  |
| 14 | 1 | 1 | 1 | 0 | Undefined |  |
| 15 | 1 | 1 | 1 | 1 | Undefined |  |
| (1) The reception of a Group can have different effects on the actions subsequently taken: interactive  The information is treated directly by the receiving system and results in either tuning or change of settings, display or storage. Off-line  The information is stored for further reference and will be basically used to update a database in the receiving system. Various  The actual effect on or at the receiving system depends on the application and the type of data transmitted. |  |  |  |  |  |  |

## 3.1	Usage of Groups

TABLE  7

Usage of Groups for different applications

| Application | Primary Groups | Secondary Groups |
| --- | --- | --- |
| LF-MF station identification | 0,2 | 1,4,8,10 |
| HF station identification | 8,2 | 1,4,10 |
| LF-MF TMC | 0,3,2 | 1,4,8,10 |
| LF-MF-dGPS | 0,9,2,5 | 1,4,8,10 |
| LF-MF-scheduling | 0,2,6,7 | 1,4,8,10 |
| HF-scheduling | 8,2,6,7 | 1,4,10 |

## 3.2	Group sequences

Unlike RDS applications, a fixed Group sequence cannot be given for AMDS. The sequence depends on the amount of data that is to be transmitted and the envisaged reaction time at the receivers.

If for example, it is necessary to have a PS-name to appear on the receivers display within 3 seconds, Group 0 is transmitted at least every 6th Group (error-free conditions). In a heavily distorted environment as in HF, it is advisable to send every 3rd Group in the sequence a Group 8.

Basis for the calculation of Group sequences is the Group duration of TG  0.47 second (s) under error-free conditions. The Group sequence can then be derived from the formula:

Noc=TRTG	(3)

where:

NOC	is the repetition rate of the particular Group within a sequence

TR		is the wanted reaction time at the receiving terminal in (s)

TG		is the Group duration in (s)

In HF-applications it has been found that a dynamic Group sequence is appropriate, for example:

Transmission is due to start at 1200 UTC.

TABLE  8

Dynamic Group sequences

| Time (UTC) | Group sequence | Task |
| --- | --- | --- |
| 1159-1200 | 8,2,8,2 | Station name and alternative frequencies |
| 1200-1201 | 8,2,1,8,2,1 | Station name, alternative frequencies and radiotext |
| 1201-1204 | 8,2,6,7,8,6,7 | Station name and scheduling information |
| 1204-1205 | 8,2,8,2 | Station name and alternative frequencies |

# 4	Description of Groups

## 4.1	Group type 0/Basic tuning and switching information BTI

Usage interactive

The Group type 0 contains the programme service name (PS), identification signals for traffic transmitters (TMC, TP, TA) and an identification for the audio bandwidth of the sound programme signal (BW). With two flags (PIX and PSX) an extended PI and/or extended PS service can be marked.

Group 0/Block 1

| GT | PI | PIX | PSX | PS Character 1 | PS Character 2 |
| --- | --- | --- | --- | --- | --- |

| 4 | 16 | 1 | 1 | 7 | 7 |
| --- | --- | --- | --- | --- | --- |

Group 0/Block 2

| GT | TA | TP | TMCF | BW | PS Character 3 | PS Character 4 | PS Character 5 | PS Character 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| FIGURE 5 4 | 1 | 1 | 1 | 1 | 7 | 7 | 7 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Group 0

TABLE  9

Data in Group 0

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI | 0000 0001 ... 1111 1111 | PI code |
| TMCF | 0 1 | Programme with coded traffic messages |
| TP | 0 1 | Programme with spoken traffic messages |
| TA | 0 1 | There is no voiced traffic message (off) There is a voiced traffic message (on) |
| BW | 0 1 | AF bandwidth of the sound programme signal: 4.5 kHz Ext. AF bandwidth of the sound programme signal: 7 kHz |
| PIX | 0 1 | No ECC will be transmitted ECC is transmitted in Group 8 |
| PSX | 0 1 | PS using 6 characters PS using 8 characters (see also Group 8) |
| PS |  | Programme service name characters 1 ... 6 |

PS

According to the code tables of ISO 646, the programme service name is transmitted in 7-bit characters to be indicated on the receiver display. For each name basically 6 characters including spaces are allowed. The position of the individual characters in the name corresponds to the assigned number. The transmission of the name starts with character 1. The bit transmitted first is the most significant bit of a character.

PSX

Two additional PS characters can be transmitted in Group 8. The number of characters of PS is marked by the PS extension flag (PSX). Characters 1 to 6 are transmitted in Group 0 and the characters 7 and 8 in Group 8.

PIX

With the PIX flag the transmission of the extended country code for PI is marked.

TMCF

Traffic Message Channel identification represents a switching signal to identify channels, which at certain times, transmit coded traffic messages. This identification could be used for an automatic TMC transmitter search tuning.

TP

Traffic Programme identification represents a switching signal to identify channels which, at certain times, transmit spoken traffic messages. An indicator lamp or similar device could be used in the receiver to inform the motorist that the channel received, at certain times, will transmit traffic messages. This identification could be used for an automatic search tuning of a traffic channel.

TA

The Traffic Announcement identification represents a switching signal to identify traffic announcements currently being transmitted. The signal could be used in the receiver as follows:

–	automatic activation in the case of traffic announcements, when the receiver is in ready-to-receive mode and has been muted;

–	automatic switch-over from tape to traffic announcement.

BW

Bandwidth identification represents a switching signal to identify the audio bandwidth of the sound programme signal. This signal identifies LF and MF transmitters which operate with an extended bandwidth during the day. This identification could be used for adapting the IF selection filter to the actual audio bandwidth of the sound programme signal.

## 4.2	Group type 1/Radiotext RT

Usage interactive

Group type 1 is used to transmit radiotext (RT).

Group 1/Block 1

| GT | PI/ BIMSB | TE | TN | TF | TSA a3..a0 | RT character 1 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 16 | 1 | 2 | 1 | 4 | 8 |

Group 1/Block 2

| FIGURE 6 GT | RT character 2 | RT character 3 | RT character 4 | RT character 5 |
| --- | --- | --- | --- | --- |
| 4 | 8 | 8 | 8 | 8 |

Group 1

TABLE  10

Data in Group 1

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB | 0000 0001 ... 1111 1111 | PI code or BIMSB code |
| TE | 0 1 | Last text-segment of the text signalled by TN is being transmitted |
| TN | 00 .. 11 | Number of text being transmitted |
| TF | 0 1 | Text signalled by TN has changed |
| TSA | 0000 1111 | Text segment address of the text signalled by TN |
| RT |  | Radio text characters 1 ... 5 of segment TSA |

According to the code tables of ISO 646, radio text is transmitted using 8-bit characters. The transmission starts with the most significant bit of a character.

TSA

The text segment address in Block 1 is used to position the text segment consisting of the characters in Blocks 1 and 2 (characters 1 to 5) in the display or in the memory. Determined by the range of the segment addresses (0-15) and the number of characters (5) contained in a Group, it is possible to transmit radio texts up to a maximum length of 80 characters.

If the display comprises less than 80 characters the receiver/decoder should be equipped with a memory in order to display parts of the RT successively.

TN

Addresses a text 0 ... 3 that will be affected by the data transmitted.

TF

Changing the text flag (TF) in Block 1 will identify a new radio text for the signalled TN. The text flag is used by the receiver/decoder to clear the display memory.

TE

Indicates that the last segment of a text addressed by TN is being transmitted.

RT is considered to be especially useful for home receivers equipped with an appropriate display and for car receivers equipped with a speech generator.

## 4.3	Group type 2/Alternative frequencies AF

Usage interactive

Group type 2 is used to transmit alternative frequencies (AF).

Group 2/Block 1

| GT | PI/ BIMSB | AF Code 1 | AF Code 2 |
| --- | --- | --- | --- |

| 4 | 16 | 8 | 8 |
| --- | --- | --- | --- |

Group 2/Block 2

| GT | AF Code 3 | AF Code 4 | AF Code 5 | AF Code 6 |
| --- | --- | --- | --- | --- |

| FIGURE 7 4 | 8 | 8 | 8 | 8 |
| --- | --- | --- | --- | --- |

Group 2

TABLE  11

Data in Group 2

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB | 0000 0001 ... 1111 1111 | PI code or BIMSB code |
| AF | 0000 0001 ... 1111 1111 | Alternative frequency code 1 ... 6 |

Codes for alternative frequencies

Six AF codes or up to 6 alternative frequencies can be transmitted within a Group. All specified AF codes and all currently specified frequency codes are indicated in Table 8.

LF band

LF frequencies (153-279 kHz) are represented by an 8-bit code with a channel spacing of 9 kHz (ITU Regions 1 and 3).

CodeLF  1  (fLF – 153)/9	(4)

fLF  153  (CodeLF – 1) . 9	(5)

MF band

MF frequencies (531-1 602 kHz) are represented by an 8-bit code with a channel spacing of 9 kHz (ITU Regions 1 and 3).

CodeMF  16  (fMF – 531)/9	(6)

fMF  531  (CodeMF – 16) . 9	(7)

HF band

HF frequencies (2 300-26 100 kHz) are represented by two 8-bit codes with a channel spacing of 5 kHz. They are characterized by the fact that the first 8-bit code is within the range of 141-159. In this case, the first code must be evaluated together with the subsequent code. Block-exceeding pairings are not permitted.

CodeHF  35 674  fHF/5

1st CodeHF  INT(CodeHF/256)	(8)

2nd CodeHF  CodeHF MOD 256

fHF  ((1st CodeHF – 139) . 256  2nd CodeHF – 90)) . 5	(9)

NOTE – fLF , fMF and fHF are given in kHz.

0-2 295 kHz band with a channel spacing of 5 kHz

This frequency band is an extension of the HF band down to lower frequencies. It provides for the transmission of alternative frequencies up to 2 295 kHz for radio services or MF transmitters in ITU Region 2 with 10 kHz channel spacing.

Code and frequency equations are as for HF.

VHF band

The VHF channels (87.5-107.9 MHz) are represented by two 8-bit codes with a channel spacing of 100 kHz. They are characterized by the fact that the first 8-bit code is 160. In this case, the first code and the subsequent code identify an VHF channel. Block-exceeding pairings are not permitted.

1st CodeVHF  160
		2nd Code  (fVHF – 87.5) . 10	(10)

fVHF  87.5  2nd CodeVHF/10	(11)

Filler code

The filler code (code  136) is used to substitute AF codes not being used within the Group.

Number code

The number code indicates how many frequencies, except for filler codes, are included in the AF list. AF lists may include up to 31 frequencies. The number code is transmitted at the beginning of the AF list.

Code  Number  224	(12)

Number  Code – 224	(13)

AF list

The AF list identifies transmitters which broadcast an identical sound programme in the same or in an adjacent receiving area. Receiver/decoder equipped with a memory for AF can reduce the time for switching over to another transmitter.

NOTE – fVHF is given in MHz.

TABLE  12

Coding of alternative frequencies

| Frequencies Remarks | 1st Code Binary | 2nd Code Binary | Decimal equivalent |
| --- | --- | --- | --- |
| free | 0000 0000 |  | 0 |
| LF band |  |  |  |
| 153 kHz | 0000 0001 |  | 1 |
| … | … |  |  |
| 279 kHz | 0000 1111 |  | 15 |
| MF band |  |  |  |
| 531 kHz | 0001 0000 |  | 16 |
| … | … |  |  |
| 1 602 kHz | 1000 0111 |  | 135 |
| Filler code | 1000 1000 |  | 136 |
| free | 1000 1001 1000 1010 |  | 137 138 |
| 0-2 295 kHz | 5 kHz spacing |  |  |
| 0 kHz | 1000 1011 | 0101 1010 | 35674 |
| … | … | … | … |
| 2 295 kHz | 1000 1101 | 0010 0101 | 36133 |
| HF band |  |  |  |
| 2 300 kHz | 1000 1101 | 0010 0110 | 36134 |
| … | … | … | … |
| 26 100 kHz | 1001 1111 | 1011 1110 | 40894 |
| VHF band |  |  |  |
| 87.5 MHz | 1010 0000 | 0000 0000 | 40960 |
| … |  | … | … |
| 107.9 MHz | 1010 0000 | 1100 1100 | 41164 |
| free | 1010 0001 … 1101 1111 |  | 205 . 223 |
| Number Codes |  |  |  |
| No AF exists | 1110 0000 |  | 224 |
| 1 AF | 1110 0001 |  | 225 |
| … | … |  |  |
| 31 AF | 1111 1111 |  | 255 |

## 4.4	Group type 3/Traffic message channel TMC

Usage interactive

Group type 3 is used to transmit coded traffic messages (TMC).

Group 3/Block 1

| GT | PI | AFT | X | TMC |
| --- | --- | --- | --- | --- |

| 4 | 16 | 8 | 3 | 5 |
| --- | --- | --- | --- | --- |

Group 3/Block 2

| GT | TMC |
| --- | --- |

| FIGURE 8 4 | 32 |
| --- | --- |

Group 3

The data format of AMDS TMC is identical with RDS TMC (information length  37 bits).

TABLE  13

Data in Group 3

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI | 0000 0001 ... 1111 1111 | PI code |
| AFT | 0000 0001 ... 1000 0111 | Alternative frequency coded as Group 2, limited to LF-MF |
| TMC | ... | TMC data |
| X |  | Unused capacity |

The alternative frequencies for TMC (AFT) only refer to channels transmitting identical traffic messages. These channels do not necessarily transmit the same sound programme. The AFT are based on the same coding as used for alternative frequencies AF. The use of AFT is restricted to LF and MF bands.

## 4.5	Group type 4/In-house applications IH

Usage various

Group type 4 is used to transmit coded in-house applications (IH).

Group 4/Block 1

| GT | PI/ BIMSB | IH |
| --- | --- | --- |

| 4 | 16 | 16 |
| --- | --- | --- |

Group 4/Block 2

| GT | IH |
| --- | --- |

| FIGURE 9 4 | 32 |
| --- | --- |

Group 4

There are 48 bits available for IH: 16 bits in Block 1 and 32 bits in Block 2. The contents of the bits for IH can be determined by the broadcaster.

TABLE  14

Data in Group 4

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB | 0000 0001 ... 1111 1111 | PI Code or BIMSB code |
| IH | ... | In-house data |

Application examples for IH

–	Identification of the sound programme.

–	Remote operation of transmitter networks.

–	Paging for the operational staff.

## 4.6	Group type 5/Transparent Data Channel TDC

Usage various

Group type 5 is used to transmit any kind of transparent data which can utilize the maximum capacity of the channel.

Group 5/Block 1

| GT | transparent DATA |
| --- | --- |
| 4 | 32 |

Group 5/Block 2

| FIGURE 10 GT | transparent DATA |
| --- | --- |
| 4 | 32 |

Group 5

NOTE 1 – This Group carries no PI or BI code to allow maximum capacity for transparent data.

NOTE 2 – Identification of the transmission has to be provided by transmission of Group 0 or Group 8 in the Group sequence.

## 4.7	Group type 6/Scheduling Information SI

Usage off-line

Group type 6 is used to transmit broadcast schedule information, to allow receivers to set up a database for easy access and selection of the wanted transmission.

Group 6/Block 1

| GT | PI/ BIMSB (ON) | CF | DF | ECC/ BILSB (ON) | START MSB |
| --- | --- | --- | --- | --- | --- |
| 4 | 16 | 1 | 1 | 8 | 6 |

Group 6/Block 2

| FIGURE 11 GT | START LSB | END | FMSB | FLSB | DOW1 |
| --- | --- | --- | --- | --- | --- |
| 4 | 3 | 9 | 8 | 8 | 4 |

Group 6

TABLE  15

Coding in Group 6

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB (ON) | 0000 0001 ... 1111 1111 | PI code or BIMSB code of other network |
| ECC BILSB (ON) | 0000 0001 ... 1111 1111 | ECC or BILSB code of other network |
| CF | 0 1 | PI-code environment BI-code environment |
| DF | 0 1 | Entry is not limited by a date Entry will have start- or end-date (transmitted in Group 7) |
| START | 0 0000 0000 ... 1 0010 0000 | 0000 UTC ... Start time of transmission in 5-minute interval 2355 UTC (see Formula (14)) |
| END | 0 0000 0000 ... 1 0010 0000 | 0000 UTC ... End time of transmission in 5-minute interval 2355 UTC (see Formula (14)) |
| FMSB | 0000 0000 ... 1111 1111 | Frequency code MSB coded according to Table 12 |
| FLSB | 0000 0000 ... 1111 1111 | Frequency code LSB coded according to Table 12 |

TABLE  16

Coding DOW1 in Group 6

| Data | Binary value | Meaning |
| --- | --- | --- |
| DOW1 (day of week) | 0000 0001 0010 0011 0100 0101 0110 0111 1000 1001 1010 1011 1100 1101 1110 1111 | Daily service Monday Tuesday Wednesday Thursday Friday Saturday Sunday Saturday and Sunday Weekdays only (Monday through Friday) Friday, Saturday and Sunday Monday and Tuesday Tuesday and Wednesday Wednesday and Thursday Thursday and Friday Friday and Saturday |
|  |  | Other combinations of operations have to be transmitted in separate Groups of type 6 using this coding or via Group 7 (see example). |

Example coding DOW1

Given a transmission on Monday, Wednesday, Thursday and Friday this will result in 3 separate Groups of type 6, coded as:

First	Group 6	0001 (Monday)

Second	Group 6	0011 (Wednesday)

Third	Group 6	1110 (Thursday and Friday)

### 4.7.1	Coding of START and END

The coding of START and END time of a transmission is based on 5-minute interval.

START/END = INT((HOURUTC⋅60+ MINUTEUTC)5)	(14)

## 4.8	Group type 7/Scheduling information supplementary SIS

Usage off-line

Group type 7 is used to transmit supplementary broadcaster schedule information, to allow receivers to set up a database for easy access and selection of the wanted transmission. The contents includes the target CIRAF-zones and the transmitter location. Using this information, the receiver could select the transmitter serving the area he is situated in and – or being closest to his location. The start time of the transmission has to be transmitted in order to provide a reference to the SI - entry, in Group 6.

Group 7/Block 1

| GT | PI/ BIMSB (ON) | CF | DF | ECC/ BILSB (ON) | START MSB |
| --- | --- | --- | --- | --- | --- |
| 4 | 16 | 1 | 1 | 8 | 6 |

Group 7/Block 2

| FIGURE 12 GT | START LSB | UC1 | Applications are defined by usage code UC1 |
| --- | --- | --- | --- |
| 4 | 3 | 4 | 25 |

Group 7

TABLE  17

Data in Group 7

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB (ON) | 0000 0001 ... 1111 1111 | PI Code or BIMSB code of other network |
| ECC BILSB (ON) | 0000 0001 ... 1111 1111 | ECC or BILSB code of other network |
| CF | 0 1 | PI-code environment BI-code environment |
| DF | 0 1 | Entry is not limited by a date Entry is limited by a start/end date (transmitted in Group 7) |
| START | 0 0000 0000 ... 1 0010 0000 | 0000 UTC ... start time of transmission 2355 UTC (see Formula (14)) |
| UC1 | 0000 ... 1111 | Usage code |

### 4.8.1	Information addressed by the usage code UC1

The information transmitted in Block 2 is determined by the usage code 1 (UC1, 4 bits) and is independent from the identification code. Sixteen applications can be addressed by the usage code.

TABLE  18

Coding of UC1 in Group 7

| UC1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | CIRAF 1 |  |  |  |  |  |  | CIRAF 2 |  |  |  |  |  |  | CIRAF 3 |  |  |  |  |  |  |  |  |  | P |  | S |  | C |  | X |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 1 |  | 1 |  | 1 |  | 1 |
| 0 | 0 | 0 | 1 | CIRAF 4 |  |  |  |  |  |  | CIRAF 5 |  |  |  |  |  |  | CIRAF 6 |  |  |  |  |  |  |  |  |  | P |  | S |  | X |  | X |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 1 |  | 1 |  | 1 |  | 1 |
| 0 | 0 | 1 | 0 | DSTART |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DOW1 |  |  |  |  |  |  |  |  |  | S |  |
| 4 |  |  |  | 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 1 |  |
| 0 | 0 | 1 | 1 | DEND |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DOW1 |  |  |  |  |  |  |  |  |  | S |  |
| 4 |  |  |  | 17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 1 |  |
| 0 | 1 | 0 | 0 | CIRAFTX |  |  |  |  |  |  | LAT |  |  |  |  |  |  |  |  |  | LON |  |  |  |  |  |  |  |  |  |  |  | X |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| 0 | 1 | 0 | 1 | not defined yet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 | not defined yet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | Reserved for broadcasters use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UC1  0

Three CIRAF zones of the intended Target area can be transmitted.

UC1  1

Three additional CIRAF zones of the intended Target area can be transmitted.

UC1  2

The START date and comprehensive days of week can be transmitted.

UC1  3

The END date and comprehensive days of week can be transmitted.

START and END date are coded with 17 bits as Julian date according to RDS specifications.

UC1  4

The CIRAF zone and the latitude, longitude of the transmitter location can be transmitted. (See Formulae (15) and (16)).

UC1  5 to 14

Not defined yet.

UC1  15

Information can be transmitted which are only related to the broadcaster.

TABLE  19

Data of UC1 in Group 7

| Data | Binary value | Meaning |
| --- | --- | --- |
| P | 0 1 | Permanent entry Entry is limited by start and/or end date |
| S | 0 1 | Special transmission |
| C | 0 1 | Only CIRAF 1 to 3 will be transmitted CIRAF 4 .. 6 will be transmitted in Group 7 UC1 = 1 |
| DOW2 | 1000000 ... 1111111 1000000 0100000 0010000 0001000 0000100 0000010 0000001 0000000 1111111 | Each bit representing one day of the week MSB = Monday (see example) 7 days a week Monday Tuesday Wednesday Thursday Friday Saturday Sunday Undefined Every day |
| LAT | 0101 1010 ... 1110 1010 | Latitude of transmitter 90 degrees NORTH ... Step 1 degree (see Formula (15)) Latitude of transmitter 90 degrees SOUTH |
| LON | 0 1011 0100 ... 1 1011 0100 | Longitude of transmitter 180 degrees EAST ... Step 1 degree (see Formula (16)) Longitude of transmitter 180 degrees WEST |
| DSTART |  | Date of the begin of the transmission (see RDS specification for coding Julian date) |
| DEND |  | Date of the end of the transmission (see RDS specification for coding Julian date) |
| CIRAF | 000 0001 ... 101 0101 | Target areas of the transmission Decimal values between 1 and 85 CIRAF1 ... 6 |
| CIRAFTX | 000 0001 ... 101 0101 | Geographical area of the transmitter location Decimal values between 1 and 85 |

Example for the treatment of DOW2 in a receiver

Given a transmission on Tuesday, Wednesday and Friday

code for Tuesday	0100000

code for Wednesday	0010000

code for Friday	0000100

————

resultant code	0110100  derived by a logical OR of all codes

### 4.8.2	Coding of latitude and longitude

Latitude (decimal) must be in the range of 90 South (-90) to 90 North (+90).

LAT(north)  INT(latitude0.5)
		LAT(south)  -INT(latitude0.5)	(15)

Longitude (decimal) must be in the range of 180 West (-180) to 180 East (180).

LON(east)  INT(longitude0.5)
		LON(west)  -INT(longitude0.5)	(16)

## 4.9	Group type 8/Additional tuning information ATI

Usage interactive

Group type 8 is used to transmit additional tuning information. To support fast tuning applications the identification code (PI or BI), marked by the code flag (CF) is also transmitted.

Depending on the identification code different information will be transmitted in Block 1. In case of PI (CF  0) the extended country code (ECC) can be transmitted and in case of BI (CF  1) the BILSB.

In both identification modes the programme type (PTY) is transmitted. The applied coding of PTY corresponds to RDS.

Group 8/Block 1

| GT | PI/ BIMSB | CF | X | ECC/ BILSB | PTY 1 | X |
| --- | --- | --- | --- | --- | --- | --- |

| 4 | 16 | 1 | 1 | 8 | 5 | 1 |
| --- | --- | --- | --- | --- | --- | --- |

Group 8/Block 2

| GT | UC2 | Applications are defined by the usage code UC2 |
| --- | --- | --- |

| 4 | 4 | 28 |
| --- | --- | --- |

FIGURE  13

Group 8

TABLE  20

Data in Group 8

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB | 0000 0001 ... 1111 1111 | PI code or BIMSB code |
| ECC BILSB | 0000 0001 ... 1111 1111 | ECC or BILSB code |
| CF | 0 1 | PI-code environment BI-code environment |
| PTY1 | 00000 ... 11111 | Programme type information (see RDS specifications) |
| UC2 | 0000 ... 1111 | Usage code |

### 4.9.1	Information addressed by the usage code UC2

The information transmitted in Block 2 is determined by the usage code 2 (UC2, 4 bits) and is independent from the identification code. Sixteen applications can be addressed by the usage code.

TABLE  21

Coding of UC2 in Group 8

| UC2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | PS char.7 |  |  |  |  |  |  | PS char.8 |  |  |  |  |  |  |  |  |  | (PTY 2) |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 1 | PTYN.1 |  |  |  |  |  |  | PTYN.2 |  |  |  |  |  |  |  |  |  | PTYN.3 |  |  |  |  |  |  |  |  |  | PTYN.4 |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 0 | PTYN.5 |  |  |  |  |  |  | PTYN.6 |  |  |  |  |  |  |  |  |  | PTYN.7 |  |  |  |  |  |  |  |  |  | PTYN.8 |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 0 | 0 | 1 | 1 | CIRAF 1 |  |  |  |  |  |  | CIRAF 2 |  |  |  |  |  |  |  |  |  | CIRAF 3 |  |  |  |  |  |  |  |  |  | CIRAF 4 |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 0 | 1 | 0 | 0 | CIRAF 5 |  |  |  |  |  |  | CIRAF 6 |  |  |  |  |  |  |  |  |  | CIRAF 7 |  |  |  |  |  |  |  |  |  | CIRAF 8 |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 0 | 1 | 0 | 1 | PS 1 |  |  |  |  |  |  | PS 2 |  |  |  |  |  |  |  |  |  | PS 3 |  |  |  |  |  |  |  |  |  | PS 4 |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 | 0 | PS 5 |  |  |  |  |  |  | PS 6 |  |  |  |  |  |  |  |  |  | PS 7 |  |  |  |  |  |  |  |  |  | PS 8 |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 7 |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 0 | 1 | 1 | 1 | START |  |  |  |  |  |  |  |  |  |  | END |  |  |  |  |  |  |  |  |  |  | CIRAF 1 |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 4 |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| 1 | 0 | 0 | 0 | FMSB |  |  |  |  |  |  |  |  | FLSB |  |  |  |  |  |  |  |  |  | STARTN |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 4 |  |  |  | 8 |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| 1 | 0 | 0 | 1 | not defined yet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 0 | not defined yet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | Reserved for broadcasters use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UC2  0

Two additional PS characters (characters 7 and 8) and PTY 2 can be transmitted. PTY 2 is used to characterize the programme in detail, e.g. Pop and News. PTY 2 should not be used until the RDS specification is finalized.

This usage code is restricted to PI usage only.

UC2  1 and UC2  2

The PTY name (2  4 characters) can be transmitted. The PTY name can be used additionally to define more clearly the programme type. Unused characters must be transmitted as space ASCII character (hex. 20).

UC2  3

The broadcast target area can be transmitted (CIRAF-zone: zone 1  primary, zones 2-4  additional). Using this information, the receiver could select transmissions aimed at a specific target area carrying a specific type of programme.

UC2  4

Additional target areas (CIRAF-zones 5-8) can be transmitted in conjunction with UC2  3. Using this information, the receiver could select transmissions aimed at a specific target area carrying a specific type of programme.

UC2  5

The first 4 characters of the programme service name are transmitted.

UC2  6

Characters 5 through 8 of the programme service name are transmitted.

UC2  7

Start and stop time as well as the primary CIRAF-zone of the broadcast target area as defined in UC2  3 CIRAF1 is transmitted. This will allow these information to be stored in the receiver in order to create a self-learning system.

UC2  8

The frequency of the next scheduled transmission carrying the same programme is transmitted. This allows to give an information on how to continue the reception when a frequency change is due. The STARTN time of this next scheduled transmission provides an indication, when this entry becomes valid.

UC2  9 to 14

Not defined yet.

UC2  15

Information can be transmitted which are only related to the broadcaster.

TABLE  22

Data of UC2 in Group 8

| Data | Binary value | Meaning |
| --- | --- | --- |
| PTY 2 | 00000 ... 11111 | (see RDS specifications) |
| PTYN.1 ... PTYN.8 | ... | Programme type information in text coded according to ISO 646 |
| CIRAF 1 ... CIRAF 8 | 000 0001 ... 101 0101 | Target area of transmission (1 to 85) |
| PS | .. | Programme service name (see 5.1 Group 0) |
| START | 0 0000 0000 ... 1 0001 1111 | 0000 UTC ... (coding see Formula (14)) 2355 UTC |
| END | 0 0000 0000 ... 1 0001 1111 | 0000 UTC ... (coding see Formula (14)) 2355 UTC |
| FMSB | 0000 0000 ... 1111 1111 | Frequency code MSB of the following programme coded according to Table 12 |
| FLSB | 0000 0000 ... 1111 1111 | Frequency code LSB of the following programme coded according to Table 12 |
| STARTN | 0 0000 0000 ... 1 0001 1111 | Start time (UTC) of the next scheduled transmission (coding see Formula (14)) |

## 4.10	Group type 9/differential GPS-data dGPS

Usage interactive

Group type 9 is used to transmit differential GPS-Data.

Group 9/Block 1

| GT | PI | AFDG Code | X | dGPS |
| --- | --- | --- | --- | --- |

| 4 | 16 | 8 | 3 | 5 |
| --- | --- | --- | --- | --- |

Group 9/Block 2

| GT | dGPS |
| --- | --- |

| 4 | 32 |
| --- | --- |

FIGURE  14

Group 9

AFDG

Alternative frequency carrying dGPS-Data are coded according to Table 12, Group 2.

The coding of dGPS-Data has to be finalized after the RDS applications are defined.

TABLE  23

Data in Group 9

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI | 0000 0001 ... 1111 1111 | PI code |
| AFDG | 0000 0000 ... 1111 1111 | Alternative frequency carrying dGPS coded according to Table 12 The frequency range is limited to LF and MF |
| dGPS | ... | Differential GPS data The coding is to be finalized after RDS applications are defined |
| X |  | Unused capacity |

## 4.11	Group type 10/TIME information UTC

Usage interactive

Group 10 can be used to transmit Time (UTC), Local time offset and Date to the receiver in order to set the internal clock.

Group 10/Block 1

| GT | PI/ BIMSB | CF | X | ECC/ BILSB | OS | LOS |
| --- | --- | --- | --- | --- | --- | --- |

| 4 | 16 | 1 | 1 | 8 | 1 | 5 |
| --- | --- | --- | --- | --- | --- | --- |

Group 10/Block 2

| GT | HOUR | MINUTE | JULIAN | X |
| --- | --- | --- | --- | --- |

| 4 | 5 | 6 | 17 | 4 |
| --- | --- | --- | --- | --- |

FIGURE  15

Group 10

TABLE  24

Data in Group 10

| Data | Binary value | Meaning |
| --- | --- | --- |
| PI/ BIMSB | 0000 0001 ... 1111 1111 | PI code or BIMSB code |
| ECC BILSB | 0 0000 0000 ... 1 0010 0000 | ECC or BILSB code |
| CF | 0 1 | PI-code environment BI-code environment |
| OS | 0 1 | Time offset to UTC positive (+) Time offset to UTC negative (-)(1) |
| LOS | 00000 ... 11000 | Offset in number of half hours Local time to UTC(1) |
| HOUR | 00000 ... 10111 | 0000 UTC ... Hour (UTC) 2300 UTC |
| MINUTE | 00 0000 ... 11 1011 | 00 ... Minute (UTC) 59 |
| JULIAN | 17 bits | Julian date (see RDS specification) |
| (1) For services crossing time zones LOS and OS should be set to 0 (zero). |  |  |

# 5	Glossary of terms

AF	alternative frequency

AFDG	alternative frequency for dGPS

AFT	alternative frequency carrying TMC

AM	amplitude modulation

AMDS	AM-data system

AM-DSB	amplitude double sideband

ATI	additional tuning information

BILSB	broadcast identification code LSB

BIMSB	broadcast identification code MSB

BTI	basic tuning and switching information

BW	flag indicating a particular bandwidth

CF	flag indicating the use of PI or BI

CIRAF	reception zone for broadcasts ranging from 1 to 85

CIRAFTX	geographical zone in which the transmitter is located

DEND	end date of an entry

DF	flag indicating a date-limited entry

dGPS	differential Global Positioning System

DOW1	day of week (limited coding)

DOW2	day of week (full coding)

DSTART	start date of an entry

ECC	extended country code

END	end of a transmission (UTC)

FLSB	frequency code (LSB)

FMSB	frequency code (MSB)

GT	Group type

HOUR	hour in UTC

IF	intermediate frequency

IH	in house

JULIAN	Julian date

LAT	geographical latitude 90N ... 90S (decimal)

LON	geographical longitude 180E ... 180W (decimal)

LOS	local time offset

LSB	least significant bits

MINUTE	minute in UTC

MSB	most significant bits

ON	other network or transmission of the broadcaster

OS	time offset sign

P	flag indicating a permanent scheduling entry

PI	programme identification code

PIX	flag indicating the use of extended country code ECC

PS	programme service name

PSX	flag indicating the use of extended PS

PTY1	programme type

PTY2	second programme type

PTYN	programme type name

RT	radio text

S	flag indicating a special transmission

SI	scheduling information

SIS	supplementary scheduling information

SSB	single sideband

START	start time of a transmission (UTC)

STARTN	start time of next scheduled transmission

TA	flag identifying traffic announcement

TDC	transparent data channel

TE	flag indicating end of radiotext

TMC	traffic message channel

TMCF	flag identifying traffic message transmissions

TN	number of radiotext

TP	flag identifying traffic programme

TSA	text segment address

UC1	usage code 1

UC2	usage code 2

UTC	time information

X	not defined

# 6	Index of Tables

Table 1 – Application of AMDS systems

Table 2 – Data elements

Table 3 – Offset words

Table 4 – PI structure

Table 5 – BI structure

Table 6 – Group types

Table 7 – Usage of Groups for different applications

Table 8 – Dynamic Group sequences

Table 9 – Data in Group 0

Table 10 – Data in Group 1

Table 11 – Data in Group 2

Table 12 – Coding of alternative frequencies

Table 13 – Data in Group 3

Table 14 – Data in Group 4

Table 15 – Coding in Group 6

Table 16 – Coding DOW1 in Group 6

Table 17 – Data in Group 7

Table 18 – Coding of UC1 in Group 7

Table 19 – Data of UC1 in Group 7

Table 20 – Data in Group 8

Table 21 – Coding of UC2 in Group 8

Table 22 – Data of UC2 in Group 8

Table 23 – Data in Group 9

Table 24 – Data in Group 10

Table 25 – Index of ALPHA-2 country codes (ISO 3166:1993)

# 7	Index of Figures

Figure 1 – Dependence of the permitted phase deviation value  on the transmission bit rate (Br)

Figure 2 – Basic circuit diagram of monophonic AM sound broadcasting system (AMDS) for supplementary data transmission

Figure 3 – Baseband coding structure

Figure 4 – Data format and addressing

Figure 5 – Group 0

Figure 6 – Group 1

Figure 7 – Group 2

Figure 8 – Group 3

Figure 9 – Group 4

Figure 10 – Group 5

Figure 11 – Group 6

Figure 12 – Group 7

Figure 13 – Group 8

Figure 14 – Group 9

Figure 15 – Group 10

# 8	Index of formulas

Formula 1

Formula 2

Formula 3

Formula 4

Formula 5

Formula 6

Formula 7

Formula 8

Formula 9

Formula 10

Formula 11

Formula 12

Formula 13

Formula 14

Formula 15

Formula 16

APPENDIX  A

TABLE  25

Index of ALPHA-2 country codes (ISO 3166:1993)

This index does not constitute the official list of names of entities

| Country | Country | 2-Letter | Country | Country |
| --- | --- | --- | --- | --- |
| Code | Code | Code | English | French |
| decimal | Hex. |  | name | name |
| 0 | 0 | .. | not used |  |
| 1 | 1 | AD | Andorra | Andorre |
| 2 | 2 | AE | United Arab Emirates | Emirats arabes unis |
| 3 | 3 | AF | Afghanistan | Afghanistan |
| 4 | 4 | AG | Antigua and Barbuda | Antigua-et-Barbuda |
| 5 | 5 | Al | Anguilla | Anguilla |
| 6 | 6 | AL | Albania | Albanie |
| 7 | 7 | AM | Armenia | Arménie |
| 8 | 8 | AN | Netherlands Antilles | Antilles néerlandaises |
| 9 | 9 | AO | Angola | Angola |
| 10 | 0A | AQ | Antarctica | Antarctique |
| 11 | 0B | AR | Argentina | Argentine |
| 12 | 0C | AS | American Samoa | Samoa américaines |
| 13 | 0D | AT | Austria | Autriche |
| 14 | 0E | AU | Australia | Australie |
| 15 | 0F | AW | Aruba | Aruba |
| 16 | 10 | AZ | Azerbaijan | Azerbaïdjan |
| 17 | 11 | BA | Bosnia and Herzegovina | Bosnie-Herzégovine |
| 18 | 12 | BB | Barbados | Barbade |
| 19 | 13 | BD | Bangladesh | Bangladesh |
| 20 | 14 | BE | Belgium | Belgique |
| 21 | 15 | BF | Burkina Faso | Burkina Faso |
| 22 | 16 | BG | Bulgaria | Bulgarie |
| 23 | 17 | BH | Bahrain | Bahreïn |
| 24 | 18 | BI | Burundi | Burundi |
| 25 | 19 | BJ | Benin | Bénin |
| 26 | 1A | BM | Bermuda | Bermudes |
| 27 | 1B | BN | Brunei Darussalam | Brunéi Darussalam |
| 28 | 1C | BO | Bolivia | Bolivie |
| 29 | 1D | BR | Brazil | Brésil |
| 30 | 1E | BS | Bahamas | Bahamas |
| 31 | 1F | BT | Bhutan | Bhoutan |
| 32 | 20 | BV | Bouvet Island | Bouvet Ile |
| 33 | 21 | BW | Botswana | Botswana |
| 34 | 22 | BY | Belarus | Bélarus |
| 35 | 23 | BZ | Belize | Belize |

TABLE  25 (continued )

| Country | Country | 2-Letter | Country | Country |
| --- | --- | --- | --- | --- |
| Code | Code | Code | English | French |
| decimal | Hex. |  | name | name |
| 36 | 24 | CA | Canada | Canada |
| 37 | 25 | CC | Cocos (Keeling) Islands | Cocos (Keeling) Iles des |
| 38 | 26 | CF | Central African Republic | Centrafricaine République |
| 39 | 27 | CG | Congo | Congo |
| 40 | 28 | CH | Switzerland | Suisse |
| 41 | 29 | CI | Côte d'Ivoire | Côte d'Ivoire |
| 42 | 2A | CK | Cook Islands | Iles Cook |
| 43 | 2B | CL | Chile | Chili |
| 44 | 2C | CM | Cameroon | Cameroun |
| 45 | 2D | CN | China | Chine |
| 46 | 2E | CO | Colombia | Colombie |
| 47 | 2F | CR | Costa Rica | Costa Rica |
| 48 | 30 | CU | Cuba | Cuba |
| 49 | 31 | CV | Cape Verde | Cap-Vert |
| 50 | 32 | CX | Christmas Island | Ile Christmas |
| 51 | 33 | CY | Cyprus | Chypre |
| 52 | 34 | CZ | Czech Republic | République tchèque |
| 53 | 35 | DE | Germany | Allemagne |
| 54 | 36 | DJ | Djibouti | Djibouti |
| 55 | 37 | DK | Denmark | Danemark |
| 56 | 38 | DM | Dominica | Dominique |
| 57 | 39 | DO | Dominican Republic | République Dominicaine |
| 58 | 3A | DZ | Algeria | Algérie |
| 59 | 3B | EC | Ecuador | Equateur |
| 60 | 3C | EE | Estonia | Estonie |
| 61 | 3D | EG | Egypt | Egypte |
| 62 | 3E | EH | Western Sahara | Sahara occidental |
| 63 | 3F | ER | Eritrea | Erythrée |
| 64 | 40 | ES | Spain | Espagne |
| 65 | 41 | ET | Ethiopia | Ethiopie |
| 66 | 42 | FI | Finland | Finlande |
| 67 | 43 | FJ | Fiji | Fidji |
| 68 | 44 | FK | Falkland Islands (Malvinas) | Iles Falkland (Malvinas) |
| 69 | 45 | FM | Micronesia (Federated States of) | Micronésie (Etats fédérés de) |
| 70 | 46 | FO | Faroe Islands | Iles Féroé |
| 71 | 47 | FR | France | France |
| 72 | 48 | FX | France Metropolitan | France métropolitaine |
| 73 | 49 | GA | Gabon | Gabon |
| 74 | 4A | GB | United Kingdom | Royaume-Uni |
| 75 | 4B | GD | Grenada | Grenade |
| 76 | 4C | GE | Georgia | Géorgie |
| 77 | 4D | GF | French Guyana | Guyane française |
| 78 | 4E | GH | Ghana | Ghana |
| 79 | 4F | GI | Gibraltar | Gibraltar |
| 80 | 50 | GL | Greenland | Groenland |

TABLE  25 (continued )

| Country | Country | 2-Letter | Country | Country |
| --- | --- | --- | --- | --- |
| Code | Code | Code | English | French |
| decimal | Hex. |  | name | name |
| 81 | 51 | GM | Gambia | Gambie |
| 82 | 52 | GN | Guinea | Guinée |
| 83 | 53 | GP | Guadeloupe | Guadeloupe |
| 84 | 54 | GQ | Equatorial Guinea | Guinée équatoriale |
| 85 | 55 | GR | Greece | Grèce |
| 86 | 56 | GS | South Georgia and the South Sandwich Islands | Géorgie du Sud et les îles Sandwich du Sud |
| 87 | 57 | GT | Guatemala | Guatemala |
| 88 | 58 | GU | Guam | Guam |
| 89 | 59 | GW | Guinea-Bissau | Guinée-Bissau |
| 90 | 5A | GY | Guyana | Guyana |
| 91 | 5B | HK | Hong Kong | Hong Kong |
| 92 | 5C | HM | Heard Island and McDonald Islands | Ile Heard et Iles McDonald |
| 93 | 5D | HN | Honduras | Honduras |
| 94 | 5E | HR | Croatia | Croatie |
| 95 | 5F | HT | Haiti | Haïti |
| 96 | 60 | HU | Hungary | Hongrie |
| 97 | 61 | ID | Indonesia | Indonésie |
| 98 | 62 | IE | Ireland | Irlande |
| 99 | 63 | IL | Israel | Israël |
| 100 | 64 | IN | India | Inde |
| 101 | 65 | IO | British Indian Ocean Territory | Territoire britannique de l'Océan Indien |
| 102 | 66 | IQ | Iraq | Iraq |
| 103 | 67 | IR | Iran (Islamic Republic of) | Iran (République islamique d') |
| 104 | 68 | IS | Iceland | Islande |
| 105 | 69 | IT | Italy | Italie |
| 106 | 6A | JM | Jamaica | Jamaïque |
| 107 | 6B | JO | Jordan | Jordanie |
| 108 | 6C | JP | Japan | Japon |
| 109 | 6D | KE | Kenya | Kenya |
| 110 | 6E | KG | Kyrgyz Republic | République Kirghiz |
| 111 | 6F | KH | Cambodia | Cambodge |
| 112 | 70 | KI | Kiribati | Kiribati |
| 113 | 71 | KM | Comoros | Comores |
| 114 | 72 | KN | Saint Kitts and Nevis | Saint-Kitts-et-Nevis |
| 115 | 73 | KP | Korea (Democratic People's of) | Corée (République populaire démocratique de) |
| 116 | 74 | KR | Korea (Republic of) | Corée (République de) |
| 117 | 75 | KW | Kuwait | Koweït |
| 118 | 76 | KY | Cayman Islands | Iles Caïmans |
| 119 | 77 | KZ | Kazakstan | Kazakstan |
| 120 | 78 | LA | Lao (People's Democratic Republic) | Lao (République démocratique populaire) |
| 121 | 79 | LB | Lebanon | Liban |
| 122 | 7A | LC | Saint Lucia | Sainte-Lucie |
| 123 | 7B | LI | Liechtenstein | Liechtenstein |

TABLE  25 (continued )

| Country | Country | 2-Letter | Country | Country |
| --- | --- | --- | --- | --- |
| Code | Code | Code | English | French |
| decimal | Hex. |  | name | name |
| 124 | 7C | LK | Sri Lanka | Sri Lanka |
| 125 | 7D | LR | Liberia | Libéria |
| 126 | 7E | LS | Lesotho | Lesotho |
| 127 | 7F | LT | Lithuania | Lituanie |
| 128 | 80 | LU | Luxembourg | Luxembourg |
| 129 | 81 | LV | Latvia | Lettonie |
| 130 | 82 | LY | Libyan Arab Jamahiriya | Jamahiriya arabe libyenne |
| 131 | 83 | MA | Morocco | Maroc |
| 132 | 84 | MC | Monaco | Monaco |
| 133 | 85 | MD | Moldova (Republic of) | Moldova (République de) |
| 134 | 86 | MG | Madagascar | Madagascar |
| 135 | 87 | MH | Marshall Islands | Iles Marshall |
| 136 | 88 | ML | Mali | Mali |
| 137 | 89 | MM | Myanmar | Myanmar |
| 138 | 8A | MN | Mongolia | Mongolie |
| 139 | 8B | MO | Macau | Macao |
| 140 | 8C | MP | Northern Mariana Islands | Iles Mariannes du Nord |
| 141 | 8D | MQ | Martinique | Martinique |
| 142 | 8E | MR | Mauritania | Mauritanie |
| 143 | 8F | MS | Montserrat | Montserrat |
| 144 | 90 | MT | Malta | Malte |
| 145 | 91 | MU | Mauritius | Maurice |
| 146 | 92 | MV | Maldives | Maldives |
| 147 | 93 | MW | Malawi | Malawi |
| 148 | 94 | MX | Mexico | Mexique |
| 149 | 95 | MY | Malaysia | Malaisie |
| 150 | 96 | MZ | Mozambique | Mozambique |
| 151 | 97 | NA | Namibia | Namibie |
| 152 | 98 | NC | New Caledonia | Nouvelle-Calédonie |
| 153 | 99 | NE | Niger | Niger |
| 154 | 9A | NF | Norfolk Island | Ile Norfolk |
| 155 | 9B | NG | Nigeria | Nigéria |
| 156 | 9C | NI | Nicaragua | Nicaragua |
| 157 | 9D | NL | Netherlands | Pays-Bas |
| 158 | 9E | NO | Norway | Norvège |
| 159 | 9F | NP | Nepal | Népal |
| 160 | A0 | NR | Nauru | Nauru |
| 161 | A1 | NU | Niue | Nioué |
| 162 | A2 | NZ | New Zealand | Nouvelle-Zélande |
| 163 | A3 | OM | Oman | Oman |
| 164 | A4 | PA | Panama | Panama |
| 165 | A5 | PE | Peru | Pérou |
| 166 | A6 | PF | French Polynesia | Polynésie française |
| 167 | A7 | PG | Papua New Guinea | Papouasie-Nouvelle-Guinée |
| 168 | A8 | PH | Philippines | Philippines |

TABLE  25 (continued )

| Country | Country | 2-Letter | Country | Country |
| --- | --- | --- | --- | --- |
| Code | Code | Code | English | French |
| decimal | Hex. |  | name | name |
| 169 | A9 | PK | Pakistan | Pakistan |
| 170 | AA | PL | Poland | Pologne |
| 171 | AB | PM | Saint Pierre and Miquelon | Saint-Pierre-et-Miquelon |
| 172 | AC | PN | Pitcairn | Pitcairn |
| 173 | AD | PR | Puerto Rico | Porto Rico |
| 174 | AE | PT | Portugal | Portugal |
| 175 | AF | PW | Palau | Palau |
| 176 | B0 | PY | Paraguay | Paraguay |
| 177 | B1 | QA | Qatar | Qatar |
| 178 | B2 | RE | Réunion | Réunion |
| 179 | B3 | RO | Romania | Roumanie |
| 180 | B4 | RU | Russian Federation | Russie (Fédération de) |
| 181 | B5 | RW | Rwanda | Rwanda |
| 182 | B6 | SA | Saudi Arabia | Arabie saoudite |
| 183 | B7 | SB | Solomon Islands | Iles Salomon |
| 184 | B8 | SC | Seychelles | Seychelles |
| 185 | B9 | SD | Sudan | Soudan |
| 186 | BA | SE | Sweden | Suède |
| 187 | BB | SG | Singapore | Singapour |
| 188 | BC | SH | Saint Helena | Sainte-Hélène |
| 189 | BD | SI | Slovenia | Slovénie |
| 190 | BE | SJ | Swalbard and Jan Mayen | Svalbard et ïle Jan Mayen |
| 191 | BF | SK | Slovakia | Slovaquie |
| 192 | C0 | SL | Sierra Leone | Sierra Leone |
| 193 | C1 | SM | San Marino | Saint-Marin |
| 194 | C2 | SN | Senegal | Sénégal |
| 195 | C3 | SO | Somalia | Somalie |
| 196 | C4 | SR | Suriname | Suriname |
| 197 | C5 | ST | Sao Tome and Principe | Sao-Tomé-et-Principe |
| 198 | C6 | SV | El Salvador | El Salvador |
| 199 | C7 | SY | Syrian Arab Republic | Syrienne (République arabe) |
| 200 | C8 | SZ | Swaziland | Swaziland |
| 201 | C9 | TC | Turks and Caicos Islands | Turks et Caiques Iles |
| 202 | CA | TD | Chad | Tchad |
| 203 | CB | TF | French Southern Territories | Terres australes françaises |
| 204 | CC | TG | Togo | Togo |
| 205 | CD | TH | Thailand | Thaïlande |
| 206 | CE | TJ | Tajikistan | Tadjikistan |
| 207 | CF | TK | Tokelau | Tokélaou |
| 208 | D0 | TM | Turkmenistan | Turkménistan |
| 209 | D1 | TN | Tunisia | Tunisie |
| 210 | D2 | TO | Tonga | Tonga |
| 211 | D3 | TP | East Timor | Timor oriental |
| 212 | D4 | TR | Turkey | Turquie |

TABLE  25 (end )

| Country | Country | 2-Letter | Country | Country |
| --- | --- | --- | --- | --- |
| Code | Code | Code | English | French |
| decimal | Hex. |  | name | name |
| 213 | D5 | TT | Trinidad and Tobago | Trinité-et-Tobago |
| 214 | D6 | TV | Tuvalu | Tuvalu |
| 215 | D7 | TW | Taiwan (Province of China) | Taiwan (Province de Chine) |
| 216 | D8 | TZ | Tanzania (United Republic of) | Tanzanie (République-Unie de) |
| 217 | D9 | UA | Ukraine | Ukraine |
| 218 | DA | UG | Uganda | Ouganda |
| 219 | DB | UM | United States Minor Outlying Islands | Iles mineures éloignées des Etats-Unis |
| 220 | DC | US | United States | Etats-Unis |
| 221 | DD | UY | Uruguay | Uruguay |
| 222 | DE | UZ | Uzbekistan | Ouzbékistan |
| 223 | DF | VA | Vatican City State (Holy See) | Vatican (Etat de la Cité du) (Saint-Siège) |
| 224 | E0 | VC | Saint Vincent and the Grenadines | Saint-Vincent-et-Grenadines |
| 225 | E1 | VE | Venezuela | Venezuela |
| 226 | E2 | VG | Virgin Islands (British) | Iles Vierges (britanniques) |
| 227 | E3 | VI | Virgin Islands (US) | Iles Vierges (Etats-Unis) |
| 228 | E4 | VN | Viet Nam | Viet Nam |
| 229 | E5 | VU | Vanuatu | Vanuatu |
| 230 | E6 | WF | Wallis and Futuna Islands | Iles Wallis et Futuna |
| 231 | E7 | WS | Samoa | Samoa |
| 232 | E8 | YE | Yemen | Yémen |
| 233 | E9 | YT | Mayotte | Mayotte |
| 234 | EA | YU | Yugoslavia | Yougoslavie |
| 235 | EB | ZA | South Africa | Afrique du Sud |
| 236 | EC | ZM | Zambia | Zambie |
| 237 | ED | ZR | Zaire | Zaïre |
| 238 | EE | ZW | Zimbabwe | Zimbabwe |
| 239 | EF |  | Not assigned |  |
| 240 | F0 |  | Not assigned |  |
| 241 | F1 |  | Not assigned |  |
| 242 | F2 |  | Not assigned |  |
| 243 | F3 |  | Not assigned |  |
| 244 | F4 |  | Not assigned |  |
| 245 | F5 |  | Not assigned |  |
| 246 | F6 |  | Not assigned |  |
| 247 | F7 |  | Not assigned |  |
| 248 | F8 |  | Not assigned |  |
| 249 | F9 |  | Not assigned |  |
| 250 | FA |  | Not assigned |  |
| 251 | FB |  | Not assigned |  |
| 252 | FC |  | Not assigned |  |
| 253 | FD |  | Not assigned |  |
| 254 | FE |  | Not assigned |  |
| 255 | FF |  | Not used |  |

*	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2002 in accordance with Resolution ITU-R 44.