RECOMMENDATION  ITU-R  BT.1120-4

Digital interfaces for HDTV studio signals

(Question ITU-R 42/6)

(1994-1998-2000-2003)

The ITU Radiocommunication Assembly,

considering

a)	that in the scope of Recommendation ITU-R BT.709, studio standards for HDTV have been developed for 1125- and 1250- line systems, which comprise systems related to conventional television as well as systems with the square pixel common image format (CIF) including progressive scanning;

b)	that Recommendation ITU-R BT.709 contains the following HDTV studio standards to cover a wide range of applications:

for systems related to conventional television:

–	1125 total line, 2:1 interlace scanning, 60 fields/s, 1035 active line standard;

–	1250 total line, 2:1 interlace scanning, 50 fields/s, 1152 active line standard.

for systems with CIF (1920  1080):

–	1125 total lines and 1080 active lines;

–	picture rates of 60, 50, 30, 25 and 24 Hz, including progressive, interlace and segmented frame transport;

c)	that in Recommendation ITU-R BT.709, the 1920  1080 HD-CIF is given as a preferred format for new installations, where interoperability with other applications is important, and work is being directed with the aim of reaching a unique worldwide standard;

d)	that the HD-CIF systems provide a common data rate feature, which allows for the use of a unique digital interface;

e)	that a whole range of equipment based on the above systems has been developed or is being developed and is commercially available now or soon, including all that necessary for broadcasting chains and for industrial applications;

f)	that many programmes are being produced in the above systems using the above equipments and that in the development of broadcasting and other services there is an increasing need for HDTV production installations;

g)	that the use of digital technology and digital interconnection is highly desirable to reach and maintain the level of performance required for HDTV;

h)	that there are clear advantages for establishing interface specifications for HDTV production installations,

recommends

1	that the specifications described in this Recommendation should be used for the basic digital coding as well for the bit-parallel and bit-serial interfaces for HDTV studio signals.

PART  1

Interfaces for HDTV signals conforming to
Recommendation ITU-R BT.709, Part 1

# 1	Digital representation

## 1.1	Coding characteristics

The signals to be digitized should comply with the characteristics described in Recommendation ITU-R BT.709, Part 1.

## 1.2	Construction of digital signals

See Part 2, § 1.2.

TABLE  1

Digital coding parameters

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 1 | Coded signals Y, CB, CR ou R, G, B | These signals are obtained from gamma pre-corrected signals, namely EY', ECB', ECR' or ER', EG', EB' Also see Recommendation ITU-R BT.709, Part 1 |  |
| 2 | Sampling lattice – R, G, B, Y | Orthogonal, line and picture repetitive |  |
| 3 | Sampling lattice – CB, CR | Orthogonal, line and picture repetitive, co-sited with each other and with alternate Y samples. The first active colour-difference samples are co-sited with the first active Y sample |  |
| 4 | Number of active lines | 1035 | 1152 |
| 5 | Sampling frequency (1) – R, G, B, Y (MHz) | 74.25 | 72 |
| 6 | Sampling frequency (1) – CB, CR | Half of luminance sampling frequency |  |
| 7 | Number of samples/line – R, G, B, Y – CB, CR | 2200 1100 | 2304 1152 |
| 8 | Number of active samples/line – R, G, B, Y – CB, CR | 1920 960 |  |
| 9 | Position of the first active Y, CB, CR sampling instants with respect to the analogue sync timing reference OH (2) (see Fig. 6) | 192 T | 256 T |
| 10 | Coding format | Uniformly quantized PCM for each of the video component signals 8 or 10 bit/sample 10 bit preferable |  |

TABLE  1  (end )

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 11 | Quantization level assignment (3) – Video data – Timing reference | 1.00 through 254.75 0.00 and 255.75 (4) |  |
| 12 | Quantization levels (5) – Black level R, G, B, Y – Achromatic level CB, CR – Nominal peak – R, G, B, Y CB, CR | 16.00 128.00 235.00 16.00 and 240.00 |  |
| 13 | Filter characteristics | See Recommendation ITU-R BT.709 |  |
| (1) The sampling clock must be locked to the line frequency. The tolerance on frequency is ±0.001% for 1125/60/2:1 and ±0.0001% for 1250/50/2:1, respectively. (2) T denotes the duration of the luminance sampling clock or the reciprocal of the luminance sampling frequency. (3) To reduce confusion when using 8-bit and 10-bit systems together, the two LSBs of the 10-bit system are read as two fractional bits. The quantization scale in an 8-bit system ranges from 0 to 255 in steps of 1, and in a 10-bit system from 0.00 to 255.75 in steps of 0.25. When 8-bit words are presented in a 10-bit system, two LSBs of zeros are to be appended to the 8-bit words. (4) In the case of a 8-bit system, eight MSBs are used. (5) These levels refer to precise nominal video levels. Signal processing may occasionally cause the signal level to deviate outside these ranges. |  |  |  |

# 2	Digital interface

The interface provides a unidirectional interconnection between a single source and a single destination. The data signals are in the form of binary information and are coded accordingly:

–	video data (8-bit or 10-bit words);

–	timing reference and identification codes (8-bit or 10-bit words except for 1250/50/2:1, which use 10-bit words only);

–	ancillary data (see Recommendation ITU-R BT.1364).

## 2.1	Video data

Y, CB, CR signals are handled as 20-bit words by time-multiplexing CB and CR components. Each 20-bit word corresponds to a colour-difference sample and a luminance sample. The multiplex is organized as:

(CB1 Y1) (CR1 Y2) (CB3 Y3) (CR3 Y4) ...

where Yi indicates the i-th active sample of a line, while CBi and CRi indicate the colour-difference samples of CB and CR components co-sited with the Yi sample. Note that the index “i” on colour-difference samples takes only odd values due to the half-rate sampling of the colour-difference signals.

The data words corresponding to digital levels 0.00 through 0.75 and 255.00 through 255.75 are reserved for data identification purposes and must not appear as video data.

For 1125/60/2:1, R, G, B signals are handled as 30-bit words in addition to the above 20-bit words for Y, CB, CR signals.

## 2.2	Video timing relationship with analogue waveform

The digital line occupies m clock periods. It begins at f clock periods prior to the reference transition (OH) of the analogue synchronizing signal in the corresponding line. The digital active line begins at g clock periods after the reference transition (OH). The values for m,  f  and g are listed in Table 2. See Fig. 6 and Table 2 for detailed timing relationships in the line interval.

The start of digital field is fixed by the position specified for the start of the digital line. See Fig. 1 and Table 3 for detailed relationships in the field interval.

TABLE  2

Line interval timing specifications

| Symbol | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
|  | Interlace ratio | 2:1 |  |
|  | Number of active Y samples per line | 1920 |  |
|  | Luminance sampling frequency (MHz) | 74,25 | 72 |
| a | Analogue line blanking (s) | 3.771 | 6.00 |
| b | Analogue active line (s) | 25.859 | 26.00 |
| c | Analogue full line (s) | 29.630 | 32.00 |
| d | Duration between end of analogue active video and start of EAV (T) | 0-6 | 24 |
| e | Duration between end of SAV and start of analogue active video (T) | 0-6 | 24 |
| f | Duration between start of EAV and analogue timing reference OH (T) | 88 | 128 |
| g | Duration between analogue timing reference OH and end of SAV (T) | 192 | 256 |
| h | Video data block (T) | 1928 |  |
| i | Duration of EAV (T) | 4 |  |
| j | Duration of SAV (T) | 4 |  |
| k | Digital line blanking (T) | 280 | 384 |
| l | Digital active line (T) | 1920 |  |
| m | Digital line (T) | 2200 | 2304 |
| NOTE 1 – The parameter values for analogue specifications expressed by the symbols a, b and c indicate the nominal values. NOTE 2 – T denotes the duration of the luminance sampling clock or the reciprocal of the luminance sampling frequency. |  |  |  |

## 2.3	Video timing reference codes (SAV and EAV)

There are two timing reference codes, one at the beginning of each video data block (start of active video, SAV) and the other at the end of each video data block (end of active video, EAV). These codes are contiguous with the video data, and continue during the field/frame blanking interval, as shown in Fig. 1.

Each code consists of a four-word sequence. The bit assignment of the word is given in Table 14. The first three words are the fixed preamble and the fourth word carries the information that defines field identification (F), field/frame blanking period (V), and line blanking period (H). In an 8-bit implementation bits Nos. 9 to 2 inclusive are used; note in 1250/50/2:1 all 10 bits are required.

The bits F and V change state synchronously with EAV at the beginning of the digital line.

The value of protection bits, P0 to P3, depends on the F, V and H as shown in Table 15. The arrangement permits one-bit errors to be corrected and two-bit errors to be detected at the receiver, but only in the 8 MSBs, as shown in Table 16.

TABLE  3

Field interval timing specifications

| Symbol | Definition | Digital line number |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
|  | Number of active lines | 1035 | 1152 |
| L1 | First line of field No. 1 | 1 |  |
| L2 | Last line of digital field blanking No. 1 | 40 | 44 |
| L3 | First line of field No. 1 active video | 41 | 45 |
| L4 | Last line of field No. 1 active video | 557 | 620 |
| L5 | First line of digital field blanking No. 2 | 558 | 621 |
| L6 | Last line of field No. 1 | 563 | 625 |
| L7 | First line of field No. 2 | 564 | 626 |
| L8 | Last line of digital field blanking No. 2 | 602 | 669 |
| L9 | First line of field No. 2 active video | 603 | 670 |
| L10 | Last line of field No. 2 active video | 1120 | 1245 |
| L11 | First line of digital field blanking No. 1 | 1121 | 1246 |
| L12 | Last line of field No. 2 | 1125 | 1250 |
| NOTE 1 – Digital field blanking No. 1 denotes the field blanking period that is prior to the active video of field No. 1, and digital field blanking No. 2 denotes that prior to the active video of field No. 2. |  |  |  |

## 2.4	Ancillary data

See Part 2, § 2.4.

## 2.5	Data words during blanking

See Part 2, § 2.5.

# 3	Bit-parallel interface

For the system of 1125/60/2:1, the bits of the digital code words which describe the video signal are transmitted in parallel by means of 20 or 30 shielded conductor pairs. The 20 conductor pairs are used for the transmission of the signal set consisting of luminance Y and time-multiplexed colour-difference CB/CR components. The 30 conductor pairs are used for the transmission of R, G, B signals or Y, CB/CR components with an additional data stream (auxiliary channel). An additional shielded conductor pair carries the synchronous clock at 74.25 MHz.

For the 1250/50/2:1 system, the bits of digital code words that describe the video signal are transmitted in parallel by means of 20 signal pairs, where each pair carries a stream of bits, 10 pairs for luminance data and 10 pairs for time-multiplexed colour-difference data. The 20 pairs can also carry ancillary data. A 21st pair provides a synchronous clock at 36 MHz.

Data signals are transmitted in non-return-to-zero (NRZ) form in real time (unbuffered).

## 3.1	Clock signal and clock-to-data timing relationship

For the system of 1125/60/2:1, the transmitted clock signal is a square wave, of which positive transitions occur midway between the data transitions as shown in Fig. 8 and Table 4.

For 1250/50/2:1, the transmitted clock signal is a 36 MHz square wave of unity mark/space ratio, the transitions of which are coincident with the transition of the data (see Fig. 2). A logical high state of the clock is concurrent with Y and CB data samples and a logical low state with Y and CR data samples, as shown in Fig. 2 and Table 4.

TABLE  4

Clock signal specifications

| Parameter | Value |  |
| --- | --- | --- |
|  | 1125/60/2:1 | 1250/50/2:1 |
| Sampling frequency for Y, R, G, B signals (MHz) | 74.25 | 72 |
| Clock period Tck Nominal value (ns) | 1/(2200 fH) 13.468 | 1/(1152 fH) 27.778 |
| Clock pulse width, t | 0.5 Tck |  |
| Tolerance | 0.11 Tck | (nominal) |
| Clock jitter | Within 0.04 Tck | Within 0.5 ns |
|  | from the average time of transition over one field in interlace systems, and over one frame in progressive systems |  |
| Data timing, Td Tolerance | 0.5 Tck 0.075 Tck | 0.25 Tck (nominal) |
| NOTE 1 – fH denotes the line frequency. NOTE 2 – Values are specified at the sending end (source). |  |  |

## 3.2	Electrical characteristics of the interface

The interface employs 21 line drivers and line receivers, in the case of the transmission of Y and CB/CR components. Each line driver has a balanced output and the corresponding line receiver has a balanced input. For 1125/60/2:1, the interface employs 31 line drivers and line receivers, in the case of R, G and B components or Y, CB/CR with an additional data stream (auxiliary channel).

Although the use of ECL technology is not mandatory, the line driver and receiver must be ECL 10 k compatible for 1125/60/2:1, and ECL 100 k compatible for 1250/50/2:1, i.e. they must permit the use of ECL for either drivers or receivers.

The receiver must sense correctly the data when a random signal produces conditions represented by the eye diagram of Fig. 3.

TABLE  5

Line driver characteristics

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 1 | Output impedance () | 110 maximum | 100 maximum |
| 2 | Common mode voltage (1) (V) | –1.29  15% | –1.3  15% |
| 3 | Signal amplitude(2) (V) | 0.6 to 2.0 p-p | 0.8 to 2.0 p-p |
| 4 | Rise and fall times (3) |  0.15 Tck |  3 ns |
| 5 | Difference between rise and fall times |  0.075 Tck |  1.0 ns |
| NOTE 1 – Tck denotes the clock period (see Table 4). (1) Measured relative to ground. (2) Measured across a resistive load having the nominal impedance of the assumed cables, that is 110  for 1125/60/2:1, and 100  for 1250/50/2:1. (3) Measured between the 20% and 80% points across a resistive load having the nominal impedance of the assumed cable. |  |  |  |

TABLE  6

Line driver characteristics

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| 1 | Input impedance () | 110  10% | 100  10% |
| 2 | Maximum input signal voltage (V) | 2.0 p-p |  |
| 3 | Minimum input signal voltage (mV) | 185 p-p |  |
| 4 | Maximum common mode voltage (1) (V) | 0.3 | 0.5 |
| 5 | Differential delay Tmin (2) | 0.3 Tck | 4.5 ns |
| NOTE 1 – Tck denotes the clock period (see Table 4). (1) Comprising interference in the range DC to line frequency (fH). (2) Data must be correctly sensed when the differential delay between the received clock and data is within this range (see Fig. 3). |  |  |  |

## 3.3	Mechanical characteristics

### 3.3.1	Connector

The interface uses a multi-contact connector. Connectors are locked by two screws on the cable connectors and two threaded bolts on the equipment. Cable connectors employ pin contacts and equipment connectors employ socket contacts. Shielding of the connectors and cables is mandatory.

For 1125/60/2:1, a 93-contact connector is used. Contact assignments are indicated in Tables 20 and 21. The mechanical specifications for the connectors are shown in Figs. 11, 12 and 13.

For 1250/50/2:1, a 50-contact type D subminiature connector is used. Contact assignments are indicated in Table 7 and Fig. 4 (for information, suggested contact assignment for a printed circuit board (PCB) header are shown in Fig. 5).

### 3.3.2	Interconnecting cable

For 1125/60/2:1, two types of multichannel cable, either 21 or 31 channels, can be used in accordance with the transmission signal set (see Table 21). The cable consists of twisted pairs with an individual shield for each pair. It also contains an overall shield. The nominal characteristic impedance of each twisted pair is 110 . The cable shall possess the characteristics that satisfy the conditions of the eye diagram shown in Fig. 3 up to a maximum cable length of 20 m.

For 1250/50/2:1, a cable with 21-channel balanced conductor pairs is used. The nominal characteristic impedance of each conductor pair is 100 . Cable length up to 30 m may be employed when a high-quality cable is used.

TABLE  7

Connector contact assignment for 1250/50/2:1

| Contact | Signal line | Contact | Signal line | Contact | Signal line |
| --- | --- | --- | --- | --- | --- |
| 1 | Clock A (CKA) |  |  | 34 | Clock B |
| 2 | GND | 18 | GND | 35 | GND |
| 3 | Data 9A (D9A) | 19 | GND | 36 | Data 9B |
| 4 | Data 8B | 20 | Data 8A | 37 | Data 7A |
| 5 | Data 6A | 21 | Data 7B | 38 | Data 6B |
| 6 | Data 5B | 22 | Data 5A | 39 | Data 4A |
| 7 | Data 3A | 23 | Data 4B | 40 | Data 3B |
| 8 | Data 2B | 24 | Data 2A | 41 | Data 1A |
| 9 | Data 0A | 25 | Data 1B | 42 | Data 0B |
| 10 | GND | 26 | GND | 43 | GND |
| 11 | Data 19A | 27 | GND | 44 | Data 19B |
| 12 | Data 18B | 28 | Data 18A | 45 | Data 17A |
| 13 | Data 16A | 29 | Data 17B | 46 | Data 16B |
| 14 | Data 15B | 30 | Data 15A | 47 | Data 14A |
| 15 | Data 13A | 31 | Data 14B | 48 | Data 13B |
| 16 | Data 12B | 32 | Data 12A | 49 | Data 11A |
| 17 | Data 10A | 33 | Data 11B | 50 | Data 10B |
| NOTE 1 – Data 9-Data 0 represent each bit of the luminance signal (Y ), and Data 19-Data 10 that of time-multiplexed colour-difference signal (CR /CB ). The suffix 19 to 0 indicates the bit number (bit 19 denotes MSB for CR/CB and bit 9 MSB for Y ). A and B correspond to the terminals A and B of Fig. 9, respectively. |  |  |  |  |  |

# 4	Bit-serial interface

## 4.1	Data format

The bit-serial data consists of video data, video timing reference codes, line number data, error detection codes, ancillary data and blanking data. Each data has a word-length of 10 bits, and is represented as parallel data before serialization. Two parallel streams (i.e. luminance data Y and colour-difference data CB/CR) are multiplexed and serialized in accordance with § 4.2.

### 4.1.1	Video data

The video data should be 10-bit words representing Y, CB /CR of the video systems defined in § 1.

### 4.1.2	Video timing reference codes

The video timing reference codes, SAV and EAV have the same format as that defined in § 2.

### 4.1.3	Line number data

The line number data is composed of two words indicating the line number. The bit assignment of the line number data is shown in Table 22. The line number data should be located immediately after EAV.

### 4.1.4	Error detection codes

See Part 2, § 4.1.4.

### 4.1.5	Ancillary data

See Part 2, § 4.1.5.

### 4.1.6	Blanking data

See Part 2, § 4.1.6.

## 4.2	Transmission format

See Part 2, § 4.2.

### 4.2.1	Word-multiplexing

The two parallel streams should be multiplexed word by word into a single 10-bit parallel stream in the order of CB, Y, CR, Y, CB, Y, CR, Y ... (see Fig. 14 and Table 8).

TABLE  8

Data stream timing specifications (see Fig. 14)

| Symbol | Parameter | Value |  |
| --- | --- | --- | --- |
|  |  | 1125/60/2:1 | 1250/50/2:1 |
| T | Parallel clock period (ns) | 1000/74,25 | 1000/72 |
| Ts | Multiplexed parallel data clock period | T/2 |  |
| m | Digital line in parallel data stream | 2200 | 2304 |
| k | Digital line blanking in parallel data stream | 280 | 384 |
| n | Ancillary data or blanking data in parallel data stream | 268 | 372 |
| ms | Digital line in multiplexed parallel data stream | 4400 | 4608 |
| ks | Digital line blanking in multiplexed parallel data stream | 560 | 768 |
| ns | Ancillary data or blanking data in multiplexed parallel data stream | 536 | 744 |

### 4.2.2	Serializing

See Part 2, § 4.2.2.

### 4.2.3	Channel coding

See Part 2, § 4.2.3.

### 4.2.4	Serial clock

Table 9 specifies the serial clock frequencies, which are twenty times the frequency of the parallel clock (see Table 4).

TABLE  9

Serial clock frequency

| Parameter | Value |  |
| --- | --- | --- |
|  | 1125/60/2:1 | 1250/50/2:1 |
| Serial clock frequency (GHz) | 1.485 | 1.400 |

### 4.2.5	Bit-serial digital check field

See Part 2, § 4.2.5.

## 4.3	Coaxial cable interfaces

See Part 2, § 4.3.

### 4.3.1	Line driver characteristics (source)

See Part 2, § 4.3.1.

### 4.3.2	Line receiver characteristics (destination)

See Part 2, § 4.3.2.

### 4.3.3	Transmission line characteristics

See Part 2, § 4.3.3.

### 4.3.4	Connector

See Part 2, § 4.3.4.

## 4.4	Optical fibre interfaces

See Part 2, § 4.4.

PART  2

Interfaces for HDTV signals conforming to
Recommendation ITU-R BT.709, Part 2

This part specifies digital interfaces for the systems listed in Table 10. For the 60, 30 and 24 Hz systems, picture rates having those values divided by 1.001 are also included. Parameter values for these systems are presented in parentheses.

TABLE  10

HDTV systems based on CIF (see Recommendation ITU-R BT.709, Part 2)

| System | Capture (Hz) | Transport |
| --- | --- | --- |
| 60/P | 60 progressive | Progressive |
| 30/P | 30 progressive | Progressive |
| 30/PsF | 30 progressive | Segmented frame |
| 60/I | 30 interlace | Interlace |
| 50/P | 50 progressive | Progressive |
| 25/P | 25 progressive | Progressive |
| 25/PsF | 25 progressive | Segmented frame |
| 50/I | 25 interlace | Interlace |
| 24/P | 24 progressive | Progressive |
| 24/PsF | 24 progressive | Segmented frame |

# 1	Digital representation

## 1.1	Coding characteristics

The signals to be digitized should comply with the characteristics described in Recommendation ITU-R BT.709, Part 2.

## 1.2	Construction of digital signals

Digital representation of R, G, B, Y, CR and CB may be obtained using the following relationship. Further study is required in terms of conversion between the data obtained with 8-bit and 10-bit quantization.

Rd		[ Int { ( 219  D )  ER'  ( 16  D ) + 0.5 } ] / D

Gd		[ Int { ( 219  D )  EG'  ( 16  D ) + 0.5 } ] / D

Bd		[ Int { ( 219  D )  EB'  ( 16  D ) + 0.5 } ] / D

Yd		[ Int { ( 219  D )  EY'  ( 16  D ) + 0.5 } ] / D

CBd		[ Int { ( 224  D )  ECB'  ( 128  D ) + 0.5 } ] / D

CRd		[ Int { ( 224  D )  ECR'  ( 128  D ) + 0.5 } ] / D

where D takes either the value 1 or 4, corresponding to 8-bit or 10-bit quantization respectively; EG', EB', ER' and EY' denote analogue R, G, B and luminance signals that have been normalized to span the range 0.0 to 1.0, while ECR' and ECB' denote analogue colour-difference signals that have been normalized to span the range – 0.5 to 0.5.

# 2	Digital interface

The interface provides a unidirectional interconnection between a single source and a single destination. The data signals are in the form of binary information and are coded accordingly:

–	video data (8-bit or 10-bit words);

–	timing reference and identification codes (8-bit or 10-bit words);

–	ancillary data (see Recommendation ITU-R BT.1364).

## 2.1	Video data

Y, CB and CR signals are handled as 20-bit words by time-multiplexing CB and CR components. Each 20-bit word corresponds to a colour-difference sample and a luminance sample. The multiplex is organized as:

(CB1 Y1) (CR1 Y2) (CB3 Y3) (CR3 Y4) ...

where Yi indicates the i-th active sample of a line, while CBi and CRi indicate the colour-difference samples of CB and CR components co-sited with the Yi sample. Note that the index i on colour-difference samples takes only odd values due to the half-rate sampling of the colour-difference signals.

The data words corresponding to digital levels 0.00 through 0.75 and 255.00 through 255.75 are reserved for data identification purposes and must not appear as video data.

R, G, B signals are handled as 30-bit words in addition to the above 20-bit words for Y, CB, CR signals.

TABLE  11

Digital coding parameters

| Item | Parameter | System |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 1 | Coded signals Y, CB, CR or R, G, B | These signals are obtained from gamma pre-corrected signals, namely , , or , , . Also see Recommendation ITU-R BT.709, Part 2 |  |  |  |  |  |  |  |  |  |
| 2 | Sampling lattice – R, G, B, Y | Orthogonal, line and picture repetitive |  |  |  |  |  |  |  |  |  |
| 3 | Sampling lattice – CB, CR | Orthogonal, line and picture repetitive, co-sited with each other and with alternate (1) Y samples |  |  |  |  |  |  |  |  |  |
| 4 | Number of active lines | 1080 |  |  |  |  |  |  |  |  |  |
| 5 | Sampling frequency (2) (MHz) – R, G, B, Y | 148.5 (148.5/1.001) | 74.25 (74.25/1.001) |  |  | 148.5 | 74.25 |  |  | 74.25 (74.25/1.001) |  |
|  | – CB, CR (3) | 74.25 (74.25/1.001) | 37.125 (37.125/1.001) |  |  | 74.25 | 37.125 |  |  | 37.125 (37.125/1.001) |  |
| 6 | Number of samples/line – R, G, B, Y – CB, CR | 2 200 1100 |  |  |  | 2 640 1320 |  |  |  | 2750 1375 |  |
| 7 | Number of active samples/line – R, G, B, Y – CB, CR | 1920 960 |  |  |  |  |  |  |  |  |  |
| 8 | Position of the first active Y, CB, CR sampling instants with respect to the analogue sync timing eference OH(4) (see Fig. 6) | 192 T |  |  |  |  |  |  |  |  |  |

TABLE  11  (end )

| Point | Parameter | System |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| 9 | Coding format | Uniformly quantized PCM for each of the video component signals 8- or 10-bit/sample |  |  |  |  |  |  |  |  |  |
| 10 | Quantization level assignment (5) – Video data – Timing reference | 1.00 through 254.75 0.00 and 255.75 (6) |  |  |  |  |  |  |  |  |  |
| 11 | Quantization levels (7) – Black level R, G, B, Y – Achromatic level CB, CR – Nominal peak – R, G, B, Y – CB, CR | 16.00 128.00 235.00 16.00 and 240.00 |  |  |  |  |  |  |  |  |  |
| 12 | Filter characteristics | See Recommendation ITU-R BT.709 |  |  |  |  |  |  |  |  |  |
| (1) The first active colour-difference samples are co-sited with the first active Y sample. (2) The sampling clock must be locked to the line frequency. The tolerance on frequency is ±0.001%. (3) CB, CR sampling frequency is half of luminance sampling frequency. (4) T denotes the duration of the luminance sampling clock or the reciprocal of the luminance sampling frequency. (5) To reduce confusion when using 8-bit and 10-bit systems together, the two LSBs of the 10-bit system are read as two fractional bits. The quantization scale in an 8-bit system ranges from 0 to 255 in steps of 1, and in a 10-bit system from 0.00 to 255.75 in steps of 0.25. When 8-bit words are treated in 10-bit system, two LSBs of zeros are to be appended to the 8-bit words. (6) In the case of 8-bit system, eight MSBs are used. (7) These levels refer to precise nominal video levels. Signal processing may occasionally cause the signal level to deviate outside these ranges. |  |  |  |  |  |  |  |  |  |  |  |

## 2.2	Video timing relationship with analogue waveform

The digital line occupies m clock periods. It begins at f clock periods prior to the reference transition (OH) of the analogue synchronizing signal in the corresponding line. The digital active line begins at g clock periods after the reference transition (OH). The values for m, f and g are listed in Table 12. See Fig. 6 and Table 12 for detailed timing relationships in the line interval.

For interlace and segmented frame systems, the start of digital field/segment is fixed by the position specified for the start of the digital line. See Fig. 7a) and Table 13a) for detailed relationships in the field/segment interval.

For progressive systems, the start of the digital frame is fixed by the position specified for the start of the digital line. See Fig. 7b) and Table 13b) for detailed relationships in the frame interval.

## 2.3	Video timing reference codes SAV and EAV

There are two timing reference codes, one at the beginning of each video data block SAV and the other at the end of each video data block EAV. These codes are contiguous with the video data, and continue during the field/frame/segment blanking interval, as shown in Fig. 7.

TABLE  12

Line interval timing specifications

| Symbol | Parameter | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
|  | Number of active Y samples per line | 1920 |  |  |  |  |  |  |  |  |  |
|  | Luminance sampling frequency (MHz) | 148.5 (148.5/ 1.001) | 74.25 (74.25/1.001) |  |  | 148.5 | 74.25 |  |  | 74.25 (74.25/1.001) |  |
| a | Analogue line blanking (T) | +12 280 –0 |  |  |  | +12 280 –0 |  |  |  | +12 280 –0 |  |
| b | Analogue active line (T) | +0 1920 –12 |  |  |  |  |  |  |  |  |  |
| c | Analogue active line (T) | 2200 |  |  |  | 2640 |  |  |  | 2750 |  |
| d | Duration between end of analogue active video and start of EAV (T) | 0-6 |  |  |  |  |  |  |  |  |  |
| e | Duration between end of SAV and start of analogue active video (T) | 0-6 |  |  |  |  |  |  |  |  |  |
| f | Duration between start of EAV and analogue timing reference OH (T) | 88 |  |  |  | 528 |  |  |  | 638 |  |
| g | Duration between analogue timing reference OH and end of SAV (T) | 192 |  |  |  |  |  |  |  |  |  |
| h | Video data block (T) | 1928 |  |  |  |  |  |  |  |  |  |
| i | Duration of EAV (T) | 4 |  |  |  |  |  |  |  |  |  |
| j | Duration of SAV (T) | 4 |  |  |  |  |  |  |  |  |  |
| k | Digital line blanking (T) | 280 |  |  |  | 720 |  |  |  | 830 |  |
| l | Digital active line (T) | 1920 |  |  |  |  |  |  |  |  |  |
| m | Digital line (T) | 2200 |  |  |  | 2640 |  |  |  | 2750 |  |
| NOTE 1 – The parameter values for analogue specifications expressed by the symbols a, b and c indicate the nominal values. NOTE 2 – T denotes the duration of the luminance clock or the reciprocal of the luminance sampling frequency. |  |  |  |  |  |  |  |  |  |  |  |

Each code consists of a four-word sequence. The bit assignment of the word is given in Table 14. The first three words are fixed preamble and the fourth word carries the information that defines field identification (F), field/frame blanking period (V), and line blanking period (H). In a 8-bit implementation bits Nos. 9 to 2 inclusive are used.

The bits F and V change state synchronously with EAV at the beginning of the digital line.

The value of protection bits, P0 to P3, depends on the F, V and H as shown in Table 15. The arrangement permits one-bit errors to be corrected and two-bit errors to be detected at the receiver, but only in the 8 MSBs, as shown in Table 16.

TABLE  13

a)	Field/segment interval timing specifications for interlace 
		and segmented frame scanning systems

| Symbol | Definition | Digital line number |
| --- | --- | --- |
|  | Number of active lines | 1080 |
| L1 | First line of field/segment No. 1 | 1 |
| L2 | Last line of digital field/segment blanking No. 1 | 20 |
| L3 | First line of field/segment No. 1 active video | 21 |
| L4 | Last line of field/segment No. 1 active video | 560 |
| L5 | First line of digital field/segment blanking No. 2 | 561 |
| L6 | Last line of field/segment No. 1 | 563 |
| L7 | First line of field/segment No. 2 | 564 |
| L8 | Last line of digital field/segment blanking No. 2 | 583 |
| L9 | First line of field/segment No. 2 active video | 584 |
| L10 | Last line of field/segment No. 2 active video | 1123 |
| L11 | First line of digital field/segment blanking No. 1 | 1124 |
| L12 | Last line of field/segment No. 2 | 1125 |
| NOTE 1 – Digital field/segment blanking No. 1 denotes the field/segment blanking period that is prior to the active video of field/segment No. 1, and digital field/segment blanking No. 2 denotes that prior to the active video of field/segment No. 2. |  |  |

b)	Frame interval timing specifications for progressive systems

| Symbol | Definition | Digital line number |
| --- | --- | --- |
|  | Number of active lines | 1080 |
| L1 | First line of frame | 1 |
| L2 | Last line of digital frame blanking | 41 |
| L3 | First line of active video | 42 |
| L4 | Last line of active video | 1121 |
| L5 | First line of digital frame blanking | 1122 |
| L6 | Last line of frame | 1125 |

TABLE  14

Bit assignment for video timing reference codes

| Word | Bit number |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9 (MSB) |  | 8 | 7 | 6 |  | 5 | 4 | 3 | 2 |  | 1 | 0 (LSB) |
| First | 1 |  | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 |  | 1 | 1 |
| Second | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Third | 0 |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |  | 0 | 0 |
| Fourth | 1 |  | F | V | H |  | P3 | P2 | P1 | P0 |  | 0 | 0 |
| Interlace and segmented frame system |  | F = 1 during field/segment No. 2 = 0 during field/segment No. 1 |  |  |  | V = 1 during field/segment blanking = 0 elsewhere |  |  |  |  | H = 1 in EAV = 0 in SAV |  |  |
| Progressive system |  | F = 0 |  |  |  | V = 1 during frame blanking = 0 elsewhere |  |  |  |  | H = 1 in EAV = 0 in SAV |  |  |
| NOTE 1 – P0, P1, P2, P3 in the fourth word are the protection bits (see Table 15). |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  15

Protection bits for SAV and EAV

|  | SAV/EAV bit status |  |  | Protection bits |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 9 (fixed) | 8 (F) | 7 (V) | 6 (H) | 5 (P3) | 4 (P2) | 3 (P1) | 2 (P0) | 1 (fixed) | 0 (fixed) |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |

TABLE  16

Error corrections using protection bits (P3-P0)

| Received bits 5-2 | Received bits 8-6 for F, V and H |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| for P3-P0 | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| 0000 | 000 | 000 | 000 | – | 000 | – | – | 111 |
| 0001 | 000 | – | – | 111 | – | 111 | 111 | 111 |
| 0010 | 000 | – | – | 011 | – | 101 | – | – |
| 0011 | – | – | 010 | – | 100 | – | – | 111 |
| 0100 | 000 | – | – | 011 | – | – | 110 | – |
| 0101 | – | 001 | – | – | 100 | – | – | 111 |
| 0110 | – | 011 | 011 | 011 | 100 | – | – | 011 |
| 0111 | 100 | – | – | 011 | 100 | 100 | 100 | – |
| 1000 | 000 | – | – | – | – | 101 | 110 | – |
| 1001 | – | 001 | 010 | – | – | – | – | 111 |
| 1010 | – | 101 | 010 | – | 101 | 101 | – | 101 |
| 1011 | 010 | – | 010 | 010 | – | 101 | 010 | – |
| 1100 | – | 001 | 110 | – | 110 | – | 110 | 110 |
| 1101 | 001 | 001 | – | 001 | – | 001 | 110 | – |
| 1110 | – | – | – | 011 | – | 101 | 110 | – |
| 1111 | – | 001 | 010 | – | 100 | – | – | – |
| NOTE 1 – The error correction applied provides a DEDSEC (double error detection – single error correction) function. The received bits denoted by “–” in the table, if detected, indicate that an error has occurred but cannot be corrected. |  |  |  |  |  |  |  |  |

## 2.4	Ancillary data

Ancillary data may optionally be included in the blanking intervals of a digital interface according to this Recommendation. The ancillary signals should comply with the general rules of Recommendation ITU-R BT.1364.

The horizontal blanking interval between the end of EAV and the start of SAV may be employed to convey ancillary data packets.

Ancillary data packets may be conveyed in the vertical blanking interval between the end of SAV and the start of EAV as follows:

	in a progressive system during lines 7 through 41 inclusive;

	in an interlaced system during lines 7 through 20 inclusive and lines 569 through 583 inclusive;

	on any line that is outside the vertical extent of the picture as noted above and that is not employed to convey vertical blanking interval signals that can be represented in the analogue domain through direct (D/A) conversion (such as digital vertical interval time code (D-VITC)).

## 2.5	Data words during blanking

The data words occurring during digital blanking intervals that are not used for the timing reference codes (SAV and EAV), or for ancillary data (ANC) are filled with words corresponding to the following blanking levels, appropriately placed in the multiplexed data:

16.00	for Y, R, G, B signals

128.00	for CB /CR (time-multiplexed colour-difference signal).

# 3	Bit-parallel interface

The bits of the digital code words which describe the video signal are transmitted in parallel by means of 20 or 30 shielded conductor pairs. The 20 conductor pairs are used for the transmission of the signal set consisting of luminance Y and time-multiplexed colour-difference CB /CR components. The 30 conductor pairs are used for the transmission of R, G, B signals or Y, CB /CR components with an additional data stream (auxiliary channel). An additional shielded conductor pair carries the synchronous clock at 148.5 MHz (148.5/1.001 MHz) for 60/P and 50/P, and 74.25 MHz (74.25/1.001 MHz) for the other systems.

Data signals are transmitted in NRZ form in real time (unbuffered).

## 3.1	Clock signal and clock-to-data timing relationship

The transmitted clock signal is a square wave, of which positive transitions occur midway between the data transitions as shown in Fig. 8 and Table 17.

TABLE  17

Clock signal specifications

| Parameter | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| Sampling frequency for Y, R, G, B signals (MHz) | 148.5 (148.5/1.001) | 74.25 (74.25/1.001) |  |  | 148.5 | 74.25 |  |  | 74.25 (74.25/1.001) |  |
| Clock period, Tck | 1/(2200 fH) |  |  |  | 1/(2640 fH) |  |  |  | 1/(2750 fH) |  |
| Nominal value (ns) | 6,734 (6,741) | 13.468 (13.481) |  |  | 6.734 | 13.468 |  |  | 13.468 (13.481) |  |
| Clock pulse width, t Tolerance | 0.5 Tck 0.11 Tck |  |  |  |  |  |  |  |  |  |
| Clock jitter | Within 0.04 Tck |  |  |  |  |  |  |  |  |  |
|  | from the average time of transition over one field/segment in interlace and segmented frame systems, and over one frame in progressive systems |  |  |  |  |  |  |  |  |  |
| Data timing, Td Tolerance | 0.5 Tck 0.075 Tck |  |  |  |  |  |  |  |  |  |
| NOTE 1 – fH denotes the line frequency. NOTE 2 – Values are specified at the sending end (source). |  |  |  |  |  |  |  |  |  |  |

## 3.2	Electrical characteristics of the interface

The interface employs 21 line drivers and line receivers, in the case of the transmission of Y and CB /CR components. Each line driver has a balanced output and the corresponding line receive has a balanced input. The interface employs 31 line drivers and line receivers, in the case of R, G and B components or Y, CB /CR with an additional data stream (auxiliary channel).

Although the use of ECL technology is not mandatory, the line driver and receiver must be ECL 10 k compatible for the systems using the synchronous clock at 74.25 MHz (74.25/1.001 MHz), i.e., they must permit the use of ECL for either drivers or receivers.

The receiver must sense correctly the data when a random signal produces conditions represented by the eye diagram of Fig. 10.

TABLE  18

Line driver characteristics

| Item | Parameter | Value |
| --- | --- | --- |
| 1 | Output impedance () | 110 maximum |
| 2 | Common mode voltage(1) (V) | –1.29  15% |
| 3 | Signal amplitude(2) (V) | 0.6 to 2.0 p-p |
| 4 | Rise and fall times(3) |  0.15 Tck |
| 5 | Difference between rise and fall times |  0.075 Tck |
| NOTE 1 – Tck denotes the clock period (see Table 17). (1) Measured relative to ground. (2) Measured across a resistive load having the nominal impedance of the assumed cables, that is 110 . (3) Measured between the 20% and 80% points across a resistive load having the nominal impedance of the assumed cable. |  |  |

TABLE  19

Line receiver characteristics

| Item | Parameter | Value |
| --- | --- | --- |
| 1 | Input impedance () | 110  10 |
| 2 | Maximum input signal voltage (V) | 2.0 p-p |
| 3 | Minimum input signal voltage (mV) | 185 p-p |
| 4 | Maximum common mode voltage(1) (V) | 0.3 |
| 5 | Differential delay Tmin(2) | 0.3 Tck |
| NOTE 1 – Tck denotes the clock period (see Table 17). (1) Comprising interference in the range DC to line frequency (fH). (2) Data must be correctly sensed when the differential delay between the received clock and data is within this range (see Fig. 10). |  |  |

## 3.3	Mechanical characteristics (see Note 1)

### 3.3.1	Connector

The interface uses a multi-contact connector. Connectors are locked by two screws on the cable connectors and two threaded bolts on the equipment. Cable connectors employ pin contacts and equipment connectors employ socket contacts. Shielding of the connectors and cables is mandatory.

A 93-contact connector is used. Contact assignments are indicated in Tables 20 and 21. The mechanical specifications for the connectors are shown in Figs. 11, 12 and 13.

NOTE 1 – For new designs, bit-serial interface described in § 4 is preferred.

TABLE  20

Connector contact assignment

| Con- tact | Signal line | Con- tact | Signal line | Con- tact | Signal line | Con- tact | Signal line | Con- tact | Signal line | Con- tact | Signal line |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Clock A | 17 | GND | 33 | Clock B |  |  |  |  |  |  |
| 2 | XD 9A | 18 | GND | 34 | XD 9B | 49 | YD 4A | 64 | GND | 79 | YD 4B |
| 3 | XD 8A | 19 | GND | 35 | XD 8B | 50 | YD 3A | 65 | GND | 80 | YD 3B |
| 4 | XD 7A | 20 | GND | 36 | XD 7B | 51 | YD 2A | 66 | GND | 81 | YD 2B |
| 5 | XD 6A | 21 | GND | 37 | XD 6B | 52 | YD 1A | 67 | GND | 82 | YD 1B |
| 6 | XD 5A | 22 | GND | 38 | XD 5B | 53 | YD 0A | 68 | GND | 83 | YD 0B |
| 7 | XD 4A | 23 | GND | 39 | XD 4B | 54 | ZD 9A | 69 | GND | 84 | ZD 9B |
| 8 | XD 3A | 24 | GND | 40 | XD 3B | 55 | ZD 8A | 70 | GND | 85 | ZD 8B |
| 9 | XD 2A | 25 | GND | 41 | XD 2B | 56 | ZD 7A | 71 | GND | 86 | ZD 7B |
| 10 | XD 1A | 26 | GND | 42 | XD 1B | 57 | ZD 6A | 72 | GND | 87 | ZD 6B |
| 11 | XD 0A | 27 | GND | 43 | XD 0B | 58 | ZD 5A | 73 | GND | 88 | ZD 5B |
| 12 | YD 9A | 28 | GND | 44 | YD 9B | 59 | ZD 4A | 74 | GND | 89 | ZD 4B |
| 13 | YD 8A | 29 | GND | 45 | YD 8B | 60 | ZD 3A | 75 | GND | 90 | ZD 3B |
| 14 | YD 7A | 30 | GND | 46 | YD 7B | 61 | ZD 2A | 76 | GND | 91 | ZD 2B |
| 15 | YD 6A | 31 | GND | 47 | YD 6B | 62 | ZD 1A | 77 | GND | 92 | ZD 1B |
| 16 | YD 5A | 32 | GND | 48 | YD 5B | 63 | ZD 0A | 78 | GND | 93 | ZD 0B |
| NOTE 1 – XD 9-XD 0, YD 9-YD 0, and ZD 9-ZD 0 represent each bit of the component signals. The suffix 9 to 0 indicates the bit number (bit 9 denotes MSB). A and B correspond to the terminals A and B of Fig. 9, respectively. The relationship between XD, YD, ZD and component signals are specified in Table 21. NOTE 2 – The shield of each pair uses the ground contact (GND) located between A and B contacts for the signal, e.g., contact No. 17 is used for the shield of the clock signal. The overall shield of the cable is electrically connected to connector hood, which is grounded to the frame of the equipment. |  |  |  |  |  |  |  |  |  |  |  |

### 3.3.2	Interconnecting cable

Two types of multi-channel cable, either 21 or 31 channels, can be used in accordance with the transmission signal set (see Table 21). The cable consists of twisted pairs with an individual shield for each pair. It also contains an overall shield. The nominal characteristic impedance of each twisted pair is 110 W. The cable shall possess the characteristics that satisfy the conditions of the eye diagram shown in Fig. 10 up to a maximum cable length of 20 m for the system using the synchronous clock at 74.25 MHz (74.25/1.001 MHz), and 14 m for the systems using the synchronous clock at 148.5 MHz (148.5/1.001 MHz).

TABLE  21

Transmission signal set and signal line assignment

| Transmission signal set | Component | Signal line assignment |  | Cable |
| --- | --- | --- | --- | --- |
|  |  | 10-bit system | 8-bit system |  |
| Y, CR/CB | Y | XD 9-XD 0 | XD 9-XD 2 |  |
|  | CR/CB | ZD 9-ZD 0 | ZD 9-ZD 2 | 21 pairs |
| Y, CR/CB with auxiliary channel | Y | XD 9-XD 0 | XD 9-XD 2 |  |
|  | CR/CB | ZD 9-ZD 0 | ZD 9-ZD 2 |  |
|  | Auxiliary channel | YD 9-YD 0 | YD 9-YD 2 | 31 pairs |
|  | G | XD 9-XD 0 | XD 9-XD 2 |  |
| R, G, B | B | YD 9-YD 0 | YD 9-YD 2 |  |
|  | R | ZD 9-ZD 0 | ZD 9-ZD 2 |  |

# 4	Bit-serial interface

Specifications for 60/P and 50/P are under study.

## 4.1	Data format

The bit-serial data consists of video data, video timing reference codes, line number data, error detection codes, ancillary data and blanking data. Each data has a word-length of 10 bits, and is represented as parallel data before serialization. Two parallel streams (i.e. luminance data Y and colour-difference data CB /CR) are multiplexed and serialized in accordance with § 4.2.

### 4.1.1	Video data

The video data should be 10-bit words representing Y, CB /CR of the video systems defined in § 1.

### 4.1.2	Video timing reference codes

The video timing reference codes, SAV and EAV have the same format as that defined in § 2.

### 4.1.3	Line number data

The line number data is composed of two words indicating the line number. The bit assignment of the line number data is shown in Table 22. The line number data should be located immediately after EAV.

TABLE  22

Bit assignment of the line number data

|  | b9 | b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Word | (MSB) |  |  |  |  |  |  |  |  | (LSB) |
| LN0 | Not b8 | L6 | L5 | L4 | L3 | L2 | L1 | L0 | R | R |
| LN1 | Not b8 | R | R | R | L10 | L9 | L8 | L7 | R | R |
| L0 (LSB)-L10 (MSB): line number in binary code. R: reserved (set to zero). |  |  |  |  |  |  |  |  |  |  |

### 4.1.4	Error detection codes

The error detection codes, cyclic redundancy check codes (CRCC), which are used to detect errors in active digital line, EAV and line number data, consist of two words and are determined by the following polynomial generator equation:

EDC(x)    x18    x5    x4    1

Initial value of the codes is set to zero. The calculation starts at the first word of the digital active line and ends at the final word of the line number data. Two error detection codes are calculated, one for luminance data (YCR) and one for colour-difference data (CCR). The bit assignment of the error detection codes is shown in Table 23. The error detection codes should be located immediately after the line number data.

TABLE  23

Bit assignment for error detection codes

|  | b9 | b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Word | (MSB) |  |  |  |  |  |  |  |  | (LSB) |
| YCR0 | Not b8 | CRCC8 | CRCC7 | CRCC6 | CRCC5 | CRCC4 | CRCC3 | CRCC2 | CRCC1 | CRCC0 |
| YCR1 | Not b8 | CRCC17 | CRCC16 | CRCC15 | CRCC14 | CRCC13 | CRCC12 | CRCC11 | CRCC10 | CRCC9 |
| CCR0 | Not b8 | CRCC8 | CRCC7 | CRCC6 | CRCC5 | CRCC4 | CRCC3 | CRCC2 | CRCC1 | CRCC0 |
| CCR1 | Not b8 | CRCC17 | CRCC16 | CRCC15 | CRCC14 | CRCC13 | CRCC12 | CRCC11 | CRCC10 | CRCC9 |
| NOTE 1 – CRC0 is the MSB of error detection codes. |  |  |  |  |  |  |  |  |  |  |

### 4.1.5	Ancillary data

The ancillary data should comply with general rules of Recommendation ITU-R BT.1364.

### 4.1.6	Blanking data

The blanking data words during digital blanking intervals that are not used for SAV, EAV, the line number data, the error detection codes and the ancillary data, should be filled with the 10-bit words corresponding to the following quantization levels:

–	16.00 for Y data

–	128.00 for CB /CR data.

## 4.2	Transmission format

The two parallel data streams are transmitted over a single channel in bit-serial form after word-multiplexing, parallel-to-serial conversion and scrambling.

### 4.2.1	Word-multiplexing

The two parallel streams should be multiplexed word by word into a single 10-bit parallel stream in the order of CB, Y, CR, Y, CB, Y, CR, Y ... (See Fig. 14 and Table 25).

### 4.2.2	Serializing

The LSB of each 10-bit word in the word-multiplexed parallel stream should be transmitted first in the bit-serial format.

### 4.2.3	Channel coding

The channel coding scheme should be scrambled NRZ inverted (NRZI). The serialized bit stream should be scrambled using the following generator polynomial equation:

G(x)    (x9  +  x4  +  1)  (x  +  1)

The input signal to the scrambler shall be positive logic. (The high voltage represents data 1 and the lowest voltage represents data 0.)

### 4.2.4	Serial clock

Table 24 specifies the serial clock frequencies, which are twenty times the frequency of the parallel clock (see Table 17).

TABLE  24

Serial clock frequency

| Parameter | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60/P | 30/P | 30/PsF | 60/I | 50/P | 25/P | 25/PsF | 50/I | 24/P | 24/PsF |
| Serial clock frequency (GHz) | Under study | 1.485 (1.485/1.001) |  |  | Under study | 1.485 |  |  | 1.485 (1.485/1.001) |  |

### 4.2.5	Bit-serial digital check field

Digital test signals suitable for testing cable equalization and phase locked loop (PLL) lock-in are described in Annex 1.

TABLE  25

Data stream timing specifications (see Fig. 14)

| Symbol | Parameter | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60/P(1) | 30/P | 30/PsF | 60/I |  | 50/P(1) | 25/P | 25/PsF | 50/I | 24/P |  | 24/PsF |
| T | Parallel clock period (ns) | 1000/148.5 (1001/148.5) | 1000/74.25 (1001/74.25) |  |  |  | 1000/148.5 | 1000/74.25 |  |  | 1000/74.25 (1001/74.25) |  |  |
| Ts | Multiplexed parallel data clock period | T/2 |  |  |  |  |  |  |  |  |  |  |  |
| m | Digital line in parallel data stream | 2200 |  |  |  | 2640 |  |  |  |  |  | 2750 |  |
| k | Digital line blanking in parallel data stream | 280 |  |  |  | 720 |  |  |  |  |  | 830 |  |
| n | Ancillary data or blanking data in parallel data stream | 268 |  |  |  | 708 |  |  |  |  |  | 728 |  |
| ms | Digital line in multiplexed parallel data stream | 4400 |  |  |  | 5280 |  |  |  |  |  | 5500 |  |
| ks | Digital line blanking in multiplexed parallel data stream | 560 |  |  |  | 1440 |  |  |  |  |  | 1660 |  |
| ns | Ancillary data or blanking data in multiplexed parallel data stream | 536 |  |  |  | 1416 |  |  |  |  |  | 1456 |  |
| (1) Parameter values for these systems are under study. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 4.3	Coaxial cable interfaces

The coaxial cable interfaces consist of one source and one destination in a point-to-point connection. The coaxial cable interfaces specify the characteristics of line driver (source), line receiver (destination), transmission line and connectors.

### 4.3.1	Line driver characteristics (source)

Table 26 specifies the line driver characteristics. The line driver should have an unbalanced output circuit.

TABLE  26

Line driver characteristics

| Item | Parameter | Value |
| --- | --- | --- |
| 1 | Output impedance | 75 W nominal |
| 2 | DC offset(1) | 0.0 V ± 0.5 V |
| 3 | Signal amplitude(2) | 800 mVp-p ± 10% |
| 4 | Return loss |  15 dB(3), ³ 10 dB(4) |
| 5 | Rise and fall times(5) |  270 ps (20% to 80%) |
| 6 | Difference between rise and fall time | £ 100 ps |
| 7 | Output jitter(6) | f1 = 10 Hz f3 = 100 kHz f4 = 1/10 of the clock rate A1 = 1 UI (UI: unit interval) A2 = 0.2 UI |
| (1) Defined by mid-amplitude point of the signal. (2) Measured across a 75 W resistive load connected through a 1 m coaxial cable. (3) In the frequency range of 5 MHz to fc/2. (fc: serial clock frequency) (4) In the frequency range of fc/2 to fc. (5) Determined between the 20% and 80% amplitude points and measured across a 75 W resistive load. Overshoot of the rising and falling edges of the waveform shall not exceed 10% of the amplitude. (6) 1 UI corresponds to 1/fc. Specification of jitter and jitter measurements methods shall comply with Recommendation ITU-R BT.1363 – Jitter specifications and methods for jitter measurement of bit-serial signals conforming to Recommendations ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120. Output amplitude excursions due to signals with a significant dc component occurring for a horizontal line (pathological signals) shall not exceed 50 mV above or below the average peak-peak signal envelope. (In effect, this specification defines a minimum output coupling time constant.) |  |  |

### 4.3.2	Line receiver characteristics (destination)

Table 27 specifies the line receiver characteristics. The line receiver should have an unbalanced input circuit. It must sense correctly the received data when connected to a line driver operating at the extreme voltage limits permitted by § 4.3.1, and when connected through a cable having the worst condition permitted by § 4.3.3.

TABLE  27

Line receiver characteristics

| Item | Parameter | Value |  |
| --- | --- | --- | --- |
| 1 | Input impedance | 75 W nominal |  |
| 2 | Return loss |  15 dB(1),  10 dB(2) |  |
|  |  | 2.5 Vmax | DC |
| 3 | Interfering signal |  2.5 Vp-p | Below 5 kHz |
|  |  |  100 mVp-p | 5 kHz to 27 MHz |
|  |  |  40 mVp-p | Above 27 MHz |
| 4 | Input jitter(3) | To be defined |  |
| (1) In the frequency range of 5 MHz to fc/2. (2) In the frequency range of fc/2 to fc. (3) Specification of jitter and jitter measurements methods shall comply with Recommendation ITU-R BT.1363. Values for input jitter need to be defined. Input jitter is measured with a short cable (2 m). |  |  |  |

### 4.3.3	Transmission line characteristics

Relevant specifications are given in Table 28.

TABLE  28

Transmission line characteristics

| Item | Parameter | Value |
| --- | --- | --- |
| 1 | Transmission loss(1) |  20 dB at 1/2 clock frequency |
| 2 | Return loss |  15 dB(2),  10 dB(3) |
| 3 | Impedance | 75 W nominal |
| (1) Loss characteristics of f. (2) In the frequency range of 5 MHz to fc/2. (3) In the frequency range of fc/2 to fc. |  |  |

### 4.3.4	Connector

The connector should have the mechanical characteristics conforming to the standard BNC type defined in IEC 169-8 (International Electrotechnical Committee) and its electrical characteristics should provide for a characteristic impedance of 75 W and for a usable frequency range of up to 2.4 GHz.

## 4.4	Optical fibre interfaces

Optical interfaces should use single mode optical interfaces only and should comply with general rules of Recommendation ITU-R BT.1367 – Serial digital fibre transmission system for signals conforming to Recommendations ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120.

To make use of this Recommendation the following specifications are necessary:

Rise and fall times:	 270 ps (20 to 80)

Output jitter (see Note 1):	f1	 10 Hz
f3	 100 kHz
f4	 1/10 of the clock rate
A1	 0,135 I UI
A2	 0,135 UI

Input jitter needs to be defined. Input jitter is measured with a short cable (2 m).

NOTE 1 – Specification of jitter and jitter measurements methods shall comply with Recommendation ITU-R BT.1363.

Annex 1

Bit-serial digital checkfield for use in
the HDTV digital interfaces

# 1	Scope

This Annex specifies digital test signals suitable for evaluating the low-frequency response of equipment handling HDTV serial digital video signals. Although a range of signals will produce the desired low-frequency effects, two specific signals are defined to test cable equalization and PLL lock-in, respectively. In the past, these two signals have been colloquially called “pathological signals.”

# 2	General considerations

Stressing of the automatic equalizer is accomplished by using a signal with the maximum number of ones or zeros, with infrequent single clock period pulses to the opposite level. Stressing of the  PLL is accomplished by using a signal with a maximum low-frequency content; that is, with a maximum time between level transitions.

2.1	Channel coding of the serial digital signal defined in this Recommendation utilizes scrambling and encoding into NRZI accomplished by a concatenation of the two following functions:

G1 (x)    x9    x4    1		G2 (x)   x   1

As a result of the channel coding, long runs of zeros in the G2 (x) output data can be obtained when the scrambler, G1 (x), is in a certain state at the time when the specific words arrive. That certain state will be present on a regular basis; therefore, continuous application of the specific data words will regularly produce the low-frequency effects.

2.2	Although the longest run of parallel data zeros (40 consecutive zeros) will occur during the EAV/SAV timing reference sequence (TRS) words, the frequency with which the scrambling of the TRS words coincide with the required scrambler state to permit either stressing condition is low. In the instances where this coincident occurs, the generation of the stressing condition is so time limited that equalizers and PLLs are not maximally stressed.

2.3	In the data portions of digital video signals (excluding TRS words in EAVs or SAVs, and ANC data flag words), the sample values are restricted to exclude data levels 0.00 to 0.75 and 255.00 to 255.75 (000h to 003h and 3FCh to 3FFh in 10-bit hexadecimal representation and 00.0h to 00.Ch and FF.0h to FF.Ch, in 8.2 hexadecimal notation) (see Note 1). the result of this restriction is that the longest run of zeros, at the scrambler input, is 16 (bits), occurring when a sample value of 128.00 (200h or 80.0h) is followed by a value between 1.00 (004h or 01.0h) and 1.75 (007h or 01.Ch). This situation can produce up to 26 consecutive zeros at the NRZI output, which is (also) not a maximally stressed case.

NOTE 1 – Within this Annex, the contents of digital word are expressed in both decimal and hexadecimal form. In decimal form, the eight MSBs are considered to be an integer part while the two additional bits are considered to be fractional parts. In hexadecimal form, both 10-bit hexadecimal and 8.2 hexadecimal notation are used. For example, the bit pattern 1001000101 would be expressed as 145.25, 245h or 91.4h.

2.4	Other specific data words in combination with specific scrambler states can produce a repetitive low-frequency serial output signal until the next EAV or SAV affects the scrambler state. It is these combinations of data words that form the basis of the test signals defined by this Annex.

2.5	Because of the Y/C interleaved nature of the component digital signal, it is possible to obtain nearly any permutation of word pair data values over the entire active picture area by defining a particular flat colour field in a noise-free environment. Certain of these permutations of word pair data values will produce the desired low-frequency effects.

# 3	Checkfield data

3.1	Receiver equalizer testing is accomplished by producing a serial digital signal with maximum d.c. content. Applying the sequence 192.00 (300h or C0.0h), 102.00 (198h or 66.0h) continuously to the C and Y samples (respectively) during the active line will produce a signal of 19 consecutive high (low) states followed by one low (high) state in a repetitive manner, once the scrambler attains the required starting condition. Either polarity of the signal can be realized, indicated by the level of the 19 consecutive states. By producing approximately half of a field of continuous lines containing this sequence, the required scrambler starting condition will be realized on several lines, and this will result in the generation of the desired equalizer testing condition.

3.2	Receiver PLL testing is accomplished by producing a serial digital signal with maximum low-frequency content and minimum high-frequency content (i.e., lowest frequency of level transitions). Applying the sequence 128.00 (200h or 80.0h), 68.00 (110h or 44.0h) continuously to the C and Y samples (respectively) during the active line will produce a signal of 20 consecutive high (low) states followed by 20 low (high) states in a repetitive manner, once the scrambler attains the required starting condition. By producing approximately half of a field of continuous lines containing this sequence, the required scrambler starting condition will be realized on several lines, and this will result in the generation of the desired PLL testing condition.

3.3	Because the equalizer test works by producing a serial digital signal with a bias, steps must be taken to ensure that both polarities of bias are realized. To change the polarity of the bias from one frame to the next, the sum total of all the bits in all the data words in all the lines in a video field must be odd.

To ensure that the polarity of the bias can change often, a single Y sample data word in the signal is changed from 120.00 (198h or 66.0h) to 100.00 (190h or 64.0h) (a net change of 1 data bit), once every other frame. This causes the bias polarity to alternate at a frame rate regardless of whether the original frame bit sum is even or odd. The data word in which the value substitution is made is the first Y sample in the first active picture line of every other frame. The specific word and line for each signal format is listed in Table 29 as the polarity control word.

3.4	The sequence 192.00 (300h or C0.0h), 102.00 (198h or 66.0h) and 128.00 (200h or 80.0h), 68.00 (110h or 44.0h) applied to C and Y samples results in shades of purple and gray, respectively. Reversing the C and Y ordering for each of these two sequences results in lighter and darker shades of green, respectively. Table 29 illustrates one ordering of each of the two sequences, but either ordering of the data values for each sequence is permitted by this Annex.

If the ordering described in § 3.1 is reversed, then the polarity control word described in § 3.3 is changed to 128.00 (200h or 80.0h). The polarity control word in either case is located at the first Y sample in the first active picture line in the field(s) specified in § 3.3.

# 4	Serial digital interface (SDI) checkfield

Distribution of data in the SDI checkfield is shown in Fig. 15 for the signal standards. Specific distributions of sample values are shown in Table 29. In each field, the line where the signal transitions from the equalizer test signal data pattern to the PLL test signal data pattern is specified as a range of lines, rather than as a single specific line. Although the specific line selected within the specified range is not technically significant, the transition point should be consistent from frame-to-frame and from field-to-field (in the case of interlaced signal formats).

TABLE  29

SDI checkfield sample values

|  |  | Part 1 |  | Part 2 |  |
| --- | --- | --- | --- | --- | --- |
| System |  | 1125/60/2:1 | 1250/50/2:1 | 60/I, 30/PsF, 50/I, 25/PsF, 24/PsF | 60/P, 30/P, 50/P, 25/P, 24/P |
| Number of active Y samples per line |  | 1920 |  |  |  |
| Number of active lines |  | 1035 | 1152 | 1080 |  |
|  | First line | 41 (field 1) | 45 (field 1) | 21 (field/segment 1) | 42 |
|  |  | 603 (field 2) | 670 (field 2) | 584 (field/segment 2) |  |
|  | Last line (range) | 295-302 (field 1) | 329-335 (field 1) | 287-293 (field/segment 1) | 578-585 |
|  |  | 858-865 (field 2) | 954-960 (field 2) | 850-856 (field/segment 2) |  |
| Equalizer test signal | Data values(1) | Samples |  |  |  |
|  | 192.00 CB | 0 ... 3836 |  |  |  |
|  | 102.00 Y | 1 ... 3837 |  |  |  |
|  | 192.00 CR | 2 ... 3838 |  |  |  |
|  | 102.00 Y | 3 ... 3839 |  |  |  |
|  | Polarity control word | (Every other frame) |  |  |  |
|  | Data value(1),(2) 100.00 Y | Line 41 Sample 1 | Line 45 Sample 1 | Line 21 Sample 1 | Line 42 Sample 1 |
|  | First line (range)(3) | 296-303 (field 1) | 330-336 (field 1) | 288-294 (field/segment 1) | 579-586 |
|  |  | 859-866 (field 2) | 955-961 (field 2) | 851-857 (field/segment 2) |  |
|  | Last line | 557 (field 1) | 620 (field 1) | 560 (field/segment 1) | 1121 |
| PLL |  | 1120 (field 2) | 1245 (field 2) | 1123 (field/segment 2) |  |
| test signal | Data values(1) | Samples |  |  |  |
|  | 128,00 CB | 0 ... 3836 |  |  |  |
|  | 68,00 Y | 1 ... 3837 |  |  |  |
|  | 128,00 CR | 2 ... 3838 |  |  |  |
|  | 68,00 Y | 3 ... 3839 |  |  |  |
| (1) The ordering of data values for each of the pairs of sample values may be reversed. If the ordering of the samples is reversed from the ordering in this Table, then the polarity control word value is (128.00 Y ) (see § 3.4). (2) The polarity change word is a substitution of the first active picture area Y sample, made in the first active picture line of every other frame (see § 3.3). (3) A range of line numbers for transitioning between the two test patterns is provided. The transition point within these ranges must be consistent across all fields (see § 4). |  |  |  |  |  |