RECOMMENDATION  ITU-R  BT.1122*

USER  REQUIREMENTS  FOR  EMISSION  AND  SECONDARY  DISTRIBUTION
SYSTEMS  FOR SDTV,  HDTV  AND  HIERARCHICAL  CODING  SCHEMES

(Question ITU-R 209/11)

(1994)

Rec. ITU-R BT.1122

The ITU Radiocommunication Assembly,

considering

a)	that Recommendations ITU-R BT.601 and ITU-R BT.709 define the parameters of Y, CB, CR-based digital luminance and colour difference signals;

b)	that signals conforming to that standard are required to be transmitted through digital secondary distribution networks or emission systems (terrestrial, satellite, cable, etc.);

c)	that coding algorithms have been devised and standards established or proposed to enable such transmission to be effected using bit rate reduction techniques;

d)	that prototype codec equipment using these algorithms is being developed and needs to be assessed;

e)	that general advice on methods of assessment is contained within ITU-R texts, and that, in particular, subjective evaluation methods are defined in Recommendations ITU-R BT.500 and ITU-R BT.710;

f)	that such assessment will need to take account of basic picture quality and the failure characteristic in the presence of errors on the transmission and emission link;

g)	that both the design of codecs and their assessment will need to take account of user requirements;

h)	that in order to be complete, the user requirements should specify the test procedures and test material that should be used to check that the requirements are being met;

j)	that many organizations are requesting an early establishment of user requirements,

recommends

1.	that the following user requirements should govern the specification design and testing of systems for the secondary distribution and emission of Y, CB, CR-based SDTV, HDTV and hierarchical television signals.

TABLE  1

User requirements for secondary distribution and emission

| User requirements | Specification |
| --- | --- |
| Functional requirements |  |
| Input television signal format | - – SDTV: 4:2:2 of Rec. ITU-R BT.601 |
|  | - – HDTV: studio standard pictures of Rec. ITU-R BT.709 |
| - Input audio signal format | - – to be specified |
| - Number of audio channels | - – to be specified |
| - Ancillary data | - – to be specified |
| - Access control | - – to be specified |
| - HDTV/SDTV compatibility | - – not required for some HDTV applications |
| - Scalability | - – to be specified |
| - Performance requirements |  |
| - Requirements assumed for a complete broadcast chain |  |
| - Codecs in cascade | - – cascade of one contribution codec, one primary distribution codec and one secondary distribution codec |
| - Basic quality | - – better than 18% of the DSCQS scale(1) for the following test material: ------ (to be specified) |
| - Change in overall delay after major disturbance | - – less than 20 ms |
| - Total delay from origination to home | - – to be specified |
| - Requirements for secondary distribution |  |
| - Good reception condition | - – £ 12% of the DSCQS scale (1) - – for the following test material: ------ (to be specified) |
| - Poor reception condition (2) | - – £ 36% of the DSCQS scale (1) for the same test material and viewing conditions |
| - Lower resolution display (3) | - – £ 12% of the DSCQS scale (1) for the following test material (... to be specified) and reference picture down-converted |
| - Basic audio quality | - – to be specified |
| - Vision failure characteristics | - – to be specified |
| - Image recovery time | - – 500 ms after a break of 50 ms |
| - Audio recovery time | - – to be specified |
| - Vision/audio failure characteristics | - – vision failure first |
| - Audio video relative delay | - – less than ± 2 ms |
| - Design requirements |  |
| - Hardware complexity | - – coder/decoder asymmetry to be favoured, i.e., a complex coder but simple receiver |
| - DSCQS: double stimulus continuous quality scale - DSIS: double stimulus impairment scale - (1) Performance requirements – all quality assessment ratings in this section are carried out using the procedures given in Recommendations ITU-R BT.500 and ITU-R BT.710 using the subjective assessment methods indicated. (2) Picture quality under rugged receiving conditions – under difficult receiving conditions, in some systems, a lower resolution picture is extracted and up-converted for display. For this mode of operation, quality assessment should use the higher resolution picture as reference. (3) Display at lower resolution – this condition applies when the receiver is capable of displaying only at lower resolution. For this mode of operation, quality assessment should use a down-converted version of the higher resolution picture as reference. |  |

_________________

* 	This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 9.