RECOMMENDATION  ITU-R  BT.1206

SPECTRUM  SHAPING  LIMITS  FOR  DIGITAL  TERRESTRIAL
TELEVISION  BROADCASTING

(Question ITU-R 121/11)

(1995)

Rec. ITU-R BT.1206

The ITU Radiocommunication Assembly,

considering

a)	that digital terrestrial television broadcasting will share frequency bands with analogue television;

b)	that for efficient planning for terrestrial broadcasting the spectrum limits for digital television should be defined for maximum compatibility,

recommends

1	that the spectrum characteristics for digital terrestrial transmission should conform to the limits defined in:

–	Annex 1 for 6 MHz channels;

–	Annex 2 for 7 MHz channels;

–	Annex 3 for 8 MHz channels.

ANNEX  1

Spectrum shaping limits for digital terrestrial television 
systems using 6 MHz channels

(Under study)

ANNEX  2

Spectrum shaping limits for digital terrestrial
television systems using 7 MHz channels

# 1	Types of 7 MHz system covered

The spectrum shaping limits described in this Annex are applicable to 7 MHz multicarrier orthogonal frequency division multiplex (OFDM) systems, irrespective of the number of carriers employed.

# 2	Sampling the transmitter output

To examine the spectrum, the output port of the transmitter (including any RF channel-defining filters) is connected to a spectrum analyser via an attenuator, or to an artificial load with some means of monitoring the emissions with a spectrum analyser. A spectrum analyser with variable persistence or digital storage is used, and its controls are adjusted as shown in Table 1, for OFDM systems.

TABLE  1

Spectrum analyser settings for OFDM systems

| RF centre frequency | Centre frequency of standard TV channel |
| --- | --- |
| Amplitude scale (dB/division) | 10 |
| Resolution bandwidth (kHz) | 10 |
| Total span (MHz) | 10 |
| Total sweep time (ms) | 300 |
| Video filter (kHz) | 10 |

# 3	Setting the spectrum analyser reference level

For multicarrier OFDM systems the spectrum analyser is adjusted so that the maximum level displayed corresponds to the 0 dB reference line. For comparison purposes, the signal power displayed on the spectrum analyser, corresponding to the reference level, can be calculated from:

Reference level  10 log10 P(av) · P(meas) /B(act)               dBW

where:

P(av) :	true average (heating) power (W) of OFDM signal measured in full system bandwidth

B(meas) :	equivalent noise bandwidth of spectrum analyser (use of spectrum analyser resolution bandwidth yields results with an accuracy of approximately  2dB)

B(act) :	total active bandwidth of the OFDM signal spectrum.

The recorded spectral plot is compared with Fig. 1 to ensure that the spectrum is contained entirely within the mask.

FIGURE 1...[D01] = 3 CM

ANNEX  3

Spectrum shaping limits for digital terrestrial
television systems using 8 MHz channels

# 1	Types of 8 MHz systems covered

The spectrum shaping limits described in this Annex are applicable to 8 MHz multicarrier OFDM systems, irrespective of the number of carriers employed.

# 2	Sampling the transmitter output

The transmitter output is sampled as described in § 2 of Annex 2. The spectrum analyser controls are adjusted as shown in Table 1.

# 3	Setting the spectrum analyser reference level

The spectrum analyser reference level is set as described for multicarrier OFDM systems in § 3 of Annex 2.

The recorded spectral plot is compared with Fig. 2 to ensure that the spectrum plot is contained entirely within the mask.

FIGURE 2...[D02] = 3 CM