RECOMMENDATION  ITU-R  BT.1210-3

Test materials to be used in subjective assessment

(Question ITU-R 81/6)

(1995-1997-2000-2004)

The ITU Radiocommunication Assembly,

considering

a)	that television system performance must be expressed by both objective and subjective measures;

b)	that the picture quality of the system is evaluated mainly by subjective assessment;

c)	that in subjective assessment, test procedures, viewing conditions and the test materials used may have a great influence on the assessment result;

d)	that Recommendations ITU-R BT.500, ITU-R BT.710, ITU-R BT.1128 and ITU-R BT.1129 describe subjective assessment methods for television pictures;

e)	that digital bit-rate reduction systems, the picture quality of which is generally dependent on, and sensitive to, the picture contents, are increasingly introduced in the broadcasting chain;

f)	that the use of worldwide common test materials is one of the bases necessary to attain a universal assessment result;

g)	that a set of materials that provide the characteristics suitable for test purposes must be properly chosen in the assessment;

h)	that the test materials must be up to date in accordance with progress in television technology,

recommends

1	that the test materials listed in Annex 1 should be preferably used in picture quality evaluation test of television systems;

2	that the test materials should be accompanied by the information shown in Annex 1 as it is useful in the selection of the materials and the statistics that serve to characterize them; candidates for suitable statistics are given in Annex 2;

3	that the latest information on test materials should be provided by the ITU-R computer system so that organizations wanting such materials can get the proper information through on-line access;

4	that the ITU-R computer system should accept the information about candidate test materials on a “draft” basis so that newly developed materials can be registered;

5	that test materials that are not listed in Annex 1, but used in picture quality evaluation tests, should be documented along with Annex 1 and be included in one ITU list, particularly if the test results have been submitted to the ITU,

further recommends

that further development should be undertaken for the identification of test materials beyond those listed in Annex 1, in particular for the advances of high definition television (HDTV), which reflects the progress in television technology.

Annex 1

List of test materials

TABLE  1

Test materials for standard definition TV (SDTV)

| Scene No. | Title | Contents | Represen- tative of | Attributes to be examined | Motion | Source | Availability |  | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 525 | 625 |  |  |  |  |
| 1 | Formal pond | To be filled | To be filled | Luminance resolution | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 2 | Boats | To be filled | To be filled | Luminance and colour resolution | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 3 | Clown | To be filled | To be filled | Horizontal resolution | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 4 | Boy with toys | To be filled | To be filled | Skin and colour edges | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 5 | Girl with toys | To be filled | To be filled | Skin and colour edges | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 6 | Young couple | To be filled | To be filled | Luminance and fine detail | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 7 | Blackboard | To be filled | To be filled | Colour, vertical resolution | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 8 | Tree | To be filled | To be filled | Luminance patterns | Still | Slide | o | o | To be filled | To be filled | D1 tape | To be filled |
| 9 | Old master | To be filled | To be filled | Chromakey FG | Still | Video | – | o | To be filled | To be filled | D1 tape | To be filled |
| 10 | Old master | To be filled | To be filled | Chromakey BG | Still | Video | – | o | To be filled | To be filled | D1 tape | To be filled |
| 11 | Still life | To be filled | To be filled | Chromakey FG | Still | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 12 | Still life | To be filled | To be filled | Chromakey BG | Still | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 13 | Kiel Harbour-1 | To be filled | To be filled | High resolution | Still | 8  10 slide | o | o | To be filled | To be filled | D1 tape | To be filled |

TABLE  1 (continued )

| Scene No. | Title | Contents | Represen- tative of | Attributes to be examined | Motion | Source | Availability |  | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 525 | 625 |  |  |  |  |
| 14 | Sailboat | To be filled | To be filled | Luminance resolution | Slow | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 15 | Flower garden | To be filled | To be filled | Colour details | Slow pan | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 16 | Susie | To be filled | To be filled | Skin tones | Slow | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 17 | Diva with noise | To be filled | To be filled | Rapid entropy changes | Prod. wipe | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 18 | Dinner party | To be filled | To be filled | Chroma key BG |  | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 19 | Boy with toys | To be filled | To be filled | Skin and colour edges | Pan (H, V) | Slide | – | o | To be filled | To be filled | D1 tape | To be filled |
| 20 | Old master | To be filled | To be filled | Chroma key FG | Slow pan | Video | – | o | To be filled | To be filled | D1 tape | To be filled |
| 21 | Old master | To be filled | To be filled | Chroma key BG | Slow pan | Video | – | o | To be filled | To be filled | D1 tape | To be filled |
| 22 | Clown | To be filled | To be filled | Luminance and colour horizontal resolution | Pan (H, V) | Slide | – | o | To be filled | To be filled | D1 tape | To be filled |
| 23 | BBC disc | To be filled | To be filled | Random movement | Circular | Video | – | o | To be filled | To be filled | D1 tape | To be filled |
| 24 | Kiel Harbour-2 | To be filled | To be filled | Cycle motion (narrow filter) | Rapid rocking | Component | o | o | To be filled | To be filled | D1 tape | To be filled |
| 25 | Kiel Harbour-3 | To be filled | To be filled | Cycle motion (wide filter) | Rapid rocking | Component | o | o | To be filled | To be filled | D1 tape | To be filled |
| 26 | Kiel Harbour-4 | To be filled | To be filled | High resolution in H, V, T dimensions | Slow pan/zoom | Component | o | o | To be filled | To be filled | D1 tape | To be filled |
| 27 | Balls of wool | To be filled | To be filled | Moving colours | Medium | Video | o | o | To be filled | To be filled | D1 tape | To be filled |

TABLE  1 (continued )

| Scene No. | Title | Contents | Represen- tative of | Attributes to be examined | Motion | Source | Availability |  | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 525 | 625 |  |  |  |  |
| 28 | Popple | To be filled | To be filled | Moving colours | Pan/rotate | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 29 | Table tennis | To be filled | To be filled | Multiple rapid motions Standards conversion | Pan/zoom/cut | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 30 | Mobile and calendar | To be filled | To be filled | Random motion of objects Standards conversion | Slow | Video | o | o | To be filled | To be filled | D1 tape | To be filled |
| 31 | Autumn leaves | To be filled | To be filled | Colour details | Slow pan/zoom | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 32 | Summer flowers | To be filled | To be filled | Saturated colours, texture | Slow pan | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 33 | Birches | To be filled | To be filled | Luminance details, sky | Slow tilt up | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 34 | Horse riding | To be filled | To be filled | Landscape | Zoom | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 35 | Bicycles | To be filled | To be filled | Bicycle wheels | Complex, fast | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 36 | Ferris wheel | To be filled | To be filled | Luminance and colour details | Fast, complex | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 37 | Shinjuku | To be filled | To be filled | Horizontal and vertical detail | Slow pan | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 38 | Football | To be filled | To be filled | Sports | Rapid motion | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 39 | Cheerleaders | To be filled | To be filled | Fast, complex | Zoom | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 40 | Ciao! | To be filled | To be filled | CK, FG, luminance, colour details | Slow pan/zoom | Camera | o | o | To be filled | To be filled | D1 tape | To be filled |
| 41 | Ciao! | To be filled | To be filled | CK, BG, luminance, colour details | Slow pan/zoom | Camera | o | o | To be filled | To be filled | D1 tape | To be filled |

TABLE  1 (end )

| Scene No. | Title | Contents | Represen- tative of | Attributes to be examined | Motion | Source | Availability |  | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 525 | 625 |  |  |  |  |
| 42 | Portrait de famille | To be filled | To be filled | Progressive utilization | Wipe | Camera/SE | o | – | To be filled | To be filled | D1 tape | To be filled |
| 43 | Diva | To be filled | To be filled | Cuts on titles/busy scene | Cuts | Camera/SE | o | – | To be filled | To be filled | D1 tape | To be filled |
| 44 | Tempête | To be filled | To be filled | H, V, luminance, colour details | Random motion | Camera | o | o | To be filled | To be filled | D1 tape | To be filled |
| 45 | Tempête with noise | To be filled | To be filled | H, V, luminance, colour details | Random motion | Camera | o | o | To be filled | To be filled | D1 tape | To be filled |
| 46 | TV trip | To be filled | To be filled | 3D graph, H, V, luminance, colour details | Zoom/rotate | Graphics | o | – | To be filled | To be filled | D1 tape | To be filled |
| 47 | Cruising | To be filled | To be filled | Animated freeze frames | 2-10 freezes | Camera | o | o | To be filled | To be filled | D1 tape | To be filled |
| 48 | Decoded NTSC | To be filled | To be filled | Cross luminance colour | Slow pan/zoom | Camera | o | – | To be filled | To be filled | D1 tape | To be filled |
| 49 | Decoded PAL | To be filled | To be filled | Cross luminance colour | Slow pan/zoom | Camera | – | o | To be filled | To be filled | D1 tape | To be filled |
| 50 | Un générique | To be filled | To be filled | Rolling and crawling titles | Crawl/roll | Camera/CG | o | o | To be filled | To be filled | D1 tape | To be filled |
| 51 | Error recovery | To be filled | To be filled | Frame and calibration | Slow | Camera/Key | o | o | To be filled | To be filled | D1 tape | To be filled |
| 52 | Text for 625 diva | To be filled | To be filled | Cuts on titles | Cuts | SE |  | o | To be filled | To be filled | D1 tape | To be filled |
| 53 | Basketball | Basketball match | Sport | Complex large area motion, high detail background | High | CCD camera recorded directly as YUV in D1 format | – | o | To be filled | Free | D1 tape | DoCA(1) |
| BBC: British Broadcasting Corporation CG: Computer graphics SE: Special effects (1) Available from Commercial Television Australia Tel.: 61 2 9960 2622 Fax: 61 2 9969 3520. |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE  2

Test materials for conventional TV

| Scene No. | Title | Contents | Represen- tative of | Attributes to be examined | Motion | Source | Availability |  | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 525 | 625 |  |  |  |  |
|  | Stills |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Formal pond | To be filled | To be filled | Luminance resolution | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 2 | Boats with lighthouse | To be filled | To be filled | Luminance and colour resolution | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 3 | Clown | Clown making up | To be filled | Horizontal resolution | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 4 | Boy with toys | Boy playing with several coloured toys | Skin and colour edges | To be filled | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 5 | Girl with toys | Girl playing with several coloured toys | Skin and colour edges | To be filled | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 6 | Young couple | Man and woman with striped clothes | Fine detail | Luminance resolution | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 7 | Toys and blackboards | To be filled | To be filled | Colour, vertical resolution | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 8 | Tree | Luminance patterns | To be filled | To be filled | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 9 | Male head | To be filled | To be filled | To be filled | Still | Slide |  |  | To be filled | To be filled | D1/MT | To be filled |
| 10 | Kiel Harbour | Detailed view of the Kiel Harbour | Fine resolution | Spatial resolution | Still | Slide | o | o | To be filled | To be filled | D1/MT | To be filled |
| 11 | Latin text | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | D1/MT | To be filled |
| 12 | Graph | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | D1/MT | To be filled |

TABLE  2 (end )

| Scene No. | Title | Contents | Represen- tative of | Attributes to be examined | Motion | Source | Availability |  | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 525 | 625 |  |  |  |  |
| 13 | Test card | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | D1/MT | To be filled |
| 14 | Zone plate | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | To be filled | D1/MT | To be filled |
|  | Stills – Chroma key |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Comb and pencil | To be filled | To be filled | To be filled | Still | To be filled |  |  | To be filled | To be filled | D1/MT | To be filled |
| 16 | Twigs and ribbon | To be filled | To be filled | To be filled | Still | To be filled |  |  | To be filled | To be filled | D1/MT | To be filled |
| 17 | Old master – foreground | To be filled | Chroma key FG | To be filled | Still | To be filled | – | o | To be filled | To be filled | D1/MT | To be filled |
| 18 | Old master – background | To be filled | Chroma key BG | To be filled | Still | To be filled | – | o | To be filled | To be filled | D1/MT | To be filled |
| 19 | Dummy with comb | To be filled | To be filled | To be filled | Still | To be filled |  |  | To be filled | To be filled | D1/MT | To be filled |
|  | Moving sequences |  |  |  |  |  |  |  |  |  |  |  |
| 20 | BBC disc 1 | Rotating disc with several features attached on it. The axis of rotation is parallel to the viewing axis | Circular motion | To be filled | To be filled | To be filled | – | o | To be filled | To be filled | D1/MT | To be filled |
| 21 | BBC disc 2 | Idem. Increased speed | Circular motion | To be filled | To be filled | To be filled |  |  | To be filled | To be filled | D1/MT | To be filled |
| 22 | BBC disc 3 | Idem. Increased speed | Circular motion | To be filled | To be filled | To be filled |  |  | To be filled | To be filled | D1/MT | To be filled |

TABLE  3

Test materials of still picture for high definition television (HDTV)

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Woman | Bust shot of a woman with a red flower against uniform background | – Studio still portrait | – False contour, Y/C phase difference – Flesh tint and texture reproduction | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 2 | Yacht harbour | Long shot of a yacht harbour with highly detailed yachts | – Outdoor still | – Waveform distortion – Aliasing – Sharpness | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 3 | Sweaters and bag | Loose shot of sweaters, towels of various colours, a leather bag, metal pitcher, etc. | – Studio still – Textured objects | – Texture reproduction – Lustre | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 4 | Eiffel tower | Long shot of Eiffel tower and landscape of Paris behind it | – Outdoor still | – Resolution – Sensation of reality | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 5 | A hat shop | Scene of a stall and a couple talking beside it | – Drama | – Gray scale reproduction – Interlace artefacts | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 6 | A couple in the snow | Loose shot, taken against the light, of a couple facing each other in the snow | – Scene taken against the light | – Gray scale reproduction – Uniformity – Large area flicker | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 7 | Guide board | Full shot of a guide board written in various kinds of characters in different sizes | – Outdoor still | – Waveform distortion – Registration legibility | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 8 | Tulip garden | Wide shot of a garden with red and yellow tulips in various sizes | – Highly saturated colours | – Chrominance/luminance resolution – Colour reproduction | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| 9 | Chromakey | A woman sitting against blue back with detailed flowers | – Chromakey | – Chromakey process – False contour | Still | Slide | See Appendix 1 to Annex 2 | See Annex 3 | Slide/MT/ CD-ROM | ITE |
| ITE: The Institute of Image Information and Television Engineers, 3-5-8 Shibakoen, Minato-ku, Tokyo 105-0011, Japan, Phone: +81 3 3432 4677, Fax: +81 3 3432 4675, E-mail: ite@ite.or.jp. NOTE 1 – Still pictures of 1 920 samples  1 035 lines and 1 920 samples  1 080 lines are available in MT and CD-ROM, respectively. |  |  |  |  |  |  |  |  |  |  |

TABLE  4

Test materials for 1125/60 HDTV and 525/60 television

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery Format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cognac and fruit | Scene of cognac and fruit on the table | Texture | General picture quality | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 2 | Flower basket | Picture of colourful flowers | Colour reproduction, texture | General picture quality | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 3 | Woman with bird cage | Waist shot of a woman carrying a red bird cage | Aliaising error | General picture quality | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 4 | Entrance hall | Loose shot of three women chatting indoors | Drama, gray scale | General picture quality | Fix | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 5 | Boy and toys | Loose shot of a boy playing with toys | Motion adaptive processing | General picture quality | Tilt, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 6 | Intersection | Scene of traffic at an intersection | Motion adaptive processing | General picture quality | Fix | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 7 | European market | Loose shot of a market on a street | Colour reproduction, signal processing | General picture quality | Pan, slow, zoom | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 8 | Walk through the square | Loose shot of a girl walking through a square | Resolution, signal processing | General picture quality | Fix | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 9 | Calligraphy practice | Tight shot of a woman practicing calligraphy | Texture, distortion | General picture quality | Zoom, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 10 | Streetcar | Scene of streetcar running on a street | Motion adaptive processing | General picture quality Standards conversion | Fix | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 11 | Buildings along the canal | Scene of many buildings along a canal | Resolution, distortion | General picture quality | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |

TABLE  4 (continued )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery Format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | Harbour scene | Scene of a harbour with buildings and many people on a street | Resolution, signal processing | General picture quality | Pan, zoom | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 13 | Church | Scene of a Gothic cathedral | Sharpness, distortion | General picture quality | Tilt | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 14 | Yacht harbour | Scene of many yachts docking at harbour | Sharpness, distortion | General picture quality | Pan, slow | Camera | See Appendix to Annex 2 | See Annex 3 | DVTR | ITE |
| 15 | Yachting | Frame in and out shot of a yacht with a woman | Distortion, signal processing | General picture quality | Pan, zoom | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 16 | Whale show | Scene of a whale jumping up in an aquarium | Signal processing | General picture quality | Pan | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 17 | Rustling leaves | Scene of branches and leaves rustling by wind | Signal processing | General picture quality | Fix | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 18 | Wintertime river | Scene of a boat on a calm river in winter season | Gray scale | General picture quality | Pan | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 19 | Opening ceremony | Opening ceremony of an athletic meet | Resolution, signal processing | General picture quality | Fix | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 20 | Soccer action | Loose shot of soccer game | Resolution, signal processing | General picture quality Standards conversion | Pan | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 21 | Baseball | Loose shot of a night game of baseball | Motion reproduction | General picture quality | Pan | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 22 | Marching in | Entrance march at an athletic meet | Resolution, signal processing | Performance of digital signal processing | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 23 | Green leaves | Scene of a tree-lined road with green leaves | Resolution, signal processing | Performance of digital signal processing | Zoom, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |

TABLE  4 (continued )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery Format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | Swinging | Loose shot of women in swinging chair | Distortion, signal processing | Performance of digital signal processing | Zoom, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 25 | Japanese room | Full shot of a woman walking in a Japanese-style room | Resolution, distortion | Distortion and aliasing | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 26 | Japanese harp | Full shot of a woman playing the koto in front of a paper sliding door | Distortion, skin colour | Distortion and aliasing | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 27 | Today's catch | Scene of a fishing boat and the unloading of fish | Distortion | Aliasing | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 28 | Summertime tanning | Waist shot of a woman on a yacht | Texture, skin colour | Skin colour and texture | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 29 | Knitting | Balls of yarn on a sofa and a woman knitting | Texture, colour reproduction | Colour production and texture | Pan, zoom, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 30 | Crowded crosswalk | Scene of pedestrians on a crosswalk | Signal processing | Performance of motion-compensated processing | Slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 31 | Flamingos | Loose shot of flamingos on the grass | Resolution | Resolution for moving area | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 32 | Buddhist images | Stone Buddhist images covered with moss | Texture, signal processing | Performance of motion-compensated processing | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 33 | Group game | A mass game at an athletic meet | Signal processing | Performance of motion-compensated processing and of digital signal processing | Pan, zoom | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 34 | Ice hockey | Scene of a face-off at a hockey game | Resolution, signal processing | Performance of motion-compensated processing | Pan | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |

TABLE  4 (continued )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery Format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35 | Horse race | Horse racing at the start | Signal processing | Performance of motion-compensated processing | Pan, fast | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 36 | Aeroplane landing | An aeroplane landing on a runway | Signal processing | Performance of motion-compensated processing | Pan, fast | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 37 | Driving | A car moving along a tree-lined road | Signal processing | Performance of motion-compensated processing | Track | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 38 | Skyscrapers | Scene of hazy buildings | Signal processing | Performance of motion-compensated processing | Pan, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 39 | Weather report | Scene of a weather report with a CG picture in the background behind a reporter | Sharpness | Sharpness | Slow | Camera/CG | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 40 | Flash photography | Taking pictures of a woman using flashes | Signal processing | Performance of digital signal processing | Slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 41 | Racetrack with credits | Scene of a sports stadium with superimposition | Superimposition, signal processing | Performance of motion-compensated processing and of digital signal processing | Pan, slow | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 42 | View from sky with credits | Aerial scene of a fiord with superimposition scrolling upwards | Superimposition, signal processing | Performance of motion-compensated processing | Slow | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 43 | Bronze with credits | A bronze statue with superimposition moving horizontally | Superimposition, signal processing | Performance of motion-compensated processing | Pan, slow | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 44 | Chromakey (fish bowl) | A scene of a woman and a fishbowl, for a chromakey test | Chromakey test | Performance of chroma key | Slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 45 | Chromakey (flowers) | A scene of a woman arranging flowers and a lace curtain for a chromakey test | Chromakey test | Performance of chroma key | Slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |

TABLE  4 (continued )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery Format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | Chromakey (sprinkling) | A composed scene of a woman and sprinkling water | Composed scene | Performance of digital signal processing | Random | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 47 | Chromakey (picture cuts) | A composed scene of a woman and short cut images | Composed scene | Performance of motion-compensated processing and of digital signal processing | Cuts | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 48 | Overlap | Repeated superimpositions of gray patterns on general images | Overlap | Performance of motion-compensated processing | Overlaps | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 49 | Character pattern | Scene of character patterns in various sizes | Character pattern | Legibility | Zoom, slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 50 | Rotating disk (without shutter) | A rotating disk attached with still pictures (in the shutter speed of 1/60 s) | Shutter speed effect | Performance of motion-compensated processing | Rotating | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 51 | Rotating disk (with 1/250 s shutter) | The same as the above (in the shutter speed of 1/250 s) | Shutter speed effect | Performance of motion-compensated processing | Rotating | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 52 | Rotating disk (with 1/1000 s shutter) | The same as the above (in the shutter speed of 1/1000 s) | Shutter speed effect | Performance of motion-compensated processing | Rotating | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 53 | Pendulum (without shutter) | A pendulum swinging in front of a still picture (in the shutter speed of 1/60 s) | Shutter speed effect | Performance of motion-compensated processing | Swinging | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 54 | Pendulum (with 1/250 s shutter) | The same as the above (in the shutter speed of 1/250 s) | Shutter speed effect | Performance of motion-compensated processing | Swinging | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 55 | Pendulum (with 1/1000 s shutter) | The same as the above (in the shutter speed of 1/1000 s) | Shutter speed effect | Performance of motion-compensated processing | Swinging | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 56 | Woman in flowers | A waist shot of a woman in front of a garden of flowers | Noise, signal processing | Performance of digital signal processing | Slow | Camera | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |

TABLE  4 (end )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery Format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57 | Woman in flowers with noise (S/N = 35 dB) | Adding noise of S/N = 35 dB to the above scene | Noise, signal processing | Performance of digital signal processing | With noise | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 58 | Woman in flowers with noise (S/N = 30 dB) | The same as the above (S/N = 30 dB) | Noise, signal processing | Performance of digital signal processing | With noise | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 59 | Woman in flowers with noise (S/N = 25 dB) | The same as the above (S/N = 25 dB) | Noise, signal processing | Performance of digital signal processing | With noise | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| 60 | Woman in flowers with noise (S/N = 20 dB) | The same as the above (S/N = 20 dB) | Noise, signal processing | Performance of digital signal processing | With noise | Camera/SE | See Appendix 1 to Annex 2 | See Annex 3 | DVTR | ITE |
| CG: Computer graphics. DVTR: Digital video tape recorder; 1-inch digital VTR for 1125/60, D1 for 525/60/2:1, 2  D1 for 525/60/1:1. ITE: The Institute of Image Information and Television Engineers, 3-5-8 Shibakoen, Minato-ku, Tokyo 105-0011, Japan, Phone: +81 3 3432 4677, Fax: +81 3 3432 4675, E-mail: ite@ite.or.jp. SE: Special effects. NOTE 1 – 1125/60/2:1 (Rec. ITU-R BT.709), 525/60/2:1 (Rec. ITU-R BT.601) and 525/60/1:1 (Rec. ITU-R BT.1358) are available. The sequences of 525/60/2:1 and 525/60/1:1 are those down-converted from the 1125/60/2:1 HDTV materials. Picture aspect ratio is 4:3 and 16:9 for 525/60/2:1, and 16:9 for 525/60/1:1. |  |  |  |  |  |  |  |  |  |  |

TABLE  5

Test materials for 1250/50 HDTV

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | English street | Natural street scene with mainly unsaturated colours; it includes zooming and motion | – Outdoor shooting – Documentary – Zoom | – Dynamic resolution – Block-switching – Artefacts | Zoom | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 2 | Geranium (BBC) | Natural scene with erratic motion in saturated coloured area; it includes a scene cut | – Outdoor shooting – Nature documentary | – Chrominance block switching – Colour blindness | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 3 | Arcade (RAI) | A background with complex texture foreground movement. Spectacular reflections on foreground rippled water | – Outdoor shooting – Panning | – Dynamic resolution – Block switching – Artefacts | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 4 | Noël (BBC) | Close-up of talking head. Background is in soft focus. Slight camera motion | – Studio production – Talk shows – News, etc. | – Resolution at static motion boundary – Natural facial motion – Chrominance resolution | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 5 | Singers (EU95) | Pan/zoom across detailed background with foreground singers. Foreground and background in focus | – Studio production – Culture programmes | – Portrayal of moving detail – Block switching – Artefacts – Natural facial motion | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |

TABLE  5 (continued )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Winter Olympic (Thompson) | Pan across high coloured background to follow skier | – Outdoor shooting – Sports coverage | – Chrominance block switching – Colour blindness – Chrominance noise – Standards conversion | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 7 | Olympic ceremony (Barcelona, 1250) | Wide angle and close-up shots of brightly dressed performers | – Outdoor shooting – Scene with large area of saturated colours | – Coherence luminance chrominance block decision – Chrominance noise | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 8 | Tennis (BBC) | Wide angle shot of complex motion. Camera pan to track players | – Outdoor spectacle – Sports events – Logo insertion | – Block switching – Dynamic resolution – Standards conversion | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 9 | Seats and grass (Barcelona, 1250) | Pan over empty stadium seating onto grass | – Outdoor sports – Field events | – Portrayal of moving fine detail – Behaviour for periodic structures – Luminance noise – Standards conversion | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 10 | Kiel Harbour (FI-DBP) | Still of harbour scene with non-saturated colours and highly detailed yachts on calm water | – Outdoor stills | – Resolution of still pictures – Block switching – Noise | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |

TABLE  5 (end )

| Scene No. | Title | Contents | Representative of | Attributes to be examined | Motion | Source | Statistic | Copyright status | Delivery format | Provider name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | Tram (EU95) | Still of street scene. Fine text and periodic structures in buildings | – Outdoor stills | – Resolution of still pictures – Block switching – Artefacts | To be filled | To be filled | To be filled | To be filled | 4  D1 Exabyte | EBU |
| 12 | Mobile and calendar 2 | Miniature train circulating in front of a coloured setting | To be filled | – Saturated colours – Zoom – Standards conversion | To be filled | To be filled | To be filled | To be filled | 4  D1 | CCETT |
| 13 | Bicross | Mountain bike race | To be filled | – Foliage – Quick motion | To be filled | To be filled | To be filled | To be filled | 4  D1 | CCETT |
| 14 | Saint Jacques | Plane landing | To be filled | – Quick motion | To be filled | To be filled | To be filled | To be filled | 4  D1 | CCETT |
| 15 | Saint Malo | Sailing boats into harbour and buildings | To be filled | – Fine details – Irregular motions | To be filled | To be filled | To be filled | To be filled | 4  D1 | CCETT |
| 16 | Table tennis 2 | Play | To be filled | – Saturated colours – Irregular motions – Standards conversion | To be filled | To be filled | To be filled | To be filled | 4  D1 | CCETT |
| CCETT: Centre commun d'études de télédiffusion et télécommunications (Joint Research Center for Broadcast and Telecommunications) EBU: European Broadcasting Union RAI: Radiotelevisione Italiana |  |  |  |  |  |  |  |  |  |  |

Annex 2

Choice of statistics that can serve for
characterization of materials

# 1	Introduction

Many test sequences for subjective assessment are described in Annex 1, and have been used in various evaluation tests. These sequences contain many different attributes as a whole, while each sequence provides a few attributes. It is important in actual evaluation tests to choose the sequences suitable for the test purposes. Statistics are useful in choosing proper sequences for the assessment purpose, as well as in analysing the quality degradation of the system to be evaluated.

In selecting the statistics described below, it is considered that there are increasing opportunities to conduct the evaluation tests of digital bit-rate reduction systems, the picture quality of which is generally sensitive to the picture contents, and therefore proper test materials must be used in the evaluation tests.

# 2	Statistics of test materials

## 2.1	Entropy of PCM data

This entrophy, E, represents the amount of information of a picture. It is defined by equation (1):

E=−i=IminImaxP(i) × logP(i)                bit/pixel	(1)

where:

Imin :	minimum level of the video signal

Imax :	maximum level of the video signal

P(i) :	probability occurrence for the video signal level i.

## 2.2	AC energy

This represents the degree of picture activity, such as fineness, and is defined to be the square sum of the DCT coefficients except for the DC coefficient, as shown below:

AC = 1Nk=1Nack  ACmax	(2)

where:

ack = m=07n=07C(m,n)2−C0. 02

and C(m, n) denotes the DCT coefficients and N the number of blocks in a field (or frame). ACmax is a normalizing factor and the theoretically maximum value of AC energy is taken here, which is obtained with such a picture that a half area within a block is black and the rest of the area within the block is white.

AC energy is calculated under several conditions, i.e. intra/inter-frame/field. Figure 1a) shows examples of the AC energy for intra-frame (frame DCT) and field DCT with motion-compensated frame difference (field DCT-MCFD).

## 2.3	Spectral entropy

This represents the degree of randomness of the DCT coefficients, and is useful to estimate the necessary bit rate of a DCT-based bit reduction system. It is defined by equation (3):

SE = 1Nk=1N(sek)2	(3)

where:

sek = –m=07n=07|C(m,n)|A log2C(m,n)A

A = m=07n=07Cm,n

Spectral entropy is calculated under several conditions, i.e. intra/inter-frame/field. Fig. 1b) shows examples of the spectral entropy.

## 2.4	Motion vector

This represents a motion of an object by a two-dimensional parameter on a block-by-block basis. Block matching is one of the commonly used methods for motion estimation.

To express the degree of motion of the picture, two kinds of statistics are used, i.e. mean magnitude of the vectors averaged over a frame/field and standard deviation within the frame/field. They are calculated separately in horizontal and vertical directions as follows:

μX=1Nk=1NXk     and     μY=1Nk=1NYk	(4)

σX2=1Nk=1NXk2−μX2     and     σY2=1Nk=1NYk2−μY2	(5)

where:

Xk and Yk :	horizontal and vertical components of the vector in a block

X and Y :	mean magnitude of Xk and Yk averaged over a field

X and Y :	standard deviation of Xk and Yk, respectively.

The mean magnitudes represent the degree of motions as a whole, while the standard deviations represent the degree of non-uniform motions.

Figures 1c) and 1d) show examples of the motion vector, calculated using block matching.

## 2.5	Motion-compensated prediction error power

The motion-compensated frame/field-difference signal, i.e. prediction error, is expressed by equation (6):

ek(x, y)=f0 (x, y)−f1 (x−uk, y−vk)	(6)

where ek (*), f0 (*) and f1 (*) denote the motion-compensated frame/field-difference signal in the k-th block, the current frame/field signal, and the previous frame/field signal, respectively, while uk and vk the horizontal and vertical components of motion vector in the block.

The power of the prediction error, EP, is defined to be the mean square value of the difference signals as follows:

EP=1Nk=1Nepk	(7)

where:

epk=1X×Yx=1X y=1Ye(x,y)2

X and Y denote horizontal and vertical sizes of a block.

This statistic value could be useful to estimate whether the sequence is critical for a bit reduction system using motion compensation.

Figure 1e) shows an example of the motion-compensated prediction error power.

## 2.6	Criticality by rate-buffer occupancy method

The Independent Television Commission of the United Kingdom and the EBU conducted a study on criticality under the RACE MOSAIC project. The MPEG-1  coding algorithm with subjectively optimized quantizers was used to measure the criticality of broadcast programmes and test sequences. The criticality was calculated by means of a parallel processing computer using the rate-buffer occupancy, and was expressed in terms of bit/pixel. An example of criticality measurement is shown in Fig. 2.

## 2.7	Criticality by fixed quantizer method

This criticality is defined as “the number of output bits per pixel from MPEG-2 encoder with a fixed quantizer”. The quantizer characteristics are linear and comply with the MPEG-2 standard. The parameter value of “quantizer_scale” gives a quantizer step closely related to picture quality.

NOTE 1 – The results are shown only for the luminance signals of the scene No. 20 “Soccer action” in Table 4 of Annex 1. The ordinate of each chart indicates the statistic value, and the abscissa the field number, counting from the beginning of the sequence (900 fields correspond to 15 s). It can be seen that there are two large horizontal motions around the field numbers 200 and 600, which suppress fine detail components due to the integration effect of the camera, resulting in reduced AC energy and spectral entropy.

Examples of the criticality are shown in Fig. 3. The criticality was measured in each frame under the following condition: quantizer_scale  12 (q_scale_code  6 and q_scale_type  0), field-based forward prediction and intra macroblock refreshment with a cycle of 0.5 s. Figure 3a) shows the fluctuation of the criticality over a 5 s interval. The criticality distribution of broadcast television programmes was measured on an NHK channel for one week, a total of 130 h, from 15 through 22  February 1995. In the measurement, composite NTSC signals were converted into component Y/C signals. The frequency of occurrence of the criticality for television programmes was calculated every 5  10–3 bit/pixel. The criticality distribution for different programme genres is shown in Fig. 3b). Sports programmes are relatively critical, while drama programmes are less critical. The statistical distribution of the criticality for overall television programmes is shown in Fig. 3c). This figure also shows the criticality for test sequences.

Appendix 1
to Annex 2

Statistics of the test materials

This Appendix reports some of the statistics of the test materials listed in Annex 1.

Annex  3

Status for usage of the test materials
(see Note 1)

Use of the test materials described in Tables 3 and 4 is restricted to the following purposes only.

a)	Technical evaluation, including:

–	research and development of equipment and systems;

–	testing of equipment in development and production process;

–	testing of transmission condition for broadcasting and telecommunication;

–	maintenance of equipment.

b)	Technical demonstration, including:

–	presentation at technical conference and workshop;

–	presentation of performance and functionality of equipment at exhibition.

Inclusion in commercial products and promotional demonstration of commercial products are not allowed.

NOTE 1 – Usage restrictions as proposed above can be considered as falling under the following category:

Commercial restricted

The test materials may be used for research projects, verification of equipment specifications and public demonstration of commercial products. Inclusion in commercial products is not allowed. Reproductions for distribution may only be made by the copyright holder or authorized distributor.