RECOMMENDATION  ITU-R  BT.1304*

Checksum for error detection and status information in interfaces conforming with Recommendations ITU-R BT.656 and ITU-R BT.799

(Question ITU-R 42/6)

(1997)

The ITU Radiocommunication Assembly,

considering

a)	that many countries are installing digital television production facilities based on the use of digital video components conforming to Recommendations ITU-R BT.601 and ITU-R BT.656;

b)	that there exists the capacity within a signal conforming to Recommendation ITU-R BT.656 for additional data signals to be multiplexed with the video data signal itself;

c)	that there are operational and economic benefits to be achieved by the multiplexing of ancillary data signals with the video data signal;

d)	that the operational benefits are increased if a minimum of different formats are used for ancillary data signals;

e)	that some countries are already using ancillary data signals embedded in the video data signal,

recommends

1	that for optional error detection and provision of status information the methods described in Annex 1 for the generation of checkwords and status flags should be used in interfaces conforming with Recommendations ITU-R BT.656 and ITU-R BT.799.

Annex 1

Checksum for error detection and status information

# 1	Introduction

This specification describes the generation of error detection checkwords and related status flags to be used optionally in conjunction with the serial digital interface described in Recommendation ITU-R BT.656.

Two checkwords are defined: one based on a field of active picture video data words and the other on a full field of data words. This two-word approach provides continuing error detection for the active picture when the digital signal has passed through processing equipment that has changed the data outside the active picture area without recalculating the full-field checkword.

Three sets of flags are provided to feed forward information regarding detected errors to help facilitate identification of faulty equipment and the type of fault. One set of flags is associated with each of the two field related checkwords. A third set of flags is used to provide similar information based on evaluating all of the ancillary data checksums within a field. Implementation of these flags is optional.

The checkwords and flags are combined in an error detection data packet which is included as ancillary data. At the receiver, a recalculation of checkwords may be compared to the error detection data packet information to determine if a transmission error has occurred.

# 2	Location of checkwords, flags, identification number, and included samples

## 2.1	Location of checkwords, flags and identification number

Data packets containing the calculated checkwords, error information flags and identification number are located in the ancillary data area of the vertical interval as shown graphically in Figs. 1a and 1b and specified in Table 1 for 525/59.94 systems and Table 2 for 625/50 systems. These locations are chosen to respect the location of source switching points.

The error detection data packet is located immediately prior to the start of active video (SAV) code words in the line prior to that during which source switching occurs.

## 2.2	Samples included in checkword calculations

Starting and ending samples for active picture and full-field checkword calculations are shown in Table 3. For the active picture checkword, only the samples in the digital active picture area of each line are included in the calculation.

Full-field checkwords include all the samples in all lines except the line containing the error detection data packet and the two following lines.

TABLE  1

Location of 525/59.94 system checkwords

| Error checking data locations: line 9, fields I and III and line 272, fields II and IV |  |
| --- | --- |
| Data item | Word No. |
| Ancillary Data Flag word 1 | 1 689 (00.0h) |
| Ancillary Data Flag word 2 | 1 690 (FF.Ch) |
| Ancillary Data Flag word 3 | 1 691 (FF.Ch) |
| Data Identifier | 1 692 (7D.0h) |
| Data Block Number | 1 693 (80.0h) |
| Data Count | 1 694 (44.0h) |
| Active picture data word 0 | 1 695 |
| Active picture data word 1 | 1 696 |
| Active picture data word 2 | 1 697 |
| Full-field data word 0 | 1 698 |
| Full-field data word 1 | 1 699 |
| Full-field data word 2 | 1 700 |
| Ancillary data error flags | 1 701 |
| Active picture error flags | 1 702 |
| Full-field error flags | 1 703 |
| Reserved words (7 total) | 1 704-1 710 |
| Checksum for this ancillary data packet | 1 711 |
| NOTE 1 – Values in parentheses are sample values. |  |

TABLE  2

Location of 625/50 system checkwords

| Error checking data locations: line 5, field I and line 318, fields II |  |
| --- | --- |
| Data item | Word No. |
| Ancillary Data Flag word 1 | Y 850 (00.0h) |
| Ancillary Data Flag word 2 | Cr 425 (FF.Ch) |
| Ancillary Data Flag word 3 | Y 851 (FF.Ch) |
| Data Identifier | Cb 426 (7D.0h) |
| Data Block Number | Y 852 (80.0h) |
| Data Count | Cr 427 (44.0h) |
| Active picture data word 0 | Y 853 |
| Active picture data word 1 | Cb 428 |
| Active picture data word 2 | Y 854 |
| Full-field data word 0 | Cr 429 |
| Full-field data word 1 | Y 855 |
| Full-field data word 2 | Cb 430 |
| Ancillary data error flags | Y 856 |
| Active picture error flags | Cr 431 |
| Full-field error flags | Y 857 |
| Reserved words (7 total) | Cb 432 |
| Checksum for this ancillary data packet | Y 858 |
| NOTE 1 – Values in parentheses are sample values. |  |

TABLE  3

Checkword included samples

| Data item | 13.5 MHz component |
| --- | --- |
| 525/59.94 systems |  |
| First full-field sample, lines 12 and 275 | 1 444 |
| First active picture sample, lines 21 and 284 | 0 |
| Last active picture sample, lines 262 and 525 | 1 439 |
| Last full-field sample, lines 8 and 271 | 1 439 |
| 625/50 systems |  |
| First full-field sample, lines 8 and 321 | Cb 361 |
| First active picture sample, lines 24 and 336 | Cb 0 |
| Last active picture sample, lines 310 and 622 | Y 719 |
| Last full-field sample, lines 4 and 317 | Y 719 |

# 3	Definitions of data words

## 3.1	Checkword values

Each checkword value consists of 16 bits of data calculated using the Cyclic Redundancy Check (CRC) polynomial generation method. The equation and a conceptual logic diagram (see Fig. 2) are shown below:

Checkword (16 bit)    x16    x12    x5    1

All data words with values between FF.0h and FF.8h inclusive are recoded to FF.Ch at the input to the CRC polynomial generator. This recoding is necessary only at the polynomial generator, for the purposes of ensuring compatibility between 8-bit and 10-bit equipment and between different implementations of serial digital interface time reference signals and ancillary data flag coding.

## 3.2	Ancillary data housekeeping, checkwords and flags

Definitions of each ancillary data word specified are shown in Table 4. Zeros are used in the two least significant bits (LSBs) of all words in the data packet to provide compatibility with 8-bit equipment. Bit b7 (V) in picture/field data word 3 is a 1 if a valid CRC has been calculated. A P in b8 provides even parity for b7 through b0; that is, the total number of 1s in b8-b0 is an even number. The most significant bit (MSB), b9, is the logical inverse of b8. A checksum is the last word in the error detection data packet.

The checksum word is used to determine the validity of the included words. The checksum word consists of 9 bits and is the sum of the 9 LSBs (b8-b0) of all words including the Data Identifier (DID) and the Reserved Words. The sum is preset to all zeros at the start of each calculation and the carry ignored.

TABLE  4

Definition of ancillary data words

| Data item | b9 MSB | b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 LSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ancillary data header, ancillary data flag (ADF), word 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ancillary data header, ADF, word 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Ancillary data header, ADF, word 3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Data identifier (7D.0h) | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| Data block number (80.0h) | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Data count (44.0h) | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| Active picture data count word 0 CRC<5:0> | P/ | P | c | c | c | c | c | c | 0 | 0 |
| Active picture data count word 1 CRC<11:6> | P/ | P | c | c | c | c | c | c | 0 | 0 |
| Active picture data count word 0 CRC<15:12> | P/ | P | V | 0 | c | c | c | c | 0 | 0 |
| Full-field data word 0 CRC<5:0> | P/ | P | c | c | c | c | c | c | 0 | 0 |
| Full-field data word 0 CRC<11:6> | P/ | P | c | c | c | c | c | c | 0 | 0 |
| Full-field data word 0 CRC<15:12> | P/ | P | V | 0 | c | c | c | c | 0 | 0 |
| Ancillary data error flags | P/ | P | 0 | ues | ida | idh | eda | edh | 0 | 0 |
| Active picture error flags | P/ | P | 0 | ues | ida | idh | eda | edh | 0 | 0 |
| Full-field error flags | P/ | P | 0 | ues | ida | idh | eda | edh | 0 | 0 |
| Reserved words (7 total) | 1/ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Checksum | S8/ | S8 | S7 | S6 | S5 | S4 | S3 | S2 | S1 | S0 |
| NOTE 1 – P and V are defined in § 3.2. |  |  |  |  |  |  |  |  |  |  |

## 3.3	Error flags

### 3.3.1	Definition of individual error status flags

All error flags indicate only the status of the previous field; that is, each flag is set or cleared on a field-by-field basis. A logical 1 is the set state and a logical 0 is the unset state. The flags are defined as follows:

edh – error detected here:	signifies that a serial transmission error was detected. In the case of ancillary data, this means that one or more ancillary data blocks did not match its checksum.

eda – error detected already:	signifies that a serial transmission error has been detected at a prior point in the signal path. If a device receives a signal in which the edh flag has been set, then that device will forward the data with the eda flag set and the edh flag set to zero if there is no further error in the data.

idh – internal error detected here:	signifies that a hardware error unrelated to serial transmission has been detected within a device. This is provided specifically for devices which have internal error-checking facilities, as an error reporting mechanism.

ida – internal error detected already:	signifies that an idh flag was received and there was a hardware device failure at a prior point in the signal chain.

ues – unknown error status:	signifies that a serial signal was received from a device not supporting this error-detection practice.

### 3.3.2	Degrees of implementation

Individual error status flags, or all such flags, may not be supported by all equipment. To maintain compatibility with full implementations of this specification, in all such cases where one or more error status flags are not supported, the unsupported flags are set to zero.

*	 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2003 in accordance with Resolution ITU-R 44.