RECOMMENDATION  ITU-R  BT.1360

CAPTURE  CHARACTERISTICS  FOR  HIGH-DEFINITION  IMAGES

(Question ITU-R 27/11)

(1998)

Rec. ITU-R BT.1360

The ITU Radiocommunication Assembly,

considering

a)	that the image characteristics for capturing high-definition images should be defined to facilitate the maximum commonality for a variety of applications;

b)	that the values should be chosen to have an harmonious relationship to digital coding systems;

c)	that there is a need for effective methods of exchange of the images between different applications;

d)	that there are substantial benefits to broadcasters and equipment manufactures if the image characteristics for high-definition images can be common for different environments,

recommends

that the following image characteristics be used for capturing high-definition images for television and other appropriate video applications for future implementation.

TABLE  1

Image area

| 1.1 | Picture aspect ratio | 16:9 |
| --- | --- | --- |
| 1.2 | Number of active samples horizontally | 1920 |
| 1.3 | Number of active samples vertically | 1080 |

TABLE  2

Colour primaries

|  |  | CIE x | CIE y |
| --- | --- | --- | --- |
| 2.1 | Red primary | 0.640 | 0.330 |
| 2.2 | Green primary | 0.300 | 0.600 |
| 2.3 | Blue primary | 0.150 | 0.060 |
| 2.4 | Assumed chromaticity for equal primary signals (reference white) ER = EG = EB | D65 x y 0.3127 0.3290 |  |

For a detailed description of colorimetry, see Recommendation ITU-R BT.1361.

NOTE 1 – For the production and international exchange of HDTV programmes, detailed specifications are given in Recommendation ITU-R BT.709, Part II.

NOTE 2 – These values may be used as a foundation for the development of extremely high-definition format.

ANNEX  1

List of systems using these image capture characteristics:

1)	Recommendation ITU-R BT.709, Part II;

2)	SMPTE 274M;

3)	SMPTE 295M.