RECOMMENDATION  ITU-R  BT.1366-2

Transmission of time code and control code in the ancillary data space of a digital television stream according to Recommendations ITU-R BT.656*,
ITU-R BT.799** and ITU-R BT.1120***

(Question ITU-R 42/6)

(1998-2007-2008)

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words “shall” or some other obligatory language such as “must” and the negative equivalents are used to express requirements. The use of such words shall in no way be construed to imply partial or total compliance with this Recommendation.

Scope

This Recommendation defines a transmission format for conveyance of linear (LTC) or vertical interval (VITC) time-code data formatted according to Recommendation ITU-R BR.780-2 in 8- or 10-bit digital television data interfaces according to Recommendations ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120.

The ITU Radiocommunication Assembly,

considering

a)	that the use of time-code signals is well-established in the area of post-production;

b)	that many countries have installed digital television production facilities based on the use of digital video components conforming to Recommendations ITU-R BT.601, ITU-R BT.656 or ITU-R BT.1120;

c)	that there exists the capacity within a signal conforming to Recommendations ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120 for additional ancillary data signals to be multiplexed with the video data signal;

d)	that there are operational and economic benefits to be achieved by the multiplexing of ancillary data signals with the video data signal;

e)	that the operational benefits are increased if a minimum of different formats are used for ancillary data signals;

f)	that the exchange of programme material between and within organizations is facilitated if a common format of time-code signal is used;

g)	that extension of the capacity of the time-code signal to carry additional information is desirable;

h)	that progressive image production beyond 30 Hz requires the use of ancillary time-code packets,

recommends

1	that the time-code ancillary data signal format described in Annex 1 to this Recommendation should be used for the interfaces defined in Recommendations ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120.

Annex 1

# 1	Introduction

This Recommendation defines a transmission format for conveyance of linear (LTC) or vertical interval (VITC) time-code data formatted according to ITU-R BR.780-2 in 8- or 10-bit digital television data interfaces according to ITU-R BT.656, ITU-R BT.799 and ITU-R BT.1120.

Time-code information is transmitted in the ancillary data space as defined in Recommendation ITU-R BT.1364. Multiple codes can be transmitted within a single digital video data stream. Other time information, such as real-time clock, DTTR tape timer information, and other user-defined information, may also be carried in the ancillary time-code packet instead of time code. The actual information transmitted through the interface is identified by the coding of a distributed binary bit.

# 2	Normative references

## 2.1	Normative provisions

The following Recommendations contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of the publication, the editions indicated were valid. All standards and Recommendations are subject to revision, and parties to agreements based on this practice are encouraged to investigate the possibility of applying the most recent edition of the standards and Recommendations indicated below.

## 2.2	Normative references

Recommendation ITU-R BR.780-2 – Time and control code standards, for production applications in order to facilitate the international exchange of television programmes on magnetic tapes.

Recommendation ITU-R BT.1364 – Format of ancillary data signals carried in digital component studio interfaces.

# 3	Ancillary time-code format (ATC)

3.1	One ancillary data packet of constant length excluding ancillary data flag shall fully represent an ancillary time-code (ATC) word.

3.2	The ancillary time-code packet shall be type 2, having a data identification (DID) and a secondary data identification (SDID). The DID and SDID shall be set to:

DID = 60h

SDID = 60h

3.3	The data count (DC) value for ancillary time code shall be set to:

DC = 10h

# 4	Format of user data words in ancillary time-code packet

4.1	All user data words in the ancillary time-code packets are formatted as shown in Table 1.

NOTE 1 – References to user data word (UDW) bits in this Recommendation are for a 10-bit UDW word. Correspondence between an 8-bit word and a 10-bit word is shown in Table 1.

TABLE 1

User data words format

| UDW10 bit (10-bit words) | UDW8 bit (8-bit words) | Assignment |
| --- | --- | --- |
| b0 (LSB) | N/A | Set to “0” in 10-bit words. N/A in 8-bit words |
| b1 | N/A | Set to “0” in 10-bit words. N/A in 8-bit words |
| b2 | b0 | Set to “0” in 10-bit and 8-bit words |
| b3 | b1 | Distributed binary bit (DBB) |
| b4 | b2 | ANC binary group LSB |
| b5 | b3 | ANC binary group |
| b6 | b4 | ANC binary group |
| b7 | b5 | ANC binary group MSB |
| b8 | b6 | 10 bit systems: Even parity for data contained in UDW bit 7 through bit 0 8 bit systems: Even parity for data contained in UDW bit 5 through bit 0 |
| b9 (MSB) | b7 | 10 bit: Not bit 8, 8 bit: Not bit 6 |

4.1.1	Bit b7 through bit b4 of the UDW10-1 through UDW10-16 shall contain the time-code information and additional information as per Recommendation ITU-R BR.780-2.

4.2	Bit b3 of the UDW10-1 through UDW10-16 form two groups of distributed binary bits DBB 1 and DBB 2 (see Table 3).

4.2.1	The first group of distributed binary bits (DBB 1) is formed by bit 3 of UDW10-1 through UDW10-8, where UDW10-1 (b3) represents the LSB and UDW10-8 (b3) represents the MSB.

4.2.2	The second group of distributed binary bits (DBB 2) is formed by bit 3 of UDW10-9 through UDW10-16, where UDW10-9 (b3) represents the LSB and UDW10-16 (b3) represents the MSB.

4.3	Bits b7 through b4 form an ancillary binary group into which the time code is mapped. Bits b4 of the UDW10 represents the LSB of this group.

4.4	Information coded in the distributed binary bit group is defined in Table 3.

4.4.1	Bits b4 through b0 of the distributed binary bit group DBB 2 convey VITC line number location indicating the position of VITC data on the output digital video signal interface within the vertical blanking interval. The line select number depends on the television system and shall be constrained to a range as shown in Table 2.

TABLE 2

Line select number

|  |  |  |  |  |  |  |  | VITC line select |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 525/60I |  | 625/50I |  |  |
|  |  |  |  |  |  |  |  | bit b5 = x | bit b5 = 1 |  | bit b5 = x | bit b5 = 1 |
| DBB 2 bits b4 through b0 |  |  |  |  |  |  |  | VITC on line N | Repeated VITC on line (N+2) |  | VITC on line N | Repeated VITC on line (N+2) |
| B4 | b3 | b2 |  | b1 |  | b0 |  | field 1 (odd) field 2 (even) | field 1 (odd) field 2 (even) |  | field 1 (odd) field 2 (even) | field 1 (odd) field 2 (even) |
| 0 | 0 | 1 |  | 1 |  | 0 |  | – | – |  | 6/319 | 8/321 |
| 0 | 0 | 1 |  | 1 |  | 1 |  | – | – |  | 7/320 | 9/322 |
| 0 | 1 | 0 |  | 0 |  | 0 |  | – | – |  | 8/321 | 10/323 |
| 0 | 1 | 0 |  | 0 |  | 1 |  | – | – |  | 9/322 | 11/324 |
| 0 | 1 | 0 |  | 1 |  | 0 |  | 10/273 | 12/275 |  | 10/323 | 12/325 |
| 0 | 1 | 0 |  | 1 |  | 1 |  | 11/274 | 13/276 |  | 11/324 | 13/326 |
| 0 | 1 | 1 |  | 0 |  | 0 |  | 12/275 | 14/277 |  | 12/325 | 14/327 |
| 0 | 1 | 1 |  | 0 |  | 1 |  | 13/276 | 15/278 |  | 13/326 | 15/328 |
| 0 | 1 | 1 |  | 1 |  | 0 |  | 14/277 | 16/279 |  | 14/327 | 16/329 |
| 0 | 1 | 1 |  | 1 |  | 1 |  | 15/278 | 17/280 |  | 15/328 | 17/330 |
| 1 | 0 | 0 |  | 0 |  | 0 |  | 16/279 | 18/281 |  | 16/329 | 18/331 |
| 1 | 0 | 0 |  | 0 |  | 1 |  | 17/280 | 19/282 |  | 17/330 | 19/332 |
| 1 | 0 | 0 |  | 1 |  | 0 |  | 18/281 | 20/283 |  | 18/331 | 20/333 |
| 1 | 0 | 0 |  | 1 |  | 1 |  | 19/282 | – |  | 19/332 | 21/334 |
| 1 | 0 | 1 |  | 0 |  | 0 |  | 20/283 | – |  | 20/333 | 22/335 |
| 1 | 0 | 1 |  | 0 |  | 1 |  | – | – |  | 21/334 | – |
| 1 | 0 | 1 |  | 1 |  | 0 |  | – | – |  | 22/335 | – |
| NOTE – x = irrelevant. |  |  |  |  |  |  |  |  |  |  |  |  |

4.4.2	Bit b5 of DBB 2 when set to “1” shall signify that the VITC word carried in the ancillary time-code word, when converted to an analogue video output signal, shall be inserted on the selected line number and shall be repeated again on the selected line number +2 (see Table 2, bit b5 = 1).

4.4.3	Bits b7 and b6 of the DBB 2 word represent different time-code condition bits (see Table 3). Data errors indicated by the error detection system of the received time-code signal at the input receiving interface to the ancillary time-code formatter and the type of processing of receiving user bits shall be signalled by these bits in the transmitted ATC word. The coding of these two bits is shown in Table 4.

TABLE 3

Distributed binary bit group coding

| DBB group | Bit 3 of UDW | Distributed binary bit (DBB) MSB LSB | Definition |
| --- | --- | --- | --- |
|  |  | 0 0 0 0 0 0 0 0 | Longitudinal time code |
|  |  | 0 0 0 0 0 0 0 1 | Vertical interval time code No. 1 |
|  |  | 0 0 0 0 0 0 1 0 | Vertical interval time code No. 2 |
| DBB 1 | UDW10-1 through UDW10-8 | 0 0 0 0 0 0 1 1 through 0 0 0 0 0 1 1 1 | User defined |
|  |  | 0 0 0 0 1 0 0 0 through 0 1 1 1 1 1 1 1 | Locally generated time address and user data (user defined) |
|  |  | 1 0 0 0 0 0 0 0 through 1 1 1 1 1 1 1 1 | Reserved |
|  | UDW10-9 | b0 | VITC line select (LSB) (Note) |
|  | UDW10-10 | b1 | VITC line select (Note) |
|  | UDW10-11 | b2 | VITC line select (Note) |
| DBB 2 | UDW10-12 | b3 | VITC line select (Note) |
|  | UDW10-13 | b4 | VITC line select (MSB) (Note) |
|  | UDW10-14 | b5 | VITC line duplication (Note) |
|  | UDW10-15 | b6 | Time-code validity |
|  | UDW10-16 | b7 | (User bits) process bit |
| NOTE 1 – These bits are not used in interfaces which are in accordance with Recommendation ITU-R BT.1120, and shall be set to logical zero. |  |  |  |

4.5	Mapping of the time-code data into the UDW 1 thorough UDW 16 of the ancillary time-code data packet is shown in Table 5.

TABLE 4

Coding of validity and process bits

| VITC validity bit (b6) and process bit (b7) | Definition |
| --- | --- |
| b6 = 0 | No time-code error received or locally generated time-code address |
| b6 = 1 | Transmitted time code interpolated from previous time code (received a time-code error) |
| b7 = 0 | Binary group of user bits in time-code data stream are processed to compensate for latency |
| b7 = 1 | Binary groups of user bits in time-code data stream are only retransmitted (no delay compensation) |

TABLE 5

Mapping of time-code data into UDW

| UDW |  | Time-code bit | Time-code definitions (as per Rec. ITU-R BR.780-2) |
| --- | --- | --- | --- |
| 1 | b4 b5 b6 b7 | 0 1 2 3 | Units of frames 1 Units of frames 2 Units of frames 4 Units of frames 8 |
| 2 | b4 b5 b6 b7 | 4 5 6 7 | LSB binary group 1 xxx binary group 1 xxx binary group 1 MSB binary group 1 |
| 3 | b4 b5 b6 b7 | 8 9 10 11 | Tens of frames 10 Tens of frames 20 Flag Flag |
| 4 | b4 b5 b6 b7 | 12 13 14 15 | LSB binary group 2 xxx binary group 2 xxx binary group 2 MSB binary group 2 |
| 5 | b4 b5 b6 b7 | 16 17 18 19 | Units of seconds 1 Units of seconds 2 Units of seconds 4 Units of seconds 8 |
| 6 | b4 b5 b6 b7 | 20 21 22 23 | LSB binary group 3 xxx binary group 3 xxx binary group 3 MSB binary group 3 |
| 7 | b4 b5 b6 b7 | 24 25 26 27 | Tens of seconds 10 Tens of seconds 20 Tens of seconds 40 Flag |
| 8 | b4 b5 b6 b7 | 28 29 30 31 | LSB binary group 4 xxx binary group 4 xxx binary group 4 MSB binary group 4 |
| 9 | b4 b5 b6 b7 | 32 33 34 35 | Units of minutes 1 Units of minutes 2 Units of minutes 4 Units of minutes 8 |
| 10 | b4 b5 b6 b7 | 36 37 38 39 | LSB binary group 5 xxx binary group 5 xxx binary group 5 MSB binary group 5 |
| 11 | b4 b5 b6 b7 | 40 41 42 43 | Tens of minutes 10 Tens of minutes 20 Tens of minutes 40 Flag |

TABLE 5 (end)

| UDW |  | Time-code bit | Time-code definitions (as per Rec. ITU-R BR.780-2) |
| --- | --- | --- | --- |
| 12 | b4 b5 b6 b7 | 44 45 46 47 | LSB binary group 6 xxx binary group 6 xxx binary group 6 MSB binary group 6 |
| 13 | b4 b5 b6 b7 | 48 49 50 51 | Units of hours 1 Units of hours 2 Units of hours 4 Units of hours 8 |
| 14 | b4 b5 b6 b7 | 52 53 54 55 | LSB binary group 7 xxx binary group 7 xxx binary group 7 MSB binary group 7 |
| 15 | b4 b5 b6 b7 | 56 57 58 59 | Tens of hours 10 Tens of hours 20 Flag Flag |
| 16 | b4 b5 b6 b7 | 60 61 62 63 | LSB binary group 8 xxx binary group 8 xxx binary group 8 MSB binary group 8 |
| NOTE 1 – Appropriate flag information for each television system as per Recommendation ITU-R BR.780-2 is inserted into the corresponding positions of Table 5 marked as “flag.” |  |  |  |

# 5	Transmission of ancillary time-code packets

5.1	Multiple transmissions of ancillary time-code packets per video frame code information are permissible under the provisions of this Recommendation.

NOTE 1 – This Recommendation permits transmission of different ATC packets within a single video frame; as for example an ATC packet containing LTC information and a second ATC packet containing VITC information. The time-code information in these two ATC packets shall correspond to the relevant video frame (see § 6.2).

5.2	Transmission of ancillary time-code packets should be at least once per frame for LTC data word and once per field for VITC data word.

5.2.1	Only the 64 information bits of time code are transferred to the ATC. The LTC sync word (bits 64-79) and the VITC (“1”/”0”) sync bit pairs and CRC word are omitted from the ancillary time-code packets.

# 6	Ancillary time-code packets location

6.1	Insertion of ancillary time-code (ATC) packets into any available location in the digital data stream is permitted under the provisions of this recommendation, but it is recommended that packet insertion occurs within the vertical blanking interval as defined by the line standard in use (prior) (see Note 1) to the normal vertical switching point defined in SMPTE RP 168-2002 (Definition of vertical Interval Switching Point for Synchronous Switching).

NOTE 1 – ATC information should correspond directly to the video after the vertical switching point. For that reason the insertion of the ATC after the normal vertical switching point should be considered as the preferred location.

6.1.1	For systems implementing ITU-R BT.1120 the following ATC insertion points should be considered as the preferred location. ATC packets shall be inserted into the Y channel of the interface.

| Type of time code | Location for multiplexing in 1125-line interlace systems | Location for multiplexing in 1125-line progressive and PsF systems |
| --- | --- | --- |
| Packet for LTC | Horizontal ancillary data space of line 10 |  |
| Packet for VITC 1 | Horizontal ancillary data space of line 9 |  |
| Packet for VITC 2 | Horizontal ancillary data space of line 571 | Horizontal ancillary data space of line 11 |
| Packet for others | Available any horizontal ancillary data space except lines 9, 10 and 571 | Any lines except lines 9 and 10 |

6.1.2 	When dual link ITU-R BT.1120 interfaces are used for a 1125 line progressive system, the locations of embedded time-code packets in each link are the same as for 1125-line interlaced formats.

6.1.3 	When ITU-R BT.656 or ITU-R BT.799 interfaces are used for SDTV signals the preferred location for insertion of ATC packets is in vertical ancillary data space, following the second line after the line specified for switching.

6.2	Frame or field address information (LTC or VITC) contained in an ATC packet shall correspond to the respective video frame or field in which the ATC packet resides. Look-ahead compensation shall be applied to the time-code (LTC or VITC) frame count when converting between ATC and either LTC or VITC.

6.3	Transmission of the VITC word for field 1 or field 2 in the ancillary time-code word is signalled by a corresponding field flag (defined in Recommendation ITU-R BR.780-2) located in the ancillary binary group of the ATC word (see Table 5). This same flag shall be used to identify a two-frame sequence when the frame rate is greater than 30 Hz (defined in Recommendation ITU-R BR.780-2).

*	Recommendation ITU-R BT.656 – Interface for digital component video signals in 525-line and 625-line television systems operating at the 4:2:2 level of Recommendation ITU-R BT.601.

*	*	Recommendation ITU-R BT.799 – Interface for digital component video signals in 525-line and 625-line television systems operating at the 4:4:4 level of Recommendation ITU-R BT.601.

*	**	Recommendation ITU-R BT.1120 – Digital interfaces for HDTV studio signals.

ATC is used to convey time-code data formatted as LTC, VITC, or both.