RECOMMENDATION  ITU-R  BT.1368-4*, **

Planning criteria for digital terrestrial television
services in the VHF/UHF bands

(Question ITU-R 4/6)

(1998-1998-2000-2002-2004)

The ITU Radiocommunication Assembly,

considering

a)	that systems are being developed for the transmission of digital terrestrial television services in the VHF/UHF bands;

b)	that the VHF/UHF television bands are already occupied by analogue television services;

c)	that the analogue television services will remain in use for a considerable period of time;

d)	that the availability of consistent sets of planning criteria agreed by administrations will facilitate the introduction of digital terrestrial television services,

recommends

1	that the relevant protection ratios (PRs) and the relevant minimum field strength values given in Annexes 1, 2 and 3, and the additional information given in Annexes 4, 5, 6 and 7 be used as the basis for frequency planning for digital terrestrial television services.

Introduction

This Recommendation contains the following Annexes:

Annex 1	 –	Planning criteria for ATSC digital terrestrial television systems in the VHF/UHF bands

Annex 2	 –	Planning criteria for DVB-T digital terrestrial television systems in the VHF/UHF bands

Annex 3	 –	Planning criteria for ISDB-T digital terrestrial television systems in the VHF/UHF bands

Annex 4 –	Other planning factors

Annex 5 –	Subjective comparison method (SCM) with a reference interferer for assessment protection ratios for analogue television system

Annex 6 –	Test methods for protection ratio measurements for wanted digital terrestrial signals

Annex 7 –	Tropospheric and continuous interference

General

The RF protection ratio is the minimum value of wanted-to-unwanted signal ratio, usually expressed in decibels at the receiver input.

The reference level of the digital signal is defined as the r.m.s. value of the emitted signal power within the channel bandwidth. It should preferably be measured with a thermal power meter. All protection ratio values for wanted digital signals are to be measured with a 60 dBm receiver input power.

The reference level of the analogue vision-modulated signal is defined as the r.m.s. value of the vision carrier at peaks of the modulation envelope. All protection ratio values for wanted analogue signals are measured with a receiver input power of 39 dBm (70 dB(V) at 75 ).

# 1	Wanted digital terrestrial television systems

The protection ratios for digital terrestrial television systems apply to both continuous and tropospheric interference. The protection ratios refer to the centre frequency of the wanted digital terrestrial television system.

Because a digital television receiver needs to operate successfully in the presence of high level analogue signals on nearby channels, a high degree of receiver front-end linearity is required.

The protection ratios for digital terrestrial television systems as the interfering system are those for the case where the wanted and unwanted signals are not synchronized and/or do not have a common programme source. Results relevant to single frequency networks (SFN) are yet to be developed.

For the digital terrestrial television system ATSC the protection ratios are measured for a BER = 3  10–6 at the input of the MPEG-2 demultiplexer.

For the digital terrestrial television systems (digital video broadcasting-terrestrial (DVB-T) and integrated service digital broadcasting-terrestrial (ISDB-T)) the protection ratios are measured between the inner and outer codes, before Reed Solomon decoding, for a BER  2  104; this corresponds to a BER < 1  10–11 at the input of the MPEG-2 demultiplexer. For domestic receivers it may not be possible to measure the BER before Reed-Solomon decoding. The BER for such cases is under study.

To reduce the number of measurements and tables, it is proposed that protection ratio measurements for DVB-T systems should preferably be made with the following three modes shown in Table 1. Protection ratio values for the different required operational modes for fixed, portable or mobile reception can be calculated from the given measured values. A formula for calculation is still under study.

TABLE  1

Proposed preferable DVB-T mode types for measurements on protection ratios

| Modulation | Code rate | C/N (1) (dB) | Bit rate (2) (Mbit/s) |
| --- | --- | --- | --- |
| QPSK | 2/3 | 6.9 |  7 |
| 16-QAM | 2/3 | 13.1 |  13 |
| 64-QAM | 2/3 | 18.7 |  20 |
| (1) The figures are given for a Gaussian channel (including a typical implementation margin) for a BER < 1  10–11. (2) For a guard interval of 1/4. |  |  |  |

# 2	Wanted analogue terrestrial television systems

Measurements of protection ratios for the vision signal of a wanted analogue terrestrial television system should preferably be made with the subjective comparison method with a sine-wave reference interferer described in Annex 5.

The values of protection ratio quoted apply to interference produced by a single source. Except where otherwise stated, the ratios apply to tropospheric, T, interference and correspond closely to a slightly annoying impairment condition. They are considered to be acceptable only if the interference occurs for a small percentage of the time, not precisely defined but generally considered to be between 1% and 10%. For substantially non-fading unwanted signals, it is necessary to provide a higher degree of protection and ratios appropriate to continuous, C, interference should be used. (See Annex 7.)

When the wanted signal is an analogue television signal, two or more protection ratio values should be considered, one for the protection ratio of the vision signal and others for the protection ratios of sound signals. The most stringent value should then be used.

Significantly strong wanted input signals can require higher protection ratio values because of non-linear effects in the receiver.

For 625-line systems, the reference impairment levels are those which correspond to co-channel protection ratios of 30 dB and 40 dB, when two-thirds line offset is used, see Recommendation ITU-R BT.655. These conditions approximate to impairment grades 3 (slightly annoying) and 4 (perceptible but not annoying) and apply to tropospheric, T, and continuous, C, interference, respectively.

TABLE OF CONTENTS

Page

Annex 1 – Planning criteria for ATSC digital television systems in the VHF/UHF bands		12

1	Protection ratios for ATSC wanted digital terrestrial television signals		12

1.1	Protection of an ATSC digital terrestrial television signal interfered with by an ATSC digital terrestrial television signal		12

1.2	Protection of ATSC digital terrestrial television interfered with by analogue terrestrial television		13

1.2.1	Protection from co-channel interference		13

1.2.2	Protection from lower adjacent channel (N – 1) interference		13

1.2.3	Protection from upper adjacent channel (N + 1) interference		14

1.2.4	Protection from other channel interference		14

2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals		14

2.1	Protection ratios for 525-line television systems		14

2.1.1	Protection for vision signals interfered with by ATSC digital television		14

2.2	Protection ratios for 625-line television systems		15

2.2.1	Protection of wanted vision signals interfered with by ATSC digital terrestrial television		15

Page

2.2.1.1	Protection from co-channel interference		15

2.2.1.2	Protection from lower adjacent channel interference		16

2.2.1.3	Protection from upper adjacent channel interference		16

3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals		16

3.1	Protection for NTSC sound signals (BTSC MTS system and SAP) interfered with by an ATSC digital television signal		16

4	Minimum field strengths for ATSC terrestrial digital television		17

Appendix 1 to Annex 1 – Derivation by the figure of merit method		18

Annex 2 – Planning criteria for DVB-T digital television systems in the VHF/UHF bands		19

1	Protection ratios for DVB-T wanted digital terrestrial television signals		19

1.1	Protection of a DVB-T digital terrestrial television signal interfered with by a DVB-T digital terrestrial television signal		20

1.2	Protection of DVB-T digital terrestrial television interfered with by analogue terrestrial television		21

1.2.1	Protection from co-channel interference		21

1.2.2	Protection from lower adjacent channel (N – 1) interference		22

1.2.3	Protection from upper adjacent channel (N + 1) interference		23

1.2.4	Protection from overlapping channel interference		23

1.3	Protection of DVB-T digital terrestrial television signal from CW or FM signals		24

1.4	Protection of DVB-T digital terrestrial television signals from T-DAB signals		25

1.5	Protection of DVB-T from wideband signals other than terrestrial broadcasting		26

1.5.1	Protection ratios for DVB-T interfered with by the fixed service (relocatable system)		26

Page

2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals		26

2.1	Protection ratios for 625-line television systems		26

2.1.1	Protection of wanted vision signals interfered with by DVB-T digital terrestrial television		26

2.1.1.1	Protection from co-channel interference		27

2.1.1.2	Protection from lower adjacent channel interference		27

2.1.1.3	Protection from upper adjacent channel interference		28

2.1.1.4	Protection from image channel interference		28

2.1.1.5	Protection from overlapping interference		29

3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals		30

3.1	Protection for FM, AM and NICAM sound signals of analogue television systems interfered with by DVB-T digital terrestrial television signals		31

4	Protection ratios for a T-DAB signal interfered with by an unwanted digital terrestrial television signal		32

5	Minimum field strengths for DVB-T terrestrial digital television		33

Appendix 1 to Annex 2 – Derivation by the voltage method		34

Annex 3 – Planning criteria for ISDB-T digital television systems in theVHF/UHF bands		35

1	Protection ratios for ISDB-T wanted digital terrestrial television signals		35

1.1	Protection of an ISDB-T digital terrestrial television signal interfered with by an ISDB-T digital television signal		36

1.2	Protection of ISDB-T digital terrestrial television interfered with by analogue terrestrial television		37

1.2.1	Protection from co-channel interference		37

1.2.2	Protection from lower adjacent channel (N  1) interference		37

1.2.3	Protection from upper adjacent channel (N + 1) interference		38

Page

2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ISDB-T digital terrestrial television signals		38

2.1	Protection ratios for 525-line television systems		38

2.1.1	Protection for NTSC vision signals interfered with by ISDB-T digital television signals		38

3	Protection ratios for sound signals of wanted analogue terrestrial television systems interfered with by ISDB-T unwanted digital terrestrial television system		39

3.1	Protection for NTSC sound signals interfered with by a ISDB-T digital television signal		39

4	Minimum field strengths for ISDB-T terrestrial digital television		39

Appendix 1 to Annex 3 – Derivation by the figure of merit method		41

Annex 4 – Other planning factors		42

1	Field strength distribution with location		42

2	Reception using portable television equipment		43

3	Receiving antenna discrimination		44

Annex 5 – Subjective comparison method (SCM) with a reference interferer for assessment 	of protection ratios for analogue television systems		44

1	Introduction		44

2	The SCM for assessment of protection ratios using a sine-wave reference		45

2.1	General description		45

2.2	Realization of the reference interferer		46

2.3	Test conditions		46

2.4	Presentation of the results		46

3	Table of important parameters		47

Annex 6 – Test methods for protection ratio measurements for wanted digital terrestrial
	 signals		47

1	Background		47

2	The subjective failure point (SFP) method for protection ratio measurements		47

Annex 7 – Tropospheric and continuous interference		48

List of Tables

Page

Table 1	–	Proposed preferable DVB-T mode types for measurements on protection
		ratios....................................................................................................................		3

Annex 1	–	Planning criteria for ATSC digital television systems in the VHF/UHF bands		12

Table 2	–	Co-channel protection ratios (dB) for an ATSC signal interfered with by an
		ATSC signal		12

Table 3	–	Protection ratios (dB) for ATSC interfered with by an ATSC signal in the
		lower (N – 1) and upper (N + 1) adjacent channels		12

Table 4	–	Protection ratios (dB) for an ATSC signal interfered with by an ATSC signal in
		channel N  2 and other out-of-band channels		13

Table 5	–	Co-channel protection ratios (dB) for an ATSC signal interfered with by an
		analogue television signal		13

Table 6	 –	Protection ratios (dB) for lower adjacent channel (N – 1) interference for an
		ATSC signal interfered with by an analogue television signal including sound		13

Table 7	 –	Protection ratios (dB) for upper adjacent channel (N + 1) interference for an
		ATSC signal interfered with by an analogue television signal		14

Table 8	 –	Protection ratios (dB) for an ATSC 6 MHz signal interfered with by an
		M/NTSC signal at other out-of-band channels		14

Table 9	 –	Protection ratios (dB) for a wanted analogue vision signal (NTSC, 6 MHz)
		interfered with by an unwanted ATSC signal		15

Table 10	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an
		unwanted ATSC 6 MHz signal		15

Table 11	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an
		ATSC 6 MHz signal (lower adjacent channel)		16

Table 12	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by
		an ATSC 6 MHz signal (upper adjacent channel)		16

Table 13	 –	Derivation by the figure of merit method ATSC 6 MHz system		17

Annex 2	 –	Planning criteria for DVB-T digital television systems in the VHF/UHF
		bands		19

Page

Table 14	 –	Co-channel protection ratios (dB) for a DVB-T signal interfered with by a
		DVB-T signal		20

Table 15	 –	Protection ratios (dB) for a DVB-T signal interfered with by a DVB-T
		signal in the lower (N – 1) and upper (N + 1) adjacent channels		21

Table 16	 –	Co-channel protection ratios (dB) for DVB-T 7 MHz and 8 MHz signals
		interfered with by analogue television (non-controlled frequency condition)
		signals		21

Table 17	 –	Protection ratios (dB) for lower adjacent channel (N – 1) interference for
		DVB-T 7 MHz and 8 MHz signals interfered with by analogue television
		signals including sound		22

Table 18	 –	Protection ratios (dB) for upper adjacent channel (N + 1) interference for
		DVB-T 7 MHz and 8 MHz signals interfered with by an analogue television
		signal		23

Table 19	 –	Protection ratios (dB) for a DVB-T 8 MHz signal interfered with by an
		overlapping PAL B signal including sound		23

Table 20	 –	Protection ratios (dB) for a DVB-T 7 MHz signal interfered with by an
		overlapping 7 MHz analogue TV signal including sound		24

Table 21	 –	Protection ratios (dB) for a DVB-T 8 MHz signal interfered with by an
		overlapping 8 MHz analogue TV signal including sound		24

Table 22	 –	Co-channel protection ratios (dB) for a DVB-T 8 MHz 64-QAM code rate 2/3
		signal interfered with by a CW or a FM carrier (non-controlled frequency
		offset)		24

Table 23	 –	Co-channel protection ratios (dB) for a DVB-T 7 MHz 64-QAM code rate 2/3
		signal interfered with by a CW carrier (controlled frequency offset)		25

Table 24	 –	Co-channel protection ratios (dB) for a DVB-T 7 MHz and 8 MHz signal interfered with by a T-DAB signal		25

Table 25	 –	Protection ratios (dB) for a DVB-T 7 MHz and 8 MHz signal interfered with by a T-DAB signal in the lower (N – 1) on upper (N  1) adjacent channels		26

Table 26	 –	Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3 signal interfered with by emissions of the fixed service		26

Table 27	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an
		unwanted DVB-T 8 MHz signal		27

Page

Table 28	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an
		unwanted DVB-T 7 MHz signal		27

Table 29	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by
		DVB-T 7 MHz and 8 MHz signals (lower adjacent channel)		27

Table 30	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by
		DVB-T 7 MHz and 8 MHz signals (upper adjacent channel)		28

Table 31	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by a
		DVB-T 8 MHz signal (image channel)		28

Table 32	 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by a
		DVB-T 7 MHz signal (image channel)		28

Table 33	 –	Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals
		interfered with by a DVB-T 7 MHz signal (overlapping channels)		29

Table 34	 –	Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals
		interfered with by a DVB-T 8 MHz signal (overlapping channels)		30

Table 35	 –	Co-channel protection (dB) ratios for a wanted sound signal interfered with by
		DVB-T digital terrestrial television system		31

Table 36	 –	Protection ratios (dB) for a wanted FM sound signal interfered with by a
		DVB-T 7 MHz signal (overlapping channels)		32

Table 37	 –	Protection ratios (dB) for a wanted AM sound signal interfered with by a
		DVB-T 8 MHz signal for different frequency offsets (upper adjacent channel)		32

Table 38	 –	Protection ratios (dB) for a T-DAB  signal interfered with by a DVB-T 8 MHz
		signal		32

Table 39	 –	Protection ratios (dB) for a T-DAB signal interfered with by a DVB-T 7 MHz
		signal		33

Table 40	 –	Derivation by the voltage method DVB-T 8 MHz system		33

Annex 3	 –	Planning criteria for ISDB-T digital television systems in the VHF/UHF bands		35

Table 41	 –	Co-channel protection ratios (dB) for ISDB-T 6 MHz interfered with by
		ISDB-T 6 MHz		36

Page

Table 42	 –	Protection ratios (dB) for ISDB-T 6 MHz interfered with by ISDB-T 6 MHz in
		lower adjacent channel (N - 1)		36

Table 43	 –	Protection ratios (dB) for ISDB-T 6 MHz interfered with by ISDB-T 6 MHz in
		upper adjacent channel (N + 1)		36

Table 44	 –	Co-channel protection ratios (dB) for ISDB-T 6 MHz interfered with by
		analogue television		37

Table 45	 –	Protection ratios (dB) for lower adjacent channel (N - 1) interference for
		ISDB-T 6 MHz interfered with by NTSC signals including sound		37

Table 46	 –	Protection ratios (dB) for upper adjacent channel (N + 1) interference for
		ISDB-T 6 MHz interfered with by NTSC 6 MHz signal		38

Table 47	 –	Protection ratios (dB) for analogue vision signal (NTSC, 6 MHz) interfered
		with by ISDB-T signal		38

Table 48	 –	Sound quality related to the vision protection ratio of grade 3 when NTSC
		6 MHz signal interfered with by ISDB-T 6 MHz signal		39

Table 49	 –	Derivation by the voltage method ISDB-T 6 MHz system		40

Annex 5	 –	Subjective comparison method (SCM) with a reference interferer for
		assessment of protection ratios for analogue television systems		44

Table 50	 –	Basic terms and relations for the SCM		47

ANNEX  1

Planning criteria for ATSC digital television systems in the VHF/UHF bands

# 1	Protection ratios for ATSC wanted digital terrestrial television signals

Tables 2 to 4 and 5 to 8 show protection ratios for an ATSC digital terrestrial television signal interfered with by an ATSC digital terrestrial television signal, and by an analogue terrestrial television signal, respectively.

## 1.1	Protection of an ATSC digital terrestrial television signal interfered with by an ATSC digital terrestrial television signal

TABLE  2

Co-channel protection ratios (dB) for an ATSC signal 
interfered with by an ATSC signal

| Wanted signal | Unwanted signal |
| --- | --- |
|  | ATSC 6 MHz |
| ATSC 6 MHz | 15 19(1) |
| (1) Based on equally partitioned noise and interference. |  |

TABLE  3

Protection ratios (dB) for ATSC interfered with by an ATSC
signal in the lower (N – 1) and upper (N  1) adjacent channels

| Channel | N – 1 | N  1 |
| --- | --- | --- |
| ATSC 6 MHz | –27 | –27 |

The protection ratios are given in dB and apply to both continuous and tropospheric interference.

TABLE  4

Protection ratios (dB) for an ATSC signal interfered with by an ATSC
signal in channel N  2 and other out-of-band channels

| Channel | N  2 and other out-of-band channels |
| --- | --- |
| ATSC 6 MHz | –58 |

## 1.2	Protection of ATSC digital terrestrial television interfered with by analogue terrestrial television

### 1.2.1	Protection from co-channel interference

TABLE  5

Co-channel protection ratios (dB) for an ATSC signal interfered with
by an analogue television signal

| Wanted signal | Unwanted signal (Analogue TV signal including sound carriers) |  |
| --- | --- | --- |
|  | M/NTSC | PAL B |
| ATSC 6 MHz | 2 7(1) | 9 |
| (1) Using a comb filter in the digital television receiver and C/N of 19 dB. |  |  |

### 1.2.2	Protection from lower adjacent channel (N – 1) interference

TABLE  6

Protection ratios (dB) for lower adjacent channel (N – 1) interference for an ATSC signal interfered with by an analogue television signal including sound

| Wanted signal | Unwanted signal (Analogue TV signal including sound carriers) |
| --- | --- |
|  | M/NTSC |
| ATSC 6 MHz | –48 |

### 1.2.3	Protection from upper adjacent channel (N + 1) interference

TABLE  7

Protection ratios (dB) for upper adjacent channel (N  1) interference 
for an ATSC signal interfered with by an analogue television signal

| Wanted signal | Unwanted signal (Analogue TV signal including sound carriers) |
| --- | --- |
|  | M/NTSC |
| ATSC 6 MHz | –49 |

### 1.2.4	Protection from other channel interference

TABLE  8

Protection ratios (dB) for an ATSC 6 MHz signal interfered with by 
an M/NTSC signal at other out-of-band channels

| Wanted signal | Unwanted signal | Unwanted channels | Protection ratio |
| --- | --- | --- | --- |
| ATSC | M/NTSC | N  2 to N  8 | –58 |

# 2 	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals

Table 9 and Tables 10 to 12 show protection ratios for wanted 525-line and 625-line, respectively, analogue television signals interfered with by ATSC digital terrestrial television signals.

## 2.1	Protection ratios for 525-line television systems

### 2.1.1	Protection for vision signals interfered with by ATSC digital television

In this section the protection ratios for an analogue wanted signal interfered by an unwanted ATSC digital signal apply only to the interference to the vision and colour carriers.

TABLE  9

Protection ratios (dB) for a wanted analogue vision signal (NTSC, 6 MHz)
interfered with by an unwanted ATSC signal

| Unwanted digital channel | Tropospheric interference grade 3 | Continuous interference grade 4 |
| --- | --- | --- |
| N – 1 (lower) | –16 |  |
| N (co-channel) | 34 |  |
| N + 1 (upper) | –17 |  |
| N + 14 (image) | –33 |  |
| N + 15 (image) | –31 |  |
| N  2 | –24 |  |
| N  3 | –30 |  |
| N  4 | –25 |  |
| N  7 | –34 |  |
| N  8 | –32 |  |

## 2.2	Protection ratios for 625-line television systems

### 2.2.1	Protection of wanted vision signals interfered with by ATSC digital terrestrial television

In this section the protection ratios for an analogue wanted signal interfered by an unwanted digital signal relate only to the interference to the vision signal.

The protection ratio values given are related to an out-of-channel spectrum attenuation of the unwanted DVB-T transmitter of 40 dB.

### 2.2.1.1	Protection from co-channel interference

TABLE  10

Protection ratios (dB) for a wanted analogue vision signal interfered with by
an unwanted ATSC 6 MHz signal

| Wanted signal: analogue system | Unwanted signal: ATSC 6 MHz |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL | 38 | 45 |

#### 2.2.1.2	Protection from lower adjacent channel interference

TABLE  11

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
an ATSC 6 MHz signal (lower adjacent channel)

| Wanted signal: analogue system | Unwanted signal: ATSC 6 MHz signal (lower adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL | – 7 | – 1 |

#### 2.2.1.3	Protection from upper adjacent channel interference

TABLE  12

Protection ratios (dB) for a wanted analogue vision signal interfered with by
an ATSC 6 MHz signal (upper adjacent channel)

| Wanted signal: analogue system | Unwanted signal: ATSC 6 MHz signal (upper adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL | – 7 | 0 |

# 3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals

## 3.1	Protection for NTSC sound signals (BTSC MTS system and SAP) interfered with by an ATSC digital television signal (see Note 1)

In the case of an unwanted upper adjacent digital channel N + 1 the audio signals degrade before the vision signal. The protection ratio value for the interference into the BTSC MTS and SAP sound signals was measured with 12 dB. (Vision protection ratio for N + 1 is 17 dB.) The 12 dB sound protection ratio figure is related to the wanted NTSC vision carrier level.

NOTE 1 – BTSC MTS:	broadcast television system committee multichannel television sound; SAP:  sound audio programme.

# 4	Minimum field strengths for ATSC terrestrial digital television

TABLE  13

Derivation by the figure of merit method
ATSC 6 MHz system(1)

| Planning parameter(2) | Low VHF 54-88 MHz | High VHF 174-216 MHz | UHF 470-806 MHz |
| --- | --- | --- | --- |
| Frequency (MHz) | 69 | 194 | 615 |
| C/N (dB) | 19.5 | 19.5 | 19.5 |
| k (dB) | –228.6 | –228.6 | –228.6 |
| B (dB(Hz)) (6 MHz) | 67.8 | 67.8 | 67.8 |
| G1m2 (dB) | – 1.8 | 7.3 | 17.2 |
| GD (dB) | 6 | 8 | 10 |
| GI (dB) | 8.2 | 10.2 | 12.2 |
| Transmission line loss (dB) line | 1.1 | 1.9 | 3.3 |
| Antenna 300/75 balun loss (dB) balun | 0.5 | 0.5 | 0.5 |
| Receiver noise figure (dB) | 5 | 5 | 10 |
| Trx (K) | 627.1 | 627.1 | 2 610 |
| Tline (K) | 65.0 | 102.9 | 154.4 |
| LNA noise figure (dB) | 5 | 5 | 5 |
| LNA gain (dB) | 20 | 20 | 20 |
| TLNA (dB) | 627.1 | 627.1 | 627.1 |
| Tbalun (K) | 31.6 | 31.6 | 31.6 |
| Ta (K) | 9 972.1 | 569.1 | Negligible |
| Ta balun (K) | 8 885.1 | 507.1 | Negligible |
| Tline/ G (K) | 0.8 | 1.6 | 3.3 |
| Trx/ G (K) | 8.1 | 9.7 | 55.8 |
| Te (K) | 9 552.6 | 1 176.8 | 717.8 |
| 10 log(Te) (dB(K)) | 39.8 | 30.7 | 28.6 |
| GA (dB) | 7.7 | 9.7 | 11.7 |
| Erx (dB(V/m))(3) (TBC) | 35 | 33 | 39 |
| (1) The values in the Table were calculated assuming C/N with typical multipath reception impairment and equal partitioning for noise and interference. The receiving system model is a typical receiving installation located near the edge of coverage and consists of an externally mounted antenna, a low noise amplifier (LNA) mounted at the antenna, an interconnecting downlead cable and an ATSC receiver. (2) For definitions see Appendix 1 to Annex 1. (3) For formula see Appendix 1 to Annex 1. |  |  |  |

APPENDIX  1

TO  ANNEX  1

Derivation by the figure of merit method

Required field strength

Erx (dB(V/m))     (dB(W/m2))  10 log(120 )

C/N     – Glm2  GA / Te – k – Brf

Erx (dB(V/m))		 (dB(W/m2))  25.8 (dB)  120 (dB)

	145.8  C/N  Glm2 – GA / Te  10 log(k)  10 log(Brf )

Erx :	required field strength at the receive system antenna

 :	power flux-density at the receive system antenna

C/N :	carrier-to-noise ratio

Glm2:	gain of 1 m2

GA /Te :	figure of merit of the receive system

k :	Boltzmann’s constant

Brf :	system equivalent noise bandwidth.

Receive system figure of merit

(For receiving system model with LNA)

GA / Te    (G  L ) / (balun Ta  Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))

Receiver noise temperature

Trx    (10NF / 10 – 1)    290

LNA noise temperature

TLNA    (10NF / 10 – 1)    290

Transmission line noise temperature

Tline    (1 – line)    290

Balun noise temperature

Tbalun    (1 – balun)    290

Antenna noise temperature

Ta    10(6.63 – 2.77(log f ))    290     (for dipole antenna)

where f is expressed in MHz.

Antenna noise temperature (referred to LNA input)

 Ta    Ta(balun)

System noise temperature

Te		(balun Ta  Tbalun  TLNA  Tline /(line GLNA)  Trx / (line GLNA))

Te (dB(K))		10 log(balun Ta  Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))

or		10 log(Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))  Next

when Ta is not known.

Gain of 1 m2

G1m2    10 log(4  / 2)

Data

GI :	antenna gain (isotropic) (dB)

L :	transmission line loss (dB)

line :	transmission line loss (numeric ratio)

Ta :	antenna noise temperature (K)

Trx :	receiver noise temperature (K)

nf :	noise factor (numeric ratio)

NF :	noise figure (dB)

T0 :	reference temperature  290 K

 :	wavelength of frequency of operation

GA :	system gain (dB)

Te :	system noise temperature (K)

Next :	dB value representing the contribution due to external noise

k :	Boltzmann’s constant 1.38  10–23 (–228.6 dB)

B :	system equivalent noise bandwidth (dB(Hz))

balun :	antenna 300/75 Balun loss (numeric ratio)

LNA :	low noise amplifier

TLNA :	LNA noise temperature (K)

ANNEX  2

Planning criteria for DVB-T digital television systems 
in the VHF/UHF bands

# 1	Protection ratios for DVB-T wanted digital terrestrial television signals

Tables 14 to 15, 16 to 21, 22 to 23 and 24 to 25 show protection ratios for the DVB-T wanted digital terrestrial television signals interfered with:

	by DVB-T digital terrestrial television signals,

	by analogue terrestrial television signals,

	by a single continuous wave (CW) or FM carrier

	by terrestrial digital audio broadcasting (T-DAB) signals,

respectively.

## 1.1	Protection of a DVB-T digital terrestrial television signal interfered with by a DVB-T digital terrestrial television signal

TABLE  14

Co-channel protection ratios (dB) for a DVB-T signal
interfered with by a DVB-T signal

| Modulation | Code rate | Gaussian Channel | Rice channel | Rayleigh channel |
| --- | --- | --- | --- | --- |
| QPSK | 1/2 | 5 | 6 | 8 |
| QPSK | 2/3 | 7 | 8 | 11 |
| 16-QAM | 1/2 | 10 | 11 | 13 |
| 16-QAM | 2/3 | 13 | 14 | 16 |
| 16-QAM | 3/4 | 14 | 15 | 18 |
| 64-QAM | 1/2 | 16 | 17 | 19 |
| 64-QAM | 2/3 | 19 | 20 | 23 |
| 64-QAM | 3/4 | 20 | 21 | 25 |

Protection ratios are given for three types of propagation channels (i.e. Gaussian, Ricean and Rayleigh). For fixed and portable reception, the values relevant to the Ricean and Rayleigh channels, respectively, should be adopted.

The same protection ratios should be applied for DVB-T systems with 6, 7 and 8 MHz bandwidth.

Protection ratios are rounded to the nearest integer.

For overlapping channel, in absence of measurement information, and if the overlapping bandwidth between the wanted and unwanted signals is less than 1 MHz, the protection ratio, PR, should be extrapolated from the co-channel ratio figure as follows:

PR    CCI    10 log10(BO/BW )

where:

CCI :	co-channel protection ratio

BO :	bandwidth (MHz) in which the two DVB-T signals are overlapping

BW :	bandwidth (MHz) of the wanted signal

PR   	30 dB should be used when the above formula gives PR <  30 dB.

However, further studies are needed on this subject.

TABLE  15

Protection ratios (dB) for a DVB-T signal interfered with by a DVB-T signal
in the lower (N – 1) and upper (N  1) adjacent channels

| Channel | N – 1 | N  1 |
| --- | --- | --- |
| PR | – 30 | – 30 |

The protection ratio is given in dB and applies to both continuous and tropospheric interference.

The values given apply to the case where wanted and unwanted DVB-T signals have the same channel width. Other combinations of channel width need further studies.

It is known from measurements of existing receivers that they permit lower protection ratios.

But for planning purposes it is an advantage to have this value.

## 1.2	Protection of DVB-T digital terrestrial television interfered with by analogue terrestrial television

### 1.2.1	Protection from co-channel interference

TABLE  16

Co-channel protection ratios (dB) for DVB-T 7 MHz and 8 MHz signals interfered with by analogue television (non-controlled frequency condition) signals

| Modulation | Code rate | Gaussian channel | Rice channel | Rayleigh channel |
| --- | --- | --- | --- | --- |
| QPSK | 1/2 | –12 |  | –12 |
| QPSK | 2/3 | –8 |  | –8 |
| QPSK | 3/4 | –4 |  |  |
| QPSK | 5/6 | 3 |  |  |
| QPSK | 7/8 | 9 |  |  |
| 16-QAM | 1/2 | –8 |  | –8 |
| 16-QAM | 2/3 | –3 |  | 3 |
| 16-QAM | 3/4 | 0 |  | 5 |
| 16-QAM | 5/6 | 9 |  |  |
| 16-QAM | 7/8 | 16 |  |  |
| 64-QAM | 1/2 | –3 |  | 3 |
| 64-QAM | 2/3 | 3 |  | 6 |
| 64-QAM | 3/4 | 9 |  | 15 |
| 64-QAM | 5/6 | 15 |  |  |
| 64-QAM | 7/8 | 20 |  |  |

NOTE 1 – The PAL/SECAM values are valid for the following sound carrier modes:

–	MONO FM with a single sound carrier at 10 dB referred to the vision carrier;

–	DUAL FM and FM + NICAM with two sound carriers at 13 dB and 20 dB level;

–	AM + NICAM with two sound carriers at respectively 10 dB and 27 dB level.

According to the available measurements, the same protection ratio values are applicable for 2k and 8k modes.

In all Tables except Table 23, the so-called non-controlled conditions are used.

Actual measurements of protection ratio values will reflect the cyclic variation that occurs when the offset between a wanted DVB-T signal and an unwanted analogue signal is varied over a frequency range equivalent to the spacing between carriers of coded orthogonal frequency division multiplex (COFDM) systems. The protection ratios given represent a conservative, but realistic, value that covers the expected offset performance of existing receivers. The adoption of fine offset between COFDM signals and interfering analogue TV signals will permit the achievement of up to 3 dB improvement in protection ratio. The required transmitter frequency stability is similar to that of analogue precision offset, that means a range of some  1 Hz.

Protection ratios for DVB-T 6 MHz are missing due to lack of measurement results.

### 1.2.2	Protection from lower adjacent channel (N – 1) interference

TABLE 17

Protection ratios (dB) for lower adjacent channel (N – 1) interference for
DVB-T 7 MHz and 8 MHz signals interfered with by analogue 
television signals including sound

| Wanted signal |  | Unwanted signal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Constellation | Code rate | PAL B | PAL G, B1 | PAL I | PAL D, K | SECAM L | SECAM D, K |
| QPSK | 1/2 |  | –44 |  |  |  |  |
| QPSK | 2/3 | –44 | –44 |  |  |  |  |
| 16-QAM | 1/2 |  | –43 | – 43 |  |  |  |
| 16-QAM | 2/3 | –42 | –42 |  |  |  |  |
| 16-QAM | 3/4 |  | –38 |  |  |  |  |
| 64-QAM | 1/2 |  | –40 | –38 |  |  |  |
| 64-QAM | 2/3 | –35 | –35 | –34 |  | –35 | –37 |
| 64-QAM | 3/4 |  | –32 |  |  |  |  |

All values applicable for fixed and portable reception conditions.

### 1.2.3	Protection from upper adjacent channel (N  1) interference

TABLE  18

Protection ratios (dB) for upper adjacent channel (N  1) interference for DVB-T 
7 MHz and 8 MHz signals interfered with by an analogue television signal

| Wanted signal |  | Unwanted signal |
| --- | --- | --- |
| Constellation | Code rate | PAL/SECAM |
| QPSK | 2/3 | – 47 |
| 16-QAM | 2/3 | – 43 |
| 64-QAM | 2/3 | –38 |

### 1.2.4	Protection from overlapping channel interference

TABLE  19

Protection ratios (dB) for a DVB-T 8 MHz signal interfered
with by an overlapping PAL B signal including sound

| Unwanted signal: PAL B analogue TV system | Wanted signal: DVB-T, 8 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 9.75 | 9.25 | 8.75 | 8.25 | 6.75 | 3.95 | 3.75 | 2.75 | 0.75 | 2.25 | 3.25 | 4.75 | 5.25 |
| PR | 37 | 14 | 8 | 4 | 2 | 1 | 3 | 3 | 3 | 2 | 1 | 29 | 36 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

TABLE  20

Protection ratios (dB) for a DVB-T 7 MHz signal interfered with by an 
overlapping 7 MHz analogue TV signal including sound

| Unwanted signal: 7 MHz analogue TV system | Wanted signal: DVB-T, 7 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 9.25 | 8.75 | 8.25 | 7.75 | 6.25 | 3.45 | 3.25 | 2.25 | 1.25 | 0 | 1.75 | 2.75 | 4.25 | 4.75 |
| PR | 35 | 12 | 11 | 5 | 3 | 1 | 4 | 1 | 0 | 2 | 5 | 5 | 36 | 38 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

TABLE  21

Protection ratios (dB) for a DVB-T 8 MHz signal interfered with by an 
overlapping 8 MHz analogue TV signal including sound

| Unwanted signal: 8 MHz analogue TV system | Wanted signal: DVB-T, 8 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 10.25 | 9.75 | 9.25 | 8.75 | 7.25 | 3.45 | 3.25 | 2.25 | 1.25 | 0 | 1.75 | 2.75 | 4.25 | 4.75 |
| PR | 35 | 12 | 11 | 5 | 3 | 1 | 4 | 1 | 0 | 2 | 5 | 5 | 36 | –38 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

## 1.3	Protection of DVB-T from digital terrestrial television signal from CW or FM signals

TABLE  22

Co-channel protection ratios (dB) for a DVB-T 8 MHz 64-QAM code rate 2/3
signal interfered with by a CW or a FM carrier
(non-controlled frequency offset)

| Unwanted signal: CW or FM carrier | Wanted signal: DVB-T, 8 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 12 | 4.5 | 3.9 | 0 | 3.9 | 4.5 | 12 |
| PR | 38 | 33 | 3 | 3 | 3 | 33 | 38 |

The given protection ratio Tables can be used for interfering signals with narrow bandwidth e.g. analogue sound carriers or non-broadcast services.

TABLE  23

Co-channel protection ratios (dB) for a DVB-T 7 MHz 64-QAM code rate 2/3
signal interfered with by a CW carrier
(controlled frequency offset)

| Unwanted signal: CW carrier | Wanted signal: DVB-T, 7 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f (MHz) | –8 | – 4 | –3 | 0 | 3 | 4 | 8 |
| PR | – 48 | – 41 | –8 | –9 | –6 | –39 | – 48 |

The given protection ratio Tables can be used for interfering signals with narrow bandwidth e.g. analogue sound carriers or non-broadcast services. It should be noted that the fine structure of the protection ratio versus frequency offset between the OFDM signal and the interfering CW signal exhibits a cyclic variation. The values shown in Table 23 are for the optimum offset.

## 1.4	Protection of DVB-T digital terrestrial television signals from T-DAB signals

TABLE  24

Co-channel protection ratios (dB) for a DVB-T 7 MHz and 8 MHz 
signal interfered with by a T-DAB signal

| Wanted signal DVB-T |  | PR |
| --- | --- | --- |
| Constellation | Code rate |  |
| QPSK | 1/2 | 10 |
| QPSK | 2/3 | 12 |
| QPSK | 3/4 | 14 |
| 16-QAM | 1/2 | 15 |
| 16-QAM | 2/3 | 18 |
| 16-QAM | 3/4 | 20 |
| 64-QAM | 1/2 | 20 |
| 64-QAM | 2/3 | 24 |
| 64-QAM | 3/4 | 26 |
| 64-QAM | 7/8 | 31 |

NOTE 1 – The given protection ratios for DVB-T signals represent the worst case of interference from T-DAB.

TABLE  25

Protection ratios (dB) for a DVB-T 7 MHz and 8 MHz signal interfered with by 
a T-DAB signal in the lower (N – 1) or upper (N  1) adjacent channels

| Channel | N – 1 | N  1 |
| --- | --- | --- |
| PR | –30 | –30 |

The protection ratio is given in dB.

## 1.5	Protection of DVB-T from wideband signals other than terrestrial broadcasting

### 1.5.1 	Protection ratios for DVB-T interfered with by the fixed service (relocatable system)

TABLE  26

Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3 signal 
interfered with by emissions of the fixed service

| Df (MHz) | –12 | –4.5 | –3.75 | 0 | 3.75 | 4.5 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PR (dB) | –45 | –27 | 1 | 4 | 1 | –27 | –45 |
| Df is the difference between the centre frequencies. |  |  |  |  |  |  |  |

Technical characteristics of the interferer

–	Modulation 2-FSK

–	Bandwidth: 750 kHz (3 dB)

# 2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals

Tables 27 to 34 show protection ratios for a wanted 625-line analogue television signal interfered with by a DVB-T digital terrestrial television signal.

## 2.1	Protection ratios for 625-line television systems

### 2.1.1	Protection of wanted vision signals interfered with by DVB-T digital terrestrial television signal

In this section the protection ratios for an analogue wanted signal interfered by an unwanted DVB-T digital signal relate only to the interference to the vision signal.

The protection ratio values given are related to an out-of-channel spectrum attenuation of the unwanted DVB-T transmitter of 40 dB.

#### 2.1.1.1	Protection from co-channel interference

TABLE  27

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
an unwanted DVB-T 8 MHz signal

| Wanted signal: analogue system | Unwanted signal: DVB-T, 8 MHz |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B, D, D1, G, H, K/PAL | 34 | 40 |
| I/PAL | 37 | 41 |
| B, D, K, L/SECAM | 35 | 41 |

TABLE  28

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
an unwanted DVB-T 7 MHz signal

| Wanted signal: analogue system | Unwanted signal: DVB-T, 7 MHz |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL, B/SECAM | 35 | 41 |

#### 2.1.1.2	Protection from lower adjacent channel interference

TABLE  29

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
DVB-T 7 MHz and 8 MHz signals (lower adjacent channel)

| Wanted signal: analogue system | Unwanted signal: DVB-T, 7 or 8 MHz (lower adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B, D, D1, G, H, I, K/PAL | –9 | –5 |
| B, D, K, L/SECAM | –5 | –1 |

#### 2.1.1.3	Protection from upper adjacent channel interference

TABLE  30

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
DVB-T 7 MHz and 8 MHz signals (upper adjacent channel)

| Wanted signal: analogue system | Unwanted signal: DVB-T, 7 or 8 MHz (upper adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| PAL and SECAM | –8 | –5 |

#### 2.1.1.4	Protection from image channel interference

TABLE  31

Protection ratios (dB) for a wanted analogue vision signal interfered with by
a DVB-T 8 MHz signal (image channel)

| Wanted analogue system | Unwanted DVB-T channel | Tropospheric interference | Continuous interference |
| --- | --- | --- | --- |
| D1, G/PAL | N  9 | –19 | –15 |
| I/PAL | N  9 |  |  |
| L/SECAM (1) | N  9 | –24 | –22 |
| D, K/SECAM (1) | N  8, N  9 | –16 | –11 |
| D, K/PAL | N  8, N  9 |  |  |
| (1) Provisional values still under study. |  |  |  |

TABLE  32

Protection ratios (dB) for a wanted analogue vision signal interfered with by
a DVB-T 7 MHz signal (image channel)

| Wanted analogue system | Unwanted DVB-T channel | Tropospheric interference | Continuous interference |
| --- | --- | --- | --- |
| B/PAL | N  10, N  11 | –22 | –18 |

#### 2.1.1.5	Protection from overlapping interference

TABLE  33

Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals(1) 
interfered with by a DVB-T 7 MHz signal
(overlapping channels)

| Frequency of the centre of the unwanted DVB-T signal minus the vision carrier frequency of the wanted analogue television signal (MHz) | Protection ratio |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| –7.75 | –16 | –11 |
| (N – 1) – 4.75 | –9 | –5 |
| – 4.25 | –3 | 4 |
| –3.75 | 13 | 21 |
| –3.25 | 25 | 31 |
| –2.75 | 30 | 37 |
| –1.75 | 34 | 40 |
| –0.75 | 35 | 41 |
| (N ) 2.25 | 35 | 41 |
| 4.25 | 35 | 40 |
| 5.25 | 31 | 38 |
| 6.25 | 28 | 35 |
| 7.25 | 26 | 33 |
| 8.25 | 6 | 12 |
| (N  1) 9.25 | –8 | –5 |
| 12.25 | –8 | –5 |
| (1) For all SECAM systems similar values are expected. The values are still under study. |  |  |

TABLE  34

Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals(1)
interfered with by a DVB-T 8 MHz signal
(overlapping channels)

| Frequency of the centre of the unwanted DVB-T signal minus the vision carrier frequency of the wanted analogue television signal (MHz) | Protection ratio |  |
| --- | --- | --- |
|  | Tropospheric interference (2) | Continuous interference (2) |
| – 8.25 | –16 | –11 |
| (N – 1) –5.25 | –9 | –5 |
| – 4.75 | – 4 | 3 |
| – 4.25 | 12 | 20 |
| –3.75 | 24 | 30 |
| –3.25 | 29 | 36 |
| –2.25 | 33 | 39 |
| –1.25 | 34 | 40 |
| (N ) 2.75 | 34 | 40 |
| 4.75 | 34 | 39 |
| 5.75 | 30 | 37 |
| 6.75 | 27 | 34 |
| 7.75 | 25 | 32 |
| 8.75 | 5 | 11 |
| (N  1) 9.75 | –8 | –5 |
| 12.75 | –8 | –5 |
| (1) For all SECAM systems similar values are expected. The values are still under study. (2) The values for tropospheric and continuous interference have been arrived at from Table 33 by calculation. |  |  |

# 3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals

Tables 35 to 37 in this Annex show protection ratios for wanted FM, AM and NICAM television sound carriers interfered with by unwanted digital terrestrial television signals.

All protection ratios in this section refer to the level of the wanted television sound carriers. The reference level of the sound carriers is the r.m.s. value of the unmodulated carrier.

The sound quality for tropospheric interference corresponds to grade 3, for continuous interference to grade 4.

The reference (S/N) for FM sound signals are:

–	40 dB (approximates to impairment grade 3) – tropospheric case;

–	48 dB (approximates to impairment grade 4) – continuous case.

The reference S/Ns are measured as S/N peak-to-peak weighted, given in Recommendation ITU-R BS.468 and Recommendation ITU-R BS.412.

The reference FM sound signal level corresponds to a maximum frequency deviation of  50 kHz.

The reference BERs for NICAM digital sound signals are:

–	BER  1  10–4 (approximates to impairment grade 3), tropospheric case;

–	BER  1  10–5 (approximates to impairment grade 4), continuous case.

In the case of a two-sound carrier transmission, each of the two-sound signals must be considered separately. Multiplex modulated sound signals may require higher protection.

## 3.1	Protection for FM, AM and NICAM sound signals of analogue television systems interfered with by DVB-T digital terrestrial television signals

TABLE  35

Co-channel protection (dB) ratios for a wanted sound signal
interfered with by DVB-T digital terrestrial television signals

| Protection ratio related to the wanted sound carrier |  | Unwanted signal |  |
| --- | --- | --- | --- |
| Wanted sound signal |  | DVB-T 7 MHz | DVB-T 8 MHz |
| FM | Tropospheric case | 6 | 5 |
|  | Continuous case | 16 | 15 |
| AM | Tropospheric case | 21 | 20 |
|  | Continuous case | 24 | 23 |
| NICAM | Tropospheric case | 5 | 4 |
| PAL B/G | Continuous case | 6 | 5 |
| NICAM | Tropospheric case |  |  |
| System I | Continuous case |  |  |
| NICAM | Tropospheric case | 12 | 11 |
| System L | Continuous case | 13 | 12 |

TABLE  36

Protection ratios (dB) for a wanted FM sound signal interfered with by 
a DVB-T 7 MHz signal (overlapping channels)

|  |  | Frequency of the 3 dB point of DVB-T signal minus sound carrier frequency |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of the DVB-T signal relative to an FM carrier | Protection ratio related to the wanted sound carrier | –500 kHz | –250 kHz | –50 kHz | 0.0 kHz | 50 kHz | 250 kHz | 500 kHz |
| DVB-T below FM | Tropospheric case | 0 | 0 | 0 | 5 | 5 | 6 | 6 |
|  | Continuous case | 9 | 9 | 9 | 14 | 14 | 15 | 16 |
| DVB-T above FM | Tropospheric case | 5 | 5 | 4 | 3 | –9 | –22 | –32 |
|  | Continuous case | 15 | 15 | 14 | 12 | –6 | –16 | –27 |
| NOTE 1 – The protection ratio figures are related to an out-of-channel spectrum attenuation of 40 dB. NOTE 2 – This Table is still under study. |  |  |  |  |  |  |  |  |

TABLE  37

Protection ratios (dB) for a wanted AM sound signal interfered with by 
a DVB-T 8 MHz signal for different frequency offsets
(upper adjacent channel)

| Protection ratio related to the wanted sound carrier | Centre frequency of the DVB-T signal minus sound carrier frequency |  |  |
| --- | --- | --- | --- |
|  | With negative offset | No offset | With positive offset |
|  | 4.250 – 0.166 MHz = 4.084 MHz | 4.250 MHz | 4.250 + 0.166 MHz = 4.416 MHz |
| Tropospheric case | –1 | –2 | – 4 |
| Continuous case | 1 | 0 | –2 |

# 4	Protection ratios for a T-DAB signal interfered with by an unwanted digital terrestrial television signal

TABLE 38

Protection ratios (dB) for a T-DAB signal interfered with by
a DVB-T 8 MHz signal

| 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (1) (MHz) | –5 | – 4.2 | – 4 | –3 | 0 | 3 | 4 | 4.2 | 5 |
| PR | –50 | – 1 | 0 | 1 | 1 | 1 | 0 | –1 | –50 |
| (1)  f : centre frequency of the DVB-T signal minus centre frequency of the T-DAB signal. |  |  |  |  |  |  |  |  |  |

TABLE  39

Protection ratios (dB) for a T-DAB signal interfered with by
a DVB-T 7 MHz signal

| 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (1) (MHz) | – 4.5 | – 3.7 | – 3.5 | – 2.5 | 0 | 2.5 | 3.5 | 3.7 | 4.5 |
| PR | – 49 | 0 | 1 | 2 | 2 | 2 | 1 | 0 | – 49 |
| (1)  f : centre frequency of the DVB-T signal minus centre frequency of the T-DAB signal. |  |  |  |  |  |  |  |  |  |

# 5	Minimum field strengths for DVB-T terrestrial digital television

The derivation by the voltage method is given for the calculation of minimum field strength values.

TABLE  40

Derivation by the voltage method DVB-T 8 MHz system

| Frequency (MHz) | 200 |  |  | 550 |  |  | 700 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System variant guard interval 1/4 | QPSK 2/3 | 16-QAM 2/3 | 64-QAM 2/3 | QPSK 2/3 | 16-QAM 2/3 | 64-QAM 2/3 | QPSK 2/3 | 16-QAM 2/3 | 64-QAM 2/3 |
| Noise bandwidth, B (MHz) | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 | 7.6 |
| Receiver noise figure, F (dB) | 5 | 5 | 5 | 7 | 7 | 7 | 7 | 7 | 7 |
| Receiver noise input voltage, UN (1) (dB(µV)) | 8.4 | 8.4 | 8.4 | 10.4 | 10.4 | 10.4 | 10.4 | 10.4 | 10.4 |
| Receiver carrier/noise ratio(2) (C/N) (dB) | 6.9 | 13.1 | 18.7 | 6.9 | 13.1 | 18.7 | 6.9 | 13.1 | 18.7 |
| Urban noise (dB) | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Minimum receiver input voltage, Umin (dB(µV))(1) | 16.3 | 22.5 | 28.1 | 17.3 | 23.5 | 29.1 | 17.3 | 23.5 | 29.1 |
| Conversion factor(1) K (dB) | 12.4 | 12.4 | 12.4 | 20.5 | 20.5 | 20.5 | 24.5 | 24.5 | 24.5 |
| Feeder loss Af (dB) | 3 | 3 | 3 | 3 | 3 | 3 | 5 | 5 | 5 |
| Antenna gain, G (dB) | 5 | 5 | 5 | 10 | 10 | 10 | 12 | 12 | 12 |
| minimum field strength for fixed reception, Emin (dB(µV/m))(1) | 26.7 | 32.9 | 38.5 | 31.8 | 37.6 | 42.6 | 35.8 | 41.6 | 46.6 |
| (1) For formula, see Appendix 1 to Annex 2. (2) For noise bandwidth noted above. |  |  |  |  |  |  |  |  |  |

APPENDIX 1

TO ANNEX 2

Derivation by the voltage method

| Derivation by the voltage method |  | Formulae |  | (dB) |
| --- | --- | --- | --- | --- |
|  |  | P  U 2/R |  |  |
| Thermal noise power: |  | k T B |  | 10 log (k T B) |
| Receiver noise input power, PN : |  | n k T B |  | 10 log (k T B)  F |
| Thermal noise voltage, UT : |  | UT = k T B R |  |  |
| Receiver noise input voltage, UN : |  | UN = n k T B R |  | 10 log (k T B)  F  10 log (R) |
| Minimum receiver input voltage, Umin : |  | Umín = UNC/N |  | UN  C/N |
| Relationship between voltage and field strength: |  |  |  |  |
| U = Pr R = ϕ A R = E2120 π 1.64 g0 λ24 π R |  |  |  |  |
| Therefore, |  |  |  |  |
| U = Eλ2480 π2 R 1.64 g0 |  |  |  |  |
| Conversion factor, K: | K = EU = 480 π21.64 g0 λ2 R |  | K = 10 log 480 π2 − 20 logλ −10 log R − 10 log 1.64 − GD + L |  |
| Conversion factor, K0: | K0 = EU = 4 π2g0 λ2 |  | K0 = 20 log(2 πλ) − GD + L |  |
| Minimum field strength: |  |  | Emín  Umín  K0 |  |

Data			Formulae used

k :	Boltzmann’s constant (1.38 10–23)	Thermal noise:	k T0 B

T0 :	reference temperature = 290 K	Receiver noise
temperature:	Trx = T0(10F/10  – 1)

F :	receiver noise figure (dB)	g0 :	10(GD – L)/10

n :	receiver noise figure (factor)	n :	10F/10

B :	equivalent noise bandwidth (Hz)	A :		1.64 g0 λ24 π

C/N :	radio-frequency carrier/noise ratio (dB)

Pr :	minimum receiver input power	 :	E2120 π

E :	field strength

R :	impedance of half-wave dipole (R = 73 )

f  :	frequency (Hz)

GD :	antenna gain related to half-wave dipole (dB)

L :	transmission line loss (dB)

 :	power flux-density

g0 :	gain of receiving antenna system related to half-wave dipole (factor)

A :	effective antenna aperture

ANNEX  3

Planning criteria for ISDB-T digital television system in the VHF/UHF bands

# 1	Protection ratios for ISDB-T wanted digital terrestrial television signals

Tables 41 to 43 and 44 to 46 show protection ratios for an ISDB-T wanted digital terrestrial television signal interfered with by an ISDB-T digital terrestrial television signal and by an analogue terrestrial television signal, respectively.

## 1.1	Protection of an ISDB-T digital terrestrial television signal interfered with by an ISDB-T digital television signal

TABLE  41

Co-channel protection ratios (dB) for ISDB-T 6 MHz
interfered with by ISDB-T 6 MHz

| Coding rate | Modulation |  |  |
| --- | --- | --- | --- |
|  | DQPSK | 16-QAM | 64-QAM |
| 7/8 | 10 | 17 | 23 |
| 5/6 | 9 | 16 | 22 |
| 3/4 | 9 | 15 | 21 |
| 2/3 | 8 | 14 | 20 |
| 1/2 | 6 | 12 | 17 |

TABLE  42

Protection ratios (dB) for ISDB-T 6 MHz interfered with by
ISDB-T 6 MHz in lower adjacent channel (N  1)

| Coding rate | Modulation |  |  |
| --- | --- | --- | --- |
|  | DQPSK | 16-QAM | 64-QAM |
| 7/8 | 28 | 27 | 24 |
| 5/6 | 28 | 27 | 25 |
| 3/4 | 29 | 27 | 26 |
| 2/3 | 29 | 28 | 26 |
| 1/2 | 29 | 28 | 27 |

TABLE  43

Protection ratios (dB) for ISDB-T 6 MHz interfered with by
ISDB-T 6 MHz in upper adjacent channel (N + 1)

| Coding rate | Modulation |  |  |
| --- | --- | --- | --- |
|  | DQPSK | 16-QAM | 64-QAM |
| 7/8 | 29 | 28 | 27 |
| 5/6 | 29 | 28 | 27 |
| 3/4 | 29 | 28 | 27 |
| 2/3 | 30 | 29 | 27 |
| 1/2 | 30 | 29 | 28 |

## 1.2	Protection of ISDB-T digital terrestrial television interfered with by analogue terrestrial television

### 1.2.1	Protection from co-channel interference

TABLE 44

Co-channel protection ratios (dB) for ISDB-T 6 MHz
interfered with by analogue television

|  | Protection ratio |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | DQPSK |  |  |  |  | QPSK |  |  |  |  |
| Code rate | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 |
| M/NTSC | 5 | 3 | 1 | 2 | 6 | 16 | 11 | 8 | 0 | 2 |
| Modulation | 16-QAM |  |  |  |  | 64-QAM |  |  |  |  |
| Code rate | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 |
| M/NTSC | 11 | 5 | 1 | 6 | 10 | 6 | 1 | 5 | 9 | 14 |
| NOTE 1 – The sound carrier level is 6 dB lower than NTSC vision carrier level. NOTE 2 – The values in this Table give the reception threshold. Taking into account the performance variation in domestic receivers, long-term degradation of reception condition etc., a margin of several dBs should preferably be added in actual frequency planning. |  |  |  |  |  |  |  |  |  |  |

### 1.2.2	Protection from lower adjacent channel (N-1) interference

TABLE 45

Protection ratios (dB) for lower adjacent channel (N – 1) interference for ISDB-T 6 MHz interfered with by NTSC signals including sound

| Wanted signal |  | Unwanted signal |
| --- | --- | --- |
| Modulation | Coding rate | M/NTSC |
| DQPSK | 1/2 | 34 |
| DQPSK | 2/3 | 34 |
| DQPSK | 3/4 | 33 |
| 16-QAM | 1/2 | 34 |
| 16-QAM | 2/3 | 33 |
| 16-QAM | 3/4 | 32 |
| 64-QAM | 2/3 | 32 |
| 64-QAM | 3/4 | 31 |
| 64-QAM | 5/6 | 29 |
| 64-QAM | 7/8 | 29 |

### 1.2.3	Protection from upper adjacent channel (N  1) interference

TABLE  46

Protection ratios (dB) for upper adjacent channel (N  1) interference for
ISDB-T 6 MHz interfered with by NTSC 6 MHz signal

| Wanted signal |  | Unwanted signal |
| --- | --- | --- |
| Modulation | Coding rate | M/NTSC |
| DQPSK | 1/2 | 35 |
| DQPSK | 2/3 | 35 |
| DQPSK | 3/4 | 34 |
| 16-QAM | 1/2 | 35 |
| 16-QAM | 2/3 | 34 |
| 16-QAM | 3/4 | 33 |
| 64-QAM | 2/3 | 33 |
| 64-QAM | 3/4 | 33 |
| 64-QAM | 5/6 | 32 |
| 64-QAM | 7/8 | 31 |

# 2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ISDB-T digital terrestrial television signals

Table 47 shows protection ratios for a 525-line analogue television signal interfered with by an ISDB-T digital terrestrial television signal.

The protection ratio values given are related to a spectrum shoulder attenuation of the unwanted digital signal of 38 dB.

## 2.1	Protection ratios for 525-line television systems

### 2.1.1	Protection for NTSC vision signals interfered with by ISDB-T digital television signals

TABLE  47

Protection ratios (dB) for analogue vision signal (NTSC, 6 MHz)
interfered with by ISDB-T signal

| Unwanted digital channel | Tropospheric interference | Continuous interference |
| --- | --- | --- |
| N – 1 (lower) | 6 | 3 |
| N (co-channel) | 39 | 44 |
| N + 1 (upper) | 6 | 3 |

# 3	Protection ratios for sound signals of wanted analogue terrestrial television systems interfered with by ISDB-T unwanted digital terrestrial television system

## 3.1	Protection for NTSC sound signals interfered with by an ISDB-T digital television signal

As shown in Table 48, the sound signal of NTSC broadcasting is robust compared with the vision signal when interfered with by an ISDB-T signal. Therefore protection ratios for NTSC broadcasting are determined by the protection ratios for the vision signal, which are shown in Table 47.

TABLE  48

Sound quality related to the vision protection ratio of grade 3 when
NTSC 6 MHz signal interfered with by ISDB-T 6 MHz signal

|  | Measurement condition (see Note 3) | Sound quality |
| --- | --- | --- |
| Co-channel interference | D/U  39 dB | > grade 4 (S/N = 54 dB) |
| Upper adjacent channel interference | D/U  6 dB |  grade 4 (S/N  53 dB) |
| Lower adjacent channel interference | D/U  6 dB |  grade 4 (S/N  52 dB) |
| NOTE 1 – The sound carrier level is 6 dB lower than NTSC vision carrier level. NOTE 2 – The reference FM sound signal level corresponds to a maximum frequency deviation of ± 25 kHz. NOTE 3 – D/U ratio that gives vision quality of grade 3 (corresponding to protection ratio for tropospheric interference). |  |  |

# 4	Minimum field strengths for ISDB-T terrestrial digital television

To reduce the number of tables for the minimum field strengths of an ISDB-T system, the derivation of minimum field strengths is made with the typical transmission modes shown in Table 49. The minimum field strengths for the different modes can be calculated from the given values in Table 49.

TABLE  49

Derivation by the voltage method ISDB-T 6 MHz system

| Frequency (MHz) | Low VHF |  |  |  | High VHF |  |  |  |  | UHF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 |  |  |  | 200 |  |  |  |  | 600 |  |  |  |  |
| System | DQPSK 1/2 | QPSK 1/2 | 16-QAM 3/4 | 64-QAM 7/8 |  | DQPSK 1/2 | QPSK 1/2 | 16-QAM 3/4 | 64-QAM 7/8 | DQPSK 1/2 | QPSK 1/2 | 16-QAM 3/4 | 64-QAM 7/8 |  |
| Noise bandwidth, B (MHz) | 5.6 | 5.6 | 5.6 | 5.6 |  | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 |  |
| Receiver noise figure, NF (dB) | 5 | 5 | 5 | 5 |  | 5 | 5 | 5 | 5 | 7 | 7 | 7 | 7 |  |
| Receiver noise input voltage, UN (1) (dB(V)) | 7.1 | 7.1 | 7.1 | 7.1 |  | 7.1 | 7.1 | 7.1 | 7.1 | 9.1 | 9.1 | 9.1 | 9.1 |  |
| Receiver carrier/noise ratio(2) (C/N) (dB) | 6.2 | 4.9 | 14.6 | 22.0 |  | 6.2 | 4.9 | 14.6 | 22.0 | 6.2 | 4.9 | 14.6 | 22.0 |  |
| Urban noise (dB) | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |  |
| Minimum receiver input voltage, Umin (dB(V))(1) | 14.3 | 13.0 | 22.7 | 30.1 |  | 14.3 | 13.0 | 22.7 | 30.1 | 15.3 | 14.0 | 23.7 | 31.1 |  |
| Conversion factor (1) K (dB) | 6.4 | 6.4 | 6.4 | 6.4 |  | 12.4 | 12.4 | 12.4 | 12.4 | 21.9 | 21.9 | 21.9 | 21.9 |  |
| Feeder loss, Af (dB) | 3 | 3 | 3 | 3 |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| Antenna gain, G (dB) | 3 | 5 | 3 | 3 |  | 5 | 5 | 5 | 5 | 10 | 10 | 10 | 10 |  |
| Minimum field strength for fixed reception, Emin (dB(V/m))(1) | 20.7 | 23.4 | 29.1 | 36.5 |  | 24.7 | 23.4 | 33.1 | 40.5 | 30.2 | 28.9 | 38.6 | 46.0 |  |
| (1) For formula, see Appendix 1 to Annex 3. (2) For noise bandwidth noted above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX  1

TO  ANNEX  3

Derivation by the figure of merit method

Required field strength

Erx (dB(V/m))     (dB(W/m2))  10 log(120 )

C/N     – Glm2  GA / Te – k – Brf

Erx (dB(V/m))		 (dB(W/m2))  25.8 (dB)  120 (dB)

	145.8  C/N  Glm2 – GA / Te  10 log(k)  10 log(Brf )

Erx :	required field strength at the receive system antenna

 :	power flux-density at the receive system antenna

C/N :	carrier-to-noise ratio

Glm2:	gain of 1 m2

GA /Te :	figure of merit of the receive system

k :	Boltzmann’s constant

Brf :	system equivalent noise bandwidth.

Receive system figure of merit

(For receiving system model with LNA)

GA / Te    (G  L ) / (balun Ta  Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))

Receiver noise temperature

Trx    (10 NF / 10 – 1)    290

LNA noise temperature

TLNA    (10 NF / 10 – 1)    290

Transmission line noise temperature

Tline    (1 – line)    290

Balun noise temperature

Tbalun    (1 – balun)    290

Antenna noise temperature

Ta    10(6.3 – 2.77(log f ))    290     (for dipole antenna)

with f expressed in MHz.

Antenna noise temperature (referred to LNA input)

 Ta    Ta(balun)

System noise temperature

Te		(balun Ta  Tbalun  TLNA  Tline /(line GLNA)  Trx / (line GLNA))

Te (dB(K))		10 log(balun Ta  Tbalun  TLNA  Tline / (líne GLNA)  Trx / (line GLNA))

or		10 log(Tbalun  TLNA  Tline / (líne GLNA)  Trx / (line GLNA))  Next

when Ta is not known.

Gain of 1 m2

G1m2    10 log(4  / 2)

Data

GI :	antenna gain (isotropic) (dB)

L :	transmission line loss (dB)

line :	transmission line loss (numeric ratio)

Ta :	antenna noise temperature (K)

Trx :	receiver noise temperature (K)

nf :	noise factor (numeric ratio)

NF :	noise figure (dB)

T0 :	reference temperature  290 K

 :	wavelength of frequency of operation

GA :	system gain (dB)

Te :	system noise temperature (K)

Next :	dB value representing the contribution due to external noise

k :	Boltzmann’s constant 1.38  10–23 (– 228.6 dB)

B :	system equivalent noise bandwidth (dB(Hz))

balun :	antenna 300/75 Balun loss (numeric ratio)

LNA :	low noise amplifier

TLNA :	LNA noise temperature (K)

ANNEX 4

Other planning factors

# 1	Field strength distribution with location

It is to be expected that the distributions of field strength with location for digital television signals will not be the same as those applicable to analogue television signals. Recommendation ITU-R P.1546 includes the standard deviation for the analogue and for the digital case in Table 1, 2 and 3 for 100 MHz, 600 MHz and 2 000 MHz, respectively.

The results of propagation studies for digital systems are given in Fig. 1 for the VHF and UHF bands. The Figure corresponds to a standard deviation of 5.5 dB. These results may also be used to derive propagation prediction curves for location percentages other than 50%. Refer to Recommendation ITU-R P.1546 for the location percentages other than 50% for analogue and digital systems, where the digital system bandwidth is greater than 1.5 MHz.

# 2	Reception using portable equipment inside buildings

For land paths, the curves in Recommendation ITU-R P.1546 give field-strength values for a receiving antenna height above ground equal to the representative height of ground cover around the receiving antenna location. Subject to a minimum height value of 10 m, examples of reference heights are 20 m for an urban area, 30 m for a dense urban area and 10 m for a suburban area. (For sea paths, the notional value is 10 m.)

If the receiving antenna height is different from the representative height, a correction is applied to the field strength taken from the curves of Recommendation ITU-R P.1546 according to a procedure given in the Recommendation.

Losses due to penetration into a building depend significantly on the building material, angle of incidence and frequency. Consideration should also be given as to whether reception is in an interior room or in one located near an exterior wall. Whilst no single comprehensive formula is available for computing building entry loss, useful statistical information based on measured losses in several types of building, at frequencies from about 500 MHz to 5 GHz, are given in Recommendation ITU-R P.679. Once inside the building, propagation loss due to walls and floors are dealt with in Recommendation ITU-R P.1238.

# 3	Receiving antenna discrimination

Information concerning the directivity and polarization discrimination of domestic receiving antennas is given in Recommendation ITU-R BT.419.

ANNEX  5

Subjective comparison method (SCM) with a reference interferer
for assessment of protection ratios for analogue television systems

# 1	Introduction

Subjective methods for assessment of impairment grades involve extensive tests, are time consuming, require large numbers of observers and consider the full impairment grade range.

For assessing protection ratios only two fixed impairment types are necessary, approximately grade 3 for tropospheric and grade 4 for continuous, see Table 50.

This Annex gives a method of assessing protection ratios for wanted analogue TV systems based on the subjective comparison of the impairment of an interferer with that of a reference interferer. Usable and reliable results are produced with only a small number of observers and one still picture.

This subjective comparison method is appropriate for the evaluation of interference from any unwanted digital or analogue transmission system into a wanted analogue television channel. The application of a defined fixed reference interferer results in a reproducible set of figures with a low deviation (approximately  1 dB standard deviation). Only a small number of observers – three to five experts or non-experts – is necessary.

There are two reference interferers which may be used:

–	sine-wave interference

–	Gaussian-noise interferer.

Tests have shown that for unwanted digital television systems a noise reference interferer can improve the assessment decision by the observer. The use of noise reference interferer shows the same results as the defined sine-wave interferer. The disadvantage is that a more complicated test arrangement may be necessary. Further tests are necessary, especially by fixing the equivalent noise reference.

(For the time being the sine-wave reference interferer should be used until an agreement on a common test procedure and an agreement on a harmonized unified noise reference figure has been obtained.)

# 2	The SCM for assessment of protection ratios using a sine-wave reference

## 2.1	General description

Figure 2 shows the test arrangement for the subjective comparison method with sine-wave interferer. The lower three blocks are the main signal path, the wanted video source, the television transmitter and the TV receiver under test. The reference video interferer is a simple sine-wave signal. The amplitude of the sine-wave generator is switchable between tropospheric interference and continuous interference. The unwanted RF interferer is added to the wanted signal path. The amplitude and frequency of the interferer are calculated from the RF reference interferer given in Recommendation ITU-R BT.655, Annex 1, § 2.3.

The intensity of the RF interferer can be changed with an attenuator controlled by the observer. The RF interferer is adjusted to produce the same impairment grade as the reference interferer by comparing the interfered pictures on the TV screen.

The RF protection ratio is the difference between the wanted and the unwanted signal levels at the receiver input. The test arrangement can be adjusted in such a way that the value in dB shown at the attenuation box gives directly the protection ratio.

## 2.2	Realization of the reference interferer

For 625-line systems the reference impairment levels are those which correspond to co-channel protection ratios of 30 dB and 40 dB with a frequency offset between the wanted and unwanted vision carriers close to two-thirds of the line frequency but adjusted for maximum impairment. The precise frequency difference is 10 416 Hz. These conditions approximate impairment grades 3 (slightly annoying) and 4 (perceptible, but not annoying) and apply to tropospheric (1% of time) and continuous interference (50% of time), respectively. The impairment grade of the given video baseband reference interferer is independent of the analogue television system and independent of the RF modulation parameters such as modulation polarity, residual carrier etc.

The RF reference interferer can be realized as a simple sine-wave signal at baseband frequency as shown in Fig. 2. The sine-wave reference interferer has a fixed frequency of 10 416 Hz for 625-line systems or 10 500 Hz for 525-line systems –60 Hz and 10 510 Hz for 525-line systems –59.94 Hz, an amplitude of either 60 mVp-p or 20 mVp-p referring to a black-to-white level of 700 m1 Vp-p or a CVBS level of 1 Vp-p. These amplitudes correspond to the RF protection ratios of 30 dB and 40 dB respectively (2/3 line offset). The frequency stability of the sine-wave generator must be within  1 Hz.

## 2.3	Test conditions

Wanted video signal:	only an electronic test picture is required (e.g. FuBK, Philips or others).

Viewing conditions:		as given in Recommendation ITU-R BT.500.

Viewing distance:		five times the picture height.

Test receiver:			up to five different domestic sets, not older than five years, for co-channel 	measurements a professional receiver can be used.

Receiver input signal:	39 dBm (70 dB(V) at 75 ).

Observers:				five observers, experts or non-experts, are necessary. For initial tests less than five observers are possible. Each single test should be made with one observer only. Observers should be introduced to the method of assessment.

## 2.4	Presentation of the results

The results should be presented together with the following information:

–	mean and standard deviation of the statistical distribution of the protection ratio values;

–	test configuration, test picture, type of picture source;

–	number of observers;

–	reference interferer type;

–	the spectrum of the unwanted signal (RF interferer), including the out-of-channel range;

–	the used RF level for the wanted signal at the receiver input (for domestic receivers an input voltage of 39 dBm (70 dB(V) at 75  should be used);

–	when domestic sets are used, type, display size and year of production.

# 3	Table of important parameters

TABLE  50

Basic terms and relations for the SCM

| Quality impairment | Grade 3 | Grade 4 |
| --- | --- | --- |
| Interference type | Tropospheric | Continuous |
| Time allowance | 1% to 5% of time | 50% of time |
| Subjective impairment | Slightly annoying | Perceptible, but not annoying |
| Reference interferer (mVp-p) | 60 | 20 |
| RF protection ratio (dB) | 30 | 40 |

ANNEX  6

Test methods for protection ratio measurements for 
wanted digital terrestrial signals

# 1	Background

Initial studies of the protection ratios for the DVB-T system were based on a target BER of 2  10–4 measured between the inner and outer codes, before Reed-Solomon decoding. For the case of a noise-like interferer, this has been taken to correspond to a quasi-error-free (QEF) picture quality with the BER  1  10–11 at the input of the MPEG-2 demultiplexer.

# 2	The subjective failure point (SFP) method for protection ratio measurements

For domestic receivers it may not be possible to measure the BER and therefore a new method called the SFP method has been proposed for protection ratio measurements in a unified manner. The quality criterion for protection ratio measurements is to find a limit for a just error-free picture

at the TV screen. The RF protection ratio for the wanted DVB-T signal is a value of wanted-to-unwanted signal ratio at the receiver input, determined by the SFP method, and rounded to the next higher integer value.

The SFP method corresponds to the picture quality where no more than one error is visible in the picture for an average observation time of 20 s. The adjustment of the wanted and unwanted signal levels for the SFP method is to be carried out in small steps, usually in steps of 0.1 dB. For a “noise-like” interferer the difference in a value of wanted-to-unwanted signal ratio between the QEF method with a BER of 2  10–4 and the SFP method is less than 1 dB. All protection ratio values for wanted digital TV signals are measured with a receiver input power of 60 dBm.

It is proposed that the SFP method should be adopted for assessment of all DTTB systems. (For the digital system ISDB-T this method will be studied in Japan.)

ANNEX  7

Tropospheric and continuous interference

When using the protection ratios in planning, it is necessary to determine whether, in particular circumstances, the interference should be considered as tropospheric or continuous. This can be done by comparing the nuisance fields for the two conditions, the nuisance field being defined as the field strength of the interfering transmitter (at its pertinent e.r.p.) enlarged by the relevant protection ratio.

Thus, the nuisance field for continuous interference:

EC    E(50, 50)    P    AC

and the nuisance field for tropospheric interference:

ET    E(50, t)    P    AT

where:

E(50, t) :	field strength (dB(V/m)) of the interfering transmitter, normalized to 1 kW, and exceeded during t % of the time

P :	e.r.p. (dB(1 kW)) of the interfering transmitter

A :	protection ratio (dB)

C and T :	continuous and tropospheric interference, respectively.

The protection ratio for continuous interference is applicable when the resulting nuisance field is stronger than that resulting from tropospheric interference, that is, when EC  ET.

This means that AC should be used in all cases when:

E(50, 50)    AC    E(50, t)    AT

*	 	The Administrations of the Islamic Republic of Iran and Syrian Arab Republic are not in a position to be bound by “the planning criteria for digital terrestrial television services in the VHF/UHF bands” as shown in this Recommendation.

*	*	The Administrations of Egypt and Syrian Arab Republic reserve their position on all proposed co-channel protection ratios and other proposed protection ratios until being decided by the Regional Radiocommunication Conference (Geneva, 2004) (RRC-04).