RECOMMENDATION  ITU-R  BT.1549*

Data link protocol for interaction channel

(Question ITU-R 16/6)

(2001)

The ITU Radiocommunication Assembly,

considering

a)	the progress in information processing and communication technologies;

b)	the rapid progress towards enhanced and digital television delivery systems;

c)	the need within such systems for interactivity for a variety of purposes;

d)	the development of communication methods, over various delivery media, suitable for use in receiving, from viewers, return communications related to the programme material (vision, sound and data);

e)	the availability of mass storage media permitting interaction without the requirement of a return channel;

f)	the large number of domestic receivers likely to be impacted by the adoption of interactive services;

g)	the large number of domestic multimedia systems likely to be impacted by the adoption of interactive services;

h)	that return information from viewers can be also provided in a non-automatic way (viewers have to store and then take care of the delivery of the information);

j)	the existence of Recommendation ITU-R BT.1369 – Basic principles for a worldwide common family of systems for the provision of interactive television services;

k)	the need for simpler protocols than Recommendation ITU-R BT.1434, which supports Internet protocol (IP)-based protocols, such that the introduction of interactive services using a return channel might be more easily achieved;

l)	that public switched telephone network (PSTN)/integrated services digital network (ISDN) is one of most generic interaction channels in many countries;

m)	the requirement that the simpler protocols should also support a simple level of the security,

recommends

1	that the data link layer protocol for the interaction channel, specified in Annex 1, should be applied when using a low data rate modem and simple data transfer protocols.

ANNEX  1

Data link protocol for interaction channel

# 1	Scope

This Recommandation specifies the data link layer protocol which can be applied when using a low data rate modem and a simple data transfer protocol for the interaction channel (i.e. non IP-based protocols such as high level data link control (HDLC) (ISO 3309, 4335, 7809), basic mode control procedure for data communication systems (ISO 1745), etc.).

# 2	Normative references

ITU-T Recommendation X.28 – DTE/DCE interface for a start-stop mode Data Terminal Equipment accessing the Packet Assembly/Disassembly facility (PAD) in a public data network situated in the same country.

# 3	Reference models

## 3.1	Assumed connection model for interactive systems

An assumed connection model for interactive systems to which this link layer protocol can be applied is shown below.

NOTE 1 – The model is compliant with the system model of ITU-T Recommendation J.110, and matches the logical model of ITU-T Recommendation J.111 with the exception that the S2 channel for data downloading is not used.

## 3.2	Data transfer phases

Protocols using PSTN for bidirectional interactive services consist of the following five phases:

–	Phase 1:	Line connection phase

–	Phase 2:	Data link establishment phase

–	Phase 3:	Data transmission phase

–	Phase 4:	Data link termination phase

–	Phase 5:	Line disconnection phase.

# 4	Data link protocol for interaction channel

## 4.1	Conditions

### 4.1.1	Telecommunication conditions

Telecommunication conditions for the modem of the receiver are shown in Table 1.

TABLE  1

Telecommunication conditions for the modem of the receiver

|  | Parameters | Note |
| --- | --- | --- |
| Data length (character length) | 8 bits | Communication conditions for line connection |
| Parity | None |  |
| Stop bit | 1 bit |  |
| Transmission code system | Specified by each application |  |
| Local echo back | None (remote echo back may be used) |  |
| CR/LF control | From receiver to host: only CR is sent. From host to receiver: CR and LF are sent |  |
| Code for transmission delimiter | CR (0D H) |  |
| Code for new line | LF (0A H) |  |
| Code for input correction | BS (08 H) |  |
| Data transfer sequence | Specified by each application | Communication conditions for modem |
| Transmission mode | Asynchronous full duplex transmission |  |
| Transmission speed | Above ITU-T Recommendation V.22 bis (2 400 bit/s) |  |
| Flow control | RS/CS |  |
| MNP class | Class 4 or above |  |
| CR: transmission mode LF: code for new line MNP: Microcom Networking Protocol |  |  |

### 4.1.2	Modem

The modem shall support the specifications, listed above, equal to ITU-T Recommendation V.22 bis or higher (e.g. ITU-T Recommendations V.22 bis, V.32 and V.32 bis) and with the error correction scheme equal to MNP4 or higher (e.g. MNP4, MNP5, ITU-T Recommendations V.42 and V.42 bis).

### 4.1.3	Line connection and disconnection phase

During the line connection and disconnection phase, the receiver connects to and disconnects from the host using PSTN, etc. The line connection and disconnection are controlled by AT (attention) commands for modems, terminal adapter (TA) and data communication adapters (ADP) of mobile phones.

## 4.2	Protocol for data link establishment and termination phase

In this phase, which starts after the line connection has been effected, a data transmission is established between the receiver and a host. In addition, after data transmission has been completed, the link between the receiver and the host is terminated. These phases can be applied to data transfer protocols, which do not specify a destination address for each data packet. Therefore, this phase is applicable to various types of protocols for interaction channels.

When basic modems are used, error detection and correction protocols are either performed at the physical layer (MNP4), the data link layer or the network layer, whichever is specified by the operational rule.

Table 2 shows the protocol stack for this phase.

TABLE  2

Protocol stack for data link establishment and termination phase

|  | Protocol stack |  |
| --- | --- | --- |
| Application layer | Selected according to service |  |
| Data link layer | Protocol conforming to a part of ITU-T Recommendation X.28 (requires a facility for specifying the number of host servers) |  |
| Physical layer | Receiver | Host |
| Basic modem | Conforming with ITU-T Recommendation V.22 bis and later |  |

## 4.3	Host number

When receivers connect to hosts through telephone networks and so on, they should connect to the shared access point at first and send the host number command to identify host servers.

## 4.4	Sequences

### 4.4.1	Connection sequence

### 4.4.2	Break sequence

An example format of the host number command and service signal is shown in Table 3.

TABLE  3

Example format of the host number command and service signal

|  |  | Format | Note |
| --- | --- | --- | --- |
| Host number command |  | N1 N2 N3 N4 N5 N6 N7 N8 CR Characters which are echoed back N1 N2 N3 N4 N5 N6 N7 N8 CR LF | 8 alphanumeric characters are echoed back |
| Service signal | Notification of connection | CR LF COM CR LF | CR LF |
|  | Command error | CR LF ERR▲INV CR LF | ▲ means space |

### 4.4.3	Receiver behaviour after transmitting host number command

#### 4.4.3.1	Waiting for remote echo of transmitted host number

After sending the host number, receivers move into the waiting status in order to receive the remote echo. The behaviour of the receivers in this status is shown in Table 4.

TABLE  4

Behaviour of the receivers waiting for the remote echo

| Received signal | Behaviour after receiving the signal |
| --- | --- |
| Same remote echo as the transmitted host number Reception of N1 N2 N3 N4 N5 N6 N7 N8 CR LF (8 characters prior to the CRLF are compared and the other characters are neglected) | Moves into the waiting status for service signal |
| Different remote echo from the transmitted host number Reception of ■■■■CR LF (■■■■ means code strings with any length larger than 0 byte other than N1 N2 N3 N4 N5 N6 N7 N8) | Disconnects the line immediately |
| After sending host number or re-sending it, it does not receive CR LF within the specified time (time out duration: T1)(1) | Disconnects the line immediately |
| (1) The timer of the receiver which watches the communication starts from the transmission of host number command or its retransmission. (The value of T1 is specified by each application.) |  |

#### 4.4.3.2	Waiting for service signal

After receiving the same remote echo as the transmitted host number, N1 N2 N3 N4 N5 N6 N7 N8 CR LF, receivers move into the waiting status for service signal. The behaviour of the receivers in this status is shown in Table 5.

TABLE  5

Behaviour of the receivers waiting for the service signal

| Received signal | Behaviour after receiving the signal |
| --- | --- |
| Correct service signal (notification of connection) (1) Reception of CR LF COM CR LF | Moves into data transfer sequence |
| Correct service signal (command error)(1) Reception of CR LF ERR▲INV CR LF (▲ means space) | Retransmits host number immediately. Number of retransmission: 3 times. (It disconnects when CR LF ERR▲INV CR LF is received four times) |
| Service signal with errors(1) Reception of CR LF COM◊, CR LF ERR○, or CR LF□□□□CR LF (◊ means codes other than CR. ○ means codes other than space. □□□□ means strings with any length larger than 0 byte other than COM and ERR▲INV) | Disconnects the line immediately |
| After sending host number or resending it, it does not receive correct service signal within the specified time (time out duration: T1)(2) | Disconnects the line immediately |
| (1) The data which is received from the transition to the waiting status of service signal to the time when the first CR LF is received are discarded. (2) The timer of the receiver which watches the communication starts from the transmission of host number command or its retransmission. (The value of T1 is specified by each application.) |  |

### 4.4.4	Remote echo

When a receiver sends the host number command, the host side echoes back to the receiver. Thus the local echo back in the receiver is not necessary.

The host side echoes back after receiving the host number command, and sends a service signal in succession.

### 4.4.5	Start timing of the timer at the server side

The timer at the host side which watches the communication starts count-up from the completion of the line connection (end of the modem negotiation). The value of time out T1 is specified according to this start time. The timer resets after sending CR LF ERR▲INV CR LF.

APPENDIX  1

TO  ANNEX  1

Example protocols used for direct connections (data transmission phase) together with the specified data link protocol (data link 
establishment and termination phase)

In the following protocols, error detection and correction protocols are either performed at the physical layer (MNP4), the data link layer or the network layer, whichever is specified by the operational rule.

NOTE 1 – Protocols at the physical layer in this Recommendation mean those of the physical layer and transport layer in ITU-T Recommendation J.111 (or Recommendation ITU-R BT.1434) and ITU-T Recommendation J.113 (or Recommendation ITU-R BT.1435), and, Protocols at the data link and higher layers mean the network independent protocols in ITU-T Recommendation J.111. These differences come from the IP protocol layer. In this specification IP is stated at the network layer, which is stated at the higher medium layer in ITU-T Recommendation J.111.

TABLE  6

Text communications protocol stack

| NOTE 1 – This protocol stack would be possible for extension of other mobile phone networks including International Mobile Telecommunications-2000 (IMT-2000). Support for these networks may be considered for creation of the Recommendation. |  | Protocol stack |  |  |
| --- | --- | --- | --- | --- |
| Application layer |  | Selected according to service |  |  |
| Data link layer |  | Non procedure (tele-typewriter (TTY) protocol) |  |  |
| Physical layer |  | Receiver | Host |  |
| Basic modem |  | ITU-T Recommendation V.22 bis and later |  |  |
| Examples of optional modem |  |  |  |  |
|  | Advanced modem | ITU-T Recommendation V.34 and later  V.42 bis |  |  |
|  | Mobile phone (circuit switched service) | PDC(1): 9 600 bit/s | PDC: 9 600 bit/s or ITU-T Recommendation V.32 bis  V.42 bis(2) |  |
|  | PHS(3) | PIAFS(4): 32 kbit/s or more. |  |  |
| (1) Personal digital cellular: PDC of Recommendation ITU-R M.1073 – Digital cellular land mobile telecommunication systems. (2) Converted to analogue data by mobile phone network (same hereinafter). (3) Personal handy-phone system: System 6 of Recommendation ITU-R M.1033 – Technical and operational characteristics of cordless telephones and cordless telecommunication systems. (4) PHS Internet access forum standard. |  |  |  |  |

TABLE  7a

Communications protocol stacks for binary transmissions

|  |  | Protocol stack |  |
| --- | --- | --- | --- |
| Application layer |  | Selected according to service |  |
| Data link layer |  | Basic mode control procedure for data communication systems (ISO 1745) (only required functions implemented) Code-independent mode |  |
| Physical layer |  | Receiver | Host |
| Basic modem |  | ITU-T Recommendation V.22 bis and later |  |
| Examples of optional modem |  |  |  |
|  | Advanced modem | ITU-T Recommendation V.34 and later  V.42 bis |  |
|  | Mobile phone (circuit switched service) | PDC(1): 9 600 bit/s | PDC: 9 600 bit/s or ITU-T Recommendation V.32 bis  V.42 bis(2) |
|  | PHS(3) | PIAFS(4): 32 kbit/s or more. |  |
| (1), (2), (3) and (4) see footnotes (1), (2), (3) and (4) to Table 6. |  |  |  |

NOTE 1 – This protocol stack would be possible for extension of other mobile phone networks including International Mobile Telecommunications-2000 (IMT-2000). Support for these networks may be considered for creation of the Recommendation.

TABLE  7b

Communications protocol stacks for binary transmissions

|  |  | Protocol stack |  |
| --- | --- | --- | --- |
| Application layer |  | Selected according to service |  |
| Data link layer |  | Basic mode control procedure for data communication systems (ISO 1745) Code-independent mode |  |
| Physical layer |  | Receiver | Host |
| Basic modem |  | ITU-T Recommendation V.22 bis and later |  |
| Examples of optional modem |  |  |  |
|  | Advanced modem | ITU-T Recommendation V.34 and later  V.42 bis |  |
|  | Mobile phone (circuit switched service) | PDC(1): 9 600 bit/s | PDC: 9 600 bit/s or ITU-T Recommendation V.32 bis  V.42 bis(2) |
|  | PHS(3) | PIAFS(4): 32 kbit/s or more. |  |
| (1), (2), (3) and (4) see footnotes (1), (2), (3) and (4) to Table 6. |  |  |  |

NOTE 1 – This protocol stack would be possible for extension of other mobile phone networks including International Mobile Telecommunications-2000 (IMT-2000). Support for these networks may be considered for creation of the Recommendation.

*	 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2002 in accordance with Resolution ITU-R 44.