Recommendation ITU-R BT.1699-1 (09/2009)

Harmonization of declarative application formats for interactive TV

BT Series Broadcasting service (television)

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2009

 ITU 2009

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  BT.1699-1

Harmonization of declarative application* formats 
for interactive TV

(Questions ITU-R 13/6 and ITU-T 4/9)

(2005-2009)

Scope

This Recommendation is intended to harmonize the application environment for declarative applications for interactive TV. It specifies common elements, media types and APIs at the syntactic level of the declarative application environment.

The ITU Radiocommunication Assembly,

considering

a)	that digital broadcasting services (satellite, terrestrial and cable) are becoming widely available and offer multimedia applications;

b)	that multimedia applications comprising video, audio, still-picture, text, graphics, etc. associated with interactive features have been developed;

c)	that multimedia applications planned or deployed in some Regions are using the declarative application environment;

d)	that common application formats are desirable for production and international exchange of multimedia applications;

e)	that ITU-T Recommendation J.200 defines, in addition to the definition above, the high-level architecture for a harmonized set of interactive application formats and application programming interfaces (APIs) and identifies the structure of application environment comprising the procedural application environment and the declarative application environment for digital television services;

f)	that ITU-T Recommendation J.202 specifies the common core of procedural application formats in the procedural application environment for interactive TV applications;

g)	that specification of harmonized declarative content formats in the declarative application environment is also required for interactive TV applications,

recommends

1	that for interactive TV applications in the declarative application environment, the harmonized declarative application formats specified in Annexes 1-7 should be used.

Annex 1

Common core of the declarative application formats
for interactive TV

# 1	Introduction

This Recommendation identifies functional commonality among the declarative application environments for interactive TV application specifications ACAP-X, BML and DVB-HTML. Elements which are common to these three standards are identified as a “Common Core”. The value of the Common Core is to assist program authors to exchange declarative applications internationally using these standards. This Recommendation also notes features outside of the Common Core of the covered standards. The goal of this Recommendation is to note these differences to encourage efforts toward increasing commonality between the standards to further improve functionality and enhance economies of scale.

# 2	Overview

This Recommendation is intended to harmonize the application environment for declarative applications for interactive TV. It specifies common elements, media types and APIs at the syntactic level of the declarative application environment to satisfy regional application requirements for the three standards ACAP-X, BML and DVB-HTML as specified in the normative references below. This Recommendation is divided into seven annexes. Annex 2 describes the Common Core of the three standards. Annex 3 describes additional functionality outside the Common Core for BML. Annex 4 describes additional functionality outside the Common Core for ACAP-X. Annex 5 describes additional functionality outside the Common Core for DVB-HTML.

The format described in Annex 6 is an intermediate format for translation between formats including the Common Core and the standards covered in this Recommendation. The format described in Annex 7 is a framework to bind content authored in multiple formats into single content.

It is noted that there are other declarative formats such as ETSIMHEG-5, which are not covered in this Recommendation. However, the migration from environments in use to the harmonized environment is assisted by the identification of a Common Core and the translation using the intermediate format.

# 3	References

## 3.1	Normative references

| [1] BML | ARIB STD-B24 V5.2 |
| --- | --- |
| [2] ACAP-X | ATSC A/101 |
| [3] DVB-HTML | ETSI TS 102 812 V1.2.2 |
| [4] wTVML | ETSI TS 102 322 V1.1.1 |
| [5] NCL | ABNT NBR 15606-2 V2 |

Users of this Recommendation are encouraged to investigate the possibility of applying the most recent editions of the references listed above, whose maintenance is the responsibility of the issuing standard bodies. Content authors should refer to the cited documentation to ensure conformity with the semantics provided by those elements, media types and APIs.

NOTE 1 – BML, ACAP-X, DVB-HTML, wTVML and NCL standards are available via the links in Appendix 1 to this Recommendation.

NOTE 2 – By agreement between ITU-R, ABNT, ATSC, ARIB and ETSI, the versions listed in § 3.1 were authorized for use by ABNT, ATSC, ARIB and ETSI, and accepted by ITU-R for inclusion in this Recommendation. Any subsequent versions of these standards which have not been accepted and approved by ITU-R are not part of this Recommendation.

## 3.2	Informative references

| [1] ETSI-MHEG | ETSI TS 202 184 V1.1.1 |
| --- | --- |
| [2] J.202 | ITU-T J.202 |
| [3] J.200 | ITU-T J.200 |

## 3.3	Terms and definitions

See the normative references listed in § 3.1.

Annex 2

Common core

# 1	Overview

Methodology for Common Core, Common Core of Media types, XML Markup, Stylesheet Markup, Monomedia and Behavioural APIs, which are based on the commonality between ACAP-X, BML and DVB-HTML are described below. Note that BML has four content profiles. Except where otherwise noted, all four profiles of BML are assumed.

## 1.1	Methodology

### 1.1.1	Layer model

Graphics layer should lie on top of other layers such as video or text plane.

### 1.1.2	Application life cycle

There should be a mechanism to destroy an application from outside the application itself.

## 1.2	Media type

Common media types are listed in Table 1.

TABLE  1

Common media type

Image/jpeg

Image/png

Text/css

Application/xhtml+xml

## 1.3	Schema

Common schema is listed in Table 2.

TABLE 2

Common schema

http://

https://

## 1.4	XML markup

Common XML markups are listed in Table 3.

TABLE  3

Common XML markup module

Structure

Text

Hypertext

List

Presentation

Bidirectional text

Forms

Image

Client Side Image Map

Object

Frames

Target

Meta Information

Scripting

Stylesheet

Style Attribute

Link

Base

Common XML markups for BML for basic services (fixed terminal profile), ACAP-X and DVB-HTML are listed in Table 4.

TABLE  4

Common XML markup for BML for basic services, ACAP-X and DVB-HTML

| Common attributes |  |  |  |
| --- | --- | --- | --- |
| Core attributes |  |  | id class |
| Style attributes |  |  |  |
|  |  |  | style |
| Core modules |  |  |  |
| Structure module |  |  |  |
|  | body |  | %Core.attrib; |
|  | head |  |  |
|  | title |  |  |
| Text module |  |  |  |
|  | br |  | %Core.attrib |
|  | div(1) |  | %Common.attrib |
|  | p(1), ( 2) |  | %Common.attrib |
|  | span |  | %Common.attrib |
| Hypertext module |  |  |  |
|  | a |  | %Common.attrib accesskey href |
| Forms module |  |  |  |
|  | input(1) | %Common.attrib accesskey disabled readonly maxlength type value |  |
| Object module |  |  |  |
|  | object(1) | %Common.attrib data type |  |
| Metainformation module |  |  |  |
|  | meta | name content |  |
| Scripting module |  |  |  |
|  | script |  |  |
| Stylesheet module |  |  |  |
|  | style |  |  |
| (1) Only these elements can be a child element of <div>. (2) Only these elements and CDATA can be a child element of <p>. |  |  |  |

## 1.5	Stylesheet

### 1.5.1	Common stylesheet properties

Common stylesheet properties are listed in Table 5.

TABLE  5

Common stylesheet properties

| Background | Clear | Outline-color |
| --- | --- | --- |
| Background-attachment | Clip | Outline-style |
| Background-color | Color | Outline-width |
| Background-image | Content | Overflow |
| Background-position | Counter-increment | Padding |
| Background-repeat | Counter-reset | Padding-bottom |
| Border | Display | Padding-left |
| Border-bottom | Float | Padding-right |
| Border-bottom-color | Font | Padding-top |
| Border-bottom-style | Font-family | Position |
| Border-bottom-width | Font-size | Right |
| Border-color | Font-style | Text-align |
| Border-left | Font-variant | Text-decoration |
| Border-left-color | Font-weight | Text-indent |
| Border-left-style | Height | Text-transform |
| Border-left-width | Left | Top |
| Border-right | Letter-spacing | Vertical-align |
| Border-right-color | Line-height | Visibility |
| Border-right-style | List-style | White-space |
| Border-right-width | List-style-image | Width |
| Border-style | List-style-position | Word-spacing |
| Border-top | List-style-type | Z-index |
| Border-top-color | Margin | Nav-index |
| Border-top-style | Margin-bottom | Nav-left |
| Border-top-width | Margin-left | Nav-right |
| Border-width | Margin-right | Nav-up |
| Bottom | Margin-top | Nav-down |
| Caption-side | Outline |  |

Common stylesheet properties for BML for basic services, ACAP-X and DVB-HTML are listed in Table 6.

TABLE  6

Common stylesheet properties for BML for basic services,
ACAP-X and DVB-HTML

@media

Margin

Padding-top

Padding-right

Padding-bottom

Padding-left

Border-width

Border-style

Position

Left(1)

Top(1)

Width(1)

Height(1)

Z-index

Line-height

Display

Visibility

Overflow

Background-image

Background-repeat

Font-family

Font-size

Font-weight

Text-align

Letter-spacing

White-space

(1) The elements <input>, <object>, <div>, and <p> must have these property values. The elements <br>, <a>, <span> must not have these property values.

Furthermore, the following restrictions should be applied:

–	display property
Only block element can be applied for <p>, <div>, <body>, <input> and <object>.
Only inline values can be applied for <br>, <a> and <span>.

–	position property
Only absolute values can be applied for <p>, <div>, <input> and <object>.
Only static values can be applied for <br>, <span> and <a>.

### 1.5.2	Common CSS selectors

Common CSS selectors are listed in Table 7.

TABLE  7

Common CSS selectors

Universal

Type

Descendant

Class

Id

:first-child pseudo-class

:link pseudo-class

:hover pseudo-class

:active pseudo-class

:focus pseudo-class

:lang pseudo-class

:pseudo-elements (:first-child, :first-letter, :before, :after)

Common CSS selectors for BML for basic services, ACAP-X and DVB-HTML are listed in Table 8.

TABLE  8

Common CSS selectors for BML for basic services, 
ACAP-X and DVB-HTML

Universal

Type

Dynamic(:focus and :active)

Class

Id

## 1.6	Scripting language

Common scripting language is ECMAScript 2nd Edition with the following restriction:

–	Number type supports integer operation only.

Common native objects for BML for basic services, ACAP-X and DVB-HTML are listed in Table 9.

TABLE  9

Common native objects for BML for basic services, ACAP-X and DVB-HTML

| Object | Methods, properties |
| --- | --- |
| (global) | NaN parseInt(string, radix) isNaN(number) |
| Object | All |
| Object.prototype | All |
| Function | prototype length |
| Function.prototype | All |
| Array | All |
| Array.prototype | All |
| String | All |
| String.prototype | All |
| Boolean | All |
| Boolean.prototype | All |
| Number | Prototype MAX_VALUE MIN_VALUE NaN Number([value]) New number([value]) |
| Number.prototype | All |
| Date | prototype Date([year [, month [, date [, hours [, minutes [, seconds [, ms ]]]]]]]) new Date([year [, month [, date [, hours [, minutes [, seconds [, ms ]]]]]]]) |
| Date.prototype | toString() getFullYear() getUTCFullYear() getMonth() getUTCMonth() getDate() getUTCDate() getDay() getUTCDay() getHours() getUTCHours() getMinutes() getUTCMinutes() getSeconds() getUTCSeconds() getMilliseconds() getUTCMilliseconds() getImtezoneOffset() setMilliseconds(ms) setUTCMilliseconds(ms) setSeconds(sec [, ms]) setUTCSeconds(sec [, ms]) setMinutes(min, [, sec [, ms]]) setUTCMinutes(min, [, sec [, ms]]) setHours(hours, [,(min, [, sec [, ms]])] setUTCHours(hours, [,(min, [, sec [, ms]])] setDate(date) setMonth(mon [, date]) setUTCMonth(mon [, date]) setFullYear(year [, mon [, date]]) setUTCFullYear{year [, mon [, date]]} toLocaleString() toUTCString() |

For BML for basic services, the length to represent signed integer is 32 bits including sign.

## 1.7	DOM API

Common DOM APIs in DOM level 1 are listed in Table 10.

TABLE  10

Common DOM level 1 APIs

| Core fundamental | DOMException |
| --- | --- |
|  | DOMImplementation |
|  | DocumentFragment |
|  | Document |
|  | Node |
|  | NodeList |
|  | NamedNodeMap |
|  | CharacterData |
|  | Attr |
|  | Element |
|  | Text |
|  | Comment |

Common DOM level 1 APIs for the BML for basic services, ACAP-X and DVB-HTML are listed in Table 11. Interfaces listed in Table 11 that have no specified attributes or methods cover all attributes and methods of the interfaces.

TABLE  11

Common DOM level 1 APIs for BML for basic services, 
ACAP-X and DVB-HTML

|  | Interface | Attributes, Methods |
| --- | --- | --- |
| Core fundamental | DOMImplementation |  |
|  | Document | implementation documentElement |
|  | Node | parentNode firstChild lastChild previousSibling nextSibling |
|  | CharacterData | data length |
|  | Element | tagName |
|  | Text |  |

Annex 3

Additional elements, media types and APIs for BML

Elements, media types and APIs for BML in addition to those listed in Annex 2 are described below. Items marked “BD)” are common to BML and DVB-HTML. Items marked “BA)” are common to BML and ACAP-X.

# 1	Additional BML media types

Additional BML media types are listed in Table 12.

TABLE  12

Additional BML media types

Multipart/mixed

Text/xmlBD)

Text/xsl

Text/html

Text/plainBD)

Text/css

Text/X-arib-bml;charset=“euc-jp”

Text/X-arib-bml;charset=“UTF-16”

Text/X-arib-bml;charset=“Shift_JIS”

Text/X-arib-bml;charset=“UTF-8”

Text/X-arib-jis8text

Text/X-arib-ecmascript;charset=“euc-jp”

Text/X-arib-ecmascript;charset=“UTF-16”

Text/X-arib-ecmascript;charset=“Shift_JIS”

Text/X-arib-ecmascript;charset=“UTF-8”

Image/gif

Image/X-arib-png

Image/X-arib-mng

Image/X-arib-mpeg2-I

Image/X-arib-mpeg4-I-simple

Image/X-arib-mpeg4-I-core

Image/X-arib-H264-I-baseline

Image/X-arib-H264-I-main

Audio/X-arib-mpeg2-aac

Audio/X-arib-mpeg2-bc

Audio/X-arib-mpeg4

Audio/X-arib-aiff

TABLE  12 (end)

Audio/X-arib-additional

Audio/X-arib-romsound

Application/X-arib-stream-text;charset=“euc-jp”

Application/X-arib-stream-text;charset=“UTF-16”

Application/X-arib-stream-text;charset=“Shift_JIS”

Application/X-arib-stream-text;charset=“UTF-8”

Application/X-arib-stream-jis8text

Application/X-arib-stream-png

Application/X-arib-stream-jpeg

Application/X-arib-stream-mpeg2-I

Application/X-arib-stream-mpeg4-I-simple

Application/X-arib-stream-mpeg4-I-core

Application/X-arib-mpeg2-tts

Application/X-arib-bmlclut

Application/X-arib-btable

Application/X-arib-drcs

Application/X-arib-PDI

Application/X-arib-resourceList

Application/X-arib-stream-H264-I-baseline

Application/X-arib-stream-H264-I-main

Application/X-arib-mpeg2-ts

Application/X-arib-rootcertificate

Application/X-arib-contentPlayContrl

Application/X-arib-streamControlInfo

Application/X-arib-meta+xml;charset=“UTF-8”

Application/X-arib-meta+xml;charset=“UTF-16”

Video/X-arib-mpeg1

Video/X-arib-mpeg2

Video/X-arib-mpeg4-simple

Video/X-arib-mpeg4-core

Video/X-arib-H264-baseline

Video/X-arib-H264-main

# 2	Additional BML XML markup

Additional BML XML markups are listed in Table 13.

TABLE  13

Additional XML markups

| Module | Tag |
| --- | --- |
| TableBA) | All |
| Intrinsic eventsBA) | All |
| Name identificationBA) | All |
| Applet | All |
| Basic forms | All |
| Basic tableBD) | All |
| Server side image map | All |
| IframeBD) | All |
| Legacy | All |
| BML extension | Bml, bevent, beitem, body&, div&, p&, span&, a&, bdo&, object& |

# 3	Additional BML CSS properties

Additional BML CSS properties are listed in Table 14.

TABLE  14

Additional CSS properties

Clut(1)

Color-index(1)

Background-color-index(1)

Border-color-index

Border-top-color-index(1)

Border-right-color-index(1)

Border-bottom-color-index(1)

Border-left-color-index(1)

Outline-color-index

Resolution(1)

Display-aspect-ratio(1)

Grayscale-color-index(1)

Used-key-list(1)

nav-index(1)

nav-up(1)

nav-down(1)

TABLE  14 (end)

nav-left(1)

nav-right(1)

-wap-marquee

-wap-marquee-style

-wap-marquee-loop

-wap-marquee-dir

-wap-marquee-speed

-wap-accesskey

-wap-input-format

-wap-input-required

(1) These attributes are employed for BML for basic services.

# 4	Additional BML DOM APIs

Additional BML DOM level 1 APIs are listed in Table 15.

TABLE  15

Additional BML DOM level 1 APIs

| Core extensionBA) | CDATASection |
| --- | --- |
|  | DocumentType |
|  | Notation |
|  | Entity |
|  | EntityReference |
|  | ProcessingInstruction |
| HTML | HTMLCollectionBA) |
|  | HTMLDocumentBA) |
|  | HTMLElementBA) |
|  | HTMLAnchorElementBA) |
|  | HTMLFormElementBA) |
|  | HTMLInputElementBA) |
|  | HTMLOptionElementBA) |
|  | HTMLSelectElementBA) |
|  | HTMLTextAreaElementBA) |
|  | HTMLImageElementBA) |
|  | HTMLObjectElementBA) |
|  | HTMLBodyElementBA) |
|  | HTMLBlockquoteElement |
|  | HTMLPreElement |
|  | HTMLHeadingElement |

TABLE  15 (end)

| HTML (cont.) | HTMLHRElement |
| --- | --- |
|  | HTMLDivElement(1) |
|  | HTMLParagraphElement(1) |
|  | HTMLQuoteElement |
|  | HTMLBRElement(1) |
|  | HTMLModElement |
|  | HTMLBaseElement |
|  | HTMLLinkElement |
|  | HTMLDListElement |
|  | HTMLOlistElement |
|  | HTMLUListElement |
|  | HTMLLIElement |
|  | HTMLButtonElement |
|  | HTMLFieldSetElement |
|  | HTMLLabelElement |
|  | HTMLLegendElement |
|  | HTMLOptGroupElement |
|  | HTMLTableCaptionElement |
|  | HTMLTableColElement |
|  | HTMLTableElement |
|  | HTMLTableSectionElement |
|  | HTMLTableCaptionElement |
|  | HTMLTableColElement |
|  | HTMLTableElement |
|  | HTMLTableSectionElement |
|  | HTMLTableCellElement |
|  | HTMLTableRowElement |
|  | HTMLAreaElement |
|  | HTMLMapElement |
|  | HTMLParamElement |
|  | HTMLFrameSetElement |
|  | HTMLFrameElement |
|  | HTMLIFrameElement |
|  | HTMLMetaElement(1) |
|  | HTMLTitleElement(1) |
|  | HTMLScriptElement(1) |
|  | HTMLStyleElement(1) |
|  | HTMLHeadElement(1) |
|  | HTMLHtmlElement(1) |
| (1) These elements are employed for BML for basic services. |  |

BML extensions of DOM APIs are listed in Table 16.

TABLE  16

Additional BML extensions

| BML extension | BMLDocument (1) |
| --- | --- |
|  | BMLCSS2Properties(1) |
|  | BMLEvent(1) |
|  | BMLIntrinsicEvent(1) |
|  | BMLBeventEvent(1) |
|  | BMLDocument(1) |
|  | BMLElement |
|  | BMLBlockquoteElement |
|  | BMLPreElement |
|  | BMLHeadingElement |
|  | BMLHRElement |
|  | BMLDivElement(1) |
|  | BMLSpanElement(1) |
|  | BMLParagraphElement(1) |
|  | BMLQuoteElement |
|  | BMLBRElement(1) |
|  | BMLModElement |
|  | BMLAnchorElement(1) |
|  | BMLLinkElement |
|  | BMLDListElement |
|  | BMLOListElement |
|  | BMLUListElement |
|  | BMLLIElement |
|  | BMLButtonElement |
|  | BMLFieldSetElement |
|  | BMLFormElement |
|  | BMLInputElement(1) |
|  | BMLLabelElement |
|  | BMLLegenedElement |
|  | BMLOptGroupElement |
|  | BMLOptionElement |
|  | BMLSelectElement |
|  | BMLTexAreaElement |
|  | BMLTableCaptionElement |
|  | BMLTableColElement |
|  | BMLTableElement |

TABLE  16 (end)

| BML extension (cont.) | BMLTableSectionElement |
| --- | --- |
|  | BMLTableCellElement |
|  | BMLTableRowElement |
|  | BMLImageElement |
|  | BMLAreaElement |
|  | BMLMapElement |
|  | BMLObjectElement(1) |
|  | BMLFrameSetElement |
|  | BMLFrameElement |
|  | BMLIFrameElement |
|  | BMLBodyElement(1) |
|  | BMLBmlElement(1) |
|  | BMLBeventElement(1) |
|  | BMLBeitemElement(1) |
| (1) These elements are employed for BML for basic services. |  |

# 5	Markup language switch

A function added to ECMAScript to launch another declarative application environment is listed in Table 17.

TABLE 17

Markup language switch function

Number startExtraBrowser( input String browserName, input Number showAV, input String returnURI, input String uri )

Annex 4

Additional elements, media types and APIs for ACAP-X

Elements, media types and APIs for ACAP-X in addition to those listed in Annex 2 are described below. Items marked “AD)” are common to ACAP-X and DVB-HTML. Items marked “AB)” are common to ACAP-X and BML.

# 1	Additional ACAP-X media types

Additional ACAP-X media types are listed in Table 18.

TABLE  18

Additional ACAP-X media types

Application/acap-j

Application/acap-certificate

Application/acap-digest

Application/acap-permission

Application/acap-signature

Application/acap-x

Application/acap-x-metadata

Application/font-tdpfr

Application/java

Application/zip

Application/xhtml+xml

Audio/ac3

Audio/basic

Audio/mpegAD)

Image/mpegAD)

Text/ecmascriptAD)

Video/mng

Video/mpeg

Video/mpv

# 2	Additional ACAP-X XML markup

Additional ACAP-X XML markups are listed in Table 19.

TABLE  19

Additional ACAP-X XML markups

| Module | Tag |
| --- | --- |
| TableAB) | All |
| Intrinsic EventsAB) | All |
| Name IdentificationAB) | All |

# 3	Additional ACAP-X CSS properties

Additional ACAP-X CSS properties and selectors are listed in Table 20.

TABLE  20

Additional ACAP-X CSS properties and selectors

| Properties | Atsc-dynamic-refresh |
| --- | --- |
| Selectors | Child |
|  | Adjacent sibling |
|  | Attribute and attribute values |

# 4	Additional ACAP-X stylesheet attributes

Additional ACAP-X stylesheet attributes are CSS level 2, CSS-BOX, CSS-COLOR, CSS-TV, CSS-UI and their related DOM APIs.

# 5	Additional ACAP-X DOM APIs

Additional ACAP-X DOM level 2 APIs are listed in Table 21.

TABLE  21

Additional ACAP-X DOM level 2 APIs

| Core fundamentalAD) | DOMException |
| --- | --- |
|  | DOMImplementation |
|  | DocumentFragment |
|  | Document |
|  | Node |
|  | NodeList |
|  | NamedNodeMap |
|  | CharacterData |

TABLE  21 (continued)

| Core fundamentalAD) (cont.) | Attr |
| --- | --- |
|  | Element |
|  | Text |
|  | Comment |
| Core extensionAB) | CDATASection |
|  | DocumentType |
|  | Notation |
|  | Entity |
|  | EntityReference |
|  | ProcessingInstruction |
| HTMLAB) | HTMLAnchorElement |
|  | HTMLBodyElement |
|  | HTMLCollection |
|  | HTMLDocument |
|  | HTMLElement |
|  | HTMLFormElement |
|  | HTMLInputElement |
|  | HTMLObjectElement |
|  | HTMLOptionElement |
|  | HTMLSelectElement |
|  | HTMLTextAreaElement |
|  | HTMLImageElement |
| View | AbstractView |
|  | DocumentView |
| Style sheetsAD) | DocumentStyle |
|  | LinkStyle |
|  | MediaList |
|  | Stylesheet |
|  | StylesheetList |
| CSS | Counter |
|  | CSSCharsetRule |
|  | CSSFontFaceRule |
|  | CSSImportRule |
|  | CSSMediaRule |
|  | CSSPageRule |
|  | CSSPrimitiveValue |
|  | CSSRule |
|  | CSSRulesList |
|  | CSSStyleDeclaration |

TABLE  21 (end)

| CSS (cont.) | CSSStyleRule |
| --- | --- |
|  | CSSStyleSheet |
|  | CSSUnknownRule |
|  | CSSValue |
|  | CSSValueList |
|  | DocumentCSS |
|  | DOMImplementationCSS |
|  | ElementCSSInlineStyle |
|  | Rect |
|  | RGBColor |
| EventAD) | ViewCSS |
|  | Event |
|  | EventException |
|  | EventListner |
|  | EventTarget |
| EventSet | KeyEvent |
|  | KeyModifiers |
|  | MouseEventAD) |
|  | MutationEventAD) |
|  | UIEventAD) |
|  | VirtualKeys |

ACAP-X extensions of DOM APIs are listed in Table 22.

TABLE  22

Additional ACAP-X extensions

| ACAP-X Extension | DOMExceptionExt |
| --- | --- |
|  | HTMLAnchorElementExt |
|  | HTMLDocumentExt |
|  | HTMLImageElementExt |
|  | HTMLFormElementExt |
|  | HTMLObjectElementExt |
|  | HTMLTriggerObjectElementExt |
|  | HTMLOptionsCollection |
|  | DocumentViewExt |

Annex 5

Additional elements, media types, and APIs for DVB-HTML

Elements, media types and APIs for DVB-HTML in addition to those listed in Annex 2 are described below. Items marked “DB)” are common to DVB-HTML and BML. Items marked “DA)” are common to DVB-HTML and ACAP-X.

# 1	Additional DVB-HTML media types

Additional DVB-HTML media types are listed in Table 23.

TABLE  23

Additional DVB-HTML media types

Application/xml

Application/dvbj

Application/dvb.pfr

Audio/mpegDA)

Image/gif

Image/mpegDA)

Text/ecmascriptDA)

Text/plainDB)

Text/css

Text/xmlDB)

Text/dvb.utf8

Multipart/dvb.service

Video/dvb.mpeg.drip

# 2	Additional DVB-HTML XML markups

Additional DVB-HTML XML markups are listed in Table 24.

TABLE  24

Additional XML markups

Basic TableDB)

IframeDB)

# 3	Additional DVB-HTML CSS properties

Additional DVB-HTML CSS properties and selectors are listed in Table 25.

TABLE  25

Additional DVB-HTML CSS properties and selectors

| Properties | Direction |
| --- | --- |
|  | Unicode-bidi |
|  | Min-width |
|  | Max-width |
|  | Min-height |
|  | Max-height |
|  | Font-stretch |
|  | Font-size-adjust |
|  | Table-layout |
|  | Empty-cells |
|  | Speak-header |
|  | Opacity |
|  | Nav-first |
|  | Clip-video |
|  | Compose-rule |
| Selectors | Child |
|  | Adjacent sibling |
|  | Attribute and attribute values |

# 4	Additional DVB-HTML DOM APIs

## 4.1	Additional DVB-HTML DOM level 1 APIs

Additional DVB-HTML DOM level 1 APIs are listed in Table 26.

TABLE  26

Additional DVB-HTML DOM level 1 APIs

| HTML | DVBHTMLCollection |
| --- | --- |
|  | DVBHTMLDocument |
|  | DVBHTMLElement |
|  | DVBHTMLAnchorElement |
|  | DVBHTMLButtonElement |
|  | DVBHTMLFormElement |
|  | DVBHTMLInputElement |
|  | DVBHTMLOptionElement |

TABLE  26 (end)

| HTML (cont.) | DVBHTMLSelectElement |
| --- | --- |
|  | DVBHTMLTextAreaElement |
|  | DVBHTMLImageElement |
|  | DVBHTMLAreaElement |
|  | DVBHTMLMapElement |
|  | DVBHTMLObjectElement |
|  | DVBHTMLFrameSetElement |
|  | DVBHTMLFrameElement |
|  | DVBHTMLIFrameElement |

## 4.2	Additional DVB-HTML DOM level 2 APIs

Additional DVB-HTML DOM level 2 APIs are listed in Table 27.

TABLE  27

Additional DVB-HTML DOM level 2 APIs

| Core fundamentalDA) | DOMException |
| --- | --- |
|  | DOMImplementation |
|  | DocumentFragment |
|  | Document |
|  | Node |
|  | NodeList |
|  | NamedNodeMap |
|  | CharacterData |
|  | Attr |
|  | Element |
|  | Text |
|  | Comment |
| View | AbstractView |
|  | DocumentView |
| Style sheetsDA) | DocumentStyle |
|  | LinkStyle |
|  | MediaList |
|  | Stylesheet |
|  | StylesheetList |

TABLE  27 (end)

| EventDA) | DocumentEvent |
| --- | --- |
|  | Event |
|  | EventException |
|  | EventListner |
|  | EventTarget |
| EventSetDA) | MouseEvent |
|  | MutationEvent |
|  | UIEvent |

Annex 6

Presentation interoperability through translation

Some service providers may find the core functionality a little limiting for their purposes, though they still wish to target the multiple presentation engines identified in this Recommendation.

As a supplement to the core functionality, the Worldwide Television Markup Language (wTVML) specified in ETSI TS 102 322 defines a format to author such interactive services, which can then be mechanically translated to any desired presentation markup language. The wTVML format uses an XML data structure with declarative behaviour and little or no scripting, and as such is easier to translate to other markup languages. Because wTVML expresses the author’s intent, rather than implementation, the richer non-core features of each supported markup become available for use.

In addition, wTVML can also be used as a native presentation language.

When using wTVML as the intermediate format for format translation of the declarative application, the following may require careful consideration for translation of the original application to wTVML in particular organization of the application:

–	broadcast message signal, such as that carried by DSM-CC stream event;

–	additional functions in scripting language, such as that for cache control.

Annex 7

Presentation interoperability by management framework of 
multiple formats for declarative applications

Some service providers may wish to employ multiple formats identified in this Recommendation including the common core. Usage of the multiple formats can be in many ways, such as concurrent use, switch of one format to another, and so on. This means management framework for content employing multiple formats is required.

As a framework for management of multiple declarative application formats, Nested Context Language (NCL) specified in ABNT NBR 15606-2 defines a format to bind content authored in multiple formats into single content. NCL is a glue language based on XML that holds media objects together in a multimedia presentation, no matter which is each object type.

When using NCL as the framework to bind content authored in different declarative application formats, following may request careful consideration to organize binding content:

–	System time base can only be in NCL, not in each media object. In particular, time based event such as an event triggered by normal play time should be handled in NCL. LuaScript is one of the processing mechanisms for this kind of behaviour in NCL.

Appendix 1

Standards

BML

ACAP-X

DVD-HTML

wTVML

NCL

NOTE 1 – BML standard is available at <http://www.arib.or.jp/english/html/overview/sb_ej.html>.

NOTE 2 – ACAP-X standard is available at <http://www.itu.int/rec/R-REC-BT.1699-0-200502-I/en>.

NOTE 3 – DVB-HTML standard is available at <http://www.itu.int/rec/R-REC-BT.1699-0-200502-I/en>.

NOTE 4 – wTVML standard is available at http://webapp.etsi.org/workprogram/Report_workitem.asp? WKI_ID=19886.

NOTE 5 – NCL standard is available at <http://abnt.iso.org/livelink/livelink/fetch/2000/2827/ 7589984/8699711/8727725/ABNTNBR15606%2D2_2007Ing_2008Vc2_2009.pdf>.

*	ITU-T Recommendation J.200 provides the definition for “declarative application”: An application which primarily makes use of declarative information to express its behaviour; an XML document instance is an example of a declarative application.