Recommendation ITU-R BT.1848 (05/2009)

Safe areas of wide-screen 16:9 aspect ratio digital production

BT Series Broadcasting services (television)

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2009

 ITU 2009

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  BT.1848

Safe areas* of wide-screen 16:9 aspect ratio digital productions

(2008)

Scope

This Recommendation provides guidelines on safe areas of 625-line, 720-line and 1 080-line formats of wide-screen 16:9 aspect ratio digital productions.

The ITU Radiocommunication Assembly,

considering

a)	that Recommendation ITU-R BT.1379-2 – Safe areas of wide-screen 16:9 and standard 4:3 aspect ratio productions to achieve a common format during a transition period to wide-screen 16:9 broadcasting, already exists;

b)	that the ability to use a 16:9 chain for the transmission of a programme in 4:3 will encourage the introduction of new 16:9 equipment;

c)	that the ability to use a single master for simultaneous transmission to both digital 16:9 and  analogue 4:3 services would encourage the transition to 16:9 broadcasting;

d)	that the use of component digital 16:9 video chains will provide viewers watching 16:9 services with the optimum picture quality;

e)	that wide-screen formats may include 1 080-line and 720-line as well as lower resolution;

f)	that digital technology in cameras, distribution, and displays makes it possible to maintain the originally captured format throughout the broadcasting chain;

g)	that display overscan is both unnecessary and undesirable for digital television;

h)	that consumers now watch television on their computer monitors which typically use displays without overscan,

recommends

1	that, in the case of programmes intended for 625-line transmission, account should be taken of the guidelines for safe areas described in Annex 1;

2	that, in the case of programmes intended for 720-line transmission, account should be taken of the guidelines for safe areas described in Annex 2;

3	that, in the case of programmes intended for 1 080-line transmission, account should be taken of the guidelines for safe areas described in Annex 3,

further recommends

4	that manufacturers of consumer television displays should be strongly encouraged to produce displays without overscan in order for the full image as approved by the programme producer to be enjoyed by all consumers;

5	that, where available, the use of a fully component electronic production chain in the 16:9 aspect ratio should be preferred.

Annex 1

Safe areas for television programmes intended
for wide-screen 625-line transmission

These guidelines are aimed at those involved in any stage of the programme-making process and at manufacturers of production equipment for programmes intended to be broadcast using 625-line transmission systems.

All the safe areas have been specified on the premise that the overscan on modern domestic television receiver displays will normally be in the range 7.0  1% of overall picture width or height. But for any one picture edge, the overscan should not exceed 4% of total picture width or height.

Appendix 1
to Annex 1

Safe areas for television programmes made in the 625-line interlaced scan 
16:9 wide-screen format: Shoot-to-protect the 16:9 full image

Table 1 shows how the action and graphics areas are defined to protect the full 16:9 wide-screen image.

TABLE 1

|  | Vertical | Horizontal |
| --- | --- | --- |
| Action safe margin (%)(1), (2) | 3.5 | 3.5 |
| Graphics safe margin (%)(1), (3) | 5 | 5 |
| (1) Defined in Recommendation ITU-R BT.1379-2. (2) The action safe margin is 3.5% at the top, bottom and lateral parts of the original image. (3) The graphics safe margin is 5% at the top, bottom and lateral parts of the original image. |  |  |

Figure 1 shows these areas in more detail.

Figure 1

16:9 shoot-to-protect the 16:9 full image, 625-line interlaced scan

The definitions of the safe areas are given in numbers of lines and pixels, which are more definitive than the percentages used previously. However, percentages are also included because they are the basis on which comparisons are made. The line numbering has been calculated on the basis that field 1 is paired with the field 2 line below it, and the line from field 1 which is just inside the percentage box is defined as the edge of active picture.

Thus the drawings give the first and last lines and the first and last pixels which are inside the safe areas.

Appendix 2
to Annex 1

Safe areas for television programmes made in the 625-line progressive scan 
16:9 wide-screen format: Shoot-to-protect the 16:9 full image

Table 2 shows how the action and graphics areas are defined to protect the full 16:9 wide-screen image.

TABLE 2

|  | Vertical | Horizontal |
| --- | --- | --- |
| Action safe margin (% | 3.5 | 3.5 |
| Graphics safe margin (% | 5 | 5 |

Figure 2 shows these areas in more detail.

Figure 2

16:9 shoot-to-protect the 16:9 full image, 625-line progressive scan

The definitions of the safe areas are given in numbers of lines and pixels, which are more definitive than the percentages used previously. However, percentages are also included because they are the basis on which comparisons are made.

Thus the drawings give the first and last lines and the first and last pixels which are inside the safe areas.

Annex 2

Safe areas for television programmes intended
for wide-screen 720-line transmission

This guideline is aimed at those involved in any stage of the programme-making process and at manufacturers of production equipment for programmes intended to be broadcast using 720-line transmission systems.

The safe areas have been specified on the premise that the overscan on modern domestic television receiver displays will normally be in the range 7.0  1% of overall picture width or height. But for any one picture edge, the overscan should not exceed 4% of total picture width or height.

Appendix 1
to Annex 2

Safe areas for television programmes made in the 720-line progressive scan 
16:9 wide-screen format: Shoot-to-protect the 16:9 full image

Table 3 shows how the action and graphics areas are defined to protect the full 16:9 wide-screen image.

TABLE 3

|  | Vertical | Horizontal |
| --- | --- | --- |
| Action safe margin (%) | 3.5 | 3.5 |
| Graphics safe margin (%) | 5 | 5 |

Figure 3 shows these areas in more detail.

Figure 3

16:9 shoot-to-protect the 16:9 full image, 720-line progressive scan

The definitions of the safe areas are given in numbers of lines and pixels, which are more definitive than the percentages used previously. However, percentages are also included because they are the basis on which comparisons are made.

Thus the drawings give the first and last lines and the first and last pixels which are inside the safe areas.

Annex 3

Safe areas for television programmes intended
for wide-screen 1 080-line transmission

These guidelines are aimed at those involved in any stage of the programme-making process and at manufacturers of production equipment for programmes intended to be broadcast using 1 080-line transmission systems.

All the safe areas have been specified on the premise that the overscan on modern domestic television receiver displays will normally be in the range 7.0  1% of overall picture width or height. But for any one picture edge, the overscan should not exceed 4% of total picture width or height.

Appendix 1
to Annex 3

Safe areas for television programmes made in the 1 080-line interlaced scan 
16:9 wide-screen format: Shoot-to-protect the 16:9 full image

Table 4 shows how the action and graphics areas are defined to protect the full 16:9 wide-screen image.

TABLE 4

|  | Vertical | Horizontal |
| --- | --- | --- |
| Action safe margin (% | 3.5 | 3.5 |
| Graphics safe margin (% | 5 | 5 |

Figure 4 shows these areas in more detail.

FIGURE 4

16:9 shoot-to-protect the 16:9 full image, 1 080-line interlaced scan

The definitions of the safe areas are given in numbers of lines and pixels, which are more definitive than the percentages used previously. However, percentages are also included because they are the basis on which comparisons are made. The line numbering has been calculated on the basis that field 1 is paired with the field 2 line below it, and the line from field 1 which is just inside the percentage box is defined as the edge of active picture.

Thus the drawings give the first and last lines and the first and last pixels which are inside the safe areas.

Appendix 2
to Annex 3

Safe areas for television programmes made in the 1 080-line progressive scan
16:9 wide-screen format: Shoot-to-protect the 16:9 full image

Table 5 shows how the action and graphics areas are defined to protect the full 16:9 wide-screen image.

TABLE 5

|  | Vertical | Horizontal |
| --- | --- | --- |
| Action safe margin (% | 3.5 | 3.5 |
| Graphics safe margin (% | 5 | 5 |

Figure 5 shows these areas in more detail.

FIGURE 5

16:9 shoot-to-protect the 16:9 full image, 1 080-line progressive scan

The definitions of the safe areas are given in numbers of lines and pixels, which are more definitive than the percentages used previously. However, percentages are also included because they are the basis on which comparisons are made.

Thus the drawings give the first and last lines and the first and last pixels which are inside the safe areas.

*	Safe areas are located within the active image area of television production systems and ensure visibility of critical picture elements of television programmes on the majority of home television receivers. Safe areas are generally defined by safe action area and safe graphics area; safe action area is the maximum image area within which all significant action should be contained, and safe graphics area is the maximum image area within which all significant graphics should be contained.