RECOMMENDATION  ITU-R  BT.655-3

RADIO-FREQUENCY  PROTECTION  RATIOS  FOR  AM  VESTIGIAL
SIDEBAND  TERRESTRIAL  TELEVISION  SYSTEMS

(Question ITU-R 4/11)

(1986-1990-1992-1994)

Rec. ITU-R BT.655-3

The ITU Radiocommunication Assembly,

considering

a)	that accurate protection ratio values are required in order to permit the planning of terrestrial television services in an effective manner,

recommends

1.	that the protection ratios given in Annex 1 be used for planning terrestrial television services;

2.	that studies should be undertaken to complete the information on protection ratios, in particular with reference to the items identified in § 4 of Annex 1.

ANNEX  1

Radio-frequency protection ratios for terrestrial television systems

# 1.	Introduction

This Annex contains general information related to protection ratios for terrestrial television systems. It also contains a series of Appendices, each containing protection ratios required for the protection of an individual category of system or signal.

Appendices 1 and 2 contain protection ratios for 525- and 625-line analogue television systems, respectively.

Appendix 3 contains protection ratios for the sound signals of analogue television systems.

Subsequent appendices will contain protection ratios for digital television systems when the relevant values become available.

# 2.	General

The RF protection ratio is the minimum value of wanted-to-unwanted signal ratio, usually expressed in decibels at the receiver input, determined under specified conditions such that a specific reception quality is achieved at the receiver output.

2.1	The values of protection ratio quoted apply to interference produced by a single source. Except where otherwise stated, the ratios apply to tropospheric, T, interference and correspond closely to a slightly annoying impairment condition. They are considered to be acceptable only if the interference occurs for a small percentage of the time, not precisely defined but generally considered to be between 1% and 10%. For substantially non-fading unwanted signals, it is necessary to provide a higher degree of protection and ratios appropriate to continuous, C, interference should be used (see Annex 2). If the latter are not known, then the tropospheric, T, values increased by 10 dB can be applied.

Values applicable to limit of perceptibility, LP, are given for information only.

2.2	Significantly strong wanted input signals can require higher protection ratio values because of non-linear effects in the receiver.

2.3	For 625-line systems, the reference impairment levels are those which correspond to co-channel protection ratios of 30 dB and 40 dB with a frequency-offset between vision carriers close to two-thirds of the line frequency but adjusted for maximum impairment, the precise frequency difference being 10.416 kHz. These conditions approximate to impairment grades 3 (slightly annoying) and 4 (perceptible but not annoying) and apply to tropospheric, T, and continuous, C, interference, respectively.

2.4	It should be noted that the amplitude of a vision-modulated signal is defined as the r.m.s. value of the carrier at peaks of the modulation envelope (taking no account of the chrominance signal in positive-modulation systems), while that of a sound-modulated signal is the r.m.s. value of the unmodulated carrier, both for amplitude modulation and for frequency modulation.

For planning purposes, it may be assumed that the power in the chrominance channel does not exceed a value which is 16 dB lower than the power in the vision carrier during peaks of the modulation envelope.

2.5	The protection ratio values are not affected if digital data are included in the field-blanking interval of the unwanted television signal. However, certain values are affected in the case of a full-field data unwanted signal; in particular, it is not possible to achieve the full advantages of precision offset operation.

2.6	The relationship between the vision carrier frequencies of the wanted and unwanted signal is as follows (see Annex 3):

### 2.6.1	Non-controlled condition

No special control of the nominal frequency difference between the carriers of the wanted and unwanted signals.

### 2.6.2	Non-precision offset

The difference between the nominal frequencies of the wanted and unwanted carriers is suitably related to the line frequency, the tolerance of the carrier frequencies being ± 500 Hz.

The line synchronization of television receivers must be sufficiently immune to periodic interference if full advantage of carrier offset operation is to be achieved.

### 2.6.3	Precision offset (see Annex 4 for the case of 625-line systems)

The difference between the nominal frequencies of the wanted and unwanted carriers is suitably related to the line and field frequencies, but with a tolerance of each of the nominal carrier frequencies of the order of ± 1 Hz and stability of the line frequencies equal to or better than 1 ´ 10–6. In order to take full advantage of precision offset when the interfering carrier falls in the upper video range (greater than 2 MHz) of the wanted signal, a line-frequency stability of at least 2 ´ 10–7 is necessary.

# 3.	Synchronized carrier operation

Field and laboratory tests have demonstrated that synchronized carrier television systems allow a similar reduction in co-channel interference to that achieved by use of precision offset techniques, when the same television programme is transmitted. Ratios of wanted-to-unwanted signals of 28 dB and 38 dB were found to correspond to impairment grades of 3.5 and 4.5, respectively.

No degradation of picture quality was observed when the frequency difference between both vision carriers was less than 0.2 Hz and/or the phase fluctuations were less than 20°.

The use of synchronized carrier techniques simplifies the introduction of new television transmitters and transposers into existing networks.

Further studies in this field are required, especially for the case of different television programmes.

# 4.	Further studies

In a number of cases, the available protection ratio values are incomplete. In particular, this applies to:

–	data signals,

–	out-of-channel response,

–	525-line systems,

–	synchronized carrier operation,

–	protection ratio values for digital television systems,

–	protection ratio values for the protection of analogue television signals against interference from digital television signals.

In addition, it is necessary to establish the relationships between picture quality or impairment grade and protection ratio value. While the information is available for grades 3, 4 and 4.5, it is not yet available for lower grades.

APPENDIX  1

TO  ANNEX  1

Protection ratios for 525-line television systems

# 1.	Protection from co-channel interference

In this section, the protection ratio values between two television signals apply only for interference due to the modulated vision carrier of the unwanted signal.

## 1.1	Carriers separated by less than 1 000 Hz, non-controlled systems having the same or a different line-standard

Protection ratio:  45 dB, tropospheric interference.

## 1.2	Carriers separated by parts of the line frequency (fline), systems having the same line-standard, non-precision offset (see Table 1)

TABLE  1

Protection ratio, tropospheric interference carrier separation up to
about ± 36/12 fline (about ± 50 kHz)

| Offset of line frequency | 1/2, 3/2, 5/2, ... | 1/3, 2/3, 4/3, ... |
| --- | --- | --- |
| 525-line system (dB) | 25 | 28 |

# 2.	Protection from adjacent-channel interference

The given protection ratios apply to tropospheric interference and are defined in terms of wanted and unwanted vision carrier levels. For continuous interference, the values should be increased by 10 dB.

Adjacent-channel protection ratios cannot be determined directly from the overlapping channel protection ratio curves shown in § 4 because for certain systems the values may be affected by special measures in the receiver, e.g. sound traps.

## 2.1	Protection from lower adjacent-channel interference, VHF and UHF bands

The worst interference to the picture signal from another signal using the same standard results from the sound signal in the lower adjacent channel. However, an improvement of 2-3 dB can be achieved if the frequency difference between the wanted vision and unwanted sound carriers is an odd multiple of half the line frequency. During periods of no sound modulation, the improvement is as much as 10 dB.

The figure below relates to the case where the separation between the wanted vision carrier frequency and the unwanted sound carrier frequency is 1.5 MHz and the ratio between the unwanted vision and unwanted sound powers is 10 dB.

A correction must be made for different vision to sound power ratios.

Protection ratio:  –13 dB.

## 2.2	Protection from upper adjacent-channel interference, VHF and UHF bands

Protection ratio:  –10 dB.

# 3.	Protection from image-channel interference

The protection ratio will depend on the intermediate frequency and image-channel rejection of the receiver, and on the type of unwanted signal falling in the image channel. It can be determined by subtracting the image rejection figure from the required protection ratio for overlapping channels. Table 2 shows image-channel rejection values.

TABLE  2

Image-channel rejection

| Image-channel rejection (dB) | VHF | UHF |
| --- | --- | --- |
| System M (Japan) | 60 | 45 |
| Other systems |  | 40 |

# 4.	Protection from overlapping channel interference

All figures and tables in this section give protection ratio values to be applied when a CW signal lies within the vision channel of the wanted transmission, the wanted vision signal being negatively modulated.

Corrections to be made for other types of interfering signal are given in Table 3.

TABLE  3

Correction values for different unwanted signals

| Unwanted signal | Correction factors (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Wanted signal | CW | TV-negative | TV-positive | FM-sound | AM-sound |
| Vision signal negative modulated | 0 | –2 | 0 | 0 | + 4 |

Figure 1 and Table 4 show protection ratios for tropospheric interference. For continuous interference, the values should be increased by 10 dB. The unwanted signal is a CW carrier. For other types of unwanted signal, the correction factors given in Table 3 should be applied.

FIGURE 1 (avec T4)...[D01] = 9 CM

| D¦ (MHz) |  | –1.5 | –1.0 | – 0.75 | 0.3 | 1.0 | 2.5 | 3.0 | 3.5 | 3.7 | 4.1 | 4.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | NTSC (dB) |  |  |  |  |  |  |  | 50 | 50 | 45 |  |
| A | PAL (dB) | 0 | 30 | 40 | 50 | 50 | 37 | 45 | 45 | 45 |  | 15 |
| A | Monochrome (dB) |  |  |  |  |  |  |  |  |  |  |  |
| B | Monochrome (dB) | 0 | 15 |  | 33 | 33 | 25 |  |  |  |  | 15 |
| Protection ratio (dB) |  |  |  |  |  |  |  |  |  |  |  |  |
| Curves A: non-controlled condition Curves B: non-precision offset condition Curves A: (1/3, 2/3, 4/3, 5/3, all of the line frequency) |  |  |  |  |  |  |  |  |  |  |  |  |

# 5.	Television signal affected by data signals

The inclusion of digital data such as teletext in the field-blanking interval has no effect on the required protection ratios. However, the full improvement resulting from non-precision or precision offset operation is not achievable when the unwanted signal carries a full-field data signal. No information is currently available regarding the protection ratios to be applied to a wanted 525-line system when the unwanted signal is full field teletext.

APPENDIX  2

TO  ANNEX 1

Protection ratios for 625-line television systems

# 1.	Protection from co-channel interference

In this section, the protection ratios between two television signals apply only for interference due to the modulated vision carrier of the unwanted signal. Additional protection may be necessary if the wanted sound carrier is affected, or if the unwanted sound carrier lies within the wanted vision channel (e.g. the unwanted sound carrier of system G lies within the vision channel of system K). For all protection ratio values in this section, the following corrections have to be made:

–	when the wanted signal is modulated negatively and the unwanted signal is modulated positively (L/SECAM), the values should be increased by 2 dB;

–	when the wanted signal is modulated positively and the unwanted signal is modulated negatively, the values should be reduced by 2 dB.

## 1.1	Carriers separated by less than 1 000 Hz, non-controlled systems having the same or a different line-standard

Protection ratio:  45 dB, tropospheric interference.

## 1.2	Carriers separated by multiples of a twelfth of the line frequency up to about ± 36/12 fline (about ± 50 kHz)

These protection ratio values do not necessarily apply for greater carrier separations.

TABLE  5

Protection ratio between 625-line systems

| Offset (multiples of 1/12 line-frequency)(1) |  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-precision | Tropospheric interference | 45 | 44 | 40 | 34 | 30 | 28 | 27 | 28 | 30 | 34 | 40 | 44 | 45 |
| offset Transmitter | Continuous interference | 52 | 51 | 48 | 44 | 40 | 36 | 33 | 36 | 40 | 44 | 48 | 51 | 52 |
| stability ± 500 Hz | Limit of perceptibility(2) | 61 | 60 | 57 | 54 | 50 | 45 | 42 | 45 | 50 | 54 | 57 | 60 | 61 |
| Precision | Tropospheric interference | 32 | 34 | 30 | 26 | 22 | 22 | 24 | 22 | 22 | 26 | 30 | 34 | 38 |
| offset Transmitter | Continuous interference | 36 | 38 | 34 | 30 | 27 | 27 | 30 | 27 | 27 | 30 | 34 | 38 | 42 |
| stability ± 1 Hz | Limit of perceptibility(2) | 42 | 44 | 40 | 36 | 36 | 39 | 42 | 39 | 36 | 36 | 40 | 44 | 48 |
| (1) Value in the first column is only valid for the 0/12 case. All other values between 1/12 and 12/12 are the same by addition or subtraction of integral multiples of 12/12 up to ± 36/12. (2) Limit of perceptibility – only for information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 2.	Protection from adjacent-channel interference

The given protection ratios apply to tropospheric interference and are defined in terms of wanted and unwanted vision carrier levels. For continuous interference, the values should be increased by 10 dB.

Adjacent-channel protection ratios cannot be determined directly from the overlapping channel protection ratio curves shown in § 4 because for certain systems the values may be affected by special measures in the receiver, e.g. sound traps.

## 2.1	Protection from lower adjacent-channel interference

The worst interference on the picture signal from another signal, using the same standard, results from the sound signal in the lower adjacent channel. However, an improvement of 2-3 dB can be achieved if the frequency difference between the wanted vision and unwanted sound carriers is an odd multiple of half the line-frequency. During periods of no sound modulation, the improvement is as much as 10 dB.

When digital sound is introduced on System B, at a level of -20 dB relative to the vision carrier, interference to D-SECAM is not increased provided that the existing System B FM sound carrier is reduced from –10 to –13 dB relative to the vision carrier. However, further studies on this and other similar cases are needed.

### 2.1.1	VHF bands

The figures below relate to the cases where the separation between the wanted vision carrier frequency and the unwanted sound carrier frequency is 1.5 MHz and the ratio between the unwanted vision and unwanted sound powers is 10 dB.

A correction must be made for different vision-to-sound power ratios.

Protection ratio:	for frequency-modulated sound carrier:	–9 dB
for amplitude-modulated sound carrier:	–8 dB.

### 2.1.2	UHF bands

Table 6 gives the protection required by a signal of any system against a lower adjacent-channel signal of the same or any other standard, assuming a vision-to-sound power ratio of 10 dB for unwanted signals of every standard.

A correction must be made for different vision-to-sound power ratios.

TABLE  6

Protection ratio from lower adjacent-channel interference
(UHF bands) for 625-line systems

| Unwanted signal | Protection ratio (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Wanted signal | G | H | I | D, K | K1 | L |
| G | –9 | –9 | –9 | 1–9 | 1–9 | 1–5 |
| H | –9 | –9 | –9 | +13 | +13 | +17 |
| I | –9 | –9 | –9 | +13 | +13 | +17 |
| D, K | –9 | –9 | –9 | 1–9 | 1–9 | 1–5 |
| K1 | –9 | –9 | –9 | 1–9 | 1–9 | +17 |
| L | –9 | –9 | –0 | –12 | –12 | 1–8 |

## 2.2	Protection from upper adjacent-channel interference, VHF and UHF bands

Protection ratio:	for systems D and K:	1–6 dB
for other systems:	–12 dB.

# 3.	Protection from image-channel interference

The protection ratio will depend on the intermediate frequency and image-channel rejection of the receiver, and on the type of unwanted signal falling in the image channel. It can be determined by subtracting the image rejection figure from the required protection ratio for overlapping channels. Table 7 shows image-channel rejection values.

TABLE  7

Image-channel rejection

| Image-channel rejection (dB) | VHF | UHF |
| --- | --- | --- |
| Systems D and K/SECAM | 45 | 30 |
| System D/PAL | 45 | 40 |
| System I |  | 50 |
| All other systems |  | 40 |

Tables 8 and 9 give image-channel protection ratio values for the UHF bands. The wanted vision channel can be affected by the unwanted vision carrier, by the unwanted sound carrier, or by both.

The image-channel protection ratios in Tables 8 and 9 apply to tropospheric and continuous interference respectively, and are defined in terms of wanted and unwanted vision carrier levels assuming a vision-to-sound power ratio of 10 dB for every standard. A correction must be made for different vision-to-sound power ratios.

# 4.	Protection from overlapping channel interference

All figures and tables in this section give protection ratios to be applied when a CW signal lies within the vision channel of the wanted transmission, the wanted vision signal being negatively modulated.

Corrections to be made for positively modulated wanted vision signals and for other types of potentially interfering signals are as given in Table 10.

Figures 2 to 4 and Tables 11 to 13 give protection ratio values applicable for tropospheric and continuous interference, and for limit of perceptibility. The values shown refer to the case of a wanted negatively modulated vision signal affected by an unwanted CW signal. The corrections shown in Table 7 should be applied when considering other combinations of wanted and unwanted signals.

TABLE  8

Protection ratios (tropospheric) from image-channel interference 625-line systems (UHF bands)

|  | Unwanted signal |  | Protection ratio (dB) |  |  |  |  |  | Image | Remarks signals |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted signal |  |  | G, H | I | D (PAL) | D, K (SECAM) | K1 | L | channel | Wanted | Unwanted |
| G (IFv = 38.9 MHz) |  |  | –1 | – 4 |  | <–12 | –12 | –8 | n + 9 |  |  |
| H (IFv = 38.9 MHz) |  |  | –1 | – 4 |  | <1–9 | –9 | –5 | n + 9 | Vision | Sound |
| I (IFv = 39.5 MHz) |  |  | –13 | –10 | –10 | <–10 | –10 | –6 | n + 9 |  |  |
| D (PAL) |  |  | –8 | –25 | –20 | <–20 | –20 | –16 | n + 8 | Vision G, H: sound | Sound |
| (IFv = 38.0 MHz) |  |  | 3 | 3 | –23 | <–23 | 3 | 5 | n + 9 | Vision | Sound |
| D, K (IFv = 38.0 MHz) |  |  | 2 | –15 | –12 | <–12 | –12 | –8 | n + 8 | Vision G, H: sound | Sound |
| (SECAM) |  |  | 13 | 13 | –13 | <–13 | 13 | 15 | n + 9 | Vision | Vision |
|  | IFv = 40.2 MHz |  | 17 | 17 |  | <–17 | 17 | 19 | n + 10 | Vision | Vision |
|  |  |  | –3 | 1 |  | <–25 | 5 | 9 | n + 9 | Vision | Sound |
| K1 | IFv = 39.9 MHz |  | 14 | 14 |  | <–14 | 14 | 16 | n + 10 | Vision | Vision |
|  |  |  | 2 | 5 |  | <–28 | 8 | 12 | n + 9 | Vision | Sound |
|  | IFv = 32.7 MHz |  | 9 | 10 |  | <–28 | 8 | 12 | n – 9 | Vision | Sound |
|  |  |  | –17 | –17 |  | <–17 | –17 | –17 | n – 9 | Sound | Vision |
|  |  |  | –33 | –33 |  | <–33 | –33 | –33 | n – 9 | Sound | Vision |
| L (IFv = 32.7 MHz) |  |  | –3 | –2 |  | <1– 4 | – 4 | 0 | n – 9 | Vision | Sound |
|  |  |  | <–20 | <–20 |  | <–20 | <–20 | <–20 | n – 8 | Vision | Vision |

When the unwanted signal is a television signal, two calculations of protection ratio are necessary: one for the unwanted vision carrier and one for the unwanted TV sound carrier. The protection ratios shown for unwanted frequency-modulated sound carriers do not apply to non-precision and precision offset conditions. Nevertheless, a reduction of 2 dB relative to the non-controlled condition (curves A and A¢ ) is achieved within the luminance frequency range between 3/12 and 9/12 of the line frequency, and within the chrominance frequency range at 0/12, 1/12, 5/12, 6/12, 7/12, 11/12 and 12/12 of the line frequency.

The curves shown in Figs. 2 to 4 are examples that can be derived from the associated tables. They illustrate the full range of protection ratio possibilities from the worst case of non-controlled condition (curves A and A¢ ) to the best achievable using either non-precision offset (curves B and B¢ ) or precision offset (curves C and C¢ ). The curves A, B and C are related to the luminance frequency range, the curves A¢, B¢ and C¢ to the chrominance frequency range for the PAL and SECAM systems. For frequency differences below –1.25 MHz or above 6 MHz, the protection ratio can be derived by linear extrapolation to the channel limit.

TABLE  9

Protection ratios (continuous) from image-channel interference 625-line systems (UHF bands)

|  | Unwanted signal |  | Protection ratio (dB) |  |  |  |  |  | Image | Remarks signals |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wanted signal |  |  | G, H | I | D (PAL) | D, K (SECAM) | K1 | L | channel | Wanted | Unwanted |
| G (IFv = 38.9 MHz) |  |  | 6 | 2 |  | <1–5 | –5 | –1 | n + 9 |  |  |
| H (IFv = 38.9 MHz) |  |  | 6 | 2 |  | <1–1 | –1 | 3 | n + 9 | Vision | Sound |
| I (IFv = 39.5 MHz) |  |  | – 4 | –2 |  | <1–2 | –2 | +2 | n + 9 |  |  |
| D (PAL) |  |  | –1 | –15 | –10 | <–10 | –10 | –6 | n + 8 | Vision G, H: sound | Sound |
| (IFv = 38.0 MHz) |  |  | 11 | 11 | –11 | <–11 | 11 | 13 | n + 9 | Vision | Sound |
| D, K (IFv = 38.0 MHz) |  |  | 9 | –10 | 1–7 | <1–7 | –7 | –3 | n + 8 | Vision G, H: sound | Sound |
| (SECAM) |  |  | 21 | 21 | –21 | <–21 | 21 | 23 | n + 9 | Vision | Vision |
|  | IFv = 40.2 MHz |  | 25 | 25 |  | <–25 | 25 | 27 | n + 10 | Vision | Vision |
|  |  |  | 5 | 10 |  | <–14 | 14 | 18 | n + 9 | Vision | Sound |
| K1 | IFv = 39.9 MHz |  | 20 | 20 |  | <–20 | 20 | 22 | n + 10 | Vision | Vision |
|  |  |  | 12 | 15 |  | <–17 | 17 | 21 | n + 9 | Vision | Sound |
|  | IFv = 32.7 MHz |  | 18 | 18 |  | <–15 | 15 | 19 | n – 9 | Vision | Sound |
|  |  |  | –16 | –16 |  | <–16 | –16 | –16 | n – 9 | Sound | Vision |
|  |  |  | –28 | –28 |  | <–28 | –28 | –28 | n – 9 | Sound | Vision |
| L (IFv = 32.7 MHz) |  |  | 6 | 6 |  | <–23 | 3 | 7 | n – 9 | Vision | Sound |
|  |  |  | <–20 | <–20 |  | <–20 | <–20 | <–20 | n – 8 | Vision | Vision |

TABLE  10

Correction values for different wanted and unwanted signals

| Unwanted signal | Correction factors (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Wanted signal | CW | TV-negative | TV-positive | FM-sound | AM-sound |
| Vision signal negative modulated | –0 | –2 | –0 | –0 | + 4 |
| Vision signal positive modulated | –2 | – 4 | –2 | –2 | +2 |

FIGURE 2 (avec T11)...[D02] = 8 CM

| Offset |  |  | D¦ (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (multiples of 1/12 |  | Curve | Luminance range |  |  |  |  |  |  |  | PAL |  | SECAM |  |
| line-frequency) |  |  | –1.25(1) | –1.25(2) | –0.5 | 0.0 | 0.5 | 1.0 | 2.0 | 3.0 | 3.6-4.8 | 5.7-6.0(3) | 3.6-4.8(4) | 5.7-6.0(3) |
| 0 | NO | A, B¢ | 32 | 23 | 44 | 47 | 50 | 50 | 44 | 36 | 35 | 18 | 40 | 25 |
|  | PO | C¢ | 23 | 11 | 32 | 34 | 40 | 40 | 37 | 31 | 28 | 15 | 33 | 18 |
| 1 | NO |  | 31 | 20 | 43 | 46 | 49 | 49 | 42 | 34 | 39 | 20 | 40 | 25 |
|  | PO |  | 23 | 11 | 33 | 36 | 39 | 39 | 36 | 31 | 31 | 16 | 33 | 18 |
| 2 | NO |  | 28 | 17 | 39 | 42 | 45 | 45 | 39 | 32 | 42 | 22 | 40 | 25 |
|  | PO |  | 21 | 19 | 29 | 32 | 35 | 35 | 33 | 29 | 34 | 17 | 33 | 18 |
| 3 | NO | A¢ | 25 | 13 | 34 | 36 | 39 | 39 | 35 | 29 | 45 | 25 | 40 | 25 |
|  | PO | B¢ | 19 | 17 | 25 | 28 | 31 | 31 | 29 | 26 | 35 | 18 | 33 | 18 |
| 4 | NO |  | 22 | 10 | 30 | 32 | 35 | 35 | 32 | 27 | 42 | 22 | 40 | 25 |
|  | PO | C | 17 | 15 | 22 | 24 | 26 | 26 | 25 | 24 | 34 | 17 | 33 | 18 |
| 5 | NO |  | 20 | 18 | 28 | 30 | 32 | 32 | 30 | 25 | 39 | 20 | 40 | 25 |
|  | PO | C | 17 | 15 | 22 | 24 | 26 | 26 | 25 | 24 | 31 | 16 | 33 | 18 |
| 6 | NO | B, B¢ | 19 | 17 | 27 | 29 | 31 | 31 | 29 | 24 | 35 | 18 | 40 | 25 |
|  | PO | C¢ | 17 | 15 | 24 | 26 | 28 | 28 | 26 | 24 | 28 | 15 | 33 | 18 |
| 7 | NO | B¢ | 20 | 18 | 28 | 30 | 32 | 32 | 30 | 25 | 35 | 18 | 40 | 25 |
|  | PO | C, C¢ | 17 | 15 | 22 | 24 | 26 | 26 | 25 | 24 | 28 | 15 | 33 | 18 |
| 8 | NO |  | 22 | 10 | 30 | 32 | 35 | 35 | 32 | 27 | 39 | 20 | 40 | 25 |
|  | PO | C | 17 | 15 | 22 | 24 | 26 | 26 | 25 | 24 | 31 | 16 | 33 | 18 |
| 9 | NO |  | 25 | 13 | 34 | 36 | 39 | 39 | 35 | 29 | 42 | 22 | 40 | 25 |
|  | PO |  | 19 | 17 | 25 | 28 | 31 | 31 | 29 | 26 | 34 | 17 | 33 | 18 |
| 10 | NO |  | 28 | 17 | 39 | 42 | 45 | 45 | 39 | 32 | 39 | 20 | 40 | 25 |
|  | PO |  | 21 | 19 | 29 | 32 | 35 | 35 | 33 | 29 | 31 | 16 | 33 | 18 |
| 11 | NO | B¢ | 31 | 20 | 43 | 46 | 49 | 49 | 42 | 34 | 35 | 18 | 40 | 25 |
|  | PO | C¢ | 23 | 11 | 33 | 36 | 39 | 39 | 36 | 31 | 28 | 15 | 33 | 18 |
| 12 | NO | A, B¢ | 32 | 23 | 44 | 47 | 50 | 50 | 44 | 36 | 35 | 18 | 40 | 25 |
|  | PO | C¢ | 23 | 11 | 32 | 40 | 40 | 40 | 37 | 31 | 28 | 15 | 33 | 18 |
| Protection ratio (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) H, I, K1, L television systems. NO: non-precision offset (2) B, D, G, K television systems. PO: precision offset (3) B, G television systems: range is 5.3-6.0 MHz. (4) D/SECAM and K/SECAM: add 5 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FIGURE 3 (avec T12)...[D03] = 8 CM

| Offset |  |  | D¦ (MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (multiples of 1/12 |  | Curve | Luminance range |  |  |  |  |  |  |  | PAL |  | SECAM |  |
| line-frequency) |  |  | –1.25(1) | –1.25(2) | –0.5 | 0.0 | 0.5 | 1.0 | 2.0 | 3.0 | 3.6-4.8 | 5.7-6.0(3) | 3.6-4.8(4) | 5.7-6.0(3) |
| 0 | NO | A, B¢ | 40 | 32 | 50 | 54 | 58 | 58 | 54 | 44 | 45 | 30 | 45 | 30 |
|  | PO | C¢ | 30 | 22 | 37 | 38 | 44 | 44 | 42 | 36 | 34 | 21 | 37 | 21 |
| 1 | NO |  | 38 | 30 | 49 | 53 | 57 | 57 | 53 | 43 | 48 | 32 | 45 | 30 |
|  | PO |  | 29 | 22 | 38 | 40 | 42 | 42 | 41 | 36 | 36 | 22 | 37 | 21 |
| 2 | NO |  | 34 | 27 | 46 | 50 | 55 | 55 | 51 | 41 | 51 | 33 | 45 | 30 |
|  | PO |  | 27 | 20 | 34 | 36 | 38 | 38 | 37 | 34 | 39 | 24 | 37 | 21 |
| 3 | NO | A¢ | 30 | 23 | 42 | 46 | 50 | 50 | 46 | 38 | 53 | 35 | 45 | 30 |
|  | PO |  | 24 | 17 | 30 | 32 | 34 | 34 | 33 | 31 | 40 | 26 | 37 | 21 |
| 4 | NO |  | 28 | 21 | 38 | 42 | 45 | 45 | 42 | 35 | 51 | 33 | 45 | 30 |
|  | PO | C | 22 | 15 | 27 | 29 | 31 | 31 | 31 | 30 | 39 | 24 | 37 | 21 |
| 5 | NO |  | 26 | 19 | 35 | 38 | 41 | 41 | 38 | 32 | 48 | 32 | 45 | 30 |
|  | PO | C | 22 | 15 | 27 | 29 | 31 | 31 | 31 | 30 | 36 | 22 | 37 | 21 |
| 6 | NO | B, B¢ | 24 | 17 | 33 | 35 | 37 | 37 | 36 | 30 | 45 | 30 | 45 | 30 |
|  | PO | C¢ | 23 | 16 | 29 | 32 | 33 | 33 | 32 | 30 | 34 | 21 | 37 | 21 |
| 7 | NO | B¢ | 26 | 19 | 35 | 38 | 41 | 41 | 38 | 32 | 45 | 30 | 45 | 30 |
|  | PO | C, C¢ | 22 | 15 | 27 | 29 | 31 | 31 | 31 | 30 | 34 | 21 | 37 | 21 |
| 8 | NO |  | 28 | 21 | 38 | 42 | 45 | 45 | 42 | 35 | 48 | 32 | 45 | 30 |
|  | PO | C | 22 | 15 | 27 | 29 | 31 | 31 | 31 | 30 | 36 | 22 | 37 | 21 |
| 9 | NO |  | 30 | 23 | 42 | 46 | 50 | 50 | 46 | 38 | 51 | 33 | 45 | 30 |
|  | PO |  | 24 | 17 | 30 | 32 | 34 | 34 | 33 | 31 | 39 | 24 | 37 | 21 |
| 10 | NO |  | 34 | 27 | 46 | 50 | 55 | 55 | 51 | 41 | 48 | 32 | 45 | 30 |
|  | PO |  | 27 | 20 | 34 | 36 | 38 | 38 | 37 | 34 | 36 | 22 | 37 | 21 |
| 11 | NO | B¢ | 38 | 30 | 49 | 53 | 57 | 57 | 53 | 43 | 45 | 30 | 45 | 30 |
|  | PO | C¢ | 29 | 22 | 38 | 40 | 42 | 42 | 41 | 36 | 34 | 21 | 37 | 21 |
| 12 | NO | A, B¢ | 40 | 32 | 50 | 54 | 58 | 58 | 54 | 44 | 45 | 30 | 45 | 30 |
|  | PO | C¢ | 30 | 22 | 37 | 44 | 44 | 44 | 42 | 36 | 34 | 21 | 37 | 21 |
| Protection ratio (dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) H, I, K1, L television systems. NO: non-precision offset (2) B, D, G, K television systems. PO: precision offset (3) B, G television systems: range is 5.3-6.0 MHz. (4) D/SECAM and K/SECAM: add 8 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

FIGURE 4 (avec T13)...[D04] = 10 CM

| D¦ (MHz) |  | –1.25 | –1.0 | –0.5 | 0.0 | 0.5 | 1.0 | 2.0 | 3.0 | 3.6 | 4.8 | 5.7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | PAL ––––––––––––– SECAM | 44 | 53 | 59 | 63 | 67 | 67 | 62 | 53 | 62 ––––––––– 54 | 62 ––––––––– 54 | 42 ––––––––– 37 |
| B | PAL ––––––––––––– SECAM | 30 | 39 | 42 | 44 | 46 | 46 | 45 | 40 | 54 | 54 | 37 |
| C | PAL ––––––––––––– SECAM | 26 | 33 | 36 | 38 | 40 | 40 | 39 | 34 | 41 ––––––––– 44 | 41 ––––––––– 44 | 27 |
| Limit of perceptibility (dB) |  |  |  |  |  |  |  |  |  |  |  |  |

# 5.	Television signal affected by data signals

The inclusion of digital data such as teletext in the field-blanking interval has no effect on the required protection ratios. However, the full improvement resulting from non-precision or precision offset operation is not achievable when the unwanted signal carries a full-field data signal. In this case, Fig. 5 shows the minimum values for all offset and non-offset conditions given in § 4. The curves in Fig. 5 apply to full-field data signals with pulse amplitude at 66% of the peak white-to-blanking level. The values should be increased linearly for higher modulation levels.

FIGURE 5 (avec T14)...[D05] = 9 CM

| D¦ (MHz) | –1.25 | 0.0 | 0.5 | 1.0 | 2.0 | 3.0 | 3.6 | 4.8 | 5.25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tropospheric interference, T | 17 | 27 | 28 | 28 | 27 | 22 | 29 | 29 | 20 |
| Continuous interference, C | 23 | 33 | 36 | 36 | 35 | 29 | 36 | 36 | 27 |
| Protection ratio (dB) |  |  |  |  |  |  |  |  |  |

APPENDIX  3

TO  ANNEX  1

Protection ratios for sound signals

Protection ratios applicable to the wanted sound signal are given in Table 15 for both tropospheric and continuous interference. The values quoted refer to the level of the wanted sound carrier. In the case of a two-sound carrier transmission, each sound carrier must be separately considered. Multiplex modulated sound signals may require higher protection.

For any unwanted vision carrier, subtract 2 dB; for an unwanted amplitude-modulated sound carrier, add 4 dB.

The maximum deviation of the wanted FM sound carrier is assumed to be ± 50 kHz. A linear correction should be made for other deviations.

Tables 15 and 16 give protection ratio values where the television sound is affected by a single unwanted signal within the range ± 250 kHz of the wanted sound carrier. In a co-channel situation the unwanted sound directly affects the wanted sound. In addition, the unwanted vision carrier produces phase modulation of the wanted vision carrier resulting in some sound distortion in receivers using inter-carrier demodulation techniques.

Improvement of the sound protection ratio can be reached by increasing the frequency offset by a suitable multiple of 1, 2 or 3 lines frequency (see also Note 1 in Table 15).

The weighted signal-to-noise ratio will be improved by approximately 8 dB if, for example, 5/3 line-frequency offset is used instead of 2/3 line-frequency offset.

TABLE  15

Protection ratio for wanted sound carriers
Unwanted signal: CW or FM sound carrier

| Absolute value of | Protection ratio (dB) – Wanted sound signal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| the difference between unwanted carrier and | Tropospheric interference |  |  | Continuous interference |  |  |
| wanted sound carrier (kHz) | FM | AM | Digital | FM | AM | Digital |
| 120 | 32 | 49 | 10 | 39 | 56 | 11 |
| 215 | 30 | 40 | 10 | 35 | 50 | 11 |
| 250 | 22 | 10 | 10 | 24 | 15 | 11 |
| 250 | –6 | –7 | 15 | –6 | 12 | 16 |
| Note 1 – In many cases, particularly with precision offsets, the required sound protection ratio can be higher than the ratio required between the vision signals. In such instances increasing the frequency offset by a suitable multiple of one, two or three lines frequency will decrease the required sound protection ratio by more than 10 dB, the vision protection ratio remaining unchanged. Note 2 – It is desirable to obtain protection ratio values for frequency differences of 10 and 25 kHz, corresponding to offsets of 8/12 and 20/12 of line frequency. |  |  |  |  |  |  |

TABLE  16

Sound carrier protection ratio (dB)
(No difference between wanted and unwanted sound carriers)

| Wanted | Unwanted | FM/CW | AM | Digital |
| --- | --- | --- | --- | --- |
| FM | T | 32 | 36(1) | 17 |
|  | C | 39 | 43(1) | 27 |
| AM | T | 49 | 53(1) | 37 |
|  | C | 56 | 60(1) | 44 |
| Digital | T | 10 | 12(1) | 12 |
|  | C | 11 | 13(1) | 13 |
| - (1) Values are 4 dB higher than those in the first column. Note 1 – In the case of interference from an I/PAL signal with digital sound to an L-SECAM signal, the full benefit of precision offset may not be obtained because of interference to the AM sound signal. |  |  |  |  |

The reference sound quality is grade 3 for tropospheric interference and grade 4 for continuous interference.

Reference signal-to-noise ratios (S/N) for sound analogue signals, where S/N is peak-to-peak weighted (Recommendation ITU-R BS.468):

–	40 dB (approximates to impairment grade 3 (T )),

–	48 dB (approximates to impairment grade 4 (C)).

Reference bit error ratios for digital sound signals:

–	10–4 (approximates to impairment grade 3 (T )),

–	10–5 (approximates to impairment grade 4 (C)).

ANNEX  2

Tropospheric and continuous interference

When using the protection ratios in planning, it is necessary to determine whether, in particular circumstances, the interference should be considered as tropospheric or continuous. This can be done by comparing the nuisance fields for the two conditions, the nuisance field being defined as the field strength of the interfering transmitter (at its pertinent e.r.p.) enlarged by the relevant protection ratio.

Thus, the nuisance field for continuous interference:

EC  =  E(50, 50)  +  P  +  AC

and the nuisance field for tropospheric interference:

ET  =  E(50, t)  +  P  +  AT

where:

E(50, t):	field strength (dB(mV/m)) of the interfering transmitter, normalized to 1 kW, and exceeded during t % of the time

P:	e.r.p. (dB(1 kW)) of the interfering transmitter

A:	protection ratio (dB)

C and T:	continuous and tropospheric interference, respectively.

The protection ratio for continuous interference is applicable when the resulting nuisance field is stronger than that resulting from tropospheric interference, that is, when EC  >  ET.

This means that AC should be used in all cases when:

E(50, 50)  +  AC  >  E(50, t)  +  AT

ANNEX  3

Different offset conditions

The required protection ratio varies considerably depending on the frequency relationship between the wanted and the unwanted carriers and their frequency tolerance. The greatest protection is required when the frequency of one or both carriers is “non-controlled”.

Less interference is possible and therefore lower protection ratios are required for non-precision offset (line-frequency offset). Non-precision offset takes advantage of the line frequency structure of the video signal and, in particular, it is advantageous to offset the carriers by multiples of one-half or one-third of the line frequency. The long-term stability of these favourable protection ratios can only be guaranteed, however, if the frequencies of the wanted and unwanted signals are kept constant within ± 500 Hz.

Precision offset takes further advantage of the field frequency structure of the video spectrum. The least protection is required when both carriers are “precision offset” controlled within a tolerance of ± 1 Hz for the wanted and unwanted carriers.

Figure 6 shows the main characteristics of offset operation and plots in schematic form the protection ratio curves between 0/12 ¦line and 12/12 ¦line. These curves are cyclic and their extensions to the left and right are symbolized by broken lines. These various conditions illustrated are similar within the luminance range up to about ± 3 MHz.

FIGURE 6 ...[D06] = 10 CM

The upper and lower curves indicate, respectively, the protection ratios obtained with non-precision and precision offset. More precisely, these two curves trace the envelope of a series of fluctuations in the protection ratio which swings between the two curves at field frequency as represented by the thin line.

Co-channel protection ratio curves in the vicinity of 0/12, 4/12 and 6/12 line frequency (625-line systems)

Figure 7 gives examples of protection ratio curves for the three most important offset positions (0/12, 4/12 and 6/12 ¦line). The curves in each graph relate to tropospheric interference, continuous interference and the limit of perceptibility.

FIGURE 7...[D07] = 13 CM

The white and black points indicate the positions for non-precision and precision offset respectively. The reference impairment points for tropospheric and continuous interference are also indicated in the figure.

When operating TV transmitter networks with synchronized as well as phase-locked carriers, the protection ratio values are slightly reduced.

ANNEX  4

Frequencies for precision offset

Table 17 lists the possible frequencies for precision offset in the vicinity of each twelfth of line frequency. For the luminance frequency range, the frequencies shown in the table end with 25 Hz up to 6/12 ¦line and with 100 Hz beyond this frequency. Two possibilities are shown for 6/12 ¦line (7 800 and 7 825 Hz) because at this point the spectral lines are symmetrical and thus of the same amplitude. The offset frequencies are expressed in twelfths of line frequency.

Alternative frequencies in the vicinity of each offset position, which differ by integral multiples of 50 Hz and by integral multiples of 15 625 Hz from the values given, are possible. The term “precision offset” always refers to a difference between the true frequencies of the wanted and unwanted transmitters, and not to an offset of a transmitter from its nominal carrier frequency.

If the frequency difference between wanted and unwanted carrier exceeds the normalized range specified in Table 17, one has to subtract integral multiples of 15 625 Hz. For computer calculations, formulae are given below for all precision-offset frequency differences in the luminance and in the chrominance range for 625-line systems.

TABLE  17

Normalized precision offset between 0/12 and 12/12 of line frequency

Luminance range for all 625-line systems
Chrominance range for only PAL and SECAM systems

|  | Precision offset frequency (Hz) |  |  |  |
| --- | --- | --- | --- | --- |
| Offset (multiples |  | Chrominance range |  | Luminance range: |
| of 1/12 line-frequency) | Luminance range | PAL | SECAM | ¦p = 15 625 m ± 25 (2 n + 1) m £ 192, n £ 156 with m and n integers |
| 00 01 02 03 04 05 06 07 08 09 10 11 12 | 10 1 25 11 325 12 625 13 925 15 225 16 525 7 800 or 7 825 19 100 10 400 11 700 13 000 14 300 15 600 | 11 11 5 1 1 305 12 605 1 3 905 1 5 205 1 6 505 1 7 810 1 9 115 10 420 11 720 13 020 14 320 15 630 | 10 110 11 302 12 604 13 906 15 208 16 510 17 812 19 115 10 417 11 719 13 021 14 323 15 625 | Chrominance range: - – PAL systems – ¦p = 15 625 m ± 25 (2 n + 1) + k – m ³ 216 and – k = –20 for 000 £ n < 143 – k = –15 for 143 £ n < 169 – k = 0–5 for 169 £ n < 299 – k = 0+5 for 299 £ n £ 312 – SECAM systems – ¦p = 15 625 m + 2n – with m, n and k integers |

Computation of operational offset frequencies in a network with transmitter triplets

Precision offset techniques are usually introduced to provide solutions of particular interference problems between two co-channel transmitters. In operational television networks, co-channel transmitters are situated at the corner of a triangle. A typical line offset (non-precision offset) situation for such a transmitter triplet is: nominal vision carrier frequency –2/3 ¦line, ± 0 ¦line, and +2/3 ¦line or in twelfths: 8M, 0, 8P (M = minus; P = plus). A transmitter triplet A-B-C consists of three transmitter pairs A-B, A-C and B-C. Introduction of precision offset for the above-mentioned example means a possible reduction of interference for all three pairs of the transmitter triplet. In practice, only 35% of all the theoretically possible transmitter triplets have full improvement for all three pairs, the residual 65% triplets have one or two pairs in non-precision offset.

Table 18 shows a complete and normalized list of these 35% possible cases within the range between 0P and 12P which secure an improved interference situation for all three transmitter pairs within a triplet, when precision offset is used.

With a simple rule, determination of precision offset frequencies for transmitter triplets is possible. All transmitter triplets which cannot translate to the normalized cases of Table 18 contain at least one pair without precision offset.

Example

The aim of this calculation is the transformation of all three offset positions into the range between 0P and 12P (see Table 18). Each single transmitter can be moved by multiples of line frequency, that means by multiples of 12/12 (see Step 2). Moving of any twelfths is allowed, when all transmitters are moved by the same number of twelfths (see Step 1).

| Given: Transmitter triplet Given: Line offset position: | A +18M | B +18PS | 1C +12P |
| --- | --- | --- | --- |
| Step 1 Set one transmitter to 0 by linear translation: Result: | +18M +10M | +18SP +26PS | +18P +20P |
| Step 2 Translation of transmitter B and C into the range between 0P and 12P by subtracting or adding multiple of the line frequency: Result: | +10M | –24PS +12PS | –12P +18P |
| Step 3 Selection of precision offset frequencies from Table 18: | +10M | +32 625 | 10 400 Hz |
| Step 4 Step 2 has to be compensated: Result: | +10M | +31 250 +33 875 | +15 625 Hz +26 025 Hz |
| Step 5 Step 1 has to be compensated: Result: equivalent to | –23 400 –23 400 –23 418M | –23 400 +10 475 +10 8P* | –23 400 Hz –22 625 Hz +12P |

TABLE  18

Possible offset combinations allowing precision offset
for all transmitter pairs in transmitter triplets

| Case | Offset | Frequency (Hz) (625-line systems) |
| --- | --- | --- |
| 01 | 0 – 0P – 06P | 0 13 25 07 800 |
| 02 | 0 – 0P – 06P | 0 13 25 07 825 |
| 03 | 0 – 1P – 06P | 0 1 325 07 800 |
| 04 | 0 – 1P – 07P | 0 1 325 09 100 |
| 05 | 0 – 2P – 06P | 0 2 625 07 800 |
| 06 | 0 – 2P – 07P | 0 2 625 09 100 |
| 07 | 0 – 2P – 08P | 0 2 625 10 400 |
| 08 | 0 – 3P – 06P | 0 3 925 07 800 |
| 09 | 0 – 3P – 07P | 0 3 925 09 100 |
| 10 | 0 – 3P – 08P | 0 3 925 10 400 |
| 11 | 0 – 3P – 09P | 0 3 925 11 700 |
| 12 | 0 – 4P – 06P | 0 5 225 07 800 |
| 13 | 0 – 4P – 07P | 0 5 225 09 100 |
| 14 | 0 – 4P – 08P | 0 5 225 10 400 |
| 15 | 0 – 4P – 09P | 0 5 225 11 700 |
| 16 | 0 – 4P – 10P | 0 5 225 13 000 |
| 17 | 0 – 5P – 06P | 0 6 525 07 800 |
| 18 | 0 – 5P – 07P | 0 6 525 09 100 |
| 19 | 0 – 5P – 08P | 0 6 525 10 400 |
| 20 | 0 – 5P – 09P | 0 6 525 11 700 |
| 21 | 0 – 5P – 10P | 0 6 525 13 000 |
| 22 | 0 – 5P – 11P | 0 6 525 14 300 |
| 23 | 0 – 6P – 06P | 0 7 800 07 825 |
| 24 | 0 – 6P – 07P | 0 7 825 09 100 |
| 25 | 0 – 6P – 08P | 0 7 825 10 400 |
| 26 | 0 – 6P – 09P | 0 7 825 11 700 |
| 27 | 0 – 6P – 10P | 0 7 825 13 000 |
| 28 | 0 – 6P – 11P | 0 7 825 14 300 |
| 29 | 0 – 6P – 12P | 0 7 800 15 600 |
| 30 | 0 – 6P – 12P | 0 7 825 15 600 |

_________________

*	To reduce the sound interference between transmitters B and C, an offset position of 20P = 26 100 Hz (enlarged by 12P = 15 625 Hz) would be preferable. In this case picture interference is unchanged.