RECOMMENDATION  ITU-R  F.1105-2*

Fixed wireless systems for disaster mitigation
and relief operations

(Question ITU-R 239/9)

(1994-2002-2006)

Scope

This Recommendation provides characteristics of fixed wireless systems used for disaster mitigation and relief operations. Several types of such systems including transportable equipment are specified according to channel capacity, operating frequency bands, transmission distance and propagation path conditions.

Detailed descriptions of these systems are also given in Annex 1 as guidance.

The ITU Radiocommunication Assembly,

considering

a)	that rapidly deployable telecommunications are essential for disaster mitigation and relief operations in the event of natural disasters, epidemics, famines and similar emergencies;

b)	that measures to mitigate the effects of natural disasters should be made as much as possible;

c)	that high speed data and high capacity information are available due to the popularity of fibre-to-the-home, digital subscriber line, mobile phones etc. in the form of voice, character data, image, or through a variety of Internet Protocol (IP)-based services;

d)	that transportable fixed wireless equipment may be used for relief operation of either radio or cable links and may involve multi-hop applications with digital and analogue equipment;

e)	that fixed wireless equipment for disaster mitigation and relief operations may be operated in locations with differing terrain and in differing climatic zones, uncontrolled environmental conditions and/or unstable power sources;

f)	that fixed wireless equipment for disaster mitigation and relief operations may be used in areas with an unfavourable interference environment;

g)	that interoperability and internetworking between fixed wireless system for disaster mitigation and relief operations and other networks would be beneficial in emergency situations as stated in considering a);

h)	that efficient use of spectrum is required,

recognizing

a)	that the World Radiocommunication Conference (WRC-03) invites ITU-R to continue its technical studies and to make recommendations concerning technical and operational implementation, as necessary, for advanced solutions to meet the needs of public protection and disaster relief radiocommunication applications, taking into account the capabilities, evolution and any resulting transition requirements of the existing systems, particularly those of many developing countries, for national and international operations (see the relevant part of Resolution 646 (WRC-03)),

recommends

1	that for disaster mitigation and relief operations in devastated areas or restoration of a break in transmission links the following  types of fixed wireless systems as given in Table 1 should be considered;

TABLE 1

Types of fixed wireless systems for disaster mitigation
and relief operations

| Type | Feature | Application |
| --- | --- | --- |
| A | A simple wireless link which can be established rapidly for telephone communication with a governmental or international headquarters | (1) (2) |
| B | One or more local networks which connect a communications centre and up to about 10 or 20 end-user stations with telephone links | (1) |
| C | A telephone link for between about 6 and 24 channels or a data link up to the primary rate over a line-of-sight or near line-of-sight path | (1) (2) |
| D | A link over an obstructed or trans-horizon path | (2) |
| E | A high-capacity telephone link (more than 24 channels) or digital fixed wireless link (above the primary rate) | (2) |
| F | Simultaneous individual or group radiocommunications using point-to-multipoint individual radiocommunications between a central station and a number of terminals in a region | (1), (3) |
| Types A to E: transportable system Application (1): for devastated areas Application (2): for breaks in transmission links Application (3): for mitigation of disaster effects |  |  |

2	that frequency bands used for fixed wireless systems for disaster mitigation and relief operations described in Table 1 should be in accordance with the Radio Regulations for the fixed service, as well as with national and regional frequency allocations (see Table 2);

3	that radio-frequency arrangements for fixed wireless systems for disaster mitigation and relief operations in the chosen bands should be in accordance with ITU-R Recommendations (see Recommendation ITU-R F.746) and national standards;

4	that interconnection of transportable fixed wireless systems with analogue and digital cable systems at repeater stations should be made at baseband;

5	that interconnection of transportable fixed wireless systems with fibre-optic systems at repeater stations may be made at points with a significant level of optical power;

6	that for system characteristics, the information contained in § 1 of Annex 1 can be referred to as a guide for administrations and system planners;

7	that performance objectives of links which use transportable fixed wireless equipment as well as separate links formed by the transportable fixed wireless equipment during restoration should have transmission performance values sufficient for normal service (see § 3 of Annex 1);

8	that transportable fixed wireless systems, Types A to E in Table 1 including Annex 1 describing their characteristics, should be used for the access link to a base station in mobile communications that are operating in disaster relief and emergency situations.

Annex 1

Descriptions of fixed wireless systems for disaster mitigation
and relief operations

# 1	System characteristics

For each type of system in Table 1, the channel capacities, frequency bands and path distances given in Table 2 are suitable.

TABLE 2

Basic characteristics

| System type | Capacity | Example frequency bands(1) | Transmission path distance |
| --- | --- | --- | --- |
| A | 1-2 channels | HF (2-10 MHz) | Up to 250 km and beyond |
| B | Local network with 10-20 outstations (several channels) | VHF (50-88 MHz) (150-174 MHz) UHF (335-470 MHz) | Up to a few km |
| C | From 6 to 120 channels 1.5/2 or 6.3/8 Mbit/s | UHF (335-470 MHz) (1.4-1.6 GHz) SHF (7-8 GHz) (10.5-10.68 GHz) | Up to 100 km |
| D | From 12 to 480 channels 1.5/2, 6.3/8, 4 x 6.3/8 Mbit/s or 34/45 Mbit/s | UHF (800-1 000 MHz) (1.7-2.7 GHz) SHF (4.2-5 GHz) | Line-of-sight or obstructed paths |
| E | 960-2 700 FDM channels STM-0 (52 Mbit/s) or STM-1 (155 Mbit/s) | SHF (4.4-5 GHz) (7.1-8.5 GHz) (10.5-10.68 GHz) (11.7-13.2 GHz) (23 GHz) | Up to several tens of km |
| F | 6-TDMA channels e.g. up to 2 000 individual calls e.g. up to 200 group calls | VHF (54-70 MHz) | Up to 10 km (typical) Extension with repeater(s) |
| FDM: frequency division multiplexing TDMA: time division multiple access STM: synchronous transfer mode (1) Many parts of these bands are shared with satellite services. |  |  |  |

In the case of links to an earth station operating in a satellite service, the following additional restrictions should be considered:

–	space-to-Earth frequency bands should be avoided;

–	problems could arise if Earth-to-space frequency bands are used;

–	trans-horizon systems (Type D) should be avoided.

It would be preferable to avoid bands likely to be in use or planned for trunk communications. However, these bands may be used for Type E with careful consideration of interference problems by the administration.

# 2	Engineering principles

## 2.1	Low-capacity links (Type A system)

HF transportable equipment for 1 or 2 channels should employ only solid-state components and should be designed to switch off the transmitters when not in use, in order to conserve battery power, and to reduce the potential of interference.

As an example, a solid-state 100 W single-sideband terminal in a band between, say, 2 and 8 MHz operated with a whip antenna, could have a range of up to 250 km. Simplex operation (transmitter and receiver employing the same frequency) with a frequency synthesizer to ensure a wide and rapid choice of frequency when interference occurs and to facilitate setting-up in an emergency, can give up to 24 h operation from a relatively small battery (assuming that use of the transmitter is not excessive). The battery can be charged from a vehicle generator and all units can be hand-carried over rough country.

## 2.2	Local radio networks (Type B system)

Radio networks of Type B are envisaged as local centres with single-channel radiocommunication with 10 to 20 out-stations, operating on VHF or UHF up to about 470 MHz. Single-channel and multi-channel equipments similar to types used in the land mobile service could be used.

## 2.3	Links up to 120 channels (Type C system)

Equipment which is suitable for transportation by road, railway or helicopter is available. Such equipment, together with power supply equipment, can be easily and quickly installed and put into service. The equipment capacity is from about 1.5/2 to 6.3/8 Mbit/s, depending on the requirements, the topography and other factors.

d.c. operated equipment or a.c. powered equipment automatically switchable to d.c. is preferred. It can be associated with light-weight, high gain Yagi or grid antennas, giving a range of up to 100 km line-of-sight, but capable of accepting some obstruction from trees on shorter paths. Simply erected guyed or telescopic poles which can be rotated from ground level are to be preferred. If separate antennas are used for transmitting and receiving with cross-polarization, it is convenient for the transmitters to be connected to antennas which are polarized at 45 (from top right to bottom left looking along the path from behind the antenna); if transmit and receive antennas are mounted on the same sub-assembly, with male and female connectors, there can then be no confusion over the plane of polarization to be selected, since the received signal will always be cross-polarized with respect to the transmitted one.

Single frequency, or selectable pre-set frequencies are to be preferred to eliminate as many variables as possible during the initial setting-up of the equipment. The ability to appropriately select transmit and receive frequencies in the field over a wide frequency band is an advantage. Foam-filled or solid dielectric flexible cable is to be preferred as this is less liable to mechanical damage and the effects of moisture.

## 2.4	Links up to 480 channels (Type D system)

Equipment which is suitable for transportation by road or railway or by helicopter is available. Such equipment, together with power supply equipment, can be easily and quickly installed and put into service. The equipment capacity is from about 12 to 480 telephone channels, depending on the requirements, the topography and other factors. The use of receivers with low noise factors and with special demodulators and of diversity reception, enables the size of the antennas, the transmitter power and the size of the power supply equipment, to be smaller than those often used for conventional trans-horizon installations.

In line-of-sight or partially obstructed path conditions, transportable equipment with similar fast deployment capability but with transmission capacities of up to 34/45 Mbit/s is available. d.c. operated equipment or a.c. powered equipment automatically switchable to d.c. is preferred. It can be associated with light-weight grid antennas, giving a range of line-of-sight, but capable of accepting some obstruction from trees on shorter paths. Simply erected guyed or telescopic poles which can be rotated from ground level are to be preferred.

The ability to appropriately select transmit and receive frequencies in the field over a wide frequency band is an advantage.

## 2.5	High capacity links (Type E system)

For higher frequency bands and capacities of  960 telephone channels and above, it is recommended that the radio-frequency equipment is integrated directly to the antennas. For transportable equipment, preference should be given to equipment in which reflectors of diameter less than about 2 m are available. Because IF interconnection at repeaters is a desirable feature, an IF interconnection should be possible between the radio-frequency heads.

However, since the equipment which is to be bypassed in an emergency or for temporary use will most likely be at ground level, the control cable should bring the IF to the control unit at ground level. The antennas of systems used for relief operations are likely to be smaller than those of fixed microwave links and it is therefore important that the output power of the transmitters should be as high as possible and the noise factor of receivers should be as low as possible. Battery operated equipment is preferable: 12 V and/or 24 V supplies are appropriate if the batteries are to be rechargeable from the dynamos or alternators of any vehicles which are available.

An alternative arrangement would be to house the equipment in a number of containers. These would not only facilitate the transport of the equipment but each container could provide facilities for rapidly installing a number of transmitters and receivers. The maximum number of transceivers to be housed in any one container would depend on the dimensions and maximum weight adopted, allowing for transport by helicopter, aeroplane or any other means of transport. Furthermore, it is preferable to take into consideration equipment operating with ordinary commercial power supplies. Fixed wireless systems generally require line-of-sight operation. For digital fixed wireless systems, the interface should be based on the primary rate (2 Mbit/s (E1) or 1.5 Mbit/s (T1)).

## 2.6	Regional simultaneous communication system (Type F system)

This type operates as a point-to-multipoint system in ordinary time, and, in case of emergency, works in particular for disaster relief communications.

A central station (CS) in a local/municipal office usually provides public information to outdoor terminal stations (OS) or indoor receivers for daily communications between the office and residents. The CS also collects data or information for potential prevention of disasters from OS through the monitoring camera, telemeter, etc., or from disaster prevention systems used in other districts. The above information may include meteorological data or notice of storms and fires. These usual communications are performed in TDMA-TDD.

For OSs far from the CS, a repeater station (or more than one station in series) can be deployed. Repeater stations may work as an OS with a function of interactive communication.

In the case that a disaster occurs or is likely to occur, the CS transmits necessary information or warnings for storm, earthquake or tsunami to the residents by means of loud speakers or character displays that are equipped with the OS and indoor receiver. This downlink information is transmitted in simultaneous distribution mode.

Interactive communications between the CS and an individual OS is possible even when simultaneous distribution is under way, using other time slots in TDMA-TDD. Thus, important information from the damaged area can be efficiently transmitted to the CS, including relief operation status, urgently required resources or residents’ safety information.

For further information, see Appendix 1.

# 3	Transmission  performance

Systems of Type A will have a noise performance which is critically dependent upon the antennas and path length in a particular case.

Systems of Types B and C are likely to provide similar transmission quality, when in use for relief work, as in normal use. A minimum sustainable  1  10–8 BER objective can be used as guidance for digital systems.

Systems of Type D would, as with Type A, be very dependent upon the siting of the terminals and the size of antennas. A minimum sustainable  1  10–8 BER objective can be used as guidance for digital systems.

Transportable microwave equipment of Type E, because of the need to use smaller antennas and lower transmitter powers than for fixed links, would be likely to have a transmission quality below that normally required for trunk connections. Nonetheless this performance should be such that the network can still carry out all normal functions. Guidance for the performance in such emergency conditions is given as follows:

–	 1 000 pW for up to 50 km for 960 channels (4-12 GHz);

–	 5 000 pW for up to 50 km for more than 1 800 channels (4-6 GHz);

–	 5 000 pW for up to 25 km for 2 700 channels (11 GHz);

–	 1  10–8 BER for digital systems.

System of Type F requires:

-  1  10–3 BER for indoor receiver terminals.

–	 1  10–4 BER for outdoor terminals with loud speakers.

Appendix 1
to Annex 1

Features and applications of Regional Digital Simultaneous Communication
System for disaster prevention and relief operations

Regional Digital Simultaneous Communication System (RDSCS) based on ARIB STD-T86* has been developed for disaster prevention and relief operations, that is, for collection of data or information for prevention of disaster or damage by disaster, and for transmission of necessary information or alarm to the residents, besides for voice or data communications between the central office and residents.

Locating a central station in the local office and a number of terminals in the region, the system provides simultaneous or group communications besides point to multipoint individual communications between the central station and the terminals.

The central station collects data or information for prevention of or damage by disasters; from monitoring cameras, telemeters, human, etc. through the outdoor terminals using TDMA, or from other disaster prevention system through telephone or facsimile. Then the central station transmits necessary information or alarm to the residents through the outdoor terminals and the indoor receivers by means of loud speakers or character displays with simultaneous distributing mode.

Each outdoor terminal is capable of interactive communications with the central station by TDD mode. 6-time slots TDMA can provide individual communication even during the time where simultaneous distribution is under way.

Up to 2 000 individual calls or up to 200 group calls can be made through 6-TDMA channels, though these capacities depend upon manufactures model.

Through 16-QAM scheme, 45 kbit/s transmission speed is possible with 15 kHz radio channel separation, providing image data collection at the central station, and character display at the terminals.

For terminals far from the central station, a repeater that provides dropout function is installed, enabling terminals to access to a repeater as well as to the central station. Two or more repeaters could be installed in series if necessary. By adopting the repeaters, transmitter power output of each outdoor terminal could become 10 W or less. Together with TDD and TDMA operations, low power consumption of the outdoor-terminal makes it possible to use solar power supply or hybrid of solar and wind generator.

In this standard interoperability between terminals or systems of different suppliers is assured, enabling such equipment in other areas to be brought to the disaster area for relief operations.

In ordinary times, the system is utilized for warning of storm, fire etc. as well as daily communications between the local office and residents.

Summary of technical specifications:

Frequency band:	54-70 MHz

Channel separation:	15 kHz

Transmitter power:	10 W or less

Transmission speed:	45 kbit/s

Modulation scheme:	16-QAM

Communication method:	TDMA-TDD

Voice CODEC:	16 kbit/s high efficiency voice CODEC for loud speaker operation.

*	 	This Recommendation should be brought to the attention of Radiocommunication Study Group 8 (Working Party 8A) and Telecommunication Development Study Group 2.

* 	http://www.arib.or.jp/english/html/overview/itu/itu-arib_std-t86v1.0_e.pdf.