RECOMMENDATION  ITU-R F.1108

DETERMINATION  OF  THE  CRITERIA  TO  PROTECT  FIXED  SERVICE
RECEIVERS  FROM  THE  EMISSIONS  OF  SPACE  STATIONS
OPERATING  IN  NON-GEOSTATIONARY  ORBITS  IN
SHARED  FREQUENCY  BANDS

(Questions ITU-R 118/9 and ITU-R 113/9)

(1994)

Rec. ITU-R F.1108

The ITU Radiocommunication Assembly,

considering

a)	that the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) has allocated to satellite services, on a co-primary basis, spectrum that is also allocated to the fixed service (FS);

b)	that the satellite services may wish to operate with space stations in non-geostationary orbits;

c)	that emissions from space stations operating in non-geostationary orbits and sharing the same spectrum may produce interference in receiving stations of the FS;

d)	that because of the wide geographic visibility of the emissions from space stations in non-geostationary orbits, frequency coordination with stations in the FS may not be practical;

e)	that FS systems must meet performance requirements on a worst-month basis;

f)	that the performance degradation for a FS system depends on the sum of the degradations due to emissions from all space stations that are visible to it;

g)	that studies of the power flux-density at the surface of the Earth due to emissions from space stations in non-geostationary orbits can be carried out by applying statistical methods to results from computer simulations,

recommends

1.	that frequency sharing criteria for FS systems sharing spectrum with space stations in non-geostationary orbits take into account the aggregate power flux-density resulting from the emissions of the total complement of space stations visible to FS stations at any point on the Earth;

1.1	that the tolerable interference be specified in terms of a power flux-density (pfd) (W/m2) in an agreed bandwidth;

2.	that pfd limits be determined on the basis of a statistical application of the principles of Recommendation ITU R F.758 in the case of digital radio-relay systems and Recommendation ITU-R SF.357 in the case of analogue radio-relay systems (method under study);

3.	that due regard be taken of the fact that ITU-T Recommendation G.826 imposes stricter error performance objectives for digital radio-relay systems;

4.	that the pfd limits take into account the orbital parameters of space stations using the band;

4.1	that the methods in Annex 1 can be used for determining the visibility statistics of space stations operating in circular orbits;

4.2	that the degradation of the performance of digital systems due to emissions from single or multiple space stations be determined using the methods described in Annex 2;

4.3	that the degradation of the performance of analogue systems due to emissions from single or multiple space stations be determined using the methods described in Annex 3.

Note 1 – Further study should be continued to improve the methodology described in this Recommendation.

ANNEX  1

Determination of the visibility statistics of space stations 
operating in circular non-geosynchronous
orbits as seen by a terrestrial station

# 1.	Introduction

In order to develop sharing criteria between low-Earth orbiting satellites and FS systems, it is necessary to determine how often a satellite will be visible in any direction for a particular terrestrial station or position and how strong will be the interference received from it. The purpose of this Annex is to develop the equations necessary to simulate the operation of a low-Earth orbiting satellite and thereby the necessary statistics. The development is sufficiently general that the results can be applied either for a random model or for a time evolutionary model.

Section 2 of this Annex provides a development of the equations of motion of a satellite, which is in a circular orbit, in an inertial coordinate system. In § 3, these equations are transformed to a coordinate system fixed on the Earth. The azimuth and distance of the sub-satellite point from a position on the surface of the Earth are determined in § 4.  In § 5, the expressions for the elevation and off-boresight angle of the satellite are developed, and a simple criterion for testing for the visibility of a satellite that is above a particular position on the Earth is given.

A right-handed spherical coordinate system is used throughout this development for Earth-centred coordinates with (r, q, l) where r is the distance from the origin, q is the angular distance from the North Pole, and l is the angle around the pole.

# 2.	The satellite in the inertial frame

In order to determine the position of the satellite in the inertial frame, its position in the orbital plane must first be determined. For a body in a circular orbit around the Earth this description involves four Keplerian orbital parameters as follows:

Rs :	orbital radius, the distance from the centre of the Earth to the satellite

I :	inclination angle (rad), the angle between the orbital plane and the Earth’s equatorial plane. It is measured from 0 to p and is less than p/2 if the satellite is headed eastward as it crosses the equatorial plane from South to North and greater than p/2 if the satellite is headed westward as it crosses the equatorial plane from South to North

Ws :	angular distance (rad) along the equatorial plane from the zero reference to the position of the ascending node, the point where the plane of the satellite crosses the equatorial plane from South to North

M :	mean anomaly (rad), the angular arc in the satellite orbital plane measured from the ascending node to the position of the satellite

To determine the coordinates of the satellite in the inertial spherical coordinate system, one must first determine the position of the satellite referenced to Wo, the angular position or longitude of the ascending node, measured East of the first point of Aries. The position of the sub-satellite point is denoted by qs and l0.

These coordinates may be determined by spherical geometry with reference to Fig. 1. Applying the law of cosines to the arc qs gives: cos qs = sin M sin I. Since q is defined on the interval (0, p):

qs  =  arc cos (sin M sin I)	(1)

FIGURE 12/F.1108...[D01] = 6 CM

Similarly, applying the law of cosines to the arc M gives cos M = sin qs cos l0 equation (2) gives the values of l0 for the entire range (q, 2p).

(2)

# 3.	Transformation to Earth coordinates

These coordinates may be transformed simply to equivalent Earth coordinates. Since the Earth rotates eastward through 2p radians in 23 h, 56 min, and 4.09 s, the East longitude of the sub-satellite point, ls is given by:

ls  =  l0  +  Ws  –  DE t	(3)

where DE = 7.292115856 ´ 10–5 rad/s.

To complete a time description of the position of the sub-satellite point one needs to account for the position of the orbit as well as the position of the satellite on the orbit. The ascending node precesses westward at a rate of 9.964 (RE / RS)3.5 cos I degrees per day, where RE (= 6378.14 km) is the equatorial radius of the Earth. Hence, the location of the ascending node evolves in time as:

Ws  =  W0  –  DL t

where:

DL = –2.0183 × 10–6 (RE / Rs)3.5 cos I. Thus equation (3) becomes:

ls  =  l0  +  W0  –  (DL  +  DE)  t	(4)

The orbital period (s) of a satellite in a circular orbit of radius Rs is given by Ts = 9.952004586 ´ 10–3 Rs1.5, where Rs is the radius of the satellite orbit (km). Hence:

M  =  M0  +  DM t	(5)

where DM = 2p / Ts.

# 4.	Distance and azimuth to a terrestrial station

The position of the terrestrial station must first be converted from standard coordinates of latitude and longitude into spherical coordinates. If LT is the latitude and LoT is the longitude of the terrestrial station, both positive angles (degrees), the spherical coordinates of the station in radians, qT and lT, may be obtained with the following two relations.

(6)

(7)

The difference in longitude from the terrestrial station to the sub-satellite point, lD, is just

lD  =  lS  –  lT	(8)

The distance X between the terrestrial station and the sub-satellite point in radians of arc may be determined by the law of cosines, referring to Fig. 2, as:

X  =  arc cos (cos qT  cos qs  + sin qT  sin qs  cos lD)	(9)

FIGURE 2/F.1108...[D02] = 3 CM

The sub-satellite point is East of the terrestrial station if sin lD is greater than zero and is West of the terrestrial station if sin lD is less than zero. Hence the azimuth Z from the station to the sub-satellite point is obtained by applying the law of cosines to the arc qs in Fig. 2.

(10)

# 5.	Satellite elevation and angular distance from main beam

The elevation angle H of the satellite above the horizon of the terrestrial station, assuming a horizon angle of 0°, may be obtained by referring to Fig. 3.

(11)

FIGURE 3/F.1108...[D03] = 6 CM

Assume that the receiving antenna of the terrestrial antenna is aimed along the azimuth ZT with an elevation angle of HT radians above the local horizontal. The angular distance j from the main beam of this terrestrial station antenna to the satellite may be obtained by considering the spherical coordinate system centred on the terrestrial station with its axis in the zenith direction, as shown in Fig. 4. Applying the law of cosines to the side j gives:

j  =  arc cos (sin HT  sin H  +  cos HT   cos H  cos (Z  –  ZT))	(12)

FIGURE 4/F.1108...[D04] = 6.5 CM

Equations (1) to (12) provide a means for simulating the interference environment of a terrestrial station in the presence of a low-Earth orbiting satellite. Some simplifications are possible. For instance, only interference from satellites above the horizon is usually considered. From equation (11), the satellite is above the horizon for:

cos X  >  RE / Rs  =  g	(13)

By using (13) in (9), it is possible to develop an expression for the range of longitudes that are within this circle of visibility for a particular sub-satellite point latitude or mean anomaly. Hence equations (10), (11) and (12) need only be evaluated under conditions that can be predetermined.

ANNEX  2

Determination of the effects of emissions from
space stations in non-geostationary orbit on
fixed service digital microwave receivers

# 1.	Introduction

The development of criteria for band sharing between the FS and space services using low-Earth orbiting satellites (LEOS) requires an understanding of the effects that the emissions from such satellites have on the performance of digital terrestrial receivers. The present Annex describes an approach to evaluating these effects on digital radios using a computer simulation. The simulation develops the statistics of interference power that would be received at a given location on a receiving antenna pointed in any one of a set of directions. It uses the orbital equations developed in Annex 1 and allows for either a single satellite or for a uniform constellation of satellites. Specific quantities developed include:

–	the percentage of time that the received interference power exceeds a level; and

–	the fractional increase in the probability of not meeting a performance criterion.

Detailed descriptions of the elements of the simulation are given in § 2 and 3 of the present Annex; the development of the expressions for performance degradation is given in § 4. Some representative results of interest are given in § 5.

# 2.	Statistical modelling

The simulation develops interference statistics by considering the interference received from each of Na equally spaced positions on an orbit. The total set of interferences is obtained by considering N0 orbits with their ascending nodes equally spaced around the equator. While this may not be representative in many cases, it is the most favourable or most permissive assumption for sharing in that it spreads out the interference most widely across the surface of the Earth. The consequences of this assumption need to be considered together with the assumption of circular orbits in further studies.

Clearly, for a single satellite, there are Na × N0 possible interference positions. Thus the probability associated with any one of them is 1/(Na N0).

The case of multiple satellites in a uniform constellation is also easily considered. A uniform constellation consists of a set of satellites spread over a set of identical uniformly spaced orbital planes. Each orbital plane contains the same number of satellites uniformly spaced on the orbit. The most uniform coverage and the most permissive sharing is obtained when the sets of satellites are equally phased. This means that when there is a satellite on one orbit crossing the equator, there will be a satellite on each of the orbital planes crossing the equator in the same direction.

In terms of the previously defined quantities, if Na is an integral multiple of the number of satellites per orbit, Nspo, and N0 is an integral multiple of the number of orbital planes, Norb, in the constellation, the number of unique states of the constellation is just (N0 / Norb)(Na / Nspo).

# 3.	Interference determination

Satellite emissions are assumed to be limited in the conventional manner: by a low level power flux-density (pfd) for elevation angles, on the Earth, below a lower limit and by a higher pfd for elevation angles above a higher limit, with a linear escalation with elevation angle between the limits. The pfd is specified here in dB(W/(m2(MHz)).

The antenna gain, GR (j), of the terrestrial receiver is that specified by Recommendation ITU-R F.699 for a gain, diameter, and frequency as independent inputs. The effective area applicable to determine the received interference power due to a pfd at an angle j is given by:

µ §

where l is expressed (m).

# 4.	Modelling the effect of interference on digital systems

The outage probability of a digital system is often written in the following form:

(14)

where:

DFM :	dispersive fade margin (dB)

TFM :	thermal fade margin (dB)

C/I :		ratio of unfaded signal power to the noise-equivalent value of interference power (dB)

CNC :	value of carrier-to-noise ratio at which the performance criterion is just met (dB)

C :		a constant depending on climate, terrain and link parameters.

Modern digital systems usually have dispersive fade margins larger than their thermal fade margins, and are getting better. Hence, the first term in equation (14) can be ignored for interference considerations. While the noise-equivalent interference power of a specific interferer into a specific receiver may be less than the measured power, for general sharing considerations, particularly where there are multiple interferers, it can be conservatively assumed that the interference has the same effect as thermal noise with the same power. Since the difference in decibels between the unfaded carrier-to-noise ratio and the critical carrier-to-noise ratio (CNC) is the thermal noise fade margin TFM, the fractional increase in P0, the probability of exceeding the performance objective, is equal to the ratio of the interference power I to the noise power NT, where both are measured at the detector in Watts or in Watts per unit bandwidth. That is, the fractional increase is equal to I/NT, for a constant interference power I. To simplify discussion such an increase in P0 will be designated as a fractional degradation in performance (FDP) or a percentage degradation in performance.

If an interferer caused an interference power Ii for a fraction of a month, fi, and was absent for the remainder of the month, the incremental fractional degradation in performance due to this interference would be given by:

The fractional degradation in performance due to a set of events, where the ith event consists of the fraction of time that the interference had a power Ii, is given as:

(15)

where the summation is taken over all interference events.

The fractional degradation in performance may also be expressed as a loss of fade margin (dB) FML, where:

FML  =  10 log (1  +  FDP)	(16)

whereas, the mean interference level above thermal noise is given in dB as 10 log(FDP).

Although equation (14) has been used primarily with reference to the occurrence of outage, the probability of the bit error ratio of the receiver exceeding 10–3, most measures of digital radio performance scale with the outage. Hence, equation (15) may be used as a measure of the fractional degradation of performance. While the expressions in (15) and (16) provide reasonable bounds on the degradation of performance for moderate interference levels, they may underestimate the effects for interference levels, say, 20 dB higher than the thermal noise, because the dispersive effects included in the first term in (14) begin to become important and because shallow fading may not follow the Rayleigh fading law implicit in (14). However, if such high interference events occur sufficiently rarely, their effect will probably be negligible. The form of (15) makes it eminently suited for interference studies because it allows large constellations of satellites to be treated simply regardless of whether or not they are uniform.

# 5.	Sample results of simulation

This section presents some results from a number of test runs of the simulation. The objective was to identify some basic trends and problem areas. Unless otherwise specified, the terrestrial station is assumed to be at a latitude of 40° operating at 2.000 GHz with a 2.76 m diameter antenna with 33 dB gain and that the waveguide losses are 2 dB. In accordance with Recommendation ITU-R F.759, the noise figure of the receiver is taken as 4 dB. The satellite emissions fall on the pfd limit of –130 dB(W/(m2/MHz)) for elevation angles of 5° or less and increase by 0.5 dB per degree from 5° to 25°, beyond which they attain the value of –120 dB(W/(m2/MHz)). The satellite interference is sampled at 0.5° increments around the orbit for 720 orbits uniformly spaced around the equator.

Figure 5 shows the cumulative distribution of the interference power received by antennas with different pointing azimuths from a satellite in an orbit with an altitude of 800 km and an inclination of 50°. Because of the limited inclination of the orbit, the satellite is never observed near the northern horizon. Hence the antennas pointed at azimuths within 50° of the North Pole never receive interference at boresight and have truncated distributions. As a consequence, there is a correspondingly larger probability of boresight observances at somewhat larger azimuth angles. Figure 6 shows this clearly in a plot of the percentage degradation in performance for a set of angles for this case.

Figure 7 shows the cumulative distribution of received interference power for the same situation except for the orbital inclination which is 89.5°. Clearly the effect of azimuth is much less pronounced in this case.

Figure 8 illustrates the effect of satellite altitude through a plot of percentage degradation in performance versus azimuth angle. The performance degradation caused by a satellite increases linearly with orbital altitude up to nearly 10 000 km as does the area of the Earth visible to the satellite at any instant.

The effect of the latitude of the earth station on the performance degradation is illustrated by Fig. 9. While the effects are not severe, they are increasingly important closer to the poles for near polar pointing angles.

Figures 10 and 11 show the effect of a uniform constellation of 55 satellites operating on a co-channel basis as they would if they were employing CDMA. Comparing Figs. 7 and 10, one finds the interference distributions for the single satellite and multiple satellite cases to be quite similar. The comparison is more obvious in Fig. 11 where the performance degradation caused by 55 satellites is seen to be 55 times worse than that caused by a single satellite.

FIGURE 5/F.1108...[D05] = 20 CM

FIGURE 6/F.1108...[D06] =  11.5 CM

FIGURE 7/F.1108...[D07] = 20 CM

FIGURE 8/F.1108...[D08] = 13.5 CM

FIGURE 9/F.1108...[D09] = 14 CM

FIGURE 10/F.1108...[D10] = 23 CM

FIGURE 11/F.1108...[D11] = 17.5 CM

# 6.	Interim conclusions

The degradation in performance was shown to be linearly dependent on the number of satellites for satellites in a uniform constellation. In general, the degradation is additive over all satellites interfering into the pass band of a receiver.

For satellites in non-geosynchronous orbits using the same escalation in pfd with elevation angle, the fractional performance degradation experienced by terrestrial receivers at moderate latitudes has limited variation with:

–	pointing angle – within a factor of three for latitudes up to 60° and altitudes up to 800 km;

–	satellite altitude – increases approximately linearly with altitude;

–	station latitude – increases by an amount that is within a factor of about two from 20° to 60° latitude.

These conclusions could vary for different pfd escalations. Stations at high latitudes may experience greater variation in the fractional performance degradation with respect to azimuth angle for satellites in near polar orbits. Sharing may be facilitated by coupling the escalation or the pfd limit itself to orbit altitude and inclination. These possibilities merit further study.

These conclusions depend on the use of nearly circular orbits that are not geosynchronous. The use of highly elliptical orbits requires further information and further study particularly because the interference statistics may not be stable over the one-month periods during which performance criteria must be met. The means to accommodate such operation requires further study.

The use of geosynchronous orbits would increase the variations of the percentage degradation in performance with respect to both pointing angle and location of the terrestrial station. These are the types of considerations that are normally part of a detailed coordination procedure. Since coordination with every terrestrial station would seem to be impractical, the alternative would be to decrease the allowed satellite emissions (pfd). While such limits could be devised simply, further study is needed.

If both geostationary and non-geostationary satellites were used in the same frequency band the sharing considerations would become increasingly complex, especially since the interference effects of the two types of satellites are additive.

ANNEX  3

Simulation of interference into analogue radio-relay
routes from low-Earth orbiting satellites

# 1.	Introduction

This Annex 3 describes a computer program which implements the mathematical relationships developed in Annex 1 of this Recommendation. The resulting program can be used as an analysis tool for examining interference into simulated analogue radio-relay networks that share spectrum with low-Earth orbiting satellites (LEOs) representative of those that may operate in bands below 3 GHz. A number of example sharing scenario situations and their results are described.

# 2.	Description of the model

The program mathematically simulates the path of a constellation of LEOs taking into account the Earth's rotation and orbit precession effects. Interference is calculated for each 1/2 degree movement of the satellite in the constellation into each radio-relay receiver in a concentration randomly distributed radio-relay routes. The program accumulates interference density data for each radio-relay route for the period of the simulation. The program converts this data into a probability distribution for each route so that the performance of each route can be separately analysed. The results of the example scenarios described here are compared with the reference performance requirements described in Fig. 1 of Recommendation ITU-R SF.357. Recommendation ITU-R SF.357 proposes reference interference sharing criteria for analogue systems only.

## 2.1	Input

The simulation allows operator selection of the following parameters:

–	frequency,

–	latitude and longitude of the centre of the radio-relay route trendlines,

–	radio-relay receive antenna gain,

–	number of radio-routes to be analysed,

–	satellite orbit altitude (same for each satellite),

–	number of satellite orbital planes,

–	longitude of the ascending node for each plane,

–	orbit inclination (same for each plane),

–	number of satellites per plane (same for each plane),

–	high angle satellite pfd level,

–	low angle satellite pfd level,

–	length (in days) of the simulation.

The assumptions that are built into the model include:

For the radio-relay system model:

50 hop, 2 500 km routes, hop directions are selected by Monte Carlo methods.

Receiver noise temperature of 1 750 K.

Baseband 4 kHz bandwidth thermal noise per hop is 25 pW.

Receive antenna characteristics per Recommendation ITU-R F.699.

Losses (feeder, conversion) of 3 dB.

For the satellite system model :

Circular orbit only.

pfd constrained to the following mask:

pfdlow		00°  £  q  £  05°

pfd  =  pfdlow  +  0.05 (pfdhi  –  pfdlow) (q  –  5)	05°  <  q  £  25°

pfdhi		25°  <  q  £  90°

## 2.2	Output

The output of the program is a single data file named Leo.dat. Information is provided for each simulated radio-relay route. The output information is arranged to indicate the time duration of interference levels received by each route. Fifty sequential, 1 dB wide, interference ranges from 1 to 100 000 pW are supported. The program automatically increments the appropriate interference range for each route that is affected by a satellite for each 1/2° increment of orbit.

# 3.	Simulation results

Recommendation ITU-R SF.357 defines both a short- and long-term limit of interference that is allowed into an angle modulated radio-relay system in bands shared with the fixed-satellite service. A linear form of interpolation is also indicated in the Recommendation for determining allowable interference levels for time durations between the long- and short-term period. Because the program calculates the interference data as a probability distribution, it is possible to evaluate each investigated sharing scenario by comparing program results with the limits of Recommendation ITU-R SF.357.

The interference limits defined in Recommendation ITU-R SF.357 are plotted on the right hand portion of the graphs of information appearing in Figs. 12 to 16 of this Annex. The curves to the left, in each figure, represent the interference into the most affected radio-relay route for the LEO/FS sharing scenario being considered.

For example, Fig. 12 presents an analysis of the effects of interference into the FS operating at 1.5 GHz, 2.0 GHz and 2.5 GHz where all other FS and LEO parameters are fixed. Two groups of scenarios were considered. The lower set of curves in the figure represents the interference effects into the FS from a single orbiting LEO. The second group of curves represent the interference effects when sufficient number of LEOs are present in one orbit plane such that one satellite is constantly in view. A LEO system with only one satellite in constant view is a convenient reference for this comparison.

FIGURE 12/F.1108...[D12] = 19.5 CM

Figure 13 demonstrates the effects of changes in orbit altitude and low angle arrival pfd on the interference received by the FS from one LEO in continuous view. For this LEO scenario the pair of dashed curves shows (pfd = –144 dB(W/m2) (4 kHz bandwidth)) for all angles of arrival), as might be expected, that orbit altitude, i.e. 800 km and 10 330 km, is not a significant parameter.

The solid curve in Fig. 13 demonstrates that spot beams usage by LEOs operating at either altitude will greatly reduce the level of interference into the FS.

Figure 14 shows the results of an investigation of the effects of interference into the FS as a function of FS latitude. The upper three curves represent interference distributions into the FS at three different latitudes assuming the same single constant visible satellite constraint. It would appear that latitude is not a significant parameter with regard to the shape of the distributions as they are reasonably similar.

The lower group of three curves in Fig. 14 represent the distributions of received interference distributions at different latitudes from single orbiting satellites that have high orbit angles (80°). It is interesting to note here that if the curve plots were extrapolated back to the Y axis for X = 0 it would approximately represent the percent of time that the satellites were visible to the FS systems at the indicated latitudes. Conversely the inverse of that number would also approximate the number of satellites needed to achieve constant single satellite visibility. It follows from a close observation of these curve plots in Fig. 14 that fewer satellites would be needed to continuously illuminate higher latitudes systems since the distribution for the 65° latitude radio-relay routes does appear to intercept the Y axis at a much higher point.

This might be verified intuitively by considering that for every orbit of a highly inclined satellite system each satellite in the plane would be visible for a percentage of time to terrestrial sites at more northern or southern latitudes, whereas terrestrial sites at mid or lower latitudes may not be visible to any portion of some orbits. This would suggest that LEOs optimized to serve medium and lower latitudes would cause more interference into higher latitude terrestrial systems since a larger percentage of the satellites in orbit would be visible to the higher latitude terrestrial sites.

Finally, Figs. 15 and 16 illustrate the interference effects into the FS from constellations of satellites that might represent practical operating systems. Both systems are arranged such that 3 to 6 satellites are continually visible to the terrestrial site requiring service. Figure 15 investigates a satellite constellation consisting of 6 circular orbit planes with 11 satellites per plane. All planes have the same inclination (86.5°) and the same satellite altitude (780 km). Figure 16 shows the interference distribution that might be expected from a 12 satellite constellation operating at an altitude of 10 370 km. The satellites are arranged in 3 orbit planes separated by 120° with inclinations of 56° and 4 satellites per plane.

# 4.	Conclusions

Because of the time parameter introduced by LEOs, the analysis of sharing becomes very much more difficult; and, in important respects differs from sharing with geostationary satellites. At this time there is no complete methodology for establishing protection requirements for the FS in a LEO environment. This is especially true when sharing with digital radio-relay systems. Recommendation ITU-R SF.357 provides a criteria which might be used for establishing pfd limits for sharing with analogue radio-relay routes. However, since the function of Recommendation ITU-R SF.357 is to establish sharing with geostationary satellites, it may be appropriate to review it with respect to sharing with LEO satellite networks.

Assuming the validity of Recommendation ITU-R SF.357 then it would appear that, for situations where there are limited numbers of LEOs visible to the FS that analogue radio-relay networks could share with LEO systems provided that low angles of arrival pfd emissions can be controlled. It is also apparent that, for high angles of arrival, pfd emission limits from LEO systems could exceed the levels currently established for the geostationary-satellite systems. Further study is clearly required and it is proposed that the development and use of simulation tools be encouraged to conduct such studies.

FIGURE 13/F.1108...[D13] =  19 CM

FIGURE 14/F.1108...[D14] = 20 CM

FIGURE 15/F.1108...[D15] = 3 CM

FIGURE 16/F.1108...[D16] = 3 CM

_________________