RECOMMENDATION  ITU-R  F.1191

BANDWIDTHS  AND  UNWANTED  EMISSIONS  OF  DIGITAL  RADIO-RELAY  SYSTEMS*

(Question ITU-R 119/9)

(1995)

Rec. ITU-R F.1191

The ITU Radiocommunication Assembly,

considering

a)	that the definitions of necessary and occupied bandwidth and allocated band are reported in Nos. 146, 147 and 17 of the Radio Regulations (RR) respectively;

b)	that the definitions of unwanted, out-of-band and spurious emissions are reported in RR Nos. 140, 138 and 139 respectively;

c)	that it is necessary to give guidance for the application of these definitions to digital radio-relay systems (DRRS);

d)	that it is relatively unlikely that out-of-band emissions from DRRS will cause significant interference into systems operating in adjacent bands, because:

–	the power spectrum of a DRRS decays rapidly outside the occupied bandwidth;

–	the e.i.r.p. of line-of-sight DRRS is low or medium;

–	trans-horizon DRRS employing a high e.i.r.p. are not widely used;

e)	that from the viewpoint of interference into other systems sharing the same frequency band, interference due to out-of-band emissions will be, in general, less significant than that due to emissions within the necessary bandwidth;

f)	that intra-system interference related problems, which may be caused by unwanted emissions, are normally taken into account by DRRS designers;

g)	that bands are allocated to radio-relay fixed services on a primary or co-primary basis, where a radio-frequency channel arrangement has been established by a relevant ITU-R Recommendation or by a National Regulatory Authority;

h)	that only the transmitters on the radio-frequency channels at the band edges should have to comply with the general occupied bandwidth compatibility criteria as required by RR No. 147;

j)	that unwanted emissions falling outside the relevant radio-frequency band allocated to the fixed service (FS) have to comply with the general rule stated by Recommendation ITU-R SM.329;

k)	that at the allocated band edges, radio-frequency bands ZS, as defined in Recommendation ITU-R F.746, are given by the relevant ITU-R Recommendations in order to control power spill-over into adjacent allocated bands;

l)	that it is not always possible or convenient to make the occupied bandwidth of DRRS smaller than or equal to the bandwidth of the radio-frequency channel provided by the relevant radio-frequency channel arrangement established for the allocated band by ITU-R or by a National Regulatory Authority;

m)	that within the allocated band, coordination between various radio-relay systems used on the basis of a radio-frequency channel arrangement is covered by an efficient concept summarized by Recommendation ITU-R F.746 and by the statistical propagation behaviour reported in Recommendation ITU-R P.530 and Recommendation ITU-R F.1093;

n)	that DRRS, with suitable scrambling applied, have in general a transmitted spectral density and unwanted emissions with power peak factors that may be conservatively considered noise-like;

o)	that DRRS have unwanted emissions composed of both noise-like and discrete components, made up of out-of-band and spurious emissions, which are not easy to distinguish one from the other,

recommends

1	that the following general definitions apply to DRRS:

## 1.1	Occupied bandwidth

The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage b/2 of the total mean power of a given emission (Article 1, RR No. 147).

## 1.2	Necessary bandwidth

For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions (Article 1, RR No. 146).

## 1.3	Allocated frequency band

Allocation (of a frequency band): entry in the Table of Frequency Allocations of a given frequency band for the purpose of its use by one or more terrestrial or space radiocommunication services or the radioastronomy service under specific conditions. This term shall also be applied to the frequency band concerned (Article 1, RR No. 17).

For DRRS the allocated frequency band may be considered as the overall frequency band allocated to the FS on a primary or co-primary basis.

## 1.4	Radio-frequency channel bandwidth

Bandwidth equal to the frequency spacing, defined in Recommendation ITU-R F.746, of adjacent channels of the relevant radio-frequency channel arrangement established within the allocated frequency band.

## 1.5	Guardband

Bandwidth equal to the frequency spacing, defined in Recommendation ITU-R F.746 as ZS, between the nominal centre frequency of the outermost channel of a radio-frequency channel arrangement and the limit of the allocated band.

## 1.6	Unwanted emissions

Consist of spurious emissions and out-of-band emissions (Article 1, RR No. 140).

For DRRS an example of a typical scenario is reported in Fig. 2.

## 1.7	Out-of-band emission

Emission on a frequency or frequencies immediately outside the necessary bandwidth which results from the modulation process, but excluding spurious emissions (Article 1, RR No. 138).

## 1.8	Spurious emission

Emission on a frequency or frequencies which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products, but exclude out-of-band emissions (Article 1, RR No. 139);

NOTE 1 – Intermodulation products include radio-frequency channels odd order intermodulation due to non-linearities in the radio-frequency branching and antenna systems.

2	that the following specific design objectives and definitions be used for DRRS; an illustration of those objectives and definitions can be found in Fig. 1;

2.1	that, for DRRS, the value of percentage b/2 should be taken as 0.5%;

2.2	that, for DRRS, the necessary bandwidth is to be considered to have the same value as the occupied bandwidth;

2.3	that, according to the type of the utilized radio-frequency channel arrangement (Note 1), the capacity and the modulation format of the transmitted signal, similar DRRS could have a necessary bandwidth which is no more than 20% wider than the radio-frequency channel bandwidth (Note 1); however, since dissimilar systems operating in the same band could give rise to certain incompatibilities, the relationship between the RF channel bandwidth and the necessary bandwidth requires further study;

2.4	that the determination of occupied bandwidth should be done with a spectrum analyser method described in Recommendation ITU-R SM.328 or, whenever possible, by numerical evaluation or integration of the actual emitted spectrum as reported in Annex 1;

2.5	that when burst transmission is used (e.g. for TDMA DRRS) the evaluation of bandwidths and emissions should refer to the activated transmitter (“on air” condition);

2.6	that DRRS should use suitable scrambling circuitry in order to maintain all the spectral emissions (both wanted and unwanted) independent from the input data stream;

2.7	that, as far as DRRS are concerned, no distinction has to be made between out-of-band and spurious emissions, they have to be considered together as unwanted emissions;

2.8	that, above and below the limits of the necessary bandwidth, the permissible mean power level of unwanted emission should be less than or equal to 0.5% of the total transmitted mean power taken at the radio antenna port (Note 2);

2.9	that, from the viewpoint of the international regulations, it is not necessary to establish any additional limitation on the spectral shape of unwanted emissions from DRRS;

2.10	that, for operating frequencies defined by ITU-R Recommendations, in any 4 kHz band, the centre frequency of which is removed from the radio-frequency channel centre frequency by more than 250% of the RF channel bandwidth (Note 1), the ratio between the mean power output and the unwanted emission shall be greater than or equal to A (dB), limited to a maximum of 80 dB,

where:

A  =  43  +  10 log10 P               dB

A :	(dB) below the mean output power level taken at the radio antenna port

P :	mean output power (W) taken at the antenna port;

2.11	that, to reduce measuring time, the measurement band could be wider than 4 kHz providing the results are normalized to 4 kHz (Note 3);

2.12	that, without other specific agreement between administrations sharing the same band edge, the digital radio-relay transmitters operating on the outermost channel frequencies of a radio-frequency channel arrangement should have an occupied bandwidth so that the outermost half of it, when added to the absolute value of the frequency tolerance (Note 4), results in a bandwidth smaller than or equal to the value of ZS as defined in § 1.5.

FIGURE 1...[D01] = 3 CM

NOTE 1 – See Recommendation ITU-R F.746 for definitions of alternated, co-channel mode band re-use and interleaved mode band re-use radio-frequency channel arrangements. Channel bandwidth is defined as XS/2 for alternated frequency channel arrangements and XS for co-channel and interleaved frequency channel arrangements.

NOTE 2 – Due to possible compatibility problems, caution should be exercised when applying this Recommendation to high capacity systems, bands which have dissimilar systems in adjacent channels, and bands which are shared with other services.

NOTE 3 – The normalization to a 4 kHz bandwidth may be accomplished by the following relationships where BWe is defined as the equivalent evaluation bandwidth of the measurement instrument:

Power level at 4 kHz  =  Power level at BWe

for discrete components and

Power level at 4 kHz (dBm)  =  Power level at BWe (dBm)  –  10 log

for noise-like components.

Some administrations propose to use measurement bandwidth BWe of 100 kHz for frequencies less than 17.7 GHz and 1 MHz for frequencies greater than 17.7 GHz.

NOTE 4 – The precise specification of frequency tolerance values is left to the National Regulatory Authorities.

ANNEX  1

# 1	Typical emission scenario of a digital radio-relay system

Figure 2 shows the typical scenario based on typical and most important emissions of a heterodyne digital radio-relay transmitter. Other emissions (e.g. other conversion products and residual components of the carrier generation) are not shown. For directly modulated radio-frequency transmitters, some unwanted emissions (e.g. conversion products and local oscillator leakage) are not applicable.

# 2	Calculation of occupied bandwidth

Generally, a normalized power spectrum W(f ) of a modulated carrier of a DRRS can be expressed as follows:

(1)

where:

S(f ) :	frequency response of the shaping filter located in the transmitter

f :	frequency separation from the carrier

T :	pulse width.

Therefore, the occupied bandwidth, B0, can be calculated as follows:

(2)

FIGURE 2...[D02] = 3 CM

## 2.1	Case of phase and amplitude modulated signals with ideal square root cosine roll-off shaping

In many cases shaping filters of a square root cosine roll-off type are employed at the transmitter side and the ideal frequency response is given by:

(3)

where a is the roll-off factor between 0 and 1.

Substitution of equation (3) into equations (1) and (2) gives the ideal occupied bandwidth as follows:

B0  =  2 K(a) / T	(4)

where K(a) is a function of a and is calculated as shown in Table 1.

Thus, the occupied bandwidth can be calculated by using equation (4) and Table 1.

TABLE  1

The values of K(a)

| a | K(a) |
| --- | --- |
| 0.1 | 0.510 |
| 0.2 | 0.537 |
| 0.3 | 0.567 |
| 0.4 | 0.600 |
| 0.5 | 0.634 |
| 0.6 | 0.669 |
| 0.7 | 0.705 |
| 0.8 | 0.742 |
| 0.9 | 0.779 |
| 1.0 | 0.816 |

## 2.2	Case of phase and amplitude modulated signals with other shaping filters

Different practical implementations and other types of shaping filters are sometimes used. These cases need more complex numerical evaluations of equation (2) and are under study.

## 2.3	Case of frequency and phase modulated signals

These cases are under study.

# 3	Occupied bandwidth for multiple sub-carriers operation

In some cases, a digital radio-relay transmitter may carry multiple sub-carriers amplified by a common amplifier. The occupied bandwidth B0 for such operation should be calculated as follows:

B0  =  b0  +  (m  –  1) DF	(5)

where:

b0 :	occupied bandwidth of a single sub-carrier

m :	number of sub-carriers

DF :	frequency spacing between centre frequencies of adjacent sub-carriers.

The above formula assumes that the multiple sub-carriers are homogeneous and equally spaced. It should be noted that in this case the value of percentage b/2 for the total power of all sub-carriers is 0.5/m %.

_________________

* 	This Recommendation should be brought to the attention of Radiocommunication Study Group 1 (TG 1/3).