RECOMMENDATION  ITU-R  F.1243*

RADIO-FREQUENCY  CHANNEL  ARRANGEMENTS  FOR  DIGITAL
RADIO  SYSTEMS  OPERATING  IN  THE  RANGE  2 290-2 670  MHz

(Question ITU-R 136/9)

(1997)

Rec. ITU-R F.1243

The ITU Radiocommunication Assembly,

considering

a)	that as a result of the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) many administrations may wish to plan fixed services around the bands designated for new mobile and mobile-satellite services;

b)	that it is sometimes desirable to be able to interconnect low and medium capacity digital radio systems on international circuits using radio frequencies in the 2 GHz band;

c)	that many interfering effects can be substantially reduced by a carefully planned arrangement of the radio frequencies in radio systems employing several radio-frequency channels;

d)	that sharing between the fixed satellite (FS) and mobile service (MS) or mobile-satellite service (MSS) may be possible by using adequate geographical and/or frequency separation;

e)	that administrations may have different requirements for sharing spectrum with the FS, and they may not require all the spectrum allocated to MS or MSS;

f)	that the use of industrial, scientific and medical (ISM) devices in the band 2 400-2 500 MHz may cause harmful interference to systems in the FS,

recommends

1	that where the introduction of MS and MSS limits the FS to the band 2 520-2 670 MHz, a radio-frequency channel arrangement with up to five go and return channels is used (carrier spacing of 14 MHz), which is derived as follows:

let		f0	be the band centre frequency of 2 595 MHz,

fn	be the centre frequency of a radio-frequency channel in the lower half of the band,

be the centre frequency of a radio-frequency channel in the upper half of the band,

DS	(Tx/Rx duplex spacing)  74 MHz,

then the frequencies of individual channels with a carrier spacing of 14 MHz are expressed by the following relationships:

lower half of band:	fn    f0  –  79    14 n	MHz

upper half of band:	    f0  –  5    14 n	MHz

where n  1, 2, 3, 4 or 5.

The frequencies of individual channels of channel arrangements with carrier spacings of 7, 3.5 and 1.75 MHz are expressed by the following relationships:

Carrier spacing of 7 MHz:

lower half of band:	fn    f0  –  75.5    7 n	MHz

upper half of band:	    f0  –  1.5    7 n	MHz

where n  1, 2, 3, ... 10.

Carrier spacing of 3.5 MHz:

lower half of band:	fn    f0  –  73.75    3.5 n	MHz

upper half of band:	    f0  + 0.25    3.5 n	MHz

where n  1, 2, 3, ... 20.

Carrier spacing of 1.75 MHz:

lower half of band:	fn    f0  –  72.875    1.75 n	MHz

upper half of band:	    f0  + 1.125    1.75 n	MHz

where n  1, 2, 3, ... 40;

2	that where the introduction of MS and MSS limits the FS to the band 2 520-2 670 MHz, a radio-frequency channel arrangement, alternative to that reported in § 1 and using the same band centre frequency and duplex spacing, with carrier spacing of 2 MHz can be accommodated with up to 35 go and return channels, the frequencies of individual channels being expressed by the following relationships:

lower half of band:	fn    f0  –  73    2 n	MHz

upper half of band:	    f0  + 1    2 n	MHz

where n  1, 2, 3, ... 35.

For lower carrier spacings, the frequencies of individual channels are expressed by the following relationships:

Carrier spacing of 1 MHz:

lower half of band:	fn    f0  –  72.5    1 n	MHz

upper half of band:	    f0  + 1.5    1 n	MHz

where n  1, 2, 3, ... 70.

Carrier spacing of 0.5 MHz:

lower half of band:	fn    f0  –  72.25    0.5 n	MHz

upper half of band:	    f0  + 1.75    0.5 n	MHz

where n  1, 2, 3, ... 140.

Carrier spacing of 0.25 MHz:

lower half of band:	fn    f0  –  72.125    0.25 n	MHz

upper half of band:	    f0  + 1.875    0.25 n	MHz

where n  1, 2, 3, ... 280.

Channel arrangements with lower carrier spacings are possible by means of further channel subdivision;

3	that where coexistence in the sub-band 2 590-2 670 MHz of different FS applications is difficult, the radio-frequency channel arrangement be based on utilizing the band 2 290-2 360 MHz paired with the band 2 520-2 590 MHz and a homogeneous pattern with a carrier spacing of 2.5 MHz defined by the following relations:

lower half of band:	fp    fr  –  306.25    2.5p

upper half of band:	    fr  –  76.25  +  2.5p

where:

p 	1, 2, 3, … 28

fr :	reference frequency of the homogeneous pattern

fr 	2 595 MHz.

_________________

* 	This Recommendation should be brought to the attention of Radiocommunication Study Groups 10 and 11 (WP 10-11S).