RECOMMENDATION  ITU-R  F.276-2*

Frequency deviation and the sense of modulation
for analogue radio-relay systems for television

(1956-1959-1970-1974)

The ITU Radiocommunication Assembly,

considering

a)	that radio-relay systems for television using frequency modulation may form part of an international circuit;

b)	that international connections of such systems may at times have to be made at intermediate or radio frequencies;

c)	that the use of too large a frequency deviation results in an unnecessarily wide band of transmitted radio frequencies and should be avoided, because of the need for economy in the use of the radio frequency spectrum;

d)	that, for various reasons, the use of pre-emphasis may be desirable (see Recommendation ITU-R F.405),

recommends

1	that the value of the frequency deviation without pre-emphasis in radio-relay systems for television should be 8 MHz peak-to-peak, referred to the nominal peak-to-peak amplitude of the video-frequency signal (see ITU-T Recommendation J.61);

2	that when pre-emphasis, in accordance with Recommendation ITU-R F.405, is used, the relative deviation of 0 dB in Fig. 2 of that Recommendation should correspond to the value of the deviation without pre-emphasis given in § 1.

NOTE 1 – This value of deviation was established for the transmission of monochrome television signals and was subsequently adopted for colour television signals;

3	that the sense of modulation at the point of international connection should be the subject of agreement between the administrations concerned.

*	Radiocommunication Study Group 9 made editorial amendments to this Recommendation in 2001 in accordance with Resolution ITU-R 44.