RECOMMENDATION  302-2

LIMITATION  OF  INTERFERENCE  FROM
TRANS-HORIZON  RADIO-RELAY  SYSTEMS

(Question 7/9)

(1959-1982-1990)

Rec. 302-2

The CCIR,

CONSIDERING

(a)	that trans-horizon radio-relay systems can cause interference over long distances which in many cases may extend across national boundaries;

(b)	that trans-horizon radio-relay systems can cause interference to all the systems sharing the same frequency bands and particularly to space communication systems;

(c)	that trans-horizon systems need some form of diversity to circumvent fading;

(d)	that multiple-diversity can be provided without using additional frequencies, e.g. by employing spaced antennas, with or without cross-polarization,

UNANIMOUSLY  RECOMMENDS

in planning trans-horizon radio-relay systems:

1.	that account be taken of the high degree of international coordination and planning which will be involved if trans-horizon radio-relay systems of this type are to occupy the same frequency bands in nearby countries without mutual interference, and that the problem would become much more complex if, in addition, they were to occupy the same frequency bands as conventional line-of-sight systems or other services;

2.	that the utmost economy in frequency should be observed;

3.	that frequency-diversity should be avoided as far as possible, particularly in those parts of the world where the frequency spectrum is likely to become congested;

4.	that special efforts should be made to operate such radio-relay systems at the lowest practicable level of radiated power, and in particular:

4.1	in the case of a frequency band shared with space radiocommunication services (space-to-Earth), due consideration should be given to interference from the trans-horizon radio-relay system to earth stations on the basis of the coordination area for receiving earth stations detemined in accordance with Recommendation 359;

4.2	for the range of angles for which the e.i.r.p. of the trans-horizon radio-relay systems is less than +40 dBW in a 4 kHz bandwith (see Note 1), careful consideration should be exercised, comparable to that required for coordination between line-of-sight radio-relay systems and earth stations. For the range of angles for which the directional e.i.r.p. of the trans-horizon system exceeds +40 dBW in a 4 kHz bandwith, more careful consideration is required;

5.	that special efforts should be made to reduce radiation in, and reception from, undesired directions;

6.	that special efforts should be made to reduce spurious emissions to the lowest practicable level.

Note 1 – This value is the maximum allowable e.i.r.p. trasmitted towards the horizon by an earth station sharing the same frequency band with the fixed service (see Article 28, No. 2541 of the Radio Regulations).

ANNEX  I

Trans-horizon radio-relay systems have interference-producing capabilities and susceptibilities not unlike those encountered in line-of-sight radio-relay systems. Differences are primarily due to the usually higher transmitting powers, narrower antenna beamwidths and more sensitive receivers encountered in trans-horizon systems. This means that siting considerations are very important with trans-horizon systems.

To minimize interference from a trans-horizon radio-relay system line-of-sight situations are usually avoided, as are areas where the diffracted signal will be strong. Under some circumstances, it may be impossible to avoid occasional interference from signals due to diffraction, strong layer reflection, and especially ducting.

To estimate expected co-channel interference, it is necessary to calculate transmission loss by subtracting the path antenna gain Gp given in Fig. 1 of Report 285 from the estimated basic transmission loss. The interfering field depends on the mean long-term loss and any additional fluctuations. At UHF and higher frequencies, the lowest observed values of extra-diffraction transmission loss are the result of atmospheric focusing and ducting, either over sea or over land.

Field strength may be combined with antenna gain to yield azimuthal distance-interference patterns for various systems and combinations. In considering non-co-channel interference, account must also be taken of transmitter spectrum distribution and receiver passband characteristics. For antenna pattern considerations see Report 614.

Although it is not possible to recommend final channel arrangements, there is a need to select frequencies in an orderly manner on a regional basis. In arriving at such agreements between administrations, the guide-lines in Report 286 (Geneva, 1982) should be observed.

Polarization discrimination is also suggested to aid the use of space-diversity and the rejection of interference.

It has been general practice to engineer tropospheric-scatter systems on the high propagation attenuations exceeded only during small percentages of the time. It should be realized that under more favourable conditions, prevailing for the rest of the time, transmitter powers and antenna gains so justified can cause increased interference fields. It may be advisable under such conditions to reduce the transmitter power temporarily.

_________________