RECOMMENDATION  384-5*

RADIO-FREQUENCY  CHANNEL  ARRANGEMENTS  FOR  MEDIUM  AND  HIGH  CAPACITY
ANALOGUE  OR  HIGH  CAPACITY  DIGITAL  RADIO-RELAY  SYSTEMS
OPERATING  IN  THE  UPPER  6  GHz  BAND

(Study Programme 1A/9 and Question 35/9)

(1963-1966-1974-1982-1986-1990)

Rec. 384-5

The CCIR,

CONSIDERING

(a)	that radio-relay systems with a capacity of 2700 telephone channels should prove to be feasible in the 6 GHz band, if due care is exercised in the planning of radio paths to reduce multipath effects;

(b)	that it is sometimes desirable to be able to interconnect, at radio frequencies, radio-relay systems on international circuits in the 6 GHz band;

(c)	that it may be desirable to interconnect up to eight go and eight return channels in a frequency band 680 MHz wide;

(d)	that economy may be achieved if at least four go and four return channels can be interconnected between radio-relay systems, each of which uses common transmit-receive antennas;

(e)	that a common radio-frequency channel arrangement for both up to 1260 and 2700 telephone channel radio-relay systems offers considerable advantages;

(f)	that the use of certain types of digital modulation (see Report 378) permits the use of the radio-frequency channel arrangement defined for 2700 telephone channel systems for the transmission of digital channels with a bit rate of the order of 140 Mbit/s or synchronous digital hierarchy bit rates;

(g)	that for these digital 140 Mbit/s radio systems, further economies are possible by accommodating up to eight go and return channels on a single antenna with suitable performance characteristics;

(h)	that many interfering effects can be reduced substantially by a carefully planned arrangement of the radio frequencies in radio-relay systems employing several radio-frequency channels;

(j)	that the radio-frequency channels should be so arranged that an intermediate frequency of 70 MHz may be used for up to 1260 channel systems;

(k)	that the radio-frequency channels should be so arranged that an intermediate frequency of 140 MHz may be employed for 2700 channel systems,

UNANIMOUSLY  RECOMMENDS

1.	that the preferred radio-frequency channel arrangement for up to eight go and eight return channels, each accommodating 2700 telephone channels, or a bit rate of the order of 140 Mbit/s, or synchronous digital hierarchy bit rates (Note 3), and operating at frequencies in the upper 6 GHz band, should be derived as follows:

Let	f0	be the frequency (MHz) of the centre of the band of frequencies occupied;

fn	be the centre frequency (MHz) of one radio-frequency channel in the lower half of the band;

be the centre frequency (MHz) of one radio-frequency channel in the upper half of the band;

then the frequencies (MHz) of individual channels are expressed by the following relationships:

lower half of the band:	fn	=	f0  –  350  +  40 n,

upper half of the band:		=	f0  –  10  +  40 n,

where

n  =  1, 2, 3, 4, 5, 6, 7 or 8;

2.	that, in the section over which the international connection is arranged, all the go channels should be in one half of the band, and all the return channels should be in the other half of the band;

3.	that different polarizations should be used alternately for adjacent radio-frequency channels in the same half of the band;

4.	that, when common transmit-receive antennas are used, and not more than four channels are accommodated on a single antenna, it is preferred that the channel frequencies be selected by making either:

n  =  1, 3, 5 and 7 in both halves of the band

or

n  =  2, 4, 6 and 8 in both halves of the band (Note 4);

5.	that the preferred arrangement of radio-frequency polarization should be one of those shown in Fig. 1;

6.	that the preferred radio-frequency channel arrangement for up to 16 go and 16 return channels, each accommodating up to 1260 telephone channels, or the equivalent, should be obtained by interleaving additional channels between those of the main pattern and should be expressed by the following relationship:

lower half of the band:	fN	=	f0  –  350  +  20 N,

upper half of the band:		=	f0  –  10  +  20 N,

where

N  =  1, 2, 3, . . . 15, 16;

FIGURE  1......[D01]  =  9 cm

7.	that, in the section over which international connection is arranged, all the go channels should be in one half of the band and all the return channels in the other half of the band;

8.	that different polarizations should be used alternately for adjacent radio-frequency channels in the same half of the band;

9.	that when common transmit-receive antennas are used, and not more than four radio-frequency channels are accommodated on a single antenna, it is preferred that the channel frequencies be selected by making either:

N  =  1, 5, 9, 13 or
N  =  2, 6, 10, 14 or
N  =  3, 7, 11, 15 or
N  =  4, 8, 12, 16,

in both halves of the bands and the preferred arrangement of radio-frequency polarization is as shown in Fig. 2;

FIGURE  2......[D02]  =  7.5 cm

10.	that the preferred centre frequency (f0) is 6770 MHz; other centre frequencies may be used by agreement between the administrations concerned.

Note 1 – This radio-frequency channel arrangement permits all local oscillator frequencies to be derived from a common oscillator, if desired.

Note 2 – The radio-frequency channel arrangements for systems of up to 1260-channel capacity and of 2700-channel capacity may be used on intersecting routes, as long as adequate antenna discrimination is provided.

Note 3 – Actual gross bit rates including overhead may be as much as 5% or more higher than net transmission rates.

Note 4 – The use of a single antenna working allows for seven go and return channels with the channel arrangement of Fig. 1(a). The channel arrangement of Fig. 1(b) and suitable antenna performance gives higher isolation between transmit and receive channels, allowing the use of the eight go and return channels.

_________________

*	This Recommendation applies only to line-of-sight and near line-of-sight radio-relay systems.