RECOMMENDATION  ITU-R  F.396-1*

Hypothetical reference circuit for trans-horizon radio-relay systems
for telephony using frequency-division multiplex

(1963-1966)

The ITU Radiocommunication Assembly,

considering

a)	that trans-horizon radio-relay systems may form part of an international connection;

b)	that the characteristics of trans-horizon systems do not allow the application of existing hypothetical reference circuits for line-of-sight radio-relay systems;

c)	that trans-horizon systems are generally limited to 120 telephone channels not utilizing supergroup through-connection;

d)	that the specific characteristics of trans-horizon systems are usually individually optimized,

recommends

1	that a hypothetical reference circuit for trans-horizon radio-relay systems should be 2 500 km long;

2	that the hypothetical reference circuit for trans-horizon radio-relay systems should not be divided into homogeneous sections of fixed length because these systems, as distinct from line-of-sight systems, are usually composed of long radio sections, the length of which depends on local conditions and may vary considerably (e.g. between 100 and 400 km);

3	that, if a radio section under study is L km long, the hypothetical reference circuit should be composed of 2 500/L sections of this type in tandem, the value 2 500/L being taken to the nearest whole number;

4	that the hypothetical reference circuit should include:

–	3 sets of channel modulators,

–	6 sets of group modulators,

–	6 sets of supergroup modulators,

for each direction of transmission, the term “set of modulators” being taken to comprise a modulator and a demodulator.

*	Radiocommunication Study Group 9 made editorial amendments to this Recommendation in 2001 in accordance with Resolution ITU-R 44.