RECOMMENDATION  ITU-R  F.404-2*,**

Frequency deviation for analogue radio-relay systems
for telephony using frequency-division multiplex

(1956-1959-1963-1966-1970)

The ITU Radiocommunication Assembly,

considering

a)	that frequency-division multiplex systems for telephony using frequency modulation may form part of an international circuit;

b)	that it may at times be desirable to make international connections between such systems at intermediate or radio frequencies;

c)	that, to economize in the use of the frequency spectrum, it is desirable to use the smallest satisfactory frequency deviation;

d)	that the use of pre-emphasis allows a more uniform distribution of signal-to-noise ratio in the various channels of a multi-channel telephony system,

recommends

that, as far as practicable, radio-relay systems for telephony using frequency-division multiplex forming part of an international circuit should conform to the following characteristics:

# 1	Frequency deviation without pre-emphasis

| Maximum number of channels | r.m.s. deviation per channel(1) (kHz) |
| --- | --- |
| 12 24 60 120 300 600 960 1 260 1 800 2 700 | 35 35 50, 100, 200 50, 100, 200 200 200 200 140, 200 140 140 |
| (1) For 1 mW, 800 Hz tone at a point of zero reference level. |  |

Larger capacity systems are not excluded.

NOTE 1 – It is recognized that it may sometimes be desirable to use other deviations by agreement between the administrations concerned. This applies in particular to trans-horizon radio-relay systems.

# 2	Frequency deviation with pre-emphasis

Where pre-emphasis is used, the pre-emphasis characteristic should preferably be such, that the effective (r.m.s.) deviation due to the multi-channel signal is the same with and without pre-emphasis.

*	This Recommendation applies to line-of-sight and near line-of-sight radio-relay systems and, where appropriate, to trans-horizon radio-relay systems.

*	*	Radiocommunication Study Group 9 made editorial amendments to this Recommendation in 2001 in accordance with Resolution ITU-R 44.