RECOMMENDATION  594-3

ALLOWABLE  BIT ERROR  RATIOS  AT  THE  OUTPUT  OF  THE  HYPOTHETICAL
REFERENCE  DIGITAL  PATH  FOR  RADIO-RELAY  SYSTEMS  WHICH  MAY
FORM  PART  OF  AN  INTEGRATED  SERVICES  DIGITAL  NETWORK

(Question 134/9)

(1982-1986-1990-1991)

Rec. 594-3

The CCIR,

considering

a)	that the performance objectives of digital radio-relay systems should be defined;

b)	that the definition of the allowable bit-error performance of the hypothetical reference digital path (HRDP) is necessary for the design and construction of radio-relay systems;

c)	that propagation and other effects suggest that the bit-error performance should be stated statistically as a percentage of time;

d)	that a bit-error measurement requires a certain duration of time which depends on the magnitude of the error ratio;

e)	that the effect of error burst phenomena and jitter may need to be taken into account;

f)	that during normal operation, periods of high bit error ratio will occur which will cause short interruptions, these interruptions being mostly caused by adverse propagation conditions (see Note 7);

g)	that the performance objectives for an international digital connection forming part of an integrated services digital network (ISDN) have been specified by the CCITT (see Recommendation G.821);

h)	that the hypothetical reference digital path (HRDP) defined in CCIR Recommendation 556 corresponds to the “high-grade performance” specified in CCITT Recommendation G.821 and for which the CCITT has established a rule of apportionment of the total impairment allowance,

recommends

1.	that the following performance objectives are stated for each direction of the 64 kbit/s hypothetical reference digital path (HRDP) specified in CCIR Recommendation 556;

2.	that fading, interference and all other sources of performance degradation are taken into account in establishing the values given below;

3.	that the bit error ratio should not exceed the following values:

3.1	1 × 10–6 during more than 0.4% of any month; integration time 1 min; (minutes of degraded performance) (see Notes 10 and 11);

3.2	1 ´ 10–3 during more than 0.054% of any month; integration time 1 s (severely errored seconds);

4.	that the total of errored seconds should not exceed 0.32% of any month (see Notes 8 and 9);

5.	that Annex 1 should be used for guidance on factors which need to be taken into account in determining performance requirements for digital radio-relay systems applied to the HRDP.

Note 1 – The limits proposed are based on the best knowledge currently available but are subject to review in the future in the light of further studies.

Note 2 – The output bit stream from a digital radio-relay system suffers from jitter. The subject requires further study and is also being considered by the CCITT.

Note 3 – This Recommendation relates to the hypothetical reference digital path (HRDP). The values given are for use by the system designer, and it is not intended that they should be quoted in specifications of equipment or used for acceptance tests.

Note 4 – Contributions from multiplex equipment are not included.

Note 5 – The Recommendation applies only when the system is considered to be available in accordance with CCIR Recommendation 557 and includes periods of high bit error ratio exceeding 10–3 which persist for periods of less than 10 consecutive seconds. Periods of high bit error ratio which persist for 10 consecutive seconds duration or longer are taken into account by Recommendation 557.

Note 6 – The limits given in § 3.2 are based upon the 10 s non-availability criteria given in CCIR Recommendation 557, and therefore will not necessarily include all forms of degradations in performance due to adverse propagation. Degradations due to adverse propagation which persist for 10 s or longer will be limited by the requirements of Recommendation 557.

Note 7 – Adverse propagation conditions can result in a decrease of the wanted signal and/or an increase in the level of interfering signals.

Note 8 – The relationship between the errored seconds of a 64 kbit/s channel and the corresponding parameters which may be measured directly at the bit rate of the radio-relay system is still under study. For the time being, the errored seconds should be measured only at the 64 kbit/s interface.

Note 9 – The errored second objective described in § 4 could usually be satisfied when the objectives of § 3.1 and 3.2, and the residual bit error ratio (RBER) objective for 2 500 km (see CCIR Recommendation 634) are satisfied, taking into account typical cumulative error probability distributions.

Note 10 – Measurements of BER are normally made at much higher bit rate than 64 kbit/s, for example at the bit rate of the radio-relay system.

Note 11 – The seconds during which the bit error ratio exceeds 1 ´ 10–3 should not be taken into account in the integration time.

Note 12 – The requirements are intended to meet the relevant performance objectives of CCITT Recommendations G.821 and G.921 under all normally envisaged operating conditions. Recommendation G.821 remains the overriding performance objective of the network.

ANNEX  1

Factors to be taken into account when determining
performance requirements for digital radio-relay 
systems applied to the HRDP

This Annex considers factors which need to be taken into account in determining performance requirements for digital radio-relay systems applied to the hypothetical reference digital path (HRDP).

# 1.	Bit error ratio (BER) objectives, DM and SES criteria

In Recommendation G.821 the CCITT states that:

–	the bit error ratio of the 25 000 km high grade portion of the longest hypothetical reference connection should not exceed 1 ´ 10–6 for more than 4% of minutes within any month (DM);

–	for a 27 500 km hypothetical reference connection (HRX) the bit error ratio should not exceed 1 ´ 10–3 for more than 0.2% of seconds within any month (SES);

–	the bit error ratio of the 25 000 km high grade portion of the hypothetical reference connection should not exceed 1 ´ 10–3 for more than 0.04% of seconds within any month. In addition, an allowance of 0.05% is given to a 2 500 km hypothetical reference digital path (HRDP) for radio-relay systems to take account of adverse propagation conditions (SES).

Therefore, for a 2 500 km HRDP, the BER should not exceed 1 ´ 10–3 for more than 0.05% + 0.004% where the 0.004% has been obtained by linearly scaling the 0.04% of the whole 25 000 km high grade portion of the HRX.

With advances in high speed data transmission services, some users may require higher transmission quality than specified above. In particular, users may become concerned with the frequency and duration of circuit interruptions. Although it is difficult to predict the frequency of such breaks from a knowledge of the design of radio-relay systems, it may be appropriate to reduce the SES objective by considering whether the additional CCITT Recommendation G.821 allowance for adverse propagation may be discarded. If this is the case, it will be necessary to include advanced technology in the design of radio-relay systems to achieve such high performance without compromising the economics of the system.

Considering that radio-relay systems have been designed to respect the existing SES objectives and that in some cases this already requires the use of the most advanced technologies and fading counter-measures, any reduction of the SES objective should be carefully examined.

# 2.	Errored seconds (ES) objective

The bit error ratio (BER) parameter does not give an accurate estimate of the data circuit performance except when the error distribution is known. The more desirable alternative block error-ratio criterion unfortunately requires a knowledge of data block size and data rate which vary from user to user. The ES based objective is one way in which the above limitations can be resolved. The CCITT has recommended that, for ISDN services including data services, a 3.2% ES objective should be attained on the 25 000 km high-grade portion in the longest hypothetical reference connection. A value of 0.32% ES for the hypothetical reference digital path is consistent with Recommendation G.821 of the CCITT, and is specified as such in this Recommendation.

In general, there is no simple relationship between bit-error ratio performance and the ES performance during periods of multipath fading.

# 3.	Possible error burst objective

With many commonly used digital demultiplex equipments, the occurrence of error bursts on large-capacity digital radio-relay systems will cause undesirable effects in the telephony network, such as multiple call dropout. Consideration may therefore have to be given to placing a limit on these error bursts. Area (1) of Fig. 1 illustrates a possible limit which would serve to control the incidence of multiple call dropout, and is based upon proposals from the United Kingdom.

Error bursts in digital radio-relay systems which could cause multiple call dropout, occur mainly because of fading. Analysis of field experiment results by one administration has shown that as fading events decrease in severity, or as fading countermeasures are applied, then the number and length of error bursts will be reduced.

The above objective, which might incur inordinate economic penalties for digital radio-relay systems, is critically dependent upon the performance of demultiplex and signalling equipment. This would appear to be a matter for resolution with CCITT Study Group XVIII.

# 4.	Other factors

Timing jitter has an influence on digital performance, particularly on long paths, and requires further investigation.

FIGURE  1      [D01]  =  20.5 cm

It should be noted that the addition or loss of a bit, due for instance to switching without prior alignment to ensure coincidence or to a spurious impulse affecting the timing, may desynchronize the downstream transmission chain and cannot be considered an isolated error. This phenomenon affects system performance and should be studied further.

_________________