RECOMMENDATION  758

CONSIDERATIONS  IN  THE  DEVELOPMENT  OF  CRITERIA  FOR  SHARING
BETWEEN  THE  TERRESTRIAL  FIXED  SERVICE  AND  OTHER  SERVICES

(Question 127/9)

(1992)

Rec. 758

The CCIR,

considering

a)	that it is necessary to establish sharing criteria between the fixed service and other services in frequency bands where both services have a primary allocation;

b)	that sharing may be managed by determining allowable values of performance and availability degradations of analogue and digital radio-relay systems caused by interference from other radio services allocated in the same frequency bands as the fixed service on a primary basis;

c)	that interference from other services sharing the same band on a non-primary basis, emissions from other services outside the shared band, and emissions from sources other than radio services must also be considered;

d)	that principles for apportioning the performance and availability degradation over the length of the radio-relay system, and between each interference source, need to be established;

e)	that the technical characteristics of each service need to be understood in order to derive interference criteria corresponding to the allowable degradation in performance and availability of the radio-relay system;

f)	that performance and availability degradation may result from both long-term and short-term interference and hence both long-term and short-term interference criteria need to be established;

g)	that availability of a basic methodology for the development of sharing criteria may be useful to other Study Groups when formulating criteria for sharing with the fixed service,

recommends

1.	that the development of sharing criteria between the fixed service and other services should be carried out in accordance with the principles described in Annex 1;

2.	that the information provided in Annex 2 should be used as guidance to the technical characteristics and sensitive sharing parameters of fixed service systems that need to be taken into account when developing criteria for sharing with other services;

3.	that studies are required to further develop appropriate short-term interference criteria;

4.	that further studies are required to derive interference criteria that are appropriate for specific types of new services.

ANNEX  1

Basic considerations in the development of sharing criteria

# 1.	Overall performance objective

One of the functions of a radiocommunications planner is to design and implement a transmission network which meets the performance objectives laid down by the CCITT and CCIR.  It is important, therefore, that real systems can meet the appropriate design objectives, recognizing the increasing use of the radio spectrum. There are various Recommendations in Volume IX-1 which relate to the overall performance objective for various types of circuit.

## 1.1	Digital performance and availability objectives

Table 1 gives details of the performance and availability objectives applicable to digital radio-relay systems. In addition, the factors to be taken into account when determining performance requirements for digital radio-relay systems are given in Recommendations 594 and 634.

TABLE  1

Digital performance and availability objectives

| Objective | HRDP | High grade | Medium grade | Local grade |
| --- | --- | --- | --- | --- |
| Error performance | Rec. 594 | Rec. 634 | Rec. 696 | Rec. 697 |
| Availability | Rec. 557 | Rec. 695 | Rec. 696 | Rec. 697 |

## 1.2	Analogue performance and availability objectives

Table 2 gives details of the noise limits and availability objectives applicable to analogue radio-relay systems.

TABLE  2

Analogue performance and availability objectives

| Objective | Telephony | Television | Trans-horizon |
| --- | --- | --- | --- |
| Noise in HRC | Rec. 393 | Rec. 555 | Rec. 397 |
| Noise in real links | Rec. 395 | – | Rec. 593 |
| Availability | Rec. 557 | – | – |

# 2.	Sub-division of the performance and availability objectives

The previous section dealt with the overall performance objectives for digital and analogue reference connections. However, there are, in practice, a large number of potential sources of interference contributing to the degradation of performance of a radio-relay system. In order to move towards a practical method for planning, the overall performance objectives need to be sub-divided between individual sections of the overall hypothetical reference circuit. Within a section, the performance objective is then apportioned between the various sources.

## 2.1	Apportionment of section performance objective

This is covered in Report 1187 (Düsseldorf, 1990). The allowable degradation is divided into an element of X% for the fixed service portion, Y% for frequency sharing on a primary basis, and Z% for all other sources of interference (It should be noted that X% + Y% + Z% = 100%). In the case of sharing with the FSS, typically, Y = 10% (e.g. Recommendation 615).

There may be a further sub-division of the X% allowance to suit local requirements and this could be apportioned in such a way as to suit the grade of service.

A particular point to note is that an interference source (say a transmitter) may affect more than one hop of a system.

# 3.	Characteristics of interference

It is necessary to have information available on interference levels arising from other services, which would degrade system performance by specific amounts. This would be facilitated if, with the assistance from other Study Groups, a table were compiled giving information on the characteristics of emissions.

Two categories of interference are worth considering:

–	the interference arising from services sharing on a primary basis that is likely to be within the receiver bandwidth from analogue or digital modulations, in either CW or burst emissions. Reference can be made to existing text where available in Volume IX-1 and Volume IV/IX-2 (e.g. Recommendation 766);

–	emissions from systems other than those sharing on a primary basis that could be numerous and diverse, and may be considered in a similar way to the spurious emissions described in Report 937 (Düsseldorf, 1990).

Ultimately, another table could be prepared, again with the assistance of other Study Groups, which compares levels of interference or Gaussian noise required to produce a specified degradation in the channel performance.

# 4.	Limit values of interference

Following the considerations in the previous sections, one may now determine the limit values of interference allowable to a particular source. This has been done for the case of frequency sharing between the fixed-satellite service and the fixed service in the joint workings of Study Groups 4 and 9, where certain models have been established. These models may be appropriate for frequency sharing between radio-relay systems and other services in general.

Methods for characterizing interference levels into terrestrial radio-relay systems include power flux-density (pfd), power level at the input to the antenna or the power level at the receiver input. It is worth noting that both methods are in use in Volume IV/IX-2.

A single interference limit value is not adequate because of the time varying nature of the interference. Two limit values, corresponding to a long term (20% of time) and a short term (< 1% of time) have been identified in Report 448 (Düsseldorf, 1990). The exact value of the short-term time percentage is related to the performance objectives for the system under consideration. Study Groups 4 and 9 have developed this method for the specific purpose of sharing between the fixed service and the fixed-satellite service. Further study is needed to determine the extent to which the techniques developed in Study Groups 4 and 9 are applicable to the other cases. Table 3 lists the references relating to sharing between the fixed service and the fixed-satellite service, concerning interference into the fixed service.

Report 1187 (Düsseldorf, 1990) lays the foundations for the apportionment of the performance and availability objectives, from which the long-term interference limit can be calculated. In the case of Rayleigh fading, it can be shown that if the aggregate level of interference is no higher than 10 dB below the receiver noise floor, the performance degradation will not exceed 10%.

TABLE  3

Reports (Düsseldorf, 1990) and Recommendations relating to frequency
sharing between the fixed service and the fixed-satellite service

| Analogue | Digital | General |
| --- | --- | --- |
| Rec. 357 | Rep. 877 | Rec. 355 |
|  | Rec. 615 | Rep. 448 |
|  | Rep. 876 (> 40 GHz) |  |

Any temporal characteristics of exposure of the fixed service to interference will also need to be taken into account. Report 1197 (Düsseldorf, 1990), dealing with sharing with the near-Earth space research systems around 2 GHz, provides useful information on this subject.

The derivation of permitted short-term interference levels, and associated time percentages, is a complex process which includes careful examination of performance/availability objectives, and assumptions about the fading characteristics and correlation of periods of wanted signal fading and interference enhancement. The procedures described in the Volume IV/IX-2 texts and the principles described in this Annex 1 should be developed for this purpose and the tables expanded to include this important information.

# 5.	Calculation of actual interference levels

To complete the analysis of sharing, the probability of interference arriving at the input to the antenna must be evaluated. This will take into account up-to-date propagation models and path factors, which are described in the reports and Recommendations of Study Group 5. It is unlikely that a single model will suffice for all possible applications. The transmission loss calculation will also include factors such as absorption losses, diffraction losses, scattering loss, polarization coupling loss, aperture-to-medium coupling loss and the effect of multipath. Also, both aggregate and single-entry interference levels may need to be considered.

ANNEX  2

Fixed service system parameters for frequency sharing

# 1.	Introduction

In order to calculate degradations in performance and availability, it is necessary to know the characteristics of the radio-relay system being degraded. There is a large variety of radio-relay systems in operation or being developed to meet future requirements. It would be unwise, therefore, to use a single "typical" radio-relay system as a general purpose model. This Annex provides details of the key radio system parameters required for interference evaluation and calculations for frequency sharing with other services.

# 2.	Transmitter characterization

The basic transmitter parameters needed to assess interference potential to other services are:

–	carrier frequency,

–	spectral characteristics,

–	e.i.r.p.,

–	antenna radiation pattern.

Operating frequencies normally correspond to standard CCIR channel plans. The modulation type and system capacity will give a guide to the spectral characteristics of the emissions. However, detailed sharing calculations would require a template of the spectral characteristics to be specified so that any frequency offset rejection at a given wanted/interfering signal carrier frequency separation may be calculated.

The e.i.r.p. of the transmitter is calculated from the transmitter power, feeder/multiplexer losses and antenna gain. A maximum e.i.r.p. value would correspond to maximum antenna gain, minimum feeder/multiplexer losses and maximum transmitter output power, which represents the worst interference potential to other services.

Knowledge of antenna radiation patterns is essential to perform detailed sharing studies. Recommendation 699 should be used to obtain information on fixed service antenna radiation pattern envelopes in cases where measured patterns are not available.

# 3.	Receiver characterization

## 3.1	Equipment parameters

Assessment of the effects of interference into the fixed service from other services requires knowledge of the performance characteristics of the radio receiver. The following receiver parameters are important for frequency sharing studies:

–	noise figure,

–	IF bandwidth,

–	receiver thermal noise,

–	received signal power for 10–3, 10–6, 10–10 BER (digital systems),

–	nominal receiver input level.

The received signal levels and interference levels could be referenced to the LNA/mixer input of the receiver, so that they would be independent of receive antenna gain and feeder/multiplexer losses (assuming this to be the same for both transmitter/receiver).

It should also be noted that accurate sharing calculations require information on the frequency selectivity of the radio equipment.

The required signal levels for given BERs could be combined with the calculated receiver thermal noise level to obtain the required carrier-to-thermal noise ratio C/N for a given BER.

## 3.2	Permitted interference

It is necessary to specify maximum interference levels for both long- and short-term time percentages. For long-term interference, a time percentage of 20% is commonly used. Where aggregate long-term interference is specified, if interference from multiple sources can simultaneously occur, it should be noted that single entry interference criteria will be correspondingly lower. In the case of short-term interference, the time percentages of interest will be related to the system performance objectives.

The long- and short-term interference levels, and associated time percentages, must be individually derived for each system type in accordance with the principles described in Annex 1.

# Digital systems

For digital receivers, it is the total interference power falling within the receiver bandwidth that is generally of most relevance. For convenience, the equivalent power spectral density (PSD) in dB(W/MHz) can also be specified.

# Analogue telephony (FM-FDM)

In analogue systems, the presence of long-term interference will lead to permanent increased noise levels in the demodulated baseband signal. The effect of RF interference on a given telephone channel in the FM-FDM multiplex after demodulation will depend on the nature of, and frequency separation between, the wanted and interfering carrier. This interference must be considered over a bandwidth similar to that of a single baseband channel. Thus, a reference bandwidth of 4 kHz is generally of interest when specifying maximum interference levels for analogue FM-FDM systems.

# 4.	Tables of system parameters

A table can be constructed showing system parameters to be used when considering sharing between the fixed service and other services, and this should include the information discussed above.

Tables 4 to 10 show selected examples of some fixed service systems that are currently in use in some of the bands in which fixed services operate. The various radio system types are identified in the tables by modulation type and system capacity.

The nominal long-term interference criteria specified in the tables provide some guidance to the results that would be obtained from detailed calculations and can be used for information for the time being. However, for detailed sharing studies accurate criteria must be derived in accordance with the information in Annex 1, and these may differ slightly from those in the table.

It is most important that the following Notes be taken into account when considering the example tables.

Note 1 – To simplify the tables, only the interfering carrier level corresponding to the 10–3 BER is included. Equally important are the 10–6 and 10–10 BER objectives, used in the evaluation of permissible performance degradation. Typically, the carrier level corresponding to 10–6 BER is around 4 dB lower than that for 10–3 BER; the carrier level difference between the 10–6 and 10–10 BER points is also about 4 dB.

Note 2 – In the example tables a straightforward, but conservative, approach to specifying maximum permitted long-term external interference is used. This was done because the characteristics and spatial distribution of the interference sources are undefined, and it is also impractical to attempt detailed performance and availability predictions for such a large number of systems at this stage.

By referencing interference to the receiver thermal noise level the problem is greatly simplified, since the permitted interference power spectral density so derived will be dependent solely on receiver noise figure and independent of the modulation scheme of the victim system. It may be shown that, independent of the normal received carrier level, the degradation in fade margin with interference set to a given level relative to receiver thermal noise level is as follows:

| Interference level relative to receiver thermal noise (dB) | Resultant degradation in fade margin (dB) |
| --- | --- |
| –6 –10 | 1 0.5 |

Within the tables, the choice of an interference to thermal noise I/N value of –6 dB or –10 dB is selected to match the typical requirements of individual systems. For detailed sharing analyses, the interference criteria must be derived in accordance with Annex 1, to match the individual, specific, sharing scenario under consideration, and will need to be agreed between the parties concerned.

Another approach stated in Note (3) of Tables 4 to 10 can be applied, conforming to the method given in Recommendation 615.

Note 3 – Short-term interference criteria have not been included in the example tables. This information must be derived in accordance with the principles derived in Annex 1. The tables may be updated when this information becomes available, as a result of future detailed studies of sharing with specific services.

| TABLE 4 Fixed service system parameters for frequency sharing Fixed services below 3 GHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (GHz) |  | 0.81-0.96 |  |  |  |  |  |  |  |  |  |
| Modulation |  | PSK | QAM | PSK | QAM | FSK | FSK | FM-FDM | FM-FDM | FM | FM-FDM |
| Capacity |  | 1 ch | 2 ch | 24 ch | 48 ch | Data | Data | 52 | 52 | Prog | 1 ch |
| Channel spacing |  | 25 kHz | 25 kHz | 600 kHz | 12.5 kHz | 12.5 kHz | 2.5-200 kHz | 2.5-200 kHz | 500 kHz | 25 kHz | 2 MHz |
|  |  |  |  | (P-MLTP) | (P-MLTP) |  | (P-MLTP) |  |  |  |  |
| Antenna gain (max.) (dB) |  | 10 | 10 | 0 | 0 | 10 | 0 | 27 | 27 | 18 | 27 |
| Feeder/multiplexer loss (min.) (dB) |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Antenna type |  | Yagi | Yagi | omni | omni | Yagi | omni | Rec. 699 | Rec. 699 | Rec. 699 | Rec. 699 |
| Max Tx output power (dBW) |  | 7 | 7 | 30 | 30 | 7 | 20 | 13 | 13 | 10 | 13 |
| e.i.r.p. (max.) (dBW) |  | 17 | 17 | 30 | 30 | 17 | 20 | 40 | 40 | 38 | 40 |
| Receiver IF bandwidth (MHz) |  | – | – | – | – | – | – | – | – | – | – |
| Receiver noise figure (dB) |  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Receiver thermal noise (dBW) |  | – | – | – | – | – | – | – | – | – | – |
| Nominal Rx input level (dBW) |  | – | – | – | – | – |  | – | – | – | – |
| Rx input level for 10–3 BER (dBW) |  | – | – | – | – | – |  | – | – | – | – |
| Nominal short-term interference (dBW) (% time) |  | – | – | – | – | – | – | – | – | – | – |
| Nominal long-term interference (dBW) |  | – | – | – | – | – | – | – | – | – | – |
| Equivalent power (dB(W/4 kHz)) |  | – | – | – | – | – | – | – | – | – | – |
| Spectral density (dB(W/MHz)) |  | – | – | – | – | – | – | – | – | – | – |
| Refer to notes |  | – | – | – | – | – | – | – | – | – | – |

| TABLE 5 Fixed service system parameters for frequency sharing Fixed services below 3 GHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (GHz) |  | 1.45-1.53 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.67-1.69 |
| Modulation |  | FDM-FM | PSK- 4-PSK | FDM-FM | FDM-FM | 4-PSK | 4-PSK | 16-QAM | 4-PSK | PCM | MSK | 4-PSK | 4-PSK | 4-PSK |  | FDM-FM |
| Capacity |  | 36 ch | 704 kbit/s 2 Mbit/s | 8 ch | 2 ch | 9.6 kbit/s | 64 kbit/s | 64 kbit/s | 144 kbit/s | – | 2 Mbit/s | 2 ´ 2 Mbit/s | 2 ´ 2 Mbit/s |  |  | 2 ch |
| Channel spacing (MHz) |  | 0.5 | 1 | 0.2 | 0.05 | 0.025 | 0.075 | 0.0375 | 0.225 | 0.5 | 2 | 2 | 4 | 3.5 |  | 0.05 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | CS | OS |  |
| Antenna gain (max.) (dB) |  | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 33 | 16 | 16 | 16 | 17 | 27 | 33 |
| Feeder/multiplexer loss (min.) (dB) |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 5 | 5 | 5 | 0 | 0 | 1 |
| Antenna type |  | dish | dish | Yagi | Yagi | Yagi/ dish | Yagi/ dish | Yagi/ dish | Yagi/ dish | Yagi/ dish |  | Yagi | Yagi | omni/ section | dish/ horn | Yagi |
| Max Tx output power (dBW) |  | 7 | 7 | 7 | 10 | 7 | 7 | 7 | 7 | 10 | 7 | 7 | 7 | 7 | 7 | 10 |
| e.i.r.p. (max.) (dBW) |  | 39 | 39 | 39 | 42 | 39 | 39 | 39 | 39 | 42 | 20 | 20 | 20 | 24 | 34 | 42 |
| Receiver IF bandwidth (MHz) |  | 0.4 | 0.7 | 0.3 | 0.04 | 0.005 | 0.032 | 0.016 | 0.072 | 0.35 | 1.2 | 1.2 | 3 | 3.5 | 3.5 | 0.04 |
| Receiver noise figure (dB) |  | 8 | 4.5 | 8 | 8 | 4 | 4 | 4 | 4 | 8 | 4 | 4 | 4 | 3.5 | 3.5 | 8 |
| Receiver thermal noise (dBW) |  | –140 | –141 | –141 | –149 | –163 | –155 | –158 | –151 | –141 | –139 | –139 | –145 | –135 | –135 | –149 |
| Nominal Rx input level (dBW) |  | –80 | –90 | –93 | –105 | –118 | –112 | –103.5 | –106 | –90 | –126 | –124 | –123 |  |  | –105 |
| Rx input level for 10–3 BER (dBW) |  | N/A | –120 | N/A | N/A | –143 | –137 | –128.5 | –131 | – | –86 | –84 | –83 |  |  | N/A |
| Nominal short-term interference (dBW) (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long-term interference (dBW) |  | –150 | –151 | –151 | –158 | –173 | –167 | –168 | –161 | –151 | –145 | –145 | –141 | –145 | –145 | –159 |
| Equivalent power (dB(W/4 kHz)) |  | –170 | – | –170 | –169 | – | – | – | – | – |  |  |  | –174 | –174 |  |
| Spectral density (dB(W/MHz)) |  | – | –149 | – | – | –150 | –152 | –150 | –150 | –146 | –146 | –146 | –146 |  |  | –169 |
| Refer to notes |  | (2), (5) | (2), (4) | (2), (5) | (2), (5) | (2), (4) | (2), (4) | (2), (4) | (2), (4) | (2), (4) |  | (1), (4) | (1), (4) |  |  | (2), (5) |

CS:	central station

OS:	out station

N/A:	not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

(3)	Specified interference will have a relative contribution of no more than 10% of total noise.

(4)	The specified interference level is total power within the receiver bandwidth.

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value.

| TABLE 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fixed service system parameters for frequency sharing Fixed services below 3 GHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency band (GHz) |  | 1.7-2.45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | FM-TVOB | FM-FDM | FM-MLT-PT |  | FM-TV |  | FM-FDM |  | FM-FDM tropo. |  | 4-PSK tropo. |  | 4-PSK |  | 4-PSK |  | FM-FDM |  | 4-PSK |  | 4-PSK |  | FM-FDM |  | 4-PSK |  | FM-QAM |  | 9-QRP |  | 0-QPSK |  | 4-PSK |  | 4-PSK |  |  |
| Capacity |  | 625-line PAL | 60-132 ch | 94 ch |  | 625-line PAL |  | 960 ch |  | 72-312 ch |  | 2 Mbit/s |  | 34 Mbit/s |  | 8 Mbit/s |  | 1-6 ch |  | 48 ch |  | 12.6 Mbit/s |  | 600 ch |  | 2 ´ 8 Mbit/s |  | 1 TV + 2 Mbit/s |  | 4 ´ 1.54 Mbit/s |  | 45 Mbit/s |  | 8 Mbit/s |  | 2 ´ 8 Mbit/s |  |  |
| Channel spacing (MHz) |  | Variable | 14/1 | 3.5 |  | 29 |  | 29 |  | Special |  | Special |  | 29 |  | 14 |  | 0.4 |  | 2.5 |  | 28 |  | 28 |  | 14 |  | 3.5 |  | 3.5 |  | 29 |  | 7 |  | 14 |  |  |
|  |  |  |  | CS | OS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna gain (max.) (dB) |  | 25 | 31 | 10 | 19 |  | 34 |  | 34 |  | 49 |  | 45 |  | 31 |  | 30 |  | 25 |  | 29 |  | 30 |  | 35.7 |  | 28 |  | 32 |  | 32 |  | 33 |  | 28 |  | 28 |  |
| Feeder/multiplexer loss (min.) (dB) |  | 0 | 5 | 3 | 3 |  | 5 |  | 3 |  | 2 |  | 2 |  | 1 |  | 3 |  | 3 |  | 6 |  | 3.5 |  | 3.5 |  | 4 |  | 6 |  | 3 |  | 3 |  | 5 |  | 5 |  |
| Antenna type |  | 1.2 m dish | 2.4 m dish | omni | horn |  | 3.7 m dish |  | 3.7 m dish |  | 12 m dish |  | 9 m dish |  | 1.8 m dish |  | 1.2 m dish |  | Yagi |  | dish |  | dish |  | dish |  | 2.4 m dish |  | 3 m dish |  | 3 m dish |  | 3 m dish |  | 1.8 m dish |  | 1.8 m dish |  |
| Max Tx output power (dBW) |  | 7 | 7 | 4 | 4 |  | 10 |  | 7 |  | 28 |  | 30 |  | 3 |  | 0 |  | 10 |  | -9 |  | -10 |  | -5.2 |  | 3 |  | 5 |  | 6 |  | 7 |  | 7 |  | -3 |  |
| e.i.r.p. (max.) (dBW) |  | 32 | 33 | 13 | 22 |  | 39 |  | 38 |  | 75 |  | 73 |  | 34 |  | 30 |  | 32 |  | 14 |  | 16.5 |  | 27 |  | 29 |  | 37 |  | 38 |  | 40 |  | 20 |  | 23 |  |
| Receiver IF bandwidth (MHz) |  | 30 | 2.8 | 2 | 2 |  | 40 |  | 40 |  | 6 |  | 1 |  | 20 |  | 4 |  | 0.15 |  | 1.5 |  | 6.5 |  | 20 |  | 8 |  | 3 |  | 3.5 |  | 29 |  | 3 |  | 4.6 |  |
| Receiver noise figure (dB) |  | 8 | 7 | 9 | 9 |  | 10 |  | 10 |  | 2 |  | 4 |  | 4 |  | 5 |  | 4 |  | 6 |  | 9 |  | 10 |  | 4 |  | 4 |  | 5 |  | 4 |  | 4 |  | 4 |  |
| Receiver thermal noise (dBW) |  | -121 | -133 | -132 | -132 |  | -118 |  | -118 |  | -132 |  | -140 |  | -127 |  | -133 |  | -148 |  | – |  | – |  | – |  | -131 |  | -135 |  | -133 |  | -125 |  | -135 |  | -133 |  |
| Nominal Rx input level (dBW) |  | -65 | -79 | -97 | -97 |  | -68 |  | -64 |  | – |  | – |  | -73 |  | -78 |  | -78 |  | -78 |  | -88.3 |  | -78 |  |  |  |  |  | -70 |  | -75 |  | -83 |  | -83 |  |
| Rx input level for 10–3 BER (dBW) |  | N/A | N/A | N/A | N/A |  | N/A |  | N/A |  | N/A |  | -131 |  | -113 |  | -118 |  | N/A |  | – |  | – |  | N/A |  |  |  |  |  | -117 |  | -112 |  | -123 |  | -123 |  |
| Nominal short-term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long-term interference (dBW) |  | -127 | -143 | -142 | -142 |  | -128 |  | -129 |  | -138 |  | -146 |  | -137 |  | -143 |  | -158 |  |  |  |  |  |  |  | -137 |  | -141 |  | -139 |  | -135 |  | -141 |  | -139 |  |
| Equivalent power (dB(W/4 kHz)) |  | – | -172 | -169 | -169 |  | – |  | -169 |  | -172 |  | – |  | – |  | – |  | -174 |  |  |  |  |  |  |  | -170 |  | -170 |  | -168 |  | -174 |  | – |  | – |  |
| Spectral density (dB(W/MHz)) |  | -142 | – | – | – |  | -144 |  | – |  | – |  | -146 |  | -150 |  | -149 |  | – |  | – |  |  |  |  |  | -146 |  | – |  |  |  |  |  | -146 |  | -146 |  |
| Refer to notes |  | (1), (4) | (2), (5) | (2), (5) | (2), (5) |  | (2), (4) |  | (2), (5) |  | (1), (5) |  | (1), (4) |  | (2), (4) |  | (2), (4) |  | (2), (5) |  | (3), (4) |  | (3), (4) |  | (3), (5) |  | (1), (4) |  |  |  | (1) |  | (1) |  | (1) |  | (1) |  |

TVOB:	temporary TV outside broadcast (ENG) link

CS:		central station

OS:		out station

N/A:		not applicable

(1)		Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

(2)		Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

(3)		Specified interference will have a relative contribution of no more than 10% of total noise.

(4)		The specified interference level is total power within the receiver bandwidth.

(5)		The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value.

| TABLE 7 Fixed service system parameters for frequency sharing Fixed services below 3 GHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency band (GHz) |  | 1.7-2.45 |  |  |  |  |  |  | 2.45-2.69 |  |  |  |  |  |  |  |  |  |
| Modulation |  | 4-PSK |  |  | 64-QAM |  | 256-QAM |  | FM-FDM tropo. |  | MSK |  | 4-PSK |  | 4-PSK |  | FM-TVOB |  |
| Capacity |  |  |  |  | 45 Mbit/s |  | 18.5 Mbit/s |  | 72-312 Ch |  | 2 ´ 2 Mbit/s |  | 34 Mbit/s |  | 2.3 Mbit/s |  | 625-line PAL |  |
| Channel spacing (MHz) |  | 3.5 |  |  | 10 |  | 3.5 |  | Special |  | 14 |  |  |  |  |  | Variable |  |
|  |  | CS | OS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna gain (max.) (dB) |  | 17 | 27 | 33 |  | 33 |  | 49 |  | 25 |  | 35.4 |  |  |  | 18 |  |  |
| Feeder/multiplexer loss (min.) (dB) |  | 0 | 0 | 2 |  | 2 |  | 2 |  | 4 |  |  |  |  |  | 1 |  |  |
| Antenna type |  | omni/section | dish/horn | dish |  | dish |  | 12 m dish |  | 1.2 m dish |  | 3 m dish |  | Yagi |  | dish |  |  |
| Max Tx output power (dBW) |  | 7 | 7 | 1 |  | –1 |  | 28 |  | 5 |  | –2 |  |  |  | 7 |  |  |
| e.i.r.p. (max.) (dBW) |  | 24 | 34 | 34 |  | 32 |  | 75 |  | 26 |  | 33 |  |  |  | 32 |  |  |
| Receiver IF bandwidth (MHz) |  | 3.5 | 3.5 | 10 |  | 3.5 |  | 6 |  | 3 |  |  |  |  |  | 30 |  |  |
| Receiver noise figure (dB) |  | 3.5 | 3.5 | 4 |  | 4 |  | 2 |  | 4 |  |  |  |  |  | 6 |  |  |
| Receiver thermal noise (dBW) |  | –135 | –135 | –130 |  | –134.5 |  | –132 |  | –135 |  |  |  |  |  | –123 |  |  |
| Nominal Rx input level (dBW) |  | – | – | –65 |  | –65 |  | –65 |  | – |  |  |  |  |  | –55 |  |  |
| Rx input level for 10–3 BER (dBW) |  | – | – | –106 |  | –104.5 |  | N/A |  | – |  |  |  |  |  | N/A |  |  |
| Nominal short-term interference (dBW) (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long-term interference (dBW) |  | –141 | –141 | –136 |  | –140.5 |  | –138 |  | –141 |  | –111.5 |  |  |  | –123 |  |  |
| Equivalent power (dB(W/4 kHz)) |  | –170 | –170 | –170 |  | –170 |  | –172 |  | –170 |  |  |  |  |  | – |  |  |
| Spectral density (dB(W/MHz)) |  | – | – | –146 |  | –146 |  | – |  | –162 |  |  |  |  |  | –129 |  |  |
| Refer to notes |  | (1) | (1) |  |  |  |  | (1), (5) |  |  |  |  |  |  |  |  |  |  |

CS:	central station

OS:	out station

N/A:	not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

(3)	Specified interference will have a relative contribution of no more than 10% of total noise.

(4)	The specified interference level is total power within the receiver bandwidth.

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value.

| TABLE 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fixed service system parameters for frequency sharing Fixed services above 10 GHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Frequency band (GHz) |  | 10.7-11.7 |  |  | 12.2-12.44 |  | 13/14 |  |  |  |  | 14.4-15.35 |  |  | 17.7-19.7 |  |  |  |  |  |  |
|  | Modulation |  | 4-PSK | FM-FDM | FM-TV | 4-PSK | 16-QAM | 4-PSK | 4-PSK | 4-PSK | 4-PSK | FM | 64-QAM | FM-FDM | 8-PSK | 4-PSK | 4-QAM | 4-FSK | 4-PSK | 2-PSK | 4-PSK | 0-QPSK |
|  | Capacity |  | 140 Mbit/s | 960 ch | 625-line PAL | 13.9 Mbit/s | 50.4 Mbit/s | 2 Mbit/s | 8 Mbit/s | 16 Mbit/s | 34 Mbit/s | 1 Video | 140 Mbit/s | 2 700 ch | 156 Mbit/s | 140 Mbit/s | 140 Mbit/s | 8 Mbit/s | 8 Mbit/s | 8 Mbit/s | 34 Mbit/s | 44.7 Mbit/s |
|  | Channel spacing (MHz) |  | 67 | 40 | 40 | 20 | 20 | 3.5 | 7 | 14 | 28 | 28 | 28 | 40 | 40 | 110 | 55 | 20 | 20 | 20 | 27.5 | 40 |
|  | Antenna gain (max.) (dB) |  | 49 | 47 | 47 | 50 | 50 | 49 | 49 | 49 | 49 | 49 | 49 | 52 | 52 | 48 | 48 | 45 | 45 | 45 | 45 | 45 |
|  | Feeder/multiplexer loss (min.) (dB) |  | 5 | 5 | 5 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 2 | 5 | 5 | 7 | 7 | 0 | 0 | 0 | 0 | 3 |
|  | Antenna type |  | 3.7 m dish | 2.5 m dish | 2.5 m dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
|  | Max Tx output power (dBW) |  | 10 | 10 | 10 | –5 | –5 | 10 | 10 | 10 | 10 | 10 | 5 | 3 | 0 | –10 | –4 | –16 | –6 | –9 | –8 | –9 |
|  | e.i.r.p. (max.) (dBW) |  | 54 | 52 | 52 | 40 | 40 | 45 | 45 | 45 | 45 | 45 | 47 | 50 | 47 | 31 | 37 | 29 | 39 | 27 | 37 | 33 |
|  | Receiver IF bandwidth (MHz) |  | 68 | 29 | 29 | 12.3 | 17.2 | 1 | 2 | 4 | 17 | 24 | 40 | 56 | 50 | 68 | 68 | 8 | 4 | 8 | 18 | 40 |
|  | Receiver noise figure (dB) |  | 7 | 7 | 8 | 7 | 5 | 10 | 10 | 10 | 10 | 10 | 4 | 10 | 5 | 7 | 8 | 13 | 7 | 7 | 7 | 5 |
|  | Receiver thermal noise (dBW) |  | –119 | –121 | –121 | – |  | –134 | –131 | –128 | –122 | –120 | –124 |  |  | –119 | –118 | –122 | –131 | –128 | –124 | –125 |
|  | Nominal Rx input level (dBW) |  | –62 | –65 | –65 | –59 + M | –59 + M | –74 | –71 | –68 | –65 | –65 | –66 | –48 | –44 | –63 | –64 | –65 | –65 | –65 | –65 | –70 |
|  | Rx input level for 10–3 BER (dBW) |  | –104 | N/A | N/A |  |  | –116 | –113 | –111 | –109 | N/A | –101 |  |  | –103 | –104 | –106 | –116 | –116 | –113 | –106 |
|  | Nominal short-term (dBW) interference (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Nominal long-term interference (dBW) |  | –129 | –131 | –131 |  |  | –144 | –141 | –138 | –132 | –130 | –134 |  |  | –129 | –131 | –132 | –141 | –138 | –143 | –131 |
|  | Equivalent power (dB(W/4 kHz)) |  | – | –170 | – |  |  | – | – | – | – | – | – |  |  | – | – | – | – | – |  | –171 |
|  | Spectral density (dB(W/MHz)) |  | –147 | – | –146 |  |  | –144 | –144 | –144 | –144 | –144 | –150 |  |  | –147 | –149 | –141 | –147 | –147 |  |  |
|  | Refer to notes |  | (2), (4) | (2), (5) | (2), (4) | (3) | (3) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) |  |  | (2), (4) | (2), (4) | (2), (4) | (2), (4) | (2), (4) | (2), (4) | (1) |

M:	fade margin

N/A:	not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

(3)	Specified interference will have a relative contribution of no more than 10% of total noise.

(4)	The specified interference level is total power within the receiver bandwidth.

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value.

| TABLE 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fixed service system parameters for frequency sharing Fixed services above 10 GHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency band (GHz) |  | 21.12-23.6 |  |  |  |  |  |  |  |  |  |  | 25.25-27 |  |  |  |  |
| Modulation |  | 2-FSK | 2-FSK | 2-FSK | 4-PSK | 4-PSK | FM | 4-PSK | ASK | ASK | 2-FSK | 64-QAM |  | FSK | DFSK | FSK |  |
| Capacity |  | 2 Mbit/s | 4 Mbit/s | 8 Mbit/s | 34 Mbit/s | 140 Mbit/s | 1 Video | 34 Mbit/s | 2 Mbit/s | 4 ´ 2 Mbit/s | 2 Mbit/s | 140 Mbit/s |  | 6 Mbit/s |  | 8 Mbit/s |  |
| Channel spacing (MHz) |  | 7 | 7 | 14 | 28 | 112 | 28 | 28 | 28 | 28 | 5 | 11140,5 |  | 40 |  | 20 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | CS | OS | CS |  |
| Antenna gain (max.) (dB) |  | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 35 | 50 | 47 | 11138.5 |  | 20 | 47 | 47 |  |
| Feeder/multiplexer loss (min.) (dB) |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 4 | 0 | –123 |  | 0 | 0 | 0 |  |
| Antenna type |  | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |  | 90° section | dish | dish |  |
| Max Tx output power (dBW) |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | –16 | –14 | –10 | 111–4,5 |  | –8 | –10 | –10 |  |
| e.i.r.p. (max.) (dBW) |  | 50 | 50 | 50 | 50 | 50 | 50 | 47 | 15 | 32 | 37 | 11131.5 |  | 10 | 37 | 37 |  |
| Receiver IF bandwidth (MHz) |  | 2 | 4 | 8 | 17 | 70 | 24 | 18 | 5 | 14 | 2 | 12140,5 |  | 16.4 | 16.4 | 16.4 |  |
| Receiver noise figure (dB) |  | 9 | 9 | 9 | 9 | 9 | 9 | 12 | 4 | 4 | 11 | 31225,5 |  | 10 | 8 | 10 |  |
| Receiver thermal noise (dBW) |  | –132 | –129 | –126 | –123 | –116 | –121 | –119 | –133 | –128 |  | –123 |  |  |  |  |  |
| Nominal Rx input level (dBW) |  | –105 + M | –104 + M | –103 + M | –100 + M | –94 + M | –84 + M | –87 | –108 + M | –109 + M | –115 | –73 |  | –99 + M | –123 + M | –99 + M |  |
| Rx input level for 10–3 BER (dBW) |  | –108 | – | –106 | –103 | –97 | N/A | –103 | –112 | –113 |  | –96 |  |  |  |  |  |
| Nominal short-term interference (dBW) (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long-term interference (dBW) |  | –142 | –139 | –136 | –133 | –126 | –131 | –129 | –139 | –136 |  | –131 |  |  |  |  |  |
| Equivalent power (dB(W/4 kHz)) |  | – | –170 | – | – | – | – | – | – | – |  | –171 |  |  |  |  |  |
| Spectral density (dB(W/MHz)) |  | –143 | –143 | –143 | –143 | –143 | –143 | –141 | –146 | –148 |  | –147 |  |  |  |  |  |
| Refer to notes |  | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (3), (4) |  |  | (3), (4) |  | (3), (4) |  |

M:	fade margin

N/A:	not applicable

(1)		Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

(2)		Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

(3)		Specified interference will have a relative contribution of no more than 10% of total noise.

(4)		The specified interference level is total power within the receiver bandwidth.

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value.

| TABLE 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fixed service system parameters for frequency sharing 30-60 GHz fixed services |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Frequency band (GHz) |  | 37-39.5 |  |  |  |  |  | 54.25-57.2 |  |  |  |  |  |
| Modulation |  | 2-FSK | 2-FSK | 4-PSK | 4-PSK | FM | FM | 2-FSK | 2-FSK | 4-PSK | 4-PSK | FM | FM |
| Capacity |  | 2 Mbit/s | 8 Mbit/s | 34 Mbit/s | 140 Mbit/s | 1 Video | 1 Video | 2 Mbit/s | 8 Mbit/s | 34 Mbit/s | 140 Mbit/s | 1 Video | 1 Video |
| Channel spacing (MHz) |  | 7 | 14 | 28 | 140 | 28 | 56 | 14 | 14 | 28 | 140 | 28 | 56 |
| Antenna gain (max.) (dB) |  | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 |
| Feeder/multiplexer loss (min.) (dB) |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Antenna type |  | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish | dish |
| Max Tx output power (dBW) |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| e.i.r.p. (max.) (dBW) |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| Receiver IF bandwidth (MHz) |  | 2 | 8 | 17 | 70 | 16 | 40 | 2 | 8 | 17 | 70 | 16 | 40 |
| Receiver noise figure (dB) |  | 11 | 11 | 11 | 11 | 12 | 12 | 11 | 11 | 11 | 11 | 12 | 12 |
| Receiver thermal noise (dBW) |  | –130 | –124 | –121 | –114 | –120 | –116 | –130 | –124 | –121 | –114 | –120 | –116 |
| Nominal Rx input level (dBW) |  | –108 + M | –102 + M | –99 + M | –93 + M | –98 + M | –85 + M | –108 + M | –102 + M | –99 + M | –93 + M | –98 + M | –85 + M |
| Rx input level for 10–3 BER (dBW) |  | –111 | –105 | –102 | –95 | N/A | N/A | –111 | –105 | –102 | –95 | N/A | N/A |
| Nominal short-term interference (dBW) (% time) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal long-term interference (dBW) |  | –140 | –134 | –131 | –124 | –130 | –126 | –140 | –134 | –131 | –124 | –130 | –126 |
| Equivalent power (dB(W/4 kHz)) |  | – | – | – |  | – | – | – | – | – |  | – | – |
| Spectral density (dB(W/MHz)) |  | –143 | –143 | –143 | –143 | –142 | –142 | –143 | –143 | –143 | –143 | –142 | –142 |
| Refer to notes |  | (1), (4) | (1), (4) | (1), (4) |  | (1), (4) | (1), (4) | (1), (4) | (1), (4) | (1), (4) |  | (1), (4) | (1), (4) |

M:	fade margin

N/A:	not applicable

(1)	Specified interference will reduce system C/N by 1 dB (interference 6 dB below receiver thermal noise floor).

(2)	Specified interference will reduce system C/N by 0.5 dB (interference 10 dB below receiver thermal noise floor).

(3)	Specified interference will have a relative contribution of no more than 10% of total noise.

(4)	The specified interference level is total power within the receiver bandwidth.

(5)	The specified interference level should be divided by the receiver bandwidth to obtain an average spectral density. The interference spectral density, averaged over any 4 kHz within the receiver bandwidth, must not exceed this value.

_________________