RECOMMENDATION  ITU-R  M.1033

TECHNICAL  AND  OPERATIONAL  CHARACTERISTICS  OF  CORDLESS  TELEPHONES
AND  CORDLESS  TELECOMMUNICATION  SYSTEMS

(Question ITU-R 114/8)

(1994)

Rec. ITU-R M.1033

The ITU Radiocommunication Assembly,

considering

a)	that new land mobile services, such as cordless telephones and cordless telecommunication systems providing communications in areas limited to a few hundred metres radius are becoming available to a large public;

b)	that in the longer term, Future Public Land Mobile Telecommunication Systems (FPLMTS) Recommendations will encompass future needs supported by digital cordless systems; however, in the interim, a Recommendation should include digital cordless systems;

c)	that these services can use consumer-type devices which may create difficulties to administrations, and may be misused;

d)	that the utilization of the radio spectrum should be as economical as possible and that the use of multi-channel access techniques conserves frequency spectrum;

e)	that highly flexible and economical systems can be achieved without using a central controller for setting up control of the radio path;

f)	that the widespread and increasing use of these equipments and the characteristics of their utilization may create operational problems;

g)	that certain administrations are encountering problems of mutual interference and inadvertent interaction between “cordless telephone” systems;

h)	that a number of systems have already been adopted by a wide range of administrations and can form the basis for a Recommendation,

recommends

that cordless telephones and cordless telecommunication systems (cordless systems) should conform to the following objectives and technical and operational characteristics:

# 1.	General objectives

Cordless systems should be designed with the following minimum basic objectives:

–	that the radio spectrum be used efficiently;

–	that a system of high subscriber capacity be realized;

–	that simple and miniature circuits be used to ensure that the weight and size of the equipment are comparable with ordinary telephones;

–	that it can be provided economically and the cost of equipment can be reduced to be acceptable for a mass market;

–	that the system provide good quality for public communication and that a flexible system of operation can be provided that does not require complicated frequency management;

–	that the system provide as far as possible normal telephone features;

–	that the system provide security of call charges.

# 2.	Operational and management objectives

Cordless systems should allow operation and management procedures which ensure that:

–	in system operation, it is only necessary to manage the number of cordless systems within the area determined by the distance at which interference occurs, while taking into account the subscriber capacity which is determined by the number of channels and the channel traffic;

–	flexible operation can be provided, so as to permit the use of a greater number of portable sets than the subscriber capacity per zone, if the number of cordless sets in use in the adjacent zone is small.

# 3.	Access techniques

Cordless systems should include the use of multi-channel techniques and appropriate protocols which do not require the use of a central controller for the detection of idle channels.

These techniques, also called self-trunking or dynamic channel selection, may employ FDMA-FDD, FDMA-TDD, as well as multi-carrier TDMA-TDD. Depending on the application the access technique should allow also in-call handover.

# 4.	Features and areas of application

Since the start of cordless systems there has been an evolution in the areas of application which are covered. The services supported by cordless systems can therefore differ. Also, the service requirements can differ depending on the market segment to be served, for example, public, residential, business, etc. Therefore the importance of the different features of a cordless system depends on its application.

To facilitate the characterization of different cordless systems, the following description of features and applications should be used.

## 4.1	Cordless telephony (residential)

Cordless telephony in its basic form is provided by a cordless telephone, which consists in its simplest version of two parts:

–	a fixed part which is connected to a subscriber line of the ordinary telephone network, and

–	a portable set.

Each part uses multi-channel access techniques and can individually perform the following operations:

–	search for idle channels;

–	set up speech paths using the selected channel;

–	check identification codes in the signals between the fixed part and the portable set in order to ensure that only associated units will lock to each other.

More advanced installations allow the use of more portable sets.

## 4.2	Cordless PABX

In addition to basic cordless telephony, single cell or multi-cell systems can serve the needs of small business or large business respectively for cordless extensions and on-premises communications networks.

## 4.3	Seamless handover

Seamless handover, which is an optional feature, allows systems to provide uninterrupted communications throughout the coverage area while the call is in progress. Handover may occur within a single cell or between adjacent cells.

## 4.4	Cordless Local Area Networks (CLAN)

Cordless LANs or more general non-voice services, including those available in the ISDN, can be offered by fully digital systems. They are the replacement of a drop-cable connection from terminals to an ISDN or LAN backbone in order to support the “wireless office” concept.

## 4.5	Telepoint

In this application the system offers the portable set access to public base stations via a fully standardized air interface, including the necessary protocols for call set-up and authentication which should guarantee billing security. These systems may provide roaming, including international roaming.

## 4.6	Local loop replacement

In this application the system provides a replacement of the local loop (extension of the PSTN, ISDN into the customers’ premises) by radio. Smooth billing should be guaranteed by secure authentication and the system should be capable of serving a large number of users.

## 4.7	Tandeming to cellular radio

The flexibility and high quality of cordless communications offers a means of enhancing other mobile services by cordless extensions (e.g. telepoint in trains, handhelds connected to a cellular mobile).

# 5.	Existing systems

Cordless systems employing dynamic multiple access techniques and supporting varying subsets of the possible features and areas of application have been developed in many countries. Annex 1 provides information about existing systems.

ANNEX  1

Systems features and applications

This Annex gives information about seven existing cordless systems.

Table 1 shows the features and applications which can be supported by the different systems using the categories described in this Recommendation.

The major specifications of these systems are summarized in Tables 2 and 3 for analogue and digital systems respectively.

The main features of the seven systems and the references to the detailed specifications are described in Appendices 1 to 7.

TABLE  1

|  | System No. |
| --- | --- |

| Features and applications | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Cordless telephony (residential) | ´ | ´ | ´ | ´ | ´ | ´ | ´ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cordless PABX | ´ |  | ´ | ´ | ´ | ´ | ´ |
| Seamless handover |  |  |  | ´ | ´ | ´ | ´ |
| Cordless Local Area Network |  |  | ´ | ´ | ´ | ´ | ´ |
| Telepoint |  |  | ´ | ´ | ´ | ´ | ´ |
| Local loop replacement |  |  | ´ | ´ | ´ | ´ | ´ |
| Tandeming to cellular radio |  |  |  |  | ´ | ´ |  |
| Note 1 – ´ means that a particular feature or application can be supported by the system. |  |  |  |  |  |  |  |

TABLE  2

System characteristics of analogue systems

|  | System 1(1) Japan, 1988 | System 2(2) CEPT, 1987 |
| --- | --- | --- |

| Class of emission | F1D, F2 (A, B, C, D, N, X), F3E | F3E or G3E |
| --- | --- | --- |
| Multiple access scheme | FDMA | FDMA |
| Duplex type | FDD | FDD |
| Transmit frequency band (MHz) – fixed part – portable set | 380.2-381.3 253.9-255.0 | 959-960 914-915 |
| Radio frequency channel spacing (kHz) | 12.5 | 25 |
| Number of speech channels | 87 (adding two control channels) | 40 |
| Transmitted power, e.r.p. (mW) – fixed part – portable set | Maximum 10(3) Maximum 10(3) | Maximum 10 Maximum 10 |
| Typical service range (m) – indoor – outdoor | 50 100 | 50 100 |
| Voice signals – type of modulation – processing | Angular | Angular syllabic compandor ITU-T Rec. G.162 (suggested) |
| Audio signal-to-noise ratio (dB) | Minimum SINAD 20(4) | Minimum SINAD 20(5) |
| Identification code | 3 ´ 107 combinations | 1 ´ 104 combinations |

FDD: frequency division duplex (1) System 1 is in use in Japan.(2) System 2 is used by several European countries. (3) Antenna input power. (4) Suggested value using an IEC Publication 489-3 standard input signal level of 10 mV. (5) Ratio measured through psophometric weighting network (according to ITU-T Recommendation P.53 (same as O.41) using an input signal field strength of 30 dBmV/m.

TABLE  3

System characteristics of digital systems*

|  | System 3(1) ETSI, 1992 | System 4(2) Sweden, 1989 | System 5(3) ETSI, 1992 | System 6(4) Japan, 1992 | System 7(5) Canada, 1993 |
| --- | --- | --- | --- | --- | --- |

| Class of emission | F1EJT or G1EJT | F1W and F7W | F1W and F7W | G1W and G7W | F1EJT or G1EJT |
| --- | --- | --- | --- | --- | --- |
| Multiple access scheme | FDMA | Multi-carrier TDMA | Multi-carrier TDMA | Multi-carrier TDMA | FDMA |
| Duplex type | TDD | TDD | TDD | TDD | TDD |
| Frequency band (MHz) | 864.1-868.1 | 862-866 | 1 880-1 900 | »1 900 | 944.0-948.5 |
| Radio frequency channel spacing (kHz) | 100 | 1 000 | 1 728 | 300 | 100 |
| Gross bit rate per carrier (kbit/s) | 72 | 640 | 1 152 | 384 | 72 |
| Number of speech channels | 40 | 8 (per carrier) | 12 (per carrier) | 4 (per carrier) | 40 (with 60 signalling/ control channels) |
| Transmitted power, e.r.p. (mW) |  | Peak power over time-slot | Peak power over time-slot |  |  |
| – fixed part – portable set | Maximum 10 Maximum 10 | Maximum 100 Maximum 100 Average < 6(8) | Maximum 250 Maximum 250 | Maximum 10(6) (7) Maximum(6) | Maximum 10 Maximum 10 |
| Typical service range (m) – indoor – outdor | 40 200 | 40 200 | 30 200 | 50 200 | 40 200 |
| Voice signals – type of modulation – processing | 2 level FSK, Gaussian shaped ADPCM | GFSK ADPCM or CVSDM | GFSK ADPCM or CVSDM | p/4 QPSK ADPCM | 2 level FSK, Gaussian shaped ADPCM (ITU-T Rec. G.726) |
| Identification code | 1.5 ´ 106 combinations | >107 combinations | >107 combinations | >108 combinations | 1.5 ´ 106 combinations |
| * Most systems indicate as audio quality that the mean opinion score (MOS), when tested subjectively, shall not fall below the MOS of 32 kbit/s ADPCM as specified in ITU-T Recommendation G.726. TDD: time division duplex CVSDM: continuously variable slope delta modulation (1) System 3 is used in the United Kingdom, Finland, France, Belgium, Germany, Italy, Portugal, Spain, The Netherlands, Singapore, Hong Kong, Malaysia and Thailand. International roaming for Telepoint operation may be provided between European countries with the possibility of extending this to the other countries listed. (2) System 4 is used in Sweden, New Zealand, Malaysia, Thailand, Indonesia, Philippines, Hong Kong, Brazil and Saudi Arabia. (3) System 5 is the Digital European Cordless Telecommunications (DECT) system. (4) System 6 is to be used in Japan. (5) System 7 is used in Canada. (6) Antenna input power. (7) Except for outdoor fixed parts. (8) This average power assumes eight speech channels (duplex time-slots). |  |  |  |  |  |

APPENDIX  1

General description of System 1 (analogue)

# 1.	System configuration [RCR Standard, 1988]

The basic configuration of a cordless telephone System 1 employing the analogue multi-channel access technique is shown in Fig. 1.

This system does not use a central controller. In the system, two dedicated control channels are assigned.

Each cordless telephone itself performs the functions of searching for idle channels and setting up a speech path over the selected channel.

Figure 2 shows the configurations of a multi-function cordless telephone.

The following types are typical examples:

–	multi-terminal cordless telephone,

–	cordless key telephone,

–	multi-zone cordless telephone.

FIGURE 1...[D01] = 13 CM

FIGURE 2...[D02] = PAGE PLAINE

# 2.	Operational characteristics

### 2.1	Identification code

2.1.1	Only when its own identification code coincides between the fixed part and the portable set in the control channel, the fixed part and portable set shall switch to a speech channel. The length of the identification code shall be 25 bits.

2.1.2	Modulation methods and bit rates for an identification code are as follows:

Subcarrier MSK (fast FSK): 1 200, 2 400 bit/s

Direct modulation: 600, 1 200, 2 400, 4 800 bit/s

### 2.2	Idle channel decision

Carrier sense shall be performed immediately before transmission and if more than 2 mV is sensed, a radio signal shall not be emitted.

### 2.3	Occupied time of control channels

Considering the common use of dedicated control channels and easy introduction of sleep mode in portable sets, for originating calls from portable sets the occupied time shall be 1.3 s or less. For originating calls from fixed parts the occupied time shall be 4 s or less.

### 2.4	Transmitter power-off on disconnection

When disconnection is initiated, or when a radio signal is not received on any speech channel, the transmitter shall be turned off automatically.

### 2.5	Transmitter power-off in case of failure

When a failure in the radio equipment causing continuous transmitter power in the control channel is found, the transmitter shall be automatically turned off within 60 s.

REFERENCES

RCR Standard [December, 1988] 250 MHz/380 MHz Band Radio Equipment for Cordless Telephone, STD-13. Research & Development Centre for Radio Systems, Japan.

APPENDIX  2

General description of System 2 (analogue)

# 1.	System configuration

The configuration of an analogue cordless telephone of System 2 employing the multiple channel access technique is shown in Fig. 3.

The system configuration that has been adopted does not use a control channel to set up the RF connection between the fixed part and portable sets. Occupancy of an RF channel may be initiated by the fixed part or the portable set by following the same procedure.

# 2.	Call setting-up procedures

The procedures for setting up a call are given in Fig. 4.

FIGURE 3...[D03] = 13 CM

## 2.1	Waiting mode

In this mode, the receiver of each part of the cordless telephone set is constantly scanning the available RF channels, searching for a signal which contains its matching identification code.

## 2.2	Call originating mode

When the need for a radio-frequency channel arises in either of the two parts of a cordless telephone set, that part searches for an idle duplex channel by sensing the field strength on that channel.

On the idle channel, the initiating part starts transmitting its identification code. Upon detection of this code, the receiver stops scanning and initiates its transmitter to occupy the corresponding return frequency of the duplex channel and to transmit its identification code to the initiating part. As soon as the receiver of the initiating part detects its matching identification code on the return frequency of the selected duplex channel, the initiating transmitter stops transmitting the identification signals and the RF channel becomes available for the transmission of dialling tones and speech.

## 2.3	Conversation mode

To ensure continued locking during a call, the identification procedure is periodically repeated.

## 2.4	Call terminating mode

When the RF connection is to be terminated, the part of the cordless telephone set which initiated the termination procedure transmits a proper termination message including the identification code. The RF circuit is immediately disconnected and the cordless telephone set returns to the idle condition.

FIGURE 4a...[D04] = 23 CM

FIGURE 4b...[D05] = 23 CM

BIBLIOGRAPHY

ETSI [1992] Draft prI-ETS 300 235 Technical characteristics, test conditions and methods of measurement for radio aspects of cordless telephones CT1. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

APPENDIX  3

General description of System 3 (digital)

# 1.	System operation

The digital cordless telephone of System 3 operates with digitally modulated, single channel per carrier equipment using time division duplex operation and frequency division multiple access (TDD/FDMA).

The system configuration does not require the use of a control channel to set up the RF link between the fixed part and portable sets. Selection of a suitable RF channel may be initiated by either the fixed part or portable set, the same procedure being followed in each case.

The basic characteristics of this type of digital transmission link are applicable to cordless telephones operated in residential, small business and telepoint services. In telepoint operation the portable sets are able to initiate calls within 100-200 m range of a public base station.

This TDD/FDMA approach is well matched to spectrum availability and network requirements. The 100 kHz channelling has no need for dispersion equalization, permits easing of frequency tolerances towards 1 part in 105 and allows the use of relatively simple filters. There is no necessity for an RF duplexer, less carrier-frequency sources are required and its moderate delay raises no echo control or absolute delay issues. With these attributes, the RF circuitry is simplified and the balance of implementation complexity shifts to baseband which is amenable to very large scale integration.

The above presentation of the digital cordless telephone is as a single base-handset combination. A wide range of situations can be accommodated with a capability of continuous growth.

For example:

–	individual, independent, cordless extensions to a PBX;

–	local concentrations of cordless base units in nests;

–	central PBX-based multi-channel unit using a leaky feeder or discrete, distributed, antennas.

The last two clearly require synchronization between transmit and receive phases of the TDD signals within the nested unit, which is easily arranged. Another consideration is the need to exercise some control of intermodulation between multiple transmissions during transmission and reception. Although the need for intermodulation control is inherent in all such FDMA schemes and special measures may be required in combining RF channels, the effects can be contained without incurring significant cost penalties. The low power levels involved also make this aspect somewhat easier. Since PBX applications involve the provision of the PBX interface and control circuitry, any potential intermodulation control can also be incorporated.

In summary, for the digital transmission link there is a very high degree of common circuitry between handset and base hardware and a high level of commonality across all applications (including telepoint).

A common air interface standard has been developed that assures inter-working of all portable sets and base stations. Telepoint networks using System 3 will be required to implement this standard. This will allow roaming between the networks, so that any portable set will be able to be used with any network for which the user is a registered subscriber.

BIBLIOGRAPHY

ETSI [1992] I-ETS 300 131 Radio Equipment and Systems (RES); Common air interface specification to be used for the interworking between cordless telephone apparatus in the frequency band 864.1 MHz to 868.1 MHz, including public access services. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

APPENDIX  4

General description of System 4 (digital)

# 1.	System configuration

Several configurations of digital cordless telephones of System 4 employing the time division multiple access technique (TDMA) are shown in Fig. 5. The multiple base station case provides a handover facility.

With one single radio transceiver, each fixed station (FS) can simultaneously communicate on all time slots that for each moment are allocated to that base station.

FIGURE 5...[D06] = 12 CM

# 2.	Call set-up procedure

The system configuration uses a time slot allocation which is dynamic and decentralized to each base station and each portable. When a traffic channel (time slot) is wanted, the portable station (PS) communicates with the fixed station (FS) having the highest field strength, and the channel is chosen with the least interference at the position of both the portable and the fixed station. For special services, it is possible to allocate temporarily more than one time slot to a portable station.

# 3.	Interference limited capacity

For System 4 the range can be C/I limited rather than C/N limited. Thus the capacity in an operational area can be increased by denser and denser re-use by installing the fixed stations closer and closer.

# 4.	Dynamic channel allocation and handover

Since all the stations (fixed and portable) have continuous information of the status on all slots due to the TDMA mode of operation, the dynamic channel allocation can be made very efficient and fast. Also, handover within and between fixed stations can be made efficient and fast. Furthermore, simulations have shown that this system is stable under heavy traffic conditions.

# 5.	TDMA frame structure

Adjacent systems with differently specified TDMA frames and time slots can co-exist on the same frequency band as long as the frame cycle times are equal. Time slot synchronization is not needed between systems, provided that the frame cycle time is reasonably constant (±5 ppm). Antenna selection at the fixed station provides diversity gain for both directions.

# 6.	Applicable services

The system concept is applicable to large office, small office, telepoint and residential voice and data services.

BIBLIOGRAPHY

ESPA [September, 1987] Publication 5.2: Business Cordless Telephones. Association of European Manufacturers of Pocket Communications Systems, Breda, the Netherlands.

STA [1989] Specification 8211-A130: Technical requirements for connection of digital cordless telephones to the public-switched telephone network. Swedish Telecom, Stockholm, Sweden.

TVTFS [1989] Regulations on Radio Technical Requirements on Digital Cordless Telephones in the Frequency Band 862-864 MHz. Swedish Telecom Radio, Stockholm, Sweden.

APPENDIX  5

General description of System 5*(digital)

# 1.	System configuration

System 5 employs the TDMA time division duplex technique in such a way that various configurations can share the same spectrum resources. An overview of the System 5 family is given in Fig. 5. One configuration does not have to be synchronized with a neighbouring one, belonging to the same family, in order to operate in the same frequency band.

Proper synchronization though, would increase the capacity of both systems. Each fixed station can communicate simultaneously on all 12 duplex time slots and on any of the carriers, with one single radio transceiver. Handover facilities are provided in multi-base station configurations.

# 2.	Services

The system concept provides for the following voice and data services:

## 2.1	Cordless telephone

DECT will provide this in its basic form, but can also support advanced features like connection to 2 PSTN lines, a system incorporating 2 or 4 handsets, intercom facility via the base station and call transfer between handsets.

## 2.2	Cordless PABX

The DECT system is designed to handle 10 000 E per floor, assuming a GOS of 1% and voice traffic with a peak of 0.2 E per terminal. A multicell system provides for fast in-call handover between cells according to interference conditions.

## 2.3	Cordless Local Area Networks (CLAN)

DECT supports the “wireless office” concept. Terminals, connected to the backbone via the DECT system, can communicate at up to 256 kbit/s.

## 2.4	Telepoint

In its basic version, DECT telepoint supports outgoing calls only. When the infrastructure is prepared for it, extra features can be implemented including incoming calls (local log-on), and in-call handover to an adjacent telepoint base station of outgoing calls queueing for access to the radio interface. Hence, a Personal Communications Network (PCN) can be realized.

## 2.5	Local loop replacement

It is expected that the combination of cheaper digital transmission (via fibre) and larger private switches will result in a systems design wherein the radio part of DECT will be distributed, but the baseband processes, channel control and signalling will be centralized.

## 2.6	Tandeming of cordless systems with mobile radio

The protocols of DECT are designed to allow tandeming with the Pan-European digital cellular system GSM.

# 3.	Grade of service

DECT provides a radio interface access to other networks and is designed to closely match wire-connected terminals to those networks from a point of view of grade of service (GOS). GOS here stands for the overall probability of a call not being set up or, worse still, being dropped during a call, in the busy hours.

Elements, contributing to a call set-up GOS are:

–	the GOS for a PABX alone (in the range 0.1% to 0.01%);

–	congestion in the radio bearer channels;

–	loss of radiocommunication caused by poor radio signal coverage of the nominated communication area or cell.

The sum of the above contributions to the GOS for all applications of DECT shall not exceed 1%.

Dropping of a call will be far less acceptable than failure to establish a call, hence it is recommended that the probability of dropping shall not exceed 0.1% but 0.01% is preferable.

In spite of the above, however, a Telepoint service might have different characteristics due to commercial factors, independent of the DECT specifications.

# 4.	Call set-up procedure

The system configuration uses a time slot allocation which is dynamic and decentralized to each base station and each portable. When a traffic channel (time slot) is wanted, the portable station (PS) communicates with the fixed station (FS) having the highest field strength, and the channel is chosen with the least interference at the position of both the portable and the fixed station. In the case of incoming calls, the base station chooses a free channel and uses its handshake to establish a communication link with the portable, preferably using the chosen channel, if it is free.

This method of dynamic channel allocation is efficient and fast because all (fixed and portable) stations have continuous information of the status on all slots due to the TDMA mode of operation.

The system concept includes the possibility of temporarily allocating more than one time slot to a portable station, in order to allow for special services requiring a high capacity communication channel.

# 5.	Structure of the standard

DECT offers a concept for high capacity microcellular radio systems designed to operate in connection with the fixed PSTN, ISDN or other networks (e.g. office systems).

From the beginning, it was recognized that a highly structured approach to the design of the standard was crucial, due to the complex nature of the task. Also, a number of specialists in various fields would have to work on different aspects of DECT and an adequate structure would be advantageous for the organization of the work.

The OSI layered structure is basic to the DECT standard (Fig. 6).

–	The physical layer (PHL) creates a set of “bit pipes” through the radio medium.

–	The medium access control (MAC) layer chooses a suitable physical channel and prepares the information to be passed through the channel reliably. The MAC layer creates the possibility of activating multiple physical channels or to offer asymmetric throughput.

–	The data link control (DLC) layer provides reliable transport of the data through the radio medium even when cells are switched during communication (handover).

–	The network layer’s (NWL) function is to transport data between network nodes (data routes within DECT or to the outside world).

–	In the DECT system, not only communication-type functions must be performed (for instance initiating a handover). Those functions can be found in the management entity, which interfaces to all layers in DECT.

# 6.	Physical layer (PHL)

DECT is a multi-carrier time division multiple access time division duplex system.

In Europe, DECT operates in the frequency band 1 880-1 900 MHz. The standard defines 10 carrier frequencies from 1 881.792 MHz to 1 897.344 MHz, separated by 1 728 kHz.

The RF modulation is Gaussian Frequency Shift Keying (GFSK) with a relative bandwidth of BT = 0.5; the modulating signal is a bit-stream of 1 152 kbit/s. Output power per physical channel is 10 mW. In the time domain, one frame of 10 ms consists of 24 slots of 416.7 ms each, and so creates 12 duplex channels per carrier (see Fig. 7).

In each time slot, a burst of 364.6 ms can be transmitted, so the overall throughput per slot is 42.0 kbit/s. This structure offers a MAC-layer data rate of 38.8 kbit/s for a duplex channel, enough to accommodate the bit-stream from and to the ITU-T Recommendation G.721 codec, used for DECT voice communication, as well as signalling information.

FIGURE 6...[D07] = 20 CM

FIGURE 7...[D08] = 10 CM

# 7.	Medium access control layer (MAC)

In the MAC layer, radio resources are allocated by activation and de-activation of physical channels.

The process will take changes of bearer capacity needs during the connection, as well as the need to change physical resources (within a fixed station) during a connection due to unacceptable degradation of quality, i.e. intra-cell handover.

Four logical channels are multiplexed onto these physical channels (see Fig. 8):

–	the user information (I) channel;

–	the signalling (C) channel, this channel contains MAC layer and signalling from higher layers;

–	the paging (P) channel, carrying the data necessary to call portables from the network;

–	the broadcast (Q) channel, which provides the portable repeatedly with information in order to recognize the network and its capabilities.

Data streams originating in higher layers are segmented. If required, data segments may be transmitted, using more than one physical channel.

Data protection for the signalling data is provided for in the MAC layer by adding cyclic redundancy check (CRC) bits. User data may not be protected (in case of speech transmission) or protected according to service requirements.

In the MAC packet, 4 bits determined from some of the 320 I-channel bits are present (X). They are not to protect the I-channel against transmission errors but to detect partial interference with the burst independently from the user service.

Constant observation of the X bits allows early detection of interference of parts of the burst (e.g. when an unsynchronized signal of a similar system slowly drifts into the wanted signal) enabling adequate counter measures, e.g. by handing the connection over to another channel.

FIGURE 8...[D09] = 11 CM

# 8.	Management entity (MGE)

A number of functions in the DECT system involve only one side of the communication, hence they do not appear in an OSI layer.

Typically, the following functions are present:

–	radio resource management, which includes the choice of free channels or the assessment of the quality of received signals;

–	mobility management, which includes attachment (or change of attachment) to cells in idle mode, or the registration of portables into networks or location areas of this network;

–	error handling, which includes the termination of a call in case of interruption of the radio link.

Although exchange to the outside world is not an issue with the MGE, the DECT standard will include some rules of performance requirements for it.

# 9.	Data link control layer (DLC)

The data link protocol creates and maintains reliable connections between a portable and the central system. The DLC carries two types of traffic each with specific protocol requirements. Therefore, the DLC layer is separated into C- (for control) and U- (for user) planes (see Fig. 9).

## 9.1	C-plane protocols

–	Connectionless broadcast service (Lb)

–	Link access procedure for C-plane LAPC. This procedure uses variable length frames and the characteristics of it are closely matched to those of the underlying MAC service; segment sizes will be chosen so that frame boundaries always align with inherent MAC timing boundaries (this will secure synchronization).

FIGURE 9...[D10] = 20 CM

## 9.2	U-plane protocols

The protocols are defined according to the service, required by specific applications.

Among others, the following have been identified:

–	forward error correction (FEC),

–	null service, offering unmodified MAC bearers,

–	frame relay for simple packet transport,

–	rate adaption, for modem and serial-line substitution packet transport.

The simple cordless telephone service, however, does not require these protocols; they may be regarded as basic to value added services.

# 10.	Network layer (NWL)

The network layer protocol is structured as a group of connected entities (see Fig. 9). These are:

## 10.1	Call control (CC)

This protocol establishes and releases network connections, it negotiates connection capabilities, it activates and controls inter-working units and it transfers call related supplementary services.

## 10.2	Supplementary services (SS)

The SS protocol supports supplementary services like call forwarding, follow me, etc., which are call independent.

## 10.3	Connection oriented message service (COMS)

This protocol can be used, for e.g. data transfer through the DECT C-plane, by using a connection oriented DLC service.

## 10.4	Connectionless message service (CLMS)

The CLMS protocol applies in case a connectionless (multicast/broadcast) service of the DLC is used for a broadcast of network layer system information via the C-plane.

## 10.5	Mobility management (MM)

This element is crucial for interfacing DECT smoothly with networks such as GSM, Telepoint and advanced cordless PABXs.

## 10.6	Link control entity (LCE)

This entity matches and coordinates the various logical links provided by the data link layer to the needs of the network layer entities.

BIBLIOGRAPHY

ETSI [1992] ETS 300 175 Digital European Cordless Telecommunications (DECT) Common Interface. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

ETSI [1992] I-ETS 300 176 Digital European Cordless Telecommunications (DECT) Approval test specification. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

ETSI [1992] ETR 042 Digital European Cordless Telecommunications (DECT) A guide to DECT features that influence the traffic capacity and the maintenance of high radio link transmission quality, including the results of simulations. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

ETSI [1992] ETR 043 Digital European Cordless Telecommunications (DECT) Common Interface Services and Facilities requirements specification. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

List of abbreviations

CC:	call control

CLAN:	cordless local area network

CLMS:	connectionless message service

COMS:	connection oriented message service

CRC:	cyclic redundancy check

DECT:	digital European cordless telecommunications

DLC:	data link control

ETSI:	European Telecommunications Standards Institute

FEC:	forward error correction

FS:	fixed station

GFSK:	Gaussian frequency shift keying

GOS:	grade of service

GSM:	global system for mobile communications

ISDN:	integrated services digital network

LAN:	local area network

LAP:	local access procedure

LCE:	link control entity

MAC:	medium access control

MGE:	management entity

MM:	mobility management

NWL:	network layer

OSI:	open systems interconnection

PABX:	private automatic branch exchange

PCN:	personal communications network

PHL:	physical layer

PMR:	private mobile radio

PS:	portable station

PSTN:	public switched telephone network

RE:	radio exchange

SAP:	service access point

SS:	supplementary services

TDMA:	time division multiple access

APPENDIX  6

General description of System 6 (digital)

# 1.	System configuration

System 6 is a digital cordless telephone system based upon a TDMA time division duplex technique. The basic configurations of System 6 for use in homes, offices, and outdoors are shown in Figs. 10, 11 and 12, respectively.

# 2.	System concept

System 6, based on a common air interface standard, enables a personal station (PS) to access cell station (CS)s in a variety of areas such as homes, offices and outdoors. When the dynamic channel assignment is applied, the system allows the co-existence of multiple systems in an area. Microcell radio zone structure along with frequency re-use techniques would increase the system capacity and reduce the transmission power, leading to a decrease in PS size. The digital technology of System 6 assures high communication quality and a high grade of security, and expands the service to relatively higher rates of data transmission or modem signal transmission. These features make it possible to apply System 6 to, for example, OA (office automation) or FA (factory automation) systems. In System 6, direct communication between PSs is possible in case portables cannot communicate via CSs.

# 3.	Access

## 3.1	Transmission scheme

System 6 is a multi-carrier time division multiple access time division duplex system (TDMA-TDD) with multiplexing of 4 time slot channels per carrier. The modulation scheme is p/4-shifted QPSK with a roll-off factor of 0.5; the modulating signal is a symbol stream of 192 000 symbol/s corresponding to a bit-stream of 384 kbit/s. Output power per physical channel is 10 mW, except for outdoor CS. In Japan, System 6 operates in the 1.9 GHz frequency band with 300 kHz carrier frequency spacing. For voice coding, the 32 kbit/s ADPCM recommended in ITU-T Recommendation G.721 is adapted in view of lower processing delay time and voice quality requirements. Even lower rate voice coding techniques such as half rate, or quarter rate voice coding can be easily adapted, assisted by the flexibility of the TDMA-TDD frame structure.

FIGURE 10...[D11] = 6 CM

FIGURE 11...[D12] = 6 CM

FIGURE 12...[D13] = 7 CM

## 3.2	TDMA-TDD frame structure

Figure 13 shows the TDMA-TDD frame structure. In the time domain, one frame of 5 ms consists of 8 slots of 625 ms each, and so accommodates 4 duplex channels per carrier. As shown in Fig. 13, mainly two types of slot, the control slot and the communication slot, are provided. The control slot is accessed in common by a large number of radio station (CSs, or PSs) and provides the function to set up an individual communication slot between a CS and a PS. Accordingly, access to the control slot from one radio station should be made intermittently. The communication slot is individually assigned to each CS and PS pair and provides the control channel and the information channel to be used for individual call set-up control and transportation of user information respectively.

FIGURE 13...[D14] = 13 CM

# 4.	Basic principle of protocol

## 4.1 	Protocol model

The protocol structure in System 6 consists of three protocol phases: the link channel acquisition phase, service channel acquisition phase, and communication phase. The concept of this structure was introduced by taking into account that the protocol should provide:

–	flexibility for various services, even over a degraded radio control channel, compared to fixed communication link;

–	unification and simplicity to minimize the program size in the PS;

–	ease in establishing local protocol to support the users original services.

The basic signal structure incorporating those three protocol phases is shown in Fig. 14:

–	Link channel acquisition phase – Through this phase, a link channel with required quality and capacity, and the proper protocol type for the call set-up in the next phase, are acquired.

–	Service channel acquisition phase – Using the acquired link channel, a service channel with required capacity and the proper protocol type for the communication phase are acquired through this phase.

–	Communication phase – In the communication phase, it is possible to select the optimal channel and protocol for each required service class. The concept of these service classes and protocol phases is shown in Fig. 15.

FIGURE 14...[D15] = 15 CM

FIGURE 15...[D16] = 7 CM

## 4.2	Layer structure

The control channel used in the link channel acquisition phase needs to be peculiar to the radio link characteristic. For this reason, the layer structure for link channel acquisition deviates somewhat from the OSI layered structure, with two layers, layers 2 and 3, being merged, as shown in Fig. 16. On the other hand, for both the service channel acquisition phase and communication phase, layers 1 through 3 from the OSI layered structure are adopted, with layer 3 supporting network functions such as RT (radio frequency transmission management), MM (mobility management) and CC (call control) as shown in Fig. 17.

FIGURE 16...[D17] = 7 CM

FIGURE 17...[D18] = 10 CM

# 5.	Layer 1 (physical layer)

Layer 1, the physical layer, provides and receives services to and from layer 2 and the management entity. Layer 1 provides layer 2 and the management entity with the following functions:

–	transmission ability which includes synchronization and transmission of control signal and/or user information;

–	establishment and release of the control channel and/or communication channel;

–	maintenance of the radio link;

–	signal transmission for operation and maintenance of the system;

–	indication of layer 1 status to the management entity;

–	radio link error detection, which is output slot by slot.

The channel structure applied to layer 1 is based upon ITU-T Recommendation Q.1063 and Report ITU-R M.1156 – Digital Cellular Public Land Mobile Telephone Systems (DCPLMTS).

# 6.	Layer 2 (data link layer)

Layer 2, the data link layer, creates and maintains a control link between the CS and PS. The features of LAPDC (Link Access Procedure for Digital Cordless) which constitutes this layer are as follows:

–	this procedure uses the HDLC (High level Data Link Control) subset for its protocol in order to maintain data link quality against degraded radio link. The basic protocol class is BAC (Balanced operation Asynchronous balanced mode Class) with a sub-function of I (Information) response deletion;

–	a UI (Un-numbered Information) command, which transmits information without link set-up procedure, is available for quick on-line control during communication;

–	an address part, consisting of a SAPI (service access point indicator) and a command/response indicator bit;

–	a frame synchronization function supplied from layer 1, releasing the LAPDC from generation/deletion of the flag sequence and “0” insertion/deletion process for succeeding transmission data “1” s;

–	an error detection function also supplied from layer 1, using CRC (cyclic redundancy check). This releases the LAPDC from generating/checking the FCS (frame check sequence).

# 7.	Layer 3 (network layer)

Layer 3, the network layer, offers the functions of establishment, maintenance, handover and release of network connection set-up on the common air interface. It also supports the functions of PS location registration, and authentication procedure for security. The layer 3 protocol applies to the environment, procedures and messages required for the entity of network functions both on the link channel and service channel. Those functions are:

–	RT	(Radio frequency Transmission management), which supports radio resource management including radio zone assignment, radio link establishment, maintenance, handover, and release;

–	MM	(mobility management) which supports PS mobility, providing the functions of PS location registration into a network and the authentication procedure for security;

–	CC	(call control) which supports network connection control including call set-up, call maintenance, and call release.

BIBLIOGRAPHY

RCR Standard (Draft) [December, 1992] Second generation cordless telephone system (RCR STD-28). Research & Development Centre for Radio Systems, Japan.

List of abbreviations

BAC:	balanced operation asynchronous balanced mode class

CAC:	common air channel

CC:	call control

CRC:	cyclic redundancy check

CS:	cell station

FCS:	frame check sequence

G:	guard time

HDLC:	high level data link control

I:	information

ISDN:	integrated services digital network

LAPDC:	link access procedure for digital cordless

MM:	mobility management

OSI:	open systems interconnection

PR:	preamble

PS:	personal station

PSTN:	public switched telephone network

QPSK:	quadrature phase shift keying

R:	ramp time for transient response

RT:	radio frequency transmission management

SAPI:	service access point indicator

SS:	start symbol

TDD:	time division duplex

TDMA:	time division multiple access

UI:	un-numbered information

UW:	unique word

APPENDIX  7

General description of System 7 (digital)

The CT2Plus system used in Canada for cordless telephone services has been designed to provide a wide range of capabilities and yet maintain efficient use of spectrum and inexpensive portable handsets. The system is designed for residential (cordless telephone), public access (“telepoint”), and commercial and business (wireless PBX) applications. The system specifically addresses the complete range from low-end residential systems to high functionality business systems on a fully compatible basis and in a cost effective manner.

The Canadian system is derived from the CT2 CAI which is defined in the European standard ETSI ETS-300-131. The CT2 CAI has also been underwritten by a memorandum of understanding as a standard for public service applications in many areas of the world. CT2Plus is based on the CT2 CAI with compatible enhancements to meet Canadian spectrum and service requirements. The system has been implemented in Canada in the 944-948.5 MHz band however extensions are provided to readily accommodate spectrum allocations of 8 MHz or more. Other spectrum bands can be easily accommodated.

The system can be interconnected to both analogue and ISDN networks and support features such as calling line identification and “D” channel data services. CT2Plus supports a wide range of basic and optional features. These include, for example, alerting functions, alphanumeric display, telepoint authentication and registration, handset registration to multiple service providers, emergency calling and two way calling features. In addition, signalling layer 3 includes areas of the message space for expansion in order to accommodate future services and features.

The voice quality provided by the system meets the Canadian network requirements for PBX/Centrex, local metropolitan and long distance connection for both two-wire and digital connections. In particular, the low delay inherent in the FDMA multiple access technique meets the delay requirements for connection to the public switched telephone network without the need for additional echo control devices.

The system uses time division duplexing (TDD) and digital modulation to achieve high spectral efficiency and robust operation within buildings. Dynamic channel allocation techniques further assure the efficient use of spectrum. Traffic densities as high as 10 000 E per square kilometre per floor may be achieved (at 0.5% grade of service blocking).

The system fully supports in-call handover. Well-defined procedures were provided for handover as a result of interference or as a result of changes in location. The system also includes provision for cases in which communications fail and alternative procedures must be called upon. The system fully supports location tracking and roaming of subscribers. Through the use of signalling channels (occupying the band 948.0-948.5 MHz), portable units may readily locate the nearest fixed part (or base-station) and register for active service. These signalling channels also may be used to announce call delivery to the portable part and thereby improve the standby life of the portables. There are five signalling frequencies each of which supports 12 signalling channels through time division multiple access.

BIBLIOGRAPHY

DOC [1993] Radio Standards Specification, RSS-130, Digital Cordless Telephones in the band 944 to 948.5 MHz. Department of Communications, Slater Street, Ottawa, Ontario, Canada.

DOC [1993] Radio Standards Specification, RSS-130, Annex 1, CT2Plus Class 2, Specification for the Canadian Common Air Interface for digital cordless telephony, including public access services. Department of Communications, Slater Street, Ottawa, Ontario, Canada.

_________________

*	A list of abbreviations is given at the end of this Appendix.