Recommendation ITU-R M.1184-4

(08/2026)

M Series: Mobile, radiodetermination, amateur
and related satellite services

Technical characteristics of mobile-satellite systems in the frequency bands below 3 GHz for use in sharing and compatibility studies between the mobile-satellite service (MSS) and other services

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the appropriate ITU-R patent information available at https://www.itu.int/en/ITU-R/study-groups/Pages/itu-r-patent-information.aspx.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radio-wave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2026

ITU 2026

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  M.1184-4

Technical characteristics of mobile-satellite systems in the frequency bands below 3 GHz for use in sharing and compatibility studies between the mobile-satellite service (MSS) and other services

(Question ITU-R 201-1/4)

(1995-2000-2003-2018-2026)

Scope

This Recommendation contains technical characteristics of mobile-satellite systems in the frequency bands below 3 GHz as a depository for use in sharing and compatibility studies between the mobile-satellite service (MSS) and other services, where appropriate and applicable.

Keywords

Characteristics, compatibility, GSO, non-GSO, mobile-satellite service (MSS), sharing

Abbreviations/Glossary

AMS(R)S	Aeronautical mobile-satellite (R) service

AMSS	Aeronautical mobile-satellite service

CDMA	Code division multiple access

DQPSK	Differential quadrature phase shift keying

EOC	Edge of coverage

FBB	Fleet broadband

FDD	Frequency division duplex

FDMA	Frequency division multiple access

FEC	Forward error correction

GMSK	Gaussian filtered MSK

GSO	Geostationary orbit

GSPS	Global satellite phone service

ICAO	International Civil Aviation Organization

IDP	IsatData Pro

IMT	International Mobile Telecommunication

LHCP	Left-hand circular polarization

LMSS	Land mobile-satellite service

MDS	Multipoint distribution system

MES	Mobile Earth station

MMSS	Maritime mobile-satellite service

MSK	Minimum shift keying

MSS	Mobile-satellite service

NBFM	Narrow-band frequency modulation

OQPSK	Offset quadrature phase-shift keying

PIFA	Planar inverted-F antenna

QPSK	Quadrature phase-shift keying

RHCP	Right-hand circular polarization

SARPs	Standards and recommended practices

SDPSK	Symmetrical differential PSK

TDD	Time division duplex

TDMA	Time division multiple access

UHF	Ultra-high frequency

VHF	Very-high frequency

Related ITU-R Recommendations, Reports

Recommendation ITU-R M.687 – International Mobile Telecommunications-2000 (IMT-2000)

Recommendation ITU-R M.818 – Satellite operation within International Mobile Telecommunications-2000 (IMT-2000)

Recommendation ITU-R M.1850 – Detailed specifications of the radio interfaces for the satellite component of International Mobile Telecommunications-2000 (IMT-2000)

Recommendation ITU-R M.2047 – Detailed specifications of the satellite radio interfaces of International Mobile Telecommunications-Advanced (IMT-Advanced)

The ITU Radiocommunication Assembly,

considering

that, while some mobile-satellite service (MSS) system parameters are still under development, a summary of representative technical characteristics of MSS systems is useful for conducting sharing and compatibility studies by the ITU-R,

recommends

1	that the representative technical characteristics for non-GSO MSS systems, as given in Annexes 1 and 2, should be considered in conducting sharing and compatibility studies, where appropriate and applicable;

2	that the representative technical characteristics for GSO MSS networks, as given in Annex 1, should be considered in conducting sharing and compatibility studies, where appropriate and applicable.

TABLE OF CONTENTS

Page

Annex 1 – Characteristics of representative 1-3 GHz MSS GSO satellite networks and non-GSO satellite systems		3

1	Satellite orbits		3

2	Global and regional/national GSO satellite networks		3

2.1	Maritime mobile-satellite service		4

2.2	Aeronautical mobile-satellite service		4

2.3	Land mobile-satellite service		4

2.4	GSO MSS satellite network characteristics		4

3	Non-GSO MSS satellite system characteristics		13

4	Propagation factors and mobile antenna characteristics		13

Annex 2 – Technical parameters of MSS satellite networks in the frequency bands below 1 GHz		24

# Annex 1

Characteristics of representative 1-3 GHz MSS GSO satellite networks and non-GSO satellite systems

# 1	Satellite orbits

Currently, GSO satellite networks and non-GSO satellite systems are being used for MSS communications. Suitable orbits are determined by coverage requirements, service considerations and frequency sharing, as well as other considerations.

# 2	Global and regional/national GSO satellite networks

Current United Kingdom and Russian mobile-satellite networks utilize Earth-coverage antennas to provide near-global service from the GSO in the 1.6/1.5 GHz band. The Russian satellite networks are similar to those of the United Kingdom satellite networks, the characteristics of which are presented in Table 2. The United Arab Emirates operates GSO MSS satellite networks to provide coverage in Regions 1 and 3 in the 1.6/1.5 GHz band. Several administrations are implementing regional/national GSO mobile-satellite networks to provide aeronautical, land and maritime services at 1.6/1.5 GHz. In addition to the above satellite networks at 1.6/1.5 GHz, Japan operates a GSO mobile-satellite network in the 2.6/2.5 GHz bands, and China has deployed its mobile-satellite networks and systems to provide regional service in the 1.6/2.5 GHz bands. Also, administrations are implementing regional/national GSO satellite networks in 2 GHz band.

Many MSS satellite networks use spot beams to provide greater spectrum efficiency and conserve transmitted power of the satellite and mobile earth station. China is planning to deploy the future generation of mobile-satellite networks and systems to provide its global service in the 1.6/2.5 GHz bands.

## 2.1	Maritime mobile-satellite service

A maritime mobile-satellite service (MMSS) system which is based on IMT-2000 standards provides broadband data and voice, simultaneously, through a compact antenna on a global basis. Another MMSS system provides store and forward data and telex using small and low-cost equipment.

## 2.2	Aeronautical mobile-satellite service

The International Civil Aviation Organization (ICAO) Standards and Recommended Practices (SARPs) for aeronautical mobile-satellite (R) service (AMS(R)S), airborne equipment have been published in Annex 10 to the Convention on International Civil Aviation. The SARPs include a requirement for priority and pre-emption for safety communications over all other communications. All systems providing AMS(R)S services to the international civil aviation community have to conform with the applicable ICAO SARPs.

In summary, the aeronautical satellite communication systems will have to take into account the priority needs for safe operation of aircraft and the avionics will have to satisfy the severe requirements of aircraft environments.

## 2.3	Land mobile-satellite service

The land mobile-satellite service (LMSS) has proven to be an effective means for providing dependable communications in remote and sparsely populated areas, either as extensions of terrestrial VHF and UHF networks or as replacements for HF networks. Worldwide roaming capability is a mandatory function of International Mobile Telecommunications-2000 (IMT-2000), and the satellite component defined in Recommendations ITU-R M.687 and ITU-R M.818 is one of the important components that encourage the IMT-2000 capability. The interworking of the mobile-satellite system with the terrestrial system can encourage user convenience within not only IMT-2000 but also general LMSS.

## 2.4	GSO MSS satellite network characteristics

Tables 1 to 3 present representative technical characteristics of service links for selected MSS satellite networks that utilize space stations in GSOs.

TABLE 1a

Technical characteristics of GSO mobile-satellite networks (service return link)

| System Parameter | GSO |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Polarization |  |  |  |  |  |  |  |  |  |
| Feeder link | Linear | Linear | Linear | Linear | Circular | Linear | Circular | Circular | Linear |
| Service link | RHCP | RHCP | RHCP | RHCP | RHCP | Circular | LHCP | LHCP | Circular |
| Direction of transmission | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space |
| Frequency bands |  |  |  |  |  |  |  |  |  |
| Feeder link (GHz) | 5 | 12 | 11 | 11 | 4 | 4, 11, 12 | 4 | 4 | 4 |
| Service link (GHz) | 1.6 | 1.6 | 1.6 | 2.0 | 2.0 | 1.6 | 1.6 | 1.6 | 2 |
| Orbit |  |  |  |  |  |  |  |  |  |
| Altitude (km) | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 |
| Satellite separation (degrees) | 120 | 78 | Not applicable | Not applicable | * | Not applicable | 20 to 30 | 20 to 30 | 20 to 25 |
| Number of satellites | 3 | 2 | 1 | 1 | 4 to 6 | 1 or 2 | 5 to 8 | 5 to 8 | 3 or 5 |
| Orbital planes | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| Inclination angle | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| Satellite antennas |  |  |  |  |  |  |  |  |  |
| Number of beams (service link) | 180 | 50 | 7 | 28 | 250 | Over 200 | 2 | 7 | Over 100 |
| Beam size (degrees) | 1 | 1 | 6 | 2 | * | 0.7 | 7 | 6 | 0.75 |
| Satellite coverage area | Global | Regional | North America, Alaska, Hawaii | North America, Alaska, Hawaii | Global | Regional | Regional | Regional | Regional |

TABLE 1a (continued)

| System Parameter | GSO |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Average beam side lobes (dB) | * | −25 | −25 | −25 | * | −20 | −20 | −20 | −20 |
| Beam frequency reuse | 5 | 5 | 1.3 | 2 | * | 1 to 30 | * | * | 7 |
| Link characteristics |  |  |  |  |  |  |  |  |  |
| Nominal user e.i.r.p. (dBW) | 6 | 0.5 | 12.5 | 10.9 | −1 to 8 | −7.5 to 3.5 | 5 to 12 | 3 to 10 | 1 to 25 |
| EOC satellite G/T (dB(K–1)) | 10 | 11 | 3.0 | 9.8 | 11 | 15.7 | −3 | −1 | 14 |
| Transmission parameters |  |  |  |  |  |  |  |  |  |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | BPSK | BPSK | BPSK/QPSK/ 8PSK |
| Coding | FEC | FEC | * | * | FEC | FEC | FEC | FEC | FEC |
| Access scheme | CDMA | FDMA | FDMA | FDMA | FDMA/ TDMA | FDMA/ TDMA | CDMA | CDMA | FDMA/ TDMA |
| Duplex scheme | * | FDD | FDD | FDD | FDD | FDD | FULL | FULL | FDD |
| Frame length | Not applicable | Not applicable | Not applicable | Not applicable | * | 40 | 20 to 200 | 20 to 1100 | Not applicable |
| Burst rate (kbit/s) | Not applicable | Not applicable | Not applicable | Not applicable | 32 | 46.8 | 8 | 4 to 16 | Not applicable |
| Chip rate (Mchip/s) | 8.33 | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | 4.1 | 4.1 | Not applicable |
| Voice activity factor | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | Not applicable | Not applicable | 0.4 |
| RF carrier spacing (MHz) | * | 0.006 | 0.006 | 0.006 | * | 0.03125 | Not applicable | Not applicable | 0.0216 |
| RF channel spacing (MHz) | * | 0.006 | 0.006 | 0.006 | * | 0.03125 | Not applicable | Not applicable | 0.0216 |
| Modulation bandwidth (MHz) | * | 0.0045 | 0.0047 | 0.0047 | * | 0.0234 | 8.2 | 8.2 | 0.016 |

TABLE 1a (end)

| System Parameter | GSO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |  |
| Required Eb /N0 (dB) |  |  |  |  |  |  |  |  |  |  |
| Voice | 2.5 | 9.0 | 9.0 | 9.0 | 4.0 | 3.5 | Not applicable | Not applicable |  | −1 to 6.5 |
| Data | 4.1 | 9.0 | 9.0 | 9.0 | * | 5.5 to 7.0 | 7.0 | 7.0 |  | 3.1 to 11.6 |
| Maximum MES antenna discrimination towards the horizon (dBi) | * | 7 | 7 | 7 | 1.0 | 7 to 19 | * | * |  | * |
| * Value requiring further study. |  |  |  |  |  |  |  |  |  |  |

TABLE 1b

Technical characteristics of GSO mobile-satellite networks (service forward link)

| System Parameter | GSO |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Polarization |  |  |  |  |  |  |  |  |  |
| Feeder link | Linear | Linear | * | * | Circular | Linear | Circular | Circular | Linear |
| Service link | RHCP | RHCP | * | * | RHCP | Circular | RHCP | RHCP | Circular |
| Direction of transmission | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth |
| Frequency bands |  |  |  |  |  |  |  |  |  |
| Feeder link (GHz) | 6 | 14 | 13 | 13 | 6 | 6, 13, 14 | 6 | 6 | 6 |
| Service (GHz) | 2.5 | 1.5 | 1.5 | 1.9 | 2.2 | 1.5 | 2.5 | 2.5 | 2.2 |
| Orbit |  |  |  |  |  |  |  |  |  |
| Altitude (km) | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 | 36 000 |
| Satellite separation (degrees) | 120 | 78 | Not applicable | Not applicable | * | Not applicable | 20 to 30 | 20 to 30 | 20 to 25 |
| Number of satellites | 3 | 2 | 1 | 1 | 4 to 6 | 1 or 2 | 5 to 8 | 5 to 8 | 3 or 5 |

TABLE 1b (continued)

| System Parameter | GSO |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  | B | C | D |  |  | E | F | G | H | I |
| Orbital planes | Not applicable |  | Not applicable | Not applicable | Not applicable |  |  | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| Inclination angle | Not applicable |  | Not applicable | Not applicable | Not applicable |  |  | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| Satellite antennas |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of beams (service link) | 180 |  | 50 | 7 | 28 |  |  | 150 | Over 200 | 2 | 7 | Over 100 |
| Beam size (degrees) | 1 |  | 1 | * | * |  |  | * | 0.7 | 7 | 6 | 0.75 |
| Satellite coverage area | Global |  | Regional | North America, Alaska, Hawaii | North America, Alaska, Hawaii |  |  | Global | Regional | Regional | Regional | Regional |
| Average beam side lobes (dB) | * |  | −25 | −25 | −25 |  |  | * | −20 | −20 | −20 | −20 |
| Beam frequency reuse | 5 |  | 5 | 1.2 | 2 |  |  | * | 1 to 30 | * | * | 7 |
| Link characteristics |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum e.i.r.p./beam (dBW) |  | 45.8 | 53.5 | 58.4 |  | 52.8 | * |  | * | 54 | 54 | 51.5 |
| Average gain/beam (dBi) |  | * | 44 | 32 |  | 41.1 | * |  | 44.5 | 27 | 27 | 44 |
| e.i.r.p./carrier (dBW) |  | 28.8 | 30 | 30 |  | 35.5 | 42 |  | 24.5 to 45.0 | 46 to 54 | 46 to 54 | 35 to 54 |
| e.i.r.p./shadowed user (dBW) |  | * | 30 | 30 |  | 35.5 | * |  | * | * | * | * |
| e.i.r.p./unshadowed user (dBW) |  | * | 30 | 30 |  | 35.5 | * |  | * | * | * | * |
| e.i.r.p./CDMA channel (dBW) |  | Not applicable | Not applicable | Not applicable |  | Not applicable | Not applicable |  | Not applicable | 54 | 54 | Not applicable |
| pfd level per beam carrier (dB(W/(m2 · 4 kHz))) |  | * | −131 | −131.0 |  | −127.5 | −123.0 |  | −138.0 to −117.5 | −141 | −141 | * |
| Transmission parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of channels/satellite |  | * | 5 000 | 2 000 |  | 10 000 | 5 000 |  | 20 000 | 2 | 7 | 5 000 |
| User G/T (dB(K–1)) |  | −20 | −22 | −16 |  | −16 | −23 |  | −23.5 to −9.0 | −24 | −24 | −29 to -20 |

TABLE 1b (end)

| System Parameter | GSO |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Minimum elevation angle (degrees) | 5 | 5 | 5 | 5 | 10 | 20 | 10 | 10 | 10 |
| Lifetime (years) | 12 | 12 | 12 | 12 | 10 to 12 | 12 | 10 to 15 | 10 to 15 | 15 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | BPSK/QPSK/8PSK |
| Coding | FEC | FEC | FEC | FEC | FEC | FEC | FEC | FEC |  |
| Access scheme | CDMA | FDMA/ TDMA | FDMA | FDMA | TDMA | FDMA/ TDMA | CDMA | CDMA | FDMA/ TDMA |
| Frame length (ms) | * | Not applicable | Not applicable | Not applicable | * | 40 | 40 | 120 | * |
| Burst rate (kbit/s) | Not applicable | Not applicable | Not applicable | Not applicable | 32 | 46.8 | Not applicable | Not applicable | * |
| Chip rate (Mchip/s) | 8.33 | Not applicable | Not applicable | Not applicable | * | Not applicable | 4.1 | 8.2 | * |
| Voice activity factor | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.5 | Not applicable | Not applicable | 0.4 |

| Required Eb/N0 (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Voice | 2.5 | 9 | 9 | 9 | * | 3.5 | Not applicable | Not applicable | −1 to 6.5 |
| Data | 4.1 | 9 | 9 | 9 | * | 5.5 to 7.0 | 7 | 6 | 3.1 to 11.6 |
| MES geographical distribution | * | * | * | * | * | * | Regional | Regional | Regional |
| * Value requiring further study. |  |  |  |  |  |  |  |  |  |

TABLE 2

GSO satellite network technical characteristics in the 1.6/1.5 GHz band overview

|  | IDP | C | Hand held (GSPS) | Land |  | Maritime |  | Aeronautical |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | High gain | Low gain | High gain | Low gain | High gain | Low gain |
| Service | LMSS | MMSS | LMSS | LMSS | LMSS | MMSS | MMSS | AMSS AMS(R)S | AMSS AMS(R)S |
| Typical mobile station antenna gain (dBi) | 0 | 0 | 2 | 17 | 14 | 18 | 11.5 | 12 | 6 |
| Antenna type (example) | Patch | Quad helix | Quad Helix | Phased array | Phased array | Phased array | Phased array | Phased array | Phased array |
| Typical antenna size | 5 cm | 5 cm diameter | 12 cm | 50 cm diameter | 30 cm diameter | 55 cm diameter | 30 cm diameter | 2 panels 60  60 cm | 20  15 cm |
| Mobile earth station figure of merit (G/T) (dB(K–1)) | −6 | −23 | −23 | −9 | −12 | −7 | −15.5 | −13 | −19 |
| Minimum mobile earth station e.i.r.p./channel (dBW) | 8.7 | 7.3 | −2 | 18 | 13 | 16 | 12.3 | 13.8 | 9.7 |
| Maximum mobile earth station e.i.r.p./channel (dBW) | 13.5 | 11 | 5 | 21 | 16 | 23 | 16.1 | 22 | 17.1 |
| User data rate | 600 bits/s | 600 bit/s | Voice service | 500 kbit/s | 250 kbit/s | 500 kbit/s | 250 kbit/s | 500 kbit/s | 250 kbit/s |
| Modulation | 32ary FSK | BPSK | GMSK | 16-QAM | 16-QAM | 16-QAM | 16-QAM | 16-QAM | 16-QAM |
| Minimum satellite e.i.r.p./channel (dBW) | 22.30 | 16.6 | 40.5 | 40.5 | 40.5 | 40.5 | 40.5 | 40.5 | 40.5 |
| Maximum satellite e.i.r.p./channel (dBW) | 27.4 | 21.8 | 44.5 | 44.5 | 44.5 | 44.5 | 44.5 | 44.5 | 44.5 |

TABLE 2 (end)

|  | IDP | C | Hand held (GSPS) | Land |  | Maritime |  | Aeronautical |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | High gain | Low gain | High gain | Low gain | High gain | Low gain |
| Uplink channel spacing (nominal) (kHz) | 2.5 | 5 | 50 | 200 | 200 | 200 | 200 | 200 | 200 |
| Downlink channel spacing (nominal) (kHz) | 2.5 | 5 | 200 | 200 | 200 | 200 | 200 | 200 | 200 |
| Satellite peak antenna gain (dBi) | 22 | 22 | 44 | 44 | 44 | 44 | 44 | 44 | 44 |
| Satellite noise temperature (K) | 630 | 630 | 630 | 630 | 630 | 630 | 630 | 630 | 630 |
| * Value requiring further study. |  |  |  |  |  |  |  |  |  |

TABLE 3

National/regional GSO satellite network overview

|  | Australia | Canada/ United States of America | Japan | Europe | East Asia Southeast Asia | Republic of Korea(2) |
| --- | --- | --- | --- | --- | --- | --- |
| Service |  | MSS | MSS | LMSS/ MMSS | AMSS | MSS | MSS |
| Frequency bands (GHz) |  | 1.5/1.6 | 1.5/1.6 | 2.5/2.6 | 2/2.2 | 2/2.2 | 1.5/1.6 |
| Typical mobile earth station antenna gain (dBi) |  | 12 | 8 to 13 0 to 4 | Up to 12 | 0 to 12 | −2 to 13 | 16 to 21.2 |
| Antenna type (example) |  | To be determined | – Electrically scanned phased array – Mast type | Phased array | – Electrically scanned phased array – Crossed-dipole | – Electrically scanned phased array – Crossed-dipole PIFA | Passive patch array or helical |

TABLE 3 (end)

|  | Australia | Canada/ United States of America | Japan | Europe | East Asia Southeast Asia | Republic of Korea(2) |
| --- | --- | --- | --- | --- | --- | --- |
| Typical antenna size | To be determined | 25 to 50 cm diameter | 10 to 20 cm diameter | 15 to 25 cm diameter | Up to 35 cm diameter | 65 to 120 cm diameter |
| Mobile earth station figure of merit (G/T) (dB(K–1)) | −13 | −15 to −12 −23 to −18 | Not available (N/A) | −11 to −23 | −29 to −20 | −6.5 to −1 |
| Mobile earth station e.i.r.p./channel (dBW) | 15 | 10 to 16 | Up to 17 | 10 to 16 | 1 to 25 | 5 to 17 |
| User data rate | 2 400 bit/s | 2.4 to 4.8 kbit/s 4 to 8 kbit/s, voice | Up to 1.5 Mbit/s | Up to 416 kbit/s | Up to 384 kbit/s | Up to 24 kbit/s |
| Communication channel rate and modulation | 6.6 kbit/s | 4.8 to 9.6 kbit/s, OQPSK TCM | BPSK/QPSK/16-QAM/64-QAM | 1/5-9/10 coding rate BPSK/QPSK/8PSK/16APSK/32APSK | BPSK/QPSK/8PSK | QPSK |
| Typical C/N0 for communication channel (dB(Hz)) | 48 | 45 to 51 | Not available (N/A) | 41 to 61 | 44 to 56.5 | 35 to 55 |
| Satellite e.i.r.p./channel (dBW) | 22 | 23 to 29 | 69.1(1) | 25 to 48 | 35 to 54 | 28 to 36(1) |
| Channel spacing (nominal) (kHz) | 7.5 | 5 to 10 | 1 200 to 10 000 | 2.5 to 200 | 21.6 | 2 to 50 |
| Satellite peak antenna gain (dBi) | To be determined | 32 | 49 | 35 | 44/47 | 19.2/19.9 |
| pfd | * | * | * | * | * | * |
| (1) Satellite e.i.r.p. density (dB(W/1 MHz)). (2) For the feeder link in the bands 1.5 GHz (space-to-Earth) and 1.6 GHz (Earth-to-space), the e.i.r.p. density values for the satellite and earth station are 16 to 35 dBW/MHz and 43 to 56 dBW/MHz respectively, and the G/T values for the satellite and earth station are 15 to 20 dB/K and −9 to −6 dB/K respectively. * Value requiring further study. |  |  |  |  |  |  |

# 3	Non-GSO MSS satellite system characteristics

Proposed personal communication systems using non-GSO satellites, including satellites using low, medium and intermediate orbits, are expected to provide voice, data communications and positioning on a worldwide basis using mobile terminals or hand-held portable terminals using omnidirectional antennas.

Operation in a band contiguous with future land mobile communication systems would permit interoperability between non-GSO MSS and land mobile systems.

Table 4 presents representative technical characteristics of service links for selected MSS networks that utilize space stations in non-GSOs.

# 4	Propagation factors and mobile antenna characteristics

Signal level variation due to multipath effects and blockage by ship’s superstructure occurs in MMSS links. Multipath, especially sea surface-reflected multipath, is a significant factor to be considered in the design of an aeronautical mobile-satellite system. In LMSS links, foliage shadowing is a significant additional effect which increases with frequency. Furthermore, several propagation factors should be taken into account when designing a non-GSO MSS system providing personal services. These propagation factors affect system characteristics, such as link margin and transmission power control techniques.

Reference radiation patterns for various types of LMSS mobile earth station antenna are recommended to assess the interference calculation for coordination studies (see Recommendation ITU-R M.1091).

TABLE 4a

Technical characteristics of non-GSO mobile-satellite systems (service return link)

| System Parameter | A(1) | B | C | D | E |  |  |  | F | G |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Link 1 | Link 2 |
| Polarization |  |  |  |  |  |  |  |  |  |  |  |
| Feeder link | RHCP | RHCP | Circular | RHCP/ LHCP | RHCP/LHCP |  |  |  | Circular | RHCP | RHCP |
| Service link | RHCP | LHCP | Circular | LHCP | RHCP |  |  |  | RHCP | LHCP | LHCP |
| Direction of transmission | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space (service) |  |  |  | Earth-to-space | Earth-to-space | Earth-to-space |
| Frequency bands (GHz) |  |  |  |  |  |  |  |  |  |  |  |
| Feeder link | 30 | 20 | 5 | 7 |  | < 19 |  |  | 7 | 11 | 11 |
| Service link | 1.6 | 1.6 | 1.6 | 1.6 |  | 1.6 |  |  | 2 | 0.2 | 1.6 |
| Orbit |  | (2) |  |  | Eccentric | Circular |  | Elliptical (alternate orbit) |  | Circular | Circular |
| Altitude (km) | 780 | 10 355 | 2 000 | 1 414 | 520/7 846 | 7 846 |  | 4 376/7 846 | 10 355 | 1 500 | 1 500 |
| Satellite separation (degrees) | 32.7 | 90 | 45 | 60 |  |  |  |  | 72 | 30 | 30 |
| Number of satellites | 66 | 12 | 40 | 48 | 4-5 | 6-8 |  | 6-8 | 10 | 48 | 48 |
| Orbital planes | 6 | 3 | 5 | 8 | 2 | 1 |  | 1 | 2 | 4 | 4 |
| Inclination angle (degrees) | 86 | 50 | 55 | 52 | 116.6 | 0 |  | 0 | 45 | 74 | 74 |
| Satellite antennas |  |  |  |  |  |  |  |  |  |  |  |
| Number of beams (service link) | 48 | 37 | 10 | 16 | 91 in eccentric orbit and 61 in circular orbit or 19 on each satellite |  |  |  | 121 | 1 | 6 |
| Beam size (km2) | 1.8  105 to 7  105 | 9.7  105 (6.3°) | * | 6.3  105 to 2.3  106 | 7.78  105 to 2.6  106 |  |  |  | 5  105 to 2  106 | 5  107 | 8.4  106 |
| Average beam side lobes (dB) | −20 | −20 | To be determined | −15 | –15 and greater |  |  |  | −20 (peak) | −3 | −2 |
| Beam frequency reuse | 0.167 | 1 | 1 | 1 | N (where N: number of beams) |  |  |  | * | 1 | 0.6 |
| Link characteristics |  |  |  |  |  |  |  |  |  |  |  |
| Nominal user e.i.r.p. (dBW) | −4 to 6 (peak) | −5.8 to −11 | 0-10 | −3 | Baseline (19 beams) 3 mobile/ portable 13 fixed |  | Enhanced (91/61 beams) 6 hand-held/mobile/ portable 13 fixed |  | −1 (average) 7 (peak) | 6.1 | 6 |
| EOC satellite G/T (dB(K–1)) | −3 to −10 | −1.4 to 1.8 | −11 | −17 | −5.75 (G  21)(3) |  | −0.75 (G  26) (3) |  | 2 | −25.5 | −14 |

TABLE 4a (continued)

| System Parameter | H | R | K |  | N | O | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | I | M |  |  |  |
| Polarization |  |  |  |  |  |  |  |
| Feeder link | LHCP | Circular | Circular | Circular | Circular | Circular | Circular |
| Service link | RHCP | Circular | LHCP | LHCP | Circular | Circular | Circular |
| Direction of transmission | Earth-to-space | Earth-to-space | earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space | Earth-to-space |
| Frequency bands (GHz) |  |  |  |  |  |  |  |
| Feeder link | 5.2 | 19 | * | * | 5.2 | 40 | 7 |
| Service link | 1.6 | 1.9, 2.6 | 1.6 | 1.6 | 1.9 | 1.6 | 1.6 |
| Orbit | Circular | Circular | Circular | Circular | Circular | Circular | Circular |
| Altitude (km) | 1 000 | 700 | 36 000 | 21 500 | 1 500 | 1 175 | 1 400/1 414 |
| Satellite separation (degrees) | 51.4 | 27.7 | 120 | 120 | 51.4 | 36 | 90 |
| Number of satellites | 7 | 91 | 3 to 9 | 14 to 27 | 28 | 12 | 12/48 |
| Orbital planes | 7 | 7 | 3 | 3 | 4 | 6 | 3/12 |
| Inclination angle (degrees) | 83 | 82 | 55 | 55 | 82,5 | 86.5 | 87/52 |
| Satellite antennas |  |  |  |  |  |  |  |
| Number of beams (service link) | 9 | 37 | 1 to 7 | 1 | * | 52 | 256 |
| Beam size (km2) | 2.6  105 to 2  106 | 7.6  104 to 3.5  105 | 1.25 × 107 to 8.5 × 107 | 8.5 × 107 | * | 9 × 105 to 1.4 × 106 | 3.8 × 104 |
| Average beam side lobes (dB) | 15 | 20 | –20 | –20 | * |  | −15 |
| Beam frequency reuse | 0.11 | 3 | * | * | * | 4 | 1 |
| Link characteristics |  |  |  |  |  |  |  |
| Nominal user e.i.r.p. (dBW) | 8 | 1.5 | 5 to 10 | 8 to 12 | 8 to 16 | 0 to 23 | −7 to 11.6 |
| EOC satellite G/T (dB(K–1)) | −18 | −12.6 | −8.5 to −6 | −16.5 to −14.5 | −15.6 | −32 to −15.5 | −7.4 |

TABLE 4a (continued)

| System Parameter | A(1) | B | C | D |  | E | F | G |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Link 1 | Link 2 |
| Transmission parameters |  |  |  |  |  |  |  |  |  |
| Modulation | QPSK | QPSK | QPSK |  | QPSK | OQPSK spreading modulation | QPSK | QPSK | QPSK |
| Coding | FEC | FEC | FEC |  | FEC | FEC rate 1/3, K 9 | FEC | FEC | FEC |
| Access scheme | FDMA/ TDMA | FDMA/ CDMA | FDMA/ CDMA |  | FDMA/ CDMA | CDMA | FDMA/ TDMA | FDMA/ CDMA | FDMA/ CDMA |
| Duplex scheme | TDD | FDD | FDD |  | FDD | Full | FDD | Full | Full |
| Transmission parameters (cont.) |  |  |  |  |  |  |  |  |  |
| Frame length (ms) | 90 | Not applicable | Not applicable | Not applicable |  | 320 and 25.86 (random access) | 40 | 60 | 60 |
| Burst rate (kbit/s) | 50 | Not applicable | Not applicable | Not applicable |  | 0.3-9.6 | 36 | Not applicable | Not applicable |
| Chip rate (Mchip/s) | Not applicable | ~2 | 2.56 | 1.2288 |  | 1.9 and/or 7.6 | Not applicable | 0.15 | 2.4 |
| Voice activity factor | 0.4 | 0.5 | 0.4 | 0.4 |  | 0.4 | 0.4 | Not applicable | Not applicable |
| RF carrier spacing (MHz) | 0.04167 | Not applicable | To be determined | Not applicable |  | Not applicable | 0.025 | 0.05 | 0.05 |
| RF channel bandwidth (MHz) | Not applicable | 2.5 | To be determined | 1.2 |  | Not applicable | 0.025 | 0.5 | 5.8 |
| Modulation bandwidth (MHz) | 0.0315 | 2.5 | To be determined | 1.2 |  | 1.9 and/or 7.6 | 0.025 | 0.5 | 5.8 |
| Required Eb /N0(dB) | 6.1 | 4.0 | 2.8 | 4.8(4) |  | 4.5 (with margin) | 2.5 | 32.6 | 35.4 |
| Maximum MES antenna gain towards the horizon (dBi) | 0 | 0 | To be determined | * |  | 3 mobile 10 fixed 0 hand-held | 2 | 1 | 1.2 |

TABLE 4a (end)

| System Parameter | H | R | K |  | N | O | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | I | M |  |  |  |
| Transmission parameters |  |  |  |  |  |  |  |
| Modulation | BPSK | QPSK | BPSK | BPSK | π/4-DQPSK | QPSK | PSK |
| Coding | FEC | Convolutional code rate 1/2, K  7 | FEC | FEC | FEC | FEC | FEC |
| Access scheme | FDMA/ CDMA | FDMA/ CDMA | CDMA | CDMA | TDMA | FDMA/ CDMA | TDMA/FDMA/ CDMA |
| Duplex scheme | Full | FDD | Full | Full | FDD | FDD | FDD |
| Transmission parameters (cont.) |  |  |  |  |  |  |  |
| Frame length (ms) | 60 | Not applicable | 50 to 800 | 500 to 1500 | * | * | * |
| Burst rate (kbit/s) | 50 | Not applicable | 1 to 4 | 0.4 to 2 | * | * | * |
| Chip rate (Mchip/s) | 3 | 0.624 | 1.6 and/or 4.1 | 1.6 | * | * | * |
| Voice activity factor | Not applicable | Not applicable | Not applicable | Not applicable | * | * | * |
| RF carrier spacing (MHz) | 0.25 | 1.25 | Not applicable | Not applicable | * | * | * |
| RF channel bandwidth (MHz) | 2.05 | 1.25 | * | * | * | 0.125 | * |
| Modulation bandwidth (MHz) | 2.05 | 1.25 | 3.2 and/or 8.2 | 3.2 | * | 0.125 | * |
| Required Eb /N0(dB) | 16 | 6.5 | 7 | 7 | * | 5.5 | * |
| Maximum MES antenna gain towards the horizon (dBi) | 2 | 0 | * | * | * | * | * |
| (1) Satellite antenna gains adjusted to maintain near-constant received power independent of range to user. (2) System B has a 6 h sidereal orbit. (3) T  473 K. (4) Includes effect of feeder link. * Value requiring further study. |  |  |  |  |  |  |  |

TABLE 4b

Technical characteristics of non-GSO mobile-satellite systems (service forward link)

| System Parameter | A | B | C | D | E |  |  | F | G |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Link 1 | Link 2 |
| Polarization |  |  |  |  |  |  |  |  |  |  |
| Feeder link | RHCP | LHCP | Circular | RHCP/ LHCP | Dual circular |  |  | Circular | LHCP | LHCP |
| Service link | RHCP | LHCP | Circular | LHCP | RHCP |  |  | RHCP | RHCP | RHCP |
| Direction of transmission | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth (service) |  |  | Space-to-Earth | Space-to-Earth | Space-to-Earth |
| Frequency bands (GHz) |  |  |  |  |  |  |  |  |  |  |
| Feeder link | 20 | 30 | 6 | 5 |  19 |  |  | 5 | 14 | 14 |
| Service | 1.6 | 2.5 | 2.5 | 2.5 | 2.5 |  |  | 2.2 | 0.4 | 1.5 |
| Orbit |  | (2) |  |  | Eccentric | Circular | Elliptical (alternate orbit) |  | Circular | Circular |
| Altitude (km) | 780 | 10 355 | 2 000 | 1 414 | 520/7 846 | 7 846 | 4 376/7 846 | 10 355 | 1 500 | 1 500 |
| Satellite separation (degrees) | 32.7 | 90 | 45 | 60 | – | – | – | 72 | 30 | 30 |
| Number of satellites | 66 | 12 | 40 | 48 | 4-5 | 6-8 | 6-8 | 10 | 48 | 48 |
| Orbital planes | 6 | 3 | 5 | 8 | 2 | 1 | 1 | 2 | 4 | 4 |
| Inclination angle (degrees) | 86 | 50 | 55 | 52 | 116.6 | 0 | 0 | 45 | 74 | 74 |
| Satellite antennas |  |  |  |  |  |  |  |  |  |  |
| Number of beams (service link) | 48 | 37 | 10 | 16 | 91 in eccentric orbit and 61 in circular orbit or 19 on each satellite |  |  | 121 | 1 | 6 |
| Beam size (km2) | 1.8  105 to 17  105 | 9.7  105 (6.3°) | * | 6.3  105 to 2.3  106 | 7.78  105 to 2.6  106 |  |  | 5  105 to 2  106 | 5  107 | 8.4  106 |
| Average beam side lobes (dB) | –20 | –20 | To be determined | –15 | –15 and greater |  |  | –20 (peak) | –3 | –2 |
| Beam frequency reuse | 0.167 | 1 | 1 | 1 | N (where N: number of beams) |  |  | * | 1 | 0.6 |

TABLE 4b (continued)

| System Parameter | H | R | K |  | N | O | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | I | M |  |  |  |
| Polarization |  |  |  |  |  |  |  |
| Feeder link | RHCP | Circular | Circular | Circular | Circular | Circular | Circular |
| Service link | RHCP | Circular | RHCP | RHCP | Circular | Circular | Circular |
| Direction of transmission | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth | Space-to-Earth |
| Frequency bands (GHz) |  |  |  |  |  |  |  |
| Feeder link | 7 | 15 | * | * | 7 | 40 | 7 |
| Service | 2.5 | 2.1 | 2.5 | 2.5 | 2.1 | 1.5 | 2.5 |
| Orbit | Circular | Circular | Circular | Circular | Circular | Circular | Circular |
| Altitude (km) | 1 000 | 700 | 36 000 | 21 500 | 1 500 | 1 175 | 1 400/1 414 |
| Satellite separation (degrees) | 51.4 | 27.7 | 120 | 120 | 51.4 | 36 | 90 |
| Number of satellites | 7 | 91 | 3 to 9 | 27 | 28 | 12 | 12/48 |
| Orbital planes | 7 | 7 | 3 | 3 | 4 | 6 | 3/12 |
| Inclination angle (degrees) | 83 | 82 | 55 | 55 | 82.5 | 86.5 | 87/52 |
| Satellite antennas |  |  |  |  |  |  |  |
| Number of beams (service link) | 9 | 37 | 1 | 1 | 16 | 52 | 128 |
| Beam size (km2) | 2.6  105 to 2  106 | 7.6  104 to 3.5  105 | 8.5 × 107 | 8.5 × 107 | * | 9  105 to 1.4  106 | 5.4 × 104 |
| Average beam side lobes (dB) | –15 | –20 | –20 | –20 | * | * | –15 |
| Beam frequency reuse | 0.11 | 3 | * | * | * | 4 | 1 |

TABLE 4b (continued)

| System Parameter | A | B | C | D | E |  |  | F | G |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Link 1 | Link 2 |
| Link characteristics |  |  |  |  |  |  |  |  |  |  |
| Maximum e.i.r.p./beam (dBW) | * | ~52 | 27.5 | * | pdf  −142 dB(W/(m2 · 4 kHz)) |  |  | 52 | −2 | 2.8 |
| Average gain/beam (dBi) | 17-25(1) | 24-28 | 15.2 | Not applicable | 18.5 (baseline) at nadir-peak gain 28.8 (enhanced) at nadir-peak gain |  |  | 30 | 3 | 13 |
| e.i.r.p./carrier (dBW) |  |  | To be determined |  |  |  |  | 33 | −15 | −7.2 |
| Unshadowed user e.i.r.p. (dBW) | 7-15 | 20.6 | To be determined | Not applicable | 13.92-18.66 for 19 beams per satellite or 13.92-21.5 for 61/91 beams per satellite |  |  | * | Not applicable | Not applicable |
| Shadowed user e.i.r.p. (dBW) | 19-27 | 24.6 | To be determined | 0-5 | Add 2.5 dB |  |  | * | Not applicable | Not applicable |
| e.i.r.p./CDMA channel (dBW) | Not applicable | * | –7 to 6 | 0 to 16 |  | * |  | Not applicable | −5 | −10.2 |
| User G/T (dB(K–1)) | −23 | −22.2 to −24 | −22 | −23 |  | −25 to −15 |  | −24 | −23.8 | −14 |
| Minimum elevation angle (degrees) | 8.3 | 20 | 15 | 10 |  | 15 |  | 10 | 7 | 10 |
| Transmission parameters |  |  |  |  |  |  |  |  |  |  |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |  |  | QPSK | QPSK | QPSK |
| Coding | FEC | FEC | FEC | FEC | FEC rate 1/3, K  9 |  |  | FEC | FEC | FEC |
| Access scheme | FDMA/ TDMA | FDMA/ CDMA | FDMA/ CDMA | FDMA/ CDMA | CDMA |  |  | FDMA/ TDMA | FDMA/ CDMA | FDMA/ CDMA |
| Duplex scheme | TDD | FDD | FDD | FDD | FDD |  |  | FDD | Full | Full |
| Frame length (ms) | 90 | * | Not applicable | * | 320 |  |  | 40 | Not applicable | Not applicable |
| Burst rate (kbit/s) | 50 | Not applicable | Not applicable | Not applicable | 0.3 to 9.6 |  |  | 36 | Not applicable | Not applicable |

TABLE 4b (continued)

| System Parameter | H | R | K |  | N | O | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | I | M |  |  |  |
| Link characteristics |  |  |  |  |  |  |  |
| Maximum e.i.r.p./beam (dBW) | 19 | 31.7 | 36 | 32 | 25 | 28 | * |
| Average gain/beam (dBi) | 10 | 31.7 | * | * |  | * | * |
| e.i.r.p./carrier (dBW) | 15 | 31.7 | 36 | 32 |  | * | * |
| Unshadowed user e.i.r.p. (dBW) | Not applicable | Not applicable | * | * | * | * | * |
| Shadowed user e.i.r.p. (dBW) | Not applicable | Not applicable | Not applicable | Not applicable | * | * | * |
| e.i.r.p./CDMA channel (dBW) | 15 to 19 | 31.7 | 36 | 32 | * | * | * |
| User G/T (dB(K–1)) | −24 | −18 | −24 | −24 | −19.7 to −11.7 | −32 to −15.5 | −23 |
| Minimum elevation angle (degrees) | 10 | 10 | 10 | 10 |  | 10 | 10 |
| Transmission parameters |  | (2) |  |  |  |  |  |
| Modulation | QPSK | QPSK | QPSK | QPSK | π/4-DQPSK | QPSK | PSK |
| Coding | FEC | Convolutional code rate 1/2, K  7 | FEC | FEC | FEC | FEC | FEC |
| Access scheme | FDMA/ CDMA | FDMA/ CDMA | CDMA | CDMA | TDMA | FDMA/ TDMA | TDMA/ FDMA/ CDMA |
| Duplex scheme | Full | FDD | Full | Full | FDD | FDD | FDD |
| Frame length (ms) | Not applicable | Not applicable | 1000 | 1000 | * | * | * |
| Burst rate (kbit/s) | Not applicable | Not applicable | Not applicable | Not applicable | * | * | * |

TABLE 4b (continued)

| System Parameter | A | B | C | D | E | F | G |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Link 1 | Link 1 |
| Transmission parameters (cont.) |  |  |  |  |  |  |  |  |
| Chip rate (Mchip/s) | Not applicable | ~2 | 2.56 | 1.228 | 1.9 and/or 7.6 | Not applicable | 0.15 | 2.4 |
| Interleaving | * | * | * | * | Varies | None | None | None |
| Voice activity factor | 0.4 | 0.5 | 0.4 | 0.4 | 0.4 | 0.4 | Not applicable | Not applicable |
| Required Eb/N0 (dB) | 6.1 | 4 | 2.8 | 3.5(3) | 4 (with no margin) | 2.5 | 33.6 | 36.4 |
| MES geographical distribution | Worldwide | Worldwide | To be determined | * | Varies | * | Multi-country | Multi-country |
| Range of pfd (dB(W/(m2 · 4 kHz))) | To be determined | To be determined | To be determined | To be determined | To be determined | To be determined | To be determined | To be determined |

TABLE 4b (end)

| System Parameter | H | R | K |  | N | O | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | I | M |  |  |  |
| Transmission parameters (cont.) |  | (2) |  |  |  |  |  |
| Chip rate (Mchip/s) | 6 | 7.5 | 8.1 | 8.1 | * | * | * |
| Interleaving | None | Not applicable | Not applicable | Not applicable | * | * | * |
| Voice activity factor | Not applicable | Not applicable | Not applicable | Not applicable | * | * | * |
| Required Eb/N0 (dB) | –8 | 6.5 | 7 | 7 | * | * | * |
| MES geographical distribution | Mulit-country | Worldwide | Regional | Worldwide | * | * | * |
| Range of pfd (dB(W/(m2 · 4 kHz))) | To be determined | Minimum: −144.8 Maximum: −132.0 | To be determined | To be determined | * | To be determined | To be determined |
| (1) Satellite antenna gains adjusted to maintain near-constant received power independent of range to user. (2) For system R, RF carrier spacing is 7.5 MHz and RF channel bandwidth 15 MHz. (3) Includes effect of feeder link. * Value requiring further study. |  |  |  |  |  |  |  |

Annex 2

Technical parameters of MSS satellite networks in the frequency bands below 1 GHz

TABLE 5

Parameters of several non-GSO MSS satellite networks with primary frequency allocations below 1 GHz

| System | L | M |  |  |  | N | P | Q |  | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Orbital parameters |  |  |  |  |  |  |  |  |  |  |  |
| Number of satellites | 48 |  |  |  |  | 3 | 6 | 32 |  | 6 |  |
| Altitude (km) | 950 | 825 |  |  | 775 | 800 | 893 | 1 000 |  | 692, 667 |  |
| Inclination (degrees) | 50 | 45 |  | 0 | 70, 108 | 88 | 99 | 51 | 83 | 98.04 |  |
| Orbit planes | 8 | 3 |  | 1 | 2 | 3 | 2 | 6 | 2 | 2 |  |
| Satellite/plane | 6 | 8 |  |  |  | 1 | 3 | 5 | 1 | 3 |  |
| Right ascension of ascending node (degrees) | 0, 45, 90, 135, 180, 225, 270, 315 | 0, 120, 240 |  | 0 | 0, 180 | 0, 15, 90 | 9.8 | 0, 60, 120, 180, 240, 300 | 0, 90 | 143.5, 53.5 |  |
| Subscriber uplink |  |  |  |  |  |  |  |  |  |  |  |
| Band (MHz) | 148-150.05(1) |  |  |  |  | 148-150.05 | 148-148.855 | 148-150.05(1) |  | 399.9-400.05 |  |
| Tx power (W) | 7 |  | 5 |  |  | 7 | 1 | 20 |  | 10 |  |
| Tx e.i.r.p. (dBW) | 8.5 |  | 7.5 |  |  | 11.5 | −3.8 | 12 |  | 16 |  |
| Maximum Tx antenna gain (dBi) | 0 |  | 0.5 |  |  | 3 | −3 | 0 |  | 7 |  |
| Channel bandwidth (kHz) | 15 |  | 5 |  |  | 30-90 | 855 | 25 |  | 150 |  |
| Rate (kbit/s) | 9.6/OQPSK |  | 2.4/SDPSK |  |  | 9.6, 19.2/FSK | 1/QPSK | 4.8, 9.6, 19.2/GMSK |  | 4.8/MSK |  |
| Polarization (Tx wave) | Linear |  |  |  |  | RHCP | LHCP | Linear |  | RHCP |  |
| Satellite Rx G/T (dB(K–1)) | −22.9 |  | −26 |  |  | −30 | −26.1 | T  940 K |  | −18.9 |  |
| Maximum Rx antenna gain (dBi) | −2 (gain at nadir) |  | 0 |  |  |  | 5.6 | 6 maximum; −3 at nadir |  |  | 7 |
| Rx antenna pattern | Isoflux |  | Toroidal, RHCP |  |  | 10 log (cos 2 ) | 10 log (cos 2 ) | Isoflux |  |  | Cardioide |
| C/(I  N) (dB) | 5.5 |  | 10.3 |  |  | 8 | Eb /(N0  I0) = 8.7 dB | Eb /N0 = 13.5 dB |  |  | Eb /N0 = 13.4 dB |

TABLE 5 (continued)

| System | L |  |  | M |  |  | N |  |  | P |  |  | Q |  |  |  |  | S |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subscriber downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Band (MHz) | 137-138 |  | 400.15-401 |  |  | 137-138 |  |  | 400.15-401 |  |  | 137.0725-137.9275 | 137-138 |  |  |  | 400.15-401 |  | 400.6-400.9 |
| Tx power (W) | 25 |  |  |  |  | 18.2 |  |  | 6.3 |  |  | 1 | 32 |  |  |  |  |  | 10 |
| Tx e.i.r.p. (dBW) | 19.7 |  |  |  |  | 13.6 |  |  | 10 |  |  | 3.8 | 17.8 |  |  |  |  |  | 16 |
| Maximum Tx antenna gain (dBi) | −2 (gain at nadir) |  |  |  |  | 1 |  |  | 2 |  |  | 4.9 | (6 maximum, isoflux, −3 at nadir) |  |  |  |  |  | 7 |
| Channel bandwidth (kHz) | 25 |  | 35 |  |  | 15/25 |  |  | 30-85 |  |  | 855 | 25 |  |  |  | 45 |  | 300 |
| Rate (kbit/s) | 24/OQPSK 9.6/FSK |  |  |  |  | 4.8/9.6/SDPSK |  |  | 9.6, 19.2/FSK |  |  | Regenerated | 4.8, 9.6, 19.2, 30/GMSK |  |  |  | 4.8, 9.6, 19.2/GMSK |  | 4.8/MSK |
| Polarization (Tx wave) | RHCP |  |  |  |  |  |  |  |  |  |  | LHCP | RHCP |  |  |  |  |  | LHCP |
| Subscriber Rx G/T (dB(K–1)) | −30.8 |  |  |  |  | −28.6 |  |  | −20.6 |  |  | −21.2 | T = 1 565 K |  | T = 505 K |  |  |  | −20.4 |
| Maximum Rx antenna gain (dBi) | 5.7 |  |  |  |  | 0.5 |  |  | 3 |  |  | −3 | 3 |  |  |  |  |  | 7 |
| C/(I  N) (dB) | 5.1 |  |  |  |  | Rec. ITU-R M.1232 |  |  | 8 |  |  | Eb /(N0  I0) = 3.7 dB | Eb /N0 = 13.5 dB |  |  |  |  |  | Eb /N0 = 13.4 dB |
| Gateway downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Band (MHz) |  | 400.15-401 |  |  | 137-138 |  |  | 400.15-401 |  |  | 137.0725-137.9275 |  |  | 137-138 |  | 400.15-401 |  |  | 400.6-400.9 |
| Tx power (W) |  | 15 |  |  | 4.9 |  |  | 6.3 |  |  | 1 |  |  | 32 |  |  |  |  | 10 |
| Tx e.i.r.p. (dBW) |  | 17.5 |  |  | 5.0 (peak) |  |  | 10 |  |  | 3.8 |  |  | 17.8 |  |  |  |  | 18 |
| Maximum Tx antenna gain (dBi) |  | 17 |  |  | 0 |  |  | 2 |  |  | 4.8 |  |  | (6 maximum isoflux, −3 at nadir) |  |  |  |  | 7 |

TABLE 5 (end)

| System | L | M | N | P | Q |  | S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gateway downlink (cont.) |  |  |  |  |  |  |  |
| Channel bandwidth (kHz) | 60 | 50 | 30-85 | 855 | 175 | 45 | 300 |
| Rate (kbit/s) | 50/OQPSK | 57.6/OQPSK | 9.6, 19.2, 38.4/FSK | Regenerated | 112/GMSK | 30/GMSK | 4.8/MSK |
| Polarization (Tx wave) | RHCP |  |  | LHCP | RHCP |  |  |
| Gateway Rx G/T (dB(K–1)) | −18.3 | −12.8 | −9.6 | −21.2 | T  1 565 K | T  505 K | −18.9 |
| Maximum Rx antenna gain (dBi) | 5.7 | 17, RHCP | 14 | 7.6 | 12 |  | 7 |
| C/(I  N) (dB) | 8.5 | Rec. ITU-R M.1232 | 8 | C/(N0  I0)  3.7 dB(Hz) | Eb /N0  13.5 dB |  | Eb /N0  13.4 dB |
| Gateway uplink |  |  |  |  |  |  |  |
| Band (MHz) | 148-150.05 |  |  | 148-148.855 | 148-150.05 |  | 399.9-400.05 |
| Tx power (W) | 1.2 | 250 (peak) | 5 | 1 | 150 |  | 10 |
| Tx e.i.r.p. (dBW) | 13.8 | 40 (peak) | 21 | 7.3 | 32.8 |  | 18 |
| Maximum Tx antenna gain (dBi) | 18 | 17 | 14 | 8.3 | 12 |  | 7 |
| Channel bandwidth (kHz) | 50 |  | 30-90 | 855 | 50 |  | 150 |
| Rate (kbit/s) | 50/OQPSK | 57.6/OQPSK | 9.6. 19.2/FSK | 1/QPSK | 30/GMSK |  | 4.8/MSK |
| Polarization (Tx wave) | RHCP |  |  | LHCP | RHCP |  |  |
| Satellite Rx G/T (dB(K–1)) | −22.9 | −33.3 | −30 | −23.1 | T  940 K |  | −20.4 |
| Maximum Rx antenna gain (dBi) | 16 | 0, RHCP | 0 | 5.7 | 6 maximum, isoflux, −3 at nadir |  | 7 |
| C/(I  N) (dB) | 8.5 | 10.6 | 8 | C/(N0  I0)  42.5 dB(Hz) | Eb /N0  13.5 dB |  | Eb /N0  13.4 dB |
| (1) MSS networks using dynamic channel assignment techniques, such as those described in Recommendation ITU-R M.1039. |  |  |  |  |  |  |  |

________________

____________________