RECOMMENDATION  ITU-R  M.1309*

DIGITALLY  CODED  SPEECH  IN  THE  LAND  MOBILE  SERVICE

(Question ITU-R 101/8)

(1997)

Rec. ITU-R M.1309

# Summary

Many new digital systems have been developed which are rapidly overtaking analogue systems for land mobile and cordless telephone voice communications. This Recommendation provides a brief description of speech coding technologies utilized in current and emerging systems.

The ITU Radiocommunication Assembly,

considering

a)	that there is rapid development in methods for the digitization of speech;

b)	that different methods of speech coding using differing bit rates and providing various qualities of speech are available;

c)	that the perceived quality of speech when using digitized speech transmission should be equal to or better than analogue in a comparable application;

d)	that the requirements on speech codec (coder/decoder) performance may differ between various applications;

e)	that there may be advantages in adopting the land mobile service standards that are compatible with ITU-T Recommendations relevant to the fixed networks;

f)	that different methods of optimizing coding of speech and channel coding or modulation altogether are available;

g)	that the ability of codecs to adapt their rates dynamically to varying transmission conditions could lead to significant spectrum efficiency gains;

h)	that the design of the codec has an effect on the power requirements;

j)	that there may be a need to compare several proposed codecs for any given application,

recommends

1	that the requirements on speech codec should be carefully matched to the application, taking into account the desired speech quality, the spectrum available for the system, the traffic capacity and other factors, such as the power consumption of the equipment;

2	that codecs have as low a bit rate as possible to give the required radio system performance without the need for excessive forward error correction (FEC). The combined codec and FEC rate must be considered when assessing spectrum efficiency;

3	that when appropriate, costs and development times can be reduced by using previously adopted standards, further work should be encouraged to develop such standards;

4	that where possible, codecs should make use of techniques, such as, varying bit rates to dynamically adapt to the transmission conditions;

5	that a methodology for assessing and quantifying the performance of codecs should be developed to facilitate fair and equitable comparisons;

5.1	the methodology should facilitate the selection of a codec taking into account the appropriate weightings for all system parameters associated with codecs. Some of the parameters that may be considered are:

–	speech performance;

–	codec bit rate;

–	codec complexity and cost;

–	processing power required to maintain real-time function;

–	the ability of the codec rate/FEC to adapt to varying radio environments, and the effect this has on spectrum efficiency and battery duty cycle/power consumption;

–	the compatibility of the codec with other radio systems (e.g. fixed);

–	the ease with which encryption can be added;

–	robustness;

–	performance in the presence of other signals and of background noise;

–	delay of the coder;

5.2	that consideration should be given to the above parameters to be used to construct a matrix which compares codec features pairwise as outlined in the International Mobile Telecommunications (IMT-2000) selection methodology (See Recommendation ITU-R M.1225). The correct implementation of the methodology would allow a framework to be designed to ensure the best system was selected for any given application;

6	that Annex 1 contains examples of those codecs used in the land mobile service which were presented to the ITU-R; they are given here for information.

ANNEX  1

Speech coding systems used in the land mobile service

|  | Cellular (TDMA) |  |  |
| --- | --- | --- | --- |
| System name | GSM/DCS/PCS (Full rate) | GSM/DCS/PCS (Half rate) | GSM EFR US PCS 1900 EFR |
| Origin | ETSI | ETSI | USA/ETSI |
| Frequency (GHz) | 0.9/1.8/1.9 | 0.9/1.8/1.9 | 0.9/1.9 |
| Modulation | GMSK | GMSK | GMSK |
| Maximum gross trns rate (kbit/s)(1) | 33.9 | 16.95 | 29.6 |
| Codec Type | RPE-LTP | VSELP | ACELP |
| Codec rate (kbit/s) | 13 | 5.6 | 12.2 |
| FEC (kbit/s)(2) | 9.8 | 5.8 | 10.6 |
| Codec Algorithmic delay (ms)(3) | 40 | 40 | 40 |
| Radio System delay (ms)(4) | 50 | 50 | 50 |
| Estimated Quality (MOS)(5), (6) | 3.6-3.8 | 3.5-3.7 | 4.1 |
| Estimated Processing(7) | 2.5 Mips | 17.5 Mips | 15.4 WMops |
| Reference Document | ETSI/ETS 300580 ANSI J-STD-007 | ETSI/ETS 300581 | ANSI J-STD-007A ETSI/ETS 300723 |

|  | Cellular (TDMA) |  |  |  |
| --- | --- | --- | --- | --- |
| System name | D-AMPS (Full rate) | D-AMPS (Enhanced) | PDC (Full rate) | PDC (Half rate) |
| Origin | USA | USA | Japan | Japan |
| Frequency (GHz) | 0.8 | 0.8/1.9 | 0.8-0.9/1.5 | 0.8-0.9/1.5 |
| Modulation | /4 DQPSK | /4 DQPSK | /4 DQPSK | /4 DQPSK |
| Maximum gross trns rate (kbit/s)(1) | 16.2 | 16.2 | 14 | 7.0 |
| Codec Type | VSELP | ACELP | VSELP | PSI-CELP |
| Codec rate (kbit/s) | 8.0 | 7.4 | 6.7 | 3.45 |
| FEC (kbit/s)(2) | 5.0 | 5.6 | 4.5 | 2.15 |
| Codec Algorithmic delay (ms)(3) | 28 | 25 | 47 | 85 |
| Radio System delay (ms)(4) | 48 | 45 | 75 | 130 |
| Estimated Quality (MOS)(5), (6) | 3.7 | 4.1 | 3.40 | 3.34 |
| Estimated Processing(7) | 22 Wmops | 14 Wmops | 7.8 Mops | 18.7 Mops |
| Reference Document | TIA/EIA IS-85 | TIA/EIA IS-641 | RCR STD-27 | RCR STD-27 |

|  | Cellular (CDMA) |  |  |
| --- | --- | --- | --- |
| System name | Composite CDMA/TDMA | CDMA (IS-96) | CDMA (IS-127) |
| Origin | USA | USA | USA |
| Frequency (GHz) | 1.9 | 0.8-0.9/1.85-1.99 | 0.8-0.9/1.85-1.99 |
| Modulation | OQPSK | QPSK | QPSK |
| Maximum gross trns rate (kbit/s)(1) | 10.4 | 28.8 | 28.8 |
| Codec Type | CELP like | CELP | RCELP |
| Codec rate (kbit/s) | 7.2 | 8/4/2/0.8 | 8/4/0.8 |
| FEC (kbit/s)(2) | 3.2 | 19.2 | 19.2 |
| Codec Algorithmic delay (ms)(3) | 26 | 27.5 | 30 |
| Radio System delay (ms)(4) | 60 | 37.5 | 40 |
| Estimated Quality (MOS)(5), (6) | > 4.0 | 3.3 | 4.1 |
| Estimated Processing(7) | 11 Mips | 22 Mops | 20 Mops |
| Reference Document | TIA/EIA IS-661 | TIA/EIA IS-96 | TIA/EIA IS-127 |

|  | Cordless |  |  |
| --- | --- | --- | --- |
| System name | CT2 | DECT | PHS |
| Origin | ETSI | ETSI | Japan |
| Frequency (GHz) | 0.86 | 1.9 | 1.9 |
| Modulation | GFSK | GFSK | /4) DQPSK |
| Maximum gross trans rate (kbit/s)(1) | 72 | 96 | 48 |
| Codec Type | ADPCM(8) | ADPCM(8) | ADPCM(8) |
| Codec rate (kbit/s) | 32 | 32 | 32 |
| FEC (kbit/s)(2) | 0 | 0 | 0 |
| Codec Algorithmic delay (ms)(3) | (9) | (9) | (9) |
| Radio System delay (ms)(4) | 0.375 | 0.375 | 0.25 |
| Estimated Quality (MOS)(5), (6) | 4.0 | 4.0 | 4.0 |
| Estimated Processing (7) | 10 Mips | 10 Mips | 1.0 Mops |
| Reference Document | Rec. ITU-T G.726 | Rec. ITU-T G.726 | Rec. ITU-T G.726 |

|  | Dispatch |  |  |  |
| --- | --- | --- | --- | --- |
| System name | Projet 25 | TETRA | IDRA | DIMRS |
| Origin | USA | ETSI | Japan | Canada |
| Frequency (GHz) | 0.15-0.9 | 0.4/0.9 | 0.8/1.5 | 0.8 |
| Modulation | C4FM/CQPSK | /4 DQPSK | M16QAM | M16QAM |
| Maximum gross trans rate (kbit/s)(1) | 9.6 | 9.0 | 10.7 | 10.7/21.33 |
| Codec Type | IMBE | ACELP | CELP/VSELP(10) | VSELP |
| Codec rate (kbit/s) | 4.4 | 4.567 | 4.7/4.2(10) | 4.2/8.0 |
| FEC (kbit/s)(2) | 2.8 | 2.633 | 2.766/3.177(10) | 3.177/6.756 |
| Codec Algorithmic delay (ms)(3) | 80 | 75 | 81/87 | 75/45 |
| Radio System delay (ms)(4) | 120 | 90 | 96/102(10) | 102/54.6 |
| Estimated Quality (MOS)(5), (6) | 3.4 | 3.3-3.5 | 3.2 | 3.20/3.98 |
| Estimated Processing(7) | 6.9 Mips | 15 Mips | 7.0/8.0 Mops(10) | 8.0 Mops |
| Reference Document | TIA/EIA IS-102.BABA | ETSI/ETS 300395 | RCR STD-32A | 68P81129E15 Motorola |

ACELP: algebraic code excited linear prediction ADPCM: adaptive differential pulse code modulation CDMA: code division multiple access CELP: code excited linear prediction CQPSK coherent quaternary phase shift keying C4FM: constant-envelope 4-level frequency modulation (FM)

DQPSK: differential quadriphase pulse shift keying GFSK: gaussian filtered frequency shift keying GMSK: gaussian filtered minimum shift keying IMBE: improved multiband excitation Mips: millions of instructions per second (required by codec) Mops: millions of operations per second (required by codec) MOS mean opinion score M16QAM: 16-states quadrature amplitude modulation OQPSK: offset quaternary phase shift keying PSI-CELP: peripheral subsystem interface – CELP QPSK: quadraphase shift keying RCELP: relaxed code excited linear prediction RPE-LTP: regular pulse excitation linear predictive coding (LPC) using long-term prediction TDMA: time division multiple access VSELP: vector sum excited linear prediction Wmops: weighted millions of operations per second (required by codec) (1) Is the maximum bit rate available to be allocated to a single user. (2) Includes only bits used for FEC excluding any other bits used for, such as, pilot or synchronization. (3) Codec Algorithmic Delay  Buffering Delay  Voice Encode  Voice Decode Delay. (4) Radio System Delay  Buffering Delay  Voice Encode Delay  FEC Encode Delay  Transmission Delay  FEC Decode Delay  Voice Decode Delay. The Radio System Delay does not include delays due to repeaters and networking. (5) Tested under no error condition. (6) MOS scores for a given codec can and do vary from test to test, based on different languages, populations, test labs and different ensembles of codecs. MOS scores from different listening tests are not directly comparable. It is recommended for future study that a more standardized voice quality testing procedure be developed. (Refer to ITU-T Recommendations P.830 regarding subjective test and P.861 regarding objective test for guidance on test methods.) (7) Processor dependant. (8) ITU-T Recommendation G.726 at 32 kbit/s. (9) Algorithmic delay is not applicable. (10) These items are not standardized and two types of coding techniques are being used in Japan.

*	This Recommendation should be brought to the attention of the Telecommunication Standardization Sector (ITU-T).