RECOMMENDATION  ITU-R  M.1390

METHODOLOGY  FOR  THE  CALCULATION  OF  IMT-2000
TERRESTRIAL  SPECTRUM  REQUIREMENTS

(1999)

Rec. ITU-R M.1390

# Introduction

IMT-2000 are third generation mobile systems which are scheduled to start service around the year 2000 subject to market considerations. They will provide access, by means of one or more radio links, to a wide range of telecommunication services supported by the fixed telecommunication networks (e.g. public-switched telephone network (PSTN)/integrated services digital network (ISDN)), and to other services which are specific to mobile users.

A range of mobile terminal types is encompassed, linking to terrestrial and/or satellite-based networks, and the terminals may be designed for mobile or fixed use.

Key features of IMT-2000 are:

–	high degree of commonality of design worldwide;

–	compatibility of services within IMT-2000 and with the fixed networks;

–	high quality;

–	use of a small pocket-terminal with worldwide roaming capability;

–	capability for multimedia applications and a wide range of services.

IMT-2000 are defined by a set of interdependent ITU Recommendations of which this one is a member.

Spectrum requirements for the terrestrial component of IMT-2000 were estimated in Report ITU-R M.1153 prior to WARC-92. Speech services were considered to be the major source of traffic at the time. As technological advancements provide additional capabilities in telecommunications users will demand more from wireless services. Future wireless services must support, not only speech but also a rich range of new services that will serve a wide range of applications. Services such as multimedia, Internet access, imaging and video conferencing will be needed in third generation wireless systems. In response to these new applications, IMT-2000 will support high rate data services. The provision of new services described in Recommendation ITU-R M.816 (Framework for services supported by IMT-2000) has an impact on the spectrum requirements for IMT-2000 systems.

There is a need to develop a new methodology for determination of spectrum requirements that can accommodate not only the new services of IMT-2000 but also the new radio transmission technologies being developed.

# Scope

This Recommendation contains a methodology for the calculation of terrestrial spectrum requirement estimates for IMT-2000. This methodology could also be used for other public land mobile radio systems. It provides a systematic approach that incorporates geographic influences, market and traffic impacts, technical and system aspects and consolidation of spectrum requirement results. The methodology is applicable to both circuit switched and packet switch-based radio transmission technologies and can accommodate services that are characterized by asymmetrical traffic flows1).

The ITU Radiocommunication Assembly,

considering

a)	that the Radio Regulations (RR) identify the bands 1 885-2 025 MHz and 2 110-2 200 MHz as intended for use on a worldwide basis by administrations wishing to implement IMT-2000, as indicated in RR S5.388, and Resolution 212 (Rev.WRC-97);

b)	that the initial implementations of IMT-2000 are expected to commence around the year 2000 subject to market considerations;

c)	that the bands identified in considering a) are used differently in various countries;

d)	that the traffic and service mix carried by IMT-2000 systems may vary from country to country, and also within countries. In some parts of the world additional spectrum may be required, whilst in other parts of the world frequency bands identified by considering a) could be adequate to meet IMT-2000 services present and future demands;

e)	the need to support the operation of IMT-2000 terminals in different regulatory environments;

f)	that the various radio access technologies that may be appropriate for IMT-2000 may have different channel bandwidth requirements, and hence varying impact on the basic frequency usage possibilities;

g)	that traffic handled by mobile systems as well as the number and diversity of services will continue to grow;

h)	that future systems may include the use of a range of cell types from indoor cells to satellite cells, which must be able to co-exist in a given location;

j)	that IMT-2000 will offer higher data rate services than earlier systems in order to meet increasing customer demands, and this could create a demand for additional spectrum beyond that earlier estimated;

k)	that efficiency of spectrum use requires consideration of the balances between IMT-2000 system costs and bandwidth needed;

l)	that the methodology in Annex 1 is considered flexible enough to accommodate either a global view or the unique requirements of regional markets relative to terrestrial spectrum needs,

recommends

1	that the methodology for the calculation of terrestrial IMT-2000 spectrum requirement estimates as specified in Annex 1 should be used by administrations as the basis for performing calculations involving estimates of future IMT-2000 terrestrial spectrum needs;

2	that the methodology in Annex 1 could also be considered for the calculation of terrestrial spectrum estimates for other public land mobile radio systems, and its use is highly encouraged.

ANNEX  1

IMT-2000 terrestrial spectrum requirement methodology

# 1	Terrestrial methodology overview

A methodology for development of a terrestrial spectrum requirement is presented below. This methodology enables the calculation of spectrum estimates to support mobile communication services of today and the future. The equation for this estimate is provided in equation 1.

This methodology is consistent with the global IMT-2000 vision and is also consistent with the services as presented in Recommendation ITU-R M.816: “Framework for services supported by IMT-2000”. The methodology is flexible enough to accommodate either a global view of spectrum needed or the unique requirements of regional markets.

The basic theme of this methodology is to determine the individual spectrum requirements for all representative combinations of specific environments and services (Fes) in a given geographical area, and to combine the set of individual spectrum requirements Fes together into a total terrestrial component spectrum requirement estimate, FTerrestrial by employing appropriate weighting factors (aes) to the summation. The factor (aes) takes into account the impact of concurrent services in a given geographical area. An additional adjustment factor () is available to apply to the composite summation to accommodate impacts such as multiple operators, spectrum sharing, and the like.

The estimation of a spectrum requirement for many years into the future is not an exact calculation. In particular, the methodology provided in this document is not intended to include the second or third order effects, but rather the calculations capture the significant first order influences which are the primary factors for terrestrial spectrum needed.

The spectrum required (FTerrestrial) in MHz is:

FTerrestrial     es Fes     es Tes/Ses	(1)

where “e” and “s” are subscripts denoting dependency on environments and services respectively.

Therefore, FTerrestrial is the total required spectrum as a weighted summation of co-existing individual Fes in the same geographical area for all environments “e” and services “s” considered relevant, adjusted for influences such as spectrum sharing, multiple operators,

where:

FTerrestrial	  Terrestrial Component Spectrum Requirement	Units: MHz

Tes	  Traffic/Celles	Units: Mbit/s/cell

Ses	  System capability 	Units: Mbit/s/MHz/cell

aes	  Weighting factor	Units: dimensionless

b	  Adjustment factor	Units: dimensionless

Equation 1 addresses both circuit and packet-switched services and includes consideration for traffic asymmetry in the uplink and downlink directions. Each of the factors of equation 1 will be defined further in the following subsections.

The calculations, parameters, and definition of inputs within the methodology are divided into four categories and serve to group similar aspects of the methodology into sub-units:

A	geographic considerations,

B	market and traffic considerations,

C	technical and system consideration,

D	spectrum results considerations.

An example is included in Appendix 1 that shows how the methodology is applied. This example is based on a representative subset of environments and services. The example is calculated with parameter values estimated from market research on public land mobile communications services, including IMT-2000, and with technical parameter values estimated from IMT-2000 radio transmission technologies, for the year 2010.

The results shown in this example should not be considered as providing an answer to the question of future spectrum requirements for public land mobile communications services, including IMT-2000, as all environments and services that must be considered for completeness have not been included in the example. Nonetheless, the example includes all environments and services required to sufficiently exercise all aspects of the methodology.

# 2	Methodology flowchart

The following material presents the methodology in “flowchart” format with a sequential listing of the steps divided among the four sub-categories. Subsequent sections of this document provide detailed information and description of the terms, parameters, calculations performed2).

# A	Geographic considerations

## A1	Select “e”

“e” - environment type: selects density and mobility.

These environments are defined by a combination of a density attribute and a mobility attribute considered jointly, and are shown in the following matrix:

| Mobility | In-building | Pedestrian | Vehicular |
| --- | --- | --- | --- |
| Density |  |  |  |
| Dense Urban (CBD) |  |  |  |
| Urban |  |  |  |
| Suburban |  |  |  |
| Rural |  |  |  |

For example, “dense urban, in-building” could be a value of “e”.

## A2	Select direction of calculation

Uplink (from the mobile station to the base station) or downlink (from the base station to the mobile station).

## A3	Establish representative cell area and geometry	Units: metres

Diameter if circular omnidirectional cell geometry; radius of vertex if sectored hexagonal cell geometry.

## A4	Calculate Cell_Area Ae 	Units: km2

Cell_Areae.

# B	Market and traffic considerations

## B1	Select “s”

s – service type: selects service type and hence Net_User_Bit_Rates (kbit/s)

## B2	Establish Population_Densitye 	Units: potential users/km2

## B3	Establish penetration_ratees 	Units: %

## B4	Calculate users/celles 	Units: users

Users/Celle s  Population_DensityePenetration_RateesCell_Areae.

## B5	Establish traffic parameters

Busy_Hour_Call_Attemptses	Units: calls in busy hour

Effective_Call_Durationes	Units: seconds

Activity_Factores			Units: dimensionless

## B6	Calculate Traffic/Useres	Units: call-seconds

Traffic/Useres  Busy_Hour_Call_AttemptsesCall_Durationes Activity_Factores.

(NOTE – May be expressed as Erlangs, where an Erlang  call-seconds/3 600.)

## B7	Calculate Offered_Traffic/Celles	Units: call-seconds/cell

Offered_Traffic/Celles  Traffic/UseresUsers/Celles.

(NOTE – May be expressed as Erlangs, where an Erlang  call-seconds/3 600.)

## B8	Establish Quality_of_Service_Functiones Parameters	Units: varied

Group_Sizees;

Blocking Criterias {Formula and Grade of Service for circuit switched; Formula and Delay for packet switched}.

# C	Technical and system considerations

## C1	Calculate number of Service_Channels/Celles
required to carry Offered_Traffic/Celles	Units: none

Service_Channels/Celles 

(Quality_of_Service_Functions 
						{Offered_Traffic/Celles*Group_Sizees; 
						Blocking Criterias})/Group_Sizees

## C2	Determine Service_Channel_Bit_Ratees
needed to carry Net_User_Bit_Rates	Units: kbit/s

## C3	Calculate Traffices	Units: Mbit/s/cell

Tes  Service_Channels/CellesService_Channel_Bit_Ratees.

(Note conversion to Mbit/s from kbit/s.)

## C4	Determine Net_System_Capabilityes Parameters	Units: varied

System Spectral Efficiency; Coding Factor; Overhead Factor; Deployment Model; and other factors.

## C5	Calculate Net_System_Capabilityes	Units: Mbit/s/MHz/cell

Ses  Function of {Spectral Efficiency; Coding Factor; Overhead Factor; Deployment Model, and other factors}.

# D	Spectrum results considerations

## D1	Calculate individual Fes Component

(Answer will be for direction of calculation chosen either uplink or downlink.)

Fes  Tes/Ses 	(either uplink or downlink)	Units: MHz

## D2	Repeat process for calculation of other direction (either downlink or uplink as appropriate)

Repeat steps A2 through D1.

## D3	Calculate Fes for the Service “s” Combining uplink and downlink components

Fes  (Fes uplink  Fes downlink)	Units: MHz

## D4	Repeat process (steps A1 through D3) for All Desired “e”, “s”

## D5	Determine weighting factor applicable to
each individual Fes: es	Units: None

## D6	Determine Adjustment Factor(s): 	Units: None

## D7	Calculate Final Total FTerrestrial Spectrum Value	Units: MHz

FTerrestrial    es Fes.

# 3	Detailed description of the methodology

# A	Geographic considerations

## A1	Environment

The initial point for consideration of terrestrial spectrum requirements is to determine the characteristics of the cells which the system will use. The system will operate in a variety of scenarios, encompassing various combinations of density and mobility. A table of possible environments is given below, although no indication has been given of which specific environments should be considered. It is thought that the matrix below is flexible enough to cover most situations encountered in deployment of a public land mobile radio system.

The variable subscript “e” represents the environment for which the calculation is performed, and the environment is defined by a combination of a density attribute and a mobility attribute considered jointly, and are show in the following matrix:

| Mobility | In-building | Pedestrian | Vehicular |
| --- | --- | --- | --- |
| Density |  |  |  |
| Dense Urban (CBD) |  |  |  |
| Urban |  |  |  |
| Suburban |  |  |  |
| Rural |  |  |  |

For example, “dense urban, in-building” could be a value of “e”.

Clearly some of these environments may be (geographically) overlapping, whilst others may be separate. For the calculation of the total spectrum required for IMT-2000, it will be necessary to determine the maximum spectrum which might realistically be needed in any one area. It is anticipated that not all combinations (values of “e”) will be needed and in most cases only a few combinations will need to be considered. For example, “dense-urban, vehicular” as a value of “e” may not be required in practice in some calculations. Therefore the first stage of the methodology is to determine the environments which could co-exist, and which would give rise to the greatest total spectrum demand.

In practice this will be a combination of overlapping dense urban and urban environments. The method to determine the total spectrum required is then applied to each of the members of this set of overlapping environments.

## A2	Select direction of calculation

Uplink (from the mobile station to the base station ) or downlink (from the base station to the mobile station).

The traffic and spectrum figures in steps A2 through D1 are calculated separately for uplink and downlink directions because of the traffic asymmetry in some services. The spectrum required for any Fes is the sum of the requirement for both directions.

## A3	Establish representative Cell_Area and geometry

For each of the “e” environments identified in A1, the cell area and geometry has to be established. Typical examples could be a circle or hexagon, either of which could be considered as a whole or could be sectored. It is possible that, for operational reasons, different environments will use cells with differing geometry, and certainly there may be a range of cell sizes.

## A4	Calculate Cell_Areae	Units: km2

Having identified the cell geometry and dimensions for each environment, it is necessary to calculate the area of the cell.

For example:

For a circular cell, Cell_Area e  p R2  p D2/4

where:	R  radius of the circle

D  diameter of the circle

For a hexagonal cell, Cell_Area e  (3/2)  (Ö3)  R2

where: R  radius (to vertex) of the hexagon.

For a cell which is a sector of a circle/hexagon, the area that should be used (Cell_Areae) is the area of the sector, and it may be sufficient to divide the area of the full circle/hexagon to obtain the sector area.

Other cell geometries and corresponding formula for calculating area may be used.

# B	Market and traffic considerations

## B1	Select “s”

“s” – service type: selects service type and hence Net_User_Bit_Rates (kbit/s).

For a given public land mobile radio service there is a set of services that are offered. Selecting a service type “s” chooses a particular service from that set for the purpose of calculation.

As an example, in IMT-2000, a reasonable set of services (the range of “s”) might be:

–	Speech	(circuit switched)

–	Simple message	(packet switched)

–	Switched data	(circuit switched)

–	Medium multimedia	(packet switched)

–	High multimedia	(packet switched)

–	High interactive multimedia	(packet switched)

## B2	Establish Population_Densitye	Units: potential users/km2

For each environment considered, it is also necessary to determine a density of population. This will be a basic figure for the number of persons per unit area within the environment under consideration.

Similar geographic locations can have differing population densities as a function of the mobility component. For example, urban-pedestrian may have a population density of 100 000 users/km2, yet the same area would not be physically able to have an urban-vehicular density of more than 3 000 users/km2.

## B3	Establish Penetration_Ratees 	Units: %

This parameter is the ratio of the number of people subscribing to the service “s” over the total population, in environment “e”.

It should be noted that the use of each service is not exclusive. Each Penetration_Ratee refers to the penetration of that service as a proportion of the total potential user base. Since users can use more than one service it is possible for the total penetration in an environment (across all services) to exceed one (100%) if a high proportion of users are using more than one service.

## B4	Calculate Users/Celles

This parameter is dependent upon the population density and the cell area for each environment “e”, and on the penetration rate for the service “s” and the environment “e”.

It represents the number of people actually subscribing to the service “s” in a cell of environment “e”.

Users/Celles  Population_Densityes  Penetration_Ratees  Cell_Areae.

## B5	Establish traffic parameters

For each service, in each environment, the following parameters must be established:

Busy_Hour_Call_Attemptses 	Units: number of
calls in busy hour

Defined as the average number of calls attempted for the average user during the busy hour. It should be noted that these calls may originate either from the user or from the network. No distinction is made here between these two sources, the result in terms of resource needed being the same. This parameter is self explanatory for circuit-switched services, and for packet-switched services a call is understood as a session.

Call_Durationes			Units: seconds

This parameter is defined as the mean actual duration of the call or of the session during the busy hour.

Activity_Factores			Units: dimensionless

Defined as the percentage of time during which the resource is actually used during the call. For example, if voice is transmitted only if the user speaks, or if a packet transmission is bursty, the transmission is only active during a relatively small amount of time.

## B6	Calculate Traffic/Useres	Units: call-seconds

This parameter is defined as the probability that the user is “offhook” and active in the busy hour for a circuit-switched call or a packet-switched session. It is clearly defined in Erlangs (call-seconds/ 3 600) for circuit-switched services and for packet-switched services has the equivalent unit of average relative activity in a period of reference of the busy hour.

Traffic/Useres  Busy_Hour_Call_Attemptses  Call_Durationes  Activity_Factores.

## B7	Calculate Offered_Traffic/Celles 	Units: call-seconds

This is the total traffic issued in a given cell of environment “e” for service “s” during the busy hour.

Offered_Traffic/Celles  Traffic/Useres  Users/Celles.

It is clearly defined in Erlangs (call-seconds/3 600) for circuit-switched services and for packet-switched services has the equivalent unit of average relative activity in a period of reference of the busy hour.

## B8	Establish Quality_of_Service_Functiones (QOSes) Parameters	Units: varied

Parameters required:

Group_Sizees

Blocking Criterias (Formula and Blocking for circuit switched)

or

Blocking Criterias (Formula and Delay for packet switched)

Discussion of Quality Of Service Aspects:

Bearer channel capabilities are characterized in terms of parameters having Quality of Service and Grade of Service significance. Establishing Quality_of_Service_Functiones parameter values (herein Quality_of_Service_Functions is used generically to apply to both Quality of Service specifically and also to Grade of Service) directly impacts on the number of service channel resources that are required to transport the User_Net_Bit_Rates streams.

These parameters are necessary to determine the actual amount of resource which is needed to carry the traffic issued from the cell. For circuit-switched services, the necessary parameter is acceptable blocking, the maximum percentage of calls which cannot be treated by the network.

For packet-switched services, the quality of service is defined in terms of maximum packet delay and packet loss probability. The acceptable values of these parameters must be established for a given Service_Type “s”.

The throughput of a packet-switched system is dependent on the choice of a suitable multiple access protocol (e.g. Aloha, PMRA, etc.). Given a particular protocol, the total throughput may be determined for a particular Service_Type “s” by application of a suitable traffic model and an appropriate packet Quality_of_Service_Functiones.

Traffic models for packet switched are dependent on many parameters, some of which might be included in the Net_System_Capabilityes variable. Some example packet-switched traffic parameters are:

–	statistical arrival times of various sessions;

–	numbers of packet bursts per session;

–	arrival times of packet bursts within a session;

–	packet size statistics.

The values of the above are also Service_Type “s” dependent. When a session consists of multiple services an aggregate traffic model should be used.

The requisite function that is used for the calculation of Quality_of_Service_Functiones relative to the number of service channels is a matter of choice of the appropriate function to match the Service_Type “s” selected. For example, Erlang B with a blocking value of some percentage (say 2% blocking) has traditionally been used for speech (circuit switched) and may be an appropriate choice to apply in the determination of spectrum associated with the speech service type. Other functions as discussed previously, which describe Quality_of_Service_Functiones appropriate to packet switched would be used in calculations of packet-based service types.

In consideration that radio transmission technologies and system deployments may provide some measure of traffic “sharing” or “redirection” among adjacent cells (perhaps in hierarchical or other arrangements) it is appropriate to consider traffic and Quality_of_Service_Functiones within a grouping of cells.

The term Group_Sizees is used to describe the number of cells considered to be grouped for the purpose of application of traffic and Quality_Of_Service_Functiones. The Group_Sizees does not imply any particular geometry, although an example could be a regular hexagonal cell grid which results in a Group_Sizees of seven, the value arising from the cell in question and the surrounding six first tier cells.

Essentially, the Traffic/Celles is multiplied by the Group_Sizees and the Blocking_Criterias function applied over this grouping. Then to obtain the Service_Channels/Celles the Group_Sizees is divided out to restore the valuation to a per cell basis.

This calculation step has the impact of some reduction in the number of Service_Channels/Celles by considering some improvement in efficiency in the traffic spread in a geographic grouping. To the extent that grouping and/or traffic sharing across geographically grouped cells is included in the System_Capabilityes parameter, then the Group_Sizees should be set to the value of one and the Blocking_Criterias function calculation performed on the traffic in a single cell.

Similarly, if the effects of a Quality_of_Service_Functiones are included in the System_Capabilityes parameter, then the Blockings function should be set to a value of one, which should in principle, also require that the Group_Sizees value be set to unity.

# C	Technical and system considerations

## C1	Calculate number of Service_Channels/Celles required
to carry Offered_Traffic/Celles	Units: none

The calculation of the number of Service_Channels/Celles is a complex function that involves use of the parameters discussed previously:

Service_Channels/Celles  (QOSes {Offered_Traffic/Celles*Group_Sizees; 
		Blocking Criterias})/Group_Sizees

Service_Channels/Celles is the actual number of “channels” that must be provisioned to carry the intended traffic. A service channel is a channel which supports a service needing to have transported the corresponding Net_User_Bit_Rates for the selected service “s”.

In general terms, a physical transmission facility offers a corresponding physical bit rate, which may be sub-divided into several sub-rate transmission pipes, each of which can support a number of service channels.

## C2	Determine Service_Channel_Bit_Ratees needed
to carry Net_User_Bit_Rates	Units: kbit/second

Due to modularity of the bit rate of the service channel, it is possible that the Service_Channel_Bit_Ratees might be equal to or greater than the corresponding Net_User_Bit_Ratees. An example of this would be a Service_Channel_Bit Ratees of 16 kbit/s to carry a 14 kbit/s Net_User_Bit_Rates, or an 80 kbit/s Service_Channel_Bit_Ratees to carry a 64 kbit/s Net_User_Bit_Rates.

Service_Channel_Bit_Ratees can also include impacts related to coding factors and channel overhead. To the extent that the actual bit rate of the service channel, coding factors and channel overhead impacts are not included in the Net_System_Capabilityes. they should be included here. Ignoring any factors related to Service_Channel_Bit_Ratees that cause it to be greater than the Net_User_Bit_Rates, the Service_Channel_Bit_Ratees is merely equal to the Net_User_Bit_Rates.

## C3	Calculate Traffices	Units: Mbit/s/cell

(Note conversion to Mbit/s from kbit/s.)

Tes  Service_Channels/CellesService_Channel_Bit_Ratees.

At this stage the traffic has been totalled for all the factors represented by the environment, service type, selected direction of transmission, cell geometry, quality of service aspects, traffic efficiencies across a group of cells, and service channel bit rate requirements.

## C4	Net_System_Capabilityes (Ses)	Units: Mbit/s/MHz/cell

(An equivalent expression for bits/s/Hz/cell.)

Determine Net_System_Capability Parameters	Units: varied

Ses is a measure of the system capacity of a specific technology. It is related to the spectral efficiency of mobile communication systems but contains many other factors. Ses has the unit dimension of Mbit/s/MHz/cell. which is a direct equivalent to bits/s/Hz/cell. Net_System_ Capabilityes is not the same as spectral efficiency of the radio transmission technology. It is comprised of a number of effects that are combined in a complex manner appropriate to the radio transmission technology and the service type “s” and environment “e”. Often the values required to determine the Net_System_Capabilityes are obtained from the results of complex system simulations.

The major components of the Net_System_Capabilityes may include the following:

1)	Radio transmission technology design or engineering impacts including but not limited to:

–	physical spectral efficiency of access technology used;

–	requirements of a specific Eb/N0;

–	requirements of a specific C/I;

–	requirements for a specific frequency reuse plan;

–	coding factors used by the radio transmission technology;

–	overhead factors used by the radio transmission technology;

–	environment – indoor, outdoor, stationary, pedestrian, vehicular.

2)	Deployment models and/or deployment technique including microcells, macrocells, hierarchical cells, or overlay cells, etc.

It therefore follows that there is a tradeoff between Net_System_Capabilityes and the quality or grade of service.

## C5	Calculate Net_System_Capabilityes

Ses  Function of {Spectral Efficiency, Coding Factor, Overhead Factor, Deployment Model, and other factors}.

This calculation proceeds using the values and parameters discussed previously as a function of appropriate combining functions.

# D	Spectrum results considerations

## D1	Calculate individual Fes Component for a given direction

–	uplink (from the mobile station to the base station); or

–	downlink (from the base station to the mobile station).

The amount of spectrum required for a given service and environment, in a given direction, is determined by dividing the Traffices (as determined in § C3) by the Net_System_Capabilityes (as determined in § C4).

Fes = Tes/Ses (either uplink or downlink)	Units: MHz

## D2	Repeat process for calculation of other direction (either downlink or uplink as appropriate)

Repeat steps A2 through D1 for the other direction (if not previously calculated).

## D3	Calculate Fes for the Service “s”, combining uplink and downlink components

The total amount of spectrum required for a given service and environment is determined by directly adding the spectrum required for the uplink and downlink components.

Fes = (Fes uplink  Fes downlink)	Units: MHz

## D4	Repeat process (steps A1 through D3) for all desired “e”, “s”

Repeat steps A1 through D3 for each combination of “e” and “s” that is being considered.

## D5	Determine weighting factor (aes) applicable to each individual Fes	Units: None

The weighting factor (aes) provides appropriate weighting in the spectrum requirements calculations and includes the following:

–	weighting to adjust for geographical offsets in overlapping environments;

–	weighting to correct for non-simultaneous busy hour traffic requirements.

The value for aes may range from zero up to unity, and the default value is 1.

## D6	Determine adjustment factor (b)	Units: None

The adjustment factor () provides for impacts such as:

–	multiple operators (reduced trunking/spectral efficiency);

–	sharing with other IMT-2000 services/systems;

–	sharing with non-IMT-2000 services/systems;

–	guard bands;

–	technology modularity. For example, if a technology uses 10 MHz frequency division duplex (FDD) channels, then the requirements will necessarily be an integer factor of 20 MHz;

–	other adjustments to be justified.

This adjustment factor is an approximation across impacts of environments “e”, services “s” and other influences. The default value for  is 1, and other values should be technically justified.

## D7	Calculate the final total FTerrestrial Spectrum Value	Units: MHz

For each environment and service, each Fes is multiplied by es, and then the individual products are added together. The result of the summation is multiplied by the adjustment factor () to derive the total terrestrial spectrum required FTerrestrial.

FTerrestrial   es Fes     MHz

APPENDIX  1

Example calculations

This example provides guidance on the application of the methodology detailed in §§ 2 and 3 of Annex 1.

The example is calculated with parameter values estimated from market research on public land mobile communications services, including IMT-2000, and with technical parameter values estimated from IMT-2000 radio transmission technologies, for the year 2010 view. The results shown in this example should not be considered as providing an answer to the question of future spectrum requirements for public land mobile communications services, including IMT-2000, as all environments and services that must be considered for completeness may have not been included in the example.

Examination of environments and services reveals that there are potentially twelve values of the subscript “e” and six values for the subscript “s” that are representative major contributors to a spectrum requirement. Hence, a complete calculation of the terrestrial spectrum requirement estimate for public land mobile communications services, including IMT-2000, would require the use of 72 terms in the summation of individual Fes terms.

This example is based on a representative subset of environments and services as presented in the matrix below and these are sufficient to exercise all aspects of the methodology.

Representative environments “e” and services “s” (values for Fes )

| Environments “e” | High density-in building (CBD) | Urban pedestrian | Urban vehicular |
| --- | --- | --- | --- |
| Services “s” |  |  |  |
| Speech (S) | Fes | Fes | Fes |
| Simple Message (SM) | Fes | Fes | Fes |
| Switched Data (SD) | Fes | Fes | Fes |
| Medium Multimedia (MMM) | Fes | Fes | Fes |
| High Multimedia (HMM) | Fes | Fes | Fes |
| High Interactive Multimedia (HIMM) | Fes | Fes | Fes |

The figures presented in the tables below are often rounded figures, but the calculation is performed with more digits to provide more accurate example results.

# A	Example for the year 2010

## A1	Environment

A subset of all environments is considered for the purpose of this example: only high density-in building also generally known as Central Business District (CBD); urban pedestrian, and urban vehicular. This subset of three environments is extracted from all possibilities because they correspond to superimposed layers in city centres.

It should be noted that no user should occupy two operational environments at a time.

## A2	Direction of calculation

–	uplink (from the mobile station to the base station); or

–	downlink (from the base station to the mobile station).

The following calculations are detailed for each of the direction.

## A3	Establish representative cell_area and geometry

The environments are defined to have the following geometry:

TABLE  1

Environment description

| Environment “e” | High density-in building (CBD) | Urban pedestrian | Urban vehicular |
| --- | --- | --- | --- |
| Geometrye | Circular | Hexagonal with three sectors | Hexagonal with three sectors |
| Cell dimensione | Diameter  100 m | Radius  600 m | Radius  600 m |

## A4	Calculate Cell_Area

Based on the cell description, the cell areas are calculated as follows:

TABLE  2

Cell_Areae

| Environment “e” | High density-in building (CBD) | Urban pedestrian | Urban vehicular |
| --- | --- | --- | --- |
| cell_areae (km2) | 7.85  10–3 | 3.12  10–1 | 3.12  10–1 |

## B1	Select service “s”

The services are as follows:

TABLE  3

Description of Service_Type “s” and corresponding Net_User_Bit_Rates

| Net_User_Bit_Ratee | Downlink (DL) net bit rate (kbit/s) | Uplink (UL) net bit rate (kbit/s) |
| --- | --- | --- |
| Service Type “s” |  |  |
| Speech (S) | 16 | 16 |
| Simple Message (SM) | 14 | 14 |
| Switched Data (SD) | 64 | 64 |
| Medium Multimedia (MMM) | 384 | 64 |
| High Multimedia (HMM) | 2 000 | 128 |
| High Interactive Multimedia (HIMM) | 128 | 128 |

## B2	Establish Population_Density

For the three environments considered, population densities can be chosen as follows:

TABLE  4

Population_Densitye

| Environment “e” | High density-in building (CBD) | Urban pedestrian | Urban vehicular |
| --- | --- | --- | --- |
| Population_Densitye | 250 000 | 100 000 | 3 000 |

## B3	Establish Penetration_Ratees

The following table describes the penetration rates used in this example calculation:

TABLE  5

Penetration_Ratees in Percent

| Environments “e” | High density-in building (CBD) | Urban pedestrian | Urban vehicular |
| --- | --- | --- | --- |
| Services “s” |  |  |  |
| Speech (S) | 73% | 73% | 73% |
| Simple Message (SM) | 40% | 40% | 40% |
| Switched Data (SD) | 13% | 13% | 13% |
| Medium Multimedia (MMM) | 15% | 15% | 15% |
| High Multimedia (HMM) | 15% | 15% | 15% |
| High Interactive Multimedia (HIMM) | 25% | 25% | 25% |

## B4	Calculate Users/Celles

The table below gives the Users/Celles calculated with the above assumptions:

TABLE  6

Users/Celles

| Environments “e” | High density-in building (CBD) number of users | Urban pedestrian number of users | Urban vehicular number of users |
| --- | --- | --- | --- |
| Services “s” |  |  |  |
| Speech (S) | 1 433 | 22 756 | 683 |
| Simple Message (SM) | 785 | 12 469 | 374 |
| Switched Data (SD) | 255 | 4 052 | 122 |
| Medium Multimedia (MMM) | 295 | 4 676 | 140 |
| High Multimedia (HMM) | 295 | 4 676 | 140 |
| High Interactive Multimedia (HIMM) | 491 | 7 793 | 234 |

## B5	Establish traffic parameters

The following traffic parameters are considered representative of the average user in each of the environment for each of the services:

TABLE  7

Busy_Hour_Call_Attemptes expressed as calls in busy hour

| Environments “e” | High density-in building (CBD) calls in busy hour |  | Urban pedestrian calls in busy hour |  |  | Urban vehicular calls in busy hour |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Services “s” |  |  |  |  |  |  |  |  |
| Speech (S) | 0.9 |  |  | 0.8 |  |  | 0.4 |  |
| Simple Message (SM) | 0.06 |  |  | 0.03 |  |  | 0.02 |  |
| Switched Data (SD) | 0.2 |  |  | 0.2 |  |  | 0.02 |  |
| Medium Multimedia (MMM) | 0.5 |  |  | 0.4 |  |  | 0.008 |  |
| High Multimedia (HMM) | 0.15 |  |  | 0.06 |  |  | 0.008 |  |
| High Interactive Multimedia (HIMM) | 0.1 |  |  | 0.05 |  |  | 0.008 |  |

TABLE  8

Call_Durationes in seconds

| Environments “e” | High density-in building (CBD) seconds |  | Urban pedestrian seconds |  |  | Urban vehicular seconds |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Services “s” |  |  |  |  |  |  |  |  |
| Speech (S) | 120 |  |  | 120 |  |  | 120 |  |
| Simple Message (SM) | 30 |  |  | 30 |  |  | 30 |  |
| Switched Data (SD) | 156 |  |  | 156 |  |  | 156 |  |
| Medium Multimedia (MMM) | 13.9 |  |  | 13.9 |  |  | 13.9 |  |
| High Multimedia (HMM) | 53.3 |  |  | 53.3 |  |  | 53.3 |  |
| High Interactive Multimedia (HIMM) | 180 |  |  | 180 |  |  | 180 |  |

TABLE  9

Activity_Factores

| Environments “e” | High density-in building (CBD) dimensionless |  | Urban pedestrian dimensionless |  | Urban vehicular dimensionless |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Simple Message (SM) | 1 | 1 | 1 | 1 | 1 | 1 |
| Switched Data (SD) | 1 | 1 | 1 | 1 | 1 | 1 |
| Medium Multimedia (MMM) | 1 | 1 | 1 | 1 | 1 | 1 |
| High Multimedia (HMM) | 1 | 1 | 1 | 1 | 1 | 1 |
| High Interactive Multimedia (HIMM) | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 |

## B6	Calculate Traffic/Useres

TABLE  10

Traffic/Useres in call-seconds

| Environments “e” | High density-in building (CBD) call-seconds |  | Urban pedestrian call-seconds |  | Urban vehicular call-seconds |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 54 | 54 | 48 | 48 | 24 | 24 |
| Simple Message (SM) | 1.8 | 1.8 | 0.9 | 0.9 | 0.6 | 0.6 |
| Switched Data (SD) | 31.2 | 31.2 | 31.2 | 31.2 | 3.12 | 3.12 |
| Medium Multimedia (MMM) | 6.95 | 6.95 | 5.56 | 5.56 | 0.111 | 0.111 |
| High Multimedia (HMM) | 8 | 8 | 3.2 | 3.2 | 0.427 | 0.427 |
| High Interactive Multimedia (HIMM) | 14.4 | 14.4 | 7.2 | 7.2 | 1.15 | 1.15 |

## B7	Calculate Offered_Traffic/Celles

TABLE  11a

Offered_Traffic/Celles in call-seconds

| Environments “e” | High density-in building (CBD) call-seconds |  | Urban pedestrian call-seconds |  | Urban vehicular call-seconds |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 7.74  104 | 7.74  104 | 1.09  106 | 1.09  106 | 1.64  104 | 1.64  104 |
| Simple Message (SM) | 1.41  103 | 1.41  103 | 1.12  104 | 1.12  104 | 2.24  102 | 2.24  102 |
| Switched Data (SD) | 7.96  103 | 7.96  103 | 1.26  105 | 1.26  105 | 3.79  102 | 3.79  102 |
| Medium Multimedia (MMM) | 2.05  103 | 2.05  103 | 2.60  104 | 2.60  104 | 1.56  101 | 1.56  101 |
| High Multimedia (HMM) | 2.36  103 | 2.36  103 | 1.50  104 | 1.50  104 | 5.98  101 | 5.98  101 |
| High Interactive Multimedia (HIMM) | 7.07  103 | 7.07  103 | 5.61  104 | 5.61  104 | 2.69  102 | 2.69  102 |

The group size is selected to be equal to 7. The Offered Traffic/Cell shown below is the traffic across all 7 cells expressed in Erlangs.

TABLE  11b

Offered_Traffic/Celles Group_Sizeesin Erlangs

| Environments “e” | High density-in building (CBD) call-seconds |  | Urban pedestrian call-seconds |  | Urban vehicular call-seconds |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 150.5 | 150.5 | 2123.88 | 2123.88 | 31.86 | 31.86 |
| Simple Message (SM) | 2.75 | 2.75 | 21.82 | 21.82 | 0.44 | 0.44 |
| Switched Data (SD) | 15.49 | 15.49 | 245.85 | 245.85 | 0.74 | 0.74 |
| Medium Multimedia (MMM) | 3.98 | 3.98 | 50.51 | 50.51 | 0.03 | 0.03 |
| High Multimedia (HMM) | 4.58 | 4.58 | 29.09 | 29.09 | 0.12 | 0.12 |
| High Interactive Multimedia (HIMM) | 13.74 | 13.74 | 109.1 | 109.1 | 0.52 | 0.52 |

## B8	Establish Quality_of_Service_Functiones (QOSes) parameters

The quality of service function for circuit switched is selected to be Erlang B with a blocking of 2%. For packet-switched services the Quality of service function is a rounding up to the next integer number.

## C1	Calculate number of Service_Channels/Celles required to carry Offered_Traffic/Celles

The number of traffic channels required in the group is presented below:

TABLE  12a

Service_Channels per Group

| Environments “e” | High density-in building (CBD) |  | Urban pedestrian |  | Urban vehicular |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 164 | 164 | 2 137 | 2 137 | 41 | 41 |
| Simple Message (SM) | 3 | 3 | 22 | 22 | 1 | 1 |
| Switched Data (SD) | 23 | 23 | 259 | 259 | 4 | 4 |
| Medium Multimedia (MMM) | 4 | 4 | 51 | 51 | 1 | 1 |
| High Multimedia (HMM) | 5 | 5 | 30 | 30 | 1 | 1 |
| High Interactive Multimedia (HIMM) | 21 | 21 | 122 | 122 | 3 | 3 |

The number of traffic channels required in the cell is service channels in the group divided by the group size of 7 and then rounded up. It is presented below:

TABLE  12b

Service_Channels/Celles

| Environments “e” | High density-in building (CBD) |  | Urban pedestrian |  | Urban vehicular |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 23.43 | 23.43 | 305.3 | 305.3 | 5.86 | 5.86 |
| Simple Message (SM) | 0.43 | 0.43 | 3.14 | 3.14 | 0.14 | 0.14 |
| Switched Data (SD) | 3.29 | 3.29 | 37.0 | 37.0 | 0.57 | 0.57 |
| Medium Multimedia (MMM) | 0.57 | 0.57 | 7.29 | 7.29 | 0.14 | 0.14 |
| High Multimedia (HMM) | 0.71 | 0.71 | 4.29 | 4.29 | 0.14 | 0.14 |
| High Interactive Multimedia (HIMM) | 3.0 | 3.0 | 17.43 | 17.43 | 0.43 | 0.43 |

## C2	Determine Service_Channel_Bit_Ratees needed to carry Net_User_Bit_Rates

In this example, it is assumed that the Service_Channel_Bit_Ratees is equal to the Net_User_Bit Rates.

TABLE  13

Service_Channel_Bit_Ratees

| Environments “e” | High density-in building (CBD) (kbit/s) |  | Urban pedestrian (kbit/s) |  |  | Urban vehicular (kbit/s) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink |  | Uplink | Downlink |  | Uplink | Downlink |
| Speech (S) | 16 | 16 |  | 16 | 16 |  | 16 | 16 |
| Simple Message (SM) | 14 | 14 |  | 14 | 14 |  | 14 | 14 |
| Switched Data (SD) | 64 | 64 |  | 64 | 64 |  | 64 | 64 |
| Medium Multimedia (MMM) | 64 | 384 |  | 64 | 384 |  | 64 | 384 |
| High Multimedia (HMM) | 128 | 2 000 |  | 128 | 2 000 |  | 128 | 2 000 |
| High Interactive Multimedia (HIMM) | 128 | 128 |  | 128 | 128 |  | 128 | 128 |

## C3	Calculate Traffices

Based on the number of channels needed and on the service channel bit rate, the traffic in each cell can be derived.

TABLE  14

Traffices

| Environments “e” | High density-in building (CBD) (Mbit/s/cell) |  | Urban pedestrian (Mbit/s/cell) |  | Urban vehicular (Mbit/s/cell) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 0.37 | 0.37 | 4.88 | 4.88 | 0.09 | 0.09 |
| Simple Message (SM) | 0.01 | 0.01 | 0.04 | 0.04 | 0.002 | 0.002 |
| Switched Data (SD) | 0.21 | 0.21 | 2.37 | 2.37 | 0.04 | 0.04 |
| Medium Multimedia (MMM) | 0.04 | 0.22 | 0.47 | 2.80 | 0.01 | 0.05 |
| High Multimedia (HMM) | 0.09 | 1.43 | 0.55 | 8.57 | 0.02 | 0.29 |
| High Interactive Multimedia (HIMM) | 0.38 | 0.38 | 2.23 | 2.23 | 0.05 | 0.05 |

## C4	Net_System_Capabiliy_ Parameterses (Ses)

A number of parameters must considered in determining key elements that influence the value of Net_System_Capabilityes.

## C5	Calculate Net_System_Capabilityes

For the purpose of this example, an improvement factor that is the expected improvement of system capabilities in the year 2010 relative to current systems is applied to current generation capabilities to obtain Net_System_Capabilityes.

In this example, the following net system capabilities have been used:

TABLE  15

Net_System_Capabilityes (Ses)

| Environments “e” | High density-in building (CBD) (kbit/s/MHz/cell) |  | Urban pedestrian (kbit/s/MHz/cell) |  | Urban vehicular (kbit/s/MHz/cell) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 67 | 67 | 67 | 67 | 67 | 67 |
| Simple Message (SM) | 73 | 73 | 73 | 73 | 73 | 73 |
| Switched Data (SD) | 73 | 73 | 73 | 73 | 73 | 73 |
| Medium Multimedia (MMM) | 73 | 73 | 73 | 73 | 73 | 73 |
| High Multimedia (HMM) | 73 | 73 | 73 | 73 | 73 | 73 |
| High Interactive Multimedia (HIMM) | 73 | 73 | 73 | 73 | 73 | 73 |

## D1	Calculate individual Fes component for a given direction (either uplink or downlink)

## D2	Repeat process for calculation of other direction (either downlink or uplink as appropriate)

Using the traffic calculated above and the example net system capability, the Fes components for each direction, each environment and each direction can be derived as follows:

TABLE 16

Individual spectrum requirements (Fes)

| Environments “e” | High density-in building (CBD) (MHz) |  | Urban pedestrian (MHz) |  | Urban vehicular (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Services “s” | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |
| Speech (S) | 5.6 | 5.6 | 72.9 | 72.9 | 1.4 | 1.4 |
| Simple Message (SM) | 0.08 | 0.08 | 0.6 | 0.6 | 0.03 | 0.03 |
| Switched Data (SD) | 2.9 | 2.9 | 32.4 | 32.4 | 0.5 | 0.5 |
| Medium Multimedia (MMM) | 0.5 | 3.0 | 6.4 | 38.3 | 0.1 | 0.8 |
| High Multimedia (HMM) | 1.3 | 19.6 | 7.5 | 117.4 | 0.3 | 3.9 |
| High Interactive Multimedia (HIMM) | 5.3 | 5.3 | 30.6 | 30.6 | 0.8 | 0.8 |

## D3	Calculate Fes for the service “s”, combining uplink and downlink components

The summation gives the following results:

TABLE  17

Fes combining the uplink and downlink components

| Environments “e” | High density-in building (CBD) (MHz) | Urban pedestrian (MHz) | Urban vehicular (MHz) |
| --- | --- | --- | --- |
| Services “s” |  |  |  |
| Speech (S) | 11.2 | 145.8 | 2.8 |
| Simple Message (SM) | 0.2 | 1.2 | 0.05 |
| Switched Data (SD) | 5.8 | 64.9 | 1.0 |
| Medium Multimedia (MMM) | 3.5 | 44.7 | 0.9 |
| High Multimedia (HMM) | 20.8 | 124.9 | 4.2 |
| High Interactive Multimedia (HIMM) | 10.5 | 61.1 | 1.5 |

## D4	Repeat process (steps A1 through D3) for all desired Fes

The results have been presented in tables that show the values for all services “s” and all environments “e”.

## D5	Determine weighting factor (aes) applicable to each individual Fes

If all services are assumed to have coincident busy hours, and the three environments are collocated in the same geographical area, the weighting factors are assumed to be one:

es    1 for all “e” and “s”

## D6	Determine adjustment factor ()

To take account of the trunking inefficiency and the guard bands, an adjustment factor of 5% can be taken.

  1.05

## D7	Calculate the final total FTerrestrial spectrum value

For this example, the summation of the components with the adjustment factor gives:

FTerrestrial  530.3 MHz

In this example, the result shown for FTerrestrial is the spectrum requirement estimate for public land mobile radio service, including IMT-2000, in the year 2010 because of assumptions made in the traffic and market forecasts, (namely, that the forecasts included first and second generation public land mobile radio services, and future expected IMT-2000 services).

In must be restated that, the results shown in this example should not be considered as providing an answer to the question of future spectrum requirements for public land mobile communications services, including IMT-2000, as all figures given in this example are still under study and all environments and services that must be considered for completeness may have not been included in the example.

1)	An example of the application of the methodology is provided in Appendix 1.

2	)	The mathematical convention of describing a complex function as a function name and a list of input parameters is used in several places in this document. It is demonstrated as follows:

Function {parameter1, parameter2, …, parameterN}.