RECOMMENDATION  ITU-R  M.1830

Technical characteristics and protection criteria of aeronautical
radionavigation service systems in the 645-862 MHz frequency band

(2007)

Scope

This Recommendation complements the technical characteristics of aeronautical radionavigation systems in the frequency band 645-862 MHz for relations not covered by RRC-06 and can be used by concerned administrations as technical guidelines for bilateral discussion and for estimations of compatibility with other radiocommunication services of administrations not party to the RRC-06 Agreement.

The ITU Radiocommunication Assembly,

considering

a)	that the aeronautical radionavigation service operates on a primary basis in the 645-862 MHz frequency band in a number of countries under No. 5.312 of the Radio Regulations (RR);

b)	that the 645-862 MHz frequency band is also used by other services on a primary basis,

recognizing

a)	that the radionavigation service is a safety service as specified by RR No. 1.59;

b)	that RR No. 4.10 states that the safety-of-life aspects of the radionavigation and other safety services require special measures to ensure their freedom from harmful interference (see also RR No. 1.169);

c)	that there is increasing need to ensure compatibility between the radionavigation services and other services which share their allocated bands;

d)	that the procedures and methodology for analysing compatibility between radars and systems in other services are contained in Recommendation ITU-R M.1461,

recommends

1	that the technical characteristics and protection criteria of aeronautical radionavigation systems in the band 645-862 MHz contained in Annexes 1 and 2 should be used in the assessment of compatibility with other services.

Annex 1

Technical characteristics of aeronautical radionavigation service systems 
in the 645-862 MHz frequency band

In accordance with RR No. 5.312, the 645-862 MHz band is allocated in a number of countries to the aeronautical radionavigation service (ARNS) on the primary basis. Within this service several types of radionavigation systems are used, including:

–	radio systems for short-range navigation (RSBN);

–	secondary radars for the air traffic control (ATC) which includes the ground radar and the onboard transponder;

–	primary airfield and route radars for the ATC.

All specified means are used to support navigation and air traffic control functions.

The basic characteristics for different types of aeronautical radionavigation stations operating in the band 645-862 MHz are presented in Table 1.

TABLE 1

Aeronautical radionavigation systems characteristics in the band 645-862 MHz

| Type of station Characteristics | RSBN | RLS 2 (Type 1) |  | RLS 2 (Type 2) |  | RLS 1 (Type 1) | RLS 1 (Type 2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Application | “Air-Ground” | Secondary radars – Type 1 (air traffic control) |  | Secondary radars – Type 2 |  | Primary radars – Type 1 | Primary radars – Type 2 |
| Transmitter characteristics |  |  |  |  |  |  |  |
| Station name | Aircraft transmitter | Ground radar transmitter | Aircraft transponder transmitter | Ground radar transmitter | Aircraft transponder transmitter | Ground radar transmitter | Ground radar transmitter |
| Place of station | Aircraft | Airfields | Aircraft | Airfields | Aircraft | Airfields | Airfields |
| Maximum effective radiated pulse power (e.r.p.) (dBW) | 30.5 | 48 | 35 | 69.5 | 34.5 | 82 | 82 |
| Pulse power (dBW) | 27 | 31 | 32 | 40 | 31 | 52.5 | 52.5 |
| Mean power (dBW) | 0.5 | 1 | 14 | 19.5 | 10.5 | 19.5 | 19.5 |
| Off-duty ratio | 447 | 1 000 | 63.1 | 112 | 112 | 1 995 | 1 995 |
| Pulse repetition cycle (ms) | 2.3 | 1.3 | 0.6 | 1.8 | 1.8 | 1.8 | 1.8 |
| Pulse length (μs) | 5.1 | 1.3 | 8.7 | 16 | 16 | 0.9-2 | 0.9-2 |
| Necessary emission bandwidth (MHz) | 3/0.7 | 4 | 4 | 3 | 8 | 6 | 3 |
| Class of emission | P0X/PXX | K0X | K0X | M1X | M1X | P0N | P0N |
| Operating frequencies (MHz) | 772, 776, 780, 784, 788, 792, 796, 800, 804, 808 | 668 | 668 | 835, 836, 837.5 | 740 | 833, 835, 836, 858 | 844, 847, 853, 859 |
| Antenna height (m) | 0 to 10 000 | 10 | 0 to 10 000 | 10 | 0 to 10 000 | 10 | 10 |
| Maximum antenna gain (dBi) | 3.5 | 17 | 3 | 29.5 | 3.5 | 29.5 | 29.5 |
| Antenna pattern | ND | 3 dB beamwidth: vert. pl. = 28° hor. pl. = 4° | ND | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° | ND | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 4° | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 4° |

TABLE 1 (end)

| Type of station Characteristics | RSBN | RLS 2 (Type 1) |  | RLS 2 (Type 2) |  | RLS 1 (Type 1) |  | RLS 1 (Type 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Direction of the antenna main beam | Lower hemisphere | Azimuth: 0-360° Revolution speed 6 rev/min. | Lower hemisphere | Azimuth: 0-360° Revolution speed 10 rev/min. | Lower hemisphere | Azimuth: 0-360° Revolution speed 6/10 rev/min. |  | Azimuth: 0-360° Revolution speed 6/10 rev/min. |
| Receiver characteristics |  |  |  |  |  |  |  |  |
| Station name | Ground radar receiver | Aircraft responder of ground radar | Ground radar receiver | Aircraft responder of ground radar | Ground radar receiver |  | Ground radar receiver | Ground radar receiver |
| Service type code | AA8 | BD | BA | BC | AA2 |  | AB | AB |
| Place of station | Airfields | Aircraft | Airfields | Aircraft | Airfields |  | Airfields | Airfields |
| Antenna height (m) | 10 | 0-10 000 | 10 | 0-10 000 | 10 |  | 10 | 10 |
| Polarization(1) | Linear, horizontal | Linear, vertical | Linear, vertical | Linear, horizontal | Linear, horizontal |  | Linear, horizontal | Linear, horizontal |
| Maximum antenna gain (dBi) | 22 | 3 | 17 | 3 | 28.4 |  | 29.5 | 29.5 |
| Antenna pattern | 3 dB beamwidth: vert. pl. = 50° hor. pl. = 4-5° | ND | 3 dB beamwidth: vert. pl. = 28° hor. pl. = 4° | ND | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° |  | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° |
| Direction of antenna main beam | Azimuth: 0-360° Revolution speed 100 rev/min. | Lower hemisphere | Azimuth: 0-360° Revolution speed 6 rev/min. | Lower hemisphere | Azimuth: 0-360° Revolution speed 10 rev/min. |  | Azimuth: 0-360° Revolution speed 6/10 rev/min. | Azimuth: 0-360° Revolution speed 6/10 rev/min. |
| (1) In the case when the interferer has orthogonal polarization in relation to the wanted signal, a value of 16 dB should be added to protected field-strength values in Tables 2 to 8 due to polarization discrimination. |  |  |  |  |  |  |  |  |

Annex 2

Protection criteria for aeronautical radionavigation service systems
in the 645-862 MHz band from DVB-T interference

The protection criteria for different types of aeronautical radionavigation stations operating in the band 645-862 MHz are presented in Tables 2 to 8.

For the sharing analyses with terrestrial broadcasting services it is important to provide protection criteria in a form indicating minimum field strength to be protected and protection ratios for different values of frequency difference between centre frequencies of DVB-T and aeronautical radionavigation station signals which takes into account impact of interference in adjacent channels.

The values for minimum field-strength values are based on experimental studies.

NOTE 1 – Further information may be required on the detailed environment and conditions of the measurements.

Protection ratios for the case of interference from DVB-T into ARNS receivers which are presented in Table 2 are based on experimental measurements, whereas protection ratios in Tables 3 to 8 are theoretical ones. Protection ratios in Tables 3 to 8 are based on perspective masks for ARNS filters in the band 645-862 MHz that significantly reduce interference susceptibility of ARNS receivers with different required emission band. The masks are presented in Fig. 1.

FIGURE 1

Perspective ARNS receiver filter mask in the band 645-862 MHz band

The masks are based on theoretically obtainable capabilities in modernization of ARNS receiving filters and actually they refer to practically ideal characteristics of filtration (rectangularity factor of at least 1.3 at –40 dB).

The masks were employed for estimation of protection ratios for ARNS receivers using the DVB-T masks for a non-critical case and sensitive cases.

TABLE 2

RSBN protection criteria – Ground reception

| Interference source | DVB-T (8 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Service type code | AA8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Protected field strength, (dB(μV/m)) | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Percentage of time (%)(1) | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Protection ratios PR in ∆f |  | ∆f(2) (MHz) | –12.0 | –10.0 | –8.0 | –6.0 | –4.0 | –2.0 | 0.0 | +2.0 | +4.0 | +6.0 | +8.0 | +10.0 | +12.0 |
|  |  | PR(3) (dB) | –65.0 | –50.0 | –27.0 | –16.0 | –5.0 | 0.0 | 0.0 | 0.0 | –5.0 | –16.0 | –40.0 | –52.0 | –65.0 |
| (1) Percentage of time for which interfering signal field strength is exceeded due to variation in propagation conditions (see Recommendation ITU-R P.1546). (2) ∆f: Frequency difference between centre frequencies of DVB-T and aeronautical radionavigation station signals. (3) PR: Protection ratio (in terms of the RRC-04 Report), provided by the ARNS receiver frequency selectivity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 3

RLS 2 (Type 1) protection criteria – Aircraft reception

| Interference source | DVB-T (8 MHz) |
| --- | --- |
| Service type code | BD |
| Protected field strength, (dB(μV/m)) | 52 – single interferer(1)/59 – aggregate interference(2) |
| Percentage of time (%) | Free space (0%) |
| Protection ratios | See Tables 3-1 and 3-2 |
| Necessary bandwidth (MHz) | 4 |
| (1) The protected field-strength values for single interferer are to be used in the planning exercise when assessing interference from digital broadcasting assignment/allotment in a one-to-one interference situation. The values for aggregate interference are provided to indicate protection required from all sources of interference and not applicable to a single DVB-T transmitter. (2) Given values are derived from worst-case analysis. |  |

TABLE 3-1

RLS 2 (Type 1) protection ratios – Aircraft reception (non-critical DVB-T mask)*

| f | –16 | –15 | –6.5 | –6 | –5.5 | –5 | –4 | –2.5 | 0 | 2.5 | 4 | 5 | 5.5 | 6 | 6.5 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –81.3 | –66.4 | –44.1 | –34 | –12 | –9 | –5.9 | –3.5 | –2.8 | –3.5 | –5.9 | –9 | –12 | –34 | –44.1 | –66.4 | –81.3 |
| * For details please refer to the RRC-06 Agreement (Annex 3). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 3-2

RLS 2 (Type 1) protection ratios – Aircraft reception (sensitive case DVB-T mask)*

| f | –16 | –15 | –6.5 | –6 | –5.5 | –5 | –4 | –2.5 | 0 | 2.5 | 4 | 5 | 5.5 | 6 | 6.5 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –90.9 | –66.5 | –44.9 | –39 | –12 | –9 | –6 | –3.5 | –2.8 | –3.5 | –6 | –9 | –12 | –39 | –44.9 | –66.5 | –90.9 |
| * For details please refer to the RRC-06 Agreement (Annex 3). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 4

RLS 2 (Type 1) protection criteria – Ground reception

| Interference source | DVB-T (8 MHz) |
| --- | --- |
| Service type code | BA |
| Protected field strength (dB(μV/m)) | 29 – single interferer(1)/33 – aggregate interference |
| Percentage of time (%) | 10 |
| Protection ratios | See Tables 4-1 and 4-2 |
| Necessary bandwidth (MHz) | 4 |
| (1) The protected field-strength values for single interferer are to be used in the planning exercise when assessing interference from digital broadcasting assignment/allotment in a one-to-one interference situation. The values for aggregate interference are provided to indicate protection required from all sources of interference and not applicable to a single DVB-T transmitter. |  |

TABLE 4-1

RLS 2 (Type 1) protection ratios – Ground reception (non-critical case DVB-T mask)

| f | –16 | –15 | –6.5 | –6 | –5.5 | –5 | –4 | –2.5 | 0 | 2.5 | 4 | 5 | 5.5 | 6 | 6.5 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –81.3 | –66.4 | –44.1 | –34 | –12 | –9 | –5.9 | –3.5 | –2.8 | –3.5 | –5.9 | –9 | –12 | –34 | –44.1 | –66.4 | –81.3 |

TABLE 4-2

RLS 2 (Type 1) protection ratios – Ground reception (sensitive case DVB-T mask)

| f | –16 | –15 | –6.5 | –6 | –5.5 | –5 | –4 | –2.5 | 0 | 2.5 | 4 | 5 | 55 | 6 | 6.5 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –90.9 | –66.5 | –44.9 | –39 | –12 | –9 | –6 | –3.5 | –2.8 | –3.5 | –6 | –9 | –12 | –39 | –44.9 | –66.5 | –90.9 |

TABLE 5

RLS 2 (Type 2) protection criteria – Aircraft reception

| Interference source | DVB-T (8 MHz) |
| --- | --- |
| Service type code | BC |
| Protected field strength (dB(μV/m)) | 73 – single interferer |
| Percentage of time (%) | Free space (0 %) |
| Protection ratios | See Tables 5-1 and 5-2 |
| Necessary bandwidth (MHz) | 3 |

TABLE 5-1

RLS 2 (Type 2) protection criteria – Aircraft reception (non-critical case DVB-T mask)

| f | –16 | –14 | –8 | –6.5 | –6 | –5 | –4 | –2 | 0 | 2 | 4 | 5 | 6 | 6.5 | 8 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –82.8 | –64 | –49.2 | –45.8 | –45.39 | –12.1 | –7.25 | –4 | –4 | –4 | –7.25 | –12.1 | –45.39 | –45.8 | –49.2 | –64 | –82.8 |

TABLE 5-2

RLS 2 (Type 2) protection ratios – Aircraft reception (sensitive case DVB-T mask)

| f | –16 | –14 | –8 | –6.5 | –6 | –5 | –4 | –2 | 0 | 2 | 4 | 5 | 6 | 6.5 | 8 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio in dB | –92.4 | –64.3 | –49.4 | –46.28 | –46.26 | –12.2 | –7.27 | –4 | –4 | –4 | –7.27 | –12.2 | –46.26 | –46.28 | –49.4 | –64.3 | –92.4 |

TABLE 6

RLS 2 (Type 2) protection criteria – Ground reception

| Interference source | DVB-T (8 MHz) |
| --- | --- |
| Service type code | AA2 |
| Protected field strength (dB(μV/m)) | 24 – single interferer(1)/28 – aggregate interference |
| Percentage of time (%) | 10 |
| Protection ratios | See Tables 6-1 and 6-2 |
| Necessary bandwidth (MHz) | 8 |
| (1) The protected field-strength values for single interferer are to be used in the planning exercise when assessing interference from digital broadcasting assignment/allotment in a one-to-one interference situation. The values for aggregate interference are provided to indicate protection required from all sources of interference and not applicable to a single DVB-T transmitter. |  |

TABLE 6-1

RLS 2 (Type 2) protection ratios – Ground reception (non-critical case DVB-T mask)

| f | –17 | –15 | –10 | –9 | –8.5 | –8 | –7 | –4 | 0 | 4 | 7 | 8 | 8.5 | 9 | 10 | 15 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –79.4 | –61.2 | –46.3 | –43.2 | –43 | –19.9 | –8.7 | –2.9 | 0 | –2.9 | –8.7 | –19.9 | –43 | –43.2 | –46.3 | –61.2 | –79.4 |

TABLE 6-2

RLS 2 (Type 2) protection ratios – Ground reception (sensitive case DVB-T mask)

| f | –17 | –15 | –10 | –9 | –8.5 | –8 | –7 | –4 | 0 | 4 | 7 | 8 | 8.5 | 9 | 10 | 15 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –89.4 | –61.3 | –46.5 | –43.4 | –43. | –20.2 | –8.7 | –2.9 | 0 | –2.9 | –8.7 | –20.2 | –43.0 | –43.4 | –46.5 | –61.3 | –89.4 |

TABLE 7

RLS 1 (Type 1) protection criteria – Ground reception

| Interference source | DVB-T (8 MHz) |
| --- | --- |
| Service type code | AB |
| Protected field strength (dB(μV/m)) | 13 – single interferer |
| Percentage of time (%) | 10 |
| Protection ratios | See Tables 7-1 and 7-2 |
| Necessary bandwidth (MHz) | 6 |

TABLE 7-1

RLS 1 (Type 1) protection ratios – Ground reception (non-critical case DVB-T mask)

| f | –17 | –15 | –9 | –7.5 | –6.5 | –6 | –4 | –1 | 0 | 1 | 4 | 6 | 6.5 | 7.5 | 9 | 15 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –80.6 | –63.79 | –47.1 | –44.4 | –11.7 | –8.8 | –4.1 | –1.1 | –1 | –1.1 | –4.1 | –8.8 | –11.7 | –44.4 | –47.1 | –63.79 | –80.6 |

TABLE 7-2

RLS 1 (Type 1) protection ratios – Ground reception (sensitive case DVB-T mask)

| f | –17 | –15 | –9 | –7.5 | –6.5 | –6 | –4 | –1 | 0 | 1 | 4 | 6 | 6.5 | 7.5 | 9 | 15 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –90.66 | –63.9 | –47.3 | –45.4 | –11.8 | –8.8 | –4.1 | –1.1 | –1 | –1.1 | –4.1 | –8.8 | –11.8 | –45.4 | –47.3 | –63.9 | –90.66 |

TABLE 8

RLS 1 (Type 2) protection criteria – Ground reception

| Interference source | DVB-T (8 MHz) |
| --- | --- |
| Service type code | AB |
| Protected field strength (dB(μV/m)) | 13 – single interferer |
| Percentage of time (%) | 10 |
| Protection ratios | See Tables 8-1 and 8-2 |
| Necessary bandwidth (MHz) | 3 |

TABLE 8-1

RLS 1 (Type 2) protection ratios – Ground reception (non-critical case DVB-T mask)

| f | –16 | –14 | –8 | –6.5 | –6 | –5 | –4 | –2 | 0 | 2 | 4 | 5 | 6 | 6.5 | 8 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –82.8 | –64 | –49.2 | –45.8 | –45.39 | –12.1 | –7.25 | –4 | –4 | –4 | –7.25 | –12.1 | –45.39 | –45.8 | –49.2 | –64 | –82.8 |

TABLE 8-2

RLS 1 (Type 2) protection ratios – Ground reception (sensitive case DVB-T mask)

| f | –16 | –14 | –8 | –6.5 | –6 | –5 | –4 | –2 | 0 | 2 | 4 | 5 | 6 | 6.5 | 8 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protection ratio (dB) | –92.4 | –64.3 | –49.4 | –46.28 | –46.26 | –12.2 | –7.27 | –4 | –4 | –4 | –7.27 | –12.2 | –46.26 | –46.28 | –49.4 | –64.3 | –92.4 |