Recommendation ITU-R M.2058-2

(02/2026)

M Series: Mobile, radiodetermination, amateur
and related satellite services

Characteristics of HF digital system, referred to as navigational data for broadcasting maritime safety and security related information from shore-to-ship in the maritime mobile service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the appropriate ITU-R patent information available at https://www.itu.int/en/ITU-R/study-groups/Pages/itu-r-patent-information.aspx.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radio-wave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2026

ITU 2026

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  M.2058-2

Characteristics of HF digital system, referred to as navigational data for broadcasting maritime safety and security related information from shore-to-ship in the maritime mobile service

(2014-2023-2026)

Scope

This Recommendation describes an HF radio system, referred to as Navigational Data HF (NAVDAT HF), for use in the maritime mobile service, operating in the frequency bands of Appendix 17 of the Radio Regulations (RR) for digital broadcasting of maritime safety and security related information from shore-to-ship.

The operational characteristics and system architecture of this radio system are included in Annexes 1 and 2. Technical characteristics and transmission structure are detailed in Annexes 3 and 4. Message file structure and a broadcast mode are introduced in Annexes 5 and 6. The frequencies listed in Annex 7, which belong to RR Appendix 17, should be utilized for operating the NAVDAT HF system. The list of subject messages is on Annex 8.

The NAVDAT HF is complementary to the NAVDAT 500 kHz, described in Recommendation ITU-R M.2010 in terms of radio coverage.

Keywords

HF, Maritime, NAVDAT, broadcast, digital

Abbreviations/Acronyms

BER	Bit error rate

BPSK	Binary phase shift keying

BW	Bandwidth

CDU	Control and display unit

CRC	Cyclic redundancy check

DRM	Digital radio mondiale

DS	Data stream

GF	Galois field or finite field

GMDSS	Global maritime distress and safety system

GNSS	Global navigation satellite system

HF	High frequency

IMO	International Maritime Organization

ITU	International Telecommunication Union

LDPC	Low-density parity-check

LF	Low frequency

MER	Modulation error ratio

MF	Medium frequency

MIS	Modulation information stream

MMSI	Maritime mobile service identity

NAVDAT	Navigational Data (the system name)

NAVTEX	Navigational Telex (the system name)

NBDP	Narrow band direct printing

NM	Nautical mile (1 852 metres)

OFDM	Orthogonal frequency division multiplexing

PEP	Peak envelope power

PRBS	Pseudo-random binary sequence

rm:	Root mean square

RS	Reed-Solomon codes

SDR	Software defined radio

SFN	Single frequency network

SIM	System of information and management

S/N or SNR	Signal-to-noise ratio

TIS	Transmitter information stream

WRC	World Radiocommunication Conference

Related ITU Recommendations, Reports

Recommendation ITU-R P.368 – Ground-wave propagation curves for frequencies between 10 kHz and 30 MHz

Recommendation ITU-R P.372 – Radio noise

Recommendation ITU-R M.493 – Digital selective-calling system for use in the maritime mobile service

Recommendation ITU-R M.585 – Assignment and use of identities in the maritime mobile service; or the revised version

Recommendation ITU-R·M.1080 – Digital selective calling system enhancement for multiple equipment installations

Recommendation ITU-R BS.1514 – System for digital sound broadcasting in the broadcasting bands below 30 MHz

Recommendation ITU-R M.2010 – Characteristics of a digital system, referred to as navigational data for broadcasting maritime safety and security related information from shore-to-ship in the 500 kHz band

Report ITU-R M.2443 – NAVDAT Guidelines

The ITU Radiocommunication Assembly,

considering

a)	that high speed data broadcast from shore-to-ships enhances operational efficiency and maritime safety;

b)	that the existing Maritime Safety Information (MSI) system operating in narrow band direct printing (NBDP) HF has limited capacity;

c)	that emerging maritime navigation systems increase the demand for data transmission from shore-to-ship;

d)	that the MF band provides a limited geographical coverage with high radio noise in some areas;

e)	that it is not always easy to install efficient MF antennas with wide bandwidth,

recognizing

a)	that Recommendation ITU-R M.2010 describes the NAVDAT system operating at 500 kHz;

b)	that Recommendation ITU-R BS.1514 contains the information about the digital radio mondiale (DRM) system referenced in Annexes 4 to 6,

noting

a)	that the NAVDAT system uses two international frequencies: 500 kHz in MF band and 4 226 kHz in HF band;

b)	that the NAVDAT system may use other allocated frequencies in the maritime MF and HF bands for national or regional broadcasts,

recommends

1	that the operational characteristics for the broadcasting of maritime safety and security related information in the HF frequency bands should be in accordance with Annex 1;

2	that the system architecture of the broadcasting system for maritime safety and security related information in the HF frequency bands should be in accordance with Annex 2;

3	that the technical characteristics and modem protocols for digital data transmission of maritime safety and security related information from shore-to-ships in the HF frequency band should be in accordance with Annexes 3 and 4;

4	that the data stream of the system and the message structure should be in accordance with Annex 5;

5	that the single frequency network (SFN) mode described in Annex 6 should be used;

6	that the frequencies listed in Annex 7, which belongs to Appendix 17 of the Radio Regulations (RR), should be utilized for operating the NAVDAT HF system;

7	that information on subject message described in Annex 8 should be considered for use.

TABLE OF CONTENTS

Page

Annex 1  Operational characteristics		5

A1-1	Type of messages and files		5

A1-2	Broadcast modes		5

A1-3	Broadcast priority		5

Annex 2  System architecture		6

A2-1	The broadcast transmission chain		6

Annex 3  NAVDAT HF technical characteristics		13

A3-1	Modulation principle		13

A3-2	Estimated usable data rate		24

A3-3	NAVDAT HF transmitter performance specification		27

A3-4	NAVDAT ship receiver		28

A3-5	Minimum NAVDAT ship receiver performance specifications		34

Annex 4  Transmission structure		35

A4-1	Frame structure		35

A4-2	Synchronization head		35

A4-3	Modulation information stream		37

A4-4	Transmitter information stream		39

A4-5	Data stream		42

A4-6	Low-density parity-check codes		47

A4-7	Cyclic redundancy check		55

Annex 5  Message file structure		55

Annex 6  Single frequency network mode of Digital Radio Mondiale		58

A6-1	Explanation of Digital Radio Mondiale		58

Annex 7  Frequencies for NAVDAT HF system		59

Annex 8  NAVDAT subject message codes		59

Annex 1

Operational characteristics

The NAVDAT HF system can use a simple time-slot allocation similar to the NAVTEX system which could be coordinated by the International Maritime Organization (IMO).

That NAVDAT HF system can also work on SFN as described in Annex 6. In this case, the transmitters are frequency synchronized and the transmit data must be the same for all transmitters.

The NAVDAT HF digital system offers a free broadcast transmission of any kind of message from shore-to-ships with possibility of encryption.

## A1-1	Type of messages and files

Any broadcasting message should be provided by a secure and controlled source.

Message types broadcast can include, but are not limited to, the following:

–	safety of navigation;

–	security;

–	piracy;

–	search and rescue;

–	meteorological messages;

–	piloting or harbour messages;

–	vessel traffic service files transfer;

–	electronic chart update packages.

NOTE – See Annex 8, which gives the message subjects and their encoding.

## A1-2	Broadcast modes

### A1-2.1	General broadcast

These messages are broadcasted for the attention of all ships.

### A1-2.2	Selective broadcast

These messages are broadcasted for the attention of a group of ships or in a specific navigation area. (See also § A3-4.1.9.)

### A1-2.3	Dedicated message

These messages are addressed to one ship, using the maritime mobile service identity.

## A1-3	Broadcast priority

NAVDAT is capable to broadcast messages with distress, urgency and safety in the order of priority of communications (refer to the documents published by IMO).

Annex 2

System architecture

## A2-1	The broadcast transmission chain

The NAVDAT system is organized by performing the following functions:

–	System of information and management (SIM):

–	collects and controls all kinds of information;

–	creates message files to be transmitted;

–	creates transmitting program according to message files priority and need of repetition;

–	ensures the operating status and broadcast quality of the shore transmitter;

–	controls the operating parameters of the shore transmitter.

–	Shore network:

–	assures the transportation of the message files from sources to the transmitters.

–	Shore transmitter:

–	receives the message files from SIM;

–	translates message files to orthogonal frequency division multiplexing (OFDM) signal;

–	transmits RF signal to the antenna for broadcast to ships;

–	ensures operating status and report to the SIM.

–	Transmission channel:

–	Transports the HF RF signal.

–	Ship receiver:

–	demodulates the RF OFDM signal;

–	reconstructs the message files;

–	sorts and makes the message files available for the dedicated equipment according to the message files applications or displays the contents of the message files.

Figures 1 and 2 show the diagram of the broadcast transmission chain.

Figure 1

NAVDAT HF broadcast transmission chain block diagram

Figure 2

Global NAVDAT broadcast chain

### A2-1.1	System of information and management

The SIM term includes:

–	all the sources that deliver file messages (e.g. meteorological office, safety and security organizations);

–	the file multiplexer which is an application running on a server;

–	the file multiplexer manager;

–	the shore transmitter manager.

All the sources are connected to the file multiplexer through a network.

Figure 3 shows the general diagram of the SIM.

Figure 3

NAVDAT system of information and management block diagram

#### A2-1.1.1	File multiplexer

The file multiplexer:

–	takes delivery of the message files from the data sources;

–	encrypts the message files when required;

–	formats the file messages with recipient information, priority status and time stamp;

–	sends the message files to the transmitter.

#### A2-1.1.2	File multiplexer manager

The file multiplexer manager is a man-machine interface that enables the user to, among other tasks:

–	have a look at the message files coming from any source;

–	specify the priority and periodicity of any message file;

–	specify the recipient of any message file;

–	manage the file message encryption.

Some of these functionalities may be automated. As an example, the priority and the periodicity of a message may be selected according to the source it comes from, or the source may specify the priority in the message.

#### A2-1.1.3	Shore transmitter manager

The shore station manager is a man-machine interface connected to the transmitter through the network; it makes it possible to supervise the transmitter status by indications such as:

–	transmit acknowledgment;

–	alarms;

–	effective RF transmit power;

–	synchronization report;

–	quality of transmission;

and to change the transmitter parameters, such as:

–	RF transmit power;

–	OFDM parameters (e.g. pilot subcarriers, modulations error coding);

–	transmission schedule.

### A2-1.2	Shore network

The shore network can use a broadband link, a low data rate link or a local file sharing.

### A2-1.3	Shore transmitter description

A coastal transmitting station consists of this minimum configuration:

–	one controller, which is a local server with access protection;

–	one OFDM modulator;

–	one RF signal generator;

–	one HF RF power amplifier;

–	one or more transmit antenna with matching unit;

–	one global navigation satellite system (GNSS) receiver or atomic clock for synchronization;

–	one monitoring receiver with its antenna.

#### A2-1.3.1	Shore system architecture

Figure 4 shows the block diagram of an HF digital transmitter.

Figure 4

NAVDAT HF digital transmitter functional block diagram

#### A2-1.3.2	Controller

This unit receives and transmits some pieces of information:

–	message files from SIM;

–	GNSS or atomic clock for synchronization;

–	HF signal from monitoring receiver;

–	HF signal generator, modulator and transmitter RF power amplifier control signals and monitoring;

–	ensure signal from the RF signal generator and RF amplifier.

The function of the controller is:

–	to check if the frequency band in use is free before transmission;

–	to synchronize all signals on the coast station from synchronization clock;

–	to control the transmission parameters, time and schedule;

–	to format the message files to be transmitted (split files into packets).

#### A2-1.3.3	Modulator

Figure 5 shows the diagram of the modulator.

Figure 5

NAVDAT HF modulator functional block diagram

##### A2-1.3.3.1	Input streams

In order to operate, the modulator needs three input streams:

–	modulation information stream (MIS);

–	transmitter information stream (TIS);

–	data stream (DS).

These streams are transcoded and then placed on the OFDM signal by the cell mapper (§ A2-1.3.3.3).

##### A2-1.3.3.1.1	Modulation information stream

This stream is used to provide information about:

–	the channel bandwidth (1, 3, 5 or 10 kHz);

–	the modulation for transmission information stream and data stream (4-, 16- or 64-QAM).

The MIS is always coded on 4-QAM subcarriers for good demodulation into the receiver.

##### A2-1.3.3.1.2	Transmitter information stream

This stream is used to provide information to the receiver about:

–	error coding for data stream according radio propagation (should be different for surface wave propagation at daytime and for surface + sky wave propagation at night-time),

–	identifier of the transmitter,

–	time.

The TIS can be coded on 4- or 16-QAM.

##### A2-1.3.3.1.3	Data stream

It contains the message files to transmit (these message files were previously formatted by the file multiplexer).

##### A2-1.3.3.2	Error encoding

The error correction scheme determines the robustness of the coding. The code rate is the ratio between useful and raw data rate. It illustrates the transmission efficiency and can vary from 0.5 to 0.75 depending on the error correction schemes and modulation patterns.

##### A2-1.3.3.3	Orthogonal frequency division multiplexing generation

The three streams (MIS, TIS and DS) are formatted:

–	encoding;

–	energy dispersal.

A cell mapper organizes the OFDM cells with the formatted streams and the pilot cells. The pilot cells are transmitted for the receiver to estimate the radio channel and synchronize on the RF signal.

An OFDM signal generator creates the OFDM baseband according to the output of the cell mapper.

#### A2-1.3.4	HF RF generator

A HF RF generator transposes the baseband signal to the final frequency RF output carrier.

An amplifier brings the RF signal to the desired power.

#### A2-1.3.5	RF power amplifier

The function of this stage is to amplify the HF signal from the generator output to the necessary level to obtain the desired radio coverage.

The OFDM transmission introduces a crest factor on the RF signal. This crest factor must be less than 10 dB at the RF amplifier output for a correct modulation error ratio (MER).

The rms RF power of the transmitter must be adapted to the overall efficiency of the antenna and the desired radio coverage.

The output rms RF power of shore transmitter can be adjusted according to the frequency band.

Maximum rms output RF power: for 4 and 6 MHz: 5 kW; for 8, 12, 16, 18/19 and 22 MHz: 10 kW.

#### A2-1.3.6	Transmit antenna with matching unit

The RF amplifier is connected to the transmit antenna through the impedance matching unit.

#### A2-1.3.7	Global navigation satellite system receiver and a backup atomic reference clock

The clock is used to synchronize the local controller and configure a high-precision reference clock when working in SFN mode.

#### A2-1.3.8	Monitoring receiver

The monitoring receiver checks that the programmed frequency is free before transmission and offers possibility to check the transmission. A remote receiver is recommended for ensuring the local signal reception quality.

### A2-1.4	Transmission channel: radio coverage estimation

The NAVDAT HF system uses surface wave propagation with vertically polarized antennas.

The ground plane of these vertical antennas should be designed to minimize the sky wave (by obtaining the lowest possible angle of radiation).

The radio coverage, in surface wave (also known as ground wave) could be calculated based on the most recent version of Recommendations ITU-R P.368 and ITU-R P.372 or appropriate simulation software (“GRWAVE”, “NOISEDAT”, “LFMF-SmoothEarth”).

The NAVDAT HF system can also use mixed propagation, surface wave and sky wave, or sky wave only.

### A2-1.5	Propagation channel

ITU has defined several criteria concerning the propagation channel from which 4 modes can be defined;

Mode A: Gaussian channels with minor fading; use with surface wave propagation;

Mode B: Time and frequency selective channels, with longer delay spread. Use with mixed surface wave and sky wave propagation;

Mode C: As mode B, but with higher Doppler spread: sky wave propagation with multi-hops;

Mode D: As mode B, but with severe delay and Doppler spread. Use with sky wave with multi-hops on several ionospheric layers,

Modes A and B are recommended for the main HF channel on 4 226 kHz.

In daytime, the ionospheric layer D is absorbent. During this period, the mode A should therefore be used.

At sunset the layer D disappears, and it is better to use the mode B during the night period.

The NAVDAT in HF frequency band can use all these modes of propagation according to the radio coverage sought.

Annex 3

NAVDAT HF technical characteristics

## A3-1	Modulation principle

The system uses OFDM which is a modulation technology for digital transmissions.

### A3-1.1	Introduction

The bandwidth of the radio transmission channel is divided in the frequency domain to form subcarriers.

The radio transmission channel occupancy is organized in the time to form OFDM symbols.

An OFDM cell is equivalent to one subcarrier in one OFDM symbol.

FIGURE 6

OFDM introduction

### A3-1.2	Principle

The OFDM uses a large number of closely-spaced (41.666 Hz (mode A), 46.875 Hz (mode B) to 68.182 Hz in mode C) orthogonal subcarriers to obtain high spectral efficiency to transmit data. These subcarriers are frequency-spaced (Fu = 1/Tu), where Tu is the duration of the useful part of an OFDM symbol.

The phases of subcarriers are orthogonal one to each other to enhance signal diversity caused by the multipath, especially in long distance.

A guard interval (Td) is inserted in the OFDM symbol to reduce multipath effect, thus reducing the inter-symbol interference.

The OFDM symbol duration is Ts = Tu + Td.

The OFDM symbols are then concatenated to make an OFDM frame.

The OFDM frame duration is Tf.

Figure 7

Spectral representation of an OFDM frame

Figure 8

Temporal representation of an OFDM frame

### A3-1.3	OFDM parameters

OFDM parameters values are listed in Table 1.

TABLE 1

OFDM parameters for all bandwidth

| Mode propagation mode | Tu (ms) | 1 / Tu (Hz) | Td (ms) | Ts = Tu + Td (ms) | Ns | Tf (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| A: surface wave | 24 | 41.666 | 2.66 | 26.66 | 15 | 400 |
| B: surface wave + sky wave | 21.33 | 46.875 | 5.33 | 26.66 | 15 | 400 |
| C: skywave | 14.66 | 68.182 | 5.33 | 20 | 20 | 400 |

Tu:	duration of the useful part of an OFDM symbol

1/Tu:	carrier spacing

Td:	duration of the guard interval

Ts:	duration of an OFDM symbol

Ns:	the number of symbols per frame

Tf:	duration of the transmission frame.

### A3-1.4	Channel bandwidth

NAVDAT digital broadcast defines different channel bandwidths and determines subcarrier numbers corresponding to the channel bandwidth. Table 2 present the relationship between channel bandwidths and OFDM sub-carrier numbers.

TABLE 2

Relationship between channel bandwidth and OFDM subcarrier numbers

| Propagation mode | Case | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
|  | Channel bandwidth | 1 kHz | 3 kHz | 5 kHz | 10 kHz |
| A: surface wave | Numbers of subcarriers | 23 | 69 | 115 | 229 |
|  | No. of subcarriers | K −11 to 11 | K −34 to 34 | K −57 to 57 | K −114 to 114 |
| B: surface wave + sky wave | Numbers of subcarriers | 19 | 61 | 103 | 207 |
|  | No. of subcarriers | K −9 to 9 | K −30 to 30 | K −51 to 51 | K −103 to 103 |
| C: sky wave | Numbers of subcarriers | 13 | 41 | 69 | 139 |
|  | No. of sub-carriers | K −6 to 6 | K −20 to 20 | K −34 to 34 | K −69 to 69 |

### A3-1.5	Modulation

Every subcarrier is modulated in amplitude and phase (quadrature amplitude modulation (QAM)).

Modulation patterns can be either 64 states (6 bits, 64-QAM), 16 states (4 bits, 16-QAM), or 4 states (2 bits, 4-QAM).

The modulation pattern depends on the desired robustness of the signal.

FIGURE 9

BPSK modulation constellation where: a = 1

Figure 10

4-QAM constellation where: a = 1  2 ; bit ordering: {i0 q0} = { y’0 y’1 }

Figure 11

16-QAM constellation where: a = 1  10 ; Bit ordering: {i0 i1 q0 q1} = { y’0 y’1 y’2 y’3 }

Figure 12

64-QAM constellation where: a = 1  42 ; Bit ordering: {i0 i1 i2 q0 q1 q2} = { y’0 y’1 y’2 y’3 y’4 y’5}; y’0 precedes y’1, y’1 precedes y’2, etc.

### A3-1.6	Synchronization

In order to allow a good demodulation of each subcarrier, the radio transmission channel response must be determined for each subcarrier and equalization is applied. Some of the subcarriers of the OFDM symbols carry pilot signals.

The pilot signals allow the receiver to:

–	detect if a signal is received;

–	estimate the frequency offset;

–	estimate the radio transmission channel.

The number of pilot signals depends on the desired robustness of the signal.

This pilot carriers are transmitted with a power gain factor of 2 in BPSK modulation.

Figure 13

Pilot OFDM signal

The pilot signal location in each OFDM symbol in a frame can be shown as follows:

Figure 14

The pilot signal location (in mode A)

Figure 15

The pilot signal location (in mode B)

Figure 16

The pilot signal location (in mode C)

where t is the direction of time domain, f is the direction of frequency domain. The first symbol of each OFDM head frame should be filled by a sequence of synchronization signal that make up the synchronization head (refer to Table 11 in Annex 4), all of which are used as a time reference to provide synchronization for the receiver. The black cell and the white cell represent the pilot signal and the data signal respectively. The pilot signal value, which is modulated in 2-QAM (BPSK) in an OFDM, is show in Table 3.

TABLE 3

Pilot sequence (in mode A)

| Number of subcarriers | Pilot sequence |
| --- | --- |
| 229 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 1 −1 −1 −1 1 1 −1 −1 −1 1 1 1 −1 −1 −1 −1 −1 −1 1 −1 1 −1 −1 1 1 1 |
| 115 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 1 −1 −1 −1 1 1 −1 −1 |
| 69 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 |
| 23 | −1 1 −1 1 |

Pilot sequence (in mode B)

| Number of subcarriers | Pilot sequence |
| --- | --- |
| 207 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 1 −1 −1 −1 1 1 −1 −1 −1 1 1 1 −1 −1 −1 −1 −1 −1 1 −1 1 −1 −1 |
| 103 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 1 −1 −1 −1 1 |
| 61 | −1 1 −1 1 −1 1 1 1 −1 1 |
| 19 | −1 1 −1 1 |

Pilot sequence (in mode C)

| Number of subcarriers | Pilot sequence |
| --- | --- |
| 139 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 1 −1 −1 −1 1 1 −1 −1 −1 1 1 1 −1 −1 −1 −1 −1 −1 1 −1 1 −1 −1 |
| 69 | −1 1 −1 1 −1 1 1 1 −1 1 1 1 1 −1 −1 −1 1 |
| 41 | −1 1 −1 1 −1 1 1 1 −1 1 |
| 13 | −1 1 −1 |

In the first symbol of each OFDM head frame, any subcarriers are used as time reference for the receiver to synchronize.

Figure 17

Synchronization symbol

### A3-1.7	Energy dispersal

The purpose of the energy dispersal is to avoid the transmission of signal patterns resulting in unwanted regularity. The individual inputs of energy dispersal scramblers should be scrambled by a modulo-2 with a pseudo-random binary sequence (PRBS), prior to channel encoding. The PRBS is defined as the output of the feedback shift register of Fig. 18. It should use a polynomial of degree 9, defined by:

Px=x9+x5+1

Figure 18

Pseudo-random binary sequence generator

### A3-1.8	Transmission mask of RF signal

Figure 19

Transmission spectral mask of NAVDAT RF signal with bandwidth F = 10 kHz 
Emission mask for 5, 3 and 1 kHz should fit within the mask for 10 kHz

### A3-1.9	Sequence for receiving scanning possibility

To allow reception of national or regional frequencies assigned to the NAVDAT system, the receiver uses a scan function.

Frequencies should then be scanned to monitor the reception of the pre-signal transmitted by the station before broadcast.

To ensure proper operation of the receiver scan function, the transmitters of active National or Regional NAVDAT coast stations should transmit, before the NAVDAT broadcast, a sequence of known data during 400 ms (pre scanning) repeated eight times for a total duration of 3.2 seconds.

To ease receiver demodulation of the NAVDAT broadcast, known data is using the same bandwidth and the same constellation as the subsequent NAVDAT broadcast.

Parameters:

Mode A during the day and Mode B at night; 10 kHz Bandwidth; 4 QAM.

The MIS and TIS will be identical to those used to broadcast messages from the NAVDAT station after the pre-scan sequence.

Known data uses a super-frame pattern of length 1.

To enable evaluation of BER, DS stream is filled with PRBS data using the polynomial:

Px=x20+x17+1

Each cell of the shift register should be pre-set to a logical 1 at the beginning of the sequence and the start of the PRBS sequence is synchronized with the beginning of each frame.

Figure 20

Transmission structure for scan facility

Insertion of MIS and TIS conforming to Table 18.

The frame structure is described in Annex 4.

## A3-2	Estimated usable data rate

In the 10 kHz channel bandwidth with HF surface wave propagation, the raw data rate available for the data stream (DS) is typically around 20 kbit/s with 16-QAM signal.

The number of subcarriers that hold data can be varied in order to adjust the channel protection. Higher channel protection (e.g. protection against multipath, fading, delay) results in a lower number of useful subcarriers.

Error correction coding must then be applied to the raw data rate to obtain the useful data rate. With a code rate of 0.5 to 0.75, the useful data rate is then between 6 and 29 kbit/s according to the mode with surface wave.

A higher code rate provides a higher useful data rate, but the radio coverage is accordingly reduced.

With the different modulation and code rate, the useful data rate is shown below.

TABLE 4

Estimated data rate for 10, 5, 3 and 1 kHz bandwidth for head frame for mode A
(given only for information)

| Mode | Spectrum occupancy (kHz) | Modulation (n-QAM) | Code rate | Estimated data rate (kbit/s) |
| --- | --- | --- | --- | --- |
| 0 | 10 | 4-QAM | 0.5 | 6.36 |
| 1 | 10 | 4-QAM | 0.75 | 9.56 |
| 2 | 10 | 16-QAM | 0.5 | 12.72 |
| 3 | 10 | 16-QAM | 0.75 | 19.12 |
| 4 | 10 | 64-QAM | 0.5 | 19.08 |
| 5 | 10 | 64-QAM | 0.75 | 28.68 |
| 6 | 5 | 4-QAM | 0.5 | 2.89 |
| 7 | 5 | 4-QAM | 0.75 | 4.35 |
| 8 | 5 | 16-QAM | 0.5 | 5.78 |
| 9 | 5 | 16-QAM | 0.75 | 8.69 |
| 10 | 5 | 64-QAM | 0.5 | 8.67 |
| 11 | 5 | 64-QAM | 0.75 | 13.04 |
| 12 | 3 | 4-QAM | 0.5 | 1.67 |
| 13 | 3 | 4-QAM | 0.75 | 2.52 |
| 14 | 3 | 16-QAM | 0.5 | 3.35 |
| 15 | 3 | 16-QAM | 0.75 | 5.03 |
| 16 | 3 | 64-QAM | 0.5 | 5.02 |
| 17 | 3 | 64-QAM | 0.75 | 7.55 |
| 18 | 1 | 4-QAM | 0.5 | 0.55 |
| 19 | 1 | 4-QAM | 0.75 | 0.84 |
| 20 | 1 | 16-QAM | 0.5 | 1.12 |
| 21 | 1 | 16-QAM | 0.75 | 1.68 |
| 22 | 1 | 64-QAM | 0.5 | 1.67 |
| 23 | 1 | 64-QAM | 0.75 | 2.52 |

TABLE 5

Estimated data rate for 10, 5, 3 and 1 kHz bandwidth for head frame for mode B

| Mode | Spectrum occupancy (kHz) | Modulation (n-QAM) | Code rate | Estimated data rate (kbit/s) |
| --- | --- | --- | --- | --- |
| 0 | 10 | 4-QAM | 0.5 | 5.705 |
| 1 | 10 | 4-QAM | 0.75 | 8.578 |
| 2 | 10 | 16-QAM | 0.5 | 11.41 |
| 3 | 10 | 16-QAM | 0.75 | 17.155 |
| 4 | 10 | 64-QAM | 0.5 | 17.115 |
| 5 | 10 | 64-QAM | 0.75 | 25.733 |
| 6 | 5 | 4-QAM | 0.5 | 2.67 |
| 7 | 5 | 4-QAM | 0.75 | 4.025 |
| 8 | 5 | 16-QAM | 0.5 | 5.34 |
| 9 | 5 | 16-QAM | 0.75 | 8.05 |
| 10 | 5 | 64-QAM | 0.5 | 8.01 |
| 11 | 5 | 64-QAM | 0.75 | 12.075 |
| 12 | 3 | 4-QAM | 0.5 | 1.46 |
| 13 | 3 | 4-QAM | 0.75 | 2.21 |
| 14 | 3 | 16-QAM | 0.5 | 2.92 |
| 15 | 3 | 16-QAM | 0.75 | 4.42 |
| 16 | 3 | 64-QAM | 0.5 | 4.38 |
| 17 | 3 | 64-QAM | 0.75 | 6.63 |
| 18 | 1 | 4-QAM | 0.5 | 0.22 |
| 19 | 1 | 4-QAM | 0.75 | 0.35 |
| 20 | 1 | 16-QAM | 0.5 | 0.44 |
| 21 | 1 | 16-QAM | 0.75 | 0.70 |
| 22 | 1 | 64-QAM | 0.5 | 0.66 |
| 23 | 1 | 64-QAM | 0.75 | 1.05 |

TABLE 6

Estimated data rate for 10, 5 and 3 kHz bandwidth for head frame for mode C

| Mode | Spectrum occupancy (kHz) | Modulation (n-QAM) | Code rate | Estimated data rate (kbit/s) |
| --- | --- | --- | --- | --- |
| 0 | 10 | 4-QAM | 0.5 | 4.60 |
| 1 | 10 | 4-QAM | 0.75 | 6.92 |
| 2 | 10 | 16-QAM | 0.5 | 9.20 |
| 3 | 10 | 16-QAM | 0.75 | 13.84 |
| 4 | 10 | 64-QAM | 0.5 | 13.80 |
| 5 | 10 | 64-QAM | 0.75 | 20.76 |
| 6 | 5 | 4-QAM | 0.5 | 2.13 |
| 7 | 5 | 4-QAM | 0.75 | 3.22 |
| 8 | 5 | 16-QAM | 0.5 | 4.26 |
| 9 | 5 | 16-QAM | 0.75 | 6.43 |
| 10 | 5 | 64-QAM | 0.5 | 6.39 |
| 11 | 5 | 64-QAM | 0.75 | 9.65 |
| 12 | 3 | 4-QAM | 0.5 | 1.14 |
| 13 | 3 | 4-QAM | 0.75 | 1.72 |
| 14 | 3 | 16-QAM | 0.5 | 2.27 |
| 15 | 3 | 16-QAM | 0.75 | 3.45 |
| 16 | 3 | 64-QAM | 0.5 | 3.41 |
| 17 | 3 | 64-QAM | 0.75 | 5.17 |

## A3-3	NAVDAT HF transmitter performance specification

TABLE 7

Minimum NAVDAT HF transmitter performance specification

| Parameters | Required results |
| --- | --- |
| Frequency band | 4 to 27.5 MHz |
| Carrier frequency error | Within ±2.5 Hz of the nominal frequency |
| Spectrum mask | Comply with the requirement of Fig. 18 |
| Transmitter third order intermodulation rejection ratio | > 40 dBc |
| Transmitter emission spurious (all power range) | −50 dB without exceeding the absolute level of 50 mW (17 dBm) |
| NOTE: The transmitter may also cover the MF band 415 to 526.5 kHz for International 500 kHz and future National NAVDAT frequencies. Refer to Recommendation ITU-R M.2010 for technical specifications. The emission class used is W7D. |  |

## A3-4	NAVDAT ship receiver

### A3-4.1	NAVDAT ship receiver description

The ship receiver block logical diagram is shown in Fig. 22.

A typical NAVDAT 500 kHz and NAVDAT HF digital receiver is composed of several basic blocks:

–	reception antenna and optional GNSS antenna;

–	RF front end;

–	demodulator;

–	file demultiplexer;

–	controller;

–	control and display unit (CDU);

–	data interface;

–	embedded table of frequencies, stations and areas (see § A3-4.1.10);

–	power supply.

The NAVDAT ship receiver can receive and decode the main international MF channel (500 kHz) and the main international HF channel (4 226 kHz) at the same time with two complete independent channels.

The first channel is constantly listening to the 500 kHz.

The second channel is constantly listening to the 4 226 kHz.

A third channel should be watching and scanning all the other NAVDAT frequencies (International, national or regional in MF and attributed HF frequencies). The design of this third channel allows reception and decoding future potential national, regional or local transmitters using MF or HF channels:

1	The maritime MF band from 415 to 526.5 kHz (except 500 kHz).

2	The frequencies assigned to NAVDAT in RR Appendix 15: 6 337.5, 8 443, 12 663.5, 16 909.5 and 22 450.5 kHz.

3	The HF-bands of RR Appendix 17 assignable for wideband data transmissions and usable by NAVDAT: 4, 6, 8, 12, 16 and 22 MHz.

The demodulated signal(s) received by scanning can be decoded simultaneously or non-simultaneously.

The choice of frequencies to be scanned should be based on information on the NAVDAT stations declared and stored by the receiver (table updated via message 63).

The receiver should first determine the NAVAREA and METAREA zone in which the vessel is located (from its position) with the possibility by the operator to add some NAVDAT stations outside of this NAVAREA / METAREA.

From the table (see § A3-4.1.10), the receiver should determine the future allocated slots and the frequencies used.

The operator can select the national or regional NAVDAT stations of interest. As soon the receiver detects the pre-scanning of these stations, it will notify the operator and decode the broadcast message.

These frequencies should then be scanned to monitor the reception of the pre-signal transmitted by the station before broadcast.

The receiving antenna is common to the three channels. It is recommended that the antenna be equipped with two outputs for sharing with another MF/HF receiver.

Below is a generic block diagram of an SDR receiver given for information.

The design of NAVDAT receivers is left to the initiative of each manufacturer.

Figure 21

Model of generic software defined radio NAVDAT receiver

FIGURE 22

NAVDAT receiver logical diagram

#### A3-4.1.1	Reception antenna and global navigation satellite antenna

The receiving antenna can be a vertical H field antenna (recommended on an EMC noisy ship) or an E field antenna. The ship’s NAVDAT receiver can also receive NAVDAT MF channels. It is recommended that the omnidirectional receiving antenna covers, at minimum, the frequency band 415 kHz to 27.5 MHz.

A GNSS antenna connected to an internal GNSS receiver (or connection to the existing ship GNSS receiver) is also needed in order to obtain ship’s position and time.

#### A3-4.1.2	RF front end

This block includes the RF filter, RF amplifier and baseband output with for all a scanning possibility.

High sensitivity and high dynamic range are necessary with protection against strong RF fields from ship transmitting antennas or lightings.

The passband of the input filters must allow reception of the maritime MF band from 415 to 526.5 kHz and all maritime HF bands.

It is recommended to place a notch filter on the MF broadcasting band (from 526.5 kHz).

Receiver design can be classic or SDR type with at least three channels (not limited).

#### A3-4.1.3	Demodulator

This stage demodulates the baseband OFDM signal and recreates the data stream that holds the transmitted message files.

It implements:

–	time/frequency synchronization;

–	channel estimation;

–	automatic modulation recovery;

–	error correction.

The NAVDAT receiver should be able to detect the following modulation parameters automatically:

–	4-, 16- or 64-QAM;

–	type of error coding.

In addition to the DS, it reports the information filled in the TIS and MIS. Furthermore, it reports complementary information about the channel such as:

–	estimated SNR;

–	BER;

–	MER.

#### A3-4.1.4	File demultiplexer

The file demultiplexer:

–	receives the message files from the controller;

–	verifies that the message files are marked for its attention (type of broadcast mode);

–	decrypts the message files if needed/able;

–	makes the message files available for the terminal application that should use the message files;

–	deletes the out-of-date message files.

Depending on the final application, the message file can be:

–	stored on an onboard server accessible through the ship network;

–	display on the receiver CDU directly;

–	sent directly to the final application.

#### A3-4.1.5	Controller

The controller:

–	extracts the message files from the DS (merge packets into files);

–	interprets the TIS and MIS and the other pieces of information given by the demodulator;

–	collects the following information from the file demultiplexer:

•	total number of decoded message files;

•	number of available message files;

•	error event (e.g. decrypt errors).

A man-machine interface may be available in order to display and check the reception parameters.

#### A3-4.1.6	Control and display unit

The receiver can provide a display and control unit. The function of this unit is:

–	display the special information and by configuring the interface to be connected to a dedicated equipment application (e.g. e-navigation) and manage the licensed contents of the ship (e.g. ship identification, encryption);

–	display and check the receiving parameters;

–	display the message content according to the application classification of the message file.

This CDU may be a special application running on an external computer, and the receiver may be a black-box device.

#### A3-4.1.7	Data interface

The receiver obtains the data from external devices such as GNSS through the data interface. The controller classifies the message files according to their applications and provides the message files to the application devices through the data interface.

The equipment should provide a data interface that complies with the requirements of the IEC 61162 series. It is advisable to provide Ethernet and USB interfaces for high-speed transmission of files and also provide a printer interface (not limited).

The equipment should be able to configure parameters of data ports for communication with other ship equipment.

The equipment should include an interface for alert management in accordance with IMO Resolution MSC.302(87) on Performance standards for bridge alert management.

#### A3-4.1.8	Power supply

The connection to the ship’s power supply must be protected against surges and EMI.

#### A3-4.1.9	Receiver ID

It should be possible to configure the receiver with:

the identity of the vessel (according to Recommendation ITU-R M.585)

the main Group identity (according to Recommendation ITU-R M.585)

additional lists of identities (MMSI’S) may be provided.

See Table 20 and note.

#### A3-4.1.10	Stored tables

The receiver should have the possibility of storing information in different memorized tables which can be updated by the reception of the message 63. This message should be authenticated by the shore authority.

For example:

1	The list of coast stations with:

Area

Country

Longitude

Latitude

Name

Slots

Frequency used

This stored table is queried when the identities of the transmitter stations are received and the complete parameters of the received NAVDAT coast station should be displayed in plain text.

2	The list of subject messages

Table with subject message 01 to 63

All the tables in memory can be updated by receiving the message 63.

#### A3-4.1.11	Storage

##### A3-4.1.11.1	Non-volatile files message memory

For each frequency provided it should be possible to record at least 100 message files in non-volatile memory. It should not be possible for the user to erase file messages from the memory. When the memory is full, the oldest file message must be replaced by the new messages.

The user should be able to mark the individual files of a message from permanent retention. These file messages can occupy up to 25% of available memory and should not be overwritten by new files. When no longer needed, the user must be able to delete the tag on these files, which can be overwritten normally.

Duplicate message could be recognized by the equipment and should not be stored.

The storage capacity of this memory should not be less than 1 GB.

##### A3-4.1.11.2	Programmable control memories

Information identifying the transmitter service area and the designator of each type of message in programmable memory should not be erased by interruptions in the power supply of less than 24 h.

The equipment should be able to store at least the time, transmitter identification, subject and content of received message. The storage capacity should not be less than 1 GB.

When power supply is interrupted unexpectedly, the equipment should protect the stored data and software parameters.

The equipment should be able to display, delete and query stored messages, and be able to output messages manually or automatically to appropriate ship equipment (such as the electronic chart and display information system (ECDIS).

#### A3-4.1.12	Alert

Upon receipt of a new search and rescue (SAR) related information message with distress or urgency priority, the receiver should give a continuous audible alarm. It should only be possible to reset this alarm manually. The position information contained in the SAR messages may be transmitted to other navigation equipment (e.g. ECDIS, electronic navigational chart plotter).

#### A3-4.1.13	Test facilities

The equipment should be provided with a facility to test that the radio receiver, the display and the non-volatile memory are functioning correctly and to display self-test results. In case of using a specific antenna it also must be checked by this process.

#### A3-4.1.14	Updates

The software/firmware of the equipment should be able to be updated. The update should be performed by using a USB port or reception of message 63 (update receiver software). This function is necessary to follow the evolutions of the GMDSS master plan for the new NAVDAT stations as well as for the future revisions of the ITU Recommendations.

#### A3-4.1.15	Scan function

As indicated in § A3-4.1 the ship's NAVDAT receiver permanently monitors the frequencies 500 and 4 226 kHz and can simultaneously decode the signals received on these two frequencies.

To allow reception of national or regional frequencies assigned to the NAVDAT system, the receiver uses a scan function on the following maritime frequency bands:

–	The MF band from 415 to 526.5 kHz (except 500 kHz).

–	The frequencies assigned to NAVDAT in RR Appendix 15: 6 337.5, 8 443, 12 663.5, 16 909.5 and 22 450.5 kHz.

–	The HF-bands of RR Appendix 17 assignable for wideband data transmissions and usable by NAVDAT: 4, 6, 8, 12, 16  and 22 MHz.

The receiver should search its stored NAVDAT station table (updated via message code 63) for all frequencies that can be sequentially scanned in relation to allocated slots (time reference).

The demodulated signal(s) received by scanning can be decoded simultaneously or non-simultaneously according to the resources of the NAVDAT receiver at that moment.

To ensure proper operation of the receiver scan function, the transmitters of active National or Regional NAVDAT coast stations should broadcast, before the NAVDAT frames, a known data repeat 8 times for a total duration of 3.2 seconds (see § A3-1.9 and Fig. 20 in Annex 3).

This should allow the receiver to detect the transmission and tune to the frequency, measure its SNR, identify the station and its NAVAREA / METAREA area.

## A3-5	Minimum NAVDAT ship receiver performance specifications

These assumed ship receiver specifications are set out below with the objective to obtain minimum S/N for a good OFDM demodulation (4-QAM, 16-QAM or 64-QAM).

The ship's NAVDAT receiver must receive the two international NAVDAT frequencies: 500 kHz and 4 226 kHz but also the MF and HF frequency band in scan mode (see Table 8).

TABLE 8

NAVDAT ship receiver minimum performance specifications

| Total frequency band Main MF frequency (centre frequency) Main HF frequency (centre frequency) | 415 to 526.5 kHz and 4 to 27.5 MHz maritime band 500 kHz 4 226 kHz |
| --- | --- |
| MF maritime band | 415 to 526.5 kHz |
| HF maritime bands | Maritime HF bands RR Appendix 17 |
| Adjacent channel protection | > 40 dB at 5 kHz |
| Noise factor | < 10 dB (< 20 dB for MF band) |
| Usable sensitivity for BER = 10−4 after error correction | < −95 dBm |
| Dynamic | > 80 dB |
| Minimal usable RF field (with adapted receiving antenna) | 20 dB(µV/m) |

Annex 4

Transmission structure

## A4-1	Frame structure

NAVDAT head frame structure contains synchronization head (the first symbol), MIS, TIS and DS shown as follows.

FIGURE 23

NAVDAT Frame structure

Head frame length is 400 ms.

Standard frame structure does not include DS stream without Synchronization head MIS or TIS.

Standard frame length is 400 ms. A sequence of one head frame and N-1 standard frame constitutes a super-frame of the length of N. The NAVDAT broadcast should use a super-frame pattern of the length of 5.

## A4-2	Synchronization head

Synchronization head is the first OFDM symbol of each head frame for the receiver to synchronize and for the information on every subcarrier is shown in Table 9.

TABLE 9

Synchronization head sequence (in mode A)

| Bandwidth and number of subcarriers | Synchronization head sequence |
| --- | --- |
| 10 kHz 229 | −1 1 1 1 1 1 1 −1 1 1 −1 −1 1 −1 1 1 1 1 1 −1 −1 1 −1 1 −1 −1 −1 1 1 1 1 −1 1 −1 −1 −1 −1 1 1 −1 1 1 1 −1 −1 −1 1 1 1 −1 1 −1 −1 1 1 −1 1 1 −1 1 1 −1 −1 −1 1 −1 1 −1 −1 1 −1 −1 1 −1 1 1 −1 −1 −1 −1 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 −1 0 −1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 −1 1 −1 1 −1 −1 −1 −1 −1 −1 1 1 −1 −1 −1 −1 −1 1 1 1 1 1 −1 1 −1 1 1 1 1 −1 1 1 1 1 −1 −1 −1 −1 1 1 1 −1 −1 1 1 1 −1 1 1 1 −1 1 1 −1 1 −1 1 1 −1 −1 1 1 −1 −1 1 −1 −1 −1 −1 1 −1 1 −1 1 −1 1 −1 −1 1 1 |
| 5 kHz 115 | 1 −1 1 1 −1 −1 −1 1 −1 1 −1 −1 1 −1 −1 1 −1 1 1 −1 −1 −1 −1 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 −1 0 1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 −1 1 −1 1 −1 −1 −1 −1 −1 −1 1 1 −1 −1 −1 −1 −1 1 1 1 1 1 −1 |
| 3 kHz 69 | 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 1 0 −1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 |
| 1 kHz 23 | 1 −1 1 −1 −1 1 −1 1 −1 1 1 0 1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 |

Synchronization head sequence (in mode B)

| Bandwidth and number of subcarriers | Synchronization head sequence |
| --- | --- |
| 10 kHz 207 | −1 1 −1 1 1 1 1 1 −1 −1 1 −1 1 −1 −1 −1 1 1 1 1 −1 1 −1 −1 −1 −1 1 1 −1 1 1 1 −1 −1 −1 1 1 1 −1 1 −1 −1 1 1 −1 1 1 −1 1 1 −1 −1 −1 1 −1 1 −1 −1 1 −1 −1 1 −1 1 1 −1 −1 −1 −1 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 −1 0 −1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 −1 1 −1 1 −1 −1 −1 −1 −1 −1 1 1 −1 −1 −1 −1 −1 1 1 1 1 1 −1 1 −1 1 1 1 1 −1 1 1 1 1 −1 −1 −1 −1 1 1 1 −1 −1 1 1 1 −1 1 1 1 −1 1 1 −1 1 −1 1 1 −1 −1 1 1 −1 −1 1 −1 −1 −1 −1 |
| 5 kHz 103 | −1 1 −1 1 −1 −1 1 −1 −1 1 −1 1 1 −1 −1 −1 −1 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 −1 0 1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 −1 1 −1 1 −1 −1 −1 −1 −1 −1 1 1 −1 −1 −1 −1 −1 |
| 3 kHz 61 | −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 1 0 −1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 |
| 1 kHz 19 | 1 −1 −1 1 −1 1 −1 1 1 0 1 −1 −1 −1 1 −1 −1 −1 −1 |

Synchronization head sequence (in mode C)

| Bandwidth and number of subcarriers | Synchronization head sequence |
| --- | --- |
| 10 kHz 139 | −1 1 1 1 −1 1 −1 −1 1 1 −1 1 1 −1 1 1 −1 −1 −1 1 −1 1 −1 −1 1 −1 −1 1 −1 1 1 −1 −1 −1 −1 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 −1 0 −1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 −1 1 −1 1 −1 −1 −1 −1 −1 −1 1 1 −1 −1 −1 −1 −1 1 1 1 1 1 −1 1 −1 1 1 1 1 −1 1 1 1 1 −1 |
| 5 kHz 69 | 1 −1 −1 1 −1 −1 −1 1 −1 1 1 1 −1 −1 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 −1 0 1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 1 1 −1 −1 1 1 −1 1 −1 1 −1 1 1 1 |
| 3 kHz 41 | 1 −1 −1 1 1 −1 −1 −1 1 1 −1 1 −1 −1 1 −1 1 −1 1 1 0 −1 1 −1 −1 −1 1 −1 −1 −1 −1 −1 1 −1 −1 −1 1 1 −1 −1 1 1 |
| 1 kHz 13 | 1 −1 1 −1 1 1 0 1 −1 −1 −1 1 −1 |

For the different channel bandwidth, the OFDM symbol index corresponding to the synchronization header is shown in Table 10.

TABLE 10

Index of the synchronization head symbols

| Mode | Ns | Index of the OFDM symbol per frame |
| --- | --- | --- |
| A | 15 | 1 |
| B | 15 | 1 |
| C | 20 | 1 |

## A4-3	Modulation information stream

### A4-3.1	Structure

MIS is used to provide the information about channel spectrum occupancy as well as TIS and DS modulation:

–	information of channel bandwidth:	2 bits;

–	information of robustness:	2 bits;

–	information of TIS modulation:	1 bit;

–	information of DS modulation: 	2 bits;

–	error encoding of DS:	1 bit;

–	Cyclic redundancy check (CRC):	8 bits;

Total bits:				16 bits.

TABLE 11

Information of channel bandwidth

| Case (see Table 2 of Annex 3) | Patterns of bits | Spectrum occupancy (kHz) |
| --- | --- | --- |
| 1 | 00 | 1 |
| 2 | 01 | 3 |
| 3 | 10 | 5 |
| 4 | 11 | 10 |

TABLE 12

Information of transmitter information stream TIS modulation

| Patterns of bits | Modulation |
| --- | --- |
| 0 | 4-QAM |
| 1 | 16-QAM |

TABLE 13

Information of data stream modulation

| Patterns of bits | Modulation |
| --- | --- |
| 00 | 4-QAM |
| 01 | 16-QAM |
| 10 | 64-QAM |

TABLE 14

Mode of robustness

| Mode | Pattern of bit |
| --- | --- |
| A | 00 |
| B | 01 |
| C | 10 |
| D | 11 |

TABLE 15

Information of data stream (DS) code rate

| Pattern of bits | Code rate |
| --- | --- |
| 0 | 0.5 |
| 1 | 0.75 |

### A4-3.2	Encoding

The MIS is encoded using a (16, 48) polar code, where the positions of the information subchannels are determined by the 0's in the following vector:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 1 1 1 0 1 0 0 0 1 0 0 0 0 0 0 0.

After standard polar encoding, the codeword should be punctured from 64 to 48 bits by discarding the bits indexed by 1-16.

## A4-4	Transmitter information stream

### A4-4.1	Structure

TIS is used to provide the information about encoding of DS, transmitter, and time for the receiver:

–	identifier of the transmitter: 	32 bits;

–	day and time:	17 bits;

–	reserved 1 (for 4-QAM):	19 bits (default: 0);

–	reserved 2 (for 16-QAM):	95 bits (default: 0);

–	CRC:			8 bits.

Total of bits:			76 for 4-QAM and 152 for 16-QAM

TABLE 16

Identifier of the transmitter

| Coding | Identifier of the transmitter |
| --- | --- |
| I | 8 bits ASCII |
| D | 8 bits ASCII |
| NAV/MET AREA | 5 bits |
| STATION NUMBER | 11 bits |
| Total | 32 bits |

The header is two 8-bits ASCII encoded characters with the capital letters I and D.

The coding of the NAV/MET areas should be done in binary on 8 bits (maximum 31 areas).

The station number allocated for a frequency should be coded on 11 bits (maximum of 2 047 stations by area).

Total of 32 bits should thus be used for the identification of each NAVDAT transmitter.

Examples of coast station identification code

A NAVDAT station located in NAVAREA/METAREA III (3) and transmitting on 4 226 kHz would have the following identity (with the numbering 85 allocated to the station):

I	01001001	8 bits ASCII

D	01000100	8 bits ASCII

3	00011	5 bits binary

85	00001010101	11 bits binary

Total		32 bits

TABLE 17

Information of time

| Parameter | Bit number | Description |
| --- | --- | --- |
| Hour of start time in UTC | 5 | hour |
| Minute of start time in UTC | 6 | minute |
| Duration of the broadcast | 6 | 0 to 59 minutes (Round up to next minute) |

### A4-4.2	Encoding

The TIS is encoded using a (76, 152) polar code. For the 4 QAM, the total bits of TIS is 76, so the 76, 152 polar code is used once; while for 16 QAM, the total bits of TIS is 152, so the 76, 152 polar code is used twice where the positions of the information subchannels are determined by the 0's in the following vector:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 1 1 1 1 1 1 1 1 1 1 1 0 1 0 0 0 1 1 1 0 1 0 0 0 1 0 0 0 0 0 0 0 1 1 1 1 1 1 1 0 1 1 1 0 1 0 0 0 1 1 1 0 1 0 0 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 1 1 1 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1.

After standard polar encoding, the codeword should be punctured from 256 bits to 152 bits by selecting the bits indexed by 1-112 and 129-168.

### A4-4.3	Position

There are 100 (MIS:48, TIS:152) carriers for MIS and TIS transmission. Table 18 gives the position of these carriers.

TABLE 18

Position of the modulation information stream and transmitter information stream carriers
For 3, 5 and 10 kHz bandwidth in mode A and B, and 1 kHz bandwidth in mode A

| Symbol | Carrier number |
| --- | --- |
| 2 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 3 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 4 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 5 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 6 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 7 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 8 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 9 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 10 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |
| 11 | −10, −8, −6, −4, −2, 2, 4, 6, 8, 10 |

For 1 kHz bandwidth in mode B

| Symbol | Carrier number |
| --- | --- |
| 2 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 3 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 4 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 5 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 6 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 7 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 8 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 9 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 10 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 11 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 12 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 13 | −8, −6, −4, −2, 2, 4, 6, 8 |
| 14 | −4, −2, 2, 4 |

For 1 kHz bandwidth in mode C

| Symbol | Carrier number |
| --- | --- |
| 2 | −6, −4, −2, 2, 4, 6 |
| 3 | −6, −4, −2, 2, 4, 6 |
| 4 | −6, −4, −2, 2, 4, 6 |
| 5 | −6, −4, −2, 2, 4, 6 |
| 6 | −6, −4, −2, 2, 4, 6 |
| 7 | −6, −4, −2, 2, 4, 6 |
| 8 | −6, −4, −2, 2, 4, 6 |
| 9 | −6, −4, −2, 2, 4, 6 |
| 10 | −6, −4, −2, 2, 4, 6 |
| 11 | −6, −4, −2, 2, 4, 6 |
| 12 | −6, −4, −2, 2, 4, 6 |
| 13 | −6, −4, −2, 2, 4, 6 |
| 14 | −6, −4, −2, 2, 4, 6 |
| 15 | −6, −4, −2, 2, 4, 6 |
| 16 | −6, −4, −2, 2, 4, 6 |
| 17 | −6, −4, −2, 2, 4, 6 |
| 18 | −4, −2, 2, 4 |

## A4-5	Data stream

### A4-5.1	Structure

Data stream generally consist of either text information or files of information. A generalized packet delivery allows the delivery of text information and files for various services in the same data stream. Services can also be carried by a series of single packets.

The structure of a packet is as follows:

–	Head	16 bits

–	Data field	n bytes

–	CRC	16 bits.

The head is made up as follows:

–	toggle bit	1 bit

–	first flag	1 bit

–	last flag	1 bit

–	packet ID	10 bits

–	padded packet indicator	1 bit

–	reserved	2 bits;

Total:	16 bits.

The packet length is fixed for each transmission and given in Table 26.

Toggle bit: This bit should be maintained in the same state as long as packets from the same text message or file are being transmitted. When a packet from a different text message or file is sent for the first time, this bit should be inverted with respect to its previous state. If a text message or file, which may consist of several packets, is repeated, then this bit should remain unchanged.

First flag, Last flag: These flags are used to identify particular packets which form a succession of packets. The flags are assigned as follows:

TABLE 19

Coding of first flag and last flag

| First flag | Last flag | The packet is |
| --- | --- | --- |
| 0 | 0 | an intermediate packet |
| 0 | 1 | the last packet of a data unit |
| 1 | 0 | the first packet of a data unit |
| 1 | 1 | the one and only packet of a data unit |

Packet ID: This 10 bits field indicates the packet ID of this packet. Initial value at the start of increments by 1 for each packet and re-cycles to zero after reaching the maximum value of 1 023.

Padded Packet Indicator: This 1-bit flag indicates whether the data field carries padding or not, as follows:

0: no padding is present, all data bytes in the data field are useful;

1: padding is present, the first two bytes give the number of useful data bytes in the data field.

Reserved: This 2 bits field is reserved for future use.

Data field: It contains the useful data intended for a particular service. It can be text information or file information (see also Table 26).

The first information in the data field is broadcast mode, which is defined in Table 20.

TABLE 20

Broadcast mode

| Mode | Pattern of bit | Coding | Comments |
| --- | --- | --- | --- |
| General | 00 | 40 bits |  |
| Selective ship | 01 | 40 bits | MMSI of the ship |
| Group of ships | 10 | 40 bits | Group of ship’s ID (main or additional for future use) |
| Selective area | 11 | 176 bits | Geographic coordinates and radius of the defined area |

In the case of selective broadcast over a specific area, this geographical area is defined as follows:

–	The zone number assigned by the server (maximum 99) + space.

–	The area is determined by four geographical points in degrees minutes seconds (DMS) starting from the highest point and turning clockwise (Latitude followed by Longitude).

For General, Selective ship and Group of ships use 40 bits.

Recommendation ITU-R M.585 recommends that X10 be reserved for future use. Recommendation ITU-R·M.1080 recommends that X10 be used for multiple installations on a boat. Each digit coded on 4 bits.

To reduce the number of bits used for the broadcast selective area, the coding is done in binary (instead of ASCII).

Table 21 shows the coding possibilities (in binary) with a total of 176 bits (instead of initial 512 bits)

TABLE 21

Geographical zone coding

|  | Zone | Latitude |  |  | Longitude |  |  | Complement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Degrees | Minutes | Seconds | Degrees | Minutes | Seconds |  |
| Bits | 7 | 8 | 6 | 6 | 9 | 6 | 6 | 5 |
| Range | 0-127 | −90° – +90° | 0-59’ | 0-59” | −180° – +180° | 0-59’ | 0-59” |  |

All numbers are unsigned integer, except degrees which are signed integers (one’s complement).

For example, for a zone N°1:

Position 1	47°42’22” N and 137°28’59” E

Position 2:	37°50’24” N and 139°00’10” E

Position 3:	32°04’57” N and 129°29’05” E

Position 4:	33°04’56” N and 127°30’28” E

Giving:

0000001

00101111  101010  010110  010001001  011100  111011

00100101  110010  011000  010001011  000000  001010

00100000  000100  111001  010000001  011101  000101

00100001  000100  111000  001111111  011110  011100

00000

And, for a zone N°2:

Position 1	47°42’22” S and 137°28’59” W

Position 2:	37°50’24” S and 139°00’10” W

Position 3:	32°04’57” S and 129°29’05” W

Position 4:	33°04’56” S and 127°30’28” W

Giving

0000010

11010000  101010  010110  101110110  011100  111011

11011010  110010  011000  101110100  000000  001010

11011111  000100  111001  101111110  011101  000101

11011110  000100  111001  110000000  011110  011100

00000

To define a circle with a given radius, the first geographic position only should be used (the bits of the other three positions are set to 0) and the radius is determined by the 5 complementary bits.

These 5 bits are used to give the radius to be drawn from this point with a multiplication coefficient of 10 (minimum 10 NM and maximum 310 NM).

Example for a radius of 100 NM: 100/10 = 10 = 01010 and for a radius of 50 NM: 50/10 = 5 = 00101.

TABLE 22

Level of message

| Coding | Definition level |
| --- | --- |
| 00 | Routine |
| 01 | Safety |
| 10 | Urgency |
| 11 | Distress |

The third piece of information specifies the subject of the message according to Table 29 (from 1 to 63) coded on 6 bits:

1 = 000001

63 = 111111

The fourth information gives the number of the message from 1 to 999 coded on 10 bits.

Example:	1 = 0000000001

999 = 1111100111

CRC: This 16-bit CRC should be calculated on the header and the data field.

### A4-5.2	Encoding

NAVDAT data stream is encoded by low-density parity-check (LDPC), and different encoding parameters should be adopted in different modes (see Table 14). Table 23 gives the LDPC parameters of mode A, B and C for all bandwidths.

TABLE 23

Low-density parity-check parameters of data stream for mode A

| Bandwidth (kHz) | Number of subcarriers per OFDM | Number of pilots | Number of data subcarriers | Modulation | TIS and MIS subcarriers | LDPC code | Code blocks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 228 | 532 | 2660 | 4-QAM | 100 | LDPC (5120,2560) | 1 |
| 10 | 228 | 532 | 2660 | 4-QAM | 100 | LDPC (5120,3840) | 1 |
| 10 | 228 | 532 | 2660 | 16-QAM | 100 | LDPC (5120,2560) | 2 |
| 10 | 228 | 532 | 2660 | 16-QAM | 100 | LDPC (5120,3840) | 2 |
| 10 | 228 | 532 | 2660 | 64-QAM | 100 | LDPC (5120,2560) | 3 |
| 10 | 228 | 532 | 2660 | 64-QAM | 100 | LDPC (5120,3840) | 3 |
| 5 | 114 | 271 | 1325 | 4-QAM | 100 | LDPC (2450,1225) | 1 |
| 5 | 114 | 271 | 1325 | 4-QAM | 100 | LDPC (2450,1838) | 1 |
| 5 | 114 | 271 | 1325 | 16-QAM | 100 | LDPC (2450,1225) | 2 |
| 5 | 114 | 271 | 1325 | 16-QAM | 100 | LDPC (2450,1838) | 2 |
| 5 | 114 | 271 | 1325 | 64-QAM | 100 | LDPC (2450,1225) | 3 |
| 5 | 114 | 271 | 1325 | 64-QAM | 100 | LDPC (2450,1838) | 3 |
| 3 | 68 | 159 | 793 | 4-QAM | 100 | LDPC (1386,693) | 1 |
| 3 | 68 | 159 | 793 | 4-QAM | 100 | LDPC (1386,1040) | 1 |
| 3 | 68 | 159 | 793 | 16-QAM | 100 | LDPC (1386,693) | 2 |
| 3 | 68 | 159 | 793 | 16-QAM | 100 | LDPC (1386,1040) | 2 |
| 3 | 68 | 159 | 793 | 64-QAM | 100 | LDPC (1386,693) | 3 |
| 3 | 68 | 159 | 793 | 64-QAM | 100 | LDPC (1386,1040) | 3 |
| 1 | 22 | 56 | 252 | 4-QAM | 100 | LDPC (304,152) | 1 |
| 1 | 22 | 56 | 252 | 4-QAM | 100 | LDPC (304,228) | 1 |
| 1 | 22 | 56 | 252 | 16-QAM | 100 | LDPC (304,152) | 2 |
| 1 | 22 | 56 | 252 | 16-QAM | 100 | LDPC (304,228) | 2 |
| 1 | 22 | 56 | 252 | 64-QAM | 100 | LDPC (304,152) | 3 |
| 1 | 22 | 56 | 252 | 64-QAM | 100 | LDPC (304,228) | 3 |

TABLE 24

Low density parity check parameters of data stream for mode B

| Bandwidth (kHz) | Number of subcarriers per OFDM | Number of pilots | Number of data subcarriers | Modulation | TIS and MIS subcarriers | LDPC code | Code blocks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 206 | 485 | 2399 | 4-QAM | 100 | LDPC (4598,2299) | 1 |
| 10 | 206 | 485 | 2399 | 4-QAM | 100 | LDPC (4598,3449) | 1 |
| 10 | 206 | 485 | 2399 | 16-QAM | 100 | LDPC (4598,2299) | 2 |
| 10 | 206 | 485 | 2399 | 16-QAM | 100 | LDPC (4598,3449) | 2 |
| 10 | 206 | 485 | 2399 | 64-QAM | 100 | LDPC (4598,2299) | 3 |
| 10 | 206 | 485 | 2399 | 64-QAM | 100 | LDPC (4598,3449) | 3 |
| 5 | 102 | 243 | 1185 | 4-QAM | 100 | LDPC (2170,1085) | 1 |
| 5 | 102 | 243 | 1185 | 4-QAM | 100 | LDPC (2170,1628) | 1 |
| 5 | 102 | 243 | 1185 | 16-QAM | 100 | LDPC (2170,1085) | 2 |
| 5 | 102 | 243 | 1185 | 16-QAM | 100 | LDPC (2170,1628) | 2 |
| 5 | 102 | 243 | 1185 | 64-QAM | 100 | LDPC (2170,1085) | 3 |
| 5 | 102 | 243 | 1185 | 64-QAM | 100 | LDPC (2170,1628) | 3 |
| 3 | 60 | 140 | 700 | 4-QAM | 100 | LDPC (1200,600) | 1 |
| 3 | 60 | 140 | 700 | 4-QAM | 100 | LDPC (1200,900) | 1 |
| 3 | 60 | 140 | 700 | 16-QAM | 100 | LDPC (1200,600) | 2 |
| 3 | 60 | 140 | 700 | 16-QAM | 100 | LDPC (1200,900) | 2 |
| 3 | 60 | 140 | 700 | 64-QAM | 100 | LDPC (1200,600) | 3 |
| 3 | 60 | 140 | 700 | 64-QAM | 100 | LDPC (1200,900) | 3 |
| 1 | 18 | 47 | 205 | 4-QAM | 100 | LDPC (210,105) | 1 |
| 1 | 18 | 47 | 205 | 4-QAM | 100 | LDPC (210,158) | 1 |
| 1 | 18 | 47 | 205 | 16-QAM | 100 | LDPC (210,105) | 2 |
| 1 | 18 | 47 | 205 | 16-QAM | 100 | LDPC (210,158) | 2 |
| 1 | 18 | 47 | 205 | 64-QAM | 100 | LDPC (210,105) | 3 |
| 1 | 18 | 47 | 205 | 64-QAM | 100 | LDPC (210,158) | 3 |

TABLE 25

Low density check parameters of data stream for mode C

| Bandwidth (kHz) | Number of subcarriers per ofdm | Number of pilots | Number of data subcarriers | Modulation | TIS and MIS subcarriers | LDPC code | Code blocks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 138 | 665 | 1957 | 4-QAM | 100 | LDPC (3714,1857) | 1 |
| 10 | 138 | 665 | 1957 | 4-QAM | 100 | LDPC (3714,2786) | 1 |
| 10 | 138 | 665 | 1957 | 16-QAM | 100 | LDPC (3714,1857) | 2 |
| 10 | 138 | 665 | 1957 | 16-QAM | 100 | LDPC (3714,2786) | 2 |
| 10 | 138 | 665 | 1957 | 64-QAM | 100 | LDPC (3714,1857) | 3 |
| 10 | 138 | 665 | 1957 | 64-QAM | 100 | LDPC (3714,2786) | 3 |
| 5 | 68 | 323 | 969 | 4-QAM | 100 | LDPC (1738,869) | 1 |
| 5 | 68 | 323 | 969 | 4-QAM | 100 | LDPC (1738,1304) | 1 |
| 5 | 68 | 323 | 969 | 16-QAM | 100 | LDPC (1738,869) | 2 |
| 5 | 68 | 323 | 969 | 16-QAM | 100 | LDPC (1738,1304) | 2 |
| 5 | 68 | 323 | 969 | 64-QAM | 100 | LDPC (1738,869) | 3 |
| 5 | 68 | 323 | 969 | 64-QAM | 100 | LDPC (1738,1304) | 3 |
| 3 | 40 | 190 | 570 | 4-QAM | 100 | LDPC (940,470) | 1 |
| 3 | 40 | 190 | 570 | 4-QAM | 100 | LDPC (940,705) | 1 |
| 3 | 40 | 190 | 570 | 16-QAM | 100 | LDPC (940,470) | 2 |
| 3 | 40 | 190 | 570 | 16-QAM | 100 | LDPC (940,705) | 2 |
| 3 | 40 | 190 | 570 | 64-QAM | 100 | LDPC (940,470) | 3 |
| 3 | 40 | 190 | 570 | 64-QAM | 100 | LDPC (940,705) | 3 |
| 1 | 12 | 57 | 171 | 4-QAM | 100 | LDPC (142,71) | 1 |
| 1 | 12 | 57 | 171 | 4-QAM | 100 | LDPC (142,107) | 1 |
| 1 | 12 | 57 | 171 | 16-QAM | 100 | LDPC (142,71) | 2 |
| 1 | 12 | 57 | 171 | 16-QAM | 100 | LDPC (142,107) | 2 |
| 1 | 12 | 57 | 171 | 64-QAM | 100 | LDPC (142,71) | 3 |
| 1 | 12 | 57 | 171 | 64-QAM | 100 | LDPC (142,107) | 3 |

## A4-6	Low-density parity-check codes

The LDPC code is a linear block code that can be uniquely defined by the parity check matrix H. Since the number of “1” in the parity check matrix H is much smaller than the number of “0”, it is called low density check code. The matrix H has double diagonal characteristics.

The check matrix H can be expressed as an exponential matrix shown as follows:

Each number represents an L × L matrix. −1 denotes an all-zero matrix, 0 denotes a unit matrix, and p denotes a permutation matrix obtained by shifting the unit matrix to the right by p. The double diagonal matrix can be divided into two parts: information block and check block, namely:

H = [Hs Hp] and the vector of the encoded output symbols also can be divided into two parts, namely: C = [S P].

According to the check equation [Hs Hp] [S P]T = 0, the corresponding parity bit can be obtained.

### A4-6.1	Code shortening and puncturing

Figure 24 illustrates the shortening of information bits by padding Npad zero bits after the K information bits, and the puncturing of Npunc parity bits from right to left.

FIGURE 24

Shortening by zero-padding and puncturing

For instance, the LDPC (2450,1225) code employs an LDPC encoder with a block size of 2560 and 1280 information bits. Prior to encoding, the 1225 information bits are padded with 55 zero bits to match the required encoder information block size. After encoding, the codeword is shortened by removing the 55 padded information bits, and further punctured by eliminating 55 parity bits from the end of the encoded sequence.

### A4-6.2	LDPC of mode A

#### A4-6.2.1	LDPC (5120,2560)

Rate 1/2 LDPC (5148, 2574) of mode A in 10 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=117), see below:

103-113-1116-155093541581222528608775-1-1224610-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10180-129710264617-1-131-19317-141981922000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-110552-1118100-1-1-17956115371933-1243110036869-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10571237644011640-1-1598-1014-103082-1651-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-130-133-1-1-12581-1-1-1-1-1-1-1-1-1-1-1-1-1894618-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-120-1-1-1-190-130-1-1-1-1-17991-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-152-1-1-1-1-1-10-1-1110564-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1108-1-1-168-1-1-1-1-1-164101-1-1-1-1-1-1-1-1113-1-1-142-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-147-1-162113-1-1-1-1-1-19046-136109-1-1-1-1-1105-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-173-1-1-10-1-1-1-1-1-1112116-1-111-1-1-1-1-10-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-115-152-1-1-198-1-1-1-1-1-1-113-157-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-14-1-148-1-1-1-1-157-1-1-1-1-1-1109-1109-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1105-196-1-1-127816-1-1-1-1-1-1-1-1-1-1-1-16297-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-18796-1-15847-1-1-1-1-1-1-1-1-1-1-1-1-1-1-183-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-182-1106-1-1-1404767-164-1-1-1-1-1-1-1-1-1-1277-137-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-13520-1-1-164-1-1-1-1-1-1-1-1-1-1-1-14814-15-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-17221-1-14682-170-1-1-1-1-1-1-1-1-1-1-193-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-117-1-1-1-1-1-1-174-1-1-1-1-15143-1-1-1-1-1-147-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-112-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-127-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-11726-1-1-1-1-1-110452-13391-11-1-1-197-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-11224-1-1-153-1-1-1-1-1-1-1-1-1-1-1-14758105-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-196113-1-176-1-1-1-1-1-1-1-1-1-1-148-1-166-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

After encoding, the code is shortened by deleting the 14 padded bits, and punctured by deleting the last 14 parity bits.

#### A4-6.2.2	LDPC (5120,3840)

Rate 3/4 LDPC (5250,3850) of mode A in 10 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=175), see below:

-11151058114-17137-118-11730-1632065165155148223510-1-1-1-1-1-123-178103318166122119-139149-1651442-1-16259-1158000-1-1-1-1-10137102-1-1111476987138172-15322243141149-1-114139-1-100-1-1-1-1164174-17128166-1-11051171161185127-1-1648513111634211-1-10-1-1-1-1-1-1-1-18-1-1-1-1-1-1139137-1-1-1-1-116161-11745-1-1-10-1-1-184-1-1-1-1150-1-183-1-1-113554-1-1-1-1-1-1-181-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1167-1-1-1-1-1-1-1-171-1-1-1-1-1-10-1-137-148-1-1-1-1-113152-1-1-1-1-1130119-1-117047-1-1-1-1-1-1-10

Before encoding, 10 zero bits are padded to the 3840 information bits. After encoding, the code is shortened by deleting the 10 padded bits, and punctured by deleting the last 120 parity bits.

#### A4-6.2.3	LDPC (2450,1225)

Rate 1/2 LDPC (2464,1232) of mode A in 5 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=56), see below:

47-148-10-14020113620512427-1-1153210-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10636-1024945533-1-131-111-1313251000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-145-113222-1-1-1103840305131-104229424335-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-121113101822915-1-122-1026-1331322-191-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-131-118-1-1-14653-1-1-1-1-1-1-1-1-1-1-1-1-1414648-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-140-129-1-1-1-1-14554-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-12-1-1-1-1-1-142-1-1495518-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-144-1-1-1-1-1-1541-1-1-1-1-1-1-1-13-1-1-17-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-138-1-12843-1-1-1-1-1-1512-12817-1-1-1-1-127-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-112-1-1-132-1-1-1-1-1-15122-1-19-1-1-1-1-128-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-110-11-1-1-117-1-1-1-1-1-1-155-120-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-127-1-11-1-1-1-1-10-1-1-1-1-1-112-124-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-119-142-1-1-11213-1-1-1-1-1-1-1-1-1-1-1-1348-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-13945-1-1362-1-1-1-1-1-1-1-1-1-1-1-1-1-1-16-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-129-140-1-1-1173417-113-1-1-1-1-1-1-1-1-1-1253-12-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1030-1-1-138-1-1-1-1-1-1-1-1-1-1-1-14439-12-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-15120-1-1850-17-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-155-1-1-1-1-1-1-10-1-1-1-1-14037-1-1-1-1-1-14-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-128-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-110-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-13813-1-1-1-1-1-13539-12913-113-1-1-134-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-15154-1-1-117-1-1-1-1-1-1-1-1-1-1-1-1361355-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-11718-1-139-1-1-1-1-1-1-1-1-1-1-13-1-145-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 7 zero bits are padded to the 1225 information bits. After encoding, the code is shortened by deleting the 7 padded bits, and punctured by deleting the last 7 parity bits.

#### A4-6.2.4	LDPC (2450,1838)

Rate 3/4 LDPC (2520,1848) of mode A in 5 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=84), see below:

-166286423-16381-167-1176-1776281157784210-1-1-1-1-1-179-17182380823114-17472-158812-1-1131-167000-1-1-1-1-1431825-1-1625413101267-1811566104130-1-17062-1-100-1-1-1-13120-1435633-1-1212625625443-1-11564471813491-1-10-1-1-1-1-1-1-1-122-1-1-1-1-1-1618-1-1-1-1-15177-1733-1-1-10-1-1-170-1-1-1-138-1-134-1-1-13047-1-1-1-1-1-1-181-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-120-1-1-1-1-1-1-1-178-1-1-1-1-1-10-1-19-162-1-1-1-1-11333-1-1-1-1-18135-1-16512-1-1-1-1-1-1-10

Before encoding, 10 zero bits are padded to the 1838 information bits. After encoding, the code is shortened by deleting the 10 padded bits, and punctured by deleting the last 60 parity bits.

#### A4-6.2.5	LDPC (1386,693)

Rate 1/2 LDPC (1408,704) of mode A in 3 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=32), see below:

2-12-18-1871819032811928215-1-115010-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-122228-111225212119-1-131-178-1116291000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1101-1412-1-1-10624142723-1010310319-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-162799310257-1-13118-106-11430-191-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-123-12-1-1-13013-1-1-1-1-1-1-1-1-1-1-1-1-125308-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-10-113-1-1-1-1-1522-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-110-1-1-1-1-1-12-1-125010-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-127-1-1-12-1-1-1-1-1-168-1-1-1-1-1-1-1-127-1-1-115-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-11-1-12011-1-1-1-1-1-12112-149-1-1-1-1-127-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-112-1-1-18-1-1-1-1-1-11922-1-19-1-1-1-1-14-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-110-11-1-1-125-1-1-1-1-1-1-115-120-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-119-1-13-1-1-1-1-11-1-1-1-1-1-128-18-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-10-110-1-1-117105-1-1-1-1-1-1-1-1-1-1-1-1138-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-11521-1-1410-1-1-1-1-1-1-1-1-1-1-1-1-1-1-130-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-121-116-1-1-111025-15-1-1-1-1-1-1-1-1-1-11013-110-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-116-1-1-130-1-1-1-1-1-1-1-1-1-1-1-1127-126-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-11128-1-12418-131-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-17-1-1-1-1-1-1-10-1-1-1-1-11629-1-1-1-1-1-128-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-128-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-12-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1225-1-1-1-1-1-1117-1521-121-1-1-110-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1276-1-1-19-1-1-1-1-1-1-1-1-1-1-1-141315-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-112-1-115-1-1-1-1-1-1-1-1-1-1-119-1-121-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 11 zero bits are padded to the 693 information bits. After encoding, the code is shortened by deleting the 11 padded bits, and punctured by deleting the last 11 parity bits.

#### A4-6.2.6	LDPC (1386,1040)

Rate 3/4 LDPC (1440,1056) of mode A in 3 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=48), see below:

-14244023-1150-11-1216-129301402533304210-1-1-1-1-1-131-1231014104326-1260-110926-1-110-119000-1-1-1-1-1194237-1-1146370127-1331542104130-1-14626-1-100-1-1-1-13144-119033-1-19382538631-1-11516474237371-1-10-1-1-1-1-1-1-1-122-1-1-1-1-1-1630-1-1-1-1-1329-113-1-1-10-1-1-134-1-1-1-138-1-146-1-1-13035-1-1-1-1-1-1-121-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-18-1-1-1-1-1-1-1-130-1-1-1-1-1-10-1-133-114-1-1-1-1-119-1-1-1-1-1923-1-12936-1-1-1-1-1-1-10

Before encoding, 16 zero bits are padded to the 1040 information bits. After encoding, the code is shortened by deleting the 16 padded bits, and punctured by deleting the last 38 parity bits.

#### A4-6.2.7	LDPC (304,152)

Rate 1/2 LDPC (320,160) of mode A in 1 kHz bandwidth with 10x20 parity-check matrix (lifting factor L=16), see below:

01-10-1-1-11412110-1-1-1-1-1-1-1-1301-114155-167-100-1-1-1-1-1-1-1-112-131-110-1-190-100-1-1-1-1-1-12-14146614151-11-1-10-1-1-1-1-1-1-1-1-17-1-1-1-1-19-1-18-10-1-1-1-1-1-1-1-114-1-19-1-1-160-1-1-10-1-1-1-1-1-170-16-1-1-14-11-1-1-1-10-1-1-12-15-1-19-1-1-10-16-1-1-1-1-10-1-1-1-1-16-1-1-1-1-16-114-1-1-1-1-1-10-1-1-184-111-1-1-1-1-114-110-1-1-1-1-10

Before encoding, 8 zero bits are padded to the 152 information bits. After encoding, the code is shortened by deleting the 8 padded bits, and punctured by deleting the last 8 parity bits.

#### A4-6.2.8	LDPC (304,228)

Rate 3/4 LDPC (322,230) of mode A in 1 kHz bandwidth with 4x14 parity-check matrix (lifting factor L=23), see below:

14121-1-191-1-12210-1-1-1111318223-141211-100-118-13-10-1-118-1200-10098-17151306-11-1-10

Before encoding, 2 zero bits are padded to the 228 information bits. After encoding, the code is shortened by deleting the 2 padded bits, and punctured by deleting the last 16 parity bits.

### A4-6.3	LDPC of Mode B

#### A4-6.3.1	LDPC (4598,2299)

Rate 1/2 LDPC (4620,2310) of mode B in 10 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=105) see below:

103-155-162-102130212490859710354063-1-1648810-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1164878-117704206780-1-194-14553-18011407000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-11103-1973322-1-1-1733810358793-184492221884-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-101865713125379285-1-11144-19826-1336964-1861-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-166-160-1-1-18881-1-1-1-1-1-1-1-1-1-1-1-1-1832527-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-171-1-1-1-112-178-1-1-1-1-15261-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-158-1-1-1-1-1-10-1-15610488-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-12-1-1-12-1-1-1-1-1-11992-1-1-1-1-1-1-1-159-1-1-10-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-138-1-1358-1-1-1-1-1-15440-1073-1-1-1-1-148-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1103-1-1-160-1-1-1-1-1-13771-1-144-1-1-1-1-163-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-187-188-1-1-1101-1-1-1-1-1-1-176-127-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-155-1-127-1-1-1-1-121-1-1-1-1-1-119-173-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-175-184-1-1-169516-1-1-1-1-1-1-1-1-1-1-1-17413-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-16066-1-14386-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1104-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-11-161-1-1-1108373-155-1-1-1-1-1-1-1-1-1-1967-1100-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-18665-1-1-194-1-1-1-1-1-1-1-1-1-1-1-1953-186-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1306-1-16443-170-1-1-1-1-1-1-1-1-1-1-184-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-162-1-1-1-1-1-1-114-1-1-1-1-15458-1-1-1-1-1-132-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-163-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-187-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-13841-1-1-1-1-1-19825-15755-176-1-1-113-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-13075-1-1-159-1-1-1-1-1-1-1-1-1-1-1-1923427-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-18753-1-167-1-1-1-1-1-1-1-1-1-1-13-1-145-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 11 zero bits are padded to the 2299 information bits. After encoding, the code is shortened by deleting the 11 padded bits, and punctured by deleting the last 11 parity bits.

#### A4-6.3.2	LDPC (4598,3449)

Rate 3/4 LDPC (4710,3454) of mode B in 10 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=157), see below:

-16966178-1152112-1125-110089-10102426707291510-1-1-1-1-1-110-11005015361026145-112110-186242-1-1113148-142000-1-1-1-1-1866742-1-12526441461300-14712215184241-1-14280-1-100-1-1-1-12054-111712021-1-1106100809392128-1-15714973621381531-1-10-1-1-1-1-1-1-1-1124-1-1-1-1-1-1152127-1-1-1-1-16725-112296-1-1-10-1-1-1147-1-1-1-121-1-195-1-1-1141135-1-1-1-1-1-1-1140-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1123-1-1-1-1-1-1-1-1130-1-1-1-1-1-10-1-183-17-1-1-1-1-115182-1-1-1-1-183119-1-13936-1-1-1-1-1-1-10

Before encoding, 5 zero bits are padded to the 3449 information bits. After encoding, the code is shortened by deleting the 5 padded bits, and punctured by deleting the last 107 parity bits.

#### A4-6.3.3	LDPC (2170,1085)

Rate 1/2 LDPC (2200,1100) of mode B in 5 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=50), see below:

4-14-11-102111215101223244023-1-129010-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1088-117147374735-1-10-1158-115163537000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1023-1171812-1-1-18181814913-14342263319-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-134373146032715-1-11114-1226-143034-1211-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-11-140-1-1-1011-1-1-1-1-1-1-1-1-1-1-1-1-13022-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-126-1-1-1-122-123-1-1-1-1-12746-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-138-1-1-1-1-1-114-1-11048-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-14-1-1-1-1-1-13412-1-1-1-1-1-1-1-149-1-1-125-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-128-1-11033-1-1-1-1-1-190-13033-1-1-1-1-113-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-18-1-1-130-1-1-1-1-1-14716-1-139-1-1-1-1-128-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-12-118-1-1-121-1-1-1-1-1-1-131-112-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-115-1-117-1-1-1-1-141-1-1-1-1-1-114-128-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-15-12-1-1-11901-1-1-1-1-1-1-1-1-1-1-1-14948-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-14521-1-11846-1-1-1-1-1-1-1-1-1-1-1-1-1-1-14-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-131-126-1-1-135283-135-1-1-1-1-1-1-1-1-1-14427-120-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-13610-1-1-124-1-1-1-1-1-1-1-1-1-1-1-143-116-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-14516-1-1448-135-1-1-1-1-1-1-1-1-1-1-14-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-127-1-1-1-1-1-1-134-1-1-1-1-1413-1-1-1-1-1-122-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-128-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-12-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1811-1-1-1-1-1-13335-1375-121-1-1-18-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-13530-1-1-129-1-1-1-1-1-1-1-1-1-1-1-12937-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-14748-1-137-1-1-1-1-1-1-1-1-1-1-13-1-135-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 15 zero bits are padded to the 1085 information bits. After encoding, the code is shortened by deleting the 15 padded bits, and punctured by deleting the last 15 parity bits.

#### A4-6.3.4	LDPC (2170,1628)

Rate 3/4 LDPC (2220,1628) of mode B in 5 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=74), see below:

-14034220-131-151-11010-14313845021164610-1-1-1-1-1-117-171242802024-1464-1201522-1-1465-111000-1-1-1-1-131219-1-11565962261-151515846438-1-13058-1-100-1-1-1-1438-1197243-1-133189342855-1-1760315629111-1-10-1-1-1-1-1-1-1-128-1-1-1-1-1-1180-1-1-1-1-15925-16735-1-1-10-1-1-14-1-1-1-118-1-134-1-1-1101-1-1-1-1-1-1-135-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-154-1-1-1-1-1-1-1-146-1-1-1-1-1-10-1-159-168-1-1-1-1-1527-1-1-1-1-15345-1-15524-1-1-1-1-1-1-10

No shortening or puncturing.

#### A4-6.3.5	LDPC (1200,600)

Rate 1/2 LDPC (1232,616) of mode B in 3 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=28), see below:

7-15-10-11307011580112147-1-115410-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1668-14182117253-1-13-145-133526000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1260-11310-1-1-1100124233-100114157-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-121112418182115-1-172-1026-151322-191-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-118-1-1-11825-1-1-1-1-1-1-1-1-1-1-1-1-1131820-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-112-11-1-1-1-1-11726-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-12-1-1-1-1-1-10-1-1212718-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-14-1-1-116-1-1-1-1-1-101-1-1-1-1-1-1-1-13-1-1-17-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-110-1-1715-1-1-1-1-1-1512-1017-1-1-1-1-127-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-112-1-1-14-1-1-1-1-1-12322-1-19-1-1-1-1-10-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-110-11-1-1-117-1-1-1-1-1-1-127-120-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-127-1-11-1-1-1-1-10-1-1-1-1-1-112-124-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-119-114-1-1-11513-1-1-1-1-1-1-1-1-1-1-1-1320-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-11117-1-182-1-1-1-1-1-1-1-1-1-1-1-1-1-1-16-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-11-112-1-1-117617-113-1-1-1-1-1-1-1-1-1-1225-12-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-122-1-1-110-1-1-1-1-1-1-1-1-1-1-1-1011-12-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-12320-1-1822-17-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-127-1-1-1-1-1-1-10-1-1-1-1-1129-1-1-1-1-1-14-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-110-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-11013-1-1-1-1-1-1711-1113-113-1-1-16-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-12326-1-1-117-1-1-1-1-1-1-1-1-1-1-1-181327-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-11718-1-111-1-1-1-1-1-1-1-1-1-1-13-1-117-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 16 zero bits are padded to the 600 information bits. After encoding, the code is shortened by deleting the 16 padded bits, and punctured by deleting the last 16 parity bits.

#### A4-6.3.6	LDPC (1200,900)

Rate 3/4 LDPC (1230,902) of mode B in 3 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=41), see below:

-12352715-1316-10-1212-130110036281310-1-1-1-1-1-129-1392121332430-1258-1313015-1-1120-133000-1-1-1-1-102726-1-18353019224-1102921146-1-12312-1-100-1-1-1-14022-141711-1-11314171035-1-13621632231-1-10-1-1-1-1-1-1-1-115-1-1-1-1-1-11526-1-1-1-1-1714-100-1-1-10-1-1-12-1-1-1-136-1-110-1-1-11924-1-1-1-1-1-1-18-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-14-1-1-1-1-1-1-1-13-1-1-1-1-1-10-1-111-114-1-1-1-1-1171-1-1-1-1-1737-1-14028-1-1-1-1-1-1-10

Before encoding, 2 zero bits are padded to the 900 information bits. After encoding, the code is shortened by deleting the 2 padded bits, and punctured by deleting the last 28 parity bits.

#### A4-6.3.7	LDPC (210,105)

Rate 1/2 LDPC (220,110) of mode B in 1 kHz bandwidth with 10x20 parity-check matrix (lifting factor L=11), see below:

03-12-1-1-181210-1-1-1-1-1-1-1-1373-1101-100-100-1-1-1-1-1-1-1-110-123-10-1-150-100-1-1-1-1-1-14-11174668-11-1-10-1-1-1-1-1-1-1-1-13-1-1-1-1-10-1-19-10-1-1-1-1-1-1-1-16-1-19-1-1-119-1-1-10-1-1-1-1-1-1010-10-1-1-14-11-1-1-1-10-1-1-12-19-1-13-1-1-11-17-1-1-1-1-10-1-1-1-1-15-1-1-1-1-16-19-1-1-1-1-1-10-1-1-153-19-1-1-1-1-18-19-1-1-1-1-10

Before encoding, 5 zero bits are padded to the 105 information bits. After encoding, the code is shortened by deleting the 5 padded bits, and punctured by deleting the last 5 parity bits.

#### A4-6.3.8	LDPC (210,158)

Rate 3/4 LDPC (224,160) of mode B in 1 kHz bandwidth with 4x14 parity-check matrix (lifting factor L=16), see below:

1212-1-114-1-1210-1-1-1103787-1466-100-13-14-110-1-114-190-1001510-153146115-11-1-10

Before encoding, 2 zero bits are padded to the 158 information bits. After encoding, the code is shortened by deleting the 2 padded bits, and punctured by deleting the last 12 parity bits.

### A4-6.4 	LDPC of Mode C

#### A4-6.4.1	LDPC (3714,1857)

Rate 1/2 LDPC (3740,1870) of mode C in 10 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=85), see below:

1-110-10-10416063404572807563-1-1145810-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-106318-18215247824-1-10-18058-1541101000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-121-16241-1-1-15833432478-12919068324-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1043501135522420-1-17119-14861-1786459-111-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-146-150-1-1-1156-1-1-1-1-1-1-1-1-1-1-1-1-1286047-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-156-1-1-1-132-163-1-1-1-1-1061-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-18-1-1-1-1-1-154-1-1218443-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-11-1-1-12-1-1-1-1-1-1147-1-1-1-1-1-1-1-139-1-1-180-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-163-1-11033-1-1-1-1-1-18415-1403-1-1-1-1-18-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-148-1-1-15-1-1-1-1-1-1241-1-149-1-1-1-1-153-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-182-183-1-1-151-1-1-1-1-1-1-111-127-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-125-1-117-1-1-1-1-16-1-1-1-1-1-149-118-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-15-179-1-1-108121-1-1-1-1-1-1-1-1-1-1-1-11448-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1571-1-15321-1-1-1-1-1-1-1-1-1-1-1-1-1-1-159-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-181-156-1-1-1403323-125-1-1-1-1-1-1-1-1-1-14442-175-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1120-1-1-129-1-1-1-1-1-1-1-1-1-1-1-13918-141-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-15561-1-14913-15-1-1-1-1-1-1-1-1-1-1-184-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-142-1-1-1-1-1-1-114-1-1-1-1-18423-1-1-1-1-1-182-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-118-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-147-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-18326-1-1-1-1-1-18345-11715-176-1-1-118-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-12560-1-1-139-1-1-1-1-1-1-1-1-1-1-1-123947-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-13243-1-167-1-1-1-1-1-1-1-1-1-1-143-1-135-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 13 zero bits are padded to the 1857 information bits. After encoding, the code is shortened by deleting the 13 padded bits, and punctured by deleting the last 13 parity bits.

#### A4-6.4.2	LDPC (3714,2786)

Rate 3/4 LDPC (3810,2794) of mode C in 10 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=127), see below:

-187303861-14652-159-11150-132609106156825110-1-1-1-1-1-126-1571227109759229-1520-1262029-1-15752-151000-1-1-1-1-1410141-1-1561041117110873-1911085777731-1-180114-1-100-1-1-1-1770-1154343-1-11410563339636-1-12777991553631-1-10-1-1-1-1-1-1-1-131-1-1-1-1-1-15278-1-1-1-1-11120-12983-1-1-10-1-1-183-1-1-1-189-1-187-1-1-195101-1-1-1-1-1-1-122-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1125-1-1-1-1-1-1-1-155-1-1-1-1-1-10-1-1108-171-1-1-1-1-199116-1-1-1-1-166119-1-18079-1-1-1-1-1-1-10

Before encoding, 8 zero bits are padded to the 2786 information bits. After encoding, the code is shortened by deleting the 8 padded bits, and punctured by deleting the last 88 parity bits.

#### A4-6.4.3	LDPC (1738,869)

Rate 1/2 LDPC (1760,880) of mode C in 5 kHz bandwidth with 22x44 parity-check matrix (lifting factor L=40), see below:

10-11-12-10921135201204101-1-139810-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1163828-1421737372-1-139-11538-1316517000-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1013-13783-1-1-118388381923-124341026319-1-100-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-133321261053715-1-13134-1826-1332914-111-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-131-110-1-1-13821-1-1-1-1-1-1-1-1-1-1-1-1-1333032-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-126-1-1-1-132-113-1-1-1-1-1376-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-118-1-1-1-1-1-134-1-11118-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-135-1-1-112-1-1-1-1-1-11432-1-1-1-1-1-1-1-119-1-1-115-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-138-1-103-1-1-1-1-1-12920-12033-1-1-1-1-13-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-128-1-1-10-1-1-1-1-1-1276-1-19-1-1-1-1-128-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-12-128-1-1-11-1-1-1-1-1-1-131-112-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-135-1-127-1-1-1-1-131-1-1-1-1-1-14-18-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-1-135-134-1-1-192621-1-1-1-1-1-1-1-1-1-1-1-1298-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1-11521-1-12826-1-1-1-1-1-1-1-1-1-1-1-1-1-1-114-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-121-116-1-1-1251833-15-1-1-1-1-1-1-1-1-1-13437-110-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-11630-1-1-114-1-1-1-1-1-1-1-1-1-1-1-1423-126-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-1-1-1036-1-12418-115-1-1-1-1-1-1-1-1-1-1-124-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-17-1-1-1-1-1-1-124-1-1-1-1-12413-1-1-1-1-1-112-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-128-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-12-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-1-1-1-13821-1-1-1-1-1-1315-1375-121-1-1-118-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-13530-1-1-19-1-1-1-1-1-1-1-1-1-1-1-112297-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10-1-1-11718-1-17-1-1-1-1-1-1-1-1-1-1-13-1-15-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-1-10

Before encoding, 11 zero bits are padded to the 869 information bits. After encoding, the code is shortened by deleting the 11 padded bits, and punctured by deleting the last 11 parity bits.

#### A4-6.4.4	LDPC (1738,1304)

Rate 3/4 LDPC (1800,1320) of mode C in 5 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=60), see below:

-10402859-1270-143-100-1533050452533423010-1-1-1-1-1-143-1235838846714-11424-11092-1-13729-143000-1-1-1-1-1314237-1-126424922487-1332754464154-1-13414-1-100-1-1-1-11944-131821-1-1451138307-1-139401164911-1-10-1-1-1-1-1-1-1-158-1-1-1-1-1-15442-1-1-1-1-15141-13715-1-1-10-1-1-134-1-1-1-150-1-158-1-1-13059-1-1-1-1-1-1-121-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-132-1-1-1-1-1-1-1-16-1-1-1-1-1-10-1-157-138-1-1-1-1-11357-1-1-1-1-14559-1-1512-1-1-1-1-1-1-10

Before encoding, 16 zero bits are padded to the 1304 information bits. After encoding, the code is shortened by deleting the 16 padded bits, and punctured by deleting the last 46 parity bits.

#### A4-6.4.5	LDPC (940,470)

Rate 1/2 LDPC (940,470) of mode C in 3 kHz bandwidth with 10x20 parity-check matrix (lifting factor L=47), see below:

026-122-1-1-15382510-1-1-1-1-1-1-1-1451612-1181823-11014-100-1-1-1-1-1-1-1-128-14641-125-1-1280-100-1-1-1-1-1-113-1830244654032-11-1-10-1-1-1-1-1-1-1-1-126-1-1-1-1-131-1-119-10-1-1-1-1-1-1-1-119-1-132-1-1-13523-1-1-10-1-1-1-1-1-1125-14-1-1-143-135-1-1-1-10-1-1-15-129-1-144-1-1-15-125-1-1-1-1-10-1-1-1-1-137-1-1-1-1-125-122-1-1-1-1-1-10-1-1-1232-128-1-1-1-1-139-11-1-1-1-1-10

No shortening or puncturing.

#### A4-6.4.6	LDPC (940,705)

Rate 3/4 LDPC (990,726) of mode C in 3 kHz bandwidth with 8x30 parity-check matrix (lifting factor L=33), see below:

-10601-110-10-102-11701721282132410-1-1-1-1-1-10-11105144211-11712-131929-1-1223-14000-1-1-1-1-16031-1-1201030-112211231119-1-1255-1-100-1-1-1-1432-17323-1-19111621831-1-1601497191-1-10-1-1-1-1-1-1-1-116-1-1-1-1-1-1129-1-1-1-1-1929-1724-1-1-10-1-1-113-1-1-1-129-1-119-1-1-13026-1-1-1-1-1-1-13-1-1-1-1-10-1-1-1-1-1-1-1-1-1-1-1-1-1-120-1-1-1-1-1-1-1-112-1-1-1-1-1-10-1-16-123-1-1-1-1-1190-1-1-1-1-11220-1-12915-1-1-1-1-1-1-10

Before encoding, 21 zero bits are padded to the 705 information bits. After encoding, the code is shortened by deleting the 21 padded bits, and punctured by deleting the last 29 parity bits.

#### A4-6.4.7	LDPC (142,71)

Rate 1/2 LDPC (160,80) of mode C in 1 kHz bandwidth with 10x20 parity-check matrix (lifting factor L=8), see below:

00-11-1-1-167510-1-1-1-1-1-1-1-1355-1645-167-100-1-1-1-1-1-1-1-14-131-12-1-100-100-1-1-1-1-1-16-14566672-11-1-10-1-1-1-1-1-1-1-1-17-1-1-1-1-11-1-10-10-1-1-1-1-1-1-1-14-1-11-1-1-160-1-1-10-1-1-1-1-1-153-13-1-1-14-11-1-1-1-10-1-1-12-15-1-11-1-1-10-16-1-1-1-1-10-1-1-1-1-16-1-1-1-1-16-16-1-1-1-1-1-10-1-1-104-13-1-1-1-1-16-12-1-1-1-1-10

Before encoding, 9 zero bits are padded to the 71 information bits. After encoding, the code is shortened by deleting the 9 padded bits, and punctured by deleting the last 9 parity bits.

#### A4-6.4.8	LDPC (142,107)

Rate 3/4 LDPC (154,110) of mode C in 1 kHz bandwidth with 4x14 parity-check matrix (lifting factor L=11), see below:

### 120-1-1110-1-1110-1-1-106023-1370-100-16-11-110-1-11-130-10047-1975532-11-1-10

Before encoding, three 0 bits are padded to the 107 information bits. After encoding, the code is shortened by deleting the three padded bits, and punctured by deleting the last nine parity bits.

## A4-7	Cyclic redundancy check

For the bit error detection in DS, the 16-bit cyclic redundancy check should be calculated at the end of each DS packet and message head. The generator polynomial should be G16x=x16+x12+x5+1.

For MIS and TIS, the 8-bit cyclic redundancy check should be calculated, and the generator polynomial should be G8x=x8+x4+x3+x2+1.

Annex 5

Message file structure

Figure 25 shows an example of how a data group is built for a message file. In the first step, a header is created to describe the body (a message file). The header contains the file’s management data. Afterwards, the header, as well as the body, are split into equally sized packets for transportation. FF and LF represent the state of the ‘first flag’ and ‘last flag’ bits for each packet.

FIGURE 25

Message file structure

TABLE 26

The structure of the message head

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Broadcast mode | 2 | 00 General broadcast 01 Selective ship 10 Group of ship 11 Selective area |
| Priority (level of message) | 2 | 00 Routine 01 Safety 10 Urgency 11 Distress |
| Subject of the message | 6 | Refer to Table 28 |
| Numbering of the message | 10 | 1 to 999 |

TABLE 26 (end)

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Broadcast count | 4 | Used for multiple broadcasts of the same file (1 to 15) |
| Length of data | 24 | The total length of the data in Bytes, and valid range = 1 ≈16777215 |
| Total packets | 14 | The total packets of the data segment and valid range 1≈ 16383 |
| Type of data | 2 | 00 Text 01 Archive file format GNU tar.gz 10 Archive file zip format 11 Reserved |
| Reserved | 8 | Reserved for future use (=0) |
| Detail of broadcast modes 00, 01 and 10 | 40 | 1 When broadcast mode = 00 All bits are = 0 2 While the Broadcast mode is 01 or 10, the identity is defined on 10 bits according to Rec. ITU-R M.585 and ITU-R·M.1080, each digit consists of 4 bits and the number of bits is 40. |
| Detail of broadcast mode 11 | 176 | The area is defined by four geographical positions on 176 bits with the ability to define a circle (see Table 20 and note) |
| CRC | 16 | The CRC calculation ranges from the Broadcast mode to the end of the detail of broadcast field. |

Total of bits 		128 (Broadcast mode 00 /01/ 10) or 264 (Broadcast mode 11)

Notes:

The body of the broadcast message contains the following information:

The subject of the message

The origin of the message (authority that wrote the message)

The date when the message was written (Year, Month, Day and Hour / minutes)

The message reference number (it is the numbering of the message) The NAVDAT server must be informed of this number when submitting the message. It should be used for the “Broadcast count” function.

The 14 bits of the total packets are determined by the length of the file to be transmitted.

The receiver must know the packet length in advance to validate the received data.

This includes the message header, which specifies the data length (length of data).

Table 27 gives the fixed packet length for the different transmission modes.

The NAVDAT server assigns each message to be broadcast a unique 10-bits number (i.e. a possibility of 999 messages by day) by incrementing from 1 to 999.

This number is totally independent of the message identification given by each authority originating the message and in the message body header.

If the same message is repeated one or more times during the day, this is indicated by the Broadcast count on 4 bits (i.e. a maximum of 15 repetitions).

Packet length

Table 27 summarizes the integer number of bytes in each configuration.

To obtain an integer number of bytes, it may be necessary to aggregate a number of frames.

For example:

MODE B, 4-QAM code rate 0.5, each frame contains 2299 bits, i.e. 287.375 bytes.

To obtain an integer number of bytes, a minimum of 8 frames must be aggregated, giving a total of 2299 bytes per packet (287.375 X 8).

The packet lengths given in Table 27 include the two CRC bytes and the two header bytes.

TABLE 27

Packet length (bytes)

| Bandwidth (kHz) | Modulation | Channel coding rate | Mode A |  | Mode B |  | Mode C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Packet length | Number of aggregated frames | Packet length | Number of aggregated frames | Packet length | Number of aggregated frames |
| 10 | 4-QAM | 0.5 | 320 | 1 | 2299 | 8 | 1857 | 8 |
| 10 | 4-QAM | 0.75 | 480 | 1 | 3449 | 8 | 1393 | 4 |
| 10 | 16-QAM | 0.5 | 640 | 1 | 2299 | 4 | 1857 | 4 |
| 10 | 16-QAM | 0.75 | 960 | 1 | 3449 | 4 | 1393 | 2 |
| 10 | 64-QAM | 0.5 | 960 | 1 | 2299 | 2 + 2/3 | 1857 | 2 + 2/3 |
| 10 | 64-QAM | 0.75 | 1440 | 1 | 3449 | 2 + 2/3 | 1393 | 1 + 1/3 |
| 5 | 4-QAM | 0.5 | 1225 | 8 | 1085 | 8 | 869 | 8 |
| 5 | 4-QAM | 0.75 | 919 | 4 | 407 | 2 | 326 | 2 |
| 5 | 16-QAM | 0.5 | 1225 | 4 | 1085 | 4 | 869 | 4 |
| 5 | 16-QAM | 0.75 | 919 | 2 | 814 | 2 | 326 | 1 |
| 5 | 64-QAM | 0.5 | 3675 | 8 | 3255 | 8 | 2607 | 8 |
| 5 | 64-QAM | 0.75 | 2757 | 4 | 1221 | 2 | 489 | 1 |
| 3 | 4-QAM | 0.5 | 693 | 8 | 150 | 2 | 235 | 4 |
| 3 | 4-QAM | 0.75 | 260 | 2 | 225 | 2 | 705 | 8 |
| 3 | 16-QAM | 0.5 | 693 | 4 | 300 | 2 | 235 | 2 |
| 3 | 16-QAM | 0.75 | 260 | 1 | 225 | 1 | 1410 | 8 |
| 3 | 64-QAM | 0.5 | 2079 | 8 | 450 | 2 | 705 | 4 |
| 3 | 64-QAM | 0.75 | 390 | 1 | 675 | 2 | 2115 | 8 |
| 1 | 4-QAM | 0.5 | 114 | 6 | 105 | 16 | 142 | 8 |
| 1 | 4-QAM | 0.75 | 114 | 4 | 158 | 8 | 107 | 8 |
| 1 | 16-QAM | 0.5 | 114 | 3 | 210 | 8 | 142 | 8 |
| 1 | 16-QAM | 0.75 | 114 | 2 | 158 | 4 | 107 | 4 |
| 1 | 64-QAM | 0.5 | 114 | 2 | 315 | 8 | 213 | 8 |
| 1 | 64-QAM | 0.75 | 171 | 2 | 237 | 4 | 321 | 8 |

Annex 6

Single frequency network mode of Digital Radio Mondiale

## A6-1	Explanation of Digital Radio Mondiale

The international digital radio broadcast standard DRM is used for digital radio broadcasting in the MF and HF bands. DRM is a proven technology that provides superior coverage, improves signal fidelity (through digital error correction coding), eliminates multi-path interference (including sky-wave interference) and thus extends coverage from sky-wave propagated signals. DRM broadcasts are implemented in both 16-QAM and 64-QAM modulation modes, depending on coverage requirements, transmitter location, power and antenna height.

### A6-1.1	Single frequency network operating mode

The NAVDAT system is capable of supporting what is called “single frequency network (SFN) operation”. This is the case where a number of transmitters transmit on the same frequency, and at the same time, identical data signals. Generally, these transmitters are arranged to have overlapped coverage areas, within which a radio should receive signals from more than one transmitter. Provided that these signals arrive within a time difference of less than the guard interval, they should provide positive signal reinforcement. Thus, the service coverage should be improved at that location compared to that obtained if there was only a single transmitter providing service to that location. By careful design, and using a number of transmitters in a SFN, a region or country may be completely covered using a single frequency, and in this application, a single time slot, thus drastically improving spectrum efficiency and release broadcast slots.

In a single frequency network all the individual transmitters must be exactly time synchronized. Every transmitter must broadcast absolutely identical OFDM symbol at the same time.

Time synchronization of all transmitted packets in the transport stream of the final data multiplex is ensured by the time signal 1 pps (pulse per second), which is acquired from the GNSS system.

The frequency stability of transmitters should be better than 2 Hz.

The basic parameter that defines the size of the SFN area is the guard interval Tg.

In OFDM modulation method, its great robustness against inter-symbol interference as an effect of multipath reception (an impact of time delayed signals – echoes) consists in largely extending the very short bit time interval Tb in the serial original data stream.

This guard interval must be carefully configured according to the position of the transmitters in relation with the coverage areas.

When building an SFN network, particular attention should be paid so that the flux of the MIS, TIS and DS are preferably generated by a common server.

Annex 7

Frequencies for NAVDAT HF system

TABLE 28

Frequencies for NAVDAT HF system

| Channel | Maritime frequency band | Central frequency (kHz) | Limits (kHz) |
| --- | --- | --- | --- |
| C1 | 4 MHz band | 4 226 | 4 221 to 4 231 |
| C2 | 6 MHz band | 6 337.5 | 6 332.5 to 6 342.5 |
| C3 | 8 MHz band | 8 443 | 8 438 to 8 448 |
| C4 | 12 MHz band | 12 663.5 | 12 658.5 to 12 668.5 |
| C5 | 16 MHz band | 16 909.5 | 16 904.5 to 16 914.5 |
| C6 | 22 MHz band | 22 450.5 | 22 445.5 to 22 455.5 |

The main international NAVDAT HF frequency is 4 226 kHz.

Annex 8

NAVDAT subject message codes

This list is given only for information.

Refer to documents published by IMO.

TABLE 29

List of NAVDAT subject message codes

| Maritime Safety Information (MSI) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subject message code | Type of message | Coding | Can be rejected |  |
|  |  |  | YES | NO |
| NAVIGATIONAL WARNINGS |  |  |  |  |
| 1 | Sub-area warning | 000001 |  | X |
| 2 | Coastal warning | 000010 |  | X |
| 3 | Local warning (only on national NAVDAT services) | 000011 |  | X |
| 4 | drifting hazards (including derelict ships, ice, mines, containers, other large items over 6 metres in length, etc.) | 000100 |  | X |
| 5 | reserved | 000101 |  |  |

TABLE 29 (continued)

| Maritime Safety Information (MSI) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subject message code | Type of message | Coding | Can be rejected |  |
|  |  |  | YES | NO |
| 6 | reserved | 000110 |  |  |
| 7 | No message on hand | 000111 |  | X |
| NAVIGATIONAL WARNINGS (following) – Positioning system Significant malfunctioning of radio-navigation services and shore-based maritime safety information radio or satellite services |  |  |  |  |
| 8 | GNSS and RNSS | 001000 |  | X |
| 9 | LORAN and E LORAN/Chayka and e Chayka | 001001 |  | X |
| 10 | Differential correction information | 001010 |  | X |
| 11 | Operating anomalies identified within ECDIS including ENC issues | 001011 |  |  |
| 12 | areas where search and rescue (SAR) and anti-pollution operations are being carried out (for avoidance of such areas) | 001100 |  | X |
| 13 | reserved | 001101 |  |  |
| 14 | reserved | 001110 |  |  |
| NAVIGATIONAL WARNINGS (following) – Act of piracy and arm robbery |  |  |  |  |
| 15 | Acts of piracy and armed robbery against ships | 001111 |  | X |
| 16 | Chart of piracy attacks | 010000 |  | X |
| 17 | reserved | 010001 |  |  |
| NAVIGATIONAL WARNINGS (following) – Tsunamis and other natural phenomena warnings |  |  |  |  |
| 18 | Tsunami warning / Abnormal changes to sea level | 010010 |  | X |
| 19 | reserved | 010011 |  |  |
| NAVIGATIONAL WARNINGS (following) – Security In accordance with the requirements of the international Ship and port facility Security Code |  |  |  |  |
| 20 | Security-related information | 010100 |  | X |
| 21 | Chart of security level areas | 010101 |  | X |
| 22 | reserved | 010110 |  |  |
| 23 | reserved | 010111 |  |  |
| NAVIGATIONAL WARNINGS (following) – HEALTH Implementation of the International Health Regulation – IHR |  |  |  |  |
| 24 | World Health Organization (WHO) health advisory information | 011000 |  | X |
| 25 | Pandemic warning | 011001 |  | X |
| 26 | reserved | 011010 |  |  |
| METEOROLOGICAL |  |  |  |  |
| 27 | Meteorological warning (Including tropical cyclone, storm, gale warning) | 011011 |  | X |
| 28 | Meteorological synopses (including weather chart) | 011100 | X |  |

TABLE 29 (continued)

| Maritime Safety Information (MSI) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subject message code | Type of message | Coding | Can be rejected |  |
|  |  |  | YES | NO |
| 29 | Meteorological forecast | 011101 |  | X |
| 30 | Current and tide | 011110 | X |  |
| 31 | Wave hight and direction | 011111 | X |  |
| 32 | reserved | 100000 |  |  |
| 33 | reserved | 100001 |  |  |
| ICE REPORT |  |  |  |  |
| 34 | Ice chart | 100010 | X |  |
| 35 | Iceberg | 100011 | X |  |
| 36 | Polar Road Information | 100100 | X |  |
| 37 | Icebreaker patrol information | 100101 | X |  |
| Search and Rescue related information |  |  |  |  |
| 38 | Distress alert relay to all ships (MAYDAY RELAY) | 100110 |  | X |
| 39 | Ship overdue (description and/or picture of the missing ship) | 100111 |  | X |
| 40 | SAR coordination (to ships involved in the SAR operation) | 101000 |  | X |
| 41 | SAR pattern (to ships involved in the SAR operation) | 101001 |  | X |
| 42 | reserved | 101010 |  |  |
| 43 | reserved | 101011 |  |  |
| Other safety-related information |  |  |  |  |
|  | Pilot service |  |  |  |
| 44 | Pilot service information | 101100 | X |  |
|  | Tug services |  |  |  |
| 45 | Tug service information | 101101 | X |  |
|  | Port support service |  |  |  |
| 46 | Time and height of the tide | 101110 | X |  |
| 47 | Local port information | 101111 | X |  |
| 48 | Hydrographic and environmental information | 110000 | X |  |
|  | Vessel Traffic Service (VTS) |  |  |  |
| 49 | VTS information | 110001 | X |  |
| 50 | MMS (RTCM SC139) | 110010 |  |  |
| 51 | reserved | 110011 |  |  |
|  | Pollution |  |  |  |
| 52 | Pollution information | 110100 |  |  |
| 53 | Pollution chart | 110101 |  |  |

TABLE 29 (end)

| Maritime Safety Information (MSI) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subject message code | Type of message | Coding | Can be rejected |  |
|  |  |  | YES | NO |
| Other information |  |  |  |  |
|  | AIS and LRIT messages |  |  |  |
| 55 | AIS | 110111 | X |  |
| 56 | LRIT | 111000 | X |  |
|  | Nautical chart and publications service |  |  |  |
| 57 | Electronic nautical chart and publications corrections | 111001 | X |  |
| 58 | Electronic Nautical chart and publications update | 111010 | X |  |
|  | Fishing information (only on national NAVDAT services) |  |  |  |
| 59 | Regulations | 111011 | X |  |
| 60 | Special maps | 111100 | X |  |
| 61 | Fishing Quota information | 111101 | X |  |
|  | Encrypted message |  |  |  |
| 62 | Receiving an encrypted message | 111110 |  |  |
| 63 | Update receiver software | 111111 |  | X |

Information is grouped by subject in the NAVDAT broadcast and each subject group is allocated a subject message code from 1 to 63.

The subject message code is used by the receiver to identify the different classes of messages as listed in this Table (from memorized information tables).

The software/firmware of the receiver should be able to be updated. The update should be performed by using a USB port or reception of message 63 (update receiver software).

This function is necessary to follow the evolutions of the GMDSS master plan for new NAVDAT stations as well as for the future revisions of the ITU Recommendations.

The group call identification format of the ship station is defined in part 1 of the Annex 1 of Recommendation ITU-R M.585.