Recommendation ITU-R M.2174-0

(02/2026)

M Series: Mobile, radiodetermination, amateur
and related satellite services

Unwanted emission characteristics of base stations using the terrestrial radio interface of IMT-2020

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the appropriate ITU-R patent information available at https://www.itu.int/en/ITU-R/study-groups/Pages/itu-r-patent-information.aspx.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radio-wave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2026

ITU 2026

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  M.2174-0

Unwanted emission characteristics of base stations using the terrestrial radio interface of IMT-2020

(2026)

Scope

This Recommendation provides the unwanted emission characteristics of base stations using radio interfaces of the terrestrial component of IMT-2020 and operating on frequencies within the bands identified for IMT, based on material submitted to the ITU by external organisations.

These unwanted emission characteristics could be used by Administrations to ensure intra-system compatibility between IMT-2020 terrestrial networks, subject to compliance with the Radio Regulations.

Keywords

Base station, emission characteristics, IMT-2020, out-of-band, spurious, unwanted

Abbreviations

ACLR	Adjacent channel leakage ratio

BS	Base stations

CA	Carrier aggregation

CACLR	Cumulative ACLR

CLTA	Co-location test antenna

DFT	Discrete fourier transform

DL	Downlink

DTT	Digital terrestrial television

EIRP	Equivalent isotropically radiated power

FR	Frequency range

GPS	Global positioning system

IMT	International Mobile Telecommunications

MBW	Measurement BANDWIDTH

NR	New radio

OoB	Out-of-band

OTA	Over-the-air

PHS	Personal handyphone system

RDN	Radio distribution network

RIB	Radiated interface boundary

RIT	Radio interface technologies

RR	Radio Regulations

SCS	Sub-carrier spacing

SRIT	Set of radio interface technologies

TAB	Transceiver array boundary

TRP	Total radiated power

TT	Test tolerances

UL	Uplink

Additional terms may be found in document 3GPP TR 21.905 “Vocabulary for 3GPP Specifications”.

Related documentation: ITU Recommendations, Reports, Document and Handbook

Recommendation ITU-R SM.329 – Unwanted emissions in the spurious domain

Recommendation ITU-R M.1036 – Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations

Recommendation ITU-R SM.1541 – Unwanted emissions in the out-of-band domain

Recommendation ITU-R M.1545 – Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000

Recommendation ITU-R M.2012 – Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications Advanced (IMT-Advanced)

Recommendation ITU-R M.2070 – Unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-Advanced

Recommendation ITU-R M.2071 – Unwanted emission characteristics of mobile stations using the terrestrial radio interfaces of IMT-Advanced

Recommendation ITU-R M.2150 – Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020)

The ITU Radiocommunication Assembly,

considering

a)	that unwanted emissions consist of both spurious and out-of-band (OoB) emissions according to No. 1.146 of the Radio Regulations (RR) and that spurious and OoB emissions are defined in RR Nos 1.145 and 1.144, respectively;

b)	that limitation of the maximum permitted levels of unwanted emissions of IMT-2020 base stations (BS) is necessary to protect other radio systems and services from interference and to enable coexistence between different technologies;

c)	that the harmonization of unwanted emission limits will facilitate global use and access to a global market; however national/regional variations in unwanted emission limits may exist;

d)	that unwanted emission limits are dependent inter alia on the transmitter emission characteristics, ITU spurious emission limits, national standards and regulations in addition to depending on services operating in other bands,

recognizing

a)	provision 3.3 of the Radio Regulations;

b)	that Recommendation ITU-R M.1036 provides the frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications in the bands identified for IMT in the Radio Regulations;

c)	that Recommendation ITU-R SM.329 provides limits for unwanted emissions in the spurious domain as well as measurement methods of spurious emissions;

d)	that Recommendation ITU-R SM.1541 provides limits for unwanted emissions in the OoB domain and encourages the development of more specific limits for each system and each frequency band;

e)	that Recommendation ITU-R M.2150 provides “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-2020 (IMT-2020)” and in particular recommends that the terrestrial radio interfaces for IMT-2020 should be “3GPP 5G-SRIT”, “3GPP 5G-RIT”, “5Gi-RIT” and “DECT 5G-SRIT”,

noting

that in order to reflect the wide applicability of IMT-2020 radio interfaces technologies and to maintain consistency with the technology specifications the notes and annexes of this Recommendation – being based on the ongoing work in standardization bodies, may contain material that reflects information related to use of these technologies in bands other than those identified for IMT,

recommends

1	that the unwanted emission characteristics of base stations that correspond to the 3GPP-5G SRIT radio interface specifications for the terrestrial component of IMT-2020 in Annex 1 should be considered to ensure compatibility between IMT-2020 terrestrial networks using 3GPP-5G SRIT radio interface;

2	that the unwanted emission characteristics of base stations that correspond to the 3GPP-5G RIT radio interface specifications for the terrestrial component of IMT-2020 in Annex 2, for the bands specified in Tables 1-2A and 1-3A should be considered to ensure compatibility between IMT-2020 terrestrial networks using 3GPP-5G RIT radio interface;

3	that the unwanted emission characteristics of base stations that correspond to the 5Gi radio interface specifications for the terrestrial component of IMT-2020 in Annex 3should be considered to ensure compatibility between IMT-2020 terrestrial networks using 5Gi radio interface;

4	that the unwanted emission characteristics of base stations that correspond to the DECT-2020 NR-RIT Component radio interface specifications for the terrestrial component of IMT-2020 in Annex 4 should be considered to ensure compatibility between IMT-2020 terrestrial network using DECT-2020 NR-RIT Component radio interface.

Annexes: 4

TABLE OF CONTENTS

Page

Annex 1 – 3GPP 5G-SRIT		6

Annex 2 – 3GPP 5G-RIT		6

1	Operating bands		6

2	Definitions, symbols and abbreviations		10

2.1	Definitions		10

2.2	Symbols		11

2.3	Abbreviations		12

2.4	Conducted and radiated reference points		12

2.5	Base station classes		14

3	NR Unwanted emission characteristics for BS type 1-C and BS type 1-H		15

3.1	Conducted operating band unwanted emissions		16

3.2	Conducted Adjacent channel leakage ratio (ACLR)		32

3.3	Conducted Cumulative adjacent channel leakage ratio (CACLR)		35

3.4	Conducted Transmitter spurious emissions		38

3.5	Conducted receiver spurious emissions		52

4	NR Unwanted emission characteristics for BS type 1-O and BS type 2-O		53

4.1	OTA Operating band unwanted emissions		54

4.2	OTA Adjacent channel leakage ratio (ACLR)		67

4.3	OTA Cumulative adjacent channel leakage ratio (CACLR)		72

4.4	OTA Transmitter spurious emissions		74

4.5	OTA receiver spurious emissions		92

5	References		94

Annex 3 – 5Gi RIT		94

Annex 4 – DECT-2020 NR-RIT Component		95

1	Unwanted emission characteristics for DECT-2020 NR		95

1.1	Operating bands		95

1.2	Definitions, symbols and abbreviations		96

2	Transmitter spectrum emission characteristics for DECT-2020		96

2.1	General		96

2.2	Occupied channel BW		97

2.3	Maximum output power		98

2.4	Out of band emissions		98

2.5	Spurious emissions		99

2.6	Adjacent channel Leakage Ratio		100

3	References		100

Annex 1

3GPP 5G-SRIT

3GPP 5G-SRIT is a set of radio interface technologies (RITs) consisting of E-UTRA/LTE as one component RIT and NR as the other component RIT. The information on unwanted emission characteristics of E-UTRA/LTE base stations is found in Annex 1 of Recommendation ITU-R M.2070. The information on unwanted emission characteristics of NR base stations is found in the following Annex 2 “3GPP 5G-RIT” of this Recommendation.

Annex 2

3GPP 5G-RIT

The Annex includes information on unwanted emission characteristics from NR carriers for NR base stations.

This Annex is outlined as follows:

–	Chapter 1 specifies the frequency ranges and operating bands for which the information on unwanted emission characteristics is presented in the Annex.

–	Chapter 2 specifies definitions, symbols and abbreviations.

–	Chapter 3 includes the NR BS unwanted emission characteristics defined at individual TAB connectors and/or OTA characteristics defined at RIB (for BS type 1-C and BS type 1-H).

–	Chapter 4 includes the NR BS unwanted emission characteristics for BS type 1-O and BS type 2-O.

–	Chapter 5 specifies references.

Values specified in the Annex incorporate test tolerances defined in Recommendation ITU-R M.1545 [4].

The Annex contains information from transposed standards which are addressed in tables and sections.

Section 3.3 in Recommendation ITU-R SM.329 should be referred to for “Category A” and “Category B” in this Annex.

# 1	Operating bands

Unwanted emission characteristics throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate are described in Table A2-1.

TABLE A2-1

NR frequency ranges

| Frequency range designation |  | Corresponding frequency range* |
| --- | --- | --- |
| FR1 |  | 410-7 125 MHz |
| FR2 | FR2-1 | 24 250-52 600 MHz |
|  | FR2-2 | 52 600-71 000 MHz |
| * Not all frequency bands in these ranges are identified for IMT in RR. |  |  |

The unwanted emission characteristics defined in the present Annex are for NR BS operating in at least one of the bands in Table A2-2 or Table A2-4.

TABLE A2-2

NR operating bands in FR1

Frequency bands utilized by NR and identified for IMT in the RR

| NR operating band number | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode | Footnote(s) Identifying the band, or part thereof, for IMT in various countries/Regions |
| --- | --- | --- | --- | --- |
| n1 | 1 920-1 980 MHz | 2 110-2 170 MHz | FDD | 5.388 |
| n2 | 1 850-1 910 MHz | 1 930-1 990 MHz | FDD | 5.384A, 5.388 |
| n3 | 1 710-1 785 MHz | 1 805-1 880 MHz | FDD | 5.384A |
| n5 | 824-849 MHz | 869-894 MHz | FDD | 5.317A |
| n7 | 2 500-2 570 MHz | 2 620-2 690 MHz | FDD | 5.384A |
| n8 | 880-915 MHz | 925-960 MHz | FDD | 5.317A |
| n12 | 699-716 MHz | 729-746 MHz | FDD | 5.313A, 5.317A |
| n13 | 777-787 MHz | 746-756 MHz | FDD | 5.313A, 5.317A |
| n14 | 788-798 MHz | 758-768 MHz | FDD | 5.313A, 5.317A |
| n18 | 815-830 MHz | 860-875 MHz | FDD | 5.317A |
| n20 | 832-862 MHz | 791-821 MHz | FDD | 5.317A |
| n25 | 1 850-1 915 MHz | 1 930-1 995 MHz | FDD | 5.384A, 5.388 |
| n26 | 814-849 MHz | 859-894 MHz | FDD | 5.317A |
| n28 | 703-748 MHz | 758-803 MHz | FDD | 5.313A, 5.317A |
| n29 | N/A | 717-728 MHz | SDL | 5.313A, 5.317A |
| n30 | 2 305-2 315 MHz | 2 350-2 360 MHz | FDD | 5.384A |
| n34 | 2 010-2 025 MHz | 2 010-2 025 MHz | TDD | 5.388 |
| n38 | 2 570-2 620 MHz | 2 570-2 620 MHz | TDD | 5.384A |
| n39 | 1 880-1 920 MHz | 1 880-1 920 MHz | TDD | 5.384A, 5.388 |
| n40 | 2 300-2 400 MHz | 2 300-2 400 MHz | TDD | 5.384A |

TABLE A2-2 (end)

| NR operating band number | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high |  | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high |  | Duplex mode |  |  | Footnote(s) Identifying the band, or part thereof, for IMT in various countries/Regions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n48 |  | 3 550-3 700 MHz |  | 3 550-3 700 MHz |  | TDD | 5.430A, 5.431B, 5.433A, 5.433B, 5.434, 5.434B |  |
| n50 |  | 1 432-1 517 MHz |  | 1 432-1 517 MHz |  | TDD | 5.341A, 5.341B, 5.341C, 5.346, 5.346A |  |
| n51 |  | 1 427-1 432 MHz |  | 1 427-1 432 MHz |  | TDD | 5.341A, 5,341B, 5,341C |  |
| n65 |  | 1 920-2 010 MHz |  | 2 110-2 200 MHz |  | FDD | 5.388 |  |
| n66 |  | 1 710-1 780 MHz |  | 2 110-2 200 MHz |  | FDD | 5.384A, 5.388 |  |
| n67 |  | N/A |  | 738-758 MHz |  | SDL | 5.313A, 5.317A |  |
| n71 |  | 663-698 MHz |  | 617-652 MHz |  | FDD | 5.296A, 5.308A, 5.307A |  |
| n74 |  | 1 427-1 470 MHz |  | 1 475-1 518 MHz |  | FDD | 5.341A, 5,341B, 5.341C, 5.346, 5.346A |  |
| n75 |  | N/A |  | 1 432-1 517 MHz |  | SDL | 5.341A, 5.341B, 5.341C, 5.346, 5.346A |  |
| n76 |  | N/A |  | 1 427-1 432 MHz |  | SDL | 5.341A, 5.341B, 5.341C |  |
| n80 |  | 1 710-1 785 MHz |  | N/A |  | SUL | 5.384A |  |
| n81 |  | 880-915 MHz |  | N/A |  | SUL | 5.317A |  |
| n82 |  | 832-862 MHz |  | N/A |  | SUL | 5.317A |  |
| n83 |  | 703-748 MHz |  | N/A |  | SUL | 5.313A, 5.317A |  |
| n84 |  | 1 920-1 980 MHz |  | N/A |  | SUL | 5.388 |  |
| n85 |  | 698-716 MHz |  | 728-746 MHz |  | FDD | 5.313A, 5.317A |  |
| n86 |  | 1 710-1 780 MHz |  | N/A |  | SUL | 5.384A |  |
| n89 |  | 824-849 MHz |  | N/A |  | SUL | 5.317A |  |
| n91 |  | 832-862 MHz |  | 1 427-1 432 MHz |  | FDD(2) | 5.317A, 5.341A, 5.431B, 5.341C |  |
| n92 |  | 832-862 MHz |  | 1 432-1 517 MHz |  | FDD(2) | 5.317A, 5.341A, 5.341B, 5.341C, 5.346, 5.346A |  |
| n93 |  | 880-915 MHz |  | 1 427-1 432 MHz |  | FDD(2) | 5.317A, 5.341A, 5.431B, 5.341C |  |
| n94 |  | 880-915 MHz |  | 1 432-1 517 MHz |  | FDD(2) | 5.317A, 5.341A, 5.341B, 5.341C, 5.346, 5.346A |  |
| n95(1) |  | 2 010-2 025 MHz |  | N/A |  | SUL | 5.388 |  |
| n97(5) |  | 2 300-2 400 MHz |  | N/A |  | SUL | 5.384A |  |
| n98(5) |  | 1 880-1 920 MHz |  | N/A |  | SUL | 5.384A, 5.388 |  |
| n101 |  | 1 900-1 910 MHz |  | 1 900-1 910 MHz |  | TDD | 5.388 |  |
| n104(8) |  | 6 425-7 125 MHz |  | 6 425-7 125 MHz |  | TDD | 5.457D, 5.457E, 5.457F |  |

TABLE A2-3

NR operating bands in FR1

Frequency bands utilized by NR and not or not fully identified for IMT in the RR

| NR operating band number | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode | Footnote(s) Identifying the band, or part thereof, for IMT in various countries/Regions |
| --- | --- | --- | --- | --- |
| n24(7) | 1 626.5-1 660.5 MHz | 1 525-1 559 MHz | FDD | Not Identified for IMT in RR |
| n41 | 2 496-2 690 MHz | 2 496-2 690 MHz | TDD | 5.384A |
| n53 | 2 483.5-2 495 MHz | 2 483.5-2 495 MHz | TDD | Not Identified for IMT in RR |
| n70 | 1 695-1 710 MHz | 1 995-2 020 MHz | FDD | 5.388 |
| n77 | 3 300-4 200 MHz | 3 300-4 200 MHz | TDD | 5.429B, 5.429D, 5.429F, 5.430A, 5.431B, 5.432A, 5.432B, 5.433A, 5.433B, 5.434, 5.434B, 5.435B |
| n78 | 3 300-3 800 MHz | 3 300-3 800 MHz | TDD | 5.429B, 5.529D, 5.429F, 5.430A, 5.431B, 5.432A, 5.432B, 5.433A, 5.433B, 5.434, 5.434B, 5.435B |
| n79 | 4 400-5 000 MHz | 4 400-5 000 MHz | TDD | 5.441A, 5.441B |
| n90 | 2 496-2 690 MHz | 2 496-2 690 MHz | TDD | 5.384A |
| n99(6) | 1 626.5-1 660.5 MHz | N/A | SUL | Not Identified for IMT in RR |
| n102(4) | 5 925-6 425 MHz | 5925-6 425 MHz | TDD(3) | Not Identified for IMT in RR |
| (1) Void (2) Variable duplex operation does not enable dynamic variable duplex configuration by the network and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. (3) Void (4) Void (5) The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. (6) Void (7) Void |  |  |  |  |

TABLE A2-4

NR operating bands in FR2

Frequency bands utilized by NR and identified for IMT in the RR

| NR operating band number | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | Footnote(s) Identifying the band, or part thereof, for IMT in various countries/Regions |
| --- | --- | --- | --- |
| n258 | 24 250-27 500 MHz | TDD | 5.532AB |
| n259 | 39 500-43 500 MHz | TDD | 5.550B |
| n260 | 37 000-40 000 MHz | TDD | 5.550B |
| n262 | 47 200-48 200 MHz | TDD | 5.553B |

TABLE A2-5

NR operating bands in FR2

Frequency bands utilized by NR and not or not fully identified for IMT in the RR

| NR operating band number | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode | Footnote(s) Identifying the band, or part thereof, for IMT in various countries/Regions |
| --- | --- | --- | --- |
| n257 | 26 500-29 500 MHz | TDD | 5.532AB |
| n261 | 27 500-28 350 MHz | TDD | Not Identified for IMT in RR |
| n263 | 57 000-71 000 MHz | TDD | 5.559AA |

# 2	Definitions, symbols and abbreviations

## 2.1	Definitions

basic limit: emissions limit relating to the power supplied by a single transmitter to a single antenna transmission line in Recommendation ITU-R SM.329 [3] used for the formulation of unwanted emission requirements for FR1.

antenna connector: connector at the conducted interface of the BS type 1-C.

BS type 1-C: NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors.

BS type 1-H: NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB.

BS type 1-O: NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB.

BS type 2-O: NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB.

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands.

NB-IoT operation in NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

Operating band: frequency range in which NR base station operates (paired or unpaired), that is defined with a specific set of technical requirements.

NOTE – The operating band(s) for a BS is declared by the manufacturer according to the designations in Tables 1-2 and 1-3.

radiated interface boundary: operating band specific radiated requirements reference where the radiated requirements apply.

NOTE – For requirements based on EIRP, the radiated interface boundary is associated to the far-field region.

TAB connector: transceiver array boundary connector.

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted RX requirements are applied.

NOTE – Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with reception on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which BS type 1-H conducted TX requirements are applied.

NOTE – Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna.

## 2.2	Symbols

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12 kHz

FDL_low		Lowest frequency of the downlink operating band

FDL_high		Highest frequency of the downlink operating band

FUL_low		Lowest frequency of the uplink operating band

FUL_high		Highest frequency of the uplink operating band

ΔfOBUE		Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

Δfmax		f_offsetmax minus half of the bandwidth of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

NRB		Transmission bandwidth configuration, expressed in resource blocks

NTXU,counted	The number of active transmitter units as calculated in clause 3.1, that are taken into account for conducted TX output power limit in clause 3.1.2, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 3.1.

## 2.3	Abbreviations

ACLR	Adjacent channel leakage ratio

CA	Carrier aggregation

CACLR	Cumulative ACLR

CLTA	Co-location test antenna

DTT	Digital terrestrial television

EIRP	Effective isotropic radiated power

FR	Frequency range

GPS	Global positioning system

OTA	Over-the-air

PHS	Personal handyphone system

RIB	Radiated interface boundary

SCS	Sub-carrier spacing

TAB	Transceiver array boundary

TRP	Total radiated power

## 2.4	Conducted and radiated reference points

The manufacturer can declare the NR BS to be one of the following BS types:

–	BS type 1-C: Requirements are applied at the BS antenna connector (port A) for a single transmitter or receiver with a full complement of transceivers for the configuration in normal operating conditions. If any external apparatus such as an amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B).

FIGURE a2-1

BS type 1-C transmitter interface

Figure a2-2

BS type 1-C receiver interface

–	BS type 1-H: Requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

Figure a2-3

Radiated and conducted reference points for BS type 1-H

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to transmit/receive parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is requirement specific.

–	BS type 1-O: Radiated characteristics for BS type 1-O are defined over the air (OTA) where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). The (spatial) characteristics in which the OTA requirements apply are requirement specific.

Figure a2-4

General architecture of BS type 1-O and BS type 2-O

–	BS type 2-O: OTA requirements defined at the RIB only, same as for the BS type 1-O.

## 2.5	Base station classes

The BS classes for NR are Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations.

BS classes for BS type 1-C and BS type 1-H are defined as indicated below:

–	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum coupling loss equal to 70 dB.

–	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum coupling loss equals to 53 dB.

–	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum coupling loss equal to 45 dB.

BS classes for BS type 1-O and BS type 2-O are defined as indicated below:

–	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

–	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

–	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

# 3	NR Unwanted emission characteristics for BS type 1-C and BS type 1-H

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The operating band unwanted emissions define unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. Unwanted emissions outside of this frequency range are defined by a spurious emissions characteristics.

NOTE – The frequency offset ΔfOBUE does not define the boundary between the out-of-band and spurious domain as set out in Recommendation ITU-R SM.1541. As a result, in the range from the operating band edge up to ΔfOBUE characteristics of unwanted emissions presented in this Annex may cover only part of the out-of-band domain frequency range. In such case, for the remaining part of out-of-band domain frequency ranges, spurious emissions characteristics should be used.

The values of ΔfOBUE are defined in Table A2-6 for the NR operating bands.

TABLE A2-6

Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-C | FDL_high – FDL_low 200 MHz | 10 |
|  | 200 MHz < FDL_high – FDL_low 900 MHz | 40 |
| BS type 1-H | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz FDL_high – FDL_low 900 MHz | 40 |

For band n96 and n102, the values of ΔfOBUE are defined in Table A2-7.

TABLE A2-7

Maximum offset of OBUE outside the downlink operating band

| Operating band | ΔfOBUE (MHz) |
| --- | --- |
| n102 | 40 |
| n96 | 50 |

## 3.1	Conducted operating band unwanted emissions

### 3.1.1	Conducted Operating band unwanted emissions for BS type 1-C

The operating band unwanted emissions for BS type 1-C for each antenna connector should be below the applicable basic limits defined in §§ 3.1.3 to 3.1.8.

### 3.1.2	Conducted Operating band unwanted emissions BS type 1-H

The operating band unwanted emissions for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in §§ 3.1.3 to 3.1.8, the power summation emissions at the TAB connectors of the TAB connector TX min cell group should not exceed a BS limit specified as the basic limit + X, where X = 10 log10(NTXU,countedpercell);

NTXU,countedpercell - scaling factor for the basic limits. NTXU,countedpercell = min(NTXU,active, 8·Ncells)/Ncells;

Ncells – the minimum number of supported geographical cells relates to the BS setting with the minimum amount of cell splitting supported with transmission on all TAB connectors supporting the operating band. The manufacturer should also declare TAB connector TX min cell groups. Every TAB connector supporting transmission in an operating band should map to one TAB connector TX min cell group supporting the same operating band. The mapping of TAB connectors to cells is implementation dependent;

NTXU,active – number of active transmitter units and is independent to the declaration of Ncells.

NOTE – Conformance to the BS type 1-H emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group should be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector should be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by −10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.1.3	Basic limits for Wide Area BS (Category A)

For BS operating in Bands n5, n8, n12, n13, n14, n18, n26, n28, n29, n71, n85, basic limits are specified in Table 6.6.4.5.2-1 of TS 38.141-1 [1].

TABLE 6.6.4.5.2-1 Wide Area BS operating band unwanted emission limits (NR bands below 1 GHz) for Category A Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −12.5 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −13 dBm (Note 3). Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -13 dBm/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. Note 3: The requirement is not applicable when fmax < 10 MHz..

For BS operating in Bands n1, n2, n3, n7, n24, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n92, n94, basic limits are specified in Table 6.6.4.5.2-2 of TS 38.141-1 [1]:

TABLE 6.6.4.5.2-2 Wide Area BS operating band unwanted emission limits (1 GHz < NR bands ≤ 3 GHz) for Category A Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −12.5 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −13 dBm (Note 3); 1 MHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −13 dBm/1 MHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Note 3: The requirement is not applicable when fmax < 10 MHz..

For BS operating in Bands n48, n77, n78, n79, basic limits are specified in Table 6.6.4.5.2-3 of TS 38.141-1 [1].

TABLE 6.6.4.5.2-3 Wide Area BS operating band unwanted emission limits (NR bands > 3 GHz) for Category A Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −12.2 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −13 dBm (Note 3); 1 MHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −13 dBm/1 MHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Note 3: The requirement is not applicable when fmax < 10 MHz..

### 3.1.4	Basic limits for Wide Area BS (Category B)

For Category B Operating band unwanted emissions, there are two options for the basic limits that may be applied regionally. Either the basic limits in § 6.6.4.2.2.1 or § 6.6.4.2.2.2 should be applied.

#### 3.1.4.1	Category B requirements (Option 1)

For BS operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, basic limits are specified in Table 6.6.4.5.3.1-1 of TS 38.141-1 [1].

TABLE 6.6.4.5.3.1-1 Wide Area BS operating band unwanted emission limits (NR bands below 1 GHz) for Category B Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −12.5 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −16 dBm (Note 3). Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -16 dBm/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Note 3: The requirement is not applicable when fmax < 10 MHz..

For BS operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n92, n94, basic limits are specified in Tables 6.6.4.5.3.1-2 of TS 38.141-1 [1].

TABLE 6.6.4.5.3.1-2 Wide Area BS operating band unwanted emission limits (1 GHz < NR bands ≤ 3 GHz) for Category B Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −12.5 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −15 dBm (Note 3); 1 MHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −15 dBm/1 MHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Note 3: The requirement is not applicable when fmax < 10 MHz..

For BS operating in Bands n48, n77, n78, n79, basic limits are specified in Table 6.6.4.5.3.1-3 of TS 38.141-1 [1].

TABLE 6.6.4.5.3.1-3 Wide Area BS operating band unwanted emission limits (NR bands >3 GHz) for Category B Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −12.2 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −15 dBm (Note 3); 1 MHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −15 dBm/1 MHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Note 3: The requirement is not applicable when fmax < 10 MHz..

#### 3.1.4.2	Category B requirements (Option 2)

For a BS operating in Bands n1, n3, n8, n65 or BS type 1-C operating in bands n7, n38 or n101 basic limits are specified in Table 6.6.4.5.3.2-1 of TS 38.141-1 [1].

TABLE 6.6.4.5.3.2-1 Regional Wide Area BS operating band unwanted emission limits for Category B Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 0.2 MHz; 0.015 MHz f_offset < 0.215 MHz; −12.5 dBm; 30 kHz. 0.2 MHz f < 1 MHz; 0.215 MHz f_offset < 1.015 MHz; 30 kHz. (Note 4); 1.015 MHz f_offset < 1.5 MHz; −24.5 dBm; 30 kHz. 1 MHz f min( 10 MHz, fmax); 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax); −11.5 dBm; 1 MHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −15 dBm (Note 3); 1 MHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be −15dBm/1MHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Note 3: The requirement is not applicable when fmax < 10 MHz. Note 4: This frequency range ensures that the range of values of f_offset is continuous..

### 3.1.5	Basic limits for Medium Range BS (Category A and Category B)

For Medium Range BS in NR bands ≤ 3 GHz, basic limits are specified in Table 6.6.4.5.4-1 and Table 6.6.4.5.4-2 of TS 38.141-1 [1].

TABLE 6.6.4.5.4-1 Medium Range BS operating band unwanted emission limits, 31< Prated,x 38 dBm (NR bands ≤ 3 GHz) Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; Prated,x-51.5dB-75f_offsetMHz-0.05dB; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); Prated,x - 58.5 dB; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; Min(Prated,x – 60 dB, −25 dBm) (Note 3); 100 kHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,x −60 dB, −25 dBm)/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. Note 3: The requirement is not applicable when fmax < 10 MHz..

TABLE 6.6.4.5.4-2 Medium Range BS operating band unwanted emission limits, Prated,x 31 dBm (NR bands ≤ 3 GHz) Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −27.5 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −29 dBm (Note 3); 100 kHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −29 dBm/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. Note 3: The requirement is not applicable when fmax < 10 MHz..

For Medium Range BS in NR bands > 3 GHz, basic limits are specified in Table 6.6.4.5.4-3 and Table 6.6.4.5.4-4 of TS 38.141-1 [1].

TABLE 6.6.4.5.4-3 Medium Range BS operating band unwanted emission limits, 31< Prated,x 38 dBm (NR bands >3 GHz) Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; Prated,x-51.2dB-75f_offsetMHz-0.05dB; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); Prated,x - 58.2 dB; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; Min(Prated,x – 60 dB, −25 dBm) (Note 3); 100 kHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,x −60 dB, −25 dBm)/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. Note 3: The requirement is not applicable when fmax < 10 MHz..

TABLE 6.6.4.5.4-4 Medium Range BS operating band unwanted emission limits, Prated,x 31 dBm (NR bands >3 GHz) Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −27.2 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −29 dBm (Note 3); 100 kHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −29 dBm/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. Note 3: The requirement is not applicable when fmax < 10 MHz...

### 3.1.6	Basic limits for Local Area BS (Category A and Category B)

For Local Area BS in NR bands ≤ 3 GHz, basic limits are specified in Table 6.6.4.5.5-1 of TS 38.141-1 [1].

TABLE 6.6.4.5.5-1 Local Area BS operating band unwanted emission limits (NR bands ≤3 GHz) Frequency offset of measurement filter −3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −35.5 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −37 dBm (Note 3). Note 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −37 dBm/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap Note 3: The requirement is not applicable when fmax < 10 MHz..

For Local Area BS in NR bands > 3 GHz, basic limits are specified in Table 6.6.4.5.5-1 of TS 38.141-1 [1].

TABLE 6.6.4.5.5-1 Local Area BS operating band unwanted emission limits (NR bands >3 GHz) Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −35.2 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −37 dBm (Note 3). Note 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −37 dBm/100 kHz. Note 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. Note 3: The requirement is not applicable when fmax < 10 MHz..

### 3.1.7	Basic limits for Local Area and Medium Range BS for band n96 and n102 (Category A and Category B)

For Local Area and Medium Range BS operating in Band n96 and Band n102, basic limits for 20 MHz, 40 MHz, 60 MHz and 80 MHz channel bandwidth are specified in Table 6.6.4.5.5A-2 of TS 38.141-1 [1]. For one non-transmitted channel basic limits are specified in Table 6.6.4.5.5A-3 of TS 38.141-1 [1], and for two non-transmitted channels basic limits are specified in Table 6.6.4.5.5A-4 of TS 38.141-1 [1].

In the case of one or two non-transmitted 20 MHz channels between transmitted channels, when a NR-U channel bandwidth of 60 MHz or 80 MHz have been assigned, the spectrum emission mask for non-transmitted channels specified in Tables 6.6.4.5.5A-3 and 6.6.4.5.5A-4 of TS 38.141-1 [1] applies for one and two non-transmitted channels respectively. The relative power of any BS emission should not exceed the most stringent levels given by Tables 6.6.4.5.5A-2 and 6.6.4.5.5A-3 of TS 38.141-1 [1]in the case of non-transmitted channels between transmitted channels.

TABLE 6.6.4.5.5A-2 Medium Range BS and Local Area BS operating band unwanted emission limits for 20 MHz, 40 MHz, 60 MHz and 80 MHz channel bandwidth for band n96 and n102 Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limits (Note 1); Measurement bandwidth. 0 MHz f < 1 MHz; 0.05 MHz f_offset < 1.05 MHz; Max(Prated,x-10log10BWchannel100kHz-20foffsetMHz-0.05+2.2dB, -40dBm); 100 kHz. 1 MHz f < min(0.5N MHz, fmax); 1.05 MHz f_offset < min((0.5N+0.05) MHz, f_offsetmax); Max(Prated,x-10log10BWChannel100kHz-17.8-80.5N-1f_offsetMHz-1.05dB, -40dBm); 100 kHz. 0.5N MHz f < min(N MHz, fmax); (0.5N+0.05) MHz f_offset < min((N+0.05) MHz, f_offsetmax); Max(Prated,x-10log10BWChannel100kHz-25.8-120.5Nf_offsetMHz-0.5N-0.05dB, -40dBm); 100 kHz. 0 MHz f < 1 MHz; 0.05 MHz f_offset < 1.05 MHz; Max(Prated,x-10log10BWchannel100kHz-20foffsetMHz-0.05+2.2dB, -40dBm); 100 kHz. 8.5N MHz f < min(10.3N MHz, fmax); (8.5N+0.05) MHz f_offset < min((10.3N+0.05) MHz, f_offsetmax); MaxPrated,x-10log10BWChannel100kHz-39.8dB,-40dBm; 100 kHz. 10.3N MHz f fmax; (10.3N+0.05) MHz f_offset < f_offsetmax; MaxPrated,x-10log10BWChannel100kHz-44.8dB,-40dBm; 100 kHz. Note 1: For a BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ N MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be MaxPrated,x-10log10BWChannel100kHz-37.8dB,-40dBm..

Table 6.6.4.5.5A-3 Medium Range BS operating band unwanted emission limits for one non-transmitted channel for 60 MHz and 80MHz channel bandwidth for band n96 and n102 Frequency offset of measurement filter -3dB point, ΔfBE_offset; Frequency offset of measurement filter centre frequency, f_BE_offset; Basic limits; Measurement bandwidth. 0 MHz fBE_offset < 1 MHz; 0.05 MHz f_BE_offset < 1.05 MHz; Prated,x-10log10BWChannel100kHz-20fBEoffsetMHz-0.05+2.2dB; 100 kHz. 1 MHz fBE_offset < 10 MHz; 1.05 MHz f_BE_offset < 10.05 MHz; Prated,x-10log10BWChannel100kHz-17.8-13f_BE_offsetMHz-1.05dB; 100 kHz. 10 MHz fBE_offset < 19 MHz; 10.05 MHz f_BE offset < 19.05 MHz; Prated,x-10log10BWChannel100kHz-20.8+13f_BE_offsetMHz-10.05dB; 100 kHz. 19 MHz fBE_offset < 19.9 MHz; 19.05 MHz f_BE_offset < 19.95 MHz; Prated,x-10log10BWChannel100kHz-17.8+20f_BE_offsetMHz-19.05dB; 100 kHz.

TABLE 6.6.4.5.5A-4 Medium Range BS and Local Area BS operating band unwanted emission limits for two non-transmitted channels of 80 MHz channel bandwidth for band n96 and n102 Frequency offset of measurement filter -3dB point, fBE_offset; Frequency offset of measurement filter centre frequency, f_BE_offset; Basic limits; Measurement bandwidth. 0 MHz fBE_offset < 1 MHz; 0.05 MHz f_BE_offset < 1.05 MHz; Prated,x-10log10BWChannel100kHz-20fBEoffsetMHz-0.05+2.2dB; 100 kHz. 1 MHz fBE_offset <10 MHz; 1.05 MHz f_BE_offset < 10.05 MHz; Prated,x-10log10BWChannel100kHz-17.8-59f_BE_offsetMHz-1.05dB; 100 kHz. 10 MHz fBE_offset <30 MHz; 10.05 MHz f_BE offset < 30.05 MHz; Prated,x-10log10BWChannel100kHz-22.8; 100 kHz. 30 MHz fBE_offset < 39 MHz; 30.05 MHz f_BE_offset < 39.05 MHz; Prated,x-10log10BWChannel100kHz-22.8+59f_BE_offsetMHz-30.05dB; 100 kHz. 39 MHz fBE_offset < 39.9 MHz; 39.05 MHz f_BE_offset < 39.95 MHz; Prated,x-10log10BWChannel100kHz-17.8+20f_BE_offsetMHz-39.05dB; 100 kHz.

### 3.1.8	Basic limits for additional requirements

In certain regions the following additional requirements may apply:

–	For protection of DTT, the requirement in clause 6.6.4.5.6.2 of TS 38.141-1 [1].

–	For operation in Band n48, the requirement in clause 6.6.4.5.6.3 of TS 38.141-1 [1].

–	For operation in Band n53, the requirement in clause 6.6.4.5.6.4 of TS 38.141-1 [1].

–	For protection of GPS, the requirement in clause 6.6.4.5.6.5 of TS 38.141-1 [1].

–	For operation with shared spectrum channel access, the requirement in clause 6.6.4.5.6.6 of TS 38.141-1 [1].

###### 6.6.4.5.6.2 Protection of DTT In certain regions the following requirement may apply for protection of DTT. For BS type 1-C or BS type 1-H operating in Band n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to Table 6.6.4.5.6.2-1, basic limit is PEM,N declared by the manufacturer. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain. Table 6.6.4.5.6.2-1: Declared emissions basic limit for protection of DTT Filter centre frequency, Ffilter; Measurement bandwidth; Declared emission basic limit (dBm). Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60; 8 MHz; PEM,N. Note: The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the BS needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in TS 36.104 [22], Annex G.. ###### 6.6.4.5.6.3 Additional operating band unwanted emissions limits for Band n48 The following requirement may apply to BS operating in Band n48 in certain regions. Emissions shall not exceed the maximum levels specified in Table 6.6.4.5.6.3-1. Table 6.6.4.5.6.3-1: Additional operating band unwanted emission limits for Band n48 Channel bandwidth; Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Minimum requirement; Measurement bandwidth (Note). All; 0 MHz f < 10 MHz; 0.5 MHz f_offset < 9.5 MHz; ‒13 dBm; 1 MHz. NOTE: The resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.. ###### 6.6.4.5.6.4 Additional operating band unwanted emissions limits for Band n53 The following requirement may apply to BS operating in Band n53 in certain regions. Emissions shall not exceed the maximum levels specified in Table 6.6.4.5.6.4-1. Table 6.6.4.5.6.4-1: Additional operating band unwanted emission limits for Band n53 Channel bandwidth (MHz); Frequency range (MHz); Frequency offset of measurement filter -3dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Minimum requirement; Measurement bandwidth (Note). 5; 2 400-2 477.5; 6 MHz f < 83.5 MHz; 6.5 MHz f_offset < 83 MHz; ‒25 dBm; 1 MHz. 10; 2 400-2 473.5; 10 MHz f < 83.5 MHz; 10.5 MHz f_offset < 83 MHz; ‒25 dBm; 1 MHz. 5; 2 477.5-2 478.5; 5 MHz f < 6 MHz; 5.5 MHz; ‒13 dBm; 1 MHz. 10; 2 473.5-2 478.5; 5 MHz f < 10 MHz; 5.5 MHz f_offset < 9.5 MHz; ‒13 dBm; 1 MHz. All; 2 478.5-2 483.5; 0 MHz f < 5 MHz; 0.5 MHz f_offset < 4.5 MHz; ‒10 dBm; 1 MHz. 5; 2 495-2 501; 0 MHz f < 6 MHz; 0.5 MHz f_offset < 5.5 MHz; ‒13 dBm; 1 MHz. 10; 2 495-2 505; 0 MHz f < 10 MHz; 0.5 MHz f_offset < 9.5 MHz; ‒13 dBm; 1 MHz. 5; 2 501-2 690; 6 MHz f < 195 MHz; 6.5 MHz f_offset < 194.5 MHz; ‒25 dBm; 1 MHz. 10; 2 505-2 690; 10 MHz f < 195 MHz; 10.5 MHz f_offset < 194.5 MHz; ‒25 dBm; 1 MHz. NOTE: The resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.. ###### 6.6.4.5.6.5 Protection of GPS The level of emissions in the 1 541-1 650 MHz band, measured in measurement bandwidth according to Table 6.6.4.5.6.5-1 shall not exceed the basic limits PEM_B24,a, PEM_B24,b, PEM_B24,c, PEM_B24,d, PEM_B24,e, and PEM_B24,f declared by the manufacturer. Table 6.6.4.5.6.5-1: Declared emissions basic limits for protection of GPS Operating Band; Frequency range (MHz); Declared emission level (dBW) (Measurement bandwidth = 1 MHz); Declared emission basic limit (dBW) of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz); Declared emission basic limit (dBW) of discrete emissions of less than 2 kHz bandwidth (Measurement bandwidth = 1 kHz). n24; 1 541-1 559; PEM_B24,a; PEM_B24,f. 1 559-1 610; PEM_B24,b; PEM_B24,d. 1 610-1 650; PEM_B24,c; PEM_B24,e. 6.6.4.5.6.6 Additional operating band unwanted emissions limits for operation with shared spectrum channel access In addition, for operation with shared spectrum channel access, the BS may have to comply with the applicable operating band unwanted emission limits established regionally, when deployed in regions where those limits apply and under the conditions declared by the manufacturer. The regional requirements may be in the form of conducted power, power spectral density, EIRP and other types of limits. In case of regulatory limits based on EIRP, assessment of the EIRP level is described in Annex F.2.

## 3.2	Conducted Adjacent channel leakage ratio (ACLR)

### 3.2.1	General requirements

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The assumed filter for the adjacent channel frequency is defined in Tables 6.6.3.5.2-1, 6.6.3.5.2-1a, 6.6.3.5.2-3 and 6.6.3.5.2-3a of TS 38.141-1 [1] and a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency.

Table 6.6.3.5.2-1: Base station ACLR limit BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted; Assumed adjacent channel carrier (informative); Filter on the adjacent channel frequency and corresponding filter bandwidth; ACLR limit. 5, 10, 15, 20; BWChannel; NR of same BW (Note 2); Square (BWConfig); 44.2 dB. 2 x BWChannel; NR of same BW (Note 2); Square (BWConfig); 44.2 dB. BWChannel /2 + 2.5 MHz; 5 MHz E-UTRA; Square (4.5 MHz); 44.2 dB (NOTE 3). BWChannel /2 + 7.5 MHz; 5 MHz E-UTRA; Square (4.5 MHz); 44.2 dB (NOTE 3). 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; BWChannel; NR of same BW (Note 2); Square (BWConfig); 43.8 dB. 2 x BWChannel; NR of same BW (Note 2); Square (BWConfig); 43.8 dB. BWChannel /2 + 2.5 MHz; 5 MHz E-UTRA; Square (4.5 MHz); 43.8 dB (NOTE 3). BWChannel /2 + 7.5 MHz; 5 MHz E-UTRA; Square (4.5 MHz); 43.8 dB (NOTE 3). Note 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). Note 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA.. Table 6.6.3.5.2-1a: Base station ACLR limit for band n96 and n102 BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted; Assumed adjacent channel carrier (informative); Filter on the adjacent channel frequency and corresponding filter bandwidth; ACLR limit. 10, 20, 40, 60, 80; BWChannel; NR of same BW (Note 2); Square (BWConfig); 35 dB. 2 x BWChannel; NR of same BW (Note 2); Square (BWConfig); 40 dB. Note 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig).. Table 6.6.3.5.2-3: Base Station ACLR limit in non-contiguous spectrum or multiple bands BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; ACLR limit. 5, 10, 15, 20; Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4); 2.5 MHz; 5 MHz NR (Note 2); Square (BWConfig); 44.2 dB. Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4); 7.5 MHz; 5 MHz NR (Note 2). 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3); 10 MHz; 20 MHz NR (Note 2); Square (BWConfig); 43.8 dB. Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3); 30 MHz; 20 MHz NR (Note 2). Note 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). Note 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. Note 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz.. Table 6.6.3.5.2-3a: Base Station ACLR limit in non-contiguous spectrum for band n96 and n102 BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; ACLR limit. 10, 20, 40, 60, 80; Wgap ≥ 60; 10 MHz; 20 MHz NR (Note 2); Square (BWConfig); 35 dB. Wgap ≥ 80; 30 MHz; 20 MHz NR (Note 2); Square (BWConfig); 40 dB. Note 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig)..

### 3.2.2	Limits and basic limits

For operation in paired and unpaired spectrum except for band n96 and n102, the ACLR should be higher than the value specified in Table 6.6.3.5.2-1 of TS 38.141-1 [1].

For operation in non-contiguous spectrum or multiple bands except for band n96 and n102, the ACLR should be higher than the value specified in Table 6.6.3.5.2-3 of TS 38.141-1 [1].

For band n96 and n102, the ACLR should be higher than the value specified in Table 6.6.3.5.2-1a of TS 38.141-1 [1].

The ACLR absolute basic limit is specified in Table 6.6.3.5.2-2 of TS 38.141-1 [1].

Table 6.6.3.5.2-2: Base station ACLR absolute basic limit BS category / BS class; ACLR absolute basic limit. Category A Wide Area BS; ‒13 dBm/MHz. Category B Wide Area BS; ‒15 dBm/MHz. Medium Range BS; ‒25 dBm/MHz. Local Area BS; ‒32 dBm/MHz.

### 3.2.3	BS type 1-C

For operation in paired and unpaired spectrum except for band n96 and n102, the ACLR test requirements for BS type 1-C are given in Tables 6.6.3.5.2-1 or 6.6.3.5.2-3 of TS 38.141-1 [1] applies per antenna connector. Conformance can be shown by meeting the ACLR limit in Tables 6.6.3.5.2-1 or 6.6.3.5.2-3 of TS 38.141-1 [1], or the absolute basic limits in Table 6.6.3.5.2-2 of TS 38.141-1 [1], whichever is less stringent.

For operation in band n96 and n102, the ACLR test requirements for BS type 1-C are given in Table 6.6.3.5.2-1a or 6.6.3.5.2-3a of TS 38.141-1 [1] applies per antenna connector. Conformance can be shown by meeting the ACLR limit in Table 6.6.3.5.2-1a or 6.6.3.5.2-3a of TS 38.141-1 [1], or the absolute basic limits in Table 6.6.3.5.2-2 of TS 38.141-1 [1], whichever is less stringent.

### 3.2.4	BS type 1-H

The ACLR absolute basic limits in Table 6.6.3.5.2-2 of TS 38.141-1 [1] + X (where X = 10log10(NTXU,countedpercell)) or the ACLR limits in Table 6.6.3.5.2-1 or 6.6.3.5.2-3 of TS 38.141-1 [1], whichever is less stringent, should apply for each TAB connector TX min cell group.

Conformance to the BS type 1-H ACLR limit can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The ratio of the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the assigned channel frequency to the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency should be greater than or equal to the ACLR limit of the BS. This should apply for each TAB connector TX min cell group.

Or

2)	The ratio of the filtered mean power at the TAB connector centred on the assigned channel frequency to the filtered mean power at this TAB connector centred on the adjacent channel frequency should be greater than or equal to the ACLR limit of the BS for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group.

In case the ACLR absolute basic limit of BS type 1-H are applied, the conformance can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency should be less than or equal to the ACLR absolute basic limit + X (where X = 10log10(NTXU,countedpercell)) of the BS. This should apply to each TAB connector TX min cell group.

Or

2)	The filtered mean power at each TAB connector centred on the adjacent channel frequency should be less than or equal to the ACLR absolute basic limit of the BS scaled by X −10log10(n) for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group, where n is the number of TAB connectors in the TAB connector TX min cell group.

## 3.3	Conducted Cumulative adjacent channel leakage ratio (CACLR)

### 3.3.1	General requirements

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in Table 6.6.3.5.2-4 of TS 38.141-1 [1] and the filters on the assigned channels are defined in Table 6.6.3.5.2-6 of TS 38.141-1 [1].

Table 6.6.3.5.2-4: Base station CACLR limit BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; CACLR limit. 5, 10, 15, 20; 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4); 2.5 MHz; 5 MHz NR (Note 2); Square (BWConfig); 44.2 dB. 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4); 7.5 MHz; 5 MHz NR (Note 2). 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3); 10 MHz; 20 MHz NR (Note 2); Square (BWConfig); 43.8 dB. 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3); 30 MHz; 20 MHz NR (Note 2). Note 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). Note 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. Note 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz.. Table 6.6.3.5.2-6: Filter parameters for the assigned channel RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap; Filter on the assigned channel frequency and corresponding filter bandwidth. NR; NR of same BW with SCS that provides largest transmission bandwidth configuration.

### 3.3.2	Limits and basic limits

For operation in non-contiguous spectrum or multiple bands except for band n96 and n102, the CACLR for NR carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap should be higher than the value specified in Table 6.6.3.5.2-4 of TS 38.141-1 [1].

For operation in non-contiguous spectrum for band n96 and n102, the CACLR for NR carriers located on either side of the sub-block gap should be higher than the value specified in Table 6.6.3.5.2-4a of TS 38.141-1 [1].

Table 6.6.3.5.2-4a: Base Station CACLR limit for band n96 and n102 BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; CACLR limit. 10, 20, 40, 60, 80; 20 ≤Wgap< 60; 10 MHz; 20 MHz NR (Note 2); Square (BWConfig); 35 dB. 40 < Wgap< 80; 30 MHz; 20 MHz NR (Note 2); Square (BWConfig); 40 dB. Note 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig)..

The CACLR absolute basic limit is specified in Table 6.6.3.5.2-5 of TS 38.141-1 [1].

Table 6.6.3.5.2-5: Base station CACLR absolute basic limit BS category / BS class; CACLR absolute basic limit. Category A Wide Area BS; ‒13 dBm/MHz. Category B Wide Area BS; ‒15 dBm/MHz. Medium Range BS; ‒25 dBm/MHz. Local Area BS; ‒32 dBm/MHz.

### 3.3.3	BS type 1-C

The CACLR test requirements for BS type 1-C are given in Table 6.6.3.5.2-4 of TS 38.141-1 [1] applies per antenna connector. Conformance can be shown by meeting the CACLR limit in Table 6.6.3.5.2-4 of TS 38.141-1 [1] or the absolute basic limits in Table 6.6.3.5.2-5 of TS 38.141-1 [1], whichever is less stringent.

### 3.3.4	BS type 1-H

The CACLR absolute basic limits in Table 6.6.3.5.2-5 of TS 38.141-1 [1] + X, (where X = 10log10(NTXU,countedpercell)) or the CACLR limits in Table 6.6.3.5.2-4 of TS 38.141-1 [1], whichever is less stringent, should apply for each TAB connector TX min cell group.

Conformance to the BS type 1-H CACLR limit can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The ratio of the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the assigned channel frequency to the sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency should be greater than or equal to the CACLR limit of the BS. This should apply for each TAB connector TX min cell group.

Or

2)	The ratio of the filtered mean power at the TAB connector centred on the assigned channel frequency to the filtered mean power at this TAB connector centred on the adjacent channel frequency should be greater than or equal to the CACLR limit of the BS for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group.

In case the CACLR absolute basic limit of BS type 1-H are applied, the conformance can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the filtered mean power measured on each TAB connector in the TAB connector TX min cell group at the adjacent channel frequency should be less than or equal to the CACLR absolute basic limit + X (where X = 10log10(NTXU,countedpercell)) of the BS. This should apply to each TAB connector TX min cell group.

Or

2)	The filtered mean power at each TAB connector centred on the adjacent channel frequency should be less than or equal to the CACLR absolute basic limit of the BS scaled by X -10log10(n) for every TAB connector in the TAB connector TX min cell group, for each TAB connector TX min cell group, where n is the number of TAB connectors in the TAB connector TX min cell group.

## 3.4	Conducted Transmitter spurious emissions

### 3.4.1	Conducted transmitter spurious emissions for BS type 1-C

The Tx spurious emissions for BS type 1-C for each antenna connector should not exceed the applicable basic limits defined in §§ 3.4.3-3.4.6.

### 3.4.2	Conducted transmitter spurious emissions for BS type 1-H

The Tx spurious emissions requirements for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in §§ 3.4.3-3.4.6, the power summation emissions at the TAB connectors of the TAB connector TX min cell group should not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE: Conformance to the BS type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group should be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector should be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by −10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.4.3	Basic limits for transmitter spurious emissions

The limits of either Table 6.6.5.5.1.1-1 in TS 38.141-1 [1] for Category A or Table 6.6.5.5.1.1-2 in TS 38.141-1 [1] for Category B should apply. The application of either Category A or Category B limits should be the same as for operating band unwanted emissions in § 3.1, and as declared by the manufacturer.

Table 6.6.5.5.1.1-1: General BS transmitter spurious emission limits in FR1, Category A Spurious frequency range; Basic limit; Measurement bandwidth; Notes. 9 kHz – 150 kHz; 1 kHz; Note 1, Note 4. 150 kHz – 30 MHz; 10 kHz; Note 1, Note 4. 30 MHz – 1 GHz; ‒13 dBm; 100 kHz; Note 1. 1 GHz – 12.75 GHz; 1 MHz; Note 1, Note 2. 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz; 1 MHz; Note 1, Note 2, Note 3. 12.75 GHz - 26 GHz; ‒13 dBm; 1 MHz; Note 1, Note 2, Note 5. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies only for band n96 and n102.. Table 6.6.5.5.1.1-2: General BS transmitter spurious emission limits in FR1, Category B Spurious frequency range; Basic limit; Measurement bandwidth; Notes. 9 kHz – 150 kHz; 1 kHz; Note 1, Note 4. 150 kHz – 30 MHz; ‒36 dBm; 10 kHz; Note 1, Note 4. 30 MHz – 1 GHz; 100 kHz; Note 1. 1 GHz – 12.75 GHz; ‒30 dBm; 1 MHz; Note 1, Note 2. 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz; 1 MHz; Note 1, Note 2, Note 3. 12.75 GHz - 26 GHz; ‒30 dBm; 1 MHz; Note 1, Note 2, Note 5. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies only for band n102..

### 3.4.4	Protection of the BS receiver of own or different BS

This requirement should be applied for NR FDD operation in order to prevent the receivers of the BSs being desensitised by emissions from a BS transmitter. It is measured at the transmit antenna connector for BS type 1-C or at the TAB connector for BS type 1-H for any type of BS which has common or separate Tx/Rx antenna connectors / TAB connectors.

The basic limits are provided in Table 6.6.5.5.1.2-1 in TS 38.141-1 [1].

Table 6.6.5.5.1.2-1: BS spurious emissions basic limits for protection of the BS receiver BS class; Frequency range; Basic limit; Measurement bandwidth. Wide Area BS; ‒96 dBm. Medium Range BS; FUL_low – FUL_high; ‒91 dBm; 100 kHz. Local Area BS; ‒88 dBm.

### 3.4.5	Additional spurious emission requirements

In certain regions the following additional requirements in TS 38.141-1 [1] may apply:

–	For BS co-existence with systems operating in other frequency bands, the requirement in Table 6.6.5.5.1.3-1.

–	For protection of PHS, the requirement in Table 6.6.5.5.1.3-2.

–	For BS operating in Band n50 and n75 within 1 432-1 452 MHz, and in Band n51 and Band n76, the requirement in Table 6.6.5.5.1.3-4.

–	For BS operating in NR Band n50 and n75 within 1 492-1 517 MHz, and in Band n74 within 14 92-1 518 MHz, the requirements in Table 6.6.5.5.1.3-5.

–	For BS operating in Band n13 and n14, the requirements in Table 6.6.5.5.1.3-6.

–	For BS operating in Band n30, the requirements in Table 6.6.5.5.1.3-7.

–	For BS operating in Band n48, the requirements in Table 6.6.5.5.1.3-8.

–	For BS operating in Band n26 for protection of 800 MHz public safety operations, the requirements in Table 6.6.5.5.1.3-9.

–	For BS operating in 3.45-3.55 GHz in Band n77 in certain regions, the requirements in Table 6.6.5.5.1.3-11.

–	For BS operating in Band n101 in CEPT countries, the requirements in Table 6.6.5.5.1.3-12.

–	For BS operating in Band n100 in CEPT countries, the requirements in Table 6.6.5.5.1.3-13.

Table 6.6.5.5.1.3-1: BS spurious emissions limits for BS for co-existence with systems operating in other frequency bands System type for NR to co-exist with; Frequency range for co-existence requirement; Basic limit; Measurement bandwidth; Note. GSM900; 921-960 MHz; −57 dBm; 100 kHz; This requirement does not apply to BS operating in band n8. 876-915 MHz; −61 dBm; 100 kHz; For the frequency range 880−915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.5.1.2.. DCS1800; 1 805-1 880 MHz; −47 dBm; 100 kHz; This requirement does not apply to BS operating in band n3.. 1 710-1 785 MHz; −61 dBm; 100 kHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.5.1.2.. PCS1900; 1 930-1 990 MHz; −47 dBm; 100 kHz; This requirement does not apply to BS operating in band n2, n25 or band n70.. 1 850-1 910 MHz; −61 dBm; 100 kHz; This requirement does not apply to BS operating in band n2 or n25 since it is already covered by the requirement in clause 6.6.5.5.1.2.. GSM850 or CDMA850; 869-894 MHz; −57 dBm; 100 kHz; This requirement does not apply to BS operating in band n5 or n26.. 824-849 MHz; −61 dBm; 100 kHz; This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band I or; 2 110-2 170 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n1 or n65. E-UTRA Band 1 or NR Band n1; 1 920-1 980 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band II or; 1 930-1 990 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n2 or n70.. E−UTRA Band 2 or NR Band n2; 1 850-1 910 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band III or; 1 805-1 880 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n3.. E-UTRA Band 3 or NR Band n3; 1 710-1 785 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band IV or; 2 110-2 155 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n66. E-UTRA Band 4; 1 710-1 755 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band V or; 869-894 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n5 or n26.. E-UTRA Band 5 or NR Band n5; 824-849 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band VI, XIX or; 860-890 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n18.. E-UTRA Band 6, 18, 19 or NR Band n18; 815-830 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n18, since it is already covered by the requirement in clause 6.6.5.2.2.. 830-845 MHz; −49 dBm; 1 MHz. UTRA FDD Band VII or; 2 620-2 690 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n7.. E-UTRA Band 7 or NR Band n7; 2 500-2 570 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n7, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band VIII or; 925-960 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n8 or n100.. E-UTRA Band 8 or NR Band n8; 880-915 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band IX or; 1 844.9-1 879.9 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n3.. E-UTRA Band 9; 1 749.9-1 784.9 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band X or; 2 110-2 170 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n66. E-UTRA Band 10; 1 710-1 770 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band XI or XXI or; 1 475.9-1 510.9 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74, n75, n92 or n94.. E-UTRA Band 11 or 21; 1 427.9-1 447.9 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94.. 1 447.9-1 462.9 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74, n75, n92 or n94.. UTRA FDD Band XII or; 729-746 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85.. E-UTRA Band 12 or NR Band n12; 699-716 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.6.5.5.1.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5).. UTRA FDD Band XIII or; 746-756 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n13.. E-UTRA Band 13 or NR Band n13; 777-787 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n13, since it is already covered by the requirement in clause 6.6.5.5.1.2. UTRA FDD Band XIV or; 758-768 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n14.. E-UTRA Band 14 or NR Band n14; 788-798 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n14, since it is already covered by the requirement in clause 6.6.5.5.1.2. E-UTRA Band 17; 734-746 MHz; −52 dBm; 1 MHz. 704-716 MHz; −49 dBm; 1 MHz; For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5).. UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20; 791-821 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n20 or n28.. 832-862 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.5.1.2.. UTRA FDD Band XXII or E-UTRA Band 22; 3 510-3 590 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n48, n77 or n78.. 3 410-3 490 MHz; −49 dBm; 1 MHz; This is not applicable to BS operating in Band n77 or n78.. E-UTRA Band 24 or NR Band n24; 1 525-1 559 MHz; −52 dBm; 1 MHz. 1 626.5-1 660.5 MHz; −49 dBm; 1 MHz. UTRA FDD Band XXV or; 1 930-1 995 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n2, n25 or n70.. E-UTRA Band 25 or NR band n25; 1 850-1 915 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n25 since it is already covered by the requirement in clause 6.6.5.5.1.2. For BS operating in Band n2, it applies for 1 910 MHz to 1915 MHz, while the rest is covered in clause 6.6.5.5.1.2.. UTRA FDD Band XXVI or; 859-894 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n5 or n26.. E-UTRA Band 26 or NR Band n26; 814-849 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n26 since it is already covered by the requirement in clause 6.6.5.5.1.2. For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.6.5.5.1.2.. E-UTRA Band 27; 852-869 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n5.. 807-824 MHz; −49 dBm; 1 MHz; This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 5).. E-UTRA Band 28 or NR Band n28; 758-803 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, n67 or n28.. 703-748 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.5.1.2. For BS operating in band n67, it applies for 703 MHz to 736 MHz.. E-UTRA Band 29 or NR Band n29; 717-728 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n29 or n85.. E-UTRA Band 30 or NR Band n30; 2 350-2 360 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n30.. 2 305-2 315 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n30, since it is already covered by the requirement in clause 6.6.5.5.1.2.. E-UTRA Band 31; 462.5-467.5 MHz; −52 dBm; 1 MHz. 452.5-457.5 MHz; −49 dBm; 1 MHz. UTRA FDD band XXXII or E-UTRA band 32; 1 452-1 496 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74, n75, n92 or n94.. UTRA TDD Band a) or E-UTRA Band 33; 1 900-1 920 MHz; −52 dBm; 1 MHz. UTRA TDD Band a) or E-UTRA Band 34 or NR band n34; 2 010-2 025 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n34.. UTRA TDD Band b) or E-UTRA Band 35; 1 850-1 910 MHz; −52 dBm; 1 MHz. UTRA TDD Band b) or E-UTRA Band 36; 1 930-1 990 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n2 or n25.. UTRA TDD Band c) or E-UTRA Band 37; 1 910-1 930 MHz; −52 dBm; 1 MHz. UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38; 2 570-2 620 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n38.. UTRA TDD Band f) or E-UTRA Band 39 or NR band n39; 1 880-1 920 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n39.. UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40; 2 300-2 400 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Bands n30 or n40.. E-UTRA Band 41 or NR Band n41; 2 496-2 690 MHz; −52 dBm; 1 MHz; This is not applicable to BS operating in Band n41 or n53.. E-UTRA Band 42; 3 400-3 600 MHz; −52 dBm; 1 MHz; This is not applicable to BS operating in Band n48, n77 or n78.. E-UTRA Band 43; 3 600-3 800 MHz; −52 dBm; 1 MHz; This is not applicable to BS operating in Band n48, n77 or n78.. E-UTRA Band 44; 703-803 MHz; −52 dBm; 1 MHz; This is not applicable to BS operating in Band n28.. E-UTRA Band 45; 1447-1467 MHz; −52 dBm; 1 MHz. E-UTRA Band 47; 5 855-5 925 MHz; −52 dBm; 1 MHz. E-UTRA Band 48 or NR Band n48; 3 550-3 700 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n48, n77 and n78.. E-UTRA Band 50 or NR band n50; 1 432-1 517 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94.. E-UTRA Band 51 or NR Band n51; 1 427-1 432 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93 or n94.. E-UTRA Band 53 or NR Band n53; 2 483.5-2 495 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n41, n53 or n90.. E-UTRA Band 65 or NR Band n65; 2 110-2 200 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n1 or n65. 1 920-2 010 MHz; −49 dBm; 1 MHz; For BS operating in Band n1, it applies for 1 980 MHz to 2010 MHz, while the rest is covered in clause 6.6.5.5.1.2. This requirement does not apply to BS operating in band n65, since it is already covered by the requirement in clause 6.6.5.5.1.2.. E-UTRA Band 66 or NR Band n66; 2 110-2 200 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n66.. 1 710-1 780 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.5.1.2.. E-UTRA Band 67 or NR Band n67; 738-758 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n28 or n67.. E-UTRA Band 68; 753-783 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n28.. 698-728 MHz; −49 dBm; 1 MHz; For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.6.5.5.1.2.. E-UTRA Band 69; 2 570-2 620 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n38.. E-UTRA Band 70 or NR Band n70; 1 995-2 020 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n2, n25 or n70. 1 695-1 710 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n70, since it is already covered by the requirement in clause 66.6.5.5.1.2.. E-UTRA Band 71 or NR Band n71; 617-652 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n71. 663-698 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n71, since it is already covered by the requirement in clause 6.6.5.5.1.2.. E-UTRA Band 72; 461-466 MHz; −52 dBm; 1 MHz. 451-456 MHz; −49 dBm; 1 MHz. E-UTRA Band 74 or NR Band n74; 1 475-1 518 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n75, n75, n92 or n94.. 1 427-1 470 MHz; −49 dBm; 1MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94.. E-UTRA Band 75 or NR Band n75; 1 432-1 517 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94.. E-UTRA Band 76 or NR Band n76; 1 427-1 432 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93 or n94.. NR Band n77; 3.3-4.2 GHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n48, n77 or n78. NR Band n78; 3.3-3.8 GHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n48, n77 or n78. NR Band n79; 4.4-5.0 GHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n79. NR Band n80; 1 710-1 785 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n81; 880-915 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n82; 832-862 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n83; 703-748 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.6.5.5.1.2. For BS operating in Band n67, it applies for 703 MHz to 736 MHz.. NR Band n84; 1 920-1 980 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n1, since it is already covered by the requirement in clause 6.6.5.5.1.2.. E-UTRA Band 85 or NR Band n85; 728-746 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85.. 698-716 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.6.5.5.1.2. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5).. NR Band n86; 1 710-1 780 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n89; 824-849 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n91; 1 427-1 432 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75 or n76.. 832-862 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n92; 1 432-1 517 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. 832-862 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n93; 1 427-1 432 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75 or n76.. 880-915 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n94; 1 432-1 517 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. 880-915 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n95; 2 010-2 025 MHz; −52 dBm; 1 MHz. NR Band n96; 5 925-7 125 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n96 or n102.. NR Band n97; 2 300-2 400 MHz; −52 dBm; 1 MHz. NR Band n98; 1 880-1 920 MHz; −52 dBm; 1 MHz. NR Band n99; 1 626.5-1 660.5 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n24, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n100; 919.4-925 MHz; −52 dBm; 1 MHz; This requirement does not apply to E-UTRA BS operating in Band n8 or n100.. 874.4-880 MHz; −49 dBm; 1 MHz; This requirement does not apply to BS operating in band n100, since it is already covered by the requirement in clause 6.6.5.5.1.2.. NR Band n101; 1 900-1 910 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n101.. NR Band n102; 5 925-6 425 MHz; −52 dBm; 1 MHz; This requirement does not apply to BS operating in Band n96 or n102.. E-UTRA Band 103; 757-758 MHz; −52 dBm; 1 MHz. 787-788 MHz; −49 dBm; 1 MHz. NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in Table 6.6.5.5.1.3-1do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see TS 38.104 [2], Table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 2: Table 6.6.5.5.1.3-1 assumes that two operating bands, where the frequency ranges in TS 38.104 [2], Table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 3: TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 4: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band. NOTE 5: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band. Table 6.6.5.5.1.3-2: BS spurious emissions basic limits for BS for co-existence with PHS Frequency range; Basic limit; Measurement bandwidth; Note. 1 884.5-1 915.7 MHz; −41 dBm; 300 kHz; Applicable when co-existence with PHS system operating in 1 884.5-1 915.7 MHz. Table 6.6.5.5.1.3-4: Additional operating band unwanted emission basic limits for BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band 51 and 76 Filter centre frequency, filter; Basic limit; Measurement bandwidth. Ffilter = 1 413.5 MHz; −42 dBm; 27 MHz. Table 6.6.5.5.1.3-5: Operating band n50, n74 and n75 declared emission above 1518 MHz Filter centre frequency, Ffilter; Declared emission basic limit (dBm); Measurement bandwidth. 1 518.5 MHz ≤ Ffilter ≤ 1 519.5 MHz; PEM, n50,a; 1 MHz. 1 520.5 MHz ≤ Ffilter ≤ 1 558.5 MHz; PEM,n50,b; 1 MHz. Table 6.6.5.5.1.3-6: BS Spurious emissions limits for protection of 700 MHz public safety operations Operating Band; Frequency range; Maximum Level; Measurement Bandwidth. n13; 763-775 MHz; −46 dBm; 6.25 kHz. n13; 793-805 MHz; −46 dBm; 6.25 kHz. n14; 769-775 MHz; −46 dBm; 6.25 kHz. n14; 799-805 MHz; −46 dBm; 6.25 kHz. Table 6.6.5.5.1.3-7: Additional NR BS Spurious emissions limits for Band n30 Frequency range; Basic limit; Measurement bandwidth. 2 200-2 345 MHz; −45 dBm. 2 362.5-2 365 MHz; −25 dBm. 2 365-2 367.5 MHz; −40 dBm; 1 MHz. 2 367.5-2 370 MHz; −42 dBm. 2 370-2 395 MHz; −45 dBm. Table 6.6.5.2.3-8: Additional BS Spurious emissions limits for Band n48 Frequency range; Maximum Level; Measurement Bandwidth (NOTE); Note. 3 530-3 720 MHz; −25 dBm; 1 MHz; Applicable 10 MHz from the assigned channel edge. 3 100-3 530 MHz 3 720-4 200 MHz; −40 dBm; 1 MHz. NOTE: The resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. Table 6.6.5.2.3-9: BS Spurious emissions limits for protection of 800 MHz public safety operations Operating Band; Frequency range; Maximum Level; Measurement Bandwidth; Note. n26; 851-859 MHz; −13 dBm; 100 kHz; Applicable for offsets > 37.5 kHz from the channel edge. Table 6.6.5.5.1.3-10: Additional BS Spurious emissions limits for Band n41 and n90 Frequency range; Basic limit; Measurement Bandwidth. 2 505-2 535 MHz; −42 dBm; 1 MHz. NOTE: This requirement applies for carriers allocated within 2 545-2 645 MHz.. Table 6.6.5.5.1.3-11: Additional BS spurious emissions limits for Band n77 Channel bandwidth [MHz]; Frequency range [MHz]; Filter centre frequency, Ffilter [MHz]; Minimum requirement [dBm]; Measurement bandwidth [MHz]. All; 3 430-3 440 3 560-3 570; 3430.5 Ffilter < 3 439.5 3560.5 Ffilter < 3 569.5; −25; 1. All; 3 430 > 3 570; Ffilter < 3 429.5 3570.5 Ffilter; −40; 1. NOTE: The resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. Table 6.6.5.5.1.3-12: Additional BS Spurious emissions limits for Band n101 Frequency range; Maximum Level; Measurement Bandwidth; Note. 1 920-1 980 MHz; −57 dBm; 5 MHz. Table 6.6.5.5.1.3-13: Additional BS Spurious emissions limits for Band n100 Frequency range; Maximum Level; Measurement Bandwidth; Note. 880-915 MHz; −62 dBm; 5 MHz.

### 3.4.6	Co-location with other base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class.

The basic limits are in Table 6.6.5.5.1.4-1 in TS 38-141-1 [1] for a BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared BS class. For a multi-band connector, the exclusions and conditions in the Note column of Table 6.6.5.5.1.4-1 should apply for each supported operating band.

Table 6.6.5.5.1.4-1: BS spurious emissions basic limits for BS co-located with another BS Type of co-located BS; Frequency range for; Basic limit; Measurement; Note. co-location requirement; WA BS; MR BS; LA BS; bandwidth. GSM900; 876-915 MHz; −98 dBm; −91 dBm; −70 dBm; 100 kHz. DCS1800; 1 710-1785 MHz; −98 dBm; −91 dBm; −80 dBm; 100 kHz. PCS1900; 1 850-1 910 MHz; −98 dBm; −91 dBm; −80 dBm; 100 kHz. GSM850 or CDMA850; 824-849 MHz; −98 dBm; −91 dBm; −70 dBm; 100 kHz. UTRA FDD Band I or E-UTRA Band 1 or NR Band n1; 1 920-1 980 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band II or E-UTRA Band 2 or NR Band n2; 1 850-1 910 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band III or E-UTRA Band 3 or NR Band n3; 1 710-1 785 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band IV or E-UTRA Band 4; 1 710-1 755 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band V or E-UTRA Band 5 or NR Band n5; 824-849 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band VI, XIX or E-UTRA Band 6, 19; 830-845 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7; 2 500-2 570 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8; 880-915 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band IX or E-UTRA Band 9; 1 749.9-1 784.9 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band X or E-UTRA Band 10; 1 710-1 770 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XI or E-UTRA Band 11; 1 427.9-1 447.9 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n50, n75, n91, n92, n93 or n94. UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12; 699-716 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13; 777-787 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14; 788-798 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 17; 704-716 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 18 or NR Band n18; 815-830 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20; 832-862 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XXI or E-UTRA Band 21; 1 447.9-1 462.9 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n50, n75, n92 or n94. UTRA FDD Band XXII or E-UTRA Band 22; 3 410-3 490 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n48, n77 or n78. E-UTRA Band 24 or NR Band n24; 1 626.5-1 660.5 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25; 1 850-1 915 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26; 814-849 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 27; 807-824 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 28 or NR Band n28; 703-748 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 30 or NR Band n30; 2 305-2 315 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 31; 452.5-457.5 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA TDD Band a) or E-UTRA Band 33; 1 900-1 920 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA TDD Band a) or E-UTRA Band 34 or NR band n34; 2 010-2 025 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n34. UTRA TDD Band b) or E-UTRA Band 35; 1 850-1 910 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA TDD Band b) or E-UTRA Band 36; 1 930-1 990 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n2 or band n25. UTRA TDD Band c) or E-UTRA Band 37; 1910-1 930 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38; 2 570-2620 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n38.. UTRA TDD Band f) or E-UTRA Band 39 or NR band n39; 1 880-1 920 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n39. UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40; 2 300-2 400 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Bands n30 or n40.. E-UTRA Band 41 or NR Band n41; 2 496-2 690 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n41 or n53. E-UTRA Band 42; 3 400-3 600 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n48, n77 or n78. E-UTRA Band 43; 3 600-3 800 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n48, n77 or n78. E-UTRA Band 44; 703-803 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n28. E-UTRA Band 45; 1 447-1 467 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 48 or NR Band n48; 3 550-3 700 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n48, n77 or n78. E-UTRA Band 50 or NR band n50; 1 432-1 517 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n51, n74, n75, n91, n92, n93 or n94. E-UTRA Band 51 or NR Band n51; 1 427-1 432 MHz; N/A; N/A; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n50, n74, n75, n76, n91, n92, n93 or n94. E-UTRA Band 53 or NR Band n53; 2 483.5-2 495 MHz; N/A; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n41, n53 or n90. E-UTRA Band 65 or NR Band n65; 1 920-2 010 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 66 or NR Band n66; 1 710-1 780 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 68; 698-728 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 70 or NR Band n70; 1 695-1 710 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 71 or NR Band n71; 663-698 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 72; 451-456 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 74 or NR Band n74; 1 427-1 470 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n50, n51, n91, n92, n93 or n94. NR Band n77; 3.3-4.2 GHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n48, n77 or n78. NR Band n78; 3.3-3.8 GHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz; This is not applicable to BS operating in Band n48, n77 or n78. NR Band n79; 4.4-5.0 GHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n80; 1 710-1 785 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n81; 880-915 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n82; 832-862 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n83; 703-748 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n84; 1 920-1 980 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. E-UTRA Band 85 or NR Band n85; 698-716 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n86; 1 710-1 780 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n89; 824-849 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n91; 832-862 MHz; N/A; N/A; −88 dBm; 100 kHz. NR Band n92; 832-862 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n93; 880-915 MHz; N/A; N/A; −88 dBm; 100 kHz. NR Band n94; 880-915 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n95; 2 010-2 025 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n96; 5 925-7 125 MHz; N/A; −90 dBm; −87 dBm; 100 kHz; This is not applicable to BS operating in Band n96 or n102. NR Band n97; 2 300-2 400 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n98; 1 880-1 920 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n99; 1 626.5-1 660.5 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NR Band n100; 874.4-880 MHz; −96 dBm; N/A; N/A; 100 kHz. NR Band n101; 1 900-1 910 MHz; −96 dBm; N/A; N/A; 100 kHz. NR Band n102; 5 925-6 425 MHz; N/A; −90 dBm; −87 dBm; 100 kHz; This is not applicable to BS operating in Band n96 or n102. E-UTRA Band 103; 787-788 MHz; −96 dBm; −91 dBm; −88 dBm; 100 kHz. NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.5.1.4-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see TS 38.104 [2] Table 5.2-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [15]. NOTE 2: Table 6.6.5.5.1.4-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in TS 38.104 [2] Table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 3: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

## 3.5	Conducted receiver spurious emissions

### 3.5.1	Conducted receiver spurious emissions BS type 1-C

The receiver spurious emissions for BS type 1-C for each antenna connector should be below the applicable basic limits defined in § 3.5.3.

### 3.5.2	Conducted receiver spurious emissions BS type 1-H

The receiver spurious emissions for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in § 3.5.3, the power summation emissions at the TAB connectors of the TAB connector TX min cell group should not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

NOTE: Conformance to the BS type 1-H emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group should be less than or equal to the limit as defined in this clause for the respective frequency span.

or

2)	The unwanted emissions power at each TAB connector should be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by –10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

### 3.5.3	Basic limits for receiver spurious emissions

Basic limits for receiver spurious emissions are specified in Table 7.6.5.1-1 of TS 38.141-1 [1].

Table 7.6.5.1-1: General BS receiver spurious emissions limits Spurious frequency range; Basic limit; Measurement bandwidth; Notes. 30 MHz-1 GHz; -57 dBm; 100 kHz; Note 1. 1 GHz-12.75 GHz; -47 dBm; 1 MHz; Note 1, Note 2. 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz; -47 dBm; 1 MHz; Note 1, Note 2, Note 3. 12.75 GHz - 26 GHz; -47 dBm; 1 MHz; Note 1, Note 2, Note 6. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the UL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. For multi-band connectors, the exclusion applies for all supported operating bands. NOTE 5: Void. NOTE 6: Applies only for band n96 and n102..

# 4	NR Unwanted emission characteristics for BS type 1-O and BS type 2-O

The maximum offset of the operating band unwanted emissions mask from the operating band edge is ΔfOBUE. The operating band unwanted emissions define unwanted emissions in each supported downlink operating band plus the frequency ranges ΔfOBUE above and ΔfOBUE below each band. Unwanted emissions outside of this frequency range are defined by a spurious emissions characteristics.

NOTE: The frequency offset ΔfOBUE does not define the boundary between the out-of-band and spurious domain as set out in Recommendation ITU-R SM.1541. As a result, in the range from the operating band edge up to ΔfOBUE characteristics of unwanted emissions presented in this Annex may cover only part of out-of-band domain frequency range. In such a case, for the remaining part of out-of-band domain frequency range spurious emissions characteristics should be used.

The value of ΔfOBUE is defined in Table A2-8 for BS type 1-O and BS type 2-O for the NR operating bands.

TABLE A2-8

Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL_high – FDL_low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL_high – FDL_low ≤ 900 MHz | 40 |
| BS type 2-O | FDL_high – FDL_low ≤ 4 000 MHz | 1 500 |

## 4.1	OTA Operating band unwanted emissions

### 4.1.1	OTA Operating band unwanted emissions BS type 1-O

For BS type 1-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to BS channel bandwidths of the outermost carrier. In addition, for a RIB operating in non-contiguous spectrum, the requirements should apply inside any sub-block gap. In addition, for a multi-band RIB, the requirements should apply inside any Inter RF Bandwidth gap.

#### 4.1.1.1	Wide Area BS (Category A)

For a RIB operating in Bands n5, n8, n12, n13, n14, n26, n28, n29, n71, n85, emissions should not exceed the maximum levels specified in Table 6.7.4.5.1.1-1 of TS 38.141-2 [2].

Table 6.7.4.5.1.1-1: Wide Area BS operating band unwanted emission limits (NR bands ≤ 1 GHz) for Category A Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 3.8 dBm - 7/5(f_offset/MHz - 0.05) dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −3.2 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −4 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -4 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.

For a RIB operating in Bands n1, n2, n3, n7, n25, n30, n34, n38, n39, n40, n41, n50, n65, n66, n70, n74, n75, n77, n78, n79, emissions should not exceed the maximum levels specified in Tables 6.7.4.5.1.1-2 to 6.7.4.5.1.1-4 of TS 38.141-2 [2].

Table 6.7.4.5.1.1-2: Wide Area BS operating band unwanted emission limits (1 GHz < NR bands ≤ 3 GHz) for Category A Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 3.8 dBm-7/5(f_offset/MHz-0.05)dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −3.2 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −4 dBm (Note 3); 1 MHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.. Table 6.7.4.5.1.1-4: Wide Area BS operating band unwanted emission limits (4.2 GHz < NR bands ≤ 6 GHz) for Category A Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 4 dBm-7/5(f_offset/MHz-0.05)dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −3 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −4 dBm (Note 3); 1 MHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -4 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

#### 4.1.1.2	Wide Area BS Category B (Option 1)

For a RIB operating in Bands n5, n8, n12, n20, n26, n28, n29, n67, n71, n85, emissions should not exceed the maximum levels specified in Table 6.7.4.5.1.2-1 of TS 38.141-2 [2].

Table 6.7.4.5.1.2-1: Wide Area BS operating band unwanted emission limits (NR bands ≤ 1 GHz) for Category B Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 3.8 dBm-7/5(f_offset/MHz-0.05)dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); ‒3.2 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; ‒7 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -7 dBm/ 100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

For a RIB operating in Bands n1, n2, n3, n7, n25, n34, n38, n39, n40, n41, n50, n65, n66, n70, n75, n77, n78, n79, emissions should not exceed the maximum levels specified in Tables 6.7.4.5.1.2-2 to 6.7.4.5.1.2-4 of TS 38.141-2 [2].

Table 6.7.4.5.1.2-2: Wide Area BS operating band unwanted emission limits (1 GHz < NR bands ≤ 3 GHz) for Category B Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 3.8 dBm-7/5(f_offset/MHz-0.05)dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −3.2 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −6 dBm (Note 3); 1 MHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.. Table 6.7.4.5.1.2-3: Wide Area BS operating band unwanted emission limits (3 GHz < NR bands ≤ 4.2 GHz) for Category B Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 4 dBm-7/5(f_offset/MHz-0.05)dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −3 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −6 dBm (Note 3); 1 MHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void. Table 6.7.4.5.1.2-4: Wide Area BS operating band unwanted emission limits (4.2 GHz < NR bands ≤ 6 GHz) for Category B Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 4 dBm-7/5(f_offset/MHz-0.05)dB; 100 kHz. 5 MHz f < min(10 MHz, fmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −3 dBm; 100 kHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −6 dBm (Note 3); 1 MHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −6 dBm/1 MHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.

#### 4.1.1.3	Wide Area BS Category B (Option 2)

For a RIB operating in bands n1, n3, n8, n65 emissions should not exceed the maximum levels specified in Table 6.7.4.5.1.3-1 of TS 38.141-2 [2].

Table 6.7.4.5.1.3-1: Regional Wide Area BS operating band unwanted emission limits for Category B Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Basic limit (Note 1, 2, 5); Measurement bandwidth. 0 MHz f < 0.2 MHz; 0.015 MHz f_offset < 0.215 MHz; −3.2 dBm; 30 kHz. 0.2 MHz f < 1 MHz; 0.215 MHz f_offset < 1.015 MHz; 30 kHz. (Note 4); 1.015 MHz f_offset < 1.5 MHz; −15.2 dBm; 30 kHz. 1 MHz f min( 10 MHz, fmax); 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax); −2.2 dBm; 1 MHz. 10 MHz f fmax; 10.5 MHz f_offset < f_offsetmax; −6 dBm (Note 3); 1 MHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be −6 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: This frequency range ensures that the range of values of f_offset is continuous. NOTE 5: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 6: Void..

#### 4.1.1.4	Medium Range BS (Category A and Category B)

For Medium Range BS class in NR bands ≤ 3 GHz, emissions should not exceed the maximum levels specified in Tables 6.7.4.5.1.4-1 and 6.7.4.5.1.4-4 of TS 38.141-2 [2].

Table 6.7.4.5.1.4-1: Medium Range BS operating band unwanted emission limits, 40 < Prated,c,TRP 47 dBm (NR bands ≤ 3 GHz) Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); Prated,c,TRP – 58.2 dB; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; Min(Prated,c,TRP −60 dB, −16 dBm) (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,c,TRP −60 dB, −16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.. Table 6.7.4.5.1.4-4: Medium Range BS operating band unwanted emission limits, Prated,c,TRP 40 dBm (NR bands ≤ 3 GHz) Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −18.2 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −20 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −20 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

For Medium Range BS class in 3 GHz <NR bands ≤ 4.2 GHz, emissions should not exceed the maximum levels specified in Tables 6.7.4.5.1.4-2 and 6.7.4.5.1.4-5 of TS 38.141-2 [2].

Table 6.7.4.5.1.4-2: Medium Range BS operating band unwanted emission limits, 40 < Prated,c,TRP 47 dBm (3 GHz < NR bands ≤ 4.2 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); Prated,c,TRP - 58 dB; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; Min(Prated,c,TRP –60 dB, −16 dBm) (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,c,TRP −60 dB, −16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.. Table 6.7.4.5.1.4-5: Medium Range BS operating band unwanted emission limits, Prated,c,TRP 40 dBm (3 GHz < NR bands ≤ 4.2 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −18 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −20 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −20 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

For Medium Range BS class in 4.2 GHz <NR bands ≤ 6 GHz, emissions should not exceed the maximum levels specified in Tables 6.7.4.5.1.4-3 and 6.7.4.5.1.4-6 of TS 38.141-2 [2].

Table 6.7.4.5.1.4-3: Medium Range BS operating band unwanted emission limits, 40 < Prated,c,TRP 47 dBm (4.2 GHz < NR bands ≤ 6 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); Prated,c,TRP - 58 dB; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; Min(Prated,c,TRP – 60 dB, -16 dBm) (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,c,TRP – 60 dB, -16 dBm)/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void.. Table 6.7.4.5.1.4-6: Medium Range BS operating band unwanted emission limits, Prated,c,TRP 40 dBm (4.2 GHz < NR bands ≤ 6 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −18 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −20 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −20 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

#### 4.1.1.5	Local Area BS (Category A and Category B)

For Local Area BS class in NR bands ≤ 3 GHz, emissions should not exceed the maximum levels specified in Table 6.7.4.5.1.5-1 of TS 38.141-2 [2].

Table 6.7.4.5.1.5-1: Local Area BS operating band unwanted emission limits (NR bands ≤ 3 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −26.2 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −28 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −28 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

For Local Area BS class in 3 GHz < NR bands ≤ 4.2 GHz, emissions should not exceed the maximum levels specified in Table 6.7.4.5.1.5-2 of TS 38.141-2 [2].

Table 6.7.4.5.1.5-2: Local Area BS operating band unwanted emission limits (3 GHz < NR bands ≤ 4.2 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −26 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −28 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −28 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

For Local Area BS class in 4.2 GHz < NR bands ≤ 6 GHz, emissions should not exceed the maximum levels specified in Table 6.7.4.5.1.5-3 of TS 38.141-2 [2].

Table 6.7.4.5.1.5-3: Local Area BS operating band unwanted emission limits (4.2 GHz < NR bands ≤ 6 GHz) Frequency offset of measurement filter -3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test requirement (Note 1, 2, 4); Measurement bandwidth. 0 MHz f < 5 MHz; 0.05 MHz f_offset < 5.05 MHz; 100 kHz. 5 MHz f < min(10 MHz, Δfmax); 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax); −26 dBm; 100 kHz. 10 MHz f fmax; 10.05 MHz f_offset < f_offsetmax; −28 dBm (Note 3); 100 kHz. NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is Δf ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be −28 dBm/100 kHz. NOTE 2: For a multi-band RIB with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 10 MHz. NOTE 4: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 5: Void..

#### 4.1.1.6	Additional requirements

In certain regions the following additional requirements may apply:

–	For protection of DTT, the requirement in clause 6.7.4.5.1.6.2 of TS 38.141-2 [2].

–	For Protection of GPS, the requirement in clause 6.7.4.5.1.6.3 of TS 38.141-2 [2].

–	For operation in Bands n50, n51, n74, n75, n76, the requirement in clause 6.7.4.5.1.6.4 of TS 38.141-2 [2].

6.7.4.5.1.6.2 Protection of DTT In certain regions the following requirement may apply for protection of DTT. For BS type 1-O operating in Band n20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to Table 6.7.4.5.1.6.2-1, shall not exceed the maximum emission TRP level shown in the table. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain. Table 6.7.4.5.1.6.2-1: Declared emissions levels for protection of DTT Case; Measurement filter centre frequency; Condition on BS maximum aggregate EIRP / 10 MHz, PEIRP_10MHz (NOTE); Maximum level PEIRP,N,MAX; Measurement bandwidth. A: for DTT frequencies where; N*8 + 306 MHz, 21 ≤ N ≤ 60; PEIRP_10MHz 59 dBm; 0 dBm; 8 MHz. broadcasting is protected; N*8 + 306 MHz, 21 ≤ N ≤ 60; 36 PEIRP_10MHz < 59 dBm; PEIRP_10MHz – 59 dBm; 8 MHz. N*8 + 306 MHz, 21 ≤ N ≤ 60; PEIRP_10MHz < 36 dBm; −23 dBm; 8 MHz. B: for DTT frequencies where; N*8 + 306 MHz, 21 ≤ N ≤ 60; PEIRP_10MHz 59 dBm; 10 dBm; 8 MHz. broadcasting is subject to an; N*8 + 306 MHz, 21 ≤ N ≤ 60; 36 PEIRP_10MHz < 59 dBm; PEIRP_10MHz – 49 dBm; 8 MHz. intermediate level of protection; N*8 + 306 MHz, 21 ≤ N ≤ 60; PEIRP_10MHz < 36 dBm; −13 dBm; 8 MHz. C: for DTT frequencies where broadcasting is not protected; N*8 + 306 MHz, 21 ≤ N ≤ 60; N.A.; 22 dBm; 8 MHz. NOTE: PEIRP_10MHz (dBm) is defined by PEIRP_10MHz = P10MHz + Gant + 9dB, where Gant is 17 dBi.. 6.7.4.5.1.6.3 Protection of GPS The level of emissions in the 1 541-1 650 MHz band, measured in measurement bandwidth according to Table 6.7.4.5.1.6.3-1 shall not exceed the maximum emission TRP limits indicated in the table. This requirement applies in the frequency range 1 541-1 650 MHz even though part of the range falls in the spurious domain. Table 6.7.4.5.1.6.3-1: Emissions levels for protection of GPS Operating Band; Frequency range (MHz); Emission level (dBW) (Measurement bandwidth = 1 MHz); Emission level (dBW) of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz); Emission level (dBW) of discrete emissions of less than 2 kHz bandwidth (Measurement bandwidth = 1 kHz). n24; 1 541-1 559; PEIRP – 17 dBi + 9 dB; PEIRP – 17 dBi + 9 dB. 1 559-1 610; PEIRP – 17 dBi + 9 dB; PEIRP – 17 dBi + 9 dB. 1 610-1 650; PEIRP – 17 dBi + 9 dB; PEIRP – 17 dBi + 9 dB. 6.7.4.5.1.6.4 Additional limits for BS operating in Bands n50, n51, n74, n75, n76 For BS operating in bands n50, n51, n74, n75 and n76 additional emission limits that might be applicable in the OBUE frequency domain are specified in clause 6.7.5.4.5.

### 4.1.2	OTA Operating band unwanted emissions BS type 2-O

For BS type 2-O, for a RIB operating in multi-carrier or contiguous CA, the requirements apply to the frequencies starting from the edge of the contiguous transmission bandwidth. In addition, for a RIB operating in non-contiguous spectrum, the requirements apply inside any sub-block gap.

#### 4.1.2.1	OTA operating band unwanted emission limits (Category A)

The power of unwanted emission should not exceed the limits in Tables 6.7.4.5.2.2-1, 6.7.4.5.2.2-2 or 6.7.4.5.2.2-3 of TS 38.141-2 [2].

Table 6.7.4.5.2.2-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(−2.3 dBm, Max(Prated,t,TRP – 32.3 dB, -9.3 dBm)); 1 MHz. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax; Min(−13 dBm, Max(Prated,t,TRP – 43 dB, −20 dBm)); 1 MHz. NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap.. Table 6.7.4.5.2.2-2: OBUE limits applicable in the frequency range 37 GHz – 43.5 GHz Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(−2.3 dBm, Max(Prated,t,TRP – 30.3 dB, −9.3 dBm)); 1 MHz. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax; Min(-13 dBm, Max(Prated,t,TRP – 41 dB, −20 dBm)); 1 MHz. NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap.. Table 6.7.4.5.2.2-3: OBUE limits applicable in the frequency range 43.5 GHz – 48.2 GHz Frequency offset of measurement filter −3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(−2.1 dBm, Max(Prated,t,TRP – 30.1 dB, −9.1 dBm)); 1 MHz. 0.1*BWcontiguous f < fmax; 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax; Min(−13 dBm, Max(Prated,t,TRP – 41 dB, −20 dBm)); 1 MHz. NOTE: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap..

#### 4.1.2.2	OTA operating band unwanted emission limits (Category B)

The power of unwanted emission should not exceed the limits in Tables 6.7.4.5.2.3-1, 6.7.4.5.2.3-2 or 6.7.4.5.2.3-3 of TS 38.141-2 [2].

Table 6.7.4.5.2.3-1: OBUE limits applicable in the frequency range 24.25 – 33.4 GHz Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(−2.3 dBm, Max(Prated,t,TRP – 32.3 dB, −9.3 dBm)); 1 MHz. 0.1*BWcontiguous f < fB; 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz; Min(−13 dBm, Max(Prated,t,TRP – 43 dB, −20 dBm)); 1 MHz. fB f < fmax; fB +5 MHz f_offset < f_ offsetmax; Min(−5 dBm, Max(Prated,t,TRP – 33 dB, −10 dBm)); 10 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz.. Table 6.7.4.5.2.3-2: OBUE limits applicable in the frequency range 37 – 43.5 GHz Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(−2.3 dBm, Max(Prated,t,TRP – 30.3 dB, −9.3 dBm)); 1 MHz. 0.1*BWcontiguous f < fB; 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz; Min(−13 dBm, Max(Prated,t,TRP – 41 dB, −20 dBm)); 1 MHz. fB f < fmax; fB +5 MHz f_offset < f_ offsetmax; Min(−5 dBm, Max(Prated,t,TRP – 31 dB, −10 dBm)); 10 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz.. Table 6.7.4.5.2.3-3: OBUE limits applicable in the frequency range 43.5 – 48.2 GHz Frequency offset of measurement filter ‒3 dB point, f; Frequency offset of measurement filter centre frequency, f_offset; Test limit; Measurement bandwidth. 0 MHz f < 0.1*BWcontiguous; 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz; Min(−2.1 dBm, Max(Prated,t,TRP – 30.1 dB, −9.1 dBm)); 1 MHz. 0.1*BWcontiguous f < fB; 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz; Min(−13 dBm, Max(Prated,t,TRP – 41 dB, −20 dBm)); 1 MHz. fB f < fmax; fB +5 MHz f_offset < f_ offsetmax; Min(−5 dBm, Max(Prated,t,TRP – 31 dB, −10 dBm)); 10 MHz. NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz..

#### 4.1.2.3	Additional OTA operating band unwanted emission limits

The following additional requirement should apply:

–	For BS operating in the frequency range 24.25-27.5 GHz, for protection of Earth Exploration Satellite Service, the requirement in Table 6.7.4.5.2.4.1-1 of TS 38.141-2 [2].

Table 6.7.4.5.2.4.1-1: BS radiated limits for protection of EESS Frequency range; Measurement filter centre frequency range; Limit; Measurement Bandwidth. 23.6-24 GHz; 23.7-23.9 GHz; −3 dBm (Note 1); 200 MHz. 23.6-24 GHz; 23.7-23.9 GHz; −9 dBm (Note 2); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027..

## 4.2	OTA Adjacent channel leakage ratio (ACLR)

### 4.2.1	BS type 1-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1-1/2a of TS 38.141-2 [2] or the OTA ACLR absolute limits in Table 6.7.3.5.1-2 of TS 38.141-2 [2] should apply, whichever is less stringent.

Table 6.7.3.5.1-1: BS type 1-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0-3 GHz) | OTA ACLR limit (3-6 GHz) |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |

Table 6.7.3.5.1-2a: BS type 1-O ACLR limit in non-contiguous spectrum or multiple bands BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; OTA ACLR limit (0-3 GHz); OTA ACLR limit (3-6 GHz). 5, 10, 15, 20; Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4); 2.5 MHz; 5 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4); 7.5 MHz; 5 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3); 10 MHz; 20 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3); 30 MHz; 20 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz.. Table 6.7.3.5.1-2: BS type 1-O ACLR absolute limit BS category / BS class; OTA ACLR absolute limit. Category A Wide Area BS; −4 dBm/MHz. Category B Wide Area BS; −6 dBm/MHz. Medium Range BS; −16 dBm/MHz. Local Area BS; −23 dBm/MHz. NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void..

The assumed filter for the adjacent channel frequency is defined in Table 6.7.3.5.1-3 of TS 38.141-2 [2] and the filters on the assigned channels are defined in Table 6.7.3.5.1-4 of TS 38.141-2 [2].

Table 6.7.3.5.1-3: BS type 1-O CACLR limit BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; OTA CACLR limit (0-3 GHz); OTA CACLR limit (3-6 GHz). 5, 10, 15, 20; 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4); 2.5 MHz; 5 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4); 7.5 MHz; 5 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100; 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3); 10 MHz; 20 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3); 30 MHz; 20 MHz NR (Note 2); Square (BWConfig); 44 dB; 43.8 dB. NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz.. Table 6.7.3.5.1-4: Filter parameters for the assigned channel RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap; Filter on the assigned channel frequency and corresponding filter bandwidth. NR; NR of same BW with SCS that provides largest transmission bandwidth configuration.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result should not be less than the OTA ACLR limit specified in Table 6.7.3.5.1-1 of TS 38.141-2 [2].

The absolute total power measurement should not exceed the OTA ACLR absolute limit specified in Table 6.7.3.5.1-2 of TS 38.141-2 [2].

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result should not be less than the OTA ACLR limit specified in Table 6.7.3.5.1-2a of TS 38.141-2 [2].

### 4.2.2	BS type 2-O

For the OTA ACLR requirement either the OTA ACLR limits in Tables 6.7.3.5.2-1/3 of TS 38.141-2 [2] or the OTA ACLR absolute limits in Table 6.7.3.5.2-2 of TS 38.141-2 [2] should apply, whichever is less stringent.

Table 6.7.3.5.2-1: BS type 2-O ACLR limit BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz); BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted; Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; OTA ACLR limit (dB). 50, 100, 200, 400; BWChannel; NR of same BW (Note 2); Square (BWConfig); 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5). NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25-33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37-43.5 GHz NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5-48.2 GHz. Table 6.7.3.5.2-2: BS type 2-O ACLR absolute limit BS class; ACLR absolute limit (Note 1); ACLR absolute limit (Note 2). Wide-area BS; −10.3 dBm/MHz; −10.1 dBm/MHz. Medium-range BS; −17.3 dBm/MHz; −17.1 dBm/MHz. Local-area BS; −17.3 dBm/MHz; −17.1 dBm/MHz. NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25-43.5 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 43.5-48.2 GHz.

Table 6.7.3.5.2-3: BS type 2-O ACLR limit in non-contiguous spectrum BS channel bandwidth of lowest/highest NR carrier transmitted (MHz); Sub-block gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; OTA ACLR limit (MHz). 50, 100; Wgap ≥ 100 (Note 5) Wgap ≥ 250 (Note 6); 25 MHz; 50 MHz NR (Note 2); Square (BWConfig); 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 7). 200, 400; Wgap ≥ 400 (Note 6) Wgap ≥ 250 (Note 5); 100 MHz; 200 MHz NR (Note 2); Square (BWConfig); 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 7). NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24-33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37-43.5 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. NOTE 7: Applicable to bands defined within the frequency spectrum range of 43.5-48.2 GHz..

The assumed filter for the adjacent channel frequency is defined in Table 6.7.3.5.2-4 of TS 38.141-2 [2] and the filters on the assigned channels are defined in Table 6.7.3.5.2-5 of TS 38.141-2 [2].

Table 6.7.3.5.2-4: BS type 2-O CACLR limit in non-contiguous spectrum BS channel bandwidth of lowest/highest NR carrier transmitted (MHz); Sub-block gap size (Wgap) where the limit applies (MHz); BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap); Assumed adjacent channel carrier; Filter on the adjacent channel frequency and corresponding filter bandwidth; OTA CACLR limit (dB). 50, 100; 50 ≤ Wgap < 100 (Note 5) 50 ≤ Wgap < 250 (Note 6); 25 MHz; 50 MHz NR (Note 2); Square (BWConfig); 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 7). 200, 400; 200 ≤ Wgap < 400 (Note 6) 200 ≤ Wgap < 250 (Note 5); 100 MHz; 200 MHz NR (Note 2); Square (BWConfig); 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 7). NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24-33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37-43.5 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50 or 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200 or 400 MHz. NOTE 7: Applicable to bands defined within the frequency spectrum range of 43.5-48.2 GHz..

Table 6.7.3.5.2-5: Filter parameters for the assigned channel RAT of the carrier adjacent to the sub-block gap; Filter on the assigned channel frequency and corresponding filter bandwidth. NR; NR of same BW with SCS that provides largest transmission bandwidth configuration.

The OTA ACLR measurement result should not be less than the OTA ACLR limit specified in Table 6.7.3.5.2-1 of TS 38.141-2 [2].

The absolute total power measurement should not exceed the OTA ACLR absolute limit specified in Table 6.7.3.5.2-2 of TS 38.141-2 [2].

For operation in non-contiguous spectrum, the OTA ACLR measurement result should not be less than the OTA ACLR limit specified in Table 6.7.3.5.2-3 of TS 38.141-2 [2].

## 4.3	OTA Cumulative adjacent channel leakage ratio (CACLR)

### 4.3.1	BS type 1-O

The OTA CACLR limits in Table 6.7.3.5.1-3 of TS 38.141-2 [2] or the OTA CACLR absolute limits in Table 6.7.3.5.1-3a of TS 38.141-2 [2] should apply, whichever is less stringent.

Table 6.7.3.5.1-3a: BS type 1-O CACLR absolute limit BS category / BS class; OTA CACLR absolute limit. Category A Wide Area BS; −4 dBm/MHz. Category B Wide Area BS; −6 dBm/MHz. Medium Range BS; −16 dBm/MHz. Local Area BS; −23 dBm/MHz. NOTE 1: The test requirement is derived from the basic limit a scaling factor of 9 dB and any applicable TT. NOTE 2: Void.

The CACLR in a sub-block gap and Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in Table 6.7.3.5.1-3 of TS 38.141-2 [2] and the filters on the assigned channels are defined in Table 6.7.3.5.1-4 of TS 38.141-2 [2].

The OTA CACLR measurement result should not less than the OTA CACLR limit specified in Table 6.7.3.5.1-3 of TS 38.141-2 [2].

The absolute total power measurement should not exceed the OTA CACLR absolute limit specified in Table 6.7.3.5.1-3a of TS 38.141-2 [2].

### 4.3.2	BS type 2-O

The OTA CACLR limits in Table 6.7.3.5.2-4 of TS 38.141-2 [2] or the OTA CACLR absolute limits in Table 6.7.3.5.2-4a of TS 38.141-2 [2] should apply, whichever is less stringent.

Table 6.7.3.5.2-4a: BS type 2-O CACLR absolute limit BS class; CACLR absolute limit (Note 1); ACLR absolute limit (Note 2). Wide area BS; −10.3 dBm/MHz; −10.1 dBm/MHz. Medium range BS; −17.3 dBm/MHz; −17.1 dBm/MHz. Local area BS; −17.3 dBm/MHz; −17.1 dBm/MHz. NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25-43.5 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 43.5-48.2 GHz.

The CACLR in a sub-block gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges.

The assumed filter for the adjacent channel frequency is defined in Table 6.7.3.5.2-4 of TS 38.141-2 [2] and the filters on the assigned channels are defined in Table 6.7.3.5.2-5 of TS 38.141-2 [2].

For operation in non-contiguous spectrum, the CACLR for carriers located on either side of the sub-block gap should be less than the value specified in Table 6.7.3.5.2-4 of TS 38.141-2 [2].

The absolute total power measurement should not exceed the OTA CACLR absolute limit specified in Table 6.7.3.5.2-4a of TS 38.141-2 [2].

## 4.4	OTA Transmitter spurious emissions

### 4.4.1	OTA transmitter spurious emissions BS type 1-O

For Category A BS the TRP of any spurious emission should not exceed the limits in Table 6.7.5.2.5.1-1 in TS 38.141-2 [2]. For Category B BS the TRP of any spurious emission should not exceed the limits in Table 6.7.5.2.5.1-2 in TS 38.141-2 [2].

Table 6.7.5.2.5.1-1: General OTA BS transmitter spurious emission limits for BS type 1-O, Category A Spurious frequency range; Test limit; Measurement bandwidth; Notes. 30 MHz – 1 GHz; −13 + X dBm; 100 kHz; Note 1, Note 6. 1 GHz – 12.75 GHz; 1 MHz; Note 1, Note 2, Note 6. 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz; 1 MHz; Note 1, Note 2, Note 3, Note 6. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 4: Void. NOTE 5: Void. NOTE 6: X = 9 dB, unless stated differently in regional regulation.. Table 6.7.5.2.5.1-2: General OTA BS transmitter spurious emission limits for BS type 1-O, Category B Spurious frequency range; Test limit; Measurement bandwidth; Notes. 30 MHz – 1 GHz; −36 + X dBm; 100 kHz; Note 1, Note 5. 1 GHz – 12.75 GHz; −30 + X dBm; 1 MHz; Note 1, Note 2, Note 5. 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz; 1 MHz; Note 1, Note 2, Note 3, Note 5. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75GHz. NOTE 4: Void. NOTE 5: X = 9 dB, unless stated differently in regional regulation..

### 4.4.2	OTA transmitter spurious emissions BS type 2-O

For Category A BS the power of any spurious emission should not exceed the limits in Table 6.7.5.2.5.2.2-1 in TS 38.141-2 [2]. For Category B BS the power of any spurious emission should not exceed the limits in Table 6.7.5.2.5.2.3-1 in TS 38.141-2 [2].

Table 6.7.5.2.5.2.2-1: General OTA BS transmitter spurious emission limits for BS type 2-O Spurious frequency range; Test limit; Measurement bandwidth; Notes. 30 MHz – 1 GHz; −13 dBm; 100 kHz; Note 1. 1 GHz – min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz); 1 MHz; Note 1, Note 2. NOTE 1: Measurement bandwidth as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1..

Table 6.7.5.2.5.2.3-1: BS radiated Tx spurious emission limits in FR2 (Category B) Frequency range (Note 4); Test limit; Measurement Bandwidth; Note. 30 MHz 1 GHz; −36 dBm; 100 kHz; Note 1. 1 GHz 18 GHz; −30 dBm; 1 MHz; Note 1. 18 GHz Fstep,1; −20 dBm; 10 MHz; Note 2. Fstep,1 Fstep,2; −15 dBm; 10 MHz; Note 2. Fstep,2 Fstep,3; −10 dBm; 10 MHz; Note 2. Fstep,4 Fstep,5; −10 dBm; 10 MHz; Note 2. Fstep,5 Fstep,6; −15 dBm; 10 MHz; Note 2. Fstep,6 min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz); −20 dBm; 10 MHz; Note 2, Note 3. NOTE 1: Bandwidth as in ITU-R SM.329 [5], s4.1 NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [26], annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 4: The step frequencies Fstep,X are defined in Table 6.7.5.2.5.2.3-2..

### 4.4.3	Protection of the BS receiver of own or different BS

This requirement should be applied for NR FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a BS type 1-O.

This requirement is a co-location requirement as defined in clause 4.9, in TS 38.104 [6]. The power levels are specified at the CLTA output, as described in 4.12.2 of TS 38.141-2 [2].

## 4.9 OTA co-location with other base stations Co-location requirements are requirements which are based on assuming the BS type 1-O is co-located with another BS of the same base station class, they ensure that both co-located systems can operate with minimal degradation to each other. Unwanted emission and out of band blocking co-location requirements are optional requirements based on declaration. TX OFF and TX IMD are mandatory requirements and have the form of a co-location requirement as it represents the worst-case scenario of all the interference cases. NOTE: Due to the low level of the unwanted emissions for the spurious emissions and TX OFF level co-location is the most suitable method to show conformance. The co-location reference antenna shall be a single column passive antenna which has the same vertical radiating dimension (h), frequency range, polarization, as the composite antenna of the BS type 1-O and nominal 65° horizontal half-power beamwidth (suitable for 3-sector deployment) and is placed at a distance d from the edge of the BS type 1-O, as shown in Figure 4.9-1.

Figure 4.9-1: Illustration of BS type 1-O enclosure and co-location reference antenna Edge-to-edge separation d between the BS type 1-O and the co-location reference antenna shall be set to 0.1 m. The BS type 1-O and the co-location reference antenna shall be aligned in a common plane perpendicular to the mechanical bore-sight direction, as shown in Figure 4.9-1. The co-location reference antenna and the BS type 1-O can have different width. The vertical radiating regions of the co-location reference antenna and the BS type 1-O composite antenna shall be aligned. For co-location requirements where the frequency range of the signal at the co-location reference antenna is different from the BS type 1-O, a co-location reference antenna suitable for the frequency stated in the requirement is assumed. OTA co-location requirements are based on the power at the conducted interface of a co-location reference antenna, depending on the requirement this interface is either an input or an output. For BS type 1-O with dual polarization the co-location reference antenna has two conducted interfaces each representing one polarization. ### 4.12.2 Co-location test antenna #### 4.12.2.1 General Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA). A CLTA is a practical antenna which can be used to test conformance to the co-location requirements. #### 4.12.2.2 Co-location test antenna characteristics A co-location test antenna is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. A CLTA shall comply with the requirements specified in Table 4.12.2.2-1. Translation of the requirements to other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified MU. NOTE: The currently defined CLTAs are suitable for testing BS type 1-O implemented with a planar antenna array. The method for testing BS with other antenna array implementations is not covered by the present release of this specification. Table 4.12.2.2-1: CLTA characteristics Parameter; In-band CLTA; Out-of-band CLTAs. Vertical radiating dimension (h); Test object vertical radiating length ±30%; Test object vertical radiating length ±30% (Note 2). Horizontal beam width; 65° ± 10°; 65° ± 10°. Vertical beam width; N/A; The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2). Polarization (Note 3); Match (Note 4); Match to in-band (Note 4). Conducted interface return loss; > 10 dB; > 10 dB. NOTE 1: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. NOTE 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise the vertical beam width definition shall be used. NOTE 3: For BS type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization. NOTE 4: Matched to the polarization of EUT antenna.. #### 4.12.2.3 Co-location test antenna alignment The alignment between the NR BS under test and the co-location test antenna is described in Table 4.12.2.3-1 and Figure 4.12.2.3-1. The same physical alignment applies to in-band and out-of-band co-location requirements. Table 4.12.2.3-1: CLTA alignment tolerances Parameter. Edge-to-edge separation between the NR BS and the CLTA, d; 0.1 m ± 0.01 m. Vertical alignment; Centre ± 0.01 m. Front alignment; Radome front ± 0.01 m.

Figure 4.12.2.3-1: Alignment of NR BS and CLTA

The total power of any spurious emission from both polarizations of the CLTA connector output should not exceed the limits in Table 6.7.5.3.5.1-1 in TS 38.141-2 [2].

Table 6.7.5.3.5.1-1: BS type 1-O OTA spurious emissions limits for protection of the BS receiver BS class; Frequency range; Maximum Level for bands below 3GHz; Maximum Level for bands between 3 and 4.2GHz; Maximum Level for bands between 4.2 and 6GHz; Measurement bandwidth. Wide Area BS; FUL_low – FUL_high; −113.9 dBm; −113.7 dBm; −113.6 dBm; 100 kHz. Medium Range BS; −108.9 dBm; −108.7 dBm; −108.6 dBm. Local Area BS; −105.9 dBm; −105.7 dBm; −105.6 dBm.

### 4.4.4	Additional spurious emissions requirements

In certain regions the following additional requirements in TS 38.141-2 [2] may apply:

For BS co-existence with systems operating in other frequency bands, the requirement in Table 6.7.5.4.5-1.

For protection of PHS, the requirement in Table 6.7.5.4.5-2

For BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band n51 and Band n76, the requirement in Table 6.7.5.4.5-3.

For BS operating in NR Band n50 and n75 within 1492-1517 MHz, and in Band n74 within 1492-1518 MHz, the requirements in Table 6.7.5.4.5-4.

For BS operating in Band n13 and n14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations, the requirements in Table 6.7.5.4.5-5.

For BS operating in Band n30, the requirements in Table 6.7.5.4.5-6.

For BS operating in Band n26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations, the requirements in Table 6.7.5.4.5-7.

Table 6.7.5.4.5-1: BS spurious emissions test limits for BS for co-existence with systems operating in other frequency bands System type for NR to co-exist with; Frequency range for co-existence requirement; Test limit; Measurement bandwidth; Notes. GSM900; 921-960 MHz; −45.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n8.. 876-915 MHz; −49.4 dBm; 100 kHz; For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.7.5.3.. DCS1800; 1 805-1 880 MHz; −35.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n3.. 1 710-1 785 MHz; −49.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.7.5.3.. PCS1900; 1 930-1 990 MHz; −35.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n2, n25 or band n70.. 1 850-1 910 MHz; −49.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n2 or n25 since it is already covered by the requirement in clause 6.7.5.3.. GSM850 or CDMA850; 869-894 MHz; −45.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n5 or n26.. 824-849 MHz; −49.4 dBm; 100 kHz; This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band I or; 2 110-2 170 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n1 or n65.. E-UTRA Band 1 or NR Band n1; 1 920-1 980 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band II or; 1 930-1 990 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n2 or n70.. E-UTRA Band 2 or NR Band n2; 1 850-1 910 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band III or; 1 805-1 880 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n3.. E-UTRA Band 3 or NR Band n3; 1 710-1 785 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band IV or; 2 110-2 155 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66.. E-UTRA Band 4; 1 710-1 755 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band V or; 869-894 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n5 or n26.. E-UTRA Band 5 or NR Band n5; 824-849 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n5 or n26, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band VI, XIX; 860-890 MHz; −40.4 dBm; 1 MHz. or E-UTRA Band 6, 18,; 815-830 MHz; −37.4 dBm; 1 MHz. 19; 830-845 MHz; −37.4 dBm; 1 MHz. UTRA FDD Band VII or; 2 620-2 690 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n7.. E-UTRA Band 7 or NR Band n7; 2 500-2 570 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n7, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band VIII or; 925-960 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n8.. E-UTRA Band 8 or NR Band n8; 880-915 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band IX or; 1 844.9-1 879.9 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n3.. E-UTRA Band 9; 1 749.9-1 784.9 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band X or; 2 110-2 170 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66. E-UTRA Band 10; 1 710-1 770 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band XI or XXI or; 1 475.9-1 510.9 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74 or n75.. E-UTRA Band 11 or; 1 427.9-1 447.9 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. 21; 1 447.9-1 462.9 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74 or n75.. UTRA FDD Band XII or; 729-746 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85.. E-UTRA Band 12 or NR Band n12; 699-716 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.7.5.3. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5).. UTRA FDD Band XIII or; 746-756 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n13.. E-UTRA Band 13 or NR Band n13; 777-787 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n13, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band XIV or; 758-768 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n14.. E-UTRA Band 14 or NR Band n14; 788-798 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n14, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 17; 734-746 MHz; −40.4 dBm; 1 MHz. 704-716 MHz; −37.4 dBm; 1 MHz; For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5).. UTRA FDD Band XX or; 791-821 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n20 or n28.. E-UTRA Band 20 or NR Band n20; 832-862 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.7.5.3.. UTRA FDD Band XXII or; 3 510-3 590 MHz; −40 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78.. E-UTRA Band 22; 3 410-3 490 MHz; −37 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78.. E-UTRA Band 24 or NR Band n24; 1 525-1 559 MHz; −40.4 dBm; 1 MHz. 1 626.5-1 660.5 MHz; −37.4 dBm; 1 MHz. UTRA FDD Band XXV or; 1 930-1 995 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n2, n25 or n70.. E−UTRA Band 25 or NR band n25; 1 850-1 915 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n25 since it is already covered by the requirement in clause 6.7.5.3. For BS operating in Band n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in clause 6.7.5.3.. UTRA FDD Band XXVI or; 859-894 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n5 or n26.. E-UTRA Band 26 or NR Band n26; 814-849 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n26 since it is already covered by the requirement in clause 6.7.5.3. For BS operating in Band n5, it applies for 814 MHz to 824 MHz, while the rest is covered in clause 6.7.5.3.. E-UTRA Band 27; 852-869 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n5.. 807-824 MHz; −37.4 dBm; 1 MHz; This requirement also applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band (Note 5).. E-UTRA Band 28 or; 758-803 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, n67 or n28.. NR Band n28; 703-748 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.7.5.3. For BS operating in band n67, it applies for 703 MHz to 736 MHz.. E-UTRA Band 29 or NR Band n29; 717-728 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n29 or n85.. E-UTRA Band 30 or; 2 350-2 360 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n30.. NR Band n30; 2 305-2 315 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n30, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 31; 462.5-467.5 MHz; −40.4 dBm; 1 MHz. 452.5-457.5 MHz; −37.4 dBm; 1 MHz. UTRA FDD band XXXII or E-UTRA band 32; 1 452-1 496 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74 or n75.. UTRA TDD Band a) or E-UTRA Band 33; 1 900-1 920 MHz; −40.4 dBm; 1 MHz. UTRA TDD Band a) or E-UTRA Band 34 or NR band n34; 2 010-2 025 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n34.. UTRA TDD Band b) or E-UTRA Band 35; 1 850-1 910 MHz; −40.4 dBm; 1 MHz. UTRA TDD Band b) or E-UTRA Band 36; 1 930-1 990 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n2 or n25.. UTRA TDD Band c) or E-UTRA Band 37; 1 910-1 930 MHz; −40.4 dBm; 1 MHz. UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38; 2 570-2 620 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n38.. UTRA TDD Band f) or E-UTRA Band 39 or NR band n39; 1 880-1 920 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n39.. UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40; 2 300-2 400 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Bands n30 or n40.. E-UTRA Band 41 or NR Band n41; 2 496-2 690 MHz; −40.4 dBm; 1 MHz; This is not applicable to BS operating in Band n41.. E-UTRA Band 42; 3 400-3 600 MHz; −40 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78.. E-UTRA Band 43; 3 600-3 800 MHz; −40 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78.. E-UTRA Band 44; 703-803 MHz; −40.4 dBm; 1 MHz; This is not applicable to BS operating in Band n28.. E-UTRA Band 45; 1 447-1 467 MHz; −40.4 dBm; 1 MHz. E-UTRA Band 47; 5 855-5 925 MHz; −39.5 dBm; 1 MHz. E-UTRA Band 48; 3 550-3 700 MHz; −40 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78.. E-UTRA Band 50 or NR Band n50; 1 432-1 517 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. E-UTRA Band 51 or NR Band n51; 1 427-1 432 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75 or n76.. E-UTRA Band 53 or NR Band n53; 2 483.5-2 495 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n41 or n90.. E-UTRA Band 65 or; 2 110-2 200 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n1 or n65.. NR Band n65; 1 920-2 010 MHz; −37.4 dBm; 1 MHz; For BS operating in Band n1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in clause 6.7.5.3. This requirement does not apply to BS operating in band n65, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 66 or; 2 110-2 200 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66.. NR Band n66; 1 710-1 780 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 67 or NR Band n67; 738-758 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n28 or n67.. E-UTRA Band 68; 753-783 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n28.. 698-728 MHz; −37.4 dBm; 1 MHz; For BS operating in Band n28, this requirement applies between 698 MHz and 703 MHz, while the rest is covered in clause 6.7.5.3.. E-UTRA Band 69; 2 570-2 620 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n38.. E-UTRA Band 70 or; 1 995-2 020 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n2, n25 or n70. NR Band n70; 1 695-1 710 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n70, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 71 or; 617-652 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n71. NR Band n71; 663-698 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n71, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 72; 461-466 MHz; −40.4 dBm; 1 MHz. 451-456 MHz; −37.4 dBm; 1 MHz. E-UTRA Band 74 or; 1 475-1 518 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n74 or n75.. NR Band n74; 1 427-1 470 MHz; −37.4 dBm; 1MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. E-UTRA Band 75 or NR Band n75; 1 432-1 517 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. E-UTRA Band 76 or NR Band n76; 1 427-1 432 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75 or n76.. NR Band n77; 3.3-4.2 GHz; −40 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78. NR Band n78; 3.3-3.8 GHz; −40 dBm; 1 MHz; This requirement does not apply to BS operating in Band n77 or n78. NR Band n79; 4.4-5.0 GHz; −39.5 dBm; 1 MHz; This requirement does not apply to BS operating in Band n79. NR Band n80; 1 710-1 785 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n81; 880-915 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n82; 832-862 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n83; 703-748 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n28, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n84; 1 920-1 980 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n1, since it is already covered by the requirement in clause 6.7.5.3.. E-UTRA Band 85 or NR Band n85; 728-746 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85.. 698-716 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n12 or n85, since it is already covered by the requirement in clause 6.7.5.3. For NR BS operating in n29, it applies 1 MHz below the Band n29 downlink operating band (Note 5).. NR Band n86; 1 710-1 780 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n66, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n89; 824-849 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n5, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n91; 1 427-1 432 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75 or n76.. 832-862 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n92; 1 432-1 517 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. 832-862 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n20, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n93; 1 427-1 432 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n75 or n76.. 880-915 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n94; 1 432-1 517 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n50, n51, n74, n75 or n76.. 880-915 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n95; 2 010-2 025 MHz; −40.4 dBm; 1 MHz. NR Band n96; 5 925-7 125 MHz; −39.5 dBm; 1 MHz; This requirement does not apply to BS operating in Band n96 or n102.. NR Band n97; 2 300-2 400 MHz; −40.4 dBm; 1 MHz; NR Band n97. NR Band n98; 1 880-1 920 MHz; −40.4 dBm; 1 MHz. NR Band n99; 1 626.5-1 660.5 MHz; −37.4 dBm; 1 MHz; This requirement does not apply to BS operating in band n24, since it is already covered by the requirement in clause 6.7.5.3.. NR Band n100; 919.4-925 MHz; −40.4 dBm; 1 MHz; This requirement does not apply to BS operating in Band n8.. 874.4-880 MHz; −37.4 dBm; 1MHz. NR Band n101; 1 900-1 910 MHz; −40.4 dBm; 1 MHz. NR Band n102; 6 425-7 125 MHz; −39.5 dBm; 1 MHz; This requirement does not apply to BS operating in Band n96 or n102.. E-UTRA Band 103; 757-758 MHz; −40.4 dBm; 1 MHz. 787-788 MHz; −37.4 dBm; 1 MHz. NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in 6.7.5.4.5-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see TS 38.104 [2], Table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 2: Table 6.7.5.4.5-1 assumes that two operating bands, where the frequency ranges in TS 38.104 [2] Table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 3: TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 4: For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band. NOTE 5: For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band. Table 6.7.5.4.5-2: BS spurious emissions test limits for BS for co-existence with PHS Frequency range; Test limit; Measurement bandwidth; Note. 1 884.5-1 915.7 MHz; −32 dBm; 300 kHz; Applicable when co-existence with PHS system operating in 1 884.5-1 915.7 MHz. Table 6.7.5.4.5-3: Additional emission test limit for BS operating in Band n50 and n75 within 1432-1452 MHz, and in Band n51 and n76 Filter centre frequency, Ffilter; Test limit (dBm); Measurement bandwidth. Ffilter = 1 413.5 MHz; −42; 27 MHz. Table 6.7.5.4.5-4: Operating band n50, n74 and n75 emission test limits above 1518 MHz Filter centre frequency, Ffilter; EIRP limit (dBm); Measurement bandwidth. 1 518.5 MHz ≤ Ffilter ≤ 1 519.5 MHz; −0.8; 1 MHz. 1 520.5 MHz ≤ Ffilter ≤ 1 558.5 MHz; −30; 1 MHz. Table 6.7.5.4.5-5: BS Spurious emissions limits for protection of 700 MHz public safety operations Operating Band; Frequency range; Maximum Level; Measurement Bandwidth. n13; 763-775 MHz; −37 dBm; 6.25 kHz. n13; 793-805 MHz; −37 dBm; 6.25 kHz. n14; 769-775 MHz; −37 dBm; 6.25 kHz. n14; 799-805 MHz; −37 dBm; 6.25 kHz. Table 6.7.5.4.5-6: Additional NR BS Spurious emissions limits for Band n30 Frequency range; Basic limit; Measurement bandwidth. 2 200-2 345 MHz; −33.4 dBm; 1 MHz. 2 362.5-2 365 MHz; −13.4 dBm. 2 365-2 367.5 MHz; −28.4 dBm. 2 367.5-2 370 MHz; −30.4 dBm. 2 370-2 395 MHz; −33.4 dBm. Table 6.7.5.4.5-7: BS OTA Spurious emissions limits for protection of 800 MHz public safety operations Operating Band; Frequency range; Maximum Level; Measurement Bandwidth; Note. n26; 851-859 MHz; −4 dBm; 100 kHz; Applicable for offsets > 37.5 kHz from the channel edge. Table 6.7.5.4.5-8: Additional BS Spurious emissions limits for Band n41 and n90 Frequency range; Test limit; Measurement Bandwidth. 2 505-2 535 MHz; −33 dBm; 1 MHz. NOTE: This requirement applies for carriers allocated within 2 545-2 645 MHz..

The following additional requirement should apply:

For BS operating in the frequency range 24.25-27.5 GHz, for protection of Earth Exploration Satellite Service, the requirements in Table 6.7.5.4.5.2-1 of TS 38.141-2 [2].

Table 6.7.5.4.5.2-1: BS spurious emissions test limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6-24 GHz; −3 dBm; 200 MHz; Note 1. 23.6-24 GHz; −9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027..

### 4.4.5	Co-location requirements

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS.

The requirements assume co-location with base stations of the same class.

NOTE: For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in clause 4.9, in TS 38.104 [6]. The power levels are specified at the CLTA output, as described in 4.12.2 of TS 38.141-2 [2].

The output of the CLTA of any spurious emission should not exceed the test limit in Table 6.7.5.5.5.1-1 in TS 38.141-2 [2].

For a multi-band RIB, the exclusions and conditions in the note column of Table 6.7.5.5.5.1-1 apply for each supported operating band.

Table 6.7.5.5.5.1-1: BS type 1-O OTA spurious emissions limits for BS co-located with another BS Type of co-located BS; Frequency range for; Test limit; Measurement; Note. co-location requirement; WA BS; MR BS; LA BS; bandwidth. GSM900; 876-915 MHz; −115.9 dBm; −108.9 dBm; −87.9 dBm; 100 kHz. DCS1800; 1 710-1 785 MHz; −115.9 dBm; −108.9 dBm; −97.9 dBm; 100 kHz. PCS1900; 1 850-1 910 MHz; −115.9 dBm; −108.9 dBm; −97.9 dBm; 100 kHz. GSM850 or CDMA850; 824-849 MHz; −115.9 dBm; −108.9 dBm; −87.9 dBm; 100 kHz. UTRA FDD Band I or E-UTRA Band 1 or NR Band n1; 1 920-1 980 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band II or E-UTRA Band 2 or NR Band n2; 1 850-1 910 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band III or E-UTRA Band 3 or NR Band n3; 1 710-1 785 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band IV or E-UTRA Band 4; 1 710-1 755 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band V or E-UTRA Band 5 or NR Band n5; 824-849 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band VI, XIX or E-UTRA Band 6, 19; 830-845 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7; 2 500-2 570 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8; 880-915 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band IX or E-UTRA Band 9; 1 749.9-1 784.9 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band X or E-UTRA Band 10; 1 710-1 770 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XI or E-UTRA Band 11; 1 427.9-1 447.9 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n50 or n75. UTRA FDD Band XII or E-UTRA Band 12; 699-716 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13; 777-787 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14; 788-798 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 17; 704-716 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 18; 815-830 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20; 832-862 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XXI or E-UTRA Band 21; 1 447.9-1 462.9 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n50 or n75. UTRA FDD Band XXII or E-UTRA Band 22; 3 410-3 490 MHz; −113.7 dBm; −108.7 dBm; −105.7 dBm; 100 kHz; This is not applicable to BS operating in Band n77 or n78. E-UTRA Band 24 or NR Band n24; 1 626.5-1 660.5 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XXV or E-UTRA Band 25; 1 850-1 915 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA FDD Band XXVI or E-UTRA Band 26 or NR Band n26; 814-849 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 27; 807-824 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 28 or NR Band n28; 703-748 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 30 or NR Band n30; 2 305-2 315 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 31; 452.5-457.5 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA TDD Band a) or E-UTRA Band 33; 1 900-1 920 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA TDD Band a) or E-UTRA Band 34; 2 010-2 025 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA TDD Band b) or E-UTRA Band 35; 1 850-1 910 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA TDD Band b) or E-UTRA Band 36; 1 930-1 990 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n2. UTRA TDD Band c) or E-UTRA Band 37; 1 910-1 930 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38; 2 570-2 620 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n38.. UTRA TDD Band f) or E-UTRA Band 39; 1 880-1 920 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. UTRA TDD Band e) or E-UTRA Band 40; 2 300-2 400 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 41 or NR Band n41; 2 496-2 690 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n41. E-UTRA Band 42; 3 400-3 600 MHz; −113.7 dBm; −108.7 dBm; −105.7 dBm; 100 kHz; This is not applicable to BS operating in Band n77 or n78. E-UTRA Band 43; 3 600-3 800 MHz; −113.7 dBm; −108.7 dBm; −105.7 dBm; 100 kHz; This is not applicable to BS operating in Band n77 or n78. E-UTRA Band 44; 703-803 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n28. E-UTRA Band 45; 1 447-1 467 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 48; 3 550-3 700 MHz; −113.7 dBm; −108.7 dBm; −105.7 dBm; 100 kHz; This is not applicable to BS operating in Band n77 or n78. E-UTRA Band 50 or NR Band n50; 1 432-1 517 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n74 or n75. E-UTRA Band 51 or NR Band n51; 1 427-1 432 MHz; N/A; N/A; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n50, n75 or n76. E-UTRA Band 53 or NR Band n53; 2 483.5-2 495 MHz; N/A; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n41 or n90. E-UTRA Band 65 or NR Band n65; 1 920-2 010 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 66 or NR Band n66; 1 710-1 780 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 68; 698-728 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 70 or NR Band n70; 1 695-1 710 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 71 or NR Band n71; 663-698 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 72; 451-456 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 74 or NR Band n74; 1 427-1 470 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz; This is not applicable to BS operating in Band n50. NR Band n77; 3.3-4.2 GHz; −113.7 dBm; −108.7 dBm; −105.7 dBm; 100 kHz; This is not applicable to BS operating in Band n77 or n78. NR Band n78; 3.3-3.8 GHz; −113.7 dBm; −108.7 dBm; −105.7 dBm; 100 kHz; This is not applicable to BS operating in Band n77 or n78. NR Band n79; 4.4-5.0 GHz; −113.6 dBm; −108.6 dBm; −105.6 dBm; 100 kHz. NR Band n80; 1 710-1 785 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n81; 880-915 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n82; 832-862 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n83; 703-748 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n84; 1 920-1 980 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. E-UTRA Band 85 or NR Band n85; 698-716 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n86; 1 710-1 780 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n89; 824-849 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n95; 2 010-2 025 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band 96; 5 925-7 125 MHz; N/A; −107.6 dBm; −104.6 dBm; 100 kHz; This is not applicable to BS operating in Band n96 or n102. NR Band n97; 2 300-2 400 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n98; 1 880-1 920 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR Band n99; 1 626.5-1 660.5 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NR band n100; 874.4-880 MHz; −113.9 dBm; NA; NA; 100 kHz. NR band n101; 1 900-1 910 MHz; −113.9 dBm; NA; NA; 100 kHz. NR Band n102; 6 425-7 125 MHz; N/A; −107.6 dBm; −104.6 dBm; 100 kHz; This is not applicable to BS operating in Band n96 or n102. E-UTRA Band 103; 787-788 MHz; −113.9 dBm; −108.9 dBm; −105.9 dBm; 100 kHz. NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in Table 6.7.5.5.5.1-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see Table 5.2-1 in TS 38.104 [2]). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [27]. NOTE 2: Table 6.7.5.5.5.1-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in Table 5.2-1 in TS 38.104 [2] would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 3: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base station), special co-location requirements may apply that are not covered by the 3GPP specifications.

## 4.5	OTA receiver spurious emissions

### 4.5.1	OTA receiver spurious emissions BS type 1-O

The receiver spurious emissions for BS type 1-O at RIB interface should be below the applicable limits defined in Table 7.7.5.1-1 of TS 38.141-2 [2].

Table 7.7.5.1-1: General OTA BS receiver spurious emission limits for BS type 1-O Spurious frequency range; Test limits (Note 6, Note 8); Measurement bandwidth; Notes. 30 MHz – 1 GHz; −36 + X dBm; 100 kHz; Note 1, Note 6. 1 GHz – 6 GHz; −30 + X dBm; 1 MHz; Note 1, Note 2, Note 6. 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz; −30 + X dBm; 1 MHz; Note 1, Note 2, Note 3, Note 6. NOTE 1: Measurement bandwidths as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 Table 1. NOTE 3: This spurious frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the UL operating band is reaching beyond 12.75 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.7.1. For multi-band RIBs, the exclusion applies for all supported operating bands. NOTE 5: Void. NOTE 6: X = 9 dB, unless stated differently in regional regulation. NOTE 7: Void. NOTE 8: Additional limits may apply regionally..

### 4.5.2	OTA receiver spurious emissions BS type 2-O

The receiver spurious emissions for BS type 2-O at RIB interface should be below the applicable limits defined in Table 7.7.5.2-1 of TS 38.141-2 [2], with the step frequencies defined in Table 7.7.5.2-2 of TS 38.141-2 [2]. In addition, the requirement to protect Earth Exploration Satellite Service should apply for BS operating in the frequency range 24.25-27.5 GHz, as defined in Table 7.7.5.2-3 of TS 38.141-2 [2].

Table 7.7.5.2-1: Radiated Rx spurious emission limits for BS type 2-O Spurious frequency range (Note 4); Limit (Note 5); Measurement Bandwidth; Note. 30 MHz 1 GHz; −36 dBm; 100 kHz; Note 1. 1 GHz 18 GHz; −30 dBm; 1 MHz; Note 1. 18 GHz Fstep,1; −20 dBm; 10 MHz; Note 2. Fstep,1 Fstep,2; −15 dBm; 10 MHz; Note 2. Fstep,2 Fstep,3; −10 dBm; 10 MHz; Note 2. Fstep,4 Fstep,5; −10 dBm; 10 MHz; Note 2. Fstep,5 Fstep,6; −15 dBm; 10 MHz; Note 2. Fstep,6 min(2nd harmonic of the upper frequency edge of the UL operating band in GHz; 60 GHz); −20 dBm; 10 MHz; Note 2, Note 3. NOTE 1: Bandwidth as in ITU-R SM.329 [2], s4.1. NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [19], Annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [2], s2.5 Table 1. NOTE 4: The step frequencies Fstep,X are defined in Table 7.7.5.2-2. NOTE 5: Additional limits may apply regionally.. Table 7.7.5.2-2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O Operating band; Fstep,1 (GHz); Fstep,2 (GHz); Fstep,3 (GHz); Fstep,4 (GHz); Fstep,5 (GHz); Fstep,6 (GHz). n257; 18; 23.5; 25; 31; 32.5; 41.5. n258; 18; 21; 22.75; 29; 30.75; 40.5. n259; 23.5; 35.5; 38; 45; 47.5; 59.5. n260; 25; 34; 35.5; 41.5; 43; 52. n261; 18; 25.5; 26.0; 29.85; 30.35; 38.35. n262; 37.2; 45.2; 45.7; 49.7; 50.2; 58.2. Table 7.7.5.2-3: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; −3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; −9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027..

# 5	References

[1]	ARIB STD-T120-38.141-1 V17.6.0, ETSI TS 138 141-1 V17.6.0 or TTA TTAT.3G-38.141-1V17.6.0) these standards refer to 3GPP TS 38.141-1 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing (Release 17)”.

[2]	ARIB STD-T120-38.141-2 V.17.6.0,  ETSI TS 138 141-2 V17.6.0 or TTA TTAT.3G-38.141-2V17.6.0 these standards refer to 3GPP TS 38.141-2 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing (Release 17)”.

[3]	Recommendation ITU-R SM.329: “Unwanted emissions in the spurious domain”.

[4]	Recommendation ITU-R M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000”.

[5]	ARIB STD-T120-38.104 V17.6.0, ETSI TS 138 104-2 V17.6.0 or TTAT.3G-38.104V17.6.0 these standards refer to 3GPP TS 38.104 V17.6.0 (2022-06), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 17)”.

Attachment 1 
to Annex 2

Definition of test tolerance

Test tolerance

With reference to Recommendation ITU-R M.1545 [4], “test tolerance” is the relaxation value referred to in recommends 2 of Recommendation ITU-R M.1545, i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per Figure 2 and Figure 3 of Annex 1 of Recommendation ITU-R M.1545. In case the core specification value is equal to the test limit (Figure 3 of Annex 2 of Recommendation ITU-R M.1545) the “test tolerances” are equal to 0 dB.

Annex 3

5Gi RIT

The unwanted emission characteristics in Annex 2 should be applied to 5Gi RIT.

Annex 4

DECT-2020 NR-RIT Component

Introduction

The DECT 5G-SRIT consists of two components:

•	DECT-2020 NR-RIT Component

•	3GPP NR-RIT Component

This Annex provides the unwanted emission characteristics of base stations (radio equipment in FT-mode) using the DECT-2020 NR radio interface.

The unwanted emission characteristics of base stations using the 3GPP NR radio interface are provided in Annex 2 to this Recommendation.

# 1	Unwanted emission characteristics for DECT-2020 NR

## 1.1	Operating bands

Operating band numbering is defined in Tables A4-1 and A4-2. Radio device may implement one or more band support depending on its capabilities.

TABLE A4-1

Operating band numbering

Frequency bands utilized by DECT-2020 NR and identified for IMT in the RR

| Band number | Receiving band (MHz) | Transmitting band (MHz) | Footnote(s) Identifying the band, or part thereof, for IMT in various countries/Regions |
| --- | --- | --- | --- |
| 1 | 1 880 to 1 900 | 1 880 to 1 900 | 5.388 |
| 2 | 1 900 to 1 920 | 1 900 to 1 920 | 5.388 |
| 5 | 450 to 470 | 450 to 470 | 5.286AA |
| 6 | 698 to 806 | 698 to 806 | 5.312B 5.317A |
| 7 | 716 to 728 | 716 to 728 | 5.312B 5.317A |
| 8 | 1 432 to 1 517 | 1 432 to 1 517 | 5.341A, 5.341B, 5.341C |
| 9 | 1 920 to 1 930 | 1 920 to 1 930 | 5.388 |
| 10 | 2 010 to 2 025 | 2 010 to 2 025 | 5.388 |
| 11 | 2 300 to 2 400 | 2 300 to 2 400 | 5.384A |
| 12 | 2 500 to 2 620 | 2 500 to 2 620 | 5.384A |
| 13 | 3 300 to 3 400 | 3 300 to 3 400 | 5.429G |
| 14 | 3 400 to 3 600 | 3 400 to 3 600 | 5.431A 5.431B |
| 15 | 3 600 to 3 700 | 3 600 to 3 700 | 5.434 |
| 16 | 4 800 to 4 990 | 4 800 to 4 990 | 5.440A 5.441A 5.441B |
| 21 | 3 700 to 3 800 | 3 700 to 3 800 | 5.435B |
| 22 | 1 910 to 1 930 | 1 910 to 1 930 | 5.388 |

TABLE A4-2

Operating band numbering

Frequency bands utilized by DECT-2020 NR and not identified for IMT in the RR

| Band number | Receiving band (MHz) | Transmitting band (MHz) |
| --- | --- | --- |
| 3 | 2 400 to 2 483,5 | 2 400 to 2 483,5 |
| 4 | 902 to 928 | 902 to 928 |
| 17 | 5 725 to 5 875 | 5 725 to 5 875 |
| 18 | 5 150 to 5 350 | 5 150 to 5 350 |
| 19 | 5 470 to 5 725 | 5 470 to 5 725 |
| 20 | 3 800 to 4 200 | 3 800 to 4 200 |

## 1.2	Definitions, symbols and abbreviations

### 1.2.1	Symbols

β	Fourier transform scaling factor [2]

μ	Subcarrier scaling factor [2]

Δfoob	Δ Frequency of the Out of Band emission [1]

BN	Nominal channel bandwidth

BG	Difference between Nominal channel bandwidth and transmission bandwidth

Fc	Carrier centre frequency

Pmax	Maximum transmission power of the radio device power class

### 1.2.2	Abbreviations

ACLR	Adjacent channel leakage ratio

DFT	Discrete fourier transform

MBW	Measurement BANDWIDTH

# 2	Transmitter spectrum emission characteristics for DECT-2020

## 2.1	General

Radio equipment transmitter spectrum emissions are occupied channel emissions, out of band and spurious emissions. The relation of these emission components is illustrated in Fig. A4-1.

The transmitter characteristics are specified at the antenna connector(s) of the radio equipment. For radio equipment with an integral antenna only, a reference antenna(s) with a gain of 0 dBi should be assumed for each antenna port(s).

Power values are applicable for operation without AAS.

Figure A4-1

Transmitter RF spectrum

## 2.2	Occupied channel BW

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to 0.5% of the total mean transmitted power.

The occupied bandwidth should be less than the nominal channel bandwidth as defined in Table A4-3.

TABLE A4-3

Channel bandwidth

| Parameter | Operating channel bandwidth I | Operating channel bandwidth II | Operating channel bandwidth III |  |  |
| --- | --- | --- | --- | --- | --- |
| Nominal channel bandwidth (MHz) | 1 728 | 3 456 | 6 912 |  |  |
| (μ,β) | (1,1) | (1,2) | (2,1) | (1,4) | (2,2) |
| Transmission channel bandwidth (MHz) | 1 539 | 3 051 | 6 075 | 6 102 | 6 156 |

## 2.3	Maximum output power

The maximum output power (Pmax) is defined as the mean power of the transmitted packet. The maximum transmitter output power is defined in Table A4-4. Radio device may use one or more power classes in its operation.

TABLE A4-4

Maximum output power

| Radio device power class | Operating channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- |
|  | 1 728 | 3 456 | 6 912 |
|  | Output power (dBm) |  |  |
| Class I | 23 | 23 | 23 |
| Class II | 21 | 21 | 21 |
| Class III | 19 | 19 | 19 |
| Class IV | 10 | 10 | 10 |
| NOTE 1: The measurement bandwidth equals to the transmission bandwidth of the operating channel bandwidth defined in Table A4-3. |  |  |  |

## 2.4	Out of band emissions

The out-of-band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

The spectrum emission mask of the radio device (RD) applies to frequencies (ΔfOOB) starting from the  edge (from Fc+BN/2 or from Fc-BN/2) of the assigned channel. For frequencies offset greater than ΔfOOB as specified in Table A4-5, the spurious requirements in § 2.5 are applicable.

TABLE A4-5

Spectrum emission limits 30 kHz measurement bandwidth

| Δfoob |  | Limit (dBm) | Measurement Bandwidth (kHz) |
| --- | --- | --- | --- |
| BG/2 ≥ 1 MHz | BG/2 < 1 MHz |  |  |
| - | 0 to BG/2 | -10-10 log10(BN/1,728) | 30 |
| - | -0 to -BG/2 | -10-10 log10(BN/1,728) | 30 |
| 0 to 1 MHz | BG/2 to 1 MHz | -21-10 log10(BN/1,728) | 30 |
| 0 to -1 MHz | -BG/2 to -1 MHz | -21-10 log10(BN/1,728) | 30 |
| NOTE 1: The first center frequency for a 30 kHz measurement filter is at Δfoob= 0,015 MHz, which is BN/2 + 0,015MHz from the carrier center frequency. NOTE 2: The first center frequency for a 30 kHz measurement filter in the range from BG/2 to 1 MHz is at Δfoob = BG/2+0,015 MHz, which is BN/2+ BG/2+0,015 MHz from the carrier center frequency. NOTE 3: Symmetrically similarly as in Note 1 and Note 2 in negative Δfoob frequencies. |  |  |  |

TABLE A4-6

Spectrum emission limits 1 MHz measurement

| Δfoob | Limit (dbm) |  | Measurement BandWidth (MBW) |
| --- | --- | --- | --- |
|  | BN≤6.912 MHz | BN>6.912 MHz |  |
| 1 MHz to BN | -10 | -10-10 log10(BN/6,912) | 1 MHz |
| –1 MHz to -BN | -10 | -10-10 log10(BN/6,912) | 1 MHz |
| BN to 2BN | -25 | -25-10log10(BN/6,912) | 1 MHz |
| -BN to -2BN | -25 | -25-10log10(BN/6,912) | 1 MHz |
| NOTE 1: The first center frequency for a 1 MHz measurement filter in the range 1 MHz to BN is at Δfoob= 1,5 MHz. NOTE 2: The first center frequency for a 1 MHz measurement filter in the range BN to 2BN at Δfoob= BN+0,5 MHz. NOTE 3: Symmetrically similarly as in NOTE 1 and NOTE 2 in negative Δfoob frequencies. |  |  |  |

The resolution bandwidth of the measuring equipment should be equal to the Measurement BandWidth (MBW). However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the MBW or rectangular type channel filter with very steep transition response can be used. When the resolution bandwidth is smaller than the MBW, the result should be integrated over the MBW in order to obtain the equivalent noise bandwidth of the MBW.

## 2.5	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in-line with Recommendation ITU-R SM.329.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than Δfoob (MHz) in Table A4-4 from the edge of the channel bandwidth. The spurious emission limits in Table A4-5 apply for all transmitter bands and channel bandwidths.

The spurious emissions should be measured during the time period where the transmitter is active excluding any transient periods.

For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

TABLE A4-7

Spurious emission limits

| Frequency range | Maximum level (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz  f < 150 kHz | −36 | 1 kHz |
| 150 kHz  f < 30 MHz | −36 | 10 kHz |
| 30 MHz  f < 1 000 MHz | −36 | 100 kHz |
| 1 GHz  f < 12.75 GHz | −30 | 1 MHz |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge in GHz | −30 | 1 MHz |

## 2.6	Adjacent channel Leakage Ratio

Adjacent Channel Leakage Ratio (ACLR) is used as a measure of the amount of transmission power leaking into adjacent channels. ACLR is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency with measurement bandwidth equal to the transmission bandwidth.

ACLR is measured with square window on adjacent channel. A DFT of the transmission signal is taken and the energy of the appropriate bins used to calculate the adjacent channel powers.

ACLR requirement of Table A4-8 should be met for all transmission powers from –40 dBm to maximum power class 𝑃𝑀ax.

TABLE A4-8

Adjacent channel leakage ratio limit

Limit (dBc)

–30

# 3	References

[1]	ETSI TS 103 636-2 v1.5.1: DECT-2020 NR New Radio (NR); Part 2: Radio reception and transmission requirements, release 1

[2]	ETSI TS 103 636-3 v1.5.1: DECT-2020 NR New Radio (NR); Part 3: Physical layer, release 1

[3]	Recommendation ITU-R SM.329: “Unwanted emissions in the spurious domain”.

[4]	Recommendation ITU-R M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000”.

Attachment 1 
to Annex 4

Definition of test tolerance

Test tolerance

With reference to Recommendation ITU-R M.1545 [10], “test tolerance” is the relaxation value referred to in recommends 2 of Recommendation ITU-R M.1545 [10], i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per Figures 2 and 3 of Annex 1 of Recommendation ITU-R M.1545 [10]. In case the core specification value is equal to the test limit (Fig. 3 of Annex 1 of Recommendation ITU-R M.1545 [10]) the “test tolerances” are equal to 0.

The latest edition of the Recommendation/Report in force should be used.

For 3GPP-5G SRIT base stations in bands not identified for IMT in the Radio Regulations, unwanted emission characteristics in Annex 1 are provided for information, which can be used for decisions at a national level.

For 3GPP-5G RIT base stations in the bands specified in Tables 1-2B and 1-3B, unwanted emission characteristics in Annex 2 are provided for information, which can be used for decisions at a national level.

For 5Gi base stations in bands not identified for IMT in the Radio Regulations, unwanted emission characteristics in Annex 3 are provided for information, which can be used for decisions at a national level.

For DECT-2020 NR-RIT Component base stations in bands not identified for IMT in the Radio Regulations, unwanted emission characteristics in Annex 4 are provided for information, which can be used for decisions at a national level.

Developed by 3GPP Proponent as “5G, Release 15 and beyond − LTE+NR SRIT”.

Developed by 3GPP Proponent as “5G, Release 15 and beyond − NR RIT”.

Developed by TSDSI as “5Gi RIT”.