RECOMMENDATION  257-2

SELECTIVE-CALLING  SYSTEM  FOR  USE  IN  THE  MARITIME  MOBILE  SERVICE

(1959-1970-1978)

Rec. 257-2

The CCIR,

CONSIDERING

(a)	that selective calling of ships by coast stations would expedite the handling of traffic in the maritime mobile service;

(b)	that a selective-calling system has the following advantages:

–	it is rapid and positive;

–	it overcomes the disadvantages of ships having to listen to all calls;

–	it reduces the difficulties caused by differences of language;

–	it reduces the time taken by coast stations to establish contact with ships and thus reduces congestion on the channels used for calling;

–	it increases operating efficiency generally;

(c)	that a number of administrations have an immediate requirement for selective-calling facilities;

(d)	that ideally, a selective-calling system should be suitable for use in all maritime mobile bands;

(e)	that a selective-calling system should have sufficient capacity of code numbers to accommodate all ships that desire to use it;

(f)	that the International Maritime Organization (IMO) has expressed in its letter of 2 November, 1966, the opinion that it would be desirable from the viewpoint of maritime safety, if selective-calling devices could include a facility for calling all ships;

(g)	that there may be a need for a facility to call predetermined groups of ships;

(h)	that it may be operationally desirable for a ship’s operator to be able to ascertain either the identity of the coast station which has called him, or the VHF channel on which he should reply;

(j)	that a selective-calling system should be suitable for use with normal types of radio equipment on ships,

UNANIMOUSLY  RECOMMENDS

1.	that, where there is a need to fulfil immediate requirements for selective calling, the system to be used should have the following characteristics;

1.1	the selective call signal should consist of five figures representing the code number assigned to a ship for selective calling;

1.2	the audio-frequency signal applied to the input of the coast station transmitter should consist of consecutive audio-frequency pulses conforming to the following:

1.2.1	the audio frequencies used to identify the figures of the code number assigned to a ship should conform to the following series:

TABLE  I

| Figure | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | Figure repetition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Audio frequency (Hz) | 1124 | 1197 | 1275 | 1358 | 1446 | 1540 | 1640 | 1747 | 1860 | 1981 | 2110 |

For example, the series of audio-frequency pulses corresponding to the selective call 12133 
would be 1124-1197-1124-1275-2110 Hz, and the series corresponding to the code number 22222 would be 1197-2110-1197-2110-1197 Hz:

1.2.2	if the series of numbers represented by the use of only two frequencies, chosen from those in § 1.2.1, are reserved for calling predetermined groups of ships, then 100 different groups of numbers are available for allocation, according to the needs of administrations;

1.2.3	the waveforms of the audio-frequency generators should be substantially sinusoidal, not exceeding 2% total harmonic distortion;

1.2.4	the audio-frequency pulses should be transmitted sequentially;

1.2.5	the difference between the maximum amplitude of any audio-frequency pulses should not exceed 1 dB;

1.2.6	the duration of each audio-frequency pulse, measured between the half-amplitude points, should be 100 ms ± 10 ms;

1.2.7	the time interval between consecutive pulses, measured between the half-amplitude points, should be 3 ms ± 2 ms;

1.2.8	the rise and the decay time of each audio-frequency pulse, measured between the 10% and 90% amplitude points, should be 1.5 ms ± 1 ms;

1.2.9	the frequency tolerance of the audio frequencies given in § 1.2.1 should be ± 4 Hz;

1.2.10	the selective call signal (ship’s code number) should be transmitted twice with an interval of 900 ms ± 100 ms between the end of the first signal and the beginning of the second signal (Fig. 1);

1.2.11	the interval between calls from a coast station to different ships should be at least 1 s (Fig. 1); but the interval between calls to the same ship, or the same group of ships, should be at least 5 s;

Figure  1    [D01]  =  6 cm

2.	that if additional information is added to the selective call signal it should be as follows:

2.1	to identify the calling coast station four figures should be transmitted;

2.2	to identify the VHF channel on which a reply is required two “zeros” followed by two “figures” should be transmitted;

2.3	the characteristics of the signals should conform to § 1.2.1 and 1.2.3 to 1.2.9 inclusive;

2.4	the composition of the signal should be as shown in the diagram (Fig. 2), the tolerance on the 350 ms interval being ± 30 ms;

Figure  2    [D02]  =  7 cm

3.	that an “all ships call” to actuate the receiving selectors on all ships, regardless of their individual code numbers, should consist of a continuous sequential transmission of the eleven audio frequencies given in § 1.2.1. The parameters of the audio-frequency pulses should be in accordance with § 1.2.3, 1.2.4, 1.2.5 and 1.2.9. The duration of each audio-frequency pulse, measured between the half-amplitude points, should be 17 ms ± 1 ms and the interval between consecutive pulses, measured between half-amplitude points, should not exceed 1 ms (Fig. 3). The total duration of this “all ships call” signal should be at least 5 s;

Figure  3    [D03]  =  3 cm

4.	that receiving selectors on ships should operate reliably in any radio conditions acceptable for satisfactory communication;

5.	that the receiving selector should be designed to accept the signals as defined in § 1 and 3. However, bearing in mind that coast stations may transmit additional signals (e.g. coast station identification), it is important to ensure that during reception of a selective call the decoder should be re-set after 250 ± 40 ms if an incorrect digit or no digit is received;

6.	that the receiving selector should be so designed, constructed and maintained that it is resistant to atmospherics and other unwanted signals including selective-calling signals other than that for which the decoder has been set up;

7.	that the receiving selector should include an audible or visual means of indicating the receipt of a call and, if required, an additional facility allowing the determination of the identity of the calling station or the VHF channel on which to reply according to the needs of administrations;

8.	in order to distinguish whether an incoming call is a normal selective call or an “all ships call”, the multiple actuation of the ship’s decoder by the “all ships call” signal (see § 3) can be used;

9.	that the indicating means mentioned in § 7 should be actuated on correct reception of the calling signal, no matter whether the correct registration has occurred on the first, or the second, or both parts of the calling signal transmitted by the coast stations;

10.	that the indicating means should remain actuated until re-set manually;

11.	that the receiving selector equipment should be as simple as is practicable, be capable of reliable operation over long periods with a minimum of maintenance, and could, with advantage, include facilities for self-testing.

Note. – The Director, CCIR, is requested to bring the contents of this Recommendation to the attention of IMO.

_________________