RECOMMENDATION  478-4

TECHNICAL  CHARACTERISTICS  OF  EQUIPMENT  AND  PRINCIPLES
GOVERNING  THE  ALLOCATION  OF  FREQUENCY  CHANNELS
BETWEEN  25  AND  1000  MHz  FOR  THE  LAND  MOBILE  SERVICE

(Question 7/8)

(1970-1974-1978-1982-1990)

Rec. 478-4

The CCIR,

CONSIDERING

(a)	that certain technical characteristics of equipment and stations in the land mobile service are of importance in connection with radio interference between the stations of different countries;

(b)	that agreement is desirable on certain technical characteristics of land mobile equipment, to minimize mutual interference and to facilitate the use of the same types of equipment in different countries in a geographical region;

(c)	that agreement is desirable on the practices governing the choice of station antenna height and effective radiated power taking into account geographical features, required communications range and system parameters;

(d)	that agreement is desirable on the practices governing the allocation of channels in the land mobile service, in order to minimize mutual interference and to obtain economy of use of the frequency spectrum;

(e)	that in some areas, different values for the technical characteristics of equipment are required, in order to minimize mutual interference;

(f)	that the values agreed upon should be based on circumstances that typify high density radio usage areas and should be a compromise between optimum spectrum utilization and cost;

(g)	that under some circumstances, e.g. where channel assignments and/or types of system operation permit, not all recommended technical characteristics are required to minimize mutual interference;

(h)	that in the land mobile service, ultimate spectrum utilization is determined by assignment techniques, suppression and rejection of unwanted radiation, and other means additional to the actual characteristics of the equipment;

(j)	that the characteristics of non-vehicular mounted portable equipment require further study;

(k)	that in Opinion 42, the IEC has been invited to advise the CCIR of any methods of measurement applicable to radio equipment used in land mobile services,

UNANIMOUSLY RECOMMENDS

1.	that the preferred technical characteristics for VHF and UHF land mobile equipment should be as follows:

1.1	Transmitter characteristics

1.1.1	Necessary bandwidth and class of emission

1.1.1.1	For class A3E: 6 kHz

1.1.1.2	For class F3E:

30 and 25 kHz channel separations: 16 kHz

20 kHz channel separation:

in frequency bands up to 160 MHz: £ 16 kHz, dependent on deviation

in frequency bands above 160 MHz: 14 kHz

12.5 kHz channel separation: 8.5 kHz

1.1.2	Frequency tolerance

Within the temperature ranges specified by each administration according to the environment, and for specified ranges of primary supply voltages, the frequency error of any carrier emission should not exceed the values given in Table I.

TABLE  I  –  Tolerances for each frequency band

|  | Frequency band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 35 MHz |  | 80 MHz |  | 160 MHz |  | 300 MHz |  | 450 MHz |  | 900 MHz |  |
| Channel spacing (kHz) | kHz (1) | 10–6 | kHz (1) | 10–6 | kHz (1) | 10–6 | kHz (1) | 10–6 | kHz (1) | 10–6 | kHz (1) | 10–6 |
| 20, 25 and 30 | 0.7 | 20 | 1.6 | 20 | 1.6 | 10 | 2.1 | 7 | 2.25 | 5 | 2.7 | 3 |
| 12.5 | – | – | 1.0 | 12 | 1.3 | 8 | – | – | 1.35 | 3 | 1.35 | 1.5 |
| (1) Approximate values. |  |  |  |  |  |  |  |  |  |  |  |  |

1.1.3	Adjacent channel power

1.1.3.1	25 and 30 kHz channel spacing:

25-500 MHz: at least 70 dB below carrier power in a bandwidth of 16 kHz

500-1000 MHz: at least 65 dB below carrier power in a bandwidth of 16 kHz

1.1.3.2	20 kHz channel spacing:

At least 70 dB below carrier power in a bandwidth of 14 kHz, Df: 4 kHz (Df: the maximum permissible frequency deviation)

At least 60 dB below carrier power in a bandwidth of 14 kHz, Df: 5 kHz (Df: the maximum permissible frequency deviation)

1.1.3.3	12.5 kHz channel spacing:

At least 60 dB below carrier power in a bandwidth of 8.5 kHz.

In each case, it is not necessary to reduce the adjacent channel power below 0.25 mW.

1.1.4	Conducted spurious emissions

Spurious emissions on discrete frequencies, when measured in a non-reactive load equal to the nominal output impedance of the transmitter, should not exceed 2.5 mW for transmitter carrier powers up to 25 W. For carrier powers in excess of 25 W, the level of any spurious emission should be at least 70 dB below the carrier power.

In some radio environments lower values may be required.

1.1.5	Cabinet radiation

The cabinet radiated power should not exceed 25 mW. In some radio environments, a lower value may be required.

1.2	Receiver characteristics

1.2.1	Reference sensitivity

The reference sensitivity should be less than 2.0 mV, e.m.f., for a given reference signal-to-noise ratio at the output of the receiver.

1.2.2	Adjacent channel selectivity

1.2.2.1	20, 25 and 30 kHz channel spacing:

The adjacent channel selectivity should not be less than 70 dB.

1.2.2.2	12.5 kHz channel spacing:

The adjacent channel selectivity should not be less than 60 dB.

1.2.3	Radio-frequency intermodulation

The intermodulation response rejection ratio should not be less than 70 dB.

1.2.4	Spurious responses

At any frequency separated from the nominal frequency of the receiver by more than one channel spacing, the spurious response rejection ratio should not be less than 70 dB.

1.2.5	Conducted spurious emissions

The power of any spurious emission measured at the antenna terminals with matched termination, on any discrete frequency, should not exceed 2.0 nW.

In some radio environments, lower values may be required.

1.2.6	Cabinet radiation

The effective radiated power of any spurious emission on any discrete frequency up to 70 MHz should not exceed 10.0 nW. Above 70 MHz, the spurious emissions should not exceed 10 nW by more than 6 dB/octave relative to the value at 70 MHz in frequency up to 1000 MHz.

In some radio environments, lower values may be required.

1.3	Station characteristics

1.3.1	Frequency characteristics

1.3.1.1	Radio-frequency band of operation:

According to the Table of Frequency Allocations contained in Article 8 of the Radio Regulations; in particular the bands of 35, 80, 160, 300, 450 and 900 MHz.

1.3.1.2	Separation of the transmit and receive frequencies for full duplex operation:

35 MHz band: 4 MHz
  80 MHz band: 3 MHz
160 MHz band: 3 MHz
300 MHz band: 4 MHz
450 MHz band: 5 MHz

The above are practical minimum values determined by cost and isolation required; smaller separations are possible using higher quality and more costly duplexers.

900 MHz band: 45 MHz

This preferred value is determined by the desirability to provide for high capacity systems with a great number of channels. However, in some systems, greater transmit/receive frequency separation might be required.

In practice, the actual separations used may be other than the values given and may be determined by other factors than were used in this Recommendation. Frequencies should preferably be assigned with a constant separation between the transmit and receive frequencies over the whole of a band or the sub-bands within a band.

1.3.2	Effective radiated power and antenna height

It is recognized that the responsibility for limiting the effective radiated power and antenna height over the average level of the ground rests with administrations, taking into account:

–	the general requirement not to radiate more power than is necessary and not to use larger antenna heights than necessary;

–	the required range and communication quality;

–	the frequency band of operation;

–	the terrain over which service is required;

–	special conditions, e.g. diversity reception at remote receiving stations;

–	the potential intra-service or inter-service effects between the mobile service and other radio services.

1.3.3	Antenna system

Vertically polarized.

2.	that reference should be made to Part A of Report 319 for information on some of the existing practices adopted by administrations in the allocation of channels in the land mobile service between 25 and 1000 MHz;

3.	that international agreement should be reached on as many aspects as possible of the practices for the allocation of channels in the land mobile service between 25 and 1000 MHz, and that reference should be made to Part A of Report 319;

4.	that reference should also be made to the relevant IEC publications on methods of measurement.

_________________