RECOMMENDATION  489-1

TECHNICAL  CHARACTERISTICS  OF  VHF  RADIOTELEPHONE  EQUIPMENT  OPERATING
IN  THE  MARITIME  MOBILE  SERVICE  IN  CHANNELS  SPACED  BY  25  kHz

(1974-1978)

Rec. 489-1

The CCIR,

CONSIDERING

(a)	that Resolution No. 308 of the World Administrative Radio Conference, Geneva, 1979, stipulates that:

–	the channel spacing for maritime mobile VHF radiotelephone services is being reduced from 50 kHz to 25 kHz;

–	the technical characteristics of equipment for 25 kHz channel spacing in these services shall be in accordance with Appendix 19 to the Radio Regulations, and that all equipment shall conform to 25 kHz standards by 1 January 1983;

(b)	that it is desirable to supplement the list of technical characteristics given in Appendix 19 to the Radio Regulations in the interests of reduction of interference and of compatibility between equipments;

(c)	that Appendix 18 to the Radio Regulations gives a table of transmitting frequencies which is based upon the principle of 25 kHz channel separations for the maritime mobile service;

(d)	that many administrations have already prepared their national specifications for this type of equipment and consequently it is not possible at the present time to reach international agreement on the detailed specification and measurement of some equipment characteristics;

(e)	that in Opinion 42, the IEC has been invited to advise the CCIR of any methods of measurement applicable to radio equipment used in land mobile services; and that such methods of measurement may also be suitable for radio equipment used in maritime mobile services,

UNANIMOUSLY  RECOMMENDS

1.	that the following characteristics should be met by VHF (metric) FM radiotelephone equipment used for the maritime mobile services operating on the frequencies specified in Appendix 18 to the Radio Regulations;

1.1	General characteristics

1.1.1	The class of emission should be F3E/G3E.*

1.1.2	The necessary bandwidth should be 16 kHz.

1.1.3	Only phase modulation (frequency modulation with a pre-emphasis characteristic of 6 dB/octave) should be used.*

1.1.4	Where duplex or semi-duplex systems are in use, the performance of the radio equipment should continue to comply with all the requirements of this Recommendation.

1.1.5	The equipment should be designed so that frequency changes between assigned channels can be carried out within 5 s.

1.1.6	Emissions should be vertically polarized at the source.*

1.2	Transmitters

1.2.1	The frequency tolerance for coast station transmitters should not exceed 5 parts in 106, and that for ship station transmitters should not exceed 10 parts in 106.*

1.2.2	Spurious emissions on discrete frequencies, when measured in a non-reactive load equal to the nominal output impedance of the transmitter, should be in accordance with the provisions of Appendix 8 of the Radio Regulations.

1.2.3	The carrier power for coast stations should not normally exceed 50 W.*

1.2.4	The carrier power for ship station transmitters should not exceed 25 W and means should be provided to readily reduce this to 1 W or less for use at short ranges.

1.2.5	The frequency deviation should not exceed ± 5 kHz.* Deviation limiting circuits should be employed such that the maximum frequency deviation attainable should be independent of the input audio frequency.

1.2.6	The upper limit of the audio-frequency band should not exceed 3 kHz.*

1.2.7	The cabinet radiated power should not exceed 25 mW. In some radio environments, lower values may be required.

1.3	Receivers

1.3.1	The reference sensitivity should be equal to or less than 2.0 mV, e.m.f., for a given reference signal-to-noise ratio at the output of the receiver.

1.3.2	The adjacent channel selectivity should be at least 70 dB.

1.3.3	The spurious response rejection ratio should be at least 70 dB.

1.3.4	The radio frequency intermodulation rejection ratio should be at least 65 dB.

1.3.5	The power of any conducted spurious emission, measured at the antenna terminals, should not exceed 2.0 nW at any discrete frequency. In some radio environments lower values may be required.

1.3.6	The effective radiated power of any cabinet radiated spurious emission on any frequency up to 70 MHz should not exceed 10 nW. Above 70 MHz, the spurious emissions should not exceed 10 nW by more than 6 dB/octave in frequency up to 1000 MHz. In some radio environments, lower values may be required.

2.	that reference should also be made to Recommendations 331 and 332 and to the relevant IEC publications on methods of measurement.

_________________

*	See also Radio Regulations, Appendix 19.

*	See also Radio Regulations, Appendix 19.