RECOMMENDATION  490

THE  INTRODUCTION  OF  DIRECT-PRINTING  TELEGRAPH  EQUIPMENT
IN  THE  MARITIME  MOBILE  SERVICE

Equivalence of terms

(Question 5/8)

(1974)

Rec. 490

The CCIR,

CONSIDERING

(a)	that the use of direct-printing telegraphy by ship and coast stations is rapidly expanding;

(b)	that there is a need for standardizing the designation of the higher and lower F1B frequencies to ensure compatible world-wide operation when using the direct-printing telegraphy system;

UNANIMOUSLY  RECOMMENDS

that the equivalence of terms as shown in the following table shall be adhered to (this Recommendation is in accordance with CCITT Recommendations U.1 and V.1.):

TABLE  I

|  | Circuits using teleprinter or punched tape equipment |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of emission | International Telegraph Alphabet No. 2 |  |  |  |  | Transmitted 7-unit signal |  | Telex |
| Higher frequency | Space | Start | No perforation | A (1) | B (2) |  | Free line condition |  |
| Lower frequency | Mark | Stop | Perforation | Z (1) | Y (2) |  | Idle circuit condition |  |
| (1) on a wire circuit. (2) on a radio channel. |  |  |  |  |  |  |  |  |