RECOMMENDATION  550-1

USE  OF  ECHO  SUPPRESSORS  IN  THE  MARITIME  MOBILE-SATELLITE  SERVICE

(Study Programme 17A/8)

(1978-1986)

Rec. 550-1

The CCIR,

CONSIDERING

(a)	that maritime mobile-satellite systems will be interconnected with the international telephone network;

(b)	that in order to achieve transmission objectives for the maritime mobile-satellite service which are in accordance with those of other international telephone connections, it is necessary to take into account the effects of echo;

(c)	CCITT Recommendation G.131 concerning the rules governing the use of half-echo suppressors;

(d)	CCITT Recommendation G.164 concerning the characteristics of half-echo suppressors;

(e)	CCITT Recommendation G.165 concerning the characteristics of echo cancellers;

(f)	CCITT Recommendation Q.115 regarding control of echo suppressors,

UNANIMOUSLY  RECOMMENDS

1.	that half-echo suppressors or echo cancellers shall be fitted in the terrestrial part of the connection;

2.	that if two-wire telephone sets are used at the ship terminal, a half-echo suppressor or an echo canceller shall be fitted at the ship end of the connection;

3.	that if echo suppressors are fitted, they should have characteristics in accordance with CCITT Recommendation G.164;

4.	that if echo cancellers are fitted, they shall have characteristics in accordance with CCITT Recommendation G.165;

5.	that it should be possible to disable echo suppressors, as described in CCITT Recommendation G.164, in both the terrestrial and the ship end of the connection;

6.	that control of echo supressors in the terrestrial connection should follow the requirements established in CCITT Recommendation Q.115;

7.	that manual disabling of echo suppressors at the ship end of the connection should be possible;

8.	that enabling of the echo suppressor, located at the ship end of the connection, must be done automatically when a call is terminated, independently of the disabling technique used.

Note 1. – See also CCITT Recommendations G.473 and M.1100 concerning the transmission plan for maritime satellite systems.

Note 2. – For disabling of echo cancellers see CCITT Recommendation G.165, § 3.5 and 4.

Note 3. – When tone disabling is used precautions must be made to ensure that the echo suppressor or echo canceller is not disabled by speech.

Note 4. – CCITT Recommendation G.161 may still be used for the design of analogue echo suppressors.