RECOMMENDATION  587-1

COAST  STATION  IDENTITIES  AND  INITIATION  OF  LOCATION
REGISTRATION  IN  AN  AUTOMATED  VHF/UHF  MARITIME  MOBILE
TELEPHONE  SYSTEM

(Question 23/8)

(1982-1986)

Rec. 587-1

The CCIR,

CONSIDERING

(a)	Recommendation 586 (Automated VHF/UHF maritime mobile telephone system);

(b)	that in accordance with Appendix 43 of the Radio Regulations (Geneva, 1979) and CCITT Recommendation E.210/F.120, coast station identities are formed as follows:

0 0 MID X6 X7 X8 X9

where the MID reflects the country in which the coast station is located and X6, X7, X8 and X9 are any figure 
from 0 to 9;

(c)	that location registration procedures are defined as those to be followed for a ship station identity to be entered in a location register of a maritime switching centre (MSC);

(d)	that location registration procedures are initiated by ship stations;

(e)	that up to 100 coastal areas may belong to one MID and no more than 100 coast stations need to be accommodated within one coastal area;

(f)	that each individual coast station must have a unique identity,

UNANIMOUSLY  RECOMMENDS

1.	that for a given MID a coastal area be identified by the digits X6 X7;

2.	that within one coastal area a coast station be identified by the digits X8 X9;

3.	that the ship station initiates a location registration procedure when it detects a change in one or more of the digits of the sequence MID X6 X7 of the identity 0 0 MID X6 X7 X8 X9 belonging to the received coast station.