RECOMMENDATION  588

CHARACTERISTICS  OF  MARITIME  RADIO  BEACONS  (REGION  1)

(Question 58/8)

(1982)

Rec. 588

The CCIR,

CONSIDERING

(a)	Recommendation No. 602 of the World Administrative Radio Conference, (Geneva, 1979);

(b)	that provisions aimed at preventing one radio beacon from causing harmful interference to another should be inserted in the Radio Regulations;

(c)	that it is nevertheless essential to fix frequency spacings with sufficient flexibility to allow for the existing technical facilities;

(d)	that these facilities will not require the maintenance in the future of as large a frequency spacing as that adopted for Region 1 in the Radio Regulations,

UNANIMOUSLY  RECOMMENDS

1.	that frequency assignments to maritime radio beacons operating in the bands between 283.5 kHz and 335 kHz should also be determined, assuming that the radiated power is maintained at the necessary minimum value to attain the desired field strength at the service range, by the need to ensure an adequate geographical separation to avoid harmful interference between radio beacons operating on the same frequency and at the same time;

2.	that frequency assignments to maritime radio beacons should be based on multiples of 100 Hz. The adjacent frequency spacing should be sufficient to prevent one radio beacon from causing harmful interference to another in the same geographical area;

3.	that the siting of maritime radio beacons should be based on regional or sub-regional arrangements;

4.	that Administrations of Regions 2 and 3 should be invited to consider that arrangements should be made to ensure that frequency allocations are made on a compatible basis throughout the world.