RECOMMENDATION  630*

MAIN  CHARACTERISTICS  OF  TWO  FREQUENCY
SHIPBORNE  INTERROGATOR  TRANSPONDERS  (SIT)

(Question 28/8)

(1986)

Rec. 630

The CCIR,

CONSIDERING

(a)	that the use of a secondary radar system would increase the ship's radar range and improve radar discrimination of vessels in the presence of interference;

(b)	that the use of a secondary radar system would make it possible to identify individual ships and to obtain navigational and other data from an interrogated vessel;

(c)	that the combined use of a secondary radar system and of an automatic radar plotting aid (ARPA) would enhance the efficiency of the latter by increasing the range, the noise immunity of the reply co-ordinate signal and absence of fluctuation in its amplitude;

(d)	that in view of CONSIDERINGs (a), (b), and (c), the use of a secondary radar system would help to improve the safety of shipping and the economics and efficiency of shipping operations;

(e)	that in the design, preference should be given to a system using two-frequency interrogation consisting of a SIT interrogating signal and a shipborne navigation radar sounding pulse in the 3 cm band;

(f)	that the operation mode and the main operating characteristics of the system, as well as the main technical characteristics of the SIT, should be co-ordinated internationally by the users to ensure the compatibility of equipment manufactured in different countries,

UNANIMOUSLY  RECOMMENDS

that for the two-frequency SIT system operating in the 10 cm band and the 3 cm band, designed to ensure safe passage of ships and regulation of the movement of ships from a central point:

–	the operating modes should conform to Annex I,

–	the operating characteristics should conform to Annex II,

–	the technical characteristics should conform to Annex III.

ANNEX  I

OPERATING  MODES

| Interrogation |  | Response |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of | Address | Content | Capacity | Data input | Reflection |  |
| working |  |  |  |  | Type | Location |
| All ships | All ships | Polar ship co-ordinates | – | – | Screen blip | PPI radar screen |
| Selective I | Ship selected by operator | Ship station identity | 9 digit number | From continuous storage unit | Figures | Panel display |
| Selective II | Ship selected by operator | Heading Speed Manoeuvre | 3 decimal digits 2 decimal digits 1 decimal digit | From ship direction and speed sensors or manually | Figures | Panel display |
| Selective III-VI | Ship selected by operator | Determined in development of system | 6 digit number | From continuous storage unit or manually | Figures | Panel display |
| Note. – Blip on PPI radar screen is shown with all types of operation. |  |  |  |  |  |  |

ANNEX  II

MAIN  OPERATING  CHARACTERISTICS  OF
SECONDARY  MARINE  RADAR  SYSTEM

| Parameter | Value |
| --- | --- |
| Range: Minimum (m) Maximum (raised antenna, height 15 m) (nautical miles) Resolution: In direction (at a radar antenna radiation pattern of width 1°) (degrees) In distance (m): – all ships operation – selective operation Distance of response signal from echo signal (m) Error in determination of ship co-ordinates |  300  010  003  200  500  200 Not worse than radar error |

ANNEX  III

MAIN  TECHNICAL  CHARACTERISTICS  OF  SIT

| Parameter | Value |  |
| --- | --- | --- |
|  | 10-cm band | 3-cm band |
| Antenna: Polarization Radiation pattern (degrees): In horizontal plane In vertical plane Transmitter: Carrier frequency (MHz) Impulsive power (W) Length of pulse (ns) Receiver: Sensitivity (dBW) Frequency band (MHz) | Vertical 360° 40°-60° 2940  2 100-200 100  20 Better than –115 2930-2950 | Horizontal 360° 40°-60° – Better than –72 9320-9500 |
| Bit rate (Mbit/s): Of interrogating signal Of response signal Interrogating signal lead time relative to associated radar sounding pulse (s) | 2 3.3 14  0.2 |  |

*	The Director, CCIR, is requested to bring this Recommendation to the attention of the International Maritime Organization (IMO) and the International Association of Lighthouse Authorities (IALA).