RECOMMENDATION  ITU-R  M.818-1

SATELLITE  OPERATION  WITHIN  INTERNATIONAL  MOBILE
TELECOMMUNICATIONS-2000  (IMT-2000)

(Question ITU-R 39/8)

(1992-1994)

Rec. ITU-R M.818-1

The ITU Radiocommunication Assembly,

considering

a)	that the ITU-R has been studying International Mobile Telecommunications-2000 (IMT-2000) and has issued Recommendation ITU-R M.687;

b)	that the ITU-R studies are continuing;

c)	the need for a flexible system structure able to match investment to the revenue growth, to adapt readily to environmental factors and to respond to new developments without restricting innovation;

d)	the relevant ITU-T Recommendations and on-going studies;

e)	that satellite operation within IMT-2000 will enhance the overall coverage and attractiveness of the services;

f)	that satellite operation within IMT-2000 could facilitate the development of telecommunication services in developing countries;

g)	that the technical characteristics of land mobile satellite systems are being studied, and some systems are coming into operation;

h)	that in order for IMT-2000 to be available to users anywhere on land, ships and aircraft, a satellite component of IMT-2000 will be required;

j)	that satellite systems using the geostationary orbit or a non-geostationary orbit (e.g. highly inclined elliptical orbit, or low-earth orbit), or combinations thereof, may provide IMT-2000 services;

k)	that the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) has identified the bands 1 885-2 025 MHz and 2 110-2 200 MHz as being available on a worldwide basis for use by the terrestrial component of IMT-2000 and that within these bands the portions 1 980-2 010 MHz and 2 170-2 200 MHz as being available on a worldwide basis for use by the satellite component of IMT-2000 from the year 2005;

l)	that in addition to the use of the bands identified in considering k), above, for the satellite component of IMT-2000, the FSS bands may be used for feeder links and other network connections in support of IMT-2000;

m)	that a pocket-sized IMT-2000 personal station (PS) used for terrestrial communication may also be configured to operate directly with satellites as personal earth stations (PES) or indirectly via a mobile earth station (MES);

n)	that users may want to be able to use the same terminals and procedures as in the fixed network ISDN to access similar telecommunication services in IMT-2000;

o)	that particularly for an integrated PES/PS, certain design, operating, and economic advantages may be realized through adoption of features that enable use of the same hardware for the terrestrial and satellite components;

p)	that the design of the terrestrial component of the IMT-2000 should not be adversely affected by the satellite component;

q)	that the maximum compatibility of hardware and software between the satellite and terrestrial components of IMT-2000 will benefit the users and facilitate international roaming;

r)	that the satellite component of IMT-2000 will need to support Universal Personal Telecommunication (UPT);

s)	that due to lack of mobile earth terminal antenna discrimination different carrier frequencies may be required for accessing the terrestrial and satellite components of the IMT-2000 to avoid interference problems,

recommends

1.	that services provided by the IMT-2000 satellite component should include but not be limited to, the following:

–	a paging (alerting) one-way data service direct from a satellite to an IMT-2000 satellite pager (SP);

–	two-way voice or non-voice services should be provided for the following configurations:

–	service directly to/from a mobile earth station (MES);

–	service directly to/from a personal earth station (PES). The PES would comprise equipment and protocols fully or partially compatible with the terrestrial-based IMT-2000 personal station;

–	service to/from users connected by a local exchange (LX) via an MES;

–	service indirectly to/from a personal station (PS) communicating via an MES. In the case of vehicles with multiple users a cell station (CS) (cell site for PSs) may be included in the vehicle between the PSs and the MES.

Figure 1 shows some examples for satellite operation involved with IMT-2000;

2.	that the signalling protocols of the satellite component of IMT-2000 should follow the OSI model, with special emphasis on establishing physical and logical modularity for those elements that may differ between satellite and terrestrial components;

3.	that links are needed between the terrestrial and satellite network control elements of IMT-2000 to facilitate handovers and exchange of location registry data and other management information;

4.	that in the frequency bands identified by WARC-92 for operation of the IMT-2000 satellite component, account should be taken of the constraints to be established for sharing with other services;

5.	that a protocol be developed to establish whether a terrestrial or satellite component should be used for a given call;

6.	that compatible but not necessarily identical multiple access schemes should be developed for the terrestrial and satellite components;

7.	that the user presentation and operation of the PES should be as similar as possible to that of the PS;

8.	that within the bands identified, common satellite frequency channels should be used to facilitate worldwide and regional planning and operation;

9.	that the following factors:

–	service,

–	equipment (hardware and software),

–	architecture,

–	interfaces and protocols,

should be addressed in the development of detailed Recommendations relating to both the satellite and terrestrial components of IMT-2000.

The subjects of these detailed Recommendations include services, architecture, interfaces, quality of service and network management;

10.	that the service provided by the satellite component should be of comparable quality to the terrestrial component of the IMT-2000, where it is possible, bearing in mind the particular constraints of satellite systems such as power, spectrum, and propagation delay;

11.	that satellite links operating in the fixed-satellite service (FSS) bands providing the IMT-2000 service indirectly via fixed earth stations (FES) are not to be considered to be part of the satellite component of IMT-2000 but constitute an FSS connection in support of IMT-2000. An example of such a connection is illustrated in Fig. 1.

FIGURE  1/M.818...[D01] = PAGE  PLEINE