RECOMMENDATION  823

TECHNICAL  CHARACTERISTICS  OF  DIFFERENTIAL  TRANSMISSIONS  FOR  GLOBAL
NAVIGATION  SATELLITE  SYSTEMS  (GNSS)  FROM  MARITIME  RADIO  BEACONS
IN  THE  FREQUENCY  BAND  285-325 kHz
(283.5-315 kHz IN  REGION  1)

(Question 58/8)

(1992)

Rec. 823

The CCIR,

considering

a)	Resolution No. 602 of the World Administrative Radio Conference for the Mobile Services (Geneva, 1987);

b)	that according to No. 466 of the Radio Regulations, in the band 285-325 kHz (283.5-315 kHz in Region 1) in the maritime radionavigation service, radio beacon stations may also transmit supplementary navigational information using narrow-band techniques, on condition that the prime function of the beacon is not significantly degraded;

c)	Recommendation 631 on the use of hyperbolic maritime radionavigation systems in the band 283.5-315 kHz;

d)	the technical characteristics set out in the Final Acts of the Regional Administrative Conference for the Planning of the Maritime Radionavigation Service (Radio beacons) in the European Maritime Area (Geneva, 1985);

e)	that the navigational accuracy expected to be available from GNSS will be 60-100 m (with 95% probability) for general use;

f)	that this accuracy, whilst adequate for most general navigation requirements, will not be enough for some specialized navigation, such as in confined waterways and harbour approaches or for the position sensor in electronic chart systems;

g)	that other specialized maritime applications, such as fishing, navigational surveying, dredging, cable and pipe laying, positioning of buoys and other offshore structures may require higher accuracy than that available from GNSS for general use;

h)	that the navigational accuracy and integrity of GNSS can be improved considerably by the transmission of differential corrections from suitably located reference stations;

j)	that technical, economic and administrative factors indicate that the use of maritime radio beacons would be one feasible solution for the transmission of differential corrections;

k)	that propagation of transmissions from maritime radio beacons is predominantly by ground wave with a usable range that does not exceed the range of applicability of the reference station corrections;

l)	that maritime radio beacons currently provide coverage of coastal waters in many parts of the world, enabling a worldwide standard for these differential transmissions to be introduced efficiently and economically;

m)	that, although present studies have specifically addressed transmission of corrections for GPS/GLONASS, the same principles apply to terrestrial radionavigation systems, such as Loran-C/Chayka,

recommends

that the technical characteristics of a differential correction service for GNSS using maritime radio beacons, in the frequency band 285-325 kHz (283.5-315 kHz in Region 1), and for associated receivers, should be in accordance with the characteristics given in Annex 1.

ANNEX  1

System characteristics for differential GNSS

# 1.	Technical characteristics

1.1	The carrier frequency of the differential correction signal of a radiobeacon station is an integer multiple of 100 Hz*.

1.2	Frequency tolerance of the carrier is ±10 Hz.

1.3	The general message format is as shown in Fig. 1 which details the first two 30 bit words of each frame or message type. Each frame is N + 2 words long, N words containing the data of the message. The minimum message types are as shown in Table 1. Details of these message type contents and formats are as shown in Figs. 2-6. The parity algorithm used links 30 bit words within and across sub-frames of 10 words, using the (32,26) Hamming Code.

FIGURE  1  [D01]  [ex D72]  =  10 CM     changé en 9cm   =  350%  pour MEP

TABLE  1

Minimum message types

| Message type number | Title |
| --- | --- |
| 11 12 13 17 16 | Differential GNSS corrections Delta differential GNSS corrections Reference station parameters Radio beacon almanacs Special message |

FIGURE  2  [D02]  [ex D73]  =  21.5 CM

FIGURE  3  [D03]  [ex D74]  =  21.5 CM  840%   dim  à  800

This message contains the difference in the pseudorange and range-rate corrections caused by the change in satellite navigation data.

FIGURE  4  [D04]  [ex D75]  =  14 CM

FIGURE  5  [D05]  [ex D76]  = 11 CM

FIGURE  6  [D06]  [ex D77]  =  8.5 CM

1.4	The transmission of differential data is continuous.

1.5	The data transmission rate is selectable from 10-200 bit/s.

1.6	Minimum shift keying (class of emission G1D) is used. The form of modulation is 90° phase retard representing binary “0”, 90° phase advance representing binary “1”.

1.7	The MSK transmission is bandpass filtered to produce a maximum occupied bandwidth of 230 Hz at the maximum transmission rate.

1.8	The transmitting station identification codes are in binary numbers.

1.9	The receiver frequency range is at least 283.5-325 kHz, selectable in 500 Hz steps.

1.10	The receiver has a dynamic range of 5 mV/m to 100 mV/m.

1.11	The receiver operates at a maximum bit error ratio of 10–3 in the presence of Gaussian noise at a signal-to-noise ratio of 7 dB.

1.12	The receiver has adequate selectivity and frequency stability to operate with transmissions 500 Hz apart having frequency tolerances of ±10 Hz and protection ratios given in Table 2.

µTABLE  2

Protection ratios for application in the European Maritime Area (EMA)

| Frequency separation between wanted and interfering signal (kHz) | Protection ratio (dB) |  |  | Existing EMA Agreement (Geneva, 1985) |
| --- | --- | --- | --- | --- |
| Wanted _ _ _ _ _ _ _ _ _ _ _ _ _ | Radio beacon (A1A) _ _ _ _ _ _ _ _ _ _ _ _ _ | Differential (G1D) _ _ _ _ _ _ _ _ _ _ _ _ _ | Differential (G1D) _ _ _ _ _ _ _ _ _ _ _ _ _ | Radio beacon (A1A) _ _ _ _ _ _ _ _ _ _ _ _ _ |
| Interfering | Differential (G1D) | Radio beacon (A1A) | Differential (G1D) | Radio beacon (A1A) |
| 0.0 0.5 1.0 1.5 | –15 –39 –60 –60 | –10 –25 –50 –60 | –10 –25 –50 –60 | –15 –39 –60 –60 |

§

1.13	Where serial data ports are provided, these are to International Electrotechnical Commission (IEC) standards.

1.14	The receiver gives a warning indication of any failure of the data link.

1.15	Where automatic frequency selection is provided, the receiver is capable of receiving and storing reference station almanac messages.

# 2.	Protection ratios

The protection ratios for application in the European Maritime Area are as in Table 2.

_________________

*	In the European Maritime Area, the carrier frequency should be an integer multiple of 500 Hz.