RECOMMENDATION  830*

OPERATIONAL  PROCEDURES  FOR  MOBILE-SATELLITE  NETWORKS  OR  SYSTEMS
IN  THE  BANDS  1 530-1 544  MHz  AND  1 626.5-1 645.5  MHz  WHICH  ARE  USED  FOR
DISTRESS  AND  SAFETY  PURPOSES  AS  SPECIFIED  FOR  GMDSS

(Question 90/8)

(1992)

Rec. 830

The CCIR,

considering

a)	Resolution No. 208 of the World Administrative Radio Conference (Geneva, 1987);

b)	that multiple mobile-satellite networks or systems are operating or being developed to operate in the 1 530-1 544 MHz and 1 626.5-1 645.5 MHz bands;

c)	that the bands 1 530-1 544 MHz (No. N3045 of the Radio Regulations (RR)) and 1 626.5-1 645.5 MHz (RR No. N3051) available for GMDSS distress and safety communications are also available for other radio services;

d)	that with the introduction of mobile-satellite networks or systems within these frequency bands, some of which may not be participants in the GMDSS, the integrity, effectiveness and protection of distress and safety communications must be continuously maintained;

e)	that maritime mobile-satellite distress and safety communications must be protected from harmful interference;

f)	that maritime distress and safety communications require priority access with real time preemptive capability or dedicated channels in the mobile-satellite service;

g)	that account must be taken of the priority of safety-related communications (Article 61 of the RR);

h)	that maritime mobile-satellite distress and safety communications must be relayed to the appropriate Rescue Coordination Centres (RCCs) in the most rapid and expedient manner;

j)	that priority relay of distress alerts from ships in distress to the appropriate RCCs must be preserved and be in accordance with Article 61 of the RR;

k)	that internetwork or intersystem linking of mobile-satellite systems may be provided via means other than the mobile-satellite service links operating in the 1.5-1.6 GHz frequency bands,

recommends

1.	that mobile-satellite networks or systems participating in the GMDSS be equipped with the means for intersystem linking between coast earth stations;

2.	that mobile-satellite networks or systems operating in the frequency bands 1 530-1 544 MHz and 1 626.5-1 645.5 MHz participating in the GMDSS be equipped with the means to ensure that maritime mobile-satellite distress and safety communications are given the necessary priority access with real time preemptive capability or dedicated channels to ensure the most expeditious handling and relay of the messages to the appropriate RCCs.

Note 1 – Section 2 does not apply to MSS systems providing distress and safety services, for which the technical and operational characteristics have already been established in accordance with the relevant provisions of the RR or of the IMO, as applicable.

3.	that communications of mobile-satellite system stations operating in the frequency bands 1 530-1 544 MHz and 1 626.5-1 645.5 MHz not participating in the GMDSS shall operate on a secondary basis to distress and safety communications of stations operating in the GMDSS. Account shall be taken of the priority of safety-related communications in the other mobile-satellite services.

*	 	The Director, CCIR, is requested to bring this Recommendation to the attention of the International Maritime Organization (IMO), the International Civil Aviation Organization (ICAO) and the CCITT.