Recommendation ITU-R P.1144-13

(11/2025)

P Series: Radio-wave propagation

Guidelines for the application of numerical methods used in propagation methods of Radiocommunication Study Group 3

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from https://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at https://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radio-wave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2025

ITU 2025

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  P.1144-13

Guidelines for the application of numerical methods used in propagation methods of Radiocommunication Study Group 3

(1995-1999-2001-2001-2007-2009-2012-2015-06/2017-12/2017-2019-2021-2023-2025)

Scope

This Recommendation provides guidelines for the application of numerical methods used within the P-series Recommendations of Radiocommunication Study Group 3.

Keywords

Spatial interpolation, numerical integration

Glossary

| Symbol | Description |
| --- | --- |
| n | number of Gaussian quadrature points (nodes) |
| Wi | Gaussian quadrature weights |
| Xi | Gaussian quadrature points |

Other symbols not listed in the Table above are intermediate in nature and have no definition.

The ITU Radiocommunication Assembly,

considering

that there is a need to assist users of the ITU-R Recommendations P-series (developed by Radiocommunication Study Group 3),

recommends

that the information contained in the annex should be considered as guidelines for the use with the various digital maps of geophysical parameters necessary for the application of the propagation methods in the P-series Recommendations.

Annex

# 1a	Bi-linear interpolation on a trapezoidal grid

Given: Values of X at four surrounding points:Lat1, LonC, Lat1, LonD, Lat0, LonA, and Lat0, LonB; i.e. XLat1, LonC, XLat1, LonD, XLat0, LonA, and XLat0, LonB.

Problem: Determine the value XLat, Lon at an intervening point Lat, Lon using bi-linear interpolation.

Figure 1

Solution: Define two auxiliary variables, t and s:

t=Lat-Lat0Lat1-Lat0
s=Lon-LonA+t LonA-LonCLonB-LonA+t LonA-LonC+LonD-LonB

and calculate:

XLat,Lon=1-s 1-t XLat0,LonA

+1-s t XLat1,LonC

+s 1-t XLat0,LonB

+t s XLat1,LonD

# 1b	Bi-linear interpolation on a square grid

figure 2

Given: Values of I at four surrounding grid points: I(R,C), I(R,C  1), I(R  1,C), and I(R  1,C  1), where R, R + 1, C, and C + 1 are integer row and column numbers.

Problem: Determine I(r,c), where r is a fractional row number between R and R + 1 and c is a fractional column number between C and C + 1, using bi-linear interpolation.

Solution: Calculate:

I(r,c)  	I(R,C) [(R  1 – r)(C  1 – c)]

 I(R  1,C) [(r – R)(C  1 – c)]

 I(R,C  1) [(R  1 – r)(c – C)]

 I(R  1,C  1) [(r – R)(c – C)]

# 2	Bi-cubic interpolation

figure 3

Given: Values of I at 16 surrounding grid points:

I(R,C), I(R,C  1), I(R,C  2), I(R,C  3),

I(R  1,C), I(R  1,C  1), I(R  1,C  2), I(R  1,C  3),

I(R  2,C), I(R  2,C  1), I(R  2,C  2), I(R  2,C  3),

I(R  3,C), I(R  C  1), I(R  3,C  2), I(R  3,C  3)

where R, R + 1, etc.; and C, C + 1, etc. are integers.

Problem: Calculate I(r,c), where r is a fractional row number between R + 1 and R + 2 and c is a fractional column number between C + 1 and C + 2, using bi-cubic interpolation.

Solution:

Step 1: For each row, X, where X  {R, R  1, R  2, R  3}, compute the interpolated value at the desired fractional column c as:

RIX,c=j=CC+3IX,j Kc-j

where:

Kδ=a+2δ3-a+3δ2+1for0≤δ≤1aδ3-5aδ2+8aδ-4afor1≤δ≤20                                                    for2≤δ

and

a=-0.5

Step 2: Calculate I(r,c) by interpolating the one-dimensional interpolations, RI(R,c), RI(R  1,c), RI(R  2,c), and RI(R  3,c) in the same manner as the row interpolations.

# 3	Gaussian quadrature integration

Gaussian quadrature integration is accurately approximate to a definite integral if the integrand, f(x), is well-approximated by a polynomial of degree 2n-1 or less over the integration interval. The value of n should be selected based on the desired approximation accuracy.

## 3.1	Single integral

A single integral can be well-approximated by Gaussian quadrature integration noting that:

abfxdx=-11b-a2fa+b2+b-a2tdt

≈i=1nWi'fXi'

where:

Wi'=b-a2 Wi

Xi'=a+b2+b-a2Xi

## 3.2	Double integral

A double integral can be well-approximated by Gaussian quadrature integration noting that:

abcdfx,y dx dy=-11-11b-a2d-c2 f(a+b2+b-a2s,c+d2+d-c2t) ds dt

≈i=1nj=1nWi' Yj' fXi', Zj'

where:

Wi'=b-a2 Wi

Xi'=a+b2+b-a2Xi

Yj'=d-c2 Wj

Zj'=c+d2+d-c2Xj

## 3.3	Algorithm to calculate Gaussian quadrature points (nodes) and weights

This algorithm calculates the points (nodes), Xi, and weights, Wi, for i=1, 2,….,n, where n is the number of Gaussian quadrature points (nodes). The variable eps is the accuracy of the machine’s floating-point system. On machines that support IEEE floating point arithmetic, eps is approximately 2.2204e-16 for double precision. The function floorx rounds x to the nearest integer less than or equal to x.

Step 1: Calculate m=floorn+12

Repeat Steps 2 to 13 for i=1 to m

Step 2: Calculate Xi=cos4i-14n+2π

Step 3: Calculate Pm1=1 and P=Xi

Repeat Steps 4 and 5 for j=2 to n

Step 4: Calculate Pm2=Pm1 and Pm1=P

Step 5: Calculate P=2-1jXi Pm1-1-1jPm2

Step 6: Calculate P'=n ∙ Xi ∙ P-Pm1Xi2-1

Step 7: Calculate Δ=PP'

Step 8: Calculate Xi=Xi-Δ

Step 9: If Δ>eps then go to Step 3, otherwise go to Step 10

Step 10: Calculate Xlast=Xi+Δ

Step 11: Calculate Pm1'=n-1 ∙ Xlast ∙ Pm1-Pm2)Xlast2-1

Step 12: Calculate Pm1=Pm1-Δ∙ Pm1'

Step 13: Calculate Wi=2 1-Xi2n ∙ Pm12

Repeat Step 14 for i=m+1 to n

Step 14: Calculate Xi=-Xfloorn2+m+1-i and Wi=Wfloorn2+m+1-i

Example values of Xi, the Gaussian quadrature points, and Wi, the Gaussian quadrature weights, are provided in a supplemental product on the ITU-R Study Group 3 website on digital products.