RECOMMENDATION  ITU-R  P.1147

PREDICTION  OF  SKY-WAVE  FIELD  STRENGTH  AT  FREQUENCIES
BETWEEN  ABOUT  150  AND  1 700  kHz

(Question ITU-R 215/3)

(1995)

Rec. ITU-R P.1147

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to give guidance to engineers in the planning of broadcast services in the LF and MF bands;

b)	that it is important, for stations working in the same or adjacent frequency channels, to determine the minimum geographical separation required to avoid interference resulting from long-distance ionospheric propagation;

c)	that portions of this band are being shared by broadcasting and other services in different Regions, an accurate method for predicting interference levels is needed to maintain efficient and orderly utilization of those portions of the spectrum,

recommends

that the following method be adopted for use, taking particular note of the cautions on accuracy in its application to certain regions as discussed in Annex 1.

# 1	Introduction

This method predicts values of the night-time sky-wave field strength for a given power radiated from one or more vertical antennas, when measured by a loop antenna at ground level aligned in a vertical plane along the great-circle path to the transmitter. It applies for paths of length 50 to 12 000 km in the LF and MF bands. For a discussion on daytime propagation, see Annex 2.

Figures 1, 2 and 3 are an essential part of the prediction method. Geomagnetic maps are included for convenience in Figs. 12, 13 and 17. The remaining figures and Appendix 1, provide additional information to simplify the use of the method.

# 2	Annual median night-time field strength

The predicted sky-wave field strength is given by:

E    V    E0  –  Lt    V    GS  –  Lp    A  –  20 log p  –  La  –  Lt  –  Lr	(1)

where:

E :	annual median of half-hourly median field strengths (dB(V/m)) for a given transmitter cymomotive force, V, and at a given time, t, relative to sunset or sunrise as appropriate

E0 :	annual median of half-hourly median field strengths (dB(V/m)) for a transmitter cymomotive force of 300 V at the reference time defined in § 2.1

V :	transmitter cymomotive force (dB above a reference cymomotive force of 300 V) (see § 2.2)

GS :	sea-gain correction (dB) (see § 2.3)

Lp :	excess polarization-coupling loss (dB) (see § 2.4)

A :	for frequencies up to and including 1 600 kHz, A  106.6 – 2 sin , where  is the (centred dipole) geomagnetic latitude of the mid-point of the path under study (see § 2.6). For frequencies greater than 1 600 kHz, A  107

La :	loss factor incorporating effects of ionospheric absorption and related factors (see § 2.6)

Lt :	hourly loss factor (dB) (see § 2.7)

Lr :	loss factor incorporating effect of solar activity (§ 2.8).

To facilitate calculation, Fig. 4 shows the quantity A – 20 log p, for   40 as a function of ground distance, d, whereas Figs. 5 to 11 show E0 as a function of ground distance, d, for various frequencies and for various geomagnetic latitudes when GS, Lp and R are all zero; where R is the twelve-month smoothed international relative sunspot number.

## 2.1	Reference time

The reference time is taken as six hours after the time at which the Sun sets at a point S on the surface of the Earth. For paths shorter than 2 000 km, S is the mid-point of the path. On longer paths, S is 750 km from the terminal where the Sun sets last, measured along the great-circle path.

## 2.2	Cymomotive force

The transmitter cymomotive force V (dB (300 V)) is given as:

V    P    GV    GH	(2)

where:

P :	radiated power (dB(1 kW))

GV :	transmitting antenna gain factor (dB) due to vertical directivity, given in Fig. 1

GH :	transmitting antenna gain factor (dB) due to horizontal directivity. For directional antennas, GH is a function of azimuth. For omnidirectional antennas, GH  0.

## 2.3	Sea gain

The sea gain GS is the additional signal gain when one or both terminals is situated near the sea, but it does not apply to propagation over fresh water. GS for a single terminal is given by:

where:

G0 :	sea gain when the terminal is on the coast and the path is unobstructed by further land (dB)

c1 :	correction to take account of the distance between the terminal and the sea

c2 :	correction to take account of the width of one or more sea channels, or the presence of islands.

If both terminals are near the sea, GS is the sum of the values for the individual terminals.

G0 is given in Fig. 2 as a function of d for LF and MF. At MF, G0  10 dB when d  6 500 km; and at LF G0  4.1 dB when d > 5 000 km, where d is the ground distance between the two terminals.

The correction c1 is given by:

(5)

where:

s1 :	distance of terminal from sea, measured along great-circle path (km)

r1 

f :	frequency (kHz)

Q1 	0.30 at LF and 1.4 at MF.

The correction c2 is given by:

where:

s2 :	distance of terminal from next section of land, measured along the great-circle path (km)

r2 

Q2 	0.25 at LF and 1.2 at MF

 :	proportion of land in the section of path between r2 and s2 (0    1).

If a computer is used but a terrain data bank is not available to calculate , then  should be made equal to 0.5, which implies that land and sea are present in equal proportions in the section of path between r2 and s2.

To facilitate calculation, Fig. 15a shows r1, the greatest distance from the sea for which sea gain has to be calculated, and Fig. 15b shows r2, the greatest distance to the next section of land for which the correction c2 is required, for various frequencies.

## 2.4	Polarization coupling loss

Lp is the excess polarization coupling loss (dB). At LF, Lp  0. At MF, Lp for a single terminal is given by one of the following two formulae:

(8)

where I is the magnetic dip, N or S (degrees) at the terminal and  is the path azimuth measured in degrees from the magnetic E-W direction, such that |  |  90. Lp should be evaluated separately for the two terminals, because of the different values of  and I that may apply, and the two Lp values added. The most accurate available values of magnetic dip and declination (e.g. see Figs. 12 and 13) should be used in determining  and I.

Figure 14 shows values of Lp calculated from equation (8).

## 2.5	Slant propagation distance, p

For paths longer than 1 000 km, p (km) is approximately equal to the ground distance, d (km) between transmitter and receiver. For shorter paths:

p    (d 2    40 000)½	(9)

Equation (9) may be used for paths of any length with negligible error. It should be used in all cases where the distances considered are both above and below 1 000 km, to avoid discontinuities in field strength as a function of distance.

FIGURE 1/P.1147...[D01] = page pleine

FIGURE 2/P.1147...[D02] = 14 CM

FIGURE 3/P.1147...[D03] = 7 CM

## 2.6	Loss factor incorporating effects of ionospheric absorption and related factors

La is the loss factor incorporating effects of ionospheric absorption, focusing, terminal losses and loss between hops of multi-hop paths.

For frequencies between 150 and 1 600 kHz:

La    k p    10–3	(10a)

and the basic loss coefficient k is given by:

k    3.2    0.19 f 0.4  tan 2 (    3)	(11a)

For frequencies greater than 1 600 kHz:

(10b)

and the basic loss coefficient k is given by:

k    (2    4.95 tan2 )	(11b)

In equations (11a) and (11b),  f  is frequency (kHz) and  is the (dipole) geomagnetic latitude of the mid-point of the path under study. Northern latitudes are considered positive, southern latitudes negative. Figure 17 may be used to obtain the geomagnetic latitude from known geographical coordinates of the mid-point. If the value of  is greater than 60, equations (11a) and (11b) are evaluated for   60. If the value of  is less than –60, equations (11a) and (11b) are evaluated for   –60. Paths longer than 3 000 km are divided into two equal sections, k for each section is determined according to equation (11a) or (11b). The algebraic average of the two values of k (keff ) is then used in calculating field strength (see also Figs. 16a and 16b).

## 2.7	Hourly loss factor

The hourly loss factor, Lt (dB) is given in Fig. 3. The time t is the time in hours relative to the sunrise or sunset time as appropriate. These are taken at the ground at the mid-path position for d  2 000 km and at 750 km from the terminal where the Sun sets or rises first for longer paths. The large values of hourly loss factor near midday are not defined (Fig. 3). For times in this period use a limit value of 30 dB. The hourly loss factor should not be calculated for high latitude paths and seasons when sunrise and sunset do not occur.

Equations generally equivalent to these curves to within 0.5 dB are given in § 1 of Appendix 1. Figure 3 represents the average annual diurnal variation.

Figure 18 shows sunset and sunrise times for a range of geographic latitudes and months. Equations equivalent to these sunset and sunrise curves are given in § 2 of Appendix 1.

## 2.8	Loss factor incorporating effect of solar activity

Lr is the loss factor incorporating effect of solar activity and is given by the following formulae:

at LF,	Lr  0

at MF,	if	|  |  45,	Lr  0

at MF,	if	|  |  45,	Lr  b(R/100) (p/1 000)               dB	(12)

where:

b  (|  | – 45)/3	except in Europe where b  1 is to be used	(13)
regardless of latitude.

Paths longer than 3 000 km are divided into two equal sections as described in § 2.6. The value of Lr for each section is derived and added together.

# 3	Day-to-day and short-period variations of night-time field strengths

The difference between field strength exceeded for 10% of the time and the annual median value, , is given by:

at LF,	  6.5	dB	(14a)

at MF,	  0.2 |  | – 2	dB	(14b)

 is greater than or equal to 6 dB but less than or equal to 10 dB.

FIGURE 4/P.1147...[D04] = 13 CM

FIGURE 5/P.1147...[D05] = page pleine

FIGURE 6/P.1147...[D06] = page pleine

FIGURE 7/P.1147...[D07] = page pleine

FIGURE 8/P.1147...[D08] = page pleine

FIGURE 9/P.1147...[D09] = page pleine

FIGURE 10/P.1147...[D10] = page pleine

FIGURE 11/P.1147...[D11] = page pleine

IGURE 12/P.1147...[D12] = page pleine

FIGURE 13/P.1147...[D13] = page pleine

FIGURE 14/P.1147...[D14] = 13 CM

FIGURE 15a/P.1147...[D15] = page pleine

FIGURE 15b/P.1147...[D16] = page pleine

FIGURE 16a/P.1147...[D17] = page pleine

FIGURE 16b/P.1147...[D18] = page pleine

FIGURE 17/P.1147...[D19] = page pleine

FIGURE 18/P.1147...[D20] = 16 cm

APPENDIX  1

This Appendix contains equations that may be used in lieu of Figs. 3 and 18 for hourly loss factor, and sunset and sunrise times respectively. For the purpose of this Appendix, the following additional symbols are used.

List of symbols

 :	geographic latitude of a point on the path (degrees)

 :	geographic longitude of a point on the path (degrees)

S :	local mean time of sunset or sunrise at a point (h).

North and East coordinates are considered positive, and South and West coordinates negative.

# 1	Hourly loss factor: Lt

These equations may be used instead of the curves in Fig. 3, within the stated limits of t. For hours between these times (i.e. near midnight) set Lt  0.

Lt  (sunset)    12.40  –  9.248 t    2.892 t2  –  0.3343 t3	for –1  t (sunset)  4

Lt  (sunrise)    9.6    12.2 t    5.62 t2    0.86 t3	for –3  t (sunrise)  1

where t is the time in hours relative to sunset or sunrise at the path mid-point.

# 2	Sunset and sunrise times

For non-polar locations, i.e. such that |  |  65, the times of sunset and sunrise can be calculated as follows, to an accuracy of  2 min:

N :	day of year, in days; e.g. 1 January  1

S :	approximate local time of event, e.g. sunset  1800 h, sunrise  0600 h

Z :	Sun’s zenith distance (degrees)  90.8333(9050 ) for sunset or sunrise.

Step 1:  Calculate observer’s longitude, B:

B     / 151               h

Step 2:  Calculate the time of event, Y:

Y    N    (S  –  B) / 24               days

Step 3:  Calculate Sun’s mean anomaly, M:

M    0.985600  Y  –  3.289               degrees

Step 4:  Calculate Sun’s longitude, L:

L    M    1.916 sin M    0.020 sin 2 M    282.634               degrees

Note in which quadrant L occurs.

Step 5:  Calculate Sun’s right ascension, RA:

tan RA    0.91746  tan L

Note that RA must be in the same quadrant as L.

Step 6:  Calculate Sun’s declination, s:

sin s  0.39782 sin L, from which:

Note that sin s may be positive or negative but cos s must always be positive.

Step 7:  Calculate Sun’s local hour angle, H:

cos H    x    (cos Z  –  sin s · sin ) / (cos s · cos )

Note that if | x |  1, there is no sunset or sunrise.

From cos H, obtain H in degrees; for sunrise 180  H  360; for sunset 0  H  180.

Step 8:  Calculate local mean time of event, S :

S    H / 15    RA / 15  –  0.065710 Y  –  6.622

Note that S is expressed in hours and that multiples of 24 should be added or subtracted until 0  S  24.

Note that S is the local time at the point concerned. The corresponding standard time is S – B  m /15 h where m is the longitude of the standard meridian for the desired time zone (degrees) so that, for example, universal time  S – B.

ANNEX  1

Accuracy of method

The method applies for paths of length 50 to 12 000 km in the LF and MF bands. However, at LF it has only been verified for paths up to 5 000 km. The accuracy of prediction varies from region to region and may be improved in certain regions by applying modifications such as those discussed below.

The asymmetry so evident in the basic loss factor (see Fig. 16a) might have been removed if “corrected geomagnetic latitude” had been used rather than geomagnetic latitude. In any case, the method should be used with caution for geomagnetic latitudes greater than 60.

Field strengths measured in the United States of America and Brazil tend to be greater at higher frequencies; the frequency variation given by equation (11a) is in the opposite sense. For these and other reasons, the method should be used provisionally in Region 2.

Equation (6) which describes how GS is modified by the distance s2 to the next section of land is derived from theory and must therefore be regarded as tentative until measurements are available.

The method predicts the field strength which is likely to be observed if the transmitter and receiver are situated on ground of average conductivity, typically 3 to 10 mS/m. In certain areas (e.g. see Recommendation ITU-R P.832), the effective ground conductivity can be as low as 0.5 mS/m or as high as 40 mS/m. If the ground conductivity at either terminal is an order of magnitude smaller than 10 mS/m, then the field strength may be up to 10 dB smaller. If the ground conductivity at both terminals is an order of magnitude smaller, then the field-strength reduction will be doubled. The amount of attenuation is a function of path length and is greatest for waves approaching grazing incidence. The method may be improved by using a correction for ground conductivity when it differs significantly from that for average ground, for example by using the information contained in Recommendation ITU-R P.684 and Report ITU-R P.575.

The method assumes that reflection takes place only via the E layer, or that E-layer reflections predominate. However, if f  (foE) sec i, where foE is the critical frequency of the E layer and i is the angle of incidence at the E layer, then the wave will penetrate the E layer and be reflected from the F layer. This is most likely to occur at the highest frequencies in the MF band at ground distances less than 500 km, especially late at night and during the sunspot minimum period. The method can still be used provided p is calculated for an F-layer reflection height of 220 km and the cymomotive force V is calculated for the corresponding angle of elevation.

Measurements made in the United States of America suggest that Fig. 3 (hourly loss factor) is likely to be accurate for frequencies near 1 000 kHz in a year of low solar activity. As the frequency deviates in either direction from about 1 000 kHz, particularly during transition hours, appreciable errors may result. These measurements also suggest that the magnitude of the effect of solar activity at two hours after sunset is considerably greater than that at six hours after sunset. Thus, in a year of high solar activity, the difference between field strengths at six hours after sunset and two hours after sunset can be considerably greater than that shown in Fig. 3.

At night, MF sky-waves propagating in temperate latitudes are strongest in spring and autumn and are weakest in summer and winter, the summer minimum being the more pronounced. The overall variation may be as much as 15 dB at the lowest frequencies in the MF band, decreasing to about 3 dB at the upper end of the band. At LF the seasonal variation at night has the opposite trend, with a pronounced summer maximum. The seasonal variation is much smaller in tropical latitudes.

ANNEX  2

A discussion on daytime sky-wave propagation

# 1	LF cases

Midday field strengths at LF are 7 to 45 dB lower than the values at midnight. The difference is dependent on frequency, distance and season (see also Recommendation ITU-R P.684).

# 2	MF cases

Available data show that midday sky-wave field strengths display a consistent seasonal variation pattern with maximum occurring in winter months. The average winter-month field strength is about 10 dB stronger than the annual median value and the winter-to-summer ratio can exceed 30 dB. The annual median value of midday field strength is about 43 dB lower than its counterpart at six hours after sunset. Field strength exceeded for 10% of the days of the year is about 13 dB stronger than the annual median value. See also Report ITU-R P.431 for observations made in Japan, the United States of America and European broadcasting areas.