Readme File for Recommendation P.1511 Data Files

The EGM2008.txt file contains hEGM2008, the undulation in the 2008 version of the U.S. National Geospatial-Intelligence Agency (NGA) Earth Gravitational Model.

The EGM2008.txt data file has 2 165 rows and 4 325 columns, where each row corresponds to a geodetic latitude, each column corresponds to a geodetic longitude, and the latitude and longitude row and column increments are 1/12° ≈ 0.08333°. The file contains two additional rows at the top and bottom and two additional columns at the left and right which wrap around the Earth to facilitate the bi-cubic interpolation procedure. For example, the (Latitude, Longitude) in the five rows and five columns in the upper left corner of the data file are as follows, where the precision has been truncated to five decimal places.

| (+89.83333°, -0.16667°) | (+89.83333°, -0.08333°) | (+89.83333°, 0°) | (+89.83333°, +0.08333°) | (+89.83333°, +0.16667°) |
| --- | --- | --- | --- | --- |
| (+89.91667°, -0.16667°) | (+89.91667°, -0.08333°) | (+89.91667°, 0°) | (+89.91667°, +0.08333°) | (+89.91667°, +0.16667°) |
| (+90.00000°,+179.83333°) | (+90.00000°,+179.91667°) | (+90.00000°,-180°) | (+90.00000°,-179.91667°) | (+90.00000°,-179.83333°) |
| (+89.91667°,+179.83333°) | (+89.91667°,+179.91667°) | (+89.91667°,-180°) | (+89.91667°,-179.91667°) | (+89.91667°,-179.83333°) |
| (+89.83333°,+179.83333°) | (+89.83333°,+179.91667°) | (+89.83333°,-180°) | (+89.83333°,-179.91667°) | (+89.83333°,-179.83333°) |

Depending on the interpolation method (e.g. the Matlab bi-cubic interpolation function interp2(..) which requires strictly monotonic latitude and longitude values), it may be convenient to assume the upper left corner corresponds to a “virtual” latitude of +90+2/12° = +90.166667° and “virtual” longitude of -180-2/12° = -180.166667°, and the row and column increments are 1/12° ≈ 0.08333°.

The undulation in the 2008 version of the U.S. National Geospatial-Intelligence Agency (NGA) Earth Gravitational Model at any location on the surface of the Earth can be obtained using the bi-cubic interpolation procedure provided in Annex 1 of Recommendation ITU-R P.1144.

Matlab users should note that the Matlab bi-cubic interpolation function interp2(..) is identical to the bi-cubic interpolation procedure provided in Annex 1 of Recommendation ITU-R P.1144; however, Octave users should note that the Octave bi-cubic interpolations function interp2(..) is different than the bi-cubic interpolation procedure in Annex 1 of Recommendation ITU-R P.1144.

The Earth Gravitational Model 2008 (EGM2008) is a product of the National Geospatial-Intelligence Agency (NGA). The ITU does not, expressed or implied, take ownership, possession, or make any other claim on EGM2008, or restrict or limit in any way the free and open use of EGM2008 by the public.